HP-BASIC  KERMIT USER GUIDE

For The Hewlett-Packard Family 9000 Series 200 and 300 Computers

HP-BASIC  WORKSTATIONS

VERSION 1.0

Andrew Campagnola

Hewlett-Packard

Measurement Systems Operation

815 SW 14 St. Loveland, Colorado  80537

Copyright (C) 1981,1988

Trustees of Columbia University in the City of New York

Permission is granted to any individual or institution to use, copy,

or redistribute this document so long as it is not sold for profit.

Kermit-RMB   Table Of Contents

Section              Topic

1.0             Kermit-RMB Title Block

2.0             History Of Kermit-RMB

3.0             Kermit-RMB Capability Summary

4.0             Computer System Requirements

5.0             Using Kermit-RMB

5.1     Examples of Kermit-RMB usage

5.2     Connecting HP Workstations for Kermit

5.2.1   Connecting the 98644 (9 Pin) Interface

5.2.2   Connecting the 98626/98628 Interfaces

5.2.3   Connecting to Modems

5.3     Testing the RS-232 Connection

6.0             The HP-BASIC File System

6.1     Specifying Filenames and Mass Storage

6.2     BASIC File Formats

6.3     Kermit Handling of Files

6.3.1   Sending Files

6.3.2   Receiving Files

7.0             Kermit Program Setup

7.1     The Kermit Distribution Disc

7.2     Printing the User Manual

7.3     Loading and Running Kermit

7.4     Minimizing Kermit Program Size

7.5     Kermit Initialization File HPK_INIT

7.6     Ram Disc Operation

8.0             Kermit-RMB Command Descriptions

8.1     Kermit Command Syntax Reference

8.2     Unique Commands for Kermit-RMB

8.3     General Kermit Command Groups

8.4     Program Management Commands

8.5     Local File Management Commands

8.6     Terminal Connection and Emulator Commands

8.7     File Transfer Commands

8.8     Ascii File Transmission Commands

8.9     Kermit SET Command Options

8.10    Status and Show Commands

8.11    Log and Close Commands

8.12    Macro Definitions

8.13    Kermit Server Commands (Not Implemented)

8.14    Remote Server Commands (Not Implemented)

9.0             Missing Features in Kermit-RMB

Future Enhancements

Bug Reports

Kermit-RMB 1.0   Users Manual                                  Page 2

1.  Kermit-RMB

This document is available either as an ordinary, plain DOS

text disk file, or a HP-LIF ASCII file.

Program:  Andrew Campagnola - Hewlett-Packard Company

Language:  Hewlett-Packard Series 200/300 BASIC (Rev.  5.x)

Version:  1.0

Release:  1 Mar 1989

2.  History

This is the first full version of KERMIT for the HP BASIC

Workstation written.  Keith Moore wrote a BASIC-CSUB which

implements the KERMIT send and receive protocol for ascii

files while at New Jersey Division Labs.  His efforts there,

and a positive response to the availability of his KERMIT for

HP BASIC combined with the universal need for a native BASIC

terminal emulator spurred on this project.

The KERMIT file transfer protocol was developed at Columbia

University in 1981 under the direction of Frank DaCruz and

first written by Bill Catchings for CP/M at the Center for

Computing Activities.  Now over 200 versions of the KERMIT

program are monitored and supported by that group.

KERMIT received its name from a muppets poster on the wall

during the conception of the protocol at Columbia in 1981.

After attempts to define K-E-R-M-I-T as an acronym failed it

was admitted that KERMIT was named after KERMIT the Frog with

Jim Henson's permission.  It was later discovered that KERMIT

is a Celtic word for "free", and other justifications arose.

Like all Kermit programs, you are encouraged to use, copy, and

distribute HP-BASIC Kermit freely.

Kermit-RMB 1.0   Users Manual                                  Page 1

3.       Kermit-RMB Capabilities At A Glance:

Local operation:                Yes

Remote operation:               Yes  Requires HP NJD REMCON BIN

Transfers text files:           Yes

Transfers binary files:         Yes  (BDAT, ASCII, HP-UX)

Wildcard send:                  No

File transfer interruption:     Yes

Filename collision avoidance:   Yes

Can time out:                   Yes

8th-bit prefixing:              Yes

Repeat count compression:       Yes  - receive only

Alternate block check types:    No

Terminal emulation:             VT100

Communication settings:         Speed, Parity, Echo   (Full duplex only)

Transmit BREAK:                 Yes

IBM mainframe communication:    No

Transaction (Packet) logging:   Yes

Session logging (raw download): Yes

Raw upload (Transmit)           Yes

Act as server:                  No

Talk to server:                 No

Advanced server functions:      No

Advanced commands for servers:  No

Local file management:          Yes

Command/init files:             Yes

Command macros:                 Yes

Extended-length packets:        No

Attribute packets:              Yes

Sliding windows:                No

Kermit-RMB is a program that implements the Kermit file

transfer protocol for the entire HP Series 200/300 BASIC

Workstation family of computers.

This user manual assumes that you are new to Kermit but

familiar with the operation of the HP BASIC Workstation and

ideas of data communication.  A very brief overview is given

here, but for details consult the early chapters of the

Kermit User Guide, or Frank DaCruz' book:

Kermit, A File Transfer Protocol, by

Frank da Cruz

Digital Press (1987)

order number EY-6705E-DP

(phone 1-800-343-8321)

which also includes excellent background tutorials on

computers, file systems, and data communication (including

modems, cabling, etc).

Kermit-RMB 1.0   Users Manual                                  Page 2

For further information about obtaining Kermit programs and

user documentation, write to:

Columbia University

Center for Computing Activities

Kermit Distribution

612 West 115th Street

New York, NY  10025  (USA)

4.0      System Requirements

Kermit-RMB was developed on a 9816 Workstation using BASIC

5.1.  The 9816 was configured with 1Mb RAM and a 98626 Serial

Interface Card. A Memory Volume was not used.

Kermit-RMB version 1.0 runs on any Series 200/300 computer

with 1 Mb of memory.  BASIC 5.x is required.  The Operating

System and Kermit program will load and run in less than 1 Mb

but without the use of a Memory Volume.

Kermit will run on any HP serial interface available for the

Series 200 and 300 computers. These include the following:

98644    Built-in 25 Pin Serial Interface on the S/310

98644    Built-in 9 Pin Serial Interface on S/320 and up

98626    50 Pin Serial Interface Card fro S/200 and S/300

98628    50 Pin Datacomm Interface Card for S/200 and S/300

Kermit-RMB implements both Ascii and Binary file transfer as

well as providing a near complete Kermit user interface.

File transfer is provided using both KERMIT Protocol Packets

(SEND-RECEIVE) and raw upload and download (TRANSMIT - LOG

SESSION).  Aside from the normal functions of Kermit for file

transfer, Kermit-RMB also provides useful functions such as:

ASCII file printing to screen and printer     PRINT / TYPE

A Hex Editor and dump utility                 DUMP

VT-100 Terminal emulation                     CONNECT

A File Type Conversion Utility                CONVERT

Kermit-RMB 1.0   Users Manual                                  Page 3

5.0   Using Kermit-RMB

Kermit-RMB performs two major functions, terminal emulation

and file transfer.  File transfer can be done using either

the Kermit file transfer protocol (SEND & RECEIVE) , or else

(without error checking), raw ASCII transmission methods.  To

use Kermit for "raw" uploading or downloading of files, see

the descriptions of the TRANSMIT and LOG SESSION commands.

Two computers use the KERMIT transfer protocol to send files to

each other by running a KERMIT program on each computer. This is

an important point - Kermit running on one computer cannot send

or receive files from another computer running some other file

transfer program (ie. Xmodem, or HP Advancelink) because the

protocols (or encoding) used is not the same. Therefore one key

advantage of using Kermit is that many computers on the market

have a version of Kermit written for them. You should be aware

that many terminal emulator programs, especially for the PC

contain the capability of performing file transfers using a

variety of protocols, including Kermit. Check into this before

assuming your terminal emulator cannot support Kermit.

If you are trying to transfer files to a computer system and do

not have a Kermit program for that system you should contact

Columbia University to see if one exists.

Without Kermit running on each computer, your only other option

is to use simple ascii transmission which does not "encode" files

using any protocol.  For example the Kermit Transmit command

could be used with most any PC Terminal Emulator that performs

"File Logging".  The disadvantage of using this method is the

lack of binary file transfers or parity error detection.

5.1  Kermit - to - Kermit File Transfer

Once you have obtained versions of Kermit for each computer

on each end of the file transfer you are ready to begin making

the RS-232 connection and transferring files. Making the physical

connection is discussed in the next section after this overview

of using Kermit.

There are two scenarios for file transfers I'll discuss here and

give examples for. One involves transferring files between two

computers that are next to each other (let's say in the same

lab), and the other scenario of transferring files between two

computers that are remote to each other (let's say from a local

instrument controller to a remote VAX in the computer center

- or in a remote building accessed via modem).  Remember -

both scenarios involve an RS-232 connection between the two

computers that must be established first.

(1) Local - to - local  File Transfer

Kermit-RMB 1.0   Users Manual                                  Page 4

Assume you have connected a Series 300 to a Personal Computer

in your lab using the HP-24542G Serial Printer Cable (This

conenection is discussed later). To transfer a file from the

Series 300 to the PC would involve the following:

A.  Start each computer, load and run Kermit on each side.

Kermit comes up on each computer with its local prompt:

MS-KERMIT> may be the prompt you see on the PC - MSDOS version

and

KERMIT-RMB> is the prompt you'll see on the HP side.

B.  Configure the receiver (PC) and begin the receive process:

You tell Kermit what to do by issuing commands to kermit at its

prompt the same way you issue commands to DOS or Unix.

MS-KERMIT> SET BAUD 9600      (19200 would work also)

MS-KERMIT> SET TIMER OFF      (Let the sender be the timer)

MS-KERMIT> RECEIVE

Kermit on the PC will begin the receive process and you will

see the Kermit receive screen which shows you the status of the

file transfer. (The screen may vary depending on the version)

At this point the MS-Kermit is waiting to receive a "start"

signal from the sending Kermit (HP side). Normally Kermit will

wait for a signal for just so long and then "timeout" and quit

trying to receive a file if it hasn't heard from the sender.

Since we shut this off by using the SET TIMER OFF command, the

MS-Kermit will now wait indefinitely for the sender to begin.

C. Configure the sender (HP) and begin the send process:

KERMIT-RMB> SET BAUD 9600     (Baud must be identical)

KERMIT-RMB> SEND TESTDATA:,700

HP Kermit begins the sending process by displaying the send file

status screen on the crt. Before actually sending the file the HP

Kermit will do a few things like make sure the file is really

there, check the file type and size of the file so it can tell

the PC Kermit what it will be sending. Then it sends the "start"

signal that the PC kermit has been waiting for, and the file

transfer begins.

The sending Hp Kermit sends the file by "chopping" it into pieces

about 100 bytes long, putting each piece into a "packet" and

sending the packet to the receiving Kermit. The receiving Kermit

unwraps the "file data" from the packet and stores each piece of

file data into the destination file. When the entire file has

Kermit-RMB 1.0   Users Manual                                  Page 5

been sent the sending kermit notifies the receiving end of this

andthen each side shuts down the file transfer and returns you to

the local prompt. You should also see some message to the effect

"file transfer complete" indicating a successful transfer.

At the prompt you can now issue another command, send or receive

another file, or quit by entering the "QUIT" command.

MS-KERMIT> QUIT

C:>

KERMIT-RMB> QUIT

Kermit Done.

(2) Local - to - Remote File Transfer

Local to Remote file transfer works identically the same with

one difference.  You probably don't want to have to walk over

to the computer center and start Kermit there every time you

want to transfer a file.  It would be alot easier if you

could "logon" to the remote computer and give it commands as

if you were sitting in front of the computer yourself.  In

order to logon to a remote computer it must be a multi-user

(multi-tasking no doubt) system.  Unix systems and VAXs are

probably the most familiar multi-user engineering systems

that you know of.  In general, Personal Computers do not

handle remote logon and use.

To logon to a remote computer you use the terminal emulator

built into Kermit to start Kermit and a file transfer

running.  The Kermit command "CONNECT" starts the terminal

emulator.  Once the file transfer is running on the remote

computer you exit the terminal emulator by typing the "escape

sequence", get back to the local Kermit prompt (KERMIT-RMB>)

and begin the file transfer on the HP end.

The following example shows this process; the other computer

is a Unix system, but the method is the same with most

others.  When you type a command, you terminate it with a

carriage return, which you can't see in the example.  The

mysterious "^]c" is Kermit-RMB's escape sequence, which you

enter by holding down the Control (Ctrl) key and pressing "]"

(right square bracket), and then typing the letter C.  This

example assumes the Kermit-RMB program is stored on disk as

the PROGram file "HPKERMIT".

Program Dialog:             Explanation:

LOAD "HPKERMIT",1

HP-9000 Kermit-RMB  Version 1.0  1 Mar 1989

? for help

Kermit-RMB 1.0   Users Manual                                  Page 6

Kermit-RMB>set baud 1200    Set the right baud rate.

Kermit-RMB>connect          Connect as a terminal.

You will see the terminal emulator screen on the HP system

with a "modeline" at the bottom telling you the baud and

terminal settings, and most important, the Kermit escape

sequence.

At this point you are connected to the remote system.

Type a carriage return to get its attention.

Login: andyc                Login to the host.

password: xxxxxx            (Passwords normally don't echo.)

%                           The remote system prompt

% kermit                    Start the remote kermit program

C-Kermit>                   The remote Kermit command prompt

C-Kermit> receive           Start the remote receive program.

^]c                         Escape back to the S200/300.

Kermit-RMB>

You have now escaped out of the terminal emulator and the

local HP Kermit prompt is again active. You are still

connected and logged on to the remote computer, and the

remote Kermit program is waiting for the "start" signal from

your end.

Kermit-RMB> send DATAFILE:,700       send a file.

(The file is transferred...)

Kermit-RMB>                  Transfer complete, prompt reappears.

In this example, the user loads "KERMIT",1 and sees the

program's herald and its prompt, "Kermit-RMB>".  Then the

appropriate communication speed ("baud rate") is set, you

connect as a terminal, and log in to your ID on the Unix

system.  To start Kermit on the Unix system the command

kermit is given, and when it starts the C-Kermit prompt

appears.  You tell it to receive a file by typing the RECEIVE

command, and then escape back to the S200/300, and tell

Kermit-RMB to send the file with the SEND command.  After the

file is transferred, the user would normally connect back to

the Unix system, exit from the Kermit program there, and log

out:

Kermit-RMB> connect      Connect again.

C-Kermit>quit

%                        The remote system prompt appears

% logout                 Logout from Unix or type Ctrl-D.

Kermit-RMB 1.0   Users Manual                                  Page 7

^]c                      Escape back to the S200/300.

Kermit-RMB>

Kermit-RMB> quit

To transfer a file in the other direction, simply exchange

the "send" and "receive" commands above.  That's the easiest

and quickest way to use Kermit.  If you have problems getting

the file transfer to work use the Kermit STATUS command to

view the current Serial Port settings and look for any

obvious incorrect settings (port, speed, parity), fix them

with SET commands (described in Section 8), and try again.

Many problems can crop up when you attempt to connect two

unlike systems over RS-232.  If all communication parameters

appear to be identical, then check the cabling between the

two systems - this is often the problem.  Next is a quick

guide to RS-232 Cabling for HP Workstations:

Kermit-RMB 1.0   Users Manual                                  Page 8

5.2  CONNECTING HP WORKSTATIONS For KERMIT

Below are instructions for connecting the HP Series 200 and

300 directly to a Personal Computer, modem, or your existing

data switch.

HP SIDE:

There are a few different serial interface connections for

the HP Workstation:

1. Built-in 98644 (9 Pin) interface on S/320,330,350,360 or

Built-in 98644 (25 Pin) on S/310.

2. 98626 Plug-in Serial I/O Card (50 Pin) for S/200/300. or

98628 Plug-in Datacomm I/O Card (50 Pin) for S/200/300.

3. Modem Connections

--------------------------------------------------------------

Note:

The following cables and pin diagrams are provided because

they are existing cable products that you may have available to

you already. The cables contain wiring which uses modem

handshake lines to manage the connection. Two computers either

local or remote can also be "direct" connected using a 3-wire

connection using pins 2,3 and 7. In this case there is no

hardware handshake, and you are responsible for manually

connecting and disconnecting.

5.2.1   To connect the S/320/330/350/360  9-Pin Port to the

Personal Computer 9-Pin COM Port, use:

HP 92221P   9-Pin Male (HP)  to  9-Pin Female (PC)

|----------------|                               |-------|

| HP 320/330/350 | --------  HP 92221P  -------- |  PC   |

|----------------| (9M)                     (9F) |-------|

Alternately,

The S/320 and on come shipped with a 9 Pin - to - 25 Pin

Serial adapter cable to provide an equivalent of the S/310 25

Pin Serial Port configuration on the other S/300 computers.

To connect the S/300 25-Pin Port to a 9 Pin Personal Computer

interface, use the Vectra PC Serial Printer Cable, or an

equivalent:

Kermit-RMB 1.0   Users Manual                                  Page 9

HP 24542G 25-Pin Male (HP)  to  9-Pin  Female (PC)

|-----------|                                   |--------|

| HP 310    | -----------  HP 24542G  --------- |   PC   |

|-----------| (25-M)                      (9-F) |--------|

|-----------|    9-->25 Adapter                 |--------|

| HP 320,30 | <-----------------< <-HP 24542G-< |   PC   |

|-----------|    HP 98561-61604                 |--------|

or obtain an equivalent according to the pin assignment

below:

(s/300)  25 Pin Male         9 Pin Female  (PC)

(1) Shell ---|

(2) TX    ------->    (2) RD

(3) RD    <-------    (3) TX

(4) RTS   ------->    (1) CD

/- (5) CTS   <-------    (4) DTR

\- (6) DSR   <--|

(7) GND   --------    (5) GND

(8) CD    <-------    (7) RTS

(9) nc                (9) RI

(20)DTR   ------->  /-(6) DSR

|-->  \-(8) CTS

(22)RI    ---|

The adapter cable for the 9 Pin Built-in HP interface is

HP 98561-61604  (9 Pin male to 25 Pin female)

9 Pin Male (CPU)      25 Pin Female (equivalent to 310)

DTR  (1) -------->   (20) DTR

TX   (2) -------->   (2)  TX         The adaptor cable

RD   (3) <--------   (3)  RD         just maps the 9

RTS  (4) -------->   (4)  RTS        Pin to the 25 Pin

CTS  (5) <--------   (5)  CTS        Interface. No

DSR  (6) <--------   (6)  DSR        jumpering is done.

GND  (7) <------->   (7)  GND

CD   (8) <--------   (8)  CD

RI   (9) <--------   (22) RI

PGND (shell) -----   (shell) PGND

|----   (1) PGND

Kermit-RMB 1.0   Users Manual                                 Page 10

CONNECTING to 25 PIN PC Interface:

The HP 13242G Printer Cable (25 Pin - 25 Pin) is used to

connect from the S/300 25 pin interface to a 25 Pin PC.

HP 13242G  25-Pin Male (HP)  to  25-Pin Female (PC)

|----------------|                               |-------|

| HP 310 (or adp)| --------  HP 13242G  -------- |  PC   |

|----------------| (25-M)                  (25-F)|-------|

The HP 92221P cable is equivalent to the combination of the

adapter (HP 98561-61604) and the 13242G Cable.

5.2.2.  Connecting to the 98626 or 98628 Interface (50 Pin)

Both the 98626 and 98628 Serial Interfaces use the 50 to 25 DTE

Pin Cable (P/N 5061-4215) to provide an equivalent S/300

25 - Pin Serial Port.

Once the 50 Pin DTE Cable is used, then the HP 24542H cable

is used to finish the connection. Below is the

pin assignment for the 50 Pin and 24542H cables.

|----------------|    HP 5061-4215       HP 24542H   |-------|

| S/200  (50-Pin)| <----------------->  >----------< |  PC   |

|----------------|   50           25      25     9   |-------|

98626/98628                               24542H

(50 PIN M)    (25 PIN M)       (25 PIN F)           (PC 9 F)

TX      (12) ----> (2)       (2)  TX    ----->    (2) RD

RD      (42) <---- (3)       (3)  RD    <-----    (3) TX

RTS     (13) ----> (4)       (4)  RTS   ----->    (1) CD    P

S   * CTS     (44) <---- (5)    /- (5)  CTS   <-----    (4) DTR   C

/   * DSR     (45) <---- (6)    \- (6)  DSR   <-|

3     DTR     (14) ----> (20)      (20) DTR   ----->  /-(6) DSR   C

0                                                |->  \-(8) CTS   O

0   * CD      (46) <---- (8)       (8)  CD    <-----    (7) RTS   M

|--    (9) RI

RI      (9)  <---- (22)      (22) RI    --|

Kermit-RMB 1.0   Users Manual                                 Page 11

GND     (48) ----- (7)       (7)  GND   ------    (5) GND

Shell   (24) ----- (1)       (1)  Shell --|

* HW Handshake Lines Used when  'SET HANDSHAKE ON'  is used

5.2.3. Modem Connections

DCE

|----------------|    HP 5061-4215       |-------|

| S/200   50-Pin | <----------------->   | MODEM |

|----------------|   50           25     |-------|

|----------------|    HP   ?             |-------|

| S/300   25-Pin | <----------------->   | MODEM |

|----------------|   25           25     |-------|

|----------------|    HP   ?             |-------|

| S/300    9-Pin | <----------------->   | MODEM |

|----------------|   9            25     |-------|

Kermit-RMB 1.0   Users Manual                                 Page 12

5.3   TESTING THE RS-232 CONNECTION:

The best way to test the RS-232 Connection is to run the

terminal emulator on the computer at each end of the

connection. Although this verifies communication is occuring,

it does not guarantee that all parameters are set correctly.

It is safe to assume that baud rate, transmit and receive

lines, and parity are compatible. This is usually enough to

allow Kermit to send and receive files. Other issues may

involve hardware handshake lines (DTR,DSR and RTS,CTS) and

the number of data bits per frame (7 or 8).

If nothing occurs after both computers "connect" then Kermit

provides some very helpful commands for determining the

problem. Below also is a list of Kermit commands for

modifying the RS-232 parameters of the Serial Port.

KERMIT-RMB> CONNECT

This will start the terminal emulator. If successful, you

will see a flashing cursor in the upper-left corner of the

screen, and along the bottom of the screen will appear a

"modeline" showing the Kermit escape sequence, baud rate,

data bits setting, parity, and ON or OFF for parity check.

If this does not occur, then serial communications never

started - this either indicates that a hardware handshake has

been enabled and the required modem signals are not present,

or there is a problem with the serial interface - selftest,

or operation.

To correct the problem, return back to Kermit by pressing the

escape sequence:  CTRL-]

You will see an 'escape options' line appear along the bottom

showing the terminal escape options. You complete the escape

option by typing one of the leters shown.

Type C to return to kermit.

At the Kermit prompt use the SET command to shut off

handshaking:

< remove the rs-232 cable attached to the serial interface>

KERMIT-RMB> SET HANDSHAKE OFF

KERMIT-RMB> SET LOCAL-ECHO ON     Turn on terminal echo

KERMIT-RMB> CONNECT               Try the connection again

If there is still no cursor, or echo of the keys you type,

there is possible indication of a program error - contact

Kermit-RMB 1.0   Users Manual                                 Page 13

Kermit Distribution for assistance.

Once the local terminal emulator is running, try connecting

the two computers together - attach the rs-232 cable to both

ports, and run the status command to verify that both Kermits

are set up correctly:

1. KERMIT-RMB> STATUS

Run the status command on each computer. It will list such

parameters as Baud Rate, Data Bits, Handshake used (RTS/CTS

etc) Software flow control (XON/XOFF etc). Make sure these

parameters are identical for each side.

2. KERMIT-RMB> SHOW  TERM

KERMIT-RMB> SHOW  COMM

KERMIT-RMB> SHOW  MODEM

These commands are similar to the status command and show

parameters related to the command - Comm, Terminal, Modem.

KERMIT COMMANDS FOR SERIAL PORT CONFIG and DEBUG

KERMIT-RMB> HELP or ?

KERMIT-RMB> STATUS

KERMIT-RMB> SHOW ?   [comm, term, modem ]

KERMIT-RMB> SET  ?   [baud, parity, handshake, flow, echo ]

KERMIT-RMB> CLEAR       (resets the serial port)

KERMIT-RMB> SET PORT ?  (displays the installed cards)

Kermit-RMB 1.0   Users Manual                                 Page 14

6.0   The HP-BASIC File System

The features of the HP-BASIC file system of greatest interest

to Kermit users are the form of the file specifications, and

the formats of the files themselves.

6.1   File Specifications

HP-BASIC file specifications  are of the form

"/PATHNAME/FILENAME:,7xx,Unit"     (assuming HP-IB 7 used)

If no path or Mass Storage Specifier (:,7xx,x) is provided

the current source directory or lif volume is used.  The

default source and destination directories are set to the

current MSI when Kermit is run.  To see the settings use the

STATUS command, and to change them use the SET SOURCE and SET

DESTINATION commands:

KERMIT-RMB> STATUS

KERMIT-RMB> SET SOURCE  :,700,0

KERMIT-RMB> SET DESTINATION  /KERMIT:,702,0

When Receiving files the Default DESTINATION MSI is used if

a msi is not specified as part of the filename specification.

For sending files the Default SOURCE MSI is used in a similar

way.

The device, path, name, and type fields may contain uppercase

letters, digits, and the special character underscore.

Illegal characters include all other special characters and

punctuation including period, colon, and semi-colon.

Files received with illegal filenames are converted to legal

LIF filenames by converting illegal characters to underscore,

and truncating the filename at the maximum allowable length.

This is 10 Characters for LIF Files and 14 for HFS.

In Kermit 1.0 all filenames are converted to uppercase when

kermit receives a send or receive command.

6.2  File Formats

HP-BASIC systems store files in 3 "user" file types and with

two data formats - ON and OFF.  This allows much flexibility

in the use of the file system, but also introduces some

complexity as far as handling data files.

Kermit-RMB 1.0   Users Manual                                 Page 15

The file types addresed by Kermit-RMB are HP-UX, BDAT, and

ASCII.  The remaining three - SYSTM, BIN, and PROG are not

user accessable from Kermit and need to be converted to the

HP-UX file type in order to be transferred.  This can be done

using the unique Kermit-RMB command Convert:

Kermit-RMB> CONVERT prog_file TO HP-UX

The CONVERT command is discussed in section 8.2 - Unique

commands for Kermit-RMB.

A brief description of the user file types may help to sort

out the complexity.

FILE TYPE          Written With             Written With

FORMAT ON                FORMAT OFF

_____________________________________________________________

ASCII        Same as Format OFF.    LIF ascii undelimited records.

4-digit length fields start

each record.

Ascii will always be stored as length-header undelimited

string data regardless of number or string representations.

The FORMAT ON parameter is ignored when ASCII files are

accessed.

HP-UX       Identical to DOS Text        Lines and Numbers

with CR-LF delimiters        use the null char

for each line.               as a delimiter.

Hp-ux files are handled differently according to the file

type set in Kermit.  The SET FILE TYPE [binary | ascii]

command is used to set this indication.

BDAT        CR-LF delimited lines -      Memory Image of data

same as hp-ux but includes   with no delimiters.

a 1-sector file header.      The internal data

is undelimited and only

decoded if the internal

file format is known.

Version 1.0 of Kermit cannot handle mixed intger,real,string

BDAT files written with Format OFF.  To transfer a BDAT-Format

OFF file you will need to manually create a file (Format OFF)

of all integers, or all reals, or convert the file to a BDAT

Format ON file.

Kermit-RMB 1.0   Users Manual                                 Page 16

The File Mode setting in Kermit determines whether the BDAT

data (format off) is to be converted to ascii numeric

representation before sending.

Kermit-RMB 1.0   Users Manual                                 Page 17

6.3 Kermit-RMB handling of files

Kermit-RMB will open the specified file being sent and

determine its file type when the file is opened.  Depending on

the file Mode (ASCII or BIN), the file data will be sent

accordingly:

Note: Throughout the Kermit File Handling sections reference

is made to File Type and File Mode. These two terms should be

understood clearly before reading further.

File Type  - Refers to the HP BASIC File Types of ASCII, BDAT

and HP-UX. These are the three user file types

and are accessable from BASIC programs. The hp-ux

file type was just intoduced in BASIC 5.0 for

compatibility with the hp-ux and DOS operating

systems. Before the hp-ux type was introduced the

ASCII type was intended for ascii file

compatibility between differing operating

systems. BDAT (Binary Data) files allow storage

of numeric data in a form quickly accessable by

BASIC.

File Mode -  Refers to the method used by Kermit when sending

files. The two Modes are ASCII and BINARY. In

ASCII Mode, all transmission is of an ascii text

representation. In BINARY Mode the transmission

is identical to file contents - which may have

been either text or unreadable binary. You will

see BINARY referred to also as BIN.

Changing File Mode

To make maters confusing, Kermit normally refers to handling

modes as "file type". The normal kermit command to change mode

is SET FILE TYPE. In Kermit-RMB the command has been changed

(aliased) to SET FILE MODE <mode>. In this and other Kermit

documentation you may see the mode and SET MODE functions

referred to as file type and SET FILE TYPE.

Kermit-RMB starts up in ascii mode and can be set this way

using the set command:

KERMIT-RMB> SET FILE MODE ASCII

To set file Mode BINARY the same Kermit command is used:

KERMIT-RMB> SET FILE MODE BIN

END-OF-FILE

Kermit-RMB 1.0   Users Manual                                 Page 18

Some DOS files use the CP/M convention of indicating the end of

a file my marking the file with a Control-Z CHR$(26) character.

If the EOF-MODE is set to CTRL-Z then CHR$(26) will be appended

to the file as it is sent.  You may want to set this flag when

sending files to a DOS PC by using the SET EOF ON command.

The EOF-MODE is ignored when receiving files - the CTRL-Z

character if sent will remain in the file and show up as a SB

special character in HP-BASIC.  It can be editied out if not

needed.

To set ctrl-Z on or off use the following KERMIT command:

Kermit-RMB> SET FILE EOF CTRL-Z

Kermit-RMB> SET FILE EOF OFF

Kermit-RMB 1.0   Users Manual                                 Page 19

6.3.1.  Sending Files

+  ASCII FILE TYPE

ASCII MODE

Contents are treated as text and read from the file line by

line.  A CR-LF sequence is appended to the end of each line

before the file is sent.  The receiving Kermit will see

ascii text delimited with standard CR-LF (^M^J) delimiters.

BINARY MODE

ASCII Files are sent the same way regardless of File Mode.

+  HP-UX FILE  TYPE

ASCII MODE

In ASCII Mode HP-UX files are read with Format-ON.

Therefore HP-UX files are expected to have CR-LF line

delimiters written with Format-ON.  If the HP-UX file was

written with Format-OFF, File Mode BIN should be set prior to

sending the file.

BINARY MODE

While sending HP-UX files with BINARY mode set the file is

read as an image and no delimiters are looked for.  The file

is transferred as a direct image of the file with no added

CR-LF item delimiters, or terminators.

+  BDAT FILE  TYPE

For BDAT file types the Kermit File Mode (ASCII or BIN) refers

to the way the file data is transmitted, not the way the file

is read.  This gives you the option to send BDAT binary image

data as either ASCII or BINARY.  The Kermit SEND command has a

special optional form for BDAT files to allow this.

SEND <BdatFile> [<File Contents Specifier>]

ASCII MODE

Kermit-RMB 1.0   Users Manual                                 Page 20

BDAT files are read according to instructions in the Kermit

SEND Command line, and the data is converted to ascii strings

and sent with CR-LF delimiters.

Kermit assumes that the file was written with Format ON unless

a special identifier is included with the SEND Command:

SEND Filename           Will read the file with FORMAT-ON

SEND Filename INT       Will access the file with Format OFF

and read the file as 16 bit integers.

SEND Filename REAL      Will access the file with Format OFF

and read the contents as 64 Bit Reals.

It is up to the Kermit-RMB user to know if the ascii file was

created and written using a FORMAT-ON or OFF attribute.

Sending BDAT files written in binary mode (Format off) without

instructing Kermit will generate the error message, "File

Contents Not Ascii" and the file transfer will abort.

In all cases the data read from the file is sent in ascii text

form.  BDAT Format-OFF data is converted (on the fly) to ascii

numeral and text data.

The 1-Sector file header on BDAT files will not be sent with

the file.

BINARY MODE

When sending a BDAT file with BINARY mode set, the contents of

the file are sent without conversion to ascii numeric

representation.  If the file was written with Format OFF -

INTEGER (16 bit) or REAL (64 bit) data, then you must indicate

this in the Kermit SEND Command Line:

SEND Filename INT

SEND Filename REAL

Kermit-RMB 1.0   Users Manual                                 Page 21

6.3.2   Receiving Files

SYNTAX:

KERMIT-RMB> RECEIVE [Filetype] [Filename] [File Len] [Recl]

A. KERMIT-RMB> RECEIVE

With no optional parameters specified, Kermit makes the

following defaults:

File type:       hp-ux

File Name:       given by sending Kermit

File Length:     given by sending Kermit, or Default 100Kb

Recl:            256

Kermit-RMB will create an HP-UX file type by default.  The

filename from the sending Kermit wil be used.  If the name is

an illegal HP-BASIC name then it will be converted to a legal

name.  name.

In this example Kermit will use the filename provided by the

remote Kermit, and use the default DESTINATION MSI when

creating and storing the file.

B.  Filetype and Filename

Kermit-RMB> RECEIVE                   Creates an hp-ux file

Kermit-RMB> RECEIVE  ASCII            Creates an ASCII File

Kermit-RMB> RECEIVE  ASCII  NEWNAME

Kermit-RMB> RECEIVE  BDAT             Creates a BDAT File

Kermit-RMB> RECEIVE  BDAT   NEWNAME

Kermit-RMB will create a file with the name specified and if a

optional file type is provided that file type will be created

instead of a hp-ux type.

FILE LENGTH:

If an HFS file is specified as the destination MSI then no

filelength needs to be specified. HFS files are extensible, and

therefore Kermit needs no knowledge about the length of the

file. If the remote Kermit sends Attribute packets which inform

the receiving kermit of the current files length, then that

figure will be used in creating the destination file.

A problem arises when receiving to a LIF type disc, and neither

the local kermit user specifies filelength, nor the remote

Kermit-RMB 1.0   Users Manual                                 Page 22

Kermit sends an attribute packet for the file length. In this

case the local kermit will create a destination file of default

length 100 Kb (if disc or ram volume size permits) and try to

accomodate the incomming file. If the incomming file is larger

than 100 Kb then a spill file will be created of 100 Kb and the

file will continue to be received. This process will continue

until the file is received, or disc space is consumed. If a

spill file cannot be created, then an Error message will be

sent to the sending Kermit which will abort the file transfer.

INCOMPLETE and DUPLICATE FILES:

Kermit provides two commands for handling incomplete and

duplicate files.

KERMIT-RMB> SET INC [KEEP | DISCARD]

Depending on the value of this parameter an aborted receive

(incomplete) file will either be erased (DISCARD) or saved in

its incomplete form on the disc (KEEP).

DUPLICATE FILE NAMES:

KERMIT-RMB> SET FILE WARN [ON | OFF]

This command instructs Kermit to either OVERWRITE a duplicate

file, or RENAME the incomming file to a unique filename.  With

Filewarn ON the file will be renamed.  If Filewarn is OFF,

then the existing file will be purged before the destination

file is created.  Loss of data could occur if file warn is

off, and the receive file process is aborted.

Kermit-RMB starts up with FILE WARN ON (rename files) and

INComplete ON (Save aborted files).

Kermit will receive all files and store them as an identical

image of the file as it was received.  The file is not changed

in any way, and End-of-line or end-of-file sequences are not

appended or stripped off.  Therefore, there is no difference

in receiving files wheter file type is set to ascii or binary.

It will be up to the HP-BASIC programmer to understand the

internal contents of a file in order to understand how it must

be accessed, and with Format ON or OFF.

C. The last permutation of the receive command is

Kermit-RMB> RECEIVE BDAT  ,  50

Kermit-RMB 1.0   Users Manual                                 Page 23

This instructs Kermit to use the filename being sent by the

sending kermit but override the HP-UX file type and create a

BDAT file type instead.  The 50 instructs Kermit to create a

file with 50 defined records, and since no record length is

specified then default of 256 bytes will be used.  The , is a

place holder for filename.  You can still specify the

destination MSI by specifying it without a filename

Kermit-RMB> RECEIVE BDAT :,702,1 , 8

In this example a BDAT file with record length of 8 bytes is

created to store the received file.  No file length was

specified in this example - Kermit depends on the sending

Kermit to notify the receiver of the proper file size to

create.  If neither occurs then 391 sectors (100 Kb) is

created by default.

Kermit-RMB 1.0   Users Manual                                 Page 24

7.0   Program Setup and Invocation

The HP-BASIC Kermit program can be run from any disk without

any special installation procedure. All the files start with

HPK____ to uniquely identify them on Columbia University's

distribution system. Once received you can rename them.

7.1  The Kermit Distribution Disc

The KERMIT Distribution disc contains the following files:

HPKERMIT        Kermit Program File

HPK_HELP        Kermit HELP File

HPK_INIT        Kermit Initialization File

HPK_DOC         Kermit User Manual

HPKGET_ME       'GET-ME' ASCII program file with document print

instructions

HPK_MISC        Source Code for compiled sections, and Hex

editor (DUMP) utility

7.2   Printing the Kermit-RMB User Manual

To print this document (which you've probably figured out by

now) use the Kermit PRINT Command:

KERMIT-RMB> PRINT  HPK_DOC:,7xx  701

Adjust the printer address and file MSI according to your

system.

To read the file on the CRT (not recommended) use the type

command:

KERMIT-RMB> TYPE  RMBK_DOC:,7xx

7.3  Loading and Running Kermit-RMB

To load and run Kermit use the BASIC LOAD Command:

LOAD "HPKERMIT:,7xx",1

You will see the Kermit prompt appear and the program banner

Kermit-RMB 1.0   Users Manual                                 Page 25

HP 9000  Kermit-RMB

Version 1.0   1 Mar 1989

?  for help

Kermit-RMB>

Kermit-RMB> is the Kermit prompt at which you can type any

Kermit command.

Kermit-RMB> VERSION              Kermit will echo the version

You may type Kermit commands repeatedly until

you are ready to quit the program, using the QUIT command.

7.4 Minimizing the Kermit Program Size

Kermit programs usually contain more than just send file and

receive file code. To minimize the amout of RAM required to run

Kermit, some functions (subprograms) have been removed from the

HPKERMIT program and placed in the HPK_MISC file, and the

HPK_HELP file.  If you are not concerned about program size,

then these functions can be loaded with the LOADSUB command,

and connected permanently by re-storing the program again with

the RE-STORE "HPKERMIT" command.  The following Subprograms

exist in these files and can be loaded:

FILE        SUB NAME           COMMAND   Description

HPK_HELP    SUB Kermit_help    HELP      Kermit Help Command

HPK_MISC    SUB Hex_edit       DUMP      Debug tool to examine

files

The HPK_MISC file also contains the BASIC source code for the

Kermit Encode_packet and Decode_packet CSUBS in HPKERMIT. These

are provided in case you want to examine, edit, or recompile in

the future.

Minimizing Kermit

There are also subprograms in Kermit which you may not intend

to make use of that can be deleted from the main program.

Following is a list of subprograms and functionality you may

want to delete from HP Kermit. I suggest you backup the

HPKERMIT file before modifying it. The number of program lines

in each subprogram is noted in ().

Kermit-RMB 1.0   Users Manual                                 Page 26

Subprogram Name       Functionality

Terminal  (680)       VT-100 terminal for logging on to remote

computers and starting Kermit there. If

you are using kermit between local

computers only you dont need this.

Transmit  (140)       Raw ASCII file transmission - not

required if you are only using SEND and

RECEIVE commands

Convert   (140)       Used with the CONVERT command to convert

file types from ASCII, BDAT, HP-UX, and

PROG.  This command is intended to be

used to convert PROG files to HP-UX, so

they can be sent by Kermit. See section

8.2 for a description of the CONVERT

command.

Phyread               Used by the Convert and Dump (Hexedit)

Phywrite              commands to access the disc volume

Get_vol_info          directory, and disc file sectors.

Get_file_info

7.5   Kermit Initialization (HPK_INIT File)

Upon initial startup, the program looks for a file on the

system's current MSI called HPK_INIT and if found executes the

kermit commands in the file.  The HPK_INIT file is a special

case of a "TAKE" file which is an ascii file containing a

Kermit Commands. See section 8.1 "Program Management Commands"

for a description of the TAKE command and writing TAKE files.

This initialization file may contain command macro

definitions, communications settings for one or more ports, or

any other Kermit-RMB commands, and you may create it using any

text editor capable of saving LIF ASCII files.

7.6    RAM Disk Operation:

If sufficient memory exists Kermit will create a RAM Disc and

use it for transfering and modifying files.

The file will first be copied to the RAM disc, and then sent

from there.

Kermit-RMB 1.0   Users Manual                                 Page 27

Kermit-RMB 1.0   Users Manual                                 Page 28

8.0    Kermit-RMB Command Descriptions

During interactive operation, all the edit function keys on the

keyboard are available to edit the command you are entering. The

recall key will recall previous commands executed. The command

line when recalled must stil contain the kermit prompt Kermit-RMB>

in order to be parsed correctly.

The notation used in command descriptions is as follows:

[square brackets]      An optional field.  This field may be

omitted.

{curly braces}         A list of alternatives, separated by commas.

Choose one of the items from the list.

The following sections describe all the HP-BASIC Kermit commands.

Since some command descriptions may contain references to other

commands that haven't been explained yet, you might find that this

manual makes more sense on a second reading. Notice that some

commands can be abreiviated using the first letter of the

command, for example:

> SET BAUD 1200          > S B 1200

> SET FILE TYPE BIN      > S F T BIN

> QUIT                   > Q

HP-BASIC Kermit supplies a good amount of the full set of

Kermit commands.  Commands with asterics are unique to

Kermit-RMB and not found in other Kermits.  Here's a summary:

8.1    Kermit-RMB command syntax

CLEAR                    Reset Serial Port Transfers

*  CLS                      Clear Screen

CLOSE,CLO                Close Log File

PACKET,PAC,P

SESSION,S,S

CONNECT,C,CON            Start Terminal Emulator

*  CONVERT

COPY

CD, MSI

DEFINE,DEF

DELETE,DEL

DO

CAT,DIR

*  DUMP

Kermit-RMB 1.0   Users Manual                                 Page 29

ECHO

EXIT

HELP,?

LOCAL

LOG

SESSION,SES

PACKET,PAC

OUTPUT

PRINT,TYPE

QUIT,Q

RECEIVE

ASCII,BDAT,HPUX

RENAME

SEND

SET,S

BAUD,SPEED,B

BLOCK-CHECK

DEBUG

DELAY

DESTINATION,DEFAULT,DES,DEF

DUPLEX

ON,FULL

OFF,HALF

ECHO,LOCAL-ECHO

OFF,REMOTE

ON,LOCAL

EOF

CTRL-Z,Z,ON

NONE,OFF,NO CTRL-Z

ESCAPE,ESC

FILE,F

NAME

TYPE,T

BINARY,IMAGE,B

WARNING,WARN

SUPERCEDE

FLOW-CONTROL,FLO,FC

XON,XOFF,X

ENQ,ENQ/ACK

HANDSHAKE,HS

INCOMPLETE

OFF,DISCARD

ON,KEEP,K

LINE,PORT

MARK

PARITY

RECEIVE,REMOTE

END-OF-PACKET,EOP,EOL

PACKET-LENGTH,PL

PAD-CHARACTER,PC

PADDING

PAUSE

START-OF-PACKET,SOP,MARK

Kermit-RMB 1.0   Users Manual                                 Page 30

TIMEOUT

RETRY,RET

SEND

END-OF-PACKET,EOP,EOL

PACKET-LENGTH,PL,LEN

PAD-CHARACTER,PC

PADDING,PAD

PAUSE

PREFIX

CONTROL

8BIT

REPEAT,REP

TIMEOUT,TIM

START-OF-PACKET,SOP

TERMINAL,T

VT100

TIMER

MACRO

STATUS

TAKE

TRANSMIT

TYPE

VERSION

Commands for TAKE files

ECHO

COMMENT                  Comment a macro file

8.2    Unique commands for Kermit-RMB:

DUMP  {filespec}

The file specified is copied to the ramdisc and dumped in

HEX/ASCII format one sector at a time. I used this to verify

that binary files were being transferred properly. The

dump utility also allows you to edit the contents of a disc

sector and write the modified sector to disc.

After the first sector is displayed the following keys ae

active:

TAB             switches edit cursor from Hex to Ascii Edit

The following functions require the CTRL key to be held down

simultaneously:

Kermit-RMB 1.0   Users Manual                                 Page 31

+               dumps next sector

-               dumps previous sector

/               dumps the directory sector holding that

file's directory entry.

*               returns you to the first data sector of file

# or N          allows you to specify a sector to be dumped

Q               quit

K               convert hex number to decimal,ascii, and

Kermit representation.

P               Dump Alpha Screen to Printer

H               Display this Help

space bar       write disc sector to disc (you will be asked

to confirm).

CONVERT  filespec  [TO]  filetype

KERMIT-RMB> CONVERT  Progfile:,702  hp-ux

The specified file is copied to the ramdisc and its

directory entry is changed to the specified filetype. The

original file is not touched unless it was already on the

ramdisc.

Caution:

This command does not convert the contents of the file!  Only

the directory entry is changed to the filetype specified.

For example, converting an ASCII filetype to HP-UX will not

change the ASCII text length headers to CR-LF text delimiters.

TRANSFERRING PROG FILES

Note: The documentation for the SEND command does not indicate

the ability to send PROG type files. This capability was not

fully debugged before releasing version 1.0. However, if the

file specified with the SEND command is a PROG file, it will

be copied to the ramdisc (the original is not touched),

converted to an hp-ux file and sent to the receiving Kermit.

The reason this capability is not documented with the SEND

command is that getting the file back as a PROG file is not

100% guaranteed. (Kind of like taking off in a jet without

landing gear). On some occaisions the file when received back

would not load correctly or run.

As soon as this capability is debugged it will be included

with the next release of Kermit. I didn't want to hold up this

release any further. If you need this capability, let Columbia

Kermit-RMB 1.0   Users Manual                                 Page 32

University, or myself know, and we'll be in touch as soon as

PROG file transfer is debugged.

Using a PC Hex Editor and the BASIC Hex editor (which was

written solely for this purpose) I've determined that the

original PROG file contents can be sent and received back

fully in tact. The problem arises in restoring the original

PROG file directory entry. It appears that the PROG

directory entry uses some other pointer than the logical

end of file in marking the file. This pointer is lost

after the file is sent, so restoring it when the file is

received is not always possible. I've used the DUMP command

to edit the PROG directory entry so that the file will load

and run correctly.

I believe that the solution may be to send the Directory entry

along with the file when it is sent, and parse it off when the

file is received again. Keep in touch.

8.3     GENERAL KERMIT COMMAND GROUPS

The Kermit Command descriptions that follow have been broken

into functional groupings under the folowing categories:

8.4     Program Management Commands

8.5     Local File Management Commands

8.6     Terminal Connection and Emulator Commands

8.7     File Transfer Commands

8.8     Ascii File Transmission Commands

8.9     Kermit SET Command Options

8.10    Status and Show Commands

8.11    Log and Close Commands

8.12    Macro Definitions

8.13    Kermit Server Commands (Not Implemented)

8.14    Remote Server Commands (Not Implemented)

Kermit-RMB 1.0   Users Manual                                 Page 33

8.4  Program Management Commands

HELP or ?

Displays a one screen introduction to frequently used Kermit

commands and their editing keys, and suggests using the

question mark command to see the terse list of primary level

Kermit commands.  Also available are HELP 1-6 screens which

are described in the general help.

VERSION

displays the Kermit-RMB program version number, which you

should know in case you are reporting bugs or seeking

technical assistance.

The EXIT Command

Syntax:  EXIT or QUIT

EXIT and QUIT are synonyms for each other.  From Kermit-RMB

they shutdown the program and reset defaults such as key

labels on, cursor on etc.

If you are in the Kermit shell from the terminal escape

CTRL-]K then EXIT returns you to the terminal session.

Don't use QUIT to exit the Kermit Shell.

The serial port RS-232 signals are left alone upon EXIT, so

that modem connections are not broken.  Kermit-RMB may be

restarted with the connection intact.  Use HANGUP to

explicitly break a modem connection; and use SHOW MODEM or

SHOW COMMUNICATIONS to view the status of modem signals

CARRIER DETECT, DATA SET (modem) READY, and CLEAR TO SEND.

The TAKE Command

Syntax:  TAKE filespec

The TAKE command gives you way a to collect Kermit-RMB commands

into a single file, so that you can execute many commands by

typing a single (TAKE) command.  TAKE instructs Kermit-RMB to

execute commands from the file that you specify.

Kermit-RMB>take modem_cmd

This directs Kermit-RMB to find the MODEM_CMD file, open it,

Kermit-RMB 1.0   Users Manual                                 Page 34

execute the commands in it, close it, and return to the

Kermit-RMB> prompt when done.

To use the HP BASIC program editor to modify the file use the

BASIC GET command:

GET "HPK_INIT"       Warning: Remeber this will erase the

Kermit program from memory.

The BASIC program editor may be used to create an init file by

beginning each line with a comment and saving the file as

ascii.

As an example of a KERMINIT command file:

10 !comment -- KERMINIT, HP-BASIC Kermit initialization file

20 !comment -- Don't overwrite my files!

25 !set warning on

30 !!  a double !!  is also a comment

40 !def HP set par odd,set loc on,set hands xon,set flo -

50 !none,set tim on ; semi-colon starts a comment

60 !def modem set port 2, set speed 1200

70 !comment -- I always start out by connecting to UNIX

80 !set port 9 !  use select code 9

90 !set speed 4800 !  a second !  starts a comment

100 !stop

INIT files are created with the following rules:

Line numbers are optional - if used start each line with a !

mark following the line number.

End the file with a STOP command.

!! and comment    are comment lines

! separates a command from its comment

Only comments after the COMMENT command will be echoed to the

Kermit-RMB 1.0   Users Manual                                 Page 35

screen when the file is executed.

The ECHO Command

Syntax:  ECHO [string]

The ECHO command writes the string to the screen, without

adding a carriage return or line feed.  ECHO may be used to

report progress during execution of a TAKE command file, or

to issue prompts during the execution of a script.

ECHO Part one completed...

The COMMENT Command

Syntax:  COMMENT text

The COMMENT command lets you add comments to a TAKE command

file.  The word COMMENT (or any unique prefix thereof) must

appear as the first word on the line. Double !! in the take

file will also act as a comment line.

8.5   Local File Management Commands

These commands are executed on your local S200/300, directly

without having to use the local command before execution.

This allows you to execute the most common file management

commands without leaving KERMIT.  All file specifications

may include device and/or directory fields.  The local file

management commands are:

CD , MSI  {path:MSI}

Changes the current working directory to the given path and

MSI.  All references to local file management commands will

refer to the current MSI. Don't confuse this with the

SET DEFAULT {MSI} which specifies the default path for SEND,

RECEIVE, TRANSMIT, and LOG commands.

DELETE , DEL , ERASE  {filespec}

Deletes the specified file. No wildcards.

CAT or DIR  [path:MSI]

Performs a CATalog of the current MSI or the MSI included in

the command specification.

Kermit-RMB 1.0   Users Manual                                 Page 36

SPACE  [MSI]

Tells how much space is available on the current disc, and what

the largest contiguous space is on a LIF disc.  The number of

sectors fragmented on the disc is also reported.

TYPE or PRINT  filespec  [device]

TYPE displays the specified local file on the screen.  PRINT

will print the file to the device specified - ie 1 or CRT,

701.

8.6   COMMANDS FOR TERMINAL CONNECTION

The CONNECT command connects your S200/300 as a terminal to

the remote system so that you may conduct a session there.

For completeness, the descriptions below contain copious

reference to the SET commands, which let you modify all sorts

of terminal and communication parameters (the SET commands are

described in a later section).  Kermit-RMB is initially set up

with the following parameters, so that you only need to issue

SET commands for those that need to be changed:

Default Kermit Startup

PORT 9   Or the lowest serial port select code found.

if more than one port is found you will be asked to

select one.

TERMINAL VT102

SPEED 2400

PARITY None  (Data Bits 8)

FLOW-CONTROL None

HANDSHAKE None

LOCAL-ECHO Off

ESCAPE Control-]

The CONNECT Command

Syntax:  CONNECT , C

The CONNECT command establishes an interactive terminal

connection to the remote system using the currently selected

communications port with all settings currently in effect for

that port, emulating the currently selected type of terminal

Kermit-RMB 1.0   Users Manual                                 Page 37

which is only VT-100 at this point.

During CONNECT, the characters you type are sent out the

communication port, and the characters that arrive at the

port are displayed on the screen.  If you SET LOCAL-ECHO ON,

Kermit-RMB itself will display the characters you type on the

screen.

Before you issue the CONNECT command, be sure to set the

correct communication speed (SET SPEED) and any other

necessary communication parameters (e.g.  SET PARITY, SET

|  LOCAL-ECHO).  If you have SET DEBUG ON, then received control

|  characters will be displayed in special notation and no

|  particular terminal will be emulated.

In addition characters that are sent to the screen will also

be recorded in a disk file if you have issued a LOG SESSION

command.

The CONNECT command turns your S200/300 into a terminal to

the other computer.  To get back to the S200/300, type the

escape sequence followed by the letter C (for "Close

connection").  Usually the escape character is Ctrl-]

(Control-Rightbracket).  That means, hold down the Ctrl key,

press "]", release both, and then type the letter C.

Kermit-RMB> C      Connect to remote system.

Conduct terminal session here...

^]c Escape back to S200/300.

Kermit-RMB> Prompt reappears.

This is called "escaping back".  You can use the SET ESCAPE

command to change the escape character to something besides

"^]".

Escape sequences:

When you press the escape sequence (CTRL-]) while in the

terminal emulator, a list of available sequence options is

displayed on the screen.  Below is a list of these options:

CTRL-] C    Cancel Terminal Emulation - return to KERMIT-RMB>

"    K    Invoke a Kermit Shell - use EXIT to return from

the Kermit shell to terminal mode.

"    B    Send a BREAK signal out the com port.

"    Q    Quit Session logging (raw download) if on

"    R    Resume Session Logging if previously

discontinued.

"    M    Toggle the modeline ON/OFF

"    E    Toggle Local-echo

Kermit-RMB 1.0   Users Manual                                 Page 38

Escape-Level Commands

The escape character, normally Control-], is used to regain the

attention of Kermit-RMB during CONNECT (you can change the

escape character using SET ESCAPE).  When you type the escape

character, Kermit-RMB waits for you to follow it with a single

character command.  For instance, the single character command

"B" sends a break signal over the serial line.  Typing any

other character (except the space bar, which is the "null

command") after the escape character will cause Kermit-RMB to

beep, but will do no harm.

?   Help -- Lists the available single-character commands.

0   (the digit zero) Transmit a NUL (ASCII 0).

B   Transmit a BREAK signal.

C   Close the connection and return to Kermit-RMB prompt level.

K   Create a Kermit Shell

M   Toggle the mode line, i.e. turn it off if it is on or vice versa.

E   Toggle Local-Echo ON/OFF

Q   Temporarily quit logging the remote session.

R   Resume logging the remote session.

When using Kermit to connect two S200/300s "back to back,"

SET LOCAL-ECHO ON so that when you CONNECT to the other

S200/300 you can see what you are typing.  You should also

SET TERMINAL NEWLINE ON, so that that a linefeed will be

automatically supplied for each carriage return you type.

TERMINAL EMULATION

Kermit-RMB emulates a minimal VT-100 terminal, and is the

only terminal emulation provided. Any escape character

received will begin decoding of an escape sequence. If a

valid VT-100 sequence is detected it is executed - if the

sequence is unknown then the entire sequence is displayed on

the CRT. Presence of these sequences during a VT-100 terminal

session will indicate those VT-100 sequences that are not

implemented.

|   The Mode Line

When you first issue the CONNECT command, a message will

display the status of the terminal line parameters such as

baud rate, parity, and escape sequence.

Esc: ^]C  baud: 9600  parity: odd  off

8.7    COMMANDS FOR FILE TRANSFER

Kermit-RMB 1.0   Users Manual                                 Page 39

Kermit-RMB's SEND and RECEIVE invoke the Kermit file

transfer protocol for error-checked transmission of files

between Kermit-RMB and another Kermit program on the other

end of the connection.  There are also commands for "raw"

transfer of files (no error checking) with systems that

don't have Kermit programs:  LOG SESSION (for capturing text

files on your S200/300) and TRANSMIT (for uploading text

files to the remote system).

During file transfer, Kermit-RMB normally displays its

progress on the screen as shown below.  The items in

the right-hand column are updated more or less at random.

File name: TESTDAT

Bytes transferred: 1460

% transferred: 10

Sending: In progress

Number of packets: 74

Number of retries: 2

Last error:

Last warning:

Kermit-RMB File Transfer Display Screen

File transfers involving floppy disks will be slow and noisy.

Hard disks are much faster (and quieter), and RAM disks

faster still (and totally silent).  But if you store new

files on a RAM disk, be sure to move them to a real disk

before turning off your S200/300.

Before attempting to transfer files to the S200/300, make sure

you have enough room on the selected device.  Kermit does not

provide a way for you to change disks during a file transfer.

However, the Kermit protocol will help you out a by creating

"spill files" which will hold the file data if the original

file created is too small to hold the incomming file.  Kermit

will also attempt to prevent transfer of files that are too big

to fit in the available space.  Kermit-RMB supports "file

attributes" exchange, and if the other Kermit supports this

option too, then the receiving program will check free disk

space before letting the transfer proceed.

Other attributes exchanged by Kermit-RMB include the file's

creation date and time, the system of origin, and the file

type.  Some of these attributes are implemented.

Kermit-RMB 1.0   Users Manual                                 Page 40

The SEND Command

Syntax:  SEND filespec1  [filetype]

The SEND command causes a file or file group to be sent from

the local HP-BASIC system to the Kermit on the remote system.

Currently Kermit-RMB does not support server commands.

filespec1 may contain the wildcard characters "*" to match

zero or more characters within a field If filespec1 contains

wildcard characters then all matching files will be sent, in

the same order that HP-BASIC would show them in a directory

listing.

If the optional filetype is specified, then only files of

that filetype will be included in the list of files to be

transferred.

If the SEND command is specified by itself on the command line,

then you will be presented with a valid syntax description help

on the screen.

If a file can't be opened for read access, the message

Cannot access file - blank filename will exit

/default/file:,msi

Correct the name of the file you want to send at this time, and

press enter.

Once you give Kermit-RMB the SEND command, the name of each file

will be displayed on your screen as the transfer begins.  Packet,

retry, and other counts will be displayed along with informational

|  messages during the transfer, in the style specified by SET

|  DISPLAY.  If the file is successfully transferred, you will

|  see "Complete", otherwise there will be an error message.

Several single-character commands may be given while a file

transfer is in progress:

|  ^X (Control-X) Stop sending the current file and go on to

the next    one, if any.

|  ^Z Stop sending this file, and don't send any further files.

|  ^C Return to Kermit-RMB command level immediately without

sending any kind of notification to the remote system.

(^Z  or even ^E is preferable.)

Kermit-RMB 1.0   Users Manual                                 Page 41

|  ^E Like ^C, but send an Error packet to the remote Kermit in

an attempt to bring it back to server or interactive

command level.

|  CR Simulate a timeout:  resend the current packet, or NAK

the expected one.

Control-X, Control-Z, and Control-E send the proper protocol

messages to the remote Kermit to bring it gracefully to the

desired state.  Control-C leaves the remote Kermit in

whatever state it happens to be in, possibly retransmitting

its last packet over and over, up to its retry limit.  You

should only have to use Control-C in dire emergencies (the

remote Kermit is stuck, the remote system crashed, etc), or

at those times when you realize that you have given a file

transfer command to Kermit-RMB without first having told the

remote Kermit about it.

The RECEIVE Command

Syntax:  RECEIVE [filespec] [filetype] [BDAT Rec Length]

The RECEIVE command tells Kermit-RMB to receive a file or

file group from the other system.  The file is stored under

the name it was transmitted with, except that any illegal

characters are translated to _.  Kermit-RMB passively waits

for the file to arrive; this command is not to be used when

talking to a Kermit server (use GET for that).  You should

already have issued a SEND command to the remote Kermit and

escaped back to Kermit-RMB before issuing the RECEIVE

command.  The RECEIVE command is intended for situations

where the file name and sending operation originates at the

other side.

If the optional filespec is provided, incoming files will be

stored under that name.  The first incoming file is renamed

and any additional files either overwrite the first (if FILE

WARNING is OFF) or are renamed slightly from the filespec

(digits are added to the end of the filename) if

SET FILE WARNING ON

has been spacified.  The filespec may include any

combination of the following fields:

Device MSI    Store the file on the designated device,

in the current directory for that device.  If no device

designator is given, store it on the current default device.

|  Directory path Store the file in the designated directory on

|  the current disk.  If no path given, store the file in the

|  current directory.

File name Store the file under the name given.  If no name

Kermit-RMB 1.0   Users Manual                                 Page 42

is given, store it under the name it was sent under,

converted, if necessary, to suit DOS conventions, and

modified, if SET WARNING ON, to avoid overwriting any file

of the same name in the same directory.

If an incoming file does not arrive in its entirety, Kermit-RMB

will normally discard it and it will not appear in your directory.

You may change this behavior by using the command SET INCOMPLETE

KEEP, which will cause as much of the file as arrived to be saved

on the disk.

The same single-character commands are available as during SEND:

|  ^X Request that the remote Kermit stop sending the current

file, and proceed to the next one immediately.  Since

this is an optional feature of the Kermit protocol, the

remote Kermit might not honor the request.

|  ^Z Request that the remote Kermit terminate the entire

transfer; this is also an optional feature that may or

may not be supported by the remote Kermit.

^C, ^E, and CR operate in the same way as they do during

SEND.  In this case, ^E should always do what ^Z is supposed

to do.

If WARNING is OFF and you type ^X or ^Z to interrupt the

transfer, you'll either get a partial new file, or else both

the old and the new file of that name will be lost,

depending on SET INCOMPLETE.  In any case, when WARNING is

off, old files with the same name as incoming files will not

survive.

8.8   Commands for Raw Uploading and Downloading

Kermit-RMB can be used to send files to, or capture files

from, remote systems that do not have Kermit programs

available.  No error checking or correction is done, so the

results can very likely contain corrupted characters, spurts

of noise, gaps, or extraneous system messages or prompts.

The command for uploading is TRANSMIT, and for downloading

LOG SESSION.

|  To minimize loss of data during these operations, be sure to

|  SET the FLOW-CONTROL and HANDSHAKE parameters to match the

|  characteristics of the system on the other end.

The TRANSMIT Command

Syntax:  TRANSMIT filespec

Kermit-RMB 1.0   Users Manual                                 Page 43

The TRANSMIT command provides a basic raw upload

facility to send ASCII, BDAT, or HP-UX files to the host

without packets, error checking, or retransmissions, but

using all the currently selected communication parameters for

flow control, parity, etc.  Information is read from the disk

file a line at a time, sent out the serial port.  A disk file

line ends with carriage-return-linefeed (CRLF).

Typically, before using this command to upload a file, you would

start a text editor (preferably a line-oriented, rather than

full-screen, editor) on the remote host and put it into text

insertion mode.  When the file has been completely transmitted,

you would manually enter the required sequence for getting the

editor out of text insertion mode, and then make any necessary

corrections by hand.

Here's an example for VAX/VMS:

Kermit-RMB>set flow xon/xoff

Kermit-RMB>connect                   Connect to VAX.

$ edt foo_txt                        Start the EDT editor.

*i Put it into "insert" mode.

^]c Escape back to Kermit-RMB.

Kermit-RMB>transmit foo_txt

Upload the file a line at a time.  ...  The lines are displayed on

your screen.

Kermit-RMB>connect

When done, connect back to the VAX.

^Z Type Ctrl-Z to exit EDT insert mode.

*exit Exit from EDT to save the file.

$

If transmission appears to be stuck, you can wake it up by

typing a carriage return on the keyboard.  You can cancel

|  the TRANSMIT command by typing a Control-C.  Control-Z's or

other control characters in the file may have adverse

effects on the host.  For this reason, you should use

TRANSMIT only for files that contain 7-bit printing ASCII

characters, spaces, tabs, carriage returns, linefeeds, and

possibly formfeeds.

Kermit-RMB 1.0   Users Manual                                 Page 44

The LOG SESSION Command

Syntax:  LOG SESSION [filespec]

The LOG SESSION command lets you copy the characters that

appear on your screen during CONNECT into the specified file

on the S200/300.  You can use this command to download files

by displaying (usually with a command like TYPE) the file on

the remote system while logging is in effect.

Example:

Kermit-RMB>LOG SESSION  :,0,0

Starts a file on the Ram Disc called SES_LOG by default. You

will need to have the terminal emulator active in order to

trap activity at the serial port.

Kermit-RMB>connect

Any activity appearing at the serial port will now be sent to

the CRT screen and to the log file SES_LOG.

If you are connected to a PC for example you can now copy the

file to the PC Serial port and capture it on the HP side:

C:\> COPY  TEXTFILE COM1:

If you were using MS-KERMIT on the PC you could use the

TRANSMIT command to copy the file to the serial port:

MS-KERMIT> TRANSMIT  TEXTFILE

The TRANSMIT command on the PC Kermit sends one line of the

file each time <enter> is pressed.

To stop session logging temporarily from the terminal emulator

you use the "Q" escape option:

CTRL-] Q       Suspends Session Logging,  but leaves the file

open.

To resume logging use the "R" option:  CTRL-] R

To close the file and end session logging use the CLOSE

command at the Kermit prompt:

CTRL-] C                       Leave the terminal emulator

KERMIT-RMB> CLOSE SESSION      and close the log file.

Kermit-RMB 1.0   Users Manual                                 Page 45

During terminal emulation, the LOG command records all the

characters that arrive from the remote host in the specified

file, If you have SET LOCAL-ECHO ON, the characters you type

will also be recorded.  Logging may be suspended and resumed

within a terminal session with the CONNECT escape-level

commands Q and R.

8.9   The SET Command

Syntax:  SET parameter [parameter] value

The SET command establishes or modifies parameters for file

transfer or terminal connection.  You can examine their

values with the SHOW or STATUS commands.  The following SET

commands are available in Kermit-RMB:

ATTRIBUTES  Controls whether Kermit-RMB uses Attribute

packets

BAUD        Communications port line speed (synonym for

SPEED)

DEBUG        Display packet contents during file transfer

DEFAULT-DISK Default disk drive for file i/o

DELAY       Wait number seconds before Sending a file

DESTINATION Default destination device for incoming files

DISPLAY     For selecting the type of file transfer display

END-OF-LINE Packet termination character

EOF         Method for determining or marking end of file

ESCAPE      Escape character for CONNECT

FLOW-CONTROL Enable or disable XON/XOFF

HANDSHAKE   Half-duplex line turnaround option

INCOMPLETE  What to do with an incompletely received file

LOCAL-ECHO  Specify which computer does the echoing during

CONNECT

MODE-LINE   Whether to display a mode line during terminal

emulation

PARITY      Character parity to use

Kermit-RMB 1.0   Users Manual                                 Page 46

PORT        Select a communications port

PROMPT      Change the "Kermit-RMB>" prompt to something

else

RECEIVE     Request remote Kermit to use specified

parameters

RETRY       Packet retransmission threshold

SEND        Use the specified parameters during file

transfer

SPEED       Communications port line speed (synonym for

BAUD)

TAKE-ECHO   Control echoing of commands from TAKE files

TERMINAL    Emulation and parameters

TIMER       Enable/disable timeouts during file transfer

WARNING     Specify how to handle filename collisions

The SET commands are now described in detail, in

alphabetical order.

SET ATTRIBUTES

|  Syntax:  SET ATTRIBUTES {ON, OFF}

Disables or enables use of Kermit file Attribute protocol

packets, which contain the size, time, and date of files

transferred with packets.  This command is a safety feature

so that a small misunderstanding with another Kermit cannot

block transfers.  SHOW FILE tells whether attributes are on

or off; they are normally ON.

SET BAUD, SPEED

Syntax:  SET BAUD  {baud rate}

SET DEBUG

Syntax:  SET DEBUG {ON, OFF}

Kermit-RMB 1.0   Users Manual                                 Page 47

With DEBUG PACKET, Kermit will display the actual packets on

your screen during file transfer.  With the normal file

transfer display, regular-length packets sent and received

are displayed in fixed-size slots.

With DEBUG ON

control characters are

displayed in uparrow ("^") notation and characters with the

8th bit set are preceded by the tilde ("~") sign, and your

session log (if any) will record 8-bit bytes, rather than

7-bit ASCII, regardless of SET DISPLAY or SET PARITY.

SET DEBUG ON turns on both SESSION and PACKET debugging, and SET

DEBUG OFF turns them both off.

SET DEFAULT-DISK

Syntax:  SET DEFAULT-DISK  {MSI}

Specify the default disk drive to use for file transfer,

directory listings, and so forth. Use CD or MSI to change

the current directory for file management commands.

SET DELAY

|   Syntax:  SET DELAY number

Wait the specified number of seconds before starting a file

transfer.  Intended for use when the other side needs appreciable

time to become ready, such as rearranging cables, changing

programs, etc.

SET END-OF-LINE

Syntax:  SET SEND END-OF-LINE number

If the remote system needs packets to be terminated by anything

other than carriage return, specify the decimal value, 0-31, of

the desired ASCII character.

SET EOF

|  Syntax:  SET EOF {CTRL-Z, NOCTRL-Z}

Controls how the end of file is handled.  CTRL-Z specifies a

Control-Z character should be appended to the end of an incoming

file.

Kermit-RMB 1.0   Users Manual                                 Page 48

SET ESCAPE

Syntax:  SET ESCAPE character

Specify the control character you want to use to "escape" from

remote connections back to Kermit-RMB.  On most systems the

default is "^]" (Control- Rightbracket), which was chosen because

it is a character you would otherwise rarely type.

SET FLOW-CONTROL

Syntax:  SET FLOW-CONTROL {XON/XOFF, ENQ/ACK, NONE}

Specify the full duplex flow control to be done on the

currently selected port.  The options are XON/XOFF and NONE.

The specified type of flow control will be done during both

terminal emulation and file transfer.  By default, XON/XOFF

flow control is selected.  XON/XOFF should not be used on

half-duplex (local echo) connections, or when the other

system does not support it.  If XON/XOFF is used, HANDSHAKE

should be set to NONE.

SET INCOMPLETE

Syntax:  SET INCOMPLETE {DISCARD, KEEP}

Specifies what to do with files that arrive incompletely:

discard them or keep them.  They are normally discarded.

SET LOCAL-ECHO

Syntax:  SET LOCAL-ECHO {ON, OFF}

Specify how characters are echoed during terminal emulation

on the currently selected port.  ON specifies that

characters are to be echoed by Kermit-RMB (because neither

the remote computer nor the communications circuitry has

been requested to echo), and is appropriate for half-duplex

connections.  LOCAL-ECHO is OFF by default, for full-duplex,

remote echo operation.

SET PARITY

Syntax:  SET PARITY {EVEN, ODD, MARK, SPACE, NONE}

Specify the character parity to be used on the currently

Kermit-RMB 1.0   Users Manual                                 Page 49

selected port.  You will need to SET PARITY to ODD, EVEN,

MARK, or possibly SPACE when communicating with a system, or

over a network, or through modems, concentrators,

multiplexers, or front ends that require or impose character

parity on the communication line.  For instance, most HP

mainframe computers use EVEN or MARK parity; Telenet

normally uses MARK parity.  If you neglect to SET PARITY

when the communications equipment requires it, the symptom

may be that terminal emulation works (well or maybe only

partially), but file transfer or script INPUT commands do

not work at all.

NONE means that no parity processing is done, and the 8th

bit of each character can be used for data when transmitting

binary files.  If parity is other than none, then there will

be 7 data bits.

If you have set parity to ODD, EVEN, MARK, or SPACE, then

Kermit-RMB will request that binary files be transferred

using 8th-bit-prefixing.  If the other Kermit knows how to

do 8th-bit-prefixing (this is an optional feature of the

Kermit protocol, and some implementations of Kermit don't

have it), then 8-bit binary files can be transmitted

successfully.  If NONE is specified, 8th-bit- prefixing will

not be requested.  Note that there is no advantage to using

parity.  It reduces Kermit's file transfer efficiency

without providing additional error detection.  The SET

PARITY command is provided only to allow Kermit to adapt to

conditions where parity is required, or 8-bit transmission

is otherwise thwarted.

|  If parity is in use, then the display during terminal

|  emulation, as well as any session log, will be 7-bit ASCII,

|  unless you have SET DEBUG ON.

SET PORT

Syntax:  SET PORT {number}

SET PROMPT

Syntax:  SET PROMPT [string]

This command allows you to change the HP-BASIC Kermit program's

prompt to any other string.

SET RECEIVE

Kermit-RMB 1.0   Users Manual                                 Page 50

Syntax:  SET RECEIVE parameter value

This command lets you modify the ways in which Kermit-RMB

asks the other Kermit to behave.  That is, it controls the

file transfer protocol options for packets sent to

Kermit-RMB by the other Kermit.  The parameters and values

you specify in the SET RECEIVE command are sent to the other

Kermit during initial negotiations.

|  END-OF-LINE number The ASCII value of terminating character

to look for on incoming packets.  Normally carriage return.

Use this command if the other Kermit is terminating its

packets with some other control character.

|  PADCHAR number Ask the remote Kermit to use the given

control character (expressed as a decimal number 0-31, or

127) for interpacket padding.  Kermit-RMB should never

require any padding.

PADDING number Ask the remote Kermit to insert the given

number of padding characters before each packet it sends.

Kermit-RMB never needs padding, but this mechanism might be

required to keep some intervening communication equipment

happy.

START-OF-PACKET number If the remote Kermit will be marking

the beginning of packets with a control character other than

Control-A, use this command to tell Kermit-RMB about it (the

number should be the decimal ASCII value of a control

character).  This will be necessary only if the hosts or

communication equipment involved cannot pass a Control-A

through as data, or if some piece of communication equipment

is echoing packets back at you.

TIMEOUT number Ask the remote Kermit to time out and

retransmit after the given number of seconds if a packet

expected from Kermit-RMB has not arrived.  Use this command

to change the other Kermit's normal timeout interval.

SET RETRY

Syntax:  SET RETRY number

Sets the number of times a packet is retransmitted before

the protocol gives up.  The number of retries can be between

1 and 63, and is 5 by default.  This is an especially useful

parameter when the communications line is noisy or the

remote host is very busy.  The initial packet of a file

exchange is given three times as many retries to allow both

systems to become ready.

Kermit-RMB 1.0   Users Manual                                 Page 51

SET SEND

Syntax:  SET SEND parameter value

The SET SEND command is used primarily to override

negotiated protocol options, or to establish them before

they are negotiated.

END-OF-LINE number ASCII value of packet terminator to put

on outbound packets.  Normally carriage return.  Use this

command if the other Kermit needs its packets terminated

with a nonstandard control character.

PACKET-LENGTH number Use this as the maximum length for

outbound packets, regardless of what the other Kermit asks

for.  Normally, you would use this command only to send

shorter packets than the other Kermit requests, because you

know something the other Kermit doesn't know, e.g.  there's

a device on the communication path with small buffers.

PADCHAR number Use the specified control character for

interpacket padding.  Some hosts may require some padding

characters (normally NUL or DEL) before a packet, and

certain front ends or other communication equipment may need

certain control characters to put them in the right modes.

The number is the ASCII decimal value of the padding

character, (0 - 31, or 127).

PADDING number How many copies of the pad character to send

before each packet, normally zero.

PAUSE number How many milliseconds to pause before sending

each packet, 0-127, normally zero.  This may help

half-duplex or slow systems prepare for reception of our

packet.  Padding characters are sent only after the time

limit expires.

QUOTE number Use the indicated printable character for

prefixing (quoting) control characters and other prefix

characters.  The only reason to change this would be for

sending a very long file that contains very many "#"

characters (the normal control prefix) as data.

START-OF-PACKET number Mark the beginning of outbound

packets with some control character other than Control-A.

This will be necessary if the remote host or the

communication channel cannot accept a Control-A as data, or

if it echoes back your packets.  The remote host must have

been given the corresponding SET RECEIVE START-OF-PACKET

command.

TIMEOUT number Change Kermit-RMB's normal timeout interval;

this command is effective only if TIMER is set to be ON; it

Kermit-RMB 1.0   Users Manual                                 Page 52

is normally ON, with a default interval of 13 seconds.

SET SPEED

Syntax:  SET SPEED rate

Set the transmission speed (in bits per second, commonly

called baud) of the currently selected terminal

communications port to 300, 1200, 1800, 2400, 4800, 9600.

19200 baud has not been tested.

Kermit-RMB appears to log packets at an effective rate of

1200 baud.  Transfers to a 2048 byte buffer allow baud rates

of about any speed, but the packet decoding will proceed at

about 1200 baud.  (uncompiled BASIC).

SET BAUD is a synonym for SET SPEED.

SET TAKE-ECHO

|   Syntax:  SET TAKE-ECHO {ON, OFF}

Specifies whether screen display should occur during

implicit or explicit TAKE operations on KERMINIT or other

Kermit-RMB command files, and during evaluation of macro

definitions by the DO command.  Handy for finding errors in

TAKE files or macro definitions.

SET TERMINAL

Syntax:  SET TERMINAL {type}

VT-100 is the only terminal available with rev. 1.0

|  NEWLINE-MODE {ON, OFF}

ON sends a carriage-return-linefeed combination (CRLF) when

you type carriage return (CR) during terminal emulation.

OFF (default) just sends a CR when you type CR.  Useful in

conjunction with SET LOCAL-ECHO ON when CONNECTing two

S200/300's back-to-back.

SET TIMER

Syntax:  SET TIMER {ON, OFF}

This command enables or disables the timer that is used

Kermit-RMB 1.0   Users Manual                                 Page 53

during file transfer to break deadlocks that occur when

expected packets do not arrive.  By default, the timer is

ON.  If the other Kermit is providing timeouts, you can

safely turn the timer OFF to avoid unnecessary

retransmissions that occur when two timers go off

simultaneously.

SET WARNING

Syntax:  SET WARNING {ON, OFF}

Specify what to do when an incoming file is about to be

stored under the same name as an existing file in the target

device and directory.  If ON, Kermit will warn you when an

incoming file has the same name as an existing file, and

automatically rename the incoming file (as indicated in the

warning message) so as not to destroy (overwrite) any

existing one.  If OFF, the pre-existing file is destroyed,

even if the incoming file does not arrive completely.

WARNING is ON by default as a safety measure, and the

|  current setting may be observed in the SHOW FILE display.

The new name is formed by adding numbers starting at the

last character of the name.

8.10    The STATUS and SHOW Commands

Syntax:  STATUS

The STATUS command displays the values of all the current

SET options.  There are currently no operands for the STATUS

|  command.  Use the SHOW command to see logically-grouped

|  settings, e.g.  SHOW COMMUNICATIONS, SHOW TERMINAL.

|  The SHOW Command

Syntax:  SHOW option

Most parameters that may be altered with SET commands are

displayed by the STATUS command.  The SHOW command is used

for displaying macro definitions, key redefinitions, file

transfer statistics, translations, and other common

groupings.

SHOW COMMUNICATIONS displays the settings of the current

serial port (port, speed, parity, echo, etc) and the status

of modem signals Carrier Detect, Data Set (modem) Ready, and

Clear To Send.

SHOW FILE displays the file transfer control settings, such

Kermit-RMB 1.0   Users Manual                                 Page 54

as the current path, file discard, attributes packets

on/off, warning, end-of-file convention, etc.

SHOW KEY allows you to determine a key's identification code

and what it will send in CONNECT mode, most useful for

obtaining the identification of a key when SET KEY commands

will be placed in a TAKE file.  This command can be done

only interactively (use a ?  to see all defined keys).

Refer to the SET KEY description for details.

SHOW LOGGING Displays the names of the session, packet, and

transaction logs, and tells whether logging is in effect.

SHOW MACROS [macroname] displays the definitions of all

currently defined macros, as well as the amount of space

left for new macro definitions.  A macro name, or

abbreviation, can be included to restrict the list, e.g.

SHOW MACRO HP will display the definition of the HP macro,

and SHOW MACRO X will list the definitions of all macros

whose names begin with X.

SHOW MODEM displays the status of the modem signals DSR

(dataset ready, modem tells the S200/300 that it is turned

on and in data mode), CTS (clear to send, modem grants the

S200/300 permission to send data), and CD (carrier detect,

local modem tells the S200/300 that it is connected to the

remote modem).  The results may be misleading if your

asynchronous adapter, or the connector or cable that is

attached to it, is strapped to supply these modem signals

itself.

SHOW PROTOCOL displays the values of the Kermit

protocol-related parameters, including all the SET SEND and

SET RECEIVE parameters, plus whether the timer, attribute

packets, and logging are enabled.

SHOW SCRIPTS displays the script-related variables.

SHOW SERVER displays which server functions are enabled and

disabled.

SHOW STATISTICS displays counts of characters sent and

received during file transfers, for both the most recent

transfer and the entire session, and an estimate of the

average baud rate while sending and listening.

SHOW TERMINAL displays the terminal settings, which terminal

is being emulated, the tab stops, etc.

SHOW TRANSLATION displays the entries in the 256 byte input

translation table.  Values are expressed numerically to

avoid confusion with different display adapters, and the

command shows only entries for which input and output codes

differ.

Kermit-RMB 1.0   Users Manual                                 Page 55

8.11  The LOG and CLOSE Commands

Syntax:  LOG {PACKET, SESSION}  [Filespec]

CLOSE {PACKET, SESSION,}

The LOG comman tells Kermit-RMB to record the terminal

session, or the file transfer protocol packets themselves in

a log file.  Open log files may be closed (and the

associated logging disabled) using the CLOSE command.  Open

log files are also closed when you EXIT from Kermit.

LOG SESSION is used to record your terminal emulation

typescript.  It was described above, in the section on file

transfer.

The LOG PACKETS Command

Syntax:  LOG PACKETS [filespec]

The packet log is for diagnostic purposes and records each

Kermit protocol packet sent and received in printable

format.  Control characters are written as caret-letter and

characters with the high bit set are shown as their 7-bit

part preceeded by a tilde.  The default filename is

PKT_LOG.  If you experience difficulty with file

transfers the packet log is valuable in discovering who said

what to whom, even though a copy of the Kermit book is

needed to unravel the meaning of each character in a packet.

8.12    Macro Definitions

Like TAKE files, macros provide a way of collecting many

commands into a single command.  The difference between a

macro and a TAKE file is that Kermit keeps all its macro

definitions in memory, and can execute them as many times as

you like, without having to look them up on disk, whereas

every time you issue a TAKE command, Kermit has to access a

disk.  But...  you can have as many TAKE command files as

you like, and they can be as long as you want, whereas

Kermit-RMB's memory for storing macro definitions is

limited.  You can put macro definitions and DO commands for

them in TAKE files, or for that matter, you can put TAKE

|  commands in macro definitions.  There is a limit of 25

defined macros at a time.

The DEFINE Command

Kermit-RMB 1.0   Users Manual                                 Page 56

Syntax:  DEFINE macro-name [command [, command [, ...]]]

Kermit-RMB command macros are constructed with the DEFINE

command.  Any Kermit-RMB commands may be included.  Example:

define telenet set parity zero, set baud 1200, connect

A macro can be undefined by typing an empty DEFINE command

for it, like

define telenet

|   A macro definition may be up to 255 character long. Use a

hyphen to continue lines longer than 80 characters.

The DO Command

Syntax:  DO macro-name [parameters...]

A Kermit-RMB macro is invoked using the DO command.  For

instance:

Kermit-RMB> DEF TEST  SET BAUD 2400 , LOG PACKET , SEND HEX

Kermit-RMB> DO TEST

8.13   Kermit Server Commands

Kermit Server capability is not implemented in version 1.0

8.14   Commands for Controlling Remote Kermit Servers

Kermit Client capability is not implemented in version 1.0

This functionality in some Kermit programs allows users to

send commands to the remote computer's operating system for

execution.

Kermit-RMB 1.0   Users Manual                                 Page 57

9.0  Future Enhancements, Missing Features and Reporting Bugs

The most important missing feature in Kermit-RMB is the Server

and Client capability. The Server operation allows a "client"

kermit process to request file send and receive from a remote

Kermit without having to interact each time to execute the

Kermit SEND or RECEIVE command. Instead, the remote Kermit is

issued the SERVER command once after which the GET and RECEIVE

commands are used to initiate file transfers.

The REMOTE command group allows you to issue Kermit

configuration commands to a remote Kermit without having to be

physically at the remote computer.

The HOST command group is a set of commands which enable you

to execute operating systems commands, such as file

management, on the remote host.

The following functions in version 1.0 of Kermit-RMB have been

implemented, but could be more fully implemented in future

revisions of the program -

- More complete VT-100 emulator

- Server/Client Packets (GET, I)

- More complete HELP facility

- Complete implementation of the SET SEND and SET RECEIVE

commands.

- Implement half-duplex communication with flow control

Reporting Bugs

If you want to report bugs, or more importantly, if you would

like to submit bug fixes, or enhancements you've made to

Kermit-RMB you should contact Columbia University Kermit

Distribution at the Center for Computing Activities, and

myself at the address on the cover of the users manual or in

the header of the HPKERMIT program file.

I'll also request your help in distributing the program and

documentation to any HP-BASIC users that you know of.

Kermit-RMB 1.0   Users Manual                                 Page 58