File  RMKIFACE.DOC

------------------		Chris Kennington	8th July 1985.

Research Machines Kermit Software Interfaces

--------------------------------------------

0.  Structure of RM Kermit.

-----------------------

The bulk of RM Kermit is written in standard C, designed to be

portable between micros and micro operating systems.  The actual

implementations carried out at RML are for the RML 480Z, a Z80-based

micro running under CP/M, and for the RM Nimbus, a 80186-based micro

running (in this instance) under MSDOS.  Since the performace and

acceptability of Kermit depends heavily on its handling of keyboard,

screen and communications line, no attempt was made to use standard

CP/M or MSDOS functions for these purposes.  Instead, a software

interface to these routines was specified and hardware-dependent

low-level routines coded to connect this interface to the necessary

functions.  (Due to the various possible hardware configurations,

there are in fact several different sets of such routines.)

There are also a small number of organizational procedures

which may differ according to the micro OS used and other

characteristics of the environment.

Since a set of routines specific to the implementation have

to be created and included, opportunity was taken to make some of

the texts displayed also variable, so that help and other information

is more closely appropriate to the version being run.

This document describes these interfaces and lists the texts.

For further information about specific cases, see file RMKGEN.DOC.

1.  Keyboard/Screen Interface.

--------------------------

1.1  Screen-Handling.

Kermit mostly divides the screen into a 4-line parameter area

at the top, a fixed divider line, and a 19/20-line scrolling area

occupying the rest of the screen.  When in one of the menu modes,

a further 5-line section plus divider is fixed at the bottom, leaving

14 or 15 lines for the menu.

Some optimisation of output has been done, to provide the fastest

display of information compatible with reasonable flexibility.  Calls to

printf() are used where it is necessary to format binary numbers

and variable strings into other text, but other procedures are used to display

fixed strings and individual characters at the current cursor position.

All map into calls to the special screen-output procedure outc(), via a

formatting routine outfc() which expands tabs etc. and deals with paging.

Output of individual characters and strings to fixed screen-locations

is handled by other procedures.

All this logic deals with output destined for the physical screen.

The built-in C-function putchar() remains the correct procedure for

output of escape-sequences etc. which are in fact intercepted by the

OS or the micro-firmware and interpreted by it rather than being passed

for display.

1.2  Keyboard-Handling

There is only one procedure required for reading the keyboard, kbdin();

it is of type char, and returns immediately with or without a character.

It is assumed that the keyboard is buffering any characters which are input

more rapidly than they are read out (though there is no expectation of readout

falling seriously behind input).  Kbdin() does not deal with null characters

(ctl-@), since the return with this is indestinguishable from the return

without a character.  See below for the data which it is expected that

the arrow and F keys will return.

1.3  Procedures.

The keyboard/screen procedures which are used by Kermit are as follows

(in all cases, the range of values taken by char variables is assumed to be

from 0 to 255):-

curset(line,col)	position cursor

char	line, col;

No return-code.

cursor(sw)		remove/replace cursor

char	sw;

If sw = 0, the cursor is removed from the screen; the coordinates

of the current position continue to be manipulated (by data-output

or otherwise), even though the cursor is not on display.

If sw != 0, it is taken as representing a cursor-character to be

displayed @ the current position.

No return-code.

char kbdin()		read char from keyboard

Returns char in range 1-255, or 0 if no character available.

keyconn()		set keyboard for connect-mode

Performs any reinitializations needed for connect-mode

(when the characters to be sent from certain keys may

differ from those defined below).

No return-code.

keyinit()		initialize

Sets up both keyboard and screen (see below).

No return-code.

keyrest()		restore

Restores keyboard and screen to their normal state.

No return-code

outc(ch)		character to screen @ cursor-position

char	ch;	character

No return-code.

A very limited set of control characters are honoured,

see below for details; others are ignored.

Characters with the top bit set are displayed as inverse

video equivalents of those without.

screen(top,bottom)	define scrolling-area

char	top, bottom;

No return-code.

This defines the scrolling area of the screen to be from "top"

to "bottom" inclusive.  The parameters may be assumed correct,

except that bottom > 23/24 implies the lowest possible line.

(Line-numbering 0 - 23/24.)

The scrolling area is full screen-width.

scroll(n)		scroll screen up/down

int	n;

No return-code.

The scrolling area of the screen is scrolled up or down by the

number of lines defined by n.  n > 0 implies normal scrolling

(data moves up); n < 0 is the opposite (data moves down);

n = 0 is a no-op.  Blank lines are scrolled in at top or bottom,

so a scroll of more than the size of the scrolling area is

a clear-screen.

vtout(loc,ch)		character to screen @ specified position

int	loc;	screen-position (in HL format as defined below)

char	ch;	character

No return-code.

All characters are treated as printable.  For characters in the

range 0-31 those defined for the RML 480Z are assumed, but

any similar set should be acceptable.

vtline(loc,str)	string to screen @ specified position

int	loc;	screen-postion, as above

char   *str;	pointer to string

No return-code.

All characters treated as printable, as above.  If the string runs

off the edge, the end is lost.

1.4  Cursor Definition.

In defining cursor positions, "HL format" means line*256 + col.

In either HL format or when specified as 2 parameters,

line and column are in range 0-23 (or 0-24) & 0-79 respectively, and refer

to the whole screen, i.e. (0,0) is the top left-hand corner, irrespective

of scrolling-area arrangements.  Validity checks are reduced to a minimum

(taking values > max to be = max and < 0 to be = 0), since Kermit is a

(presumably) developed program.

1.5  Control Characters.

CR (0x0d) is defined to move the cursor to the left margin, then move

it DOWN.  LF (0x0a) is defined to move the cursor straight down.

The UP (0x0b) & DOWN (0x0a) cursor movements cause screen scrolling if they

attempt to go off the scrolling area.  The RIGHT (0x18) & LEFT (0x08) cursor

movements are no-ops if the cursor is already on the margin.  Attempting to

store a character (by outc(ch)) when the cursor is at the right margin

causes a "CR" to the beginning of the next line (with scrolling

if on the bottom line).  DEL (7fH) is defined as a destructive

backspace, i.e. it deletes the character (if any) to the left of the

cursor, then moves the cursor LEFT.  Overtyping an existing character

has no side-effects (i.e. there is no "insert" mode).

1.6  Initialization.

Screen initialization implies clearing, homing the cursor to top

left, and setting full-screen scroll.

Keyboard initialization includes switching off any specially-treated

characters such as ctrl-A & ctrl-F.   Kermit uses the arrow-pad and F-keys;

the characters to be returned (other than in connect-mode), with and

without SHIFT, are:-

unshifted	       shifted

---------	       -------

arrow right		0f2H		0d2H

arrow left		0ecH		0ccH

arrow up		0f5H		0d5H

arrow down		0e4H		0c4H

F1			0e2H		0c2H

F2 (TAB)		 88H		 88H

F3			0eeH		0ceH

F4			 9bH		0bbH

DEL/DELT		0x7f		0xff

2.  Communications Interface.

-------------------------

2.1  Manner of Working.

RM Kermit makes no assumptions about whether polled or interrupt-driven

communications are in use.  Procedures return immediately (i.e. they do

not wait for data to be received or transmitted) with a flag indicating

success or error.

In one of the implementations there was a hardware requirement

to quiesce the communications before issuing any commands to the

disk-handling parts of CP/M.  A call s4sleep() was therefore provided,

and is called before all disk operations.  It is a side-effect of any

call to the active procedures s4get(), s4put() or s4reset() to wake up

(un-sleep) the communications.

The interface was designed to be used by other programs as well as

Kermit.  This has resulted in the existence of parameters which are

superfluous, but must be set as shown.  All procedures are type integer,

and return < 1 for an error; success is indicated by 0 or 1.  A call to

any routine when comms are not initialized is an error (-1); in most other

cases the value returned is that of a status-word (see below) which

always has the top bit set.

2.2  Procedures.

s4break(len)	/* send "break" to comms line		*/

char	len;		/* duration in sec/8		*/

(this procedure is not expected to return until the break

has been completely sent.)

s4get(n,buff)	/* read character from comms line	*/

int	n;		/* number of chars available	*/

char	*buff;		/* buffer to receive chars	*/

(in Kermit the value of n is always 1)

returns 0 if no character available, else # of chars actually

read (<= n), else error-status.

s4get7(n,buff)  /* read stripped characters from comms line	*/

int	n;		/* number of chars available	*/

char	*buff;		/* buffer to receive chars	*/

as s4get(n,buff) except that it strips the top bit;

calls to the two may be intermixed.

s4go();		/* open/initialize comms routines	*/

may be called again after s4stop() to reopen comms;

is a no-op if already open.

s4put(n,buff)	/* write characters to comms line	*/

int	n;		/* number of chars available	*/

char	*buff;		/* buffer containing chars	*/

(in Kermit the value of n is always 1)

returns 0 if unable to write because line is busy,

else # of chars actually written (<= n), else error-status.

s4set(options,control); /* set facilities			*/

int	options;	/* facilities-word as defined	*/

char	control;	/* DTR & CTS bits of Z80sio WR5	*/

s4sleep();		/* deactivate comms routines		*/

provided for cases where use of disks and communications are

mutually exclusive, otherwise may be a no-op.

s4speed(code)	/* set speed of comms line		*/

char	code;		/* RML speed-code, 0-9		*/

this routine may be called before s4init().

s4stop();		/* close down comms routines		*/

this routine is a no-op if comms are already stopped

s4test();		/* test status of communications	*/

returns status-word as defined but with top bit set,

hence always <0.

2.3  Returned Status-Word.

This integer was originally a bit-pattern with meanings assigned

to permit careful control of communications.  Kermit only requires that

a negative value (top bit set) should be returned if there is an error.

2.4  Option-Word and Control-Byte.

These were also originally designed with a wide range of possibilities.

The only bits of significance to Kermit are as shown below; all other

bits may be ignored.

Option-word:

0  -  RTS/CTS flow-control to be used

1  -  XON/XOFF flow-control to be used

8/9 -  Parity-Generation on/off

The 2-bit code for parity is:

0  -  Generation not required

1  -  Enforce odd parity

2  -  Enforce even parity

3  -  Enforce mark parity

The only bit of the control-byte which is significant is bit7;

this is set to 1 to raise DTR or an equivalent signal, 0 to drop it.

2.5  Speed-Codes.

The following speed-codes are defined (though not necessarily

all implemented in any given version):-

0  -	110baud (2 stop-bits)

1  -	300baud

2  -	600baud

3  -   1200baud

4  -   2400baud

5  -   4800baud

6  -   9600baud

7  -  19.2Kbaud

8  -  40.8Kbaud

9  -	 75baud

There is no provision for split-speed working, nor for

half-duplex line turnaround.  8-bit data is always set, and

1 stop-bit except at 110baud.

3.  Miscellaneous Interface.

------------------------

The following miscellaneous procedures are called by Kermit to

perform system-oriented functions:-

cpmgo()		/* return to micro OS		*/

this call is used instead of "exit()" to permit a clean return

to the micro's operating system after carrying out any

required housekeeping.

fpanel()		/* debug display		*/

this call is executed after an "ESC-F" break-in to access

the micros's built-in lowest-level debugging display.

It may be a no-op if no such facility is available

or required.

unsigned getram(n)	/* get storage			*/

int  n;			/* # of bytes required	*/

this call is used instead of "malloc()" to obtain dynamic

storage so that the space occupied by initialization-code

may be reused; it returns the start-address of the

block of storage.  If unable to allocate it should

printf() a failure-message and call cpmgo().  Storage

once allocated is never released.

netcool()		/* deactivate disk system	*/

this call is made before reading or writing the communications

line; it would normally be a no-op.

s4env()		/* return environment		*/

this call returns a small integer value to indicate the communications

environment in which it is running.  The following values

have been defined:-

0  -  480Z via SIO4

1  -  480Z via IDC

2  -  Nimbus via Data Comms Controller (n.y.i.)

3  -  Nimbus via Aux port

4  -  Nimbus via Piconet

5  -  480Z or Nimbus via RML Network (n.y.i.)

6  -  non-standard

(Any user extending this table will have to amend the entries

in arrays locations[] and fuzz[] in RMKDATA.C)

The versions of these routines in file RMKNIMB.C should be suitable

for most straightforward micros.

4.  Data Items.

-----------

There is no absolute need to change any of the texts displayed.

The versions found in files RMK480Z.C and RMKNIMB.C (for CP/M and

MSDOS systems respectively) may well be acceptable; the examples

below are those from the Nimbus version.

Data items which should be supplied for display are as follows:-

char head1[]		text-string to identify program, e.g.

"  RM  NIMBUS  Kermit  File  Transfer  Program,";

char prompt[]		normal prompt to be displayed by RM Kermit (there is

no ability to change this dynamically), preceded by CR, e.g.

"\rKmNimbus";

char eprompt[]		special prompt to be displayed after ESC break-in, e.g.

"\rKmF4> ";

char backto[]		final message to be displayed when Kermit terminates, e.g.

" OK - back to MSDOS; ";

If these texts are altered, it is recommended that their lengths should

not be extended.

*******************************************************