Implementation of KERMIT for Pecan Software Systems UCSD p-System

R. Tim Coslet

R_Tim_Coslet@cup.portal.com

Version 1.1

KERMIT-UCSD  is a  program  which implements the KERMIT file-

transfer protocol for use under the Version IV.x UCSD p-System on

various computers.  The distributed version has been developed on

an  Atari MEGA ST2 under  Pecan Software Systems, Inc. p-System

Version IV.2.2 with the Standard File System (SFS).

This implementation was done by  Tim Coslet  from the UCIBMPC

implementation  by  Steven   Pacenka,   which  was  based largely

on  an  implementation  by  Kate MacGregor of  Cornell  Computing

Services for the Version II.0 p-System on a Terak 8510a.

Summary of Kermit-UCSD Capabilities at a Glance

--------------------------------------------------------------

Local operation                      Yes

Remote operation                     No

Transfers text files                 Yes

Transfers binary files               Yes

Wildcard send                        Yes

^X,^Z interruptions                  Yes

Filename collision avoidance         Yes

Send Literal/Converted filenames     Yes

Can time out                         Yes

8th-Bit prefixing                    Yes

Repeat count prefixing               No

Alternate block checks               No

Communication settings               Yes, Baud & Parity

Transmit break                       Yes, dependent on REMUNIT

Support for dialout modems           No

IBM mainframe communications         Yes

Transaction logging                  No

Session logging                      No

Packet logging                       No

Debug logging                        Yes

Raw file transmit                    No

Act as server                        No

Talk to server                       Yes

Local file management                No

File Attributes packets              Yes, date only

Selectable user interfaces           Yes, Kermit & UCSD styles

Take initialization command file     Yes, *KERMITINFO.TEXT

Take commands from file              Yes

Command macros                       No

Login Scripts                        No

Large packets                        No

Extra Large packets                  No

Sliding windows                      No

Terminal emulation                   ??

Has Datamedia from UCIBMPC but I had

problems with it. Didn't need it on

Atari ST as it already partially emulates

a VT52 terminal.

CONSTRUCTING A NEW KERMIT

The program consists of several Pascal source files which

need to be compiled.  Also required is a REMUNIT.CODE to

interface with your computer's serial port.  REMUNIT's are

available for most computers running the p-System, from USUS.

This version also uses several Library Units provided by

Pecan Software Systems: KERNEL, SCREENOPS, DIRINFO, WILD,

and optionally ERRORHANDLING.

If you have the Advanced File System (AFS) you may also

use Pecan's Library Unit ATTRIBUTES. (current versions of this

unit appear to have a bug that prevents them from operating

correctly under the Standard File System (SFS).)

The source files (which are concatenated into a single large

file on the distribution tape) are:

KERMIT

UCSD Name      Distribution                  Contents

Name

--------------   ------------   --------------------------------------------

UCPECAN.D.TEXT    UCPECA.DOC    This documentation

UCPECAN.P.TEXT    UCPECA.PAS    Following source files (self separating)

KERMIT.TEXT         "           Main program

SENDER.TEXT         "           UNIT Sender (file sending)

RECEIVER.TEXT       "           UNIT Receiver (file receiving)

CLIENT.TEXT         "           UNIT Client (talks to server)

HELPER.TEXT         "           UNIT Helper (online help info)

PARSER.TEXT         "           UNIT Parser (command parsing)

INTFUTIL.TEXT       "           Interface part of KermUtil UNIT

FAKEUTIL.TEXT       "           UNIT KermUtil (fake implementation)

KERMUTIL.TEXT       "           UNIT KermUtil (misc utilities)

SETSHOW.TEXT        "           Include file for KERMIT.TEXT

KERMENUS.TEXT       "           UNIT KerMenus (UCSD style Menus)

KERMPACK.TEXT       "           UNIT KermPack (packet utilities)

KERMGLOB.TEXT       "           UNIT KermGlob (declarations)

UCPECAN.M.TEXT      "           "make" file (compiles in correct order)

UCPECAN.U.TEXT    UCPECA.UPD    Update information (V1.0 -> V1.1)

The .PAS file is a "self separating" text file containing

the Pascal source files, the .DOC file (obviously) contains

this user documentation and instructions for building UCSD

Pascal Kermit, and the .UPD file contains a list of the

changes made from V1.0 to V1.1 of Kermit-UCSD.

All of the .PAS modules should compile and execute on any

adaptation of Version IV of the UCSD p-System, provided that

an equivalent of the REMUNIT.CODE unit is available for the

particular computer being used.  (This file implements the

UCSD p-System Users' Society (USUS) "standard remote unit.")

To generate a variant of this  KERMIT  for  another  computer

running the Version IV p-System:

1.  Create  or  obtain  a  REMUNIT  that  implements  the  USUS

standard  remote unit.  Specifications and  implementations

for a variety of different  computers  are  available  from

USUS.  For  communications  above  300  baud,  the incoming

characters to REMIN: should be  processed  in  a  buffered,

interrupt-driven mode.  The  buffer  should be at least 256

characters long.

2.  Change all instances of the volume name "syslibr:" in the

.PAS file to the name of your volume containing the

Pecan Library Units.

3.  Using ASE (Advanced System Editor) separate the Pascal source

files in the "self separating" .PAS file by performing the

following steps:

a)  Create a Volume with at least 720 free blocks. Make this

Volume your "Prefix Vol" (default).

Note: This will run fastest from ramdisk. So if your

RAMDISK: Volume is large enough ( >1030 blocks )

then you may creat this Volume as a .SVOL on RAMDISK:.

If your RAMDISK: is not large enough to do this

then a Virtual Vol on HardDisk is next best. If

you have only floppies available, it will be very

slow, but will still work (provided they are large

enough. 720 blocks is the size of a SSDD "Native"

p-System floppie on the Atari ST).

b)  Make sure that the RAMDISK: still has at least 300 block

of free space left over (for the extracted source files).

c)  Put the "self separating" .PAS file on your "Prefix Vol"

as UCPECAN.P.TEXT (its UCSD Name, see table above).

d)  E(dit UCPECAN.P

e)  Press the <f1> Function Key. This will initiate the

file extraction, which takes several minutes.

f)  If it completes with NO Errors then Q(uit E(xit ASE.

Note: If errors occur attempt to correct them and try

again (e.g. "not enough room on vol" means you need

more space, try again with larger Volumes). If

problems persist you may extract the files by hand;

they are separated with lines in the format below...

{>>>> filename}

The file UCPECAN.M.TEXT must have this line removed

before X(ecuting it (this is normally done

automagically in the "self separation" process).

g)  It would be a good idea to copy the extracted .TEXT files

to another Volume at this time (i.e. backup floppy).

4.  Place all of the source Pascal files together on your usual

prefix volume along with the REMUNIT.CODE file.

5.  Configure the conditional compilation switches in these files

to the state required by your environment:

Unit         Compiler Control

--------     ----------------------------------------------

KERMUTIL     {$D OS_ERHDL+}  change + to - if ERRORHANDLING

is not available.

{$D OS_TIMER+}  change + to - if time() does

not work on your machine.

SENDER       {$D AFS-}       change - to + if AFS available

RECEIVER     {$D AFS-}       change - to + if AFS available

6.  Compile the  Pascal  units  by  selecting the X(ecute command

and entering the following execution-option string:

i=ucpecan.m

7.  At  this  point  you  can   use   the   standard   p-System

LIBRARY.CODE  program  to  combine  the  units   (including

REMUNIT) into one code file for easier and faster  loading,

or you can enter their individual .CODE file names into the

USERLIB.TEXT file on your root disk.

8.  Optionally creat a file (with ASE) called KERMITINFO.TEXT

in your boot volume containing any initialization commands

that you may need.

Example:

set system-id atari mega ST2

set file-warning on

set baud 4800

LIMITATIONS

1.  No character repeat counts are used in packets.

2.  '?'  and <esc> cannot  be  used  when  entering  a  command

line.   Some  versions of Kermit parse  commands  from  the

keyboard character  by character as they are entered.  This

version interprets the line after it has been terminated by

a carriage return.  Use the  HELP  command in place of '?',

and type out the command names in full.

3.  All linefeed characters received during TEXT file transfer

are stripped from the local file that is being  created,

since the p-System does not recognize them in text files. This

may cause difficulties in files received from certain other

computers which omit carriage returns at the ends of lines,

particularly blank lines. But if both Kermits are correctly

set for TEXT (ASCII) transfer you should not have a problem.

4.  BINARY file transfers are exact only if you use the ATTRIBUTES

Unit (which at this time only works under AFS). Without this

Unit file size will be rounded up to the next full block

(512 bytes) and filled with NUL characters: chr(0).

5.  In this version of Kermit-UCSD the "UCSD style" user interface

Unit remains resident durring file transfers (unlike the PARSER

Unit used by the "Kermit style" user interface). This could

present problems on machines using an internal Codepool. If

this causes problems for you (e.g. *STACK OVERFLOW* while

transfering files) then avoid using the menus, just use

the "normal" Kermit command mode.

6.  I was not able to get access to a "Known working" Kermit that

implements attribute packets to verify the operation of the

protocol. They have only been tested against another copy of

this program.

7.  Limited server communications are supported (only GET, PUT,

BYE, and FINISH) in this version. The CLIENT Unit contains

code for handling "X" packets sent by a server, but I only

had access to a "Known working" Kermit Server for a limited

time so this code is untested (and probably won't work).

8.  V1.0 had a problem where occasionally the Creation Date

passed in an attribute packet was not correctly recorded.

While trying to locate this problem I added some test code

to the RECEIVER Unit (debugdate) that rereads the file's

date and have been unable to duplicate the problem since

then (I was duplicating it regularly before this). I have

left this code in, since it appears to fix the problem. But

as I do not know the cause of the problem I can not be sure

that it is really fixed. If anyone using this program

encounters this problem, please send me a copy of a debug

log file showing the problem.

COMMANDS

The commands recognized by KERMIT-UCSD are listed below.

CONNECT         To make  a  "virtual  terminal"  connection  to a

remote  system.   The  CONNECT  command  may   be

abbreviated to 'CON'.   When  in  CONNECT mode, all

typed characters are sent to the serial interface

except the  escape  character  (see  SET  ESCAPE,

below).  The escape character  may be followed by

another  character   which   is   interpreted  as

follows:

c           Break  the  connection  and  "escape"

back to the micro

b           Send   a  "break"  signal  over   the

communications  line;  this  is  only

useful on mainframes such as the  IBM

3081/370/4341 et.  al.

s           Same as the  "show  all"  command  in

command  mode;  displays the  current

parameter  settings   for  the  local

KERMIT.

?           Displays the possible  characters  to

follow the escape code

<Escape char>

Two consecutive  escapes are required

to send one  such  character  to  the

communications line

EXIT            To return to main p-System command level.

QUIT            Same as EXIT.

HELP            To get a list of KERMIT commands.   HELP  can  be

followed by any  command,  in which case the help

will refer only to that command.

RECEIVE         To accept a file  from  the  remote system.  If

FILE-TYPE is TEXT the suffix ".TEXT" will be

appended, if not already present in the received

file name. If FILE-TYPE is BINARY the name will

be used as received without modification.

SEND            To send a file to the  remote  system.

Wildcards may be used in the filename to send

multiple files. The wildcard system used here

(From Pecan's WILD Library Unit) is an "extension"

of "standard UCSD" wildcards.

?  -  Matches any single character.

=  -  Matches any sequence of 0 or more chars.

%  -  Escape char. to allow special characters

to be matched literally.

{} -  Matches any in specified set.

a-c  is subrange.

,    seperates elements.

~    excludes char from set.

{a-c,~b} is same as {a,c}

*  -  Following {} matches 0 or more of

the specified characters.

+  -  Following {} matches 1 or more of

the specified characters.

GET             To request a file from a remote Kermit in SERVER mode.

PUT             To send a file to a remote Kermit in SERVER mode.

Wildcards may be used in the filename to send

multiple files.

BYE             Shutdown and logout a remote Kermit in SERVER mode.

FINISH          Shutdown a remote Kermit in SERVER mode.

SET             To  establish  system-dependent parameters.   The

SET options are as follows:

BAUD         To set the communications baud rate.

Permisable values are dependant on

your Hardware and REMUNIT.

(default is 1200).

DEBUG        To set debug mode ON or OFF (default

is OFF).

EMULATE      To set DataMedia 1520A screen control

code interpretation ON or OFF (default

is OFF).

ESCAPE       To change the escape sequence that

lets you return to the PC Kermit from

the remote host.  The default is CTRL-

] c.

FILE-NAMES   LITERAL/CONVERTED, Default is

CONVERTED, In this Kermit LITERAL

Names have Volume name Stripped, while

CONVERTED Names also have all but the

final '.' removed.

FILE-TYPE    BINARY/TEXT Default is TEXT.

FILE-WARNING ON/OFF, default is OFF.  If ON, Kermit

will rename an incoming file so as not

to write over a file that currently

exists with the same name.

IBM          ON/OFF, default is OFF.  This flag

should be ON only when transfering

files between the PC and an IBM VM/CMS

system.  It also causes the parity to

be set appropriately (mark) and activates

local echoing.

INTERFACE    KERMIT/UCSD, default is KERMIT. Permits

selection of prefered User Interface:

KERMIT command line or UCSD menus.

LOCAL-ECHO   ON/OFF, default is OFF.  When on, this

causes characters typed at the

keyboard during connect mode to be

echoed to the screen.

PARITY       EVEN, ODD, MARK, SPACE, or NONE. NONE

is the default but if the IBM flag is

set, parity is set to MARK. This flag

selects the parity for outgoing and

incoming characters during CONNECT and

file transfer to match the

requirements of the remote computer.

SYSTEM-ID    Specify the System-ID for your REMUNIT

if your REMUNIT needs it specified.

Called "model" in the REMUNIT specs.

Default System-ID is UNKNOWN

SHOW            To  see  the  values  of parameters that  can  be

modified via the SET command. For an explanation

of the parameter, see the help for the matching

SET command.

Additional SHOW options are as follows:

ALL          Show all parameters.

VERSION      Show version information.

Note: there is no SHOW INTERFACE command (it was

not needed as you already can see which

interface you are using).

TAKE            This command instructs Kermit to take further

commands from a specified file.

COMMENT         Comments a TAKE file. (ignored)

LOG             This command opens a selected log file.

LOG options are as follows:

DEBUG        open specified file for debug output.

CLOSE           This command closes a selected log file previously

opened via the LOG command.

WISH LIST FOR FUTURE VERSIONS

If anyone  tries these, please consider portability and share

your work with the rest of the p-System community.

1.  Increased memory buffering  of  sent  and received packets.

Now only the standard 512 character buffers provided by the

operating system  are utilized.  On floppy disk systems the

throughput is slowed considerably by frequent disk access.

This may require you to write your own UCSD .TEXT output

formatter (which will need to be bypassed if the file being

received is not a .TEXT or .BACK type file).

2.  Implementation  of  the  immediate '?'   (help)  and  <esc>

(abbreviate) commands in the parsing routines.

3.  A DIR command to list a local volume directory.

4.  Text  file  capture  with memory  buffering,  and  XON/XOFF

protocol.  (For remote computers not supporting Kermit.)

Note: on the Atari XON/XOFF protocol is already implemented

in the Host's Operating System (TOS).

5.  Text file transmission without protocol.

6.  SET TIMEOUT command. (Hardware timing is now available, but

the timeout is currently fixed at 5 seconds).

CREDITS AND ADDRESSES

1.  Pecan Software Systems, Inc.

1410 39th Street

Brooklyn, NY 11218

(800) 63-PECAN    (orders)

(718) 851-3100

Library Units used by this program.

2.  USUS      (UCSD p-System Users' Society)

P.O. Box 1148

La Jolla, CA 92038

REMUNIT specification and sources.

3.  R. Tim Coslet

Usenet:   R_Tim_Coslet@cup.portal.com

{sun,atari}!portal!R_Tim_Coslet

BIX:      r.tim_coslet

Programmer (me).

The above e-mail addresses may be used to contact

me if you have problems with this version of Kermit.