BBC KERMIT USER GUIDE

=====================

This  guide  describes how to use the implementation of KERMIT for the

BBC Computer produced by  the  Computing  Department's  Communications

Group at Lancaster University. BBC KERMIT may be used on BBC models B,

B+  and  B+128; and also on the Master 128. It operates with the Acorn

DFS, 1770 DFS, ADFS, Econet, and  any  other  Acorn-compatible  filing

system.

The information in this edition applies to version 1.42

EDITION 4.1

July 1986

Alan Phillips

BBC KERMIT User Guide

CONTENTS

1.  INTRODUCTION

1.1  BBC KERMIT capabilities at a glance

2.  AN OVERVIEW OF KERMIT

3.  CONTROLLING BBC KERMIT

3.1  Entering BBC KERMIT

3.1.1  The RAM version

3.1.2  The sideways ROM version

3.2  Leaving BBC KERMIT

3.3  BBC KERMIT command language

3.3.1  Command format

3.3.2  Abbreviating commands and parameters

3.3.3  Numeric parameters

3.3.4  Obtaining help

3.4  Reading commands from a file

3.5  Storing parameter settings

3.6  Setting the command screen width

3.7  Function and cursor keys

3.8  Using an auto-boot file

4.  USING BBC KERMIT AS A TERMINAL EMULATOR

4.1  Running a terminal session

4.1.1  Choosing the terminal emulation required

4.1.2  Setting the line speed

4.1.3  Setting parity

4.1.4  Selecting the flow control method

4.1.5  Specifying an "ignore" character

4.1.6  Starting terminal emulation

4.1.7  Sending a break signal

4.1.8  Using the function keys

4.1.9  Using the cursor keys

4.1.10 Pausing screen output

4.1.11 Returning to command mode

4.2  Logging output to a disc file

4.3  Logging output to a printer

4.4  Sending files to a host without KERMIT

4.5  VT52 keypad emulation

BBC KERMIT User Guide

5.  TRANSFERRING FILES WITH KERMIT

5.1  Principles

5.2  File type

5.2.1  Binary files

5.2.2  Printable text (ASCII) files

5.2.3  How to decide on the file type to use

5.3  Sending eight bit data

5.4  Starting up the mainframe KERMIT

5.5  Using BBC KERMIT with a remote server

5.5.1  Sending files to a server

5.5.2  Fetching files from a server

5.5.3  Controlling a remote server

5.5.4  Closing down the server

5.6  Using BBC KERMIT with a remote non-server

5.6.1  Sending files to a non-server

5.6.2  Receiving files from a non-server

5.7  Transferring data to and from memory

5.8  Transferring data to a parallel printer

5.9  Handling problems

5.10 Advanced facilities

5.10.1  Interrupting transfers

5.10.2  Using timeouts

5.10.3  File name translation

5.10.4  Detailed protocol control

Appendices

A1. BBC KERMIT COMMANDS

A1.1  Commands for general control of BBC KERMIT

A1.2  Commands for transferring data

A1.3  Commands for terminal emulation

A1.4  Commands for control of remote servers

A1.5  Commands for detailed protocol control

A2. OBTAINING, BUILDING AND MODIFYING BBC KERMIT

A2.1 Obtaining BBC KERMIT

A2.1.1 The source files

A2.2 Building BBC KERMIT from a hex file

A2.3 Building BBC KERMIT from source

A2.3.1 Source file layout

A2.3.2 The assembly process

A2.4 Changing KERMIT defaults

A2.4.1 Changing the source

A2.4.2 Patching the object code

A2.4.3 Format of the defaults block

A2.5 The hex to binary converter program

A2.6 Contact addresses

BBC KERMIT User Guide

A3. USING THE EDT EDITOR ON VAX/VMS

A3.1 Setting up the terminal details

A3.2 Edit keypad keys

A3.2.1 Models B, B+ and B+128

A3.2.2 The Master 128

BBC KERMIT User Guide

1: INTRODUCTION

This  user  guide  describes  the  KERMIT implementation on the BBC

Micro produced by the Communications Group of the Computing Department

at Lancaster University. It is intended to provide enough  information

for  a  novice KERMIT user to be able to transfer data to and from his

BBC micro to another KERMIT system. Other KERMIT systems are  desribed

only  in  passing:  thus you will almost certainly need to consult the

equivalent user guide for the KERMIT system on the other machine.

The guide is divided into several chapters. The next chapter  is  a

general  overview of KERMIT as a whole, and explains its advantages as

a file transfer system over "dumb  capture"  pograms.  The  succeeding

chapter describes the command language that BBC KERMIT uses. Following

that  are  chapters that describe how to use BBC KERMIT as a terminal,

and how to use it to transfer data.

The appendices comprise the "reference section". They  describe  in

full  detail  the  commands  available in BBC KERMIT, grouping them by

functionality  (i.e.  "Commands  for  file  transfer",  "Commands  for

terminal  emulation",  etc).  They  also  describe  how  to obtain BBC

KERMIT, and, having obtained it, how to build  it  from  the  assembly

language sources or modify the compiled binary version.

BBC  KERMIT is, of course, freely available to anyone who wants it.

It can be obtained from  the  KERMIT  tapes  distributed  by  Columbia

University;   alternatively,  it  can  be  picked  up  from  Lancaster

University's KERMIT distribution service. This latter  option  enables

it  to  be  acquired  either  over  file  transfer  from the Lancaster

University VAX 11/780 system, or on Acorn format discs, or  (in  small

numbers)  as  programmed  EPROMs.  The  Lancaster  KERMIT distribution

service also maintains on-line bulletin files giving  details  of  new

releases of BBC KERMIT and of reported bugs: these can be consulted in

a public-access username.

Lancaster  University  intend  to  continue  development of the BBC

KERMIT system. We welcome any comments or  suggestions  that  you  may

wish to pass on, as well as reports of bugs, problems and deficiencies

in  the  program  or  the  documentation.  The  addresses are given in

Appendix 2.

1

BBC KERMIT User Guide

1.1 BBC KERMIT CAPABILITIES AT A GLANCE

Local operation                    Yes

Remote operation                   No

Transfer text files                Yes

Transfer binary files              Yes

Wildcard send                      Yes

^X/^Z interruption                 Yes

Filename collision avoidance       Yes

Can time out                       Yes

8th-bit-prefixing                  Yes

Repeat count prefixing             No

Alternate block checks             No

Terminal emulation                 Yes

Communications settings            Yes

Transmit BREAK                     Yes

IBM mainframe communication        Yes

Transaction logging                No

Session logging (raw download)     Yes

Raw transmit                       Yes

Act as server                      No

Talk to server                     Yes

Advanced server functions          No

Advanced commands for servers      Yes

Local file management              Yes

Handle file attributes             No

Command files                      Yes

Command macros                     No

2

BBC KERMIT User Guide

2: AN OVERVIEW OF KERMIT

KERMIT is a system, devised at the Center for Computing  Activities

at  the  University  of  Columbia  in  New York (CUCCA), to permit the

simple and flexible  transfer  of  data  from  a  microcomputer  to  a

mainframe  or  another  microcomputer.  CUCCA  retain the copyright on

KERMIT (the programs are not "public domain"), but have published full

information on it and permit anyone  to  implement  it  on  their  own

machines,  provided  this is not done for commercial purposes and that

copies are sent to them for distribution. The result is that KERMIT is

now available on a very wide range of machines indeed: very few micros

and mainframes do not have a KERMIT of some sort  suitable  for  them,

and  the programs can be easily acquired from the Lancaster University

KERMIT distribution service.

The primary design aim of KERMIT is to permit the reliable transfer

of any data whatsoever between systems, and to make the data usable on

the system that receives it if this is  possible.  To  illustrate  why

this  is  important,  and  not  possible  with  simple systems, we can

consider an ordinary terminal emulation system that allows data to  be

captured into files or sent from them.

Simple  terminal  emulator  systems,  such  as  those  commercially

available for the BBC micro, do permit you to transfer  files  from  a

mainframe  in  a  rudimentary way. You would tell the emulator to copy

any characters that appear on the screen into a  file,  then  ask  the

mainframe to display the file. The reverse process would let you input

data into a mainframe file from your BBC discs.

The  problems arise in the nature of the communications system that

connect the micro to the mainframe, and how the mainframe itself  uses

this  system.  A  character of data in a file occupies one byte, which

consists of 8 binary digits or "bits". If you regard  the  pattern  of

bits representing a character as a number, this allows numbers ranging

from  0  to  255 to be used. However, many communications systems will

allow only 7 of the eight bits to be transmitted along them. The  most

significant   bit,   termed   the   "parity   bit",  is  used  by  the

communications system as an error-checking device. Thus,  even  though

you  send  a byte of 8 bits to the mainframe, it may receive only 7 of

them. This immediately restricts the range of characters that  can  be

sent to those whose codes are in the range 0 to 127.

A  further  restriction may be imposed if the communications system

uses some of those characters  for  its  own  control  purposes:  thus

systems  often  will  use  the characters whose codes are 17 and 19 to

prevent overloads occurring. In  such  systems,  you  cannot  transmit

these  characters  at  all.  To make matters even worse, some machines

will (apparently arbitrarily) decide that you could not possibly  want

to  send some characters, so, if you do send them, it will change them

into something else entirely.

As far as the BBC micro is concerned, you  could  just  about  live

with  such  problems.  The  character  range  0  to 127 covers all the

printable characters, so that  transferring  text  files  should  just

about be possible. Of course, if the communications line you are using

3

BBC KERMIT User Guide

is unreliable or noisy (a dial up connection over telephone lines, for

instance,   can   be  expected  to  garble  a  significant  number  of

characters) there is  nothing  to  prevent  data  being  corrupted  in

transmission,  so  that  you  will  never  be  sure that the data that

arrives is the data that you sent.

But although text  files  are  about  manageable,  those  including

teletext  control  codes  or  word  processor control codes are highly

unlikely to be, since such codes are likely to lie in the range 128 to

255. And tokenised BASIC programs produced by SAVE haven't a  hope  of

being transferred in any useful way at all.

KERMIT  overcomes  all  these  difficulties by encoding the data it

sends according to a standard set of rules or  "protocol".  The  rules

recognise   that   many   characters  cannot  be  transmitted  down  a

communications line, so  if  those  characters  occur,  they  will  be

translated  into something that can be transmitted. The receiving end,

of course, will translate them back again  to  what  they  were.  This

technique  enables you to send any data at all, even SAVEd BASIC files

or machine code programs. It further guarantees that the data you send

is the data that  arrives,  since  KERMIT  uses  special  methods  for

detecting  garbling and will repeat any transmissions that did not get

through correctly. KERMIT's encoding and checking techniques are  more

efficient than some other systems that offer this facility, since only

bytes that need encoding actually are encoded, thus keeping the volume

of data sent to the minimum possible.

Besides  the problems of actually transferring data corectly, there

is  the  problem  of  making  it  usable  on  the  other  end  of  the

transmission link. If you are sending, say, a SAVEd BASIC program to a

VAX,  this  isn't  a problem, since the VAX can't understand BBC BASIC

anyway. Nor does it matter if you  use  the  VAX  system  only  as  an

archive:  it's  irrelevant how the data is held on the VAX, as long as

when it is brought back to the BBC side it looks the same as  when  it

was sent.

The  usability  problem  does appear, though, if you want to move a

file from a BBC to a VAX and then actually use  it  on  the  VAX.  You

might,  for  instance, word process a file on a BBC, then send it to a

VAX to be printed. In this case, you do not want to transfer the  data

byte-for-byte,  since  the  way the BBC and the VAX denote things like

the end of each line of text will almost certainly be different.  What

you  require is that the file of printable lines on the BBC, which you

can process on that machine, becomes a file of  printable  records  on

the VAX, that can be processed there.

Using  a  dumb terminal emulator system would probably let you send

the data, but it would appear byte-for-byte as it was on the BBC.  And

probably  you  would  get  a file on the VAX with extra line-feeds and

carriage-returns that would need laborious editing  before  you  could

use the file sensibly.

With  KERMIT  the  problem  can  easily be circumvented. The KERMIT

protocols define a standard way of indicating the end of  a  printable

line.  When  you  send  a file from the BBC, BBC KERMIT will translate

whatever ends the lines of text in your file into this  standard  form

4

BBC KERMIT User Guide

before  sending  the  data.  The  receiving  end, seeing this standard

end-of-line indicator, will translate  it  into  however it  indicates

end-of-line.  You  thus  end  up  with a usable file of lines, with no

extra characters anywhere.

The requirements you must meet before using KERMIT are simple.  You

will need a BBC KERMIT in your BBC micro; a KERMIT program in whatever

mainframe  or micro you wish to transfer data to; and a way of linking

the machines, be it a network, an ordinary cable, or a  piece  of  wet

string.

For  a  micro  to micro transfer KERMIT is extremely simple to use.

You would, for example, tell one machine that it is going to receive a

file, tell the other to send it, and sit back and let them get on with

it. Micro to mainframe transfers involve an extra step, which is  also

simple.  BBC  KERMIT  includes  its own terminal emulator program: you

initially use this to log in to the mainframe as though the BBC  micro

were  an  ordinary  terminal.  Once  logged  in,  you start the KERMIT

program on the mainframe, and can then flip from  giving  commands  to

this  mainframe  KERMIT,  to giving commands to BBC KERMIT. As before,

once you have told each side to transfer a file, you just sit back and

relax while it happens.

And KERMIT provides one further facility to  help  you  spend  your

time  doing  more useful things. As well as sending one file at a time

from one machine to the other, you can send them in groups: thus,  you

could  say  "send all the files on my disc to the VAX" in one command.

The KERMIT programs will send the files one by one until all are gone,

quite automatically.

5

BBC KERMIT User Guide

3: CONTROLLING BBC KERMIT

In this section we shall look at how you start and stop BBC KERMIT,

and also how its command language operates.

6

BBC KERMIT User Guide

3.1 ENTERING BBC KERMIT

BBC KERMIT exists in two  versions:  one  that  runs  as  an  ordinary

program  in the BBC Computer's main memory, and one that is programmed

into an EPROM and runs as a sideways ROM. The versions have  the  same

file transfer facilities, but the version that runs in memory provides

only a 40-character terminal emulation on the model B as opposed to an

80-character  VT52  emulation.  On models B+ and B+128, and the Master

128, both versions give identical terminal emulation facilities.

BBC KERMIT is not compatible with the 6502 second processor, so  if

you  have  been  using  one  you  will  need to power it off and press

CONTROL-BREAK to reset the system. Similarly it will not  run  on  the

Master  Turbo-Card:  you  will  need  to  use  the  control  panel  or

*CONFIGURE command to disable the card before using BBC KERMIT.

3.1.1 The RAM version

The RAM version of BBC KERMIT will be kept on a disc, and  it  is

up  to  you what you call the file. Assuming it is called KERMIT,

you could enter it with the command

*RUN KERMIT

If the file is in the default or library directory, you could run

it by simply typing

*KERMIT

The BBC Computer User Guide and the guide  for  the  Disc  Filing

System you are using will explain this in more detail if you wish

to consult them.

3.1.2 The sideways ROM version

The  EPROM  containing  the  sideways  ROM  version of BBC KERMIT

should be fitted into one of the sideways  ROM  sockets  on  your

machine  in the same way as other sideways ROMs such as Wordwise.

If you are using a model B or B+ with a sideways  RAM  board,  or

have  a  B+128  or  Master  128, you will be able to load the BBC

KERMIT sideways ROM version to sideways RAM and run it there,  as

the program is completely unprotected.

If  you  fit  the BBC KERMIT ROM as the highest priority language

ROM in the machine, you will find that the  KERMIT  command  mode

screen  will appear when you power the machine on. Otherwise, you

will need to give an operating system command  to  enter  KERMIT:

7

BBC KERMIT User Guide

simply type

*KERMIT

and the KERMIT command mode screen will appear.

The command you give can be abbreviated if you wish, but you must

follow  the  abbreviation  with  a  ".". The minimum abbreviation

possible is "*KER.", so you could type any one of

*KER.

*KERM.

*KERMI.

or *KERMIT

if you wished.

8

BBC KERMIT User Guide

3.2 LEAVING BBC KERMIT

To leave either the RAM or sideways ROM versions of BBC KERMIT  you

would  normally use the EXIT command. This will reset the parts of the

system that KERMIT has been using, and will enter BASIC.

Alternatively, you can issue an operating system  command  such  as

*WORDWISE if you wish to enter another language ROM. Here, though, you

may  need  to  press  the BREAK  key  to  reset  the screen if the new

language ROM does not do this itself.

Note that the RAM version of  BBC  KERMIT  adjusts  OSHWM  so  that

commands  such  as *COMPACT will not corrupt the KERMIT program. OSHWM

is reset  when  you  use  the  EXIT  command  or  press CONTROL-BREAK;

however,  if  you leave KERMIT with, for example, *BASIC, OSHWM is not

reset, so you will find that you have  very  little  memory  left  for

programs. Pressing CONTROL-BREAK will correct this.

9

BBC KERMIT User Guide

3.3 BBC KERMIT COMMAND LANGUAGE

You  control  what  you want BBC KERMIT to do, and how it should do

it, by giving it commands in its "command language". The format of the

command language closely  follows  that  used  on  most  other  KERMIT

implementations on other machines.

There  are  some  diferences in what commands are available between

the sideways ROM version and the RAM  version  of  BBC  KERMIT.  These

differences will be noted in the following sections of this guide, and

are marked clearly in Appendix 1.

3.3.1. Command format

When  you start BBC KERMIT, you will see a MODE 7 screen known as

the "command screen" appear. This will be showing a  prompt  line

saying

BBC>

in  yellow, indicating that BBC KERMIT is expecting you to type a

command. You can type either a KERMIT command,  or  an  operating

system  command  preceded as usual by a "*". This allows you, for

instance, to look at a disc catalogue, or to  delete  files  from

within  KERMIT.  Like  operating system commands, KERMIT commands

can be typed in upper case, lower case, or any mixture of the two

as you please.

KERMIT's own commands are not preceded by an asterisk.  They  all

take  the  form  of  a  command  name,  such  as "SET", sometimes

followed  by  one  or  more  further  pieces  of  information  or

"parameters",  which must be separated from each other by spaces.

For example, one command you might use is

SET PARITY EVEN

to set one of the values used for communicating  with  the  other

system.  Here  the  command is "SET", and "PARITY" and "EVEN" are

parameters to the command. Since there  are  a  large  number  of

variations  to  the  SET  command, these are often referred to in

this guide as separate commands. Thus you will see references  to

the "SET FILE TYPE" command, for instance. You can regard this as

either a command whose name has spaces in it, or as a form of the

SET  command  with  its  first  parameters  "FILE" and "TYPE". In

either case, the effect will be the same.

At the end of each command you should press  the  RETURN  key  to

send  the  line  to  KERMIT for action. If you have made a typing

mistake you can use the cursor keys and the DELETE key exactly as

for other BBC micro languages. Pressing CONTROL  and  U  together

will cancel the entire input line.

10

BBC KERMIT User Guide

Appendix 1 gives the complete specification of all the BBC KERMIT

commands.  The  commands  are grouped according to their function

(such  as  "file  transfer  control"),   rather   than   in   one

alphabetical  list.  Thus  you  will  find  variations of the SET

command appearing in many places,  grouped  with  other  commands

that function in the same area.

Sometimes, not all the parameters of some commands need be typed.

In  these  cases,  BBC KERMIT will take a "default" value for the

parameter you did not supply. The reference section will tell you

which parameters can be omitted, and what values BBC KERMIT  will

assume if they are omitted.

3.3.2. Abbreviating commands and parameters

To  reduce  the  amount of typing that you need to do, BBC KERMIT

allows you to abbreviate all commands and  most  parameters.  You

may  abbreviate  to  as few characters as you wish, provided that

what you type is unique. Thus

TAKE

TAK

TA

are legal, but

S

is not, since it could stand for either SET,  SAVE  or  SHOW.  To

take a full example, you could type either

SET FILE TYPE BINARY

or abbreviate it as far as

SE FI T B

3.3.3. Numeric parameters

Except  where  explicitely stated, numeric parameters to commands

can be typed in either decimal or hexadecimal.  By  default,  BBC

KERMIT  expects  the  value to be decimal, but you can indicate a

hexadecimal number by preceding the parameter  with  either  "X",

"$" or "&".

Thus

11

BBC KERMIT User Guide

123

is a decimal value, and

&123

is a hexadecimal value.

As   an   example:  the  number  28  (decimal)  is  expressed  in

hexadecimal as 1C, so the two commands

SET SEND PADDING 28

and

SET SEND PADDING &1C

will have the same effect.

3.3.4. Obtaining help

Quite often, you may wish a reminder as  to  what  parameters  or

commands  you  can  give.  BBC KERMIT has an "auto-help" facility

that will act as an "aide-memoire" for you.

To use auto-help, simply type a "?" at any place in an input line

while typing a command. BBC KERMIT will immediately  give  you  a

list of the commands or parameters that you can use at that place

in the line, or will output a short message explaining the use of

that  parameter. It then retypes the command line up to the place

where you typed the question mark, so that you can type in one of

the options. You can, of course, change your mind and rub out all

or part of the line at this point, or press ESCAPE to  cancel  it

altogether.

For example, if you type a question mark after the prompt thus:

BBC> ?

BBC KERMIT will output a list of the commands you can use. If you

choose the LOGGING command, you might then type

BBC> LOGGING ?

BBC  KERMIT  will now tell you that you can type either ON or OFF

at this point, and will retype your line, so you will now see

BBC> LOGGING

with the cursor positioned after "LOGGING", ready for more input.

12

BBC KERMIT User Guide

If you type a question mark at a place where there  are  no  more

possible parameters, BBC KERMIT will output

No more parameters for this command

to indicate that here you can only either type RETURN to obey the

command, or change the line to something else.

The   examples   above  have  shown  the  "?"  character  as  the

"help-trigger" that invokes  auto-help.  There  may,  though,  be

times  when  you  require  to  type a "?" without it causing this

action - for example, as part of a remote  filename.  BBC  KERMIT

lets  you redefine the character that will trigger auto-help with

the SET HELP-TRIGGER command: you  can  set  it  to  any  of  the

characters "?", "&", "/", "#", or "@".

Automatic  line retyping will also occur if you make a mistake in

a command. BBC KERMIT will retype the line up to the point  where

it  found the error, so you will not need to type the entire line

again.

13

BBC KERMIT User Guide

3.4 READING COMMANDS FROM A FILE

As an alternative to typing commands in on the  keyboard,  you  can

place  them  in  a file (for example using *BUILD or a word processor)

known as a "TAKE file", and tell BBC KERMIT to read the commands  from

there  instead  of  from  the  keyboard.  This  is  done with the TAKE

command.

BBC KERMIT will read the file in  as  though  the  characters  were

coming  from the keyboard, and will obey its contents as commands. You

can include either KERMIT commands or operating system commands in the

file, the only exception being another  TAKE  command,  which  is  not

allowed.  Additionally,  you  can  place  comment lines in the file to

describe what it does: any line that starts with a colon (":") will be

ignored by KERMIT. Blank lines are also ignored.

The SET TAKE-ECHO command allows to you specify  whether  you  wish

KERMIT  to  display  the  lines  it reads from the file as it goes. By

default you will see the  commands  on  the  screen  before  they  are

obeyed.

If  an error occurs in a TAKE file, KERMIT will close it and revert

to reading  input  from  the  keyboard.  You  can  yourself  stop  the

execution of a TAKE file at any time by pressing the ESCAPE key.

The  most  common use of a TAKE file is to save you typing a number

of SET commands to configure BBC KERMIT every time you start  it:  the

job  of  configuration  can then be done with one TAKE command. (Note,

though, that the LOAD and SAVE commands provide an even better way  of

doing this).

14

BBC KERMIT User Guide

3.5 STORING PARAMETER SETTINGS

Once  you have changed a number of parameters with the SET command,

it is useful to be able to store  them  to  avoid  repitition  of  the

commands  at a later time. The SAVE command lets you record the values

of all the BBC KERMIT parameters in a disc file: subsequently the LOAD

command will read these values  back  and  make  them  operative.  You

could,  for  example,  keep  a number of sets of parameters in various

files to suit varying needs.

If you are running the sideways ROM version BBC KERMIT in  sideways

RAM  on  an  expansion board instead of in EPROM, you have the further

option of permanently changing the default settings. The  FIX  command

will  replace  the  default  values  stored within BBC KERMIT with the

values currently in use. This command is  not  available  in  the  RAM

version.

15

BBC KERMIT User Guide

3.6 SETTING THE COMMAND SCREEN WIDTH

The  normal  command  screen  used  by  BBC  KERMIT  uses  the  BBC

Computer's MODE 7, giving you a 40 character wide  screen  in  colour.

This  is  quite  adequate for operating BBC KERMIT normally, but there

are occasions, such as when you are using *TYPE to examine a file,  or

when  you  are  displaying  output from a remote KERMIT server program

(described in section 5.4.3), when it is convenient to have a line  of

80 characters.

If you have the sideways ROM version of BBC KERMIT on a model B, or

are  using  a  B+,  B+128  or  Master 128 with either version, you may

change the width of the command screen to suit  your  needs  with  the

WIDTH command. Typing

WIDTH 80

will change the command screen into an 80 character screen, and

WIDTH 40

will return to a 40 character screen.

The characteristics of the 80 byte command screen are controlled by

the  same  commands as for the 80 byte terminal emulation screen: thus

the commands

SET VDU-COLOUR

and

SET TV

control the text colour and whether a  *TV  command  is  automatically

issued.

Remember, though, that the 80 character command screen will use the

BBC Computer's main memory down to address &4000 on all models, so you

should  not  use it if you have prepared data for transfer from memory

above that address. The status screen used to display the progress  of

file  transfers will always be in MODE 7, regardless of the use of the

WIDTH command.

16

BBC KERMIT User Guide

3.7 FUNCTION AND CURSOR KEYS

In command mode, the 10 red  function  keys  on  the  BBC  Computer

keyboard  behave as they do in BASIC. You can program strings onto the

keys using the *KEY command as described  in  the  BBC  Computer  User

Guide, and pressing the keys will produce the strings. The four cursor

keys  also  have their normal function, and you can use them, the COPY

key and the DELETE key to edit your command lines as you  would  lines

of BASIC.

17

BBC KERMIT User Guide

3.8 USING AN AUTOBOOT FILE

One  way  of  starting  BBC KERMIT up and automatically setting the

parameters to suitable values is to create a !BOOT file (using  *BUILD

or a word processor, for example) on a disc containing the appropriate

commands.  You  should  consult your Disc Filing System user guide for

details of how to set up the system so you can use autoboot.

For example, you might create the file to hold

*KERMIT

SET BAUD 1200 RECEIVE

SET BAUD 75 TRANSMIT

Then, when you press SHIFT-BREAK, the commands in the file will  enter

BBC  KERMIT,  then  set  the baud rates up before returning control to

you.

18

BBC KERMIT User Guide

4: USING BBC KERMIT AS A TERMINAL EMULATOR

BBC KERMIT includes facilities that enable  you  to  use  your  BBC

computer  as  a  terminal  connected  to  a  mainframe  computer.  The

facilities are not as sophisticated as those offered by  a  ROM  whose

sole purpose is terminal emulation, since the prime use of KERMIT as a

terminal is to allow you to start running a remote KERMIT program on a

mainframe, but it is still quite usable.

If  you  are  using the RAM version of BBC KERMIT on a model B, the

only type of terminal emulation provided is as  a  40  character  wide

teletype device.

If  you  are  using  the  sideways ROM version, or have a model B+,

B+128 or Master 128 with either version, you may choose from  3  types

of  terminal  emulations.  By  default,  BBC  KERMIT  provides  an  80

character wide terminal compatible with the DEC VT52 standard. If  you

wish, you may select an 80 character wide simple teletype device, or a

40  character  wide teletype. Normally, you would need the latter mode

only if you wanted to transfer a large amount of data to or  from  the

BBC's  memory:  the 40 character wide terminal emulation uses BBC mode

7, so that memory below &7C00 is available to you.  The  80  character

wide teletype and VT52 emulations use memory down to &3D00.

19

BBC KERMIT User Guide

4.1 RUNNING A TERMINAL SESSION

In  this  section  we  shall  look  at how to set up BBC KERMIT for

terminal emulation and how to control the emulation functions.

4.1.1 Choosing the terminal emulation required

If you are using the RAM version of BBC KERMIT on a model B,  you

should  ignore  this  section, since only a 40 character teletype

emulation is provided. Although you are allowed to  select  other

emulation types, you will not be able to use them.

If  you  have  the  sideways ROM version of BBC KERMIT, or have a

model B+, B+128 or  Master  128,  you  should  first  decide  the

emulation mode you wish to choose. The command you will need here

is

SET TERMINAL

to choose the emulation you require.

For example, the command

SET TERMINAL TT80

will  turn  BBC  KERMIT  into  a  simple teletype device using an

80-column screen.

4.1.2 Setting the line speed

For both the RAM and sideways ROM versions of BBC KERMIT you  may

need to change the speed at which characters are transmitted down

the communications line. The

SET BAUD

command  accomplishes  this.  If  you have the wrong setting, you

will find either gibberish or nothing at  all  appearing  on  the

screen.  The  default  setting that BBC KERMIT uses is a speed of

9600 baud for both transmitting and receiving data.  If  you  are

unsure  of  what  setting to use, consult the Advisory or Systems

Support personnel of  your  computer  centre.  For  example,  the

command

SET BAUD 1200

would  set  up BBC KERMIT to use a baud rate of 1200 for transmit

and receive.

20

BBC KERMIT User Guide

Some  communications  equipment  may   require   that   data   is

transmitted  and  received  at different baud rates: for example,

some modems may enable the host to send data to you at 1200 baud,

but require you to send to the host at 75 baud. BBC KERMIT allows

you to select such a "split  baud  rate"  mode  by  specifying  a

further  parameter  to the SET BAUD command. In the example here,

of 1200 baud receive and 75 baud transmit,  you  should  set  the

baud rates with the two commands

SET BAUD 1200 RECEIVE

and

SET BAUD 75 TRANSMIT

4.1.3 Setting parity

For  both the sideways ROM and RAM versions of BBC KERMIT you may

need  to  change  the  "parity"  value  which  is  used  on   the

communications  line.  The parity setting determines what is done

with the eighth bit  of  every  data  byte:  some  communications

systems or mainframes may insist that this byte be always set, or

always  clear,  or should reflect the contents of the other seven

bits in some way.

You change the parity setting with the

SET PARITY

command. As with line speed, if you use  the  wrong  setting  you

will  probably  see  garbage  appearing  on  the screen, and file

transfers will not work. As an example, the command

SET PARITY NONE

would tell BBC KERMIT to use a parity value  of  NONE  (sometimes

expressed as "no parity"). You should consult the systems support

personnel  at your site if you are unsure of the correct setting.

Parity also affects how KERMIT decides to transfer some types  of

file, and you should consult section 5.3 for more information.

By default, BBC KERMIT uses a parity setting of SPACE.

4.1.4 Selecting the flow control method

When  two computers transfer data between each other, some method

21

BBC KERMIT User Guide

is needed to control the data flow,  to  stop  one  side  sending

faster  than  the  other  can receive and process that data. This

process is termed "flow control", and gives the machine  that  is

receiving the data the ability to tell the other side "don't send

any  more  data  till I tell you, as I have a backlog of bytes to

display".

BBC KERMIT supports  the  two  most  common  techniques  of  flow

control, and you can switch at will between them. The command

SET FLOW-CONTROL XON/XOFF

selects  "XON/XOFF"  flow  control,  whereby  "stop"  and "go" is

signalled by sending  special  byte  values.  Alternatively,  the

command

SET FLOW-CONTROL CTS/RTS

will  select  "CTS/RTS"  or "hardware" flow control, where "stop"

and "go" is signalled by changing voltages on the line connecting

the systems.

The  setting  you  should  use  will  depend  entirely   on   the

communications  system or mainframe you are using, and you should

consult the systems support personnel at your site for advice. If

you have the wrong setting you will probably find parts of output

lines missing when you use terminal mode.

By default, BBC KERMIT will use XON/XOFF flow control.

4.1.5 Specifying an "ignore" character

Some mainframe systems assume that  all  terminals  connected  to

them  are  produced  by  the  same  manufacturer,  and  will send

"filler" characters to them, assuming that the terminal will know

that they are not to be displayed on the screen. BBC  KERMIT,  of

course,  is  unaware  that  this  is  being done, so will take no

special action: all characters with ASCII codes greater than  31,

and  the  CR, LF, DEL, BS and FF characters will be output to the

screen. (TAB characters are expanded into spaces  and  ESCAPE  is

handled specially).

If  the  filler  character  the  mainframe  sends  is one that is

normally displayed by BBC KERMIT, you can suppress its output  by

nominating it as the "ignore character". For example,

SET IGNORE &7F

would cause DEL characters (ASCII code &7F) to be ignored. Typing

SET IGNORE

22

BBC KERMIT User Guide

on its own would switch off the ignore facility once more.

4.1.6 Starting terminal emulation

To start BBC KERMIT as a terminal emulator, you should set up the

speed,  parity,  flow  control  and  "ignore" character values as

described above, then type the command

CONNECT

This sets up the screen in the required mode and clears  it.  Any

characters  you  type  now  are sent directly to the RS423 output

port, exactly as they would be if the BBC were a  real  terminal.

You  may  now  "wake  up" the system you are connected to, and do

anything else you wish.

Note that if you have chosen a terminal emulation that BBC KERMIT

cannot provide (for instance, if you have selected VT52 mode with

the RAM version on a  model  B),  the  CONNECT  command  will  be

rejected  and  the error message "Terminal not supported" will be

displayed.

In all terminal emulation modes BBC KERMIT will display a "status

line" at the top  of  the  screen.  The  line  contains  a  brief

reminder  of the functions you can select by pressing the CONTROL

key with  a  function  key  and  indicates  what  facilities  are

currently operating.

4.1.7 Sending a break signal

Many  mainframe  computer  or  communications systems require the

terminal to send them a "break" signal for some operations.

BBC KERMIT allows you to  send  either  of  two  types  of  break

signal. Pressing CONTROL and F9 will send a "short break" signal:

this  is  the  most commonly-used type, and the break signal will

last about 240 milliseconds. In some circumstances a "long break"

signal may be needed: pressing CONTROL and F8 will send a  signal

lasting  3.5  seconds.  Your  systems support or advisory service

will be able to tell you which, if any, break signal you need  to

use.

Note  that  you should not press the key marked BREAK for sending

break signals.

23

BBC KERMIT User Guide

4.1.8 Using the function keys

In terminal emulation mode BBC KERMIT reserves  the  10  function

keys  for  its  own  use:  in VT52 emulation mode it uses them to

emulate the "keypad" keys of the  real  VT52.  However,  you  can

still  use the function keys to produce strings, but now you will

need to press the CONTROL and SHIFT keys at the same time as  the

function key concerned.

As an example, suppose that in BBC KERMIT command mode (or indeed

before  you  started  to run BBC KERMIT) you issued the Operating

System command

*KEY 0 type myfile.dat

Then, in terminal emulation mode, pressing CONTROL-SHIFT-F0 would

produce the string "type myfile.dat" on the screen exactly as  if

you had typed it from the keyboard.

4.1.9 Using the cursor keys

BBC  KERMIT  uses  the  four  cursor  keys  in  one  of two ways,

depending on what terminal emulation you have selected.

If you have selected either a 40 character  or  an  80  character

teletype emulation, the keys function as normal BBC Computer edit

keys.  You  can use them in conjunction with the COPY key to edit

lines, in exactly the same way as you would edit a BASIC program.

In VT52 emulation mode, though, the cursor keys will  by  default

produce  VT52  ESCAPE sequences that programs such as the VAX/VMS

EDT editor expect,  and  these  are  described  below.  You  can,

though,  specify  that  they  should  be used as BBC edit keys by

pressing CONTROL and F4. An "E" will appear in the status line to

remind you, and the keys will now operate as BBC edit  keys.  Any

program  you  are running on the mainframe will not be aware that

you have pressed a cursor  key.  To  return  to  using  VT52-mode

cursor keys, press CONTROL and F4 once more.

4.1.10 Pausing screen output

You have several choices if you wish to stop output to the screen

for  a  moment (for example to study part of a file that is being

listed). One method uses the standard "stop scroll" mechanism  of

the  BBC  Computer:  output will stop if you hold the CONTROL and

24

BBC KERMIT User Guide

SHIFT keys down together, and will resume when you release them.

If you wish to hold the  screen  while  you  do  something  else,

though,  a better method is to use BBC KERMIT's own mechanism. If

you press the CONTROL and F3 keys once, output to the screen will

stop. An "H" will be displayed on the  status  line  to  indicate

that  you  have suspended output. The screen will remain as it is

until you again press CONTROL and F3: unlike the first technique,

this lets you move away from the machine if you wish.

4.1.11 Returning to command mode

At some point, particularly if you  are  going  to  perform  file

transfers,  you  will  wish  to leave terminal emulation mode and

return to the command screen, since  the  commands  that  perform

file  transfer  can  be  issued only from KERMIT command mode. To

return to command mode you should press CONTROL and F0  together:

the BBC KERMIT command screen will reappear and you may issue any

BBC  KERMIT  commands  you wish, and return to terminal emulation

mode when you require.

25

BBC KERMIT User Guide

4.2 LOGGING OUTPUT TO A DISC FILE

You may wish to record your terminal session in a file on disc  for

later use: this is a convenient way of down-loading a text file from a

mainframe  if  it does not have a KERMIT facility of its own (however,

it is strongly recommended that you do use KERMIT  for  file  transfer

whenever possible).

Terminal  logging  must  be  initiated  from the BBC KERMIT command

screen before  you  issue  the  CONNECT  command  to  start   terminal

emulation. You could type, for example

LOGGING ON MYLOG

which  would  initiate logging, and open a file called MYLOG to record

the data. At this point, though, although logging is initiated, it  is

not active, so nothing will be sent to the file.

Once  you  have entered terminal emulation mode you can activate or

deactivate logging at will. Pressing CONTROL and F1 once will activate

logging: any character that appears on the screen will  be  copied  to

the  file.  Pressing CONTROL and F1 again will deactivate logging, and

no characters will go the file. You may  repeat  this  activation  and

de-activation  as  often  as  you wish. A "D" will appear at the right

hand side of the status line whenever logging is active, to remind you

that output is being sent to disc.

When you have finished your terminal session, you should return  to

the command screen by pressing CONTROL and F0, then issue the

LOGGING OFF

command  to  terminate  logging  and  close  the  file.  This  step is

important: if you don't do it some data will be lost from the file.

A possible sequence you could use to capture a  text  file  from  a

mainframe would be this:

1. Issue the command, for example,

LOGGING ON MYFILE

to open a file and initiate logging.

2. Enter terminal emulation mode with the command

CONNECT

3. Log in to the mainframe, and type the command to list a file

to  the terminal, but don't press RETURN to send the command

to the mainframe.

4. Activate logging to the disc file by  pressing  CONTROL  and

F1.

26

BBC KERMIT User Guide

5. Press RETURN to send the listing command to the mainframe.

6. When  the  file  has  been  displayed  on  the screen, press

CONTROL and F1 again to deactivate logging.

7. Return to the BBC KERMIT command screen by pressing  CONTROL

and F0.

8. Terminate logging and close the file with the command

LOGGING OFF

27

BBC KERMIT User Guide

4.3 LOGGING OUTPUT TO A PRINTER

BBC KERMIT allows you to copy all the characters that appear on the

screen to a printer, to give a hard copy log of a terminal session. At

any  time  while  you  are  in  terminal mode, pressing CONTROL and F2

together will turn on the copying of bytes to  the  printer;  pressing

CONTROL  and F2 once more will turn it off again. A "P" will appear on

the right of the status line whenever bytes are being  logged  to  the

printer.

An  alternative,  and  preferable way of obtaining a hard copy of a

file on your printer is to use the file  transfer  facilities  of  BBC

KERMIT  to  move  the  file  from  the  mainframe  to your BBC system,

specifying that the data be sent directly to the printer with the

SET DESTINATION PRINTER

command. This method, although slower, has the advantages of providing

an error-free copy of the file, and enabling  all  possible  character

values to be sent. The technique is further described in Chapter 5.

28

BBC KERMIT User Guide

4.4 SENDING A FILE TO A HOST WITHOUT KERMIT

BBC  KERMIT  contains facilities to let you transfer files from BBC

discs to another machine even if that machine does not have  a  KERMIT

program  of  its  own.  This  facility, sometimes called "raw send" is

controlled by the TRANSMIT command. You should note, though, that  ths

method  offers  no  error-correction  at all, so you should always use

KERMIT to transfer data if you can.

For example, suppose you wish to transfer the contents  of  a  file

:1.RESULTS  to a mainframe that does not have KERMIT. You can take the

following steps:

1. Use the CONNECT command to enter terminal mode, and  log  in

to the host.

2. Type  the  host  command  needed to store what you type in a

file. Your system may have an INPUT or  CREATE  command,  or

you  may  need  to  use  the editor to do this: if in doubt,

consult your systems support personnel or advisory service.

3. Press CONTROL and F0 to return to BBC KERMIT command mode.

4. Issue the TRANSMIT command. In this example, you would type

TRANSMIT :1.RESULTS

BBC KERMIT  will  now  return  to  terminal  mode,  and  the

contents  of  file :1.RESULTS will be sent to the host, just

as if you were typing at  the  keyboard.  In  order  not  to

overload  the  host  system with data arriving at too high a

rate, BBC KERMIT will pause for half-a-second after  sending

each line of the file.

5. Once the file has been sent, type whatever is needed to tell

the host that there is no more data to go into the file, and

carry on with your terminal session.

Note that you can press ESCAPE at any time to terminate sending the

file.  BBC KERMIT will once more take input from the keyboard, exactly

as normal.

It may sometimes happen that  your  file  contains  line-feed  (LF)

characters   at   the   end   of  each  line  as  well  as  the  usual

carriage-return  (CR)  characters,  and  these  can  sometimes   cause

problems  at  the  host  end. Accordingly, BBC KERMIT lets you control

what happens to them, as follows:

If the file type you have set with the SET FILE TYPE  command  is

ASCII  CR,  line-feed  characters in the file are ignored and not

sent to the host.

If the file type is anything else, the data in the file  is  sent

exactly as it is, without any changes.

29

BBC KERMIT User Guide

4.5 VT52 KEYPAD EMULATION

Since  the  model  B,  B+ and B+128 keyboards do not possess a VT52

keypad, it is necessary to emulate these keys using  the  10  function

keys.  In  VT52  terminal  emulation mode the function and cursor keys

correspond to the VT52 keypad in the following way:

BBC key      VT52 keypad key  ESCAPE sequence

F0                  0             ESC ? p

F1                  1             ESC ? q

F2                  2             ESC ? r

F3                  3             ESC ? s

F4                  4             ESC ? t

F5                  5             ESC ? u

F6                  6             ESC ? v

F7                  7             ESC ? w

F8                  8             ESC ? x

F9                  9             ESC ? y

SHIFT-F1            PF1           ESC P

SHIFT-F2            PF2           ESC Q

SHIFT-F3            PF3           ESC R

SHIFT-F4            PF4           ESC S

SHIFT-F5            MINUS         ESC ? m

SHIFT-F6            COMMA         ESC ? l

SHIFT-F7            PERIOD        ESC ? n

SHIFT-F8            ENTER         ESC ? M

UP                  UP            ESC A

DOWN                DOWN          ESC B

RIGHT               RIGHT         ESC C

LEFT                LEFT          ESC D

Note that keys PF4, MINUS and COMMA may not exist on  a  real  VT52

device,  although they are present and usable on a VT100 terminal that

is operating in VT52 mode.

The Master 128 allows you to use the above keys as  described,  but

also  lets  you use the built-in keypad, which is considerably easier.

The arrangement of the keys correspond to a real VT52 keypad  closely,

and they have the following meaning:

30

BBC KERMIT User Guide

---------------------------------

!       !       !       !       !

!  PF1  !  PF2  !  PF3  !  PF4  !

!       !       !       !       !

!       !       !       !       !

---------------------------------

!       !       !       !       !

!   7   !   8   !   9   ! MINUS !

!       !       !       !       !

!       !       !       !       !

---------------------------------

!       !       !       !       !

!   4   !   5   !   6   !       !

!       !       !       !       !

!       !       !       !       !

---------------------------------

!       !       !       !       !

!   1   !   2   !   3   ! COMMA !

!       !       !       !       !

!       !       !       !       !

---------------------------------

!       !       !               !

!   0   ! PERIOD!    ENTER      !

!       !       !               !

!       !       !               !

---------------------------------

Note that the keypad DELETE key is not used.

Appendix 3 details the use of the function keys with the EDT editor

available on the VAX/VMS Operating System.

31

BBC KERMIT User Guide

5: TRANSFERRING FILES WITH BBC KERMIT

The primary use of BBC KERMIT is to transfer files between it and a

mainframe  computer,  or  between  it  and  another microcomputer. The

methods used will be substantially the same whatever the other  system

is,  since  any  KERMIT  system should be able to communicate with any

other. Though the general techniques  will  be  the  same,  the  exact

commands  used  to control the remote KERMIT will vary from one system

to another. You will need to consult the user  guide  for  the  remote

system  to  discover  how  it should be controlled. In this section we

shall cover in detail how BBC KERMIT is controlled.

32

BBC KERMIT User Guide

5.1 PRINCIPLES

KERMIT differs from other "dumb" file transfer systems (such as you

might find in a terminal emulator ROM, for instance) in that  it  aims

to  transfer files not only reliably, but also in a usable way between

systems. Thus, if you have a program source on your BBC discs that you

can edit with some BBC micro editor, and transfer this to a mainframe,

the resulting file should also be editable on the mainframe's editors.

KERMIT will resolve all the differences in how  files  are  stored  on

your  behalf,  so  that,  in this example, the mainframe file will not

contain extra line-feed characters at  inconvenient  places  that  the

mainframe editors cannot handle.

Transferring  files with KERMIT involves several discrete steps. We

shall consider here the most common case of transfer  to  and  from  a

mainframe and look at the several steps involved in a general way.

1. You  start  BBC  KERMIT  is and set it up for the transfer. In

particular, you may wish to tell it what types of file are  to

be  moved.  You  will  also  need  to  set  the parameters for

terminal  emulation,  and,  depending  on  the  needs  of  the

communications  system  and the remote KERMIT, you may need to

change some of the  more  detailed  aspects  of  BBC  KERMIT's

operation (though this is unlikely).

2. You  enter  terminal emulation mode, and log the BBC system in

to the mainframe as though it were an ordinary terminal.

3. Using the BBC system as a terminal, you start the  mainframe's

own KERMIT program running.

4. You  can  now  give  commands  to  the  mainframe KERMIT (from

terminal emulation mode) and to the BBC  KERMIT  (from  KERMIT

command  mode)  to transfer your files. The two KERMIT systems

will communicate with each other  using  the  standard  KERMIT

protocol.

5. After  the  transfers are done, you can log the BBC system out

from the mainframe.

In practice, the steps taken will range up and down  this  list  as

required.  For  example,  BBC  KERMIT parameters can be changed at any

time, not only at the start, and if you are  moving  several  tyes  of

file you will need to change them frequently. Also, things may be made

much simpler if the mainframe KERMIT provides what is known as "server

mode  operation" - we shall discuss this later. The following sections

will discuss the various aspects of file transfer in more detail.

33

BBC KERMIT User Guide

5.2 FILE TYPE

The most important thing  that  you  will  need  to  consider  when

transferring  files  using  BBC  KERMIT  is the file type of the files

involved. As we saw in the overview of KERMIT in chapter 2, one of the

facilities that KERMIT provides is that files  are  transferred  in  a

usable  way, with the method the sending system uses to denote the end

of a line being automatically translated into the method the receiving

end uses. Many operating systems (for example MS-DOS)  don't  care  at

all  how  you  denote  the  end of a printable line, and will raise no

objections whether your file  uses  Carriage-Return  bytes,  Line-Feed

bytes  or  anything  else.  Mainframes,  though, are usually much more

choosy, and can be quite perverse. Some mainframe systems store actual

byte sequences (such as an actual Carriage-Return byte) in the file to

mark the end of a line. Others may assume a line is finished when some

byte such as Carriage-Return is input, but don't store the actual byte

in the file, using instead some method  of  recording  the  number  of

characters  in the line. And even worse, some mainframe systems expect

you to decide what they should do - this can be  particularly  awkward

if  you  are  transferring binary  files that don't contain printable,

separate records anyway.

The way the KERMIT system  in  general  handles  this  is  that all

KERMITs, when sending a file, translate their own system's end-of-file

indication  into  a standard  form.  The KERMIT that receives the file

then knows exactly where each record ends,  and  translates  the  data

into whatever format its system needs.

Now  this  would  be perfectly simple and easy, except for the fact

that on the BBC Computer there is no uniform way of denoting  the  end

of a record. Each program or ROM you may use can use whatever bytes it

likes  to  denote  the  end  of  a  record, and these are likely to be

different.  Thus   a   file   you   produce   with   *SPOOL   contains

Line-Feed/Carriage-Return  pairs  at  the end of a line, but a spooled

Wordwise file contains only Carriage-Returns. There is no way  at  all

for BBC KERMIT to know what bytes are used, so it is up to you to tell

it by setting a "file type".

How  end-of-record  is  denoted is also of importance when you move

files to BBC KERMIT from another system. Here you will want BBC KERMIT

to  translate  the  KERMIT  standard  end-of-record   indicator   into

something  suitable  for  whatever  program  or  ROM  you are going to

process the file with.

In the sections below we shall look at the possible file types  you

can  select, then examine how you can work out what the type should be

for a particular file.

5.2.1 Binary files

These files contain data that is not  primarily  printable  text,

34

BBC KERMIT User Guide

such as SAVEd BASIC programs, machine-code programs, screen dumps

or  *SAVEd  areas  of  memory. When you transfer these files, you

wish every byte in the file on one system to appear unchanged  in

the file on the other system, regardless of what it is.

You  tell  BBC KERMIT that you are handling binary files with the

command

SET FILE TYPE BINARY

which tells it not to change any data that  it  either  sends  or

receives.  You  may  need  to  issue  a comparable command to the

remote KERMIT, to  prevent  it  trying  to  manipulate  the  data

itself. Some remote KERMITs may not allow you to send pure binary

data,  though,  and  you will not be able to send binary files to

them in this case.

5.2.2 Printable text (ASCII) files

These files contain printable text, such  as  you  might  produce

with  the  *BUILD command, or by spooling a BASIC program listing

to disc. When you transfer one of these files, you will want  the

two  KERMITs  to  translate  the  way end-of-record is indicated,

rather than transfer every byte exactly as it is.

You tell BBC KERMIT that ASCII text files are to  be  transferred

with the

SET FILE TYPE ASCII

command.  This  specifies that the files are text, but BBC KERMIT

will also need to know how the end of a printed line is indicated

in this file.  The  command  shown  above  sets  the  end-of-line

indication    to   the   default   of   Line-Feed   followed   by

Carriage-Return (LFCR), but this may not be what you require. You

can tell BBC KERMIT your needs with a further parameter, which is

one of the strings LFCR, CRLF, LF or CR.

Thus, for example,

SET FILE TYPE ASCII CR

tells  BBC  KERMIT  that  files  are  text,  and  end-of-line  is

indicated  by  a  Carriage-Return  byte on its own. When you then

send a file, every CR byte is translated into the internal KERMIT

represenation of end-of-line. On receiving a file,  the  internal

KERMIT form will be translated into a CR for you.

35

BBC KERMIT User Guide

5.2.3 How to decide on the file type to use

There  is, unfortunately, no easy way of telling how the end of a

printed line is denoted in a file. However, you can  inspect  the

file with the *DUMP command with, for instance

*DUMP FILE

This (on the Acorn DFS, at any rate) will print a file in hex and

character  form  on  the screen. You can then locate the end of a

line of text and see what characters follow in the  hex  area  of

the  dump:  a CR byte appears as 0D, and an LF will appear as 0A.

In this example, if your file contained only the lines

ABCDE

FGH

IJKL

you might see the dump looking like this

0000: 41 42 43 44 45 0D 46 47  ABCDE.FG

0008: 48 0D 49 4A 4B 4C 0D **  H.IJKL..

which would indicate that the file starts with  a  line  "ABCDE",

terminated  by one CR byte, then has a line "FGH" terminated by a

CR byte, then a line "IJKL" terminated with a CR byte. You  would

then need

SET FILE TYPE ASCII CR

to transfer such a file.

The sequences you should look for in the dump are then:

0D      CR

0A      LF

0D 0A   CRLF

0A 0D   LFCR

If  you  ever  encounter  a  sequence  such as 0D 0D (CR CR) this

indicates that one line terminates with a CR, and the next  line,

also terminating with a CR, is blank.

Some  of the common files you will come across have the following

file type:

SAVEd BASIC               BINARY

Spooled BASIC listings    ASCII LFCR

Saved WORDWISE files      BINARY

Spooled WORDWISE text     ASCII CR

*BUILD files              ASCII CR

36

BBC KERMIT User Guide

ADE editor files          ASCII LFCR

*SAVEd files              BINARY

Machine code programs     BINARY

37

BBC KERMIT User Guide

5.3 SENDING EIGHT BIT DATA

As we saw in chapter 2, characters are stored in a file in "bytes",

and each byte consists of 8 separate binary digits or bits. Each  byte

can  contain a number between 0 and 255, and so there are 256 possible

characters that you can write into a file. Unfortunately, it is common

for communications systems to let through only 7 bits from each  byte,

using  the  eighth  bit  for their own purposes. Thus you can normally

send only characters whose ASCII codes are in the range 0 to 127. Many

text files on the BBC, though, and  every  binary  file  will  contain

bytes from the whole character set, with codes from 0 to 255, so there

is a potential problem in transferring such data correctly.

KERMIT  in general has a technique for overcoming this restriction,

by encoding characters in the range 128 to 255 into special  sequences

that can  be  sent  down  any  communications  line. Almost all modern

KERMITs will use  this  technique,  which  is  known  as  "eighth  bit

prefixing",  but  you  may  encounter  an older implementation on some

machine that does not support it. In  this  case  your  data  will  be

garbled  in  transmission.  There  is,  regrettably, no way round this

problem from within KERMIT, but BBC KERMIT  will  warn  you  when  the

problem  is detected: a message "WARNING : Non ASCII data encountered"

will appear on the display screen.

In order that the amount of data sent down the  communication  line

is  not  unnecessarily  large, there are some rules governing when BBC

KERMIT and the remote KERMIT will perform eighth-bit-prefixing,  since

the  technique increases the amount of data that must be sent whenever

characters that use the eighth bit are encountered.

1. If the remote system has been set up with its  own  commands

to  ask for eighth-bit-prefixing, BBC KERMIT will always use

it.

2. If you have used the SET PARITY command to  select  a  value

that  implies  that  only  7  data bits can be sent down the

communication line (i.e. if you have set the value to SPACE,

EVEN, ODD or MARK) then  BBC  KERMIT  will  attempt  to  use

eighth-bit-prefixing. Whenever you send a file from the BBC,

BBC   KERMIT   will   request   the  remote  system  to  use

eighth-bit-prefixing; whenever the remote sends  a  file  to

the BBC, BBC KERMIT will use eighth-bit-prefixing unless the

remote KERMIT has said that it does not implement it.

The status screen will tell you whether or not eighth-bit-prefixing

is  in  use  during  a  transfer  so that you will know when you might

expect problems.

38

BBC KERMIT User Guide

5.4 STARTING UP THE MAINFRAME KERMIT

In order to run the KERMIT program on  the  mainframe  system,  you

will  need  to  log  your  BBC  computer  in  as a terminal, using BBC

KERMIT's terminal emulation facilities. This is described in detail in

Chapter 4.

Once you have logged in,  you  can  start  the  mainframe's  KERMIT

program.  How this is done is of course dependent on the other system,

but the command is probably "KERMIT" or something similar. You  should

consult  the  user guide for the mainframe system to find exactly what

to do.

The mainframe KERMIT will certainly be able to operate as a  normal

KERMIT  (termed non-server  mode). In this mode, you will need to give

commands both to it and to BBC KERMIT for every file transfer (here  a

transfer of a group of files in one go counts as one operation), which

will  involve you in continual changes between BBC KERMIT command mode

and terminal mode. Alternatively, more sophisticated mainframe KERMITs

may operate in "server" mode.  Here  you  issue  one  command  to  the

mainframe  KERMIT  (usually  "SERVER"  or something similar) to put it

into server mode. Now you can control all operations from  BBC  KERMIT

command  mode:  you do not need to continually switch to terminal mode

to  give  commands  to  the  mainframe  KERMIT.  You  can  start  file

transfers, and even, with some mainframe KERMITs, manipulate files and

obtain  directory  listings  from  the mainframe side simply by giving

commands to  BBC  KERMIT.  In  general,  you  should  always  set  the

mainframe KERMIT into server mode if this is possible.

The  exact  way  in which you transfer files will depend on whether

you are talking to a server or a non-server,  and  we  shall  consider

each in turn.

39

BBC KERMIT User Guide

5.5 USING BBC KERMIT WITH A REMOTE SERVER

As  we have seen, you should put the remote KERMIT into server mode

with a command such as "SERVER". You will then probably see a  message

telling  you something like "use your micro's local escape sequence to

return to command mode": the exact text (and  whether  it  appears  at

all) depends on the remote system in use. You should press CONTROL and

F0  together,  and BBC KERMIT will enter command mode, showing you the

command mode screen.

You can now control the whole operation  from  BBC  KERMIT  command

mode.

5.5.1. Sending files to a server

To  send  a file to a server you simply use the command SEND. The

reference  section  describes  the  use  of  the   command,   its

side-effects and facilities, in detail, and you should consult it

for further information.

The  basic  use of the command is simple. To send, for example, a

file called BEEBLE, you would type

SEND BEEBLE

You will now see the file  transfer  status  screen  appear,  and

information  about  the transfer will be displayed. Normally, BBC

KERMIT will pause for 30 seconds here - this is because it has to

cater for the requirements  of  a  non-server  remote  system  as

described  below.  You can either wait the full time, after which

the transfer will start, or you can press any key  to  force  the

transfer to begin at once. You can alter this pause period if you

wish  with  the  SET  DELAY  command  -  this is described in the

reference section.

In this example, BBC KERMIT will ask the remote  system  to  call

the  file  it  receives  BEEBLE.BBC  (although  the remote system

doesn't have to if it doesn't want to or if that  isn't  a  legal

filename  for  it  to  use).  If you wanted the file on the other

system to be called something  else,  you  can  use  a  different

format of the SEND command, putting

SEND BEEBLE /44d/frogspawn/pudding

Here  BBC  KERMIT will ask the remote system to store the data in

"/44d/frogspawn/pudding". You  can  put  what  you  like  in  the

filename,  providing  that  it does not contain spaces and is not

more than 48 characters long.

Alternatively, you can make BBC KERMIT  add  a  different  suffix

from ".BBC" when it is forming the remote file name. The command

40

BBC KERMIT User Guide

SET FILE SUFFIX TOAD

would  cause  BBC KERMIT to add ".TOAD" to the name, for example.

The command also allows you to  stop  any  suffix  at  all  being

added:  you  should  consult  the  reference  section for further

details.

You can, if you wish, send a group of files in one  operation  by

specifying  "wildcard"  characters  in  the name. BBC KERMIT will

then send all the files whose names match, one  after  the  other

automatically.  Two different wildcard characters can be used, as

follows:

1. The "*" character can be used to match against any number of

characters in the names of the files in the directory. Thus,

if you specified a name

B*S

then files called BIRDS, BAGS, BLINKS, BUS and BS  would  be

transferred.

If  you specify a name simply as "*", then all files will be

transferred.

2. The "#" wildcard character can  be  used  to  match  against

exactly  one  character  in the real filenames. Thus, if you

specified a name

B#G

then files BUG and BIG would be transferred, but files  BANG

and BG would not be.

BBC  KERMIT  uses  the  facilities  of  the Disc Filing System to

locate files whose names match a wildcarded one, and depending on

its operation there may be restrictions on where you can make  it

search. If you using the Acorn DFS or DNFS, for example, only the

current directory is searched, so that although a wildcarded name

such as

DAT*

is acceptable, a name such as

:1.DAT*

cannot be used, and no matching files will be found for it.

41

BBC KERMIT User Guide

5.5.2. Fetching files from a server

Fetching files from a server is also simple. If the remote system

has a file called GROMMET.TXT, you can move it to the BBC with

GET GROMMET.TXT

The  file  transfer  status  screen will appear, and the transfer

will start immediately.

In this example, the data will be stored in a file called GROMMET

in the current directory and drive. If you want a different name,

you could put, for instance,

GET GROMMET.TXT :3.B.PLUG

and the data will be stored in file :3.B.PLUG.

In the first example, there is of course  a  risk  that  you  may

already have a file called GROMMET: this risk is increased if BBC

KERMIT has to change the file name a lot to make it acceptable to

the  Disc  Filing System. Accordingly, by default BBC KERMIT will

try to amend the name it uses if there  is  a  clash.  Characters

will  be  changed,  starting  from the right, into "&" characters

until a unique name is formed. You will  be  told  whenever  this

occurs.  If  you  wish, you can change this facility: issuing the

command

SET FILE WARNING OFF

will turn this check off, and BBC KERMIT will overwrite any  file

whose  name  is  the  same  as  the new file, losing its original

contents.

Note that BBC KERMIT will transform unacceptable  names  to  make

them  legal to the Acorn filing system standard: it will not take

advantage of filing systems that allow, for example,  file  names

longer than 7 characters.

You  can  fetch a group of files from the server in one operation

by including the remote system's wildcard character in the  name.

The remote system will then send each file in turn automatically.

5.5.3 Controlling a remote server

Many  of  the  more  modern mainframe KERMITs that support server

mode operation allow the micro KERMIT  to  perform  a  number  of

useful  operations  on  mainframe  files  through  them.  If  the

mainframe KERMIT you are using is one of these, you will be  able

42

BBC KERMIT User Guide

to  do things such as listing files and directories, deleting and

renaming files, all with commands issued  to BBC  KERMIT  without

the need to enter terminal emulation mode.

These  operations  are  all initiated with the BBC KERMIT command

REMOTE, which passes a request to the mainframe system  for  some

action.  The  command  is  always  followed by some parameters to

specify the action you are requesting, and we shall look  at  the

various  possibilities  below.  You should note, though, that not

all servers implement all the  facilities  described  here  (some

implement   none   at  all),  and  some  interpret  the  commands

differently from others. You should consult the  user  guide  for

the server in use to see the precise details.

Note here that some mainframes may take a long time to respond to

these  commands, so that you may need to turn timeouts off if you

have been using them.

You can interrupt any REMOTE command producing a large amount  of

output by pressing CONTROL and Z.

1.  Listing a directory

The command

REMOTE DIR

will  produce a directory listing from the server. The exact

layout and selection of names will  depend  on  the  server:

some will allow you to specify, for example

REMOTE DIR ABC*

to list all files with names starting "ABC".

2.  Changing the current directory

The command

REMOTE CWD

will request the server to change the default place in which

it looks for files. Typing the command on its own will reset

the  directory to whatever the default value is: if, though,

you typed something like

REMOTE CWD [.SUBDIR]

you would change the  directory  to  [.SUBDIR].  If  you  do

specify a name here, BBC KERMIT will ask you for a password:

if  your  mainframe  system requires one you should type it,

then press RETURN. The password will not be  echoed  on  the

43

BBC KERMIT User Guide

screen  as  you  type.  If  your  system  does not require a

password, simply press RETURN here.

3.  Displaying a file

The command

REMOTE TYPE

will display a file  owned  by  the  mainframe  on  the  BBC

screen. You could, for example, type

REMOTE TYPE MYLIST.LIS

to examine a file called MYLIST.LIS.

4.  Obtaining help

The command

REMOTE HELP

requests the server to display some help information to you.

You  can  either  type  the  command  by itself, or, on some

systems, you can specify a topic. Thus

REMOTE HELP

will normally provide you with a "first level" display, and

REMOTE HELP files

for example might produce some help on the specific topic of

"files".

5.  Displaying server status

Issuing the command

REMOTE STATUS

will produce  a  display  giving  details  of  the  server's

operation.

6.  Examining who is logged in

Issuing the command

44

BBC KERMIT User Guide

REMOTE WHO

will produce a list of all the users currently logged in.

7.  Copying a file

The command

REMOTE COPY

requests  the  server  to  copy a file on the mainframe. You

could type, for example

REMOTE COPY LUMBER.TXT DUTCHELM.DED

to copy file LUMBER.TXT to a new file called DUTCHELM.DED.

8.  Renaming a file

The command

REMOTE RENAME

requests the server to change the name  of  a  file  on  the

mainframe. You could type, for instance

REMOTE RENAME /wondrous/thing /old/hat

to change the name of /wondrous/thing to /old/hat.

9.  Deleting a file

The command

REMOTE DELETE

requests the server to delete a file on the mainframe. Thus,

typing

REMOTE DELETE DOCTOR.WHO

would delete the file DOCTOR.WHO.

10. Interrogating disc space usage

The command

REMOTE SPACE

45

BBC KERMIT User Guide

requests  the  server  to report on your current disc usage.

Some servers may allow you to specify a selection parameter:

thus you might be able to type

REMOTE SPACE //disc99

to see how much space you have available on a specific  disc

drive.

11. Issuing a host command

The command

REMOTE HOST

requests  the  server  to  issue  a  command  to the command

interpreter of  the  host.  The  command  to  be  issued  is

whatever  follows  REMOTE HOST on the line: thus if you were

to type

REMOTE HOST CREATE/DIR [.FRED]

when using a VAX/VMS server, the command

CREATE/DIR [.FRED]

would be issued to  the  command  interpreter  to  create  a

directory.

Note  that  care is needed in the choice of command: it must

be one that requires no input from the  terminal,  since  of

course, there is no terminal available to provide the input.

5.5.4. Closing down the server

Once you have finished moving data, you can close down the remote

KERMIT  server.  Again, you do this from BBC KERMIT command mode.

Typing the command

BYE

will tell the server to close down, and  your  terminal  will  be

automatically logged out.

Typing

FINISH

46

BBC KERMIT User Guide

will  tell  the  remote  KERMIT  to cease operating, but here the

terminal will still be logged in.

47

BBC KERMIT User Guide

5.6 USING BBC KERMIT WITH A REMOTE NON-SERVER

Transferring data to  and  from  a  non-server  is  a  little  more

complicated, since you will need to continually change from BBC KERMIT

command  mode to terminal mode and back again. With a little practice,

though, the technique becomes natural.

5.6.1. Sending files to a non-server

To send a file to a non-server you use the command SEND. However,

unlike sending files to a server, you must also tell  the  remote

KERMIT that a file is on its way.

One means of doing this is as follows:

1. In terminal mode, start the remote KERMIT program, and issue

its RECEIVE command. This tells it to expect a file from BBC

KERMIT.  The  remote system may output a message when you do

this, but more probably it will do nothing but wait for  you

do something.

2. Press  CONTROL and F0 to return BBC KERMIT to KERMIT command

mode.

3. Issue the BBC KERMIT  SEND  command.  What  happens  now  is

identical  to  the actions described above for sending files

to a server: you can use the  same  wildcard  facilities  to

select files, etc., as described in section 5.5.1.

If  you  are transferring to another micro KERMIT, you would need

to do this:

1. From BBC KERMIT command mode,  issue  the  SEND  command  as

above.

2. From  the  other  micro's  command  mode,  issue its RECEIVE

command. BBC KERMIT by default allows you 30 seconds  to  do

this: you can change this interval if you wish. The command

SET DELAY 10

for example, changes the delay to 10 seconds.

48

BBC KERMIT User Guide

5.6.2. Receiving files from a non-server

If  the  remote  KERMIT  system is not a server, you will need to

transfer files from it by the exact reverse  of  the  above  SEND

procedure:  all  you  need  to do is reverse the roles of the two

machines. Thus, you could take the following steps:

1. In terminal mode, start the remote KERMIT program, and issue

its SEND command. This tells it to transfer a  file  to  the

BBC  system.  There will normally be a delay before anything

happens - the interval may be anything from  a  few  seconds

upwards,  and is intended to let you do the next step before

the transfer starts. On many KERMITs there will be a command

to let you change the interval if you find it too short.

2. Press CONTROL and F0 to return to BBC KERMIT command mode.

3. Issue the RECEIVE command to BBC  KERMIT.  When  the  remote

system's delay time expires, it will start to send the file.

The  RECEIVE  command tells BBC KERMIT to sit and wait until

this happens.

If you are transferring files between micro systems, you could do

1. From BBC KERMIT command mode, issue the RECEIVE command.

2. On the remote system, issue the SEND command.  The  transfer

will  start  once  the  remote  system's  delay  period  has

expired.

The advantage of the above approach is that you have no  need  to

hurry  to  prepare the BBC system, since the RECEIVE command will

leave it waiting indefinitely for  you  to  get  the  other  side

going. You can't, though, use this method when you are talking to

a  mainframe KERMIT, since once you issue the RECEIVE command you

cannot enter  terminal  mode  to  issue  the  corresponding  SEND

command to the remote system.

49

BBC KERMIT User Guide

5.7 TRANSFERRING DATA TO AND FROM MEMORY

BBC  KERMIT  will transfer data not only to and from files on disc,

but also to and from the BBC's random-access memory. BBC KERMIT  knows

at all times the currently-defined "source" and "destination" of data,

which  tell  it  what  to  do when sending and receiving respectively.

Source and destination can be set to either "file"  (the  default)  or

"memory".

For example, suppose you wished to send the contents of memory from

address &1900 to &357C inclusive to the remote system. You would issue

the command

SET SOURCE MEMORY 1900 357D

to  tell BBC KERMIT that data it sends now comes from meory instead of

file, and also tells it where in memory. Note that the second  address

given  is  "357D":  this  value is the last address to be transferred,

plus one.

When you now issue the SEND command, the contents of this  area  of

memory  will  be transferred. Since there is no BBC file involved, the

form of the SEND command  changes  slightly.  There  is  now  only one

parameter,  which  you  must supply: this is the name of the file that

you wish to be used on the remote system. Thus you might type

SEND /mem/contents

to send the memory contents to a file "/mem/contents"  on  the  remote

system.

Bringing  data  from  the  remote  KERMIT  into the BBC memory is also

simple. You might, for example, issue the command

SET DESTINATION MEMORY 1B00

to tell BBC KERMIT that data now goes into memory, starting at address

&1B00. Here also there is no BBC file involved, so the  forms  of  the

GET   and  RECEIVE  commands  change  slightly.  RECEIVE  now  has  no

parameters: all you are allowed to type is

RECEIVE

The GET command now has only one parameter, which is the name  of  the

file you wish the remote server to send you. Thus, you would type

GET /mem/dump

to bring the contents of file "/mem/dump" into the BBC's memory.

There  are  some  points  that  you  should  bear  in  mind when doing

transfers to and from memory:

1. The effect of the SET SOURCE and  SET  DESTINATION  commands

50

BBC KERMIT User Guide

persist until you specify otherwise. Thus, if you transfer a

group  of  files  from  the remote system to BBC KERMIT, all

will end up in the same memory area.

2. BBC KERMIT has no way of knowing whether the  addresses  you

specify are sensible: it is up to you to get them right.

3. Although  no files are involved, the transfer of data to and

from memory still obeys the settings  defined  by  the   SET

FILE  TYPE  command. Thus if you wish to transfer the memory

contents as unchanged binary data, you must type

SET FILE TYPE BINARY

to tell BBC KERMIT this.

51

BBC KERMIT User Guide

5.8 TRANSFERRING DATA TO A PRINTER

When you transfer a file from another system to your BBC  Computer,

BBC KERMIT will allow you to send it directly to a printer rather than

to disc or memory. The command

SET DESTINATION PRINTER

will  select this option: after you have issued it, all files that you

transfer with RECEIVE or GET will go directly to the printer.  Routing

to the printer will persist until you specify another destination.

This technique is very similar in result to using the terminal mode

"log  to  printer"  facility described in chapter 4. However, terminal

logging, since it does not use the KERMIT transfer protocols,  suffers

from  all  the restrictions described in the introductory chapters. It

is prone to error if you are using a noisy  communications  line,  and

also, of course, you may not be able to transfer all the characters of

a particular file. Using the "transfer to printer" option will let you

accomplish  the  result  using  all  of  KERMIT's  error detection and

transfer facilities.

A little care may be needed when using this facility, especially if

the KERMIT on the other machine is able to time out. Printers are  far

slower  than  discs,  of  course, so it will take BBC KERMIT very much

longer to deal with output from the other system and  acknowledge  it.

The  other  system may expect a rapid response, and so may assume that

the data did not arrive and start to take recovery action. You can get

round this potential problem by either setting a very long timeout  on

the remote system, or by disabling its time-out facility altogether.

In  this  mode BBC KERMIT will treat incoming data according to the

file type you have specified  with  the  SET FILE TYPE  command,  even

though  no  file  is  involved.  Thus, for instance, you might need to

specify

SET FILE TYPE BINARY

if you are transferring a graphics dump to a printer. You can also use

the file type to control whether lines of a text file  are  terminated

with   a   Carriage-Return   and   a   Line-Feed,   or   simply  by  a

Carriage-Return. If you have your printer set to perform an  automatic

line feed when a Carriage-Return byte is printed, you should use

SET FILE TYPE ASCII CR

otherwise, you should use

SET FILE TYPE ASCII CRLF

You  should  note  here that the "printer ignore" byte set by *FX 6 is

not used by BBC KERMIT.

52

BBC KERMIT User Guide

5.9 HANDLING PROBLEMS

By design, KERMIT is a highly reliable file  transfer  system,  and

performs considerably better than any "dumb capture" facility within a

terminal  emulator.  The error-detection capabilities of KERMIT ensure

that data is transmitted  correctly:  in  the  rare  cases  where  the

communications system you are using is unreliable, KERMIT systems will

abort a file transfer rather than transfer garbage.

That  said,  there  are  some  cases where you may need to give BBC

KERMIT some assistance. The most common case will arise when one  byte

is  transmitted by one system but does not arrive at the other. KERMIT

breaks data up into small chunks called "packets", and if the  missing

byte  is  one of the ones that the KERMIT systems use to control these

packets, you may end up with a machine that  is  waiting  forever  for

something that will never arrive.

The simplest way out of this possible problem is for you to keep an

eye  on  the progress of the transfer and see when it appears to grind

to a halt. The file transfer status display  shows  you  a  continuous

count  of  the  packets as they are transmitted and received: normally

the number will increment steadily. If the number does not change  for

a  significant time (and here you must bear in mind that the mainframe

you are talking to may  be  running  very  slowly,  so  allow  a  good

interval  to  pass), you can press the RETURN key once. This tells BBC

KERMIT to drop whatever it was doing, and retransmit its  last  packet

of  data  to  the other end. If the other system was stuck waiting for

data that had been lost, the retransmission will prod  it  into  life,

and  the  automatic recovery mechanisms of KERMIT should allow the two

ends to pick up the transfer from where it stopped.

If you wish, you can automate this recovery by using the  "timeout"

facility  of  BBC  KERMIT.  This  is described below in the section on

"Advanced facilities"

53

BBC KERMIT User Guide

5.10 ADVANCED FACILITIES

BBC KERMIT permits you to use  some  more  advanced  techniques  to

control  file  transfers.  Some of these facilities involve the remote

KERMIT system, and it is possible that you will  find  a  system  that

does not implement them, since they are fairly recent additions to the

KERMIT specification. However, BBC KERMIT will be able to detect this,

and will act accordingly.

5.10.1. Interrupting transfers

If  you  discover  that you don't want a transfer to continue for

some reason, you may interrupt it at  any  point  by  pressing  a

control key. There are two possibilities here:

1. To interrupt one file

Pressing  CONTROL  and X together will interrupt the sending

or receiving of a file. If the file was one of a group (i.e.

you have specified  a  wildcard  character  to  one  of  the

systems),  BBC  KERMIT  and  the remote system will carry on

with the next file in the group.

2. To interrupt a group

Pressing CONTROL and Z together will interrupt  the  sending

and  receiving of one file, as in (1) above. However, if the

file was part of a group, the whole group is abandoned.

As  we  saw  earlier,  transfer  interruption  uses  some  fairly

recently  devised  KERMIT  facilities. BBC KERMIT will notify the

remote system that you wish to interrupt the  transfer,  and  the

remote  system will acknowledge this if it supports the facility.

However, if it does not support the interrupt facility,  it  will

not respond correctly. When BBC KERMIT detects this situation, it

will  use a different method to abort the transfer, by simulating

an irrecoverable  error.  The  transfer  will  be  terminated  as

before,  but here the systems will not be able to carry on to the

next file of a group, so that CONTROL-X and CONTROL-Z  will  both

have the same effect.

A  more  drastic  way  of interrupting a transfer is to press the

CONTROL and E keys together. This causes BBC KERMIT to notify the

remote system of an  irrecoverable  error,  which  will  cause  a

transfer  to  be abandoned. This technique, though, should really

only be used if the transfer is going wrong in some way.

If you interrupt the reception of a file  into  BBC  KERMIT  (or,

indeed if an error such as "Disc full" or "Can't extend" occurs),

54

BBC KERMIT User Guide

you  will  be left with a file on disc that contains only part of

the data that should have been sent. By default, BBC KERMIT  will

delete  this  partial  file  for  you.  If,  though,  you want to

preserve whatever data has been transferred, you  can  issue  the

command

SET INCOMPLETE KEEP

after  which BBC KERMIT will close the file normally and preserve

it. You can re-instate the normal situation at any time with  the

command

SET INCOMPLETE DELETE

5.10.2. Using timeouts

As  we  saw  above, it is possible to automate the detection that

the transfer has stopped.  This  is  done  by  defining  "timeout

periods":  if  one or other system has not received any data when

its timeout expires, it will try to re-establish contact with the

other.

You control the use of  timeouts  in  two  ways.  The  first  way

enables  you  to  tell the remote system what timeout interval it

should use when receiving data from BBC KERMIT. This  information

is  transmitted  to  the remote system when a transfer starts. By

default, BBC KERMIT asks the remote system to use a timeout of 30

seconds, but you can  change  this  with  the  SET  SEND  TIMEOUT

command.  Note,  though, that the remote system may or may not do

as it is asked: it may not support the timeout facility;  it  may

ignore  the  value  sent and use its own interval; or the timeout

facility may need to be switched on by your giving it a command.

BBC KERMIT can also timeout if data  does  not  arrive  from  the

remote  system  within  a given interval. By default, the timeout

facility is turned off,  so  BBC  KERMIT  will  never  time  out.

Issuing the command

SET TIMER ON

will  switch  the  timeout facility on, and by default BBC KERMIT

will time out if it has not received any data in 15 seconds.  The

SET  RECEIVE  TIMEOUT  command  lets  you change this interval as

required. You can turn the timeout facility off a any time with

SET TIMER OFF

Note that here the remote system may ask  BBC  KERMIT  to  use  a

specified  timeout  period  when  a  transfer starts. BBC KERMIT,

though, always ignores this request, and decides what  to  do  on

the basis of the SET commands you have issued.

55

BBC KERMIT User Guide

5.10.3. File name translation

As  the preceding sections explained, a KERMIT program that sends

a file will pass the receiving KERMIT  a  standard-form  filename

derived  from  the  real name. Thus, a remote system may tell BBC

KERMIT to receive a file called

DATA.DAT

which has the "normal" format "name.type".

By default, BBC KERMIT will translate  the  filenames  the  other

system  sends  it,  in an attempt to form a name tht is legal for

the filing system in use.  It  will  discard  the  dot  and  what

follows it, and use the rest of the name as its basis.

This  approach  is  normally exactly what you require, especially

for mainframe-to-micro transfers. However, sometimes you  may  be

able to control the name the remote system sends, and may be able

to  ensure  that  it is a legal BBC filename without translation.

For example, if you are sending a file from  one  BBC  KERMIT  to

another, the sending command

SEND MYFILE $.PROGRAM.LIST3

would pass the name "$.PROGRAM.LIST3" to the other machine - here

you  have told  the sender the name to use rather than letting it

generate a "name.type" form.

In this case, you would tell the receiving system to use the name

exactly as it is with the command

SET FILE NAME UNTRANSLATED

command. It will  store  the  incoming  data  in  a  file  called

$.PROGRAM.LIST3  -  it's  up to you to make sure this really is a

legal filename.

You can restore the normal translation action at  any  time  with

the command

SET FILE NAME NORMAL

5.10.4. Detailed protocol control

The  rules  by which files are transferred between KERMIT systems

are termed the "KERMIT protocol". These rules  define  in  detail

56

BBC KERMIT User Guide

how data should be transferred: they specify how much can be sent

in one chunk or packet; what control sequences indicate the start

and  end  of a packet; what character encoding is to be used, and

so on. In almost every case you will have no need to  change  any

of  these  settings,  since they are carefully chosen so that any

KERMIT can communicate to any other KERMIT in  just  about  every

circumstance.

However,  it is possible that you may come across cases where you

need to change some of the protocol values, either to improve the

performance of  the  file  transfer  mechanism,  or  because  the

standard settings are inappropriate and do not work.

The  protocol  values  are  changed  by  the SET command, and BBC

KERMIT  allows  you  to  change  all  the  possible  values.  The

reference  section  details  all  the  SET commands concerned and

their effects. A detailed discussion of the various possibilities

is beyond the scope  of  this  user  guide,  though,  since  some

understanding  of  the  KERMIT  protocol is needed. You will find

this explained in the "KERMIT  Protocol  Manual"  (Edition  5  or

later);  or  you  might  contact the systems support personnel at

your computer centre.

57

BBC KERMIT User Guide

APPENDIX 1 : BBC KERMIT COMMANDS

This  appendix  lists  in full detail all the commands available in

BBC KERMIT. The commands are grouped in sections appropriate to  their

usage.

Throughout  the  appendix,  commands  are presented in a formal way

that shows you exactly what you may or must type at  each  point.  For

example, you might see a command described as having a format

SET PAUSE <number>

Here  the  fixed  parts,  that  you must  type,  are shown in capitals

(although, of course, you can type the  command  in  either  upper  or

lower  case  if  you wish). The third part of this particular command,

"<number>" is shown enclosed in "<>" brackets: this indicates that you

must provide some sort of value yourself, rather than type the  actual

word shown. The fact that the parameter is described as "number" tells

you  that  a  number  of  some  sort is needed: the description of the

command will tell you what sort of number, why you should  supply  it,

and whether you could leave it out and let BBC KERMIT itself provide a

suitable value. You shouldn't include the "<>" brackets in the command

that you type.

Thus, a valid way of typing the command in this example would be

SET PAUSE 10

where  you have supplied the numeric value "10" in the expected place.

BBC KERMIT will always  tell  you  if  you  either  omit  a  necessary

parameter or supply too many parameters.

All  commands,  and  most  parameters,  can be abbreviated to their

first few characters, provided that what you type uniquely  identifies

the command or parameter you mean. The command specification will tell

you about the few cases where parameters must always be typed in full.

58

BBC KERMIT User Guide

A1.1 COMMANDS FOR GENERAL CONTROL OF BBC KERMIT

The commands listed in this section are used to control the general

operation of BBC KERMIT.

CLOSE

This  command  closes  any  files  that  may  be  open.  The only

circumstance in which this command  is  needed  is  if  you  have

entered  BBC KERMIT when the maximum number of permitted files is

already open: in such a case you would not be  able  to  transfer

any data until the files were closed.

The command has no parameters.

Example:

CLOSE

DISPLAY

This  command displays whatever follows it on the command line to

the screen. It is mostly for use within TAKE files, where it  can

be used to indicate progress, etc.

The command format is:

DISPLAY <message>

The parameters are:

<message>

The   text   to  be  displayed.  Unlike  other  command

parameters, spaces are not taken  as  delimiters  here:

everything  from  the first non-space after the command

to the end of the line is output.

Example:

DISPLAY About to start

will output "About to start" on the screen.

EXIT

This command causes BBC KERMIT to return to BASIC at the end of a

session.

The command has no parameters.

59

BBC KERMIT User Guide

Example:

EXIT

FIX

This command, available in the sideways ROM version only, updates

the default parameter table within BBC  KERMIT  with  the  values

currently  established  with  the  SET command. Since the command

needs write access to the address area &8000 to &BFFF  where  the

BBC  KERMIT  code  is  held,  it can be used only if you have BBC

KERMIT in sideways RAM.

The command will ask if you are sure that you wish to do this: if

you reply "Y" the default table will be changed. The  new  values

will then be the defaults used when you enteR BBC KERMIT again.

The command has no parameters.

Example:

FIX

JSR

This  command,  available in the sideway ROM version only, allows

you to call some machine  code  routines  you  may  have  written

yourself  and  loaded into memory. The code is entered with a JSR

instruction and should exit with an RTS.

The command format is:

JSR <address>

The parameters are as follows:

<address>

Specifies the address of the code to be entered.

Example:

JSR &2000

enters some of your code at address &2000.

LOAD

This command loads a complete set of BBC KERMIT parameters  (i.e.

the  values  established  by the SET command) from a file on disc

produced by the SAVE command. You could use this command to avoid

having to type a large number of  SET  commands  every  time  you

start BBC KERMIT up.

The command format is :

60

BBC KERMIT User Guide

LOAD <filename>

The parameters are as follows :

<filename>

Specifies  the  name  of  a  file  produced by the SAVE

command.

If the file is in the correct format, BBC KERMIT  will  load  its

contents into memory, overwriting all the previous control values

established by the SET command.

Example :

LOAD :1.K.PARAMS

would  set  all  the  KERMIT control values from the data held in

file :1.K.PARAMS.

RESET

This command restores all the KERMIT  control  values  (i.e.  the

values controlled by the SET command) to the values they had when

BBC KERMIT was started.

Using  this  command enables you to instantly restore the control

values to their initial state after changes  without  needing  to

use the SET command.

The command has no parameters.

Example:

RESET

SAVE

This  command records all the current KERMIT control values (i.e.

the values established by the SET command) in a file on disc. You

may subsequently use the LOAD command to re-establish  the  saved

values.

This  command  enables  you to configure BBC KERMIT to your exact

requirements once only, record the new settings,  then,  whenever

you  start  BBC  KERMIT  again, you can reset these values in one

operation.

The command format is :

SAVE <filename>

The parameters are :

61

BBC KERMIT User Guide

<filename>

This is the name of a  file  on  disc  into  which  the

control values will be written.

Example:

SAVE :1.K.PARAMS

will write the current control values to the file :1.K.PARAMS.

SET BAUD

This  command  allows  you to define the line speed ("baud rate")

that BBC KERMIT will use on the RS423 port. BBC KERMIT allows you

to set the receive and transmit rates to the same, or  different,

values as you wish. The command format is:

SET BAUD <number> <direction>

The parameters are:

<number>

This  specifies the baud rate to be used. It can be any

of the baud rates supported by the BBC micro, that  is,

any  of the values 75, 110, 300, 1200, 2400, 4800, 9600

or 19200.

<direction>

This specifies whether you  wish  to  set  the  receive

rate,  the  transmit  rate,  or  both and can be one of

RECEIVE, TRANSMIT or BOTH. If you omit  the  parameter,

BBC KERMIT assumes you mean BOTH.

The command takes immediate effect. By default, BBC KERMIT uses a

baud rate of 9600 baud for transmit and receive.

Examples:

SET BAUD 1200

will set the baud rate to 1200 baud for transmit and receive.

SET BAUD 75 TRANSMIT

will  set  the  baud  rate  to 75 baud for transmitting data, but

leaves the receive baud rate unchanged.

SET DEBUGGING

This command allows you turn the BBC KERMIT debugging facility on

or off. The command format is:

SET DEBUGGING <action>

62

BBC KERMIT User Guide

The parameters are:

<action>

This must be either ON or OFF.

If debugging is on, BBC KERMIT will output information during all

file transfers to enable you to trace errors. It will record  the

protocol   state,  the  contents  of  all  packets  received  and

transmitted, plus indications of its  internal  operations.  This

information is output to the printer.

When  debugging  is  on it can be suspended or activated during a

file transfer at will. Pressing  CONTROL  and  D  when  the  file

transfer  status  display  is  showing  will  suspend  debugging;

pressing it again will restart it.

By default, debugging is OFF.

Example:

SET DEBUGGING ON

will turn the debugging facility ON.

SET FLOW-CONTROL

This command defines how BBC KERMIT controls the flow of data  in

and  out  of  the  RS423  port, in order to stop either it or the

remote system becoming overloaded.

The command format is

SET FLOW-CONTROL <type>

The parameters are:

<type>

This defines the type of flow control you wish. It  can

be  either  XON/XOFF  if you wish flow control to be by

the method of sending XON and XOFF control  characters,

or  CTS/RTS  if  you  wish  flow  control to be done by

hardware signals.

The type of flow control you should select  will  depend  on  the

system  you  are  connected  to:  consult  your  systems  support

personnel if you are unsure what to use.

By default, BBC KERMIT uses XON/XOFF flow control.

Example:

SET FLOW-CONTROL CTS/RTS

63

BBC KERMIT User Guide

will select hardware flow control.

SET HANDSHAKE

This command specifies whether "handshaking" is  to  be  done  in

file  transfer  mode.  This  operation is needed if BBC KERMIT is

linked to some IBM mainframes,  and  causes  it  to  wait  for  a

"handshake"  character  to  be  sent from the mainframe before it

attempts to send a data packet.

The command format is

SET HANDSHAKE <type>

The parameters are:

<type>

The type of handshake byte the mainframe is using.  The

value should be one of NONE, BELL, CR, LF, XON or XOFF.

The  first value specifies that no handshaking is to be

done: the others specify that it is  to  be  done  with

handshake   bytes   of   &07,  &0D,  &0A,  &11  or  &13

respectively.

By default, BBC KERMIT uses a value of NONE  and  handshaking  is

not performed.

Example:

SET HANDSHAKE XON

will turn handshaking on, and specify that an XON byte (code &13)

is to be used.

SET MUX-WAIT

This  command  is  used  in cases where the communications system

that BBC KERMIT is connected to is slow to respond to characters.

It causes BBC KERMIT to wait  for  a  specified  number  of  byte

periods  before  checking that it is clear to transmit each byte.

This delay is independent of the type of flow-control selected.

The command format is:

SET MUX-WAIT <number>

The parameters are:

<number>

The number of byte periods for which  BBC  KERMIT  will

64

BBC KERMIT User Guide

wait, in the range 0 to 255.

By  default, a value of 0 is used, so BBC KERMIT imposes no delay

before transmitting each byte.

Example:

SET MUX-WAIT 1

causes BBC KERMIT to delay for one  byte  period  before  testing

that it is clear to send a character.

SET PARITY

This command lets you define how characters are sent and received

on  the  RS423  port  in  terms  of what happens to the eighth or

"parity" bit of each byte.

The command format is

SET PARITY <type>

The parameters are:

<type>

This selects the type of parity you wish to be applied.

It can be one of MARK, SPACE, ODD, EVEN or NONE.

The setting of parity you select is dependent on the  system  you

are  connected  to:  you  should  consult  your  systems  support

personnel for advice if you are unsure of what to use.

The  setting  of  parity  also  affects  how  the  file  transfer

mechanism  copes  with  data  whose  ASCII codes are in the range

128..255. If parity is anything other than NONE, BBC KERMIT  will

assume  that  it  cannot send such data directly, and will try to

arrange with the remote system to encode it in a way that can  be

transmitted  correctly.  If  parity  is NONE, however, BBC KERMIT

will assume that all data can be transmitted, and will  only  use

the  encoding technique if the remote system explicitely asks for

it. In the case of some networks this may cause confusion,  since

the  network may not use parity, and the recommended way of using

terminals might then be with parity NONE. However, the  mainframe

system  itself  may  operate  with,  say,  SPACE  parity, and may

discard the parity bit from every byte it sends or receives. This

requires BBC KERMIT to encode bytes in the range 128..255, but if

parity is NONE it will do this only if the other system asks  for

it.  To  be  safe, you would normally set parity to SPACE in such

cases.

By default, BBC KERMIT sets parity to SPACE.

Example:

65

BBC KERMIT User Guide

SET PARITY EVEN

selects EVEN parity.

SET TAKE-ECHO

This command defines whether the commands that BBC  KERMIT  obeys

from  a  TAKE  file  instead of the keyboard are displayed on the

screen as they are executed.

The command format is

SET TAKE-ECHO <action>

The parameters are

<action>

This must be either ON or OFF

Setting TAKE-ECHO to ON will cause all the commands from the TAKE

file to be displayed on screen before they are obeyed. If  it  is

set  to  OFF,  the  commands will be obeyed, but you will not see

them on the screen.

By default, TAKE-ECHO is ON.

Example:

SET TAKE-ECHO OFF

will turn off the display of commands from TAKE files.

SHOW

This command displays  the  values  of  all  the  KERMIT  control

values. The display is broken up into several screens full: after

each  screen  you are given the option of pressing the RETURN key

to see the next screen, or the  ESCAPE  key  to  quit  and  input

another command.

The  output  of  the  SHOW  command  uses a 40 byte MODE 7 screen

whether or not you have used the WIDTH command to  select  an  80

byte wide command screen.

The command has no parameters.

Example:

SHOW

66

BBC KERMIT User Guide

TAKE

This  command causes BBC KERMIT to read all further commands from

a file instead of from the keyboard. The file can be  constructed

with  an  editor,  a  word-processing  system, or with the *BUILD

command as you prefer.

The command format is:

TAKE <filename>

The parameters are:

<filename>

This specifies the name of a file containing BBC KERMIT

commands, in the same format as if they were  typed  at

the keyboard.

Once  you issue the TAKE command, BBC KERMIT will read characters

from the specified file instead of the keyboard.  Any  KERMIT  or

Operating  System  command can be issued from within a TAKE file,

except a further  TAKE  command.  By  default,  BBC  KERMIT  will

display  each  command on the screen as it is obeyed, but you can

switch this echo off with the SET TAKE-ECHO command.

You may stop the use of the TAKE file at any  point  by  pressing

the  ESCAPE  key.  Execution will also stop if any command within

the TAKE file causes  an  error,  or  if  you  interrupt  a  file

transfer with by pressing CONTROL and Z (but not CONTROL and X).

Example:

TAKE :3.THINGS

will read subsequent commands from the file :3.THINGS.

WAIT

This command, primarily used within TAKE files, causes BBC KERMIT

to  wait before proceeding with the next command. You can specify

an indefinite wait, or nominate a length of time: in either  case

pressing any key will terminate the command.

The command format is:

WAIT <time>

The parameters are:

<time>

The interval in seconds for which BBC KERMIT will wait,

in  the  range  0  to  255.  If you omit the value, BBC

KERMIT waits until a key is pressed.

67

BBC KERMIT User Guide

Example:

WAIT 30

causes BBC KERMIT to wait for 30  seconds,  or  until  a  key  is

pressed.

WIDTH

This  command  allows you to choose between a 40 character and an

80 character command screen. It  may  not  be  used  in  the  RAM

version on a model B.

Normally,  BBC  KERMIT  uses  a MODE 7 command screen, but if you

wish to *TYPE a BBC file or  are  using  the  REMOTE  command  to

obtain  information  from  a  remote  server then an 80 byte wide

screen can be more convenient. The text colour used  for  the  80

character screen is controlled by the SET VDU-COLOUR command, and

the  SET TV  command  determines  whether a *TV command is issued

automatically on changing  from  40  character  to  80  character

screens.

The command format is

WIDTH <width>

The parameters are:

<width>

Specifies  the  width  of the command screen. The value

can be either 40 or 80.

By default, BBC KERMIT uses a 40 character command screen.

Example:

WIDTH 80

selects an 80 character command screen.

68

BBC KERMIT User Guide

A1.2 COMMANDS FOR TRANSFERRING DATA

In this section we  shall  look  at  the  detailed  format  of  the

commands  that  you  use  to  transfer  files using BBC KERMIT, and to

control how BBC KERMIT will perform the transfers.

GET

This command should only be used if the remote KERMIT is  running

in  server  mode.  BBC  KERMIT  will  send  the server a command,

requesting it to transfer a file: the effect is to "pull" a  file

from the remote server to BBC KERMIT.

The format of the command is this:

GET <remote-file> <local-file>

The parameters are as follows:

<remote-file>

This  parameter is mandatory, and specifies the name of

the file on the remote machine. The format of the  name

will,  of  course,  be  that  required  by  the  remote

machine.

<local-file>

This parameter is optional, and specifies the  name  of

the  file on the BBC system into which you wish data to

be  transferred.  This  name  may  include  drive   and

directory  specifications if you wish the file to be on

a drive and directory other than the current default.

When this command is issued, BBC KERMIT will send  a  command  to

the  remote  server  to  pull the file from it. The transfer will

take place automatically, and you need do nothing further.

There are several points to be considered about  the  format  and

use of the filename parameters.

1. If   you  have  used  the  SET  DESTINATION  MEMORY  or  SET

DESTINATION PRINTER commands to transfer data into the BBC's

RAM or to a printer instead of into a  file,  you must  omit

the <local-file> parameter.

2. If  the  destination of the data is a file, and you omit the

<local-file> parameter, BBC KERMIT will try  to  generate  a

legal  BBC  filename from the <remote-file> you supplied. If

this name is the same as a file that already exists, then by

default BBC KERMIT will try to alter it until  there  is  no

clash. The display will tell you what is happening, and give

you  the  name  of  the  file  that  is actually going to be

69

BBC KERMIT User Guide

written. You may prevent this renaming  operation  with  the

SET  FILE WARNING command: if you turn the facility off, any

existing file will be overwritten.

3. If you do specify a <local-file> parameter, then BBC  KERMIT

will  use  this file for the received data whether or not it

exists already. It will not attempt to rename  the  file  to

miss an existing one.

4. You  may  GET  a  group  of  files from the remote server by

putting  that  system's   wildcard   characters   into   the

<remote-file>. The remote server will send each file in turn

automatically,  and  you need take no action until the group

transfer is complete. Note that, if you do include wildcards

in  the  <remote-file>  parameter,   you   must   omit   the

<local-file>  parameter  or  all  the  files from the remote

server will be placed in the same BBC file.

RECEIVE

This command should be used only  if  the  remote  KERMIT  is not

running  in server mode, and causes BBC KERMIT to wait for a file

transfer to be started by the remote system. You will  thus  need

to  issue  a  SEND  command  to the other KERMIT in order to make

something happen.

The command format is this:

RECEIVE <local-file>

The parameters are:

<local-file>

This parameter is optional, and specifies the name of a

file on the BBC system into which you wish data  to  be

transferred.  It  must be a legal BBC filename, and can

include drive and directory specifications.

When the command is issued, BBC KERMIT will wait passively for  a

signal  from the remote KERMIT that a file transfer is beginning.

This signal will include the name of the file that is being sent:

if you have included the <local-file> parameter, this name is for

information only, and the data will be written to  the  file  you

have identified.

If  you  do  omit  the  <local-file>  parameter,  BBC KERMIT will

attempt to  generate  a  suitable  BBC  filename  from  the  name

supplied  by the remote system. The actions taken are the same as

those described above in the GET command.

Some points that  must  be  considered  when  using  the  RECEIVE

command are:

1. If   you  have  used  the  SET  DESTINATION  MEMORY  or  SET

DESTINATION PRINTER commands to transfer data into the BBC's

70

BBC KERMIT User Guide

RAM or to a printer instead of into a  file,  you must  omit

the <local-file> parameter.

2. If  you  have  directed the remote system to send a group of

files by including a wildcard in the  filename,  you  should

omit the <local-file> parameter, or all the data will end up

in  the  one  BBC  file,  each file overwriting the previous

ones.

SEND

This command is used whether  or  not  the  remote  KERMIT  is  a

server, and sends a file or group of files to it.

The command has two possible formats:

SEND <local-file> <remote-file>

used if you are transferring data from a BBC file, and

SEND <remote-file>

if you are sending data from the BBC's memory.

The parameters are as follows:

<local-file>

This  parameter  specifies  the name of the file on the

BBC that you wish to send, and can  be  any  legal  BBC

filename.

<remote-file>

This parameter is optional if you are sending data from

a  file, and mandatory if you are sending data from the

BBC's memory. It specifies the filename that BBC KERMIT

will send to the remote system to  identify  the  file:

this  may  be  used by the remote system as the name of

the file that it  puts  the  data  into,  but  this  is

entirely  up  to  the remote system. If the filename is

legal for the remote system, the file would normally be

called by this name.

If the remote KERMIT is not running  in  server  mode,  you  must

issue  a  RECEIVE  command to the remote system to prepare it for

this SEND. You can either do  this  before  you  issue  the  SEND

command,  or  you  can  issue  the  SEND first and then issue the

RECEIVE to the other system. To  allow  you  to  do  this  latter

option,  BBC  KERMIT  will pause after you issue the SEND command

(by default for 30 seconds) before it  actually  starts  sending.

You  can  change  this pause interval with the SET DELAY command;

alternatively, pressing any key while BBC KERMIT is waiting  will

make it start the transfer immediately.

71

BBC KERMIT User Guide

Some points to note about the SEND command are:

1. If  you  omit  the  <remote-file> parameter, BBC KERMIT will

generate a "generally acceptable" name from the <local-file>

parameter and pass this to the remote system. It forms  this

name    by   stripping   off   any   drive   and   directory

specifications, then adding a filename suffix  in  the  form

".XXX"  to  what  remains.  By  default, the suffix added is

".BBC", but the SET FILE SUFFIX command allows you to change

this, or suppress it altogether.

2. You may send a group of files in one operation by  including

the  "*"  or  "#"  wildcard  characters in <local-file>. BBC

KERMIT will then send all files  in  the  current  directory

whose names match. You should not specify drive or directory

details in the name if you include wildcards: you may select

different  drives  with  the  DFS  *DRIVE command before you

issue  the  SEND  if  you  need  to.  You  must   omit   the

<remote-file> name if you include wildcards in <local-file>.

SET DELAY

This command sets the period for which BBC KERMIT will wait after

you issue a SEND command.

The command format is

SET DELAY <number>

The parameters are

<number>

The delay in seconds, in the range 0 to 255.

When you issue a SEND command, BBC KERMIT will do nothing for the

specified  period  to  enable  you  to issue commands to a remote

system. You can interrupt the delay and make BBC  KERMIT  proceed

by pressing any key.

By  default, BBC KERMIT will delay for 30 seconds after you issue

a SEND command.

Example:

SET DELAY 5

will cause BBC KERMIT to wait after a SEND command for 5 seconds,

or until a key is pressed.

SET DESTINATION FILE

This command specifies that any data received by BBC KERMIT is to

be stored in a file, rather than placed into the BBC's memory  or

72

BBC KERMIT User Guide

sent directly to the parallel printer.

The command format is

SET DESTINATION FILE

This is the default condition when BBC KERMIT is entered.

SET DESTINATION MEMORY

This  command,  available  in  the  sideways  ROM  version  only,

specifies that data received by BBC KERMIT is to be  placed  into

the BBC's memory rather than being stored in a file.

The command format is

SET DESTINATION MEMORY <address>

The parameters are:

<address>

This  specifies  the  address  where you want the first

byte received to be placed. The value must be from 1 to

4 hexadecimal digits, optionally preceded by  "X",  "$"

or "&".

After  issuing  this command, any data that you receive is placed

in memory instead of a file. The first  byte  is  placed  at  the

specified  address,  and  subsequent  bytes  are placed at higher

addresses. BBC  KERMIT  will  not  allow  the  received  data  to

overflow  into the mode 7 file transfer status display screen, so

that the highest address available is &7BFF.

BBC KERMIT makes no checks on the address supplied: it is  up  to

you  to  ensure  that  you  direct  data to a sensible place. The

destination for incoming data will remain the BBC's memory  until

you specify otherwise.

Note  that BBC KERMIT will process the data received according to

the currently defined file type, even though the  destination  is

not a file.

The default condition for BBC KERMIT is to place data in files.

The RAM version of BBC KERMIT always uses destination FILE.

Example:

SET DESTINATION MEMORY 1900

will  cause  incoming  data  to go to memory, starting at address

&1900 (the default value of PAGE for Acorn DFS systems).

73

BBC KERMIT User Guide

SET DESTINATION PRINTER

This command specifies that data received by BBC KERMIT is to  be

transferred directly to a printer rather that stored in a file or

in memory.

The command format is

SET DESTINATION PRINTER

The  default  condition  is  for BBC KERMIT to transfer data into

files.

Note that BBC KERMIT will process the data received according  to

the currently defined file type, even though no file is involved.

SET INCOMPLETE

This  command  defines  what  BBC  KERMIT  will  do if either you

interrupt the reception of a file, or if an error occurs while  a

file is being received.

The command format is

SET INCOMPLETE <action>

The parameters are

<action>

This must be either KEEP or DELETE.

If  the  action is set to DELETE, BBC KERMIT will delete any file

whose reception is interrupted and is  thus  incomplete.  If  the

action  is  set to KEEP, BBC KERMIT will close the file normally,

and you will be able to use whatever data was received before the

interruption or error.

By default, BBC KERMIT will DELETE an incomplete file.

Example:

SET INCOMPLETE KEEP

causes BBC KERMIT to leave an incomplete file on the  disc  after

the transfer.

74

BBC KERMIT User Guide

SET FILE BUFFER-SIZE

This command, available in the sideways ROM version only, defines

the size of buffer BBC KERMIT uses when reading or writing files.

The command format is

SET FILE BUFFER-SIZE <number>

The parameters are:

<number>

This  specifies  the  size  of  the buffer, in 256 byte

pages. It must be a value from 1 to 64.

BBC KERMIT uses the file buffer to read and write files in  large

chunks,  which  can  improve  performance in some cases. However,

with large buffers, although BBC KERMIT  will  access  the  discs

less  frequently, it will take longer when it does, so the remote

KERMIT may mistakenly think that contact has been lost.  If  this

occurs, the buffer size can be reduced.

By default, BBC KERMIT uses a buffer of 16 pages (4K bytes).

The  RAM  version of BBC KERMIT uses a file buffer of 8 pages (2K

bytes).

Example:

SET FILE BUFFER-SIZE 1

changes the file buffer to be 1 256 byte page long.

SET FILE SUFFIX

This command defines the characters that BBC KERMIT will add to a

BBC filename when generating a "generally acceptable" filename to

send to the remote system.

The command format is

SET FILE SUFFIX <string>

The parameters are

<string>

This is the suffix you wish added, consisting of up  to

8  characters.  The  "."  is not part of the suffix and

should not be included.

BBC  KERMIT  will,  when  generating  a  "generally   acceptable"

75

BBC KERMIT User Guide

filename,  take  the  final  part of the BBC filename, add a ".",

then add the suffix defined by this command.  This  command  lets

you  change the suffix to suit your requirements. If you omit the

string, BBC KERMIT will not add anything to the BBC  filename  at

all (not even the ".")

By default, the suffix added to the BBC filename is "BBC".

Examples:

SET FILE SUFFIX ADE

will  cause  BBC  KERMIT  to  add ".ADE" to BBC filenames for the

remote name.

SET FILE SUFFIX

will cause it not to add anything at all to the BBC filename.

SET FILE NAME

This command controls how BBC KERMIT handles the  filename  given

it by a remote system when it is receiving a file.

The command format is

SET FILE NAME <action>

The parameters are

<action>

This can be one of NORMAL or UNTRANSLATED.

If  you set the action to NORMAL, BBC KERMIT will assume that the

name given by the remote  system  has  the  standard  "name.type"

form,  and will generate a BBC filename from it. If the action is

UNTRANSLATED, BBC KERMIT will use the exact name  to  create  the

file: it is up to you to ensure that what the remote system gives

it really is a valid BBC file name.

By  default,  BBC  KERMIT  will  translate  filenames sent by the

remote system. (action NORMAL).

Example:

SET FILE NAME UNTRANSLATED

causes BBC KERMIT to use the name sent exactly as it is.

76

BBC KERMIT User Guide

SET FILE TYPE ASCII

This command  tells  BBC  KERMIT  that  files  it  transmits  and

receives  are  to taken as containing printable ASCII text and to

transform them accordingly.

The command format is

SET FILE TYPE ASCII <end-of-line>

The parameters are

<end-of-line>

This optional parameter tells BBC KERMIT how a file you

are sending denotes the end of a line of text,  or  how

you  wish  this to be denoted in a file you receive. It

can be one of LFCR, CRLF, CR or LF, where LF stands for

the Line Feed character, and CR for Carriage Return. If

you omit the parameter, BBC KERMIT assumes LFCR.

After  using  this  command,  BBC  KERMIT  treats  all  files  as

containing  printable  ASCII  text,  with  the  end  of each line

indicated by the specified sequence of control  characters.  When

sending  a  file,  it  will  transform  every  occurrence  of the

specified  sequence  into  KERMIT's  standard   "end   of   line"

indicator,  and  the  remote  KERMIT should then change this into

whatever the standard representation of end-of-line is on its own

system. When  receiving  files,  BBC  KERMIT  will  change  every

occurrence  of  the  standard KERMIT indicator into the specified

sequence of bytes.

Note  that  this  command  has  effect  even  if  the  source  or

destination  of  data  is memory rather than files. Also, you may

need to give an equivalent command to the remote KERMIT system to

make it treat the data correctly.

By default, BBC KERMIT treats files as ASCII, with  end  of  line

indicated by LFCR bytes.

Example:

SET FILE TYPE ASCII CR

specifies  a file type of ASCII, with lines being terminated with

a CR (&0D) byte.

SET FILE TYPE BINARY

This command tells BBC KERMIT that all files it receives or sends

should be treated as containing binary data.

The command format is

SET FILE TYPE BINARY

77

BBC KERMIT User Guide

After using this command, BBC KERMIT will transmit the bytes from

a file exactly as they are, and will  not  change  any  of  them.

Similarly,  the  data it receives will be written to file with no

alteration.

Note  that  this  command  has  effect  even  if  the  source  or

destination  of  data is the BBC's memory or the parallel printer

rather than a file. Also, you may  need  to  give  an  equivalent

command  to  the  remote  KERMIT system to make it treat the data

correctly.

By default, BBC KERMITs treats files as ASCII text.

Example:

SET FILE TYPE BINARY

makes BBC KERMIT treat files as containing binary data.

SET FILE WARNING

This command specifies the action that BBC KERMIT  must  take  on

receiving  a  file  if  the name it tries to store the data under

clashes with that of an existing file.

The command format is

SET FILE WARNING <action>

The parameters are

<action>

This must be either ON or OFF.

If the file warning facility is ON, BBC KERMIT will check whether

the name it wishes to use to store incoming data clashes with  an

existing  file  name.  If  it does, a warning is displayed on the

screen, and BBC KERMIT will change the  right-most  character  of

the  name  to  a  "+" and check again. If this still clashes, the

next right-most characer is changed, and so  on.  An  error  will

occur  if  no  unique name is found by the time the name has been

changed to be entirely "+" characters.

If file warning is OFF, BBC  KERMIT  will  make  no  checks.  Any

existing file will be overwritten with the new data.

By default, file warning is ON.

Example:

SET FILE WARNING OFF

78

BBC KERMIT User Guide

turns off the file warning facility.

SET RECEIVE TIMEOUT

This  command  defines  the  interval after which BBC KERMIT will

take action if it has received no data from the remote system.

The command format is

SET RECEIVE TIMEOUT <interval>

The parameters are

<interval>

Specifies the timeout period in seconds, in the range 0

to 255.

The command defines the timeout that BBC KERMIT is to use.  Note,

though,  that  BBC  KERMIT  will  not  perform any timeout at all

unless you have used the SET TIMER command to turn  the  facility

on - by default it is OFF.

The  choice  of  the  timeout period will depend on many factors.

Regard should be paid to how fast the remote system is running  -

if it is mainframe, it could be running slowly due to heavy use -

and  also to the delays that operating over a network may impose.

Care should be taken not to set the value too low, or BBC  KERMIT

will repeatedly timeout unnecessarily.

By default, a timeout interval of 15 seconds is set.

Example:

SET RECEIVE TIMEOUT 10

causes  BBC  KERMIT to timeout if no data has been received after

10 seconds, providing that the timeout facility is turned on.

SET SEND TIMEOUT

This command sets the timeout interval that BBC KERMIT  asks  the

remote system to use. The remote system should timeout if no data

has been received from BBC KERMIT after this interval.

The command format is

SET SEND TIMEOUT <interval>

The parameters are

<interval>

specifies  the  interval  required  in  seconds, in the

79

BBC KERMIT User Guide

range 0 to 94.

BBC KERMIT will pass this value  to  the  remote  system  at  the

beginning  of  every  file  transfer.  Note,  though,  that it is

entirely up to the remote system  whether  it  uses  this  value,

ignores  it, or uses a value of its own. You may need to give the

remote system commands to turn its timeout facility on.

By default, BBC KERMIT requests the remote  system  to  time  out

after 15 seconds.

Example

SET SEND TIMEOUT 10

causes  BBC  KERMIT to ask the remote system to time out after 10

seconds if no data has been received from the BBC system

SET SOURCE FILE

This  command,  available  in  the  sideways  ROM  version  only,

specifies  that  data  transmitted  to the remote system is taken

from files rather than the BBC's memory.

The command format is

SET SOURCE FILE

This is the default condition when BBC KERMIT is entered.

The RAM version of BBC KERMIT always uses source FILE.

SET SOURCE MEMORY

This command, available in the sideways ROM version only,  causes

data  transmitted to the remote system to be taken from the BBC's

memory rather than from files.

The command format is

SET SOURCE MEMORY <start-address> <end-address>

The parameters are

<start-address>

Specifies  the  address  of  the  first  byte   to   be

transferred.  The value must be from 1 to 4 hexadecimal

digits, optionally preceded by "X", "$" or "&".

<end-address>

Specifies  the  address  of  the  last   byte   to   be

transferred  plus  one.  The  value must be from 1 to 4

hexadecimal digits, optionally preceded by "X", "$"  or

80

BBC KERMIT User Guide

"&".

After  issuing  this  command, BBC KERMIT will send data from the

BBC's memory instead of from a file. the area  of  memory  to  be

transmitted is defined by the two command parameters: it is up to

you to ensure that the addresses are sensible.

Note  that  BBC  KERMIT  will  still process the data transmitted

according to the defined file type, even  though  the  source  of

data is not a file.

By default, BBC KERMIT will use source FILE.

The RAM version of BBC KERMIT always uses source FILE.

Example

SET SOURCE MEMORY 1B00 3200

will  cause  BBC KERMIT to send the data between memory addresses

&1B00 and &31FF inclusive in a SEND command.

SET TIMER

This command controls whether BBC KERMIT will timeout if no  data

is  received  from the remote system after the timeout period has

expired.

The command format is

SET TIMER <action>

The parameters are

<action>

This must be either ON or OFF.

If the timer facility is turned ON, BBC KERMIT will use the value

defined by SET RECEIVE TIMEOUT to time the remote system out.  If

it  is  OFF,  the  value is ignored, and BBC KERMIT will wait for

ever for data.

By default, the timer is OFF.

Example

SET TIMER ON

turns the timeout facility on.

81

BBC KERMIT User Guide

A1.3: COMMANDS FOR TERMINAL EMULATION

In this section we shall consider the commands that you can use  to

control the terminal emulation facilities of BBC KERMIT.

CONNECT

This  command  changes BBC KERMIT from command mode into terminal

emulation mode. Once you have issued the command the screen  will

clear  and  will  become  a  40  character  or  80 character wide

terminal screen. From this point, anything you type will  not  be

examined  by  BBC  KERMIT, but will be sent directly to the RS423

output and the system you are linked to.

You may now operate your BBC micro as  if  it  were  an  ordinary

terminal.  Depending on how you have set up BBC KERMIT, this will

be either an ordinary teletype device, or one compatible with the

DEC VT52 standard.

At any time you can return to KERMIT command mode by pressing the

CONTROL and F0 keys together. You will  then  be  able  to  issue

KERMIT  commands  as  before: the CONNECT command will re-instate

you as a terminal at the point where you left off.

Note that if you have  selected  a  terminal  type  that  is  not

possible (for exaple, you have selected a VT52 in the RAM version

on  a  model  B),  the  command  will  produce  an  error message

"Terminal not supported"

The command has no parameters.

Example:

CONNECT

Whenever you  enter  terminal  mode  with  the  CONNECT  command,

printer  logging  will  be off, disc logging will be deactivated,

and any pause you may  have  placed  on  screen  output  will  be

lifted.

LOGGING

This  command  is  used to initiate or terminate terminal session

logging, the facility by which all characters  appearing  on  the

screen in terminal mode are also written to a file.

The command has two formats:

82

BBC KERMIT User Guide

LOGGING ON <filename>

or

LOGGING OFF

The  first  format initiates terminal session logging and opens a

file of name <filename>. The second format terminates logging and

closes the file.

When you use the command to initiate  logging,  BBC  KERMIT  will

open  a  file  on  disc  in  readiness. At this point, though, no

logging will take place. To cause characters to be written to the

files, you will need to  press  the  CONTROL  and  F1  keys  from

terminal  mode  (i.e.  after  using  the  CONNECT  command).  Now

characters appearing on screen will be written  to  the  further.

Pressing  CONTROL and F1 again will stop logging; a further press

will restart it, and so on.

You should remember to always use the

LOGGING OFF

command when you have finished, or you may lose  some  data  that

has not yet been written to the disc.

Example:

LOGGING ON LOGFILE

opens a file LOGFILE for terminal logging.

SET IGNORE

This command nominates one character that will be totally ignored

when it is received from the host.

The command format is

SET IGNORE <number>

The parameters are:

<number>

The numeric code of the character to be ignored. If you

omit  the value, any previous setting will be cancelled

and all characters will be actioned.

By default, BBC KERMIT will display  all  characters  with  ASCII

codes  greater  than  31,  and  also  the  CR, LF, DEL, BS and FF

control codes. TAB  characters  are  expanded  into  spaces,  and

ESCAPE  is  either  displayed  or taken as starting a VT52 ESCAPE

sequence. All other characters are always ignored.

83

BBC KERMIT User Guide

Example:

SET IGNORE &7F

causes the DEL character (code &7F) to be  ignored  if  the  host

sends it.

SET KEY-CLICK

This  command  controls  whether  an  audible  click  is produced

whenever you press a key in terminal emulation mode.

The command format is

SET KEY-CLICK <action>

The parameters are:

<action>

This must be either ON or OFF

Setting key-click to ON will cause a click to be produced whenver

you press a key.

By default, key-click is OFF.

Example

SET KEY-CLICK ON

will turn the audible key-click ON.

SET LOCAL-ECHO

This command lets you define  how  characters  you  type  on  the

keyboard  in  terminal  emulation mode are to be displayed on the

screen. When you press a key, the character is sent to the remote

system,  which  may  then  echo   it   back   to   your   screen.

Alternatively,  it  may  not  perform this echo: in this case BBC

KERMIT must put the character on the screen itself.

The command format is

SET LOCAL-ECHO <action>

The parameters are:

<action>

This must be either ON or OFF

If local echo is ON, BBC KERMIT itself will display characters on

84

BBC KERMIT User Guide

the screen. If it is OFF, the remote system is expected  to  echo

each character back as you type it.

By default, local echo is OFF.

Example

SET LOCAL-ECHO ON

will turn local echo on.

SET TERMINAL

This  command allows you to select the type of terminal emulation

you reuire in terminal mode.

The command format is

SET TERMINAL <type>

The parameters are:

<type>

This can be one of TT40, TT80 or VT52.  The  first  two

provide  teletype  emulations  using 40 or 80 byte wide

screens, and the last gives VT52 emulation. Some  other

types  may  be  selectable by ths command, but only the

three above are used in this release of BBC KERMIT.

Note that the command will  allow  you  to  select  any  type  of

emulation,  whether  or  not  it  can  actually  be provided. The

CONNECT command will fail, though, if the type  required  is  not

available.

By  default, the sideways ROM version of BBC KERMIT provides VT52

emulation on all machines. The RAM version of BBC KERMIT provides

VT52 emulation on the B+, B+128 and Master 128, and TT40  on  the

model B.

Example

SET TERMINAL TT80

Makes  the  BBC micro appear as a simple teletype device using an

80 column screen.

85

BBC KERMIT User Guide

SET TV

This  command  lets  you  specify  whether  BBC   KERMIT   should

automatically  issue  a  *TV  command  when  it  enters  terminal

emulation mode or 80 character command screen mode.

The command format is

SET TV <number-1> <number-2>

The parameters are:

<number-1>

The first *TV parameter to be used.

<number-2>

The second *TV parameter to be used.

If you omit both parameters, BBC KERMIT will no  longer  issue  a

*TV command: however, the effect of any previously issued command

will persist. By default, BBC KERMIT issues a *TV 0 1 to turn off

interlace.

Example:

SET TV 255 1

causes  BBC  KERMIT  to  issue  a  *TV 255 1  command on entering

terminal mode or 80 character command screen mode.

SET VDU-COLOUR

This command, available in the sideways ROM  version  only,  lets

you  select  the  colour  used  for text in 80 character terminal

emulation mode or 80 character command screen mode.  The  command

has  no  effect  for  40  character terminal mode. The background

colour cannot be changed, and will always be black.

The command format is

SET VDU-COLOUR <colour>

The parameters are:

<colour>

Specifies the colour to be used, and  must  be  one  of

RED, GREEN, YELLOW, BLUE, MAGENTA, CYAN or WHITE.

By default, the text colour used is white.

Example:

SET VDU-COLOUR GREEN

will select green text on a black background.

86

BBC KERMIT User Guide

TRANSMIT

This  command  allows  you  to send a text file from the BBC to a

host that does not have KERMIT. Binary files cannot  normally  be

sent in this way.

The command format is

TRANSMIT <filename>

The parameters are:

<filename>

This specifies the file you wish to send.

When  you issue this command, BBC KERMIT will automatically enter

terminal mode and will send the contents of the file to the host.

It will remain in terminal mode after all the data has been sent,

and you will be able to type at the keyboard as  normal.  Sending

the file can be interruped at any time by pressing ESCAPE.

Example:

TRANSMIT DELEG8

will send the text file DELEG8 to the host in terminal mode.

87

BBC KERMIT User Guide

A1.4 COMMANDS FOR CONTROL OF REMOTE SERVERS

This  section specifies the command that you can use to control the

operation of a remote server.

It is important to note that servers vary in  which  commands  they

implement  from  this section, and also on the exact way in which they

action  others. All  servers  should  implement  the  BYE  and  FINISH

commands,  but  some  may  treat  them  both  as  FINISH and leave the

terminal session logged in. Some servers implement all the  facilities

described   under  the  REMOTE  command  below,  some  implement  them

partially or in different ways, and some don't implement them at  all.

In  particular,  some  servers  may  implement commands such as REMOTE

DIRECTORY but ignore any parameters you supply. The  only  way  to  be

sure is to consult the user guide for the server concerned.

If  you  do  attempt to use a server command that is not available, an

error  message  such  as  "Unimplemented  server  command"   will   be

displayed.

In  all  cases, you can interrupt a REMOTE command that is producing a

large amount of output by  pressing  CONTROL  and  Z.  The  techniques

described  in chapter 5 for handling problems are also applicable here

should the transfer of the output stop for some  reason:  however  the

REMOTE  command  does  not  provide  you  with  any  indication of the

progress or otherwise of the transfer.

Most of the REMOTE commands produce at least one line of  output  from

the  remote  server which is displayed on BBC KERMIT's command screen.

Often this output will be formatted assuming that  the  screen  is  80

characters  wide:  if  you  are  using the sideways ROM version of BBC

KERMIT, or are using a B+, B+128 or Master 128,  you  can  change  the

command screen width from 40 to 80 characters with the command

WIDTH 80

so that the output layout looks correct.

BYE

This command tells the remote server to cease operation, and also

to log out the terminal job in which it is running.

The command has no parameters.

Example:

BYE

88

BBC KERMIT User Guide

FINISH

This  command  tells the remote server to cease operation, but to

leave the terminal session logged in.

The command has no parameters.

Example:

FINISH

REMOTE COPY

This command instructs the server to copy one  file  to  another,

leaving the source file unchanged.

The command format is

REMOTE COPY <oldfile> <newfile>

The parameters are:

<oldfile>

Specifies the source file for the copy operation.

<newfile>

Specifies the name of the file the data is to be copied

into.

Example:

REMOTE COPY LURGI.ILL SICKNOTE.LIS

would  copy  the  contents  of  file  LURGI.ILL  into  a new file

SICKNOTE.LIS.

REMOTE CWD

This command instructs the server to change its  current  working

directory (or wherever it looks for files unless told otherwise).

The command format is

REMOTE CWD <directory>

The parameters are:

<directory>

This  optional  parameter  specifies  the  name  of the

directory to be used in future.

89

BBC KERMIT User Guide

If you omit the directory  name,  the  server  should  reset  its

current  directory to that in force when you logged in. If you do

specify a name, BBC KERMIT will prompt you  for  a  password,  in

case  the  server  system  requires  one. You can either type the

password, which will not be echoed  to  the  screen,  then  press

RETURN; or, if no password is needed, simply press RETURN.

Example:

REMOTE CWD [.SUBDIR]

would  tell  a  server  running  under VAX/VMS to set the current

directory to be [.SUBDIR]

REMOTE DELETE

This command instructs the server to delete the specified file.

The command format is

REMOTE DELETE <filename>

The parameters are:

<filename>

Specifies the file you wish  deleted.  The  server  may

permit you to specify a wildcarded name, or it may not.

Example:

REMOTE DELETE /temp/notalotofgood

would ask the server to delete the file /temp/notalotofgood.

REMOTE DIRECTORY

This command instructs the server to produce a directory listing,

which is displayed on BBC KERMIT's command screen.

The command format is

REMOTE DIRECTORY <filespec>

The parameters are:

<filespec>

This  optional  parameter gives a file specification or

other  string  that   refines   a   directory   listing

selection.  If  you  omit  it, the server would usually

display a list of all files in the current directory.

90

BBC KERMIT User Guide

Example:

REMOTE DIRECTORY WORD*.LIS

would ask the server for a listing of the names of all the  files

with names starting "WORD" and with file type "LIS".

REMOTE HELP

This  command  asks  the  server to display some help information

about itself.

The command format is

REMOTE HELP <topic>

The parameters are:

<topic>

This optional parameter specifies a specific  topic  on

which  help is needed: the server may refine its output

to suit, or it may ignore the selection. If you omit it

the  server   would   usually   output   some   general

information.  Unlike  other command parameters, <topic>

consists of the whole of the rest of the  command  line

and so may include spaces.

Example:

REMOTE HELP

asks the server for general help information.

REMOTE HOST

This  command  asks  the  server  to pass a command string to the

mainframe operating system's command interpreter for execution.

The command format is

REMOTE HOST <command>

The parameters are:

<command>

This is the command you wish  to  be  executed.  Unlike

other  command  parameters  it consists of the whole of

the rest of  the  command  line,  and  so  can  contain

spaces.

Some  care  is needed in the choice of command that you ask to be

executed. Since the operating system has no means of carrying  on

91

BBC KERMIT User Guide

a  dialogue  with  you,  commands  that  require  interaction, or

otherwise cannot run in batch mode are unsuitable.

Example:

REMOTE HOST %allow readaccess data.dat

would pass a command "%allow readaccess data.dat" to the server's

operating system for execution.

REMOTE RENAME

This command requests the server to change the name of a file.

The command format is

REMOTE RENAME <oldname> <newname>

The parameters are:

<oldname>

Specifies the file whose name is to be changed.

<newname>

Specifies the new name to be given to the file.

Example:

REMOTE RENAME napoleon snowball

asks the  server  to  change  the  name  of  file  "napoleon"  to

"snowball".

REMOTE SPACE

This  command  asks  the  server  to display details of your disc

usage.

The command format is

REMOTE SPACE <selector>

The parameters are:

<selector>

This optional  parameter  specifies  some  selector  by

which  the  server  refines  its  calculations  of disc

usage.

Example:

REMOTE SPACE

92

BBC KERMIT User Guide

produces a display of the disc space you are using.

REMOTE STATUS

This command requests the server to display details about its own

status.

The command has no parameters.

Example:

REMOTE STATUS

REMOTE TYPE

This command request the server to type a file on the BBC  KERMIT

command screen.

The command format is

REMOTE TYPE <filename>

The parameters are:

<filename>

The  name  of the file you wish to be typed. The server

may permit you to specify wildcards, in which case  the

matching files will be displayed one after the other.

Example:

REMOTE TYPE DATA.LIS

would type the contents of file DATA.LIS.

REMOTE WHO

This  command  requests  the  server  to display details of other

users logged in to the system.

The command has no parameters.

Example:

REMOTE WHO

93

BBC KERMIT User Guide

A1.5: COMMANDS FOR DETAILED PROTOCOL CONTROL

The commands described in this section are used to  exert  detailed

control over the KERMIT protocol. As explained earlier, it is unlikely

that  you  would  ever need to use them. You should consult the KERMIT

Protocol Manual for a detailed  description  of  the  facilities  they

control.

SET EIGHT-BIT-PREFIX

This  command  defines  the  eight-bit-prefix  character that BBC

KERMIT will ask the remote system to agree to use.

The command format is

SET EIGHT-BIT-PREFIX <character>

The parameters are

<character>

The printable character to be used.

By default, BBC KERMIT will attempt to use the "&"  character  as

its eight-bit prefix.

Example

SET EIGHT-BIT-PREFIX %

sets the eight-bit prefix character to be "%".

SET PAUSE

This  command  defines  a "turn-round" delay that BBC KERMIT will

use in packet transmission. It defines the time that  BBC  KERMIT

will  wait,  after  receiving  a  packet  from the remote system,

before transmitting the next to it. This overcomes problems  with

some  systems  that cannot accept input that comes too soon after

output.

The command format is

SET PAUSE <number>

The parameters are

<number>

The pause time in seconds, in the range 0 to 255.

94

BBC KERMIT User Guide

By default, BBC KERMIT does not pause (i.e. the  pause  value  is

0).

Example

SET PAUSE 1

causes  BBC KERMIT to pause for 1 second after receiving a packet

before transmitting one.

SET RECEIVE END-OF-LINE

This command defines the value that BBC KERMIT requests the other

end to use to signal the end of a packet.

The command format is

SET RECEIVE END-OF-LINE <number>

The parameters are

<number>

The number of the control character in the range  0  to

31.

By  default,  BBC  KERMIT  asks for packets to be terminated with

Carriage Return (&0D).

Example

SET RECEIVE END-OF-LINE &0A

requests the remote KERMIT to terminate packets  with  Line  Feed

(code &0A).

SET RECEIVE PACKET-LENGTH

This command defines the maximum packet size that BBC KERMIT will

ask  the  remote  system  to  send.  You  can use this command to

optimise transfers on a noisy communications line: short  packets

will  have  a  higher  chance  of  succesful transmission in such

circumstances.

The command format is

SET RECEIVE PACKET-LENGTH <number>

The parameters are

95

BBC KERMIT User Guide

<number>

The maximum size  of  packet  wanted  from  the  remote

system, in the range 35 to 94.

By  default,  BBC  KERMIT asks the remote system to use a maximum

packet size of 94 bytes.

Example

SET RECEIVE PACKET-LENGTH 60

causes BBC KERMIT to request the remote system to use  a  maximum

packet size of 60 bytes.

SET RECEIVE PADCHAR

This  command  defines the character that BBC KERMIT requests the

other to use when padding before sending a packet.

The command format is

SET RECEIVE PADCHAR <number>

The parameters are

<number>

The numeric code  of  the  character  to  be  used  for

padding in the range 0 to 31.

By default, BBC KERMIT will ask for a pad character of code 0.

Example

SET RECEIVE PADCHAR &07

requests the remote KERMIT to pad with &07 characters.

SET RECEIVE PADDING

This command defines the number of pad characters that BBC KERMIT

will ask the other end to send before each packet.

The command format is

SET RECEIVE PADDING <number>

The parameters are

<number>

The  number  of  pad characters to be requested, in the

96

BBC KERMIT User Guide

range 0 to 94.

By default, BBC KERMIT asks for no pad characters to be sent.

Example

SET RECEIVE PADDING 10

will cause BBC KERMIT to request the  remote  system  to  precede

each packet with 10 pad characters.

SET RECEIVE START-OF-PACKET

This  command defines to BBC KERMIT the character that the remote

system will use to indicate the start of each packet.

The command format is

SET RECEIVE START-OF-PACKET <number>

The parameters are

<number>

The numeric code of the  character  the  remote  system

will use, in the range 0 to 31.

By  default,  BBC KERMIT will expect the remote system to precede

each packet with a &01 character.

Example

SET RECEIVE START-OF-PACKET &03

tells BBC KERMIT that the remote system will precede each  packet

with a &03 character.

SET RETRIES

This  command  defines the number of times BBC KERMIT will repeat

an attempt to transmit a packet to, or receive a packet from, the

remote system. You may need to increase the  number  of  attempts

allowed if you are using a noisy communications line.

The command format is

SET RETRIES <number>

The parameters are:

<number>

The  number  of  retries  to be made, in the range 1 to

255.

97

BBC KERMIT User Guide

By default, BBC KERMIT will make 5 retry attempts (i.e.  a  total

of  6  attempts  in  all) before deciding that a packet cannot be

transmitted  or  received  correctly  and  abandoning  the   file

transfer.

Example:

SET RETRIES 16

will cause BBC KERMIT to make 16 retry attempts before abandoning

the transfer.

SET SEND END-OF-LINE

This  command  defines  the character that BBC KERMIT will use to

terminate the first packet it sends to the remote system in  each

transfer.  Subsequent packets will be terminated by the character

that the remote system requests when  it  tells  BBC  KERMIT  its

requirements.

The command format is

SET SEND END-OF-LINE <number>

The parameters are

<number>

The  numeric  code  of the character to be used, in the

range 0 to 31.

By default, BBC KERMIT uses &0D (Carriage Return).

Example

SET SEND END-OF-LINE &0A

causes the first packet sent in each transfer to  be  ended  with

Line Feed (code &0A).

SET SEND PADCHAR

This  command defines the padding character with which BBC KERMIT

will  precede  the  first  packet  to  be  sent  in  a  transfer.

Subsequent packets will be preceded by the character requested by

the remote system in its SEND-INIT packet.

The command format is

SET SEND PADCHAR <number>

98

BBC KERMIT User Guide

The parameters are

<number>

The  numeric  code  of the character to be used, in the

range 0 to 255.

By default, BBC KERMIT uses a pad character of NUL (code &00).

Example

SET SEND PADCHAR &03

sets the pad character used to be &03.

SET SEND PADDING

This command defines the number of pad characters BBC KERMIT will

send before the first packet in a  transfer.  Subsequent  packets

will  be  preceded  by  the number requested by the remote system

when it tells BBC KERMIT its requirements.

The command format is

SET SEND PADDING <number>

The parameters are

<number>

The number of pad characters to be sent, in the range 0

to 255.

By default, BBC KERMIT sends no pad characters.

Example

SET SEND PADDING 21

causes BBC KERMIT to send 21  pad  characters  before  its  first

packet.

SET SEND QUOTE

This  command  defines  the character that BBC KERMIT will use to

prefix control characters in data packets that  it  sends.  Since

every  occurence  of the quote character in a file must itself be

preceded by a quote  when  transmitted,  this  command  lets  you

minimise the data to be moved if the file contains a large number

of the normal quote character.

The command format is

SET SEND QUOTE <character>

The parameters are

99

BBC KERMIT User Guide

<character>

The printable character to be used.

By default, BBC KERMIT uses the "#" character.

Example

SET SEND QUOTE $

causes  BBC  KERMIT to prefix control characters it sends in data

packets with a "$".

SET SEND START-OF-PACKET

This command defines the character that BBC KERMIT will  send  to

indicate the start of every packet.

The command format is

SET SEND START-OF-PACKET <number>

The parameters are

<number>

The  numeric  code  of the character to be sent, in the

range 0 to 31.

By default, BBC KERMIT sends a &01 character in  front  of  every

packet.

Example

SET SEND START-OF-PACKET &04

causes  BBC  KERMIT  to  precede every packet it sends with a &04

character.

100

BBC KERMIT User Guide

APPENDIX 2 : OBTAINING, BUILDING AND MODIFYING BBC KERMIT

This appendix describes how you can get  hold  of  BBC  KERMIT  and

build  it  from  the  assembly language sources, or modify an existing

version.

101

BBC KERMIT User Guide

A2.1 OBTAINING BBC KERMIT

There are several ways of acquiring a BBC KERMIT, as follows:

1. You can obtain the files from the KERMIT tapes  distributed  by

Columbia University in New York, or by anonymous FTP from their

CU20B system.

2. You  can obtain the files from Lancaster KERMIT distribution by

file transfer over the JANET or PSS networks, by  using  KERMIT

itself, or on magnetic tape. The files are held online on a VAX

11/780 running VMS and can be freely accessed by anyone.

3. You  can  obtain  the  files  on  Acorn  DFS  format discs from

Lancaster KERMIT distribution: the  sources  and  the  compiled

binaries  will  need  1  double-sided  80  track drive, and the

documentation (in Wordwise Plus and printable format)  needs  2

more.

If  you  are  within  the  UK  you  should use option 2 or 3 above:

Columbia will refer  all  KERMIT  supply  enquiries  from  the  UK  to

Lancaster in the first instance. Also, Lancaster will generally have a

more up-to-date release of BBC KERMIT than is available from Columbia.

Options  2  and 3 will get you the source files exactly as you need

them for the ADE assembler. For reasons of space, though, the  sources

are  distributed  by Columbia as one monolithic file: you will need to

process this file with an editor on your mainframe to  split  it  into

its  components at the clearly marked points before moving it to a BBC

machine.

However you get the files,  you  will  eventually  have  a  set  of

assembler  source  files  from  which  you  need  to generate compiled

binaries. If you get BBC KERMIT on disc  from  Lancaster  distribution

you  will  find  binary  files  already  there,  called ROMnnn for the

sideways ROM version and RAMnnn for the RAM version (where nnn is  the

mark  and  version  number).  Otherwise  you  will  find  files called

BBCROM.HEX and BBCRAM.HEX, which are Intel hex dumps of  the  compiled

binaries.  You  should  get  these onto a BBC disc, then run the small

BASIC program BBCDEHEX.BAS to convert them to binary form.

The binary of the RAM version can be *RUN immediately: that for the

sideways ROM version should either be  loaded  into  sideways  RAM  at

address  &8000  or  programmed into a 16K 27128 EPROM. For those sites

not  possessing  EPROM  programming   facilities,   Lancaster   KERMIT

distribution is willing to program a small number of EPROMs for anyone

who supplies them.

A2.1.1 The source files

The BBC KERMIT assembler source is held in a fair number of files

102

BBC KERMIT User Guide

and is now very large (and still growing). The source is split by

functionality, and further divided to keep the files small enough

to  permit  easy and quick editing (we have taken about 800 lines

as the limit for a file). The organisation  is  not  fixed  -  we

shall refine it with experience and as the system changes, so the

names  are  not  guaranteed  to  be unchanged from one release to

another.

For version 1.40 the files  that  make  up  BBC  KERMIT  are  as

follows:

a. Files supplied on disc

BBC  KERMIT  as issued on disc consists of two sets of files. The

first set consists of the assembler source files and two compiled

binary files: the files are

BBCMIT    This is the first file to be assembled, containing

compilation switches and messages.

BBCDEF    Various global defines.

BBCWKS    Workspace declarations

BBCMAI    The entry code to the system.

BBCCPR    The command processor.

BBCCMD    Command routines  for  general  commands  such  as

TAKE, SEND, etc.

BBCGEN    Command  routines  for  generic  functions such as

BYE, REMOTE etc.

BBCST1    The SET command part 1.

BBCST2    The SET command part 2.

BBCSHW    The SHOW command.

BBCREC    Code for receiving files.

BBCSND    Code for sending files.

BBCCOM    Communications handling code.

BBCTM1    Terminal emulation part 1.

BBCTM2    Terminal emulation part 2.

BBCPM1    Packet handling code part 1.

BBCPM2    Packet handling code part 2.

BBCPM3    Packet handling code part 3.

BBCFIL    File handling code.

BBCSB1    General subroutines.

BBCSB2    General subroutines.

BBCSB3    General subroutines.

BBCSB4    General subroutines.

BBCTX1    Texts for messages part 1.

BBCTX2    Texts for messages part 2.

ROMnnn    The compiled binary  file  for  the  sideways  ROM

version.  Here  "nnn"  gives  the mark and version

number.

RAMnnn    The compiled binary file for the  version  running

in RAM.

A.Assem   A  compiled binary file for a 65C02 assembler that

you can use as an alternative to ADE.

103

BBC KERMIT User Guide

A.ReadMe  Notes  on  the  65C02  assembler,  describing  its

differences from ADE.

The  second  set  of the files is that comprising the User Guide.

This is supplied as Wordwise Plus source files, and as a spooled,

printable file.

b. Files on the Lancaster VAX filestore

The VAX filestore contains most of the above  files  apart  from

the  WordWise Plus source files for the User Guide. The assembler

source files have the same names as those on the discs, but  each

filename has a suffix ".ADE" added to it.

The other files are:

BBCKERMIT.DOC A  printable  copy  of  the  user  guide. This

contains no printer  control  characters,  and

uses 66 lines on each printed page.

BBCKERMIT.BWR An up-to-date list of known bugs and problems.

(Note   that  this  file  may  not  always  be

present)

BBCROM.HEX    The compiled sideways ROM version in Intel hex

format.

BBCRAM.HEX    The compiled RAM version in Intel hex format.

BBCHEX.BAS    The BASIC program used to convert  the  binary

files to Intel hex.

BBCDEHEX.BAS  A  BASIC  program to convert from Intel hex to

binary. Note that the  version  supplied  with

BBC  KERMIT  release  1.02 is not suitable for

use with release 1.30 and later versions.

BBCASM.HEX    An Intel hex form of the 65C02 assembler  that

you can use as an alternative to ADE.

BBCASM.DOC    Notes on the 65C02 assembler.

c. Files on tapes from Columbia

Here  the  files  consist of one large source file BBCKERMIT.ADE,

containing a concatenation of  the  separate  sources,  plus  the

additional  files  listed  above in (b). The Wordwise Plus format

files for the User Guide are not available from Columbia.

104

BBC KERMIT User Guide

A2.2 BUILDING BBC KERMIT FROM A HEX FILE

The easiest way to obtain a binary version  of  BBC  KERMIT,  apart

from  getting  one on a disc from Lancaster KERMIT distribution, is to

generate it from one of the Intel hex format files.

The hex files will be found on  Columbia  tapes  and  in  Lancaster

filestore  as  BBCROM.HEX  and  BBCRAM.HEX.  They  are  in  completely

standard format, so can be converted with any utility you may  already

have.  However  a  small BASIC program is provided to do this for you:

this is in file BBCDEHEX.BAS, and is also listed below.

You will need to get the Intel hex files you require to use onto  a

BBC disc in some way to start with. An ordinary terminal emulator with

dumb  capture  facility  will suffice for this. You will also need the

BBCDEHEX.BAS program: you can capture this in the same way, or,  since

it  is  short, type it in from the listing. Note that earlier releases

of this program should not be used with BBC KERMIT  version  1.30  and

later.  If  you capture the program, say into a file called DEHEX, you

will need to convert it from text form into a BASIC program in memory.

You can do this with the commands

*EXEC DEHEX

SAVE"BDEHEX"

for example.

Once the program is ready you can simply RUN it. It will ask you to

name the input and output files to use, then will do all the necessary

conversion work.

105

BBC KERMIT User Guide

A2.3 BUILDING BBC KERMIT FROM SOURCE

Generating BBC KERMIT from the assembly sources is  a  fairly  easy

process.  You  will  need  to arrange the source files on the discs to

match the CHN directives, as described below.

BBC KERMIT  was  developed  using  the  ADE  assembler/editor  ROM,

available  commercially (see Appendix 2 section 6 for the address). An

alternative to ADE is now available: a 65C02 macro  assembler  written

at  Lancaster  is  distributed  with  BBC  KERMIT. This assembler runs

either in a 27128 EPROM or  in  sideways  RAM,  and  is  substantially

faster  than ADE. From version 1.42 onwards the sources are compatible

with both assemblers.

A2.3.1 Source file layout

To  compile  BBC  KERMIT  you  will  need  either  2   80   track

single-sided  units,  or  1 80 track double-sided unit if you are

using DFS, or an Econet file server.

The source files are divided into 2 groups, reflecting  how  they

can  be  held  on 80 track BBC discs. Files from BBCMIT to BBCSND

inclusive should be held on one disc  surface.  Files  BBCCOM  to

BBCTX2 can be held on either the reverse side of this disc, or on

side  1  of  another  disc. Questions output from the BBCMIT file

during assembly allow the system to configure itself to point  to

the  right  drives:  you should reply "N" to the initial question

"All files on one disc surface?"

There should be space for the object file to  be  placed  on  the

first of the two surfaces.

Note  that  if  you are using an Econet file server you can place

all the files on one disc: in this case you should reply  "Y"  to

the question "All files on one disc surface?".

A2.3.2 The assembly process

Having  arranged  the files on your BBC discs as described above,

you can compile them very simply.

a. Using ADE

To use the ADE assembler, simply type the command

106

BBC KERMIT User Guide

*ASM BBCMIT KERMIT

Precede this command with CTRL-B if you wish  to  produce  a

listing on a printer.

The  file  BBCMIT  will  output  some  questions  to  you to

determine what it is to do. The first question is whether or

not you are using the ADE  assembler:  reply  "Y"  to  this.

Futher  questions  will  be  on the disc layout, whether you

wish to build the sideways  ROM  or  the  RAM  version,  and

whether  you  wish  a listing to be output, then will do the

assembly.

b. Using the Lancaster 65C02 assembler

To use the Lancaster  65C02  assembler,  you  will  need  to

either  program  the code into a 27128 EPROM or load it into

sideways RAM. Once there, you can compile BBC KERMIT.

To simply check the syntax of the sources, without producing

an object code file, type the command

*ASSEMBLE BBCMIT -A

File BBCMIT will ask you if you are using ADE: reply "N"  to

this.  It then asks you the same questions on disc layout as

described above.

To produce object code, the command  you  use  becomes,  for

example

*ASSEMBLE BBCMIT -A -OKERMIT

where the output code file here is called KERMIT.

Both the commands shown above produce listings to the screen

only.  To list to a printer, you should add a "-P" option to

the command: thus, for example

*ASSEMBLE BBCMIT -A -OKERMIT -P

produces object code in file KERMIT and  a  listing  to  the

printer.

Note  that  the  "-A"  option  is essential  if  you use the

Lancaster 65C02 assembler for BBC KERMIT.

Whichever assembler you use, the  listing  is  configured  to  be

produced  on  an  EPSON  FX80 or compatible printer, and switches

itself into 8 lines per  inch,  condensed  print  mode.  If  your

printer is not EPSON-compatible, you will need to change the PAGE

and SYSVDU commands in file BBCMIT to suit your requirements.

The  output  listing from ADE will need up to 132 characters on a

107

BBC KERMIT User Guide

line.  On  the  Lancaster  65C02  assembler  the  line  width  is

configurable:  if your printer only has 80 byte lines then change

the PAGE directive in file BBCMIT to read

PAGE   88,80

The listing currently takes over 2 hours to  print  on  an  EPSON

FX80, and occupies over 200 pages.

If  you  wish,  you  can  list only some of the source files in a

compilation. File BBCMIT contains a number of  boolean  switches,

one for each source file, that dictate whether that file is to be

listed if "selective listing" is specified. You should change the

switches  for the files you wish to list, then specify "selective

listing"  to  the  question  that  is  output  at  the  start  of

compilation.

108

BBC KERMIT User Guide

A2.4 CHANGING KERMIT DEFAULTS

There  are  several ways to arrange for BBC KERMIT to use different

control values. The easiest is to leave the code as it is, and use the

LOAD, SAVE or TAKE commands to reset the values as you want them  when

you  start up. You can, of course, change the source and recompile: or

you can leave the source alone and patch the object code.

BBC KERMIT maintains the current control values in a RAM table, and

the SET command manipulates the values therein. On start up, or  using

the  RESET  command,  KERMIT  copies  a block of defaults into the RAM

table. The SAVE command writes the RAM table to a file, and  the  LOAD

command reloads from a file into the table.

A2.4.1 Changing the source

You  can  change  the defaults by altering the values held in the

main defaults table. This is held in the source file BBCMAI,  and

is  clearly  marked.  Each  value  is  commented  to  show  which

parameter it defines: for possible values see below.

A2.4.2 Patching the object code

If you do not wish to recompile, you can load the object file  to

memory  and  patch  it  with a machine-code monitor such as EXMON

before  programming  another  EPROM  or  saving   it   to   disc.

Alternatively,  you  can use a default setting program written in

BASIC by Dave Walker of University  College  Cardiff  to  do  the

changes: this is available on disc in file B.Patcher, or from the

Lancaster  distribution  machine  in  file  BBCPATCHER.BAS.  This

program handles the process automatically, so there is no need to

consult the following section.

The defaults block is not in a fixed place in the program image,

but the code contains pointers to let you locate it. You can find

the base of the defaults table in the following way:

a. The sideways ROM version

1. Look at byte $8007. This gives an offset  to  the  copyright

text  in  the  ROM header. Add $8000 to the contents to form

the address of the zero byte preceding the string.

2. Follow to the end of the copyright string  (which  currently

109

BBC KERMIT User Guide

reads "(C) CUCCA 1986") and find the terminating zero byte.

3. The succeeding bytes are then as follows:

Byte 0    The mark number of this release.

1    The version number.

2-3  The  address  (low-high)  of the first byte of the

defaults table in the ROM.

4-5  The address (low-high) of the first unused byte in

the ROM.

b. The RAM version

The first bytes of the program are as follows

Byte 0-2  A JMP instruction to the first executable byte  of

the image.

3    The mark number of this release.

4    The version number.

5-6  The  address  (low-high)  of the first byte of the

defaults table in the program.

7-8  The address (low-high) of the first unused byte in

the program.

Note that if you wish to add patch code to the program,  you

should place it after the last byte of the image, then alter

the  address  held  in bytes 7-8 to be that of the next byte

after your added code. Failure to change this  address  will

result    in   your   patch   being   overwritten   by   the

dynamically-allocated file buffer when the program runs.

A2.4.3 Format of the defaults block

The data in the defaults block is defined as follows:

Byte 0     The transmit baud rate. This should be set to a number

in the range 1..8, where the value is as for the *FX 7

command. (SET BAUD n TRANSMIT)

1     The receive baud rate. This should be set to a  number

in the range 1..8, where the value is as for the *FX 8

command. (SET BAUD n RECEIVE)

2     The  local  echo  flag,  set to 0 for off or 1 for on.

(SET LOCAL-ECHO)

3     No longer used.

4     The debug flag, set to 0 for off or  1  for  on.  (SET

DEBUGGING)

110

BBC KERMIT User Guide

5     No longer used.

6     The timeout value BBC KERMIT will ask the other system

to use, in the range 0..94. (SET SEND TIMEOUT)

7     The  timeout  value  BBC KERMIT will use on the remote

system (if TIMER is on)  in  the  range  0..255.  (SET

RECEIVE TIMEOUT)

8     The  TIMER  flag,  set  to 0 for off or 1 for on. (SET

TIMER)

9     The pause BBC KERMIT uses on packet turnround, in  the

range 0..255 (SET PAUSE)

10    The  delay  after  the  SEND command is issued, in the

range 0..255 (SET DELAY)

11    No longer used.

12    No longer used.

13    The terminal mode keyclick flag, set to 0 for off or 1

for on. (SET KEY-CLICK)

14    The quote character BBC KERMIT will send. (SET QUOTE)

15    The  eight-bit  prefix  BBC  KERMIT   will   use   for

preference. (SET EIGHT-BIT-PREFIX)

16    The  end-of-line  byte  that  BBC  KERMIT uses for the

first packet it sends. (SET SEND END-OF-LINE)

17    The end-of-line byte the remote system will use.  (SET

RECEIVE END-OF-LINE)

18    The  pad  character  BBC  KERMIT  will send. (SET SEND

PADCHAR)

19    The number of pad characters  BBC  KERMIT  will  send.

(SET SEND PADDING)

20    The  pad  character  BBC  KERMIT  will  ask the remote

system to send. (SET RECEIVE PADCHAR)

21    The number of pad characters BBC KERMIT will  ask  the

other end to send. (SET RECEIVE PADDING)

22    The current parity setting. This should be one of:

0 MARK

1 SPACE

2 EVEN

3 ODD

4 NONE

23    The current file type. This should be 0 for ASCII or 1

111

BBC KERMIT User Guide

for BINARY. (SET FILE TYPE)

24    The  current  end-of-record  type used for ASCII files

(SET FILE TYPE ASCII). The value should be one of:

0 LFCR

1 CRLF

2 LF

3 CR

25    The current source of data. The value is 0 for FILE or

1 for MEMORY. (SET SOURCE)

26-27 The  base  address  for   the   source   memory   area

(low-high).  This  must  be  set if byte 25 is 1. (SET

SOURCE MEMORY)

28-29 The top address for the  source  memory  area  plus  1

(low-high).  This  must  be  set if byte 25 is 1. (SET

SOURCE MEMORY)

30    The current destination for  data  (SET  DESTINATION).

Values are

0 FILE

1 MEMORY

2 PRINTER

31-32 The  base  address  for  the  destination  memory area

(low-high). This must be set if byte  30  is  1.  (SET

DESTINATION MEMORY)

33-34 The  highest  address  for the destination memory area

(low-high). The value is normally the base of the MODE

7 screen area $7BFF, and must be set if byte 30  is  1

(SET DESTINATION MEMORY).

35    The  start  of  packet  character that BBC KERMIT will

send. (SET SEND START-OF-PACKET)

36    The start of packet character that the  remote  system

will send. (SET RECEIVE START-OF-PACKET)

37    The  maximum  packet  size  BBC KERMIT asks the remote

system to send. (SET RECEIVE PACKET-LENGTH)

38    The size of the file buffer area used  in  pages.  the

value   must   be   in  the  range  1..64.  (SET  FILE

BUFFER-SIZE)

39    The file warning flag, set to 0 for off or 1  for  on.

(SET FILE WARNING)

40    The  incomplete  file  disposition  flag, set to 0 for

DELETE or 1 for KEEP. (SET INCOMPLETE)

41    The current flow control method, set to 0 for XON/XOFF

112

BBC KERMIT User Guide

or 1 for CTS/RTS. (SET FLOW-CONTROL)

42    The significant size of the file suffix string defined

in the next field. The value is 1..8 if  a  suffix  is

wanted, or 0 if none is to be added. (SET FILE SUFFIX)

43-50 The  suffix  added  to a BBC name to generate a remote

file name. The number of characters used is defined by

byte 42. Note that the suffix does not include the "."

character. (SET FILE SUFFIX).

51    The TAKE-echo flag, set to 0 for off or 1 for on. (SET

TAKE-ECHO)

52    The colour  to  be  used  for  text  in  80  character

terminal  mode.  Colours  available  are  red,  green,

yellow, blue, magenta, cyan and  white,  corresponding

to values 1..7 (SET VDU-COLOUR)

53    *TV  flag, set to 1 if a *TV is to be done on entry to

terminal mode and 0 if it is not. (SET TV)

54-55 The two *TV parameters to be used if  byte  53  is  1.

(SET TV)

56    VDU  ignore  flag,  set to 1 if an ignore character is

defined, and 0 if it is not. (SET IGNORE)

57    The incoming terminal-mode character to be ignored  if

byte 56 is 1. (SET IGNORE)

58    The  number  of  byte  periods  to wait before testing

clear-to-send  when  transmitting  a  character.  (SET

MUX-WAIT)

59    The  character that triggers a help display in command

mode. (SET HELP-TRIGGER)

60    The maximum number of retries BBC KERMIT will make  in

an attempt to send a packet (SET RETRIES)

61    The  handshake  type in use (SET HANDSHAKE). The value

should be one of

0 NONE

1 BELL

2 CR

3 LF

4 XON

5 XOFF

62    The terminal emulation type to be used (SET TERMINAL).

The values currently allocated are:

0 TT40

1 TT80

2 VT52

113

BBC KERMIT User Guide

3 VT100

4 TEK

5 Un-named (A)

6 Un-named (B)

If the value is set to $FF, BBC Kermit  will  use  the

default appropriate to the machine it is running in.

63    Controls  how  filenames sent by the remote system are

handled (SET FILE NAME). Values allowed are:

0 Translate name from name.type format

1 Use name exactly as specified

The table format is fixed, and will not be changed  by  Lancaster

in later releases. Additional facilities will be added to the end

of  the  block  only. Note that it is up to you to make sure that

the values you put in the default  block  make  sense:  what  BBC

KERMIT  does  with  inconsistent  ones is unlikely to be anything

you'ld like it to.

114

BBC KERMIT User Guide

A2.5 THE HEX TO BINARY CONVERTER PROGRAM

The Intel hex forms of the compiled BBC KERMIT and the  Lancaster

65C02 assembler can be converted into binary files using a simple

converter  supplied as the file BBCDEHEX.BAS. This can be brought

on  to  a  BBC  disc  and  *EXEC'd  to  give  a  BASIC   program:

alternatively,  since  the  program  is short, it can be typed in

directly. For convenience, a listing of the program  is  supplied

below.

100 REM Intel hex to binary converter

120 REM Version 2 November 1985

140

160 CLOSE#0

180 DIM image 20480

200

220 INPUT "Name of input hex file?    " in$

240 INPUT "Name of output binary file? " out$

260 in%=OPENIN(in$)

280 IF in%=0 THEN PRINT"Input file does not exist":END

300

320 imagesize% = 0

340 record% = 0

360 REPEAT

380   REPEAT

400     UNTIL (BGET#in%=ASC(":")) OR EOF#in%

420   IF EOF#in% THEN PROCsave:CLOSE#0:END ELSE PROCreadrecord

440   UNTIL FALSE

460

480 END

500

520

540 DEF PROCreadrecord

560 LOCAL count%,address%,storedcsum%,type%

580 record%=record%+1

600 csum%=0

620 count%=FNgetbyte

640 address%=FNgetbyte*256+FNgetbyte

660 IF record%=1 THEN base% = address%

680 PRINT"Record ";record%;" : Size ";count%;", address

$";~address%

700

720 type% = FNgetbyte

740 IF type%<>0 AND type%<>1 THEN PRINT"*** Unknown record type

";type%;" detected":CLOSE#0:END

760 IF type% <> 0 THEN PRINT"+++ End-of-file record

detected":ENDPROC

780 PROCreaddata(count%)

800 storedcsum% = FNgetbyte

820 IF csum%<>0 THEN PRINT"*** Checksum error":CLOSE#0:END

840 ENDPROC

860

880 DEF FNgetbyte

900 LOCAL value%

115

BBC KERMIT User Guide

920 value%=FNgetnibble*16 + FNgetnibble

940 csum% = (csum% + value%) AND &FF

960 =value%

980

1000 DEF FNgetnibble

1020 LOCAL char%

1040 char% = BGET#in%

1060 IF char%>=ASC("A") THEN =char%-ASC("A")+10 ELSE

=char%-ASC("0")

1080

1100 DEF PROCreaddata(count%)

1120 LOCAL I%

1140 pointer% = address% - base%

1160 FOR I% = 1 TO count%

1180   image?pointer% = FNgetbyte

1200   address% = address% + 1

1220   pointer% = pointer% + 1

1240   NEXT

1260 IF pointer% > imagesize% THEN imagesize% = pointer%

1280 ENDPROC

1300

1320 DEF PROCsave

1340 loadaddress% = base% OR &FFFF0000

1360 OSCLI("*SAVE

"+out$+" "+STR$~image+" "+STR$~(imagesize%+image)+" "+STR$~loadad

dress%+" "+STR$~loadaddress%)

1380 ENDPROC

116

BBC KERMIT User Guide

A2.6 CONTACT ADDRESSES

If you wish to use the ADE assembler/editor, you can obtain it from

SYSTEM

12 Collegiate Crescent

Sheffield

UK

See  adverts in Acorn User for the current price. Currently BBC KERMIT

is compiled using version 2.02. ADE will continue to  be  the  primary

assembly  method  used  for  BBC  KERMIT,  but  all releases from 1.42

onwards will be also be fully  compatible  with  the  Lancaster  65C02

assembler  supplied  with  it. The 65C02 assembler does not contain an

editor, but you  can  use  any  word  processor  or  other  editor  to

manipulate the source files.

If  you  wish  advice,  or  have any bug reports or comments on BBC

KERMIT, you can contact me at Lancaster University in several ways.

Postal address

Alan Phillips

Communications Group

Department of Computing

Computer Building

Lancaster University

Lancaster UK

Telephone

0524-65201 x 4881

Electronic mail

SYSKERMIT @ LANCS.VAX1

JANET address 000010404000.FTP.MAIL

PSS address   234252400101.000010404000.FTP.MAIL

Contact by electronic mail is  the  preferred  method  if  you  can

manage it: please remember to specify your name, where you are mailing

from, and the full mail address and NRS name (if you have one) for the

reply.  I  can  reply only with difficulty and no guarantee of success

over ARPA, as the gateway I have to use is of low reliability: I  have

no  authorisation  for  uucp  so  cannot reply at all by this route. A

JANET address is preferred if you can provide one. I will  attempt  to

reply  to  all  enquiries,  and  will  usually do so within one or two

weeks: if you don't hear from me by then, you should assume that I  am

unable  to  get  mail to you for some reason and contact me by another

route.

I will be pleased to supply  details  of  our  KERMIT  distribution

service  on  request:  we  have a complete collection of all available

117

BBC KERMIT User Guide

KERMIT implementations, and can distribute  these  on  magnetic  tape,

over the JANET or PSS networks, or, for some versions, on floppy disc.

Distribution   is   free   to  all  educational  or  research  council

institutions: commercial customers are asked to pay a  small  handling

charge.  The  increasing  use  of  this  service  means that we cannot

currently undertake distribution to new customers outside  the  UK  or

Eire.

118

BBC KERMIT User Guide

APPENDIX 3 : USING THE EDT EDITOR ON VAX/VMS

If  you are using the sideways ROM version of BBC KERMIT on a model

B, or either version on a B+, B+128 or Master 128, you will be able to

use the EDT screen editor on VAX/VMS mainframes to  edit  your  files.

This Appendix describes how you should set up your job to do this, and

how the BBC Computer's function keys are used to control the editor.

If  you are using the RAM version of BBC KERMIT on a model B, which

emulates only a 40 character wide teletype, you will still be able  to

use EDT, but only in line mode.

119

BBC KERMIT User Guide

A3.1 SETTING UP THE TERMINAL DETAILS

In  order to use the full facilities of the EDT editor you must, of

course, set BBC KERMIT to provide a VT52 terminal emulation.  This  is

the default setting, but if you have changed the terminal type you can

return to VT52 mode with the command

SET TERMINAL VT52

before you use the CONNECT command to enter terminal mode.

Depending  on  how the VAX/VMS system has been set up, you may also

need to configure it to know that your terminal is a VT52 and not, for

instance, a VT100. You can see what VMS thinks  your  terminal  is  by

issuing the DCL command

$ SHOW TERMINAL

during a job. If the default terminal type is a VT52, you need take no

further action: EDT will automatically adapt itself to use the correct

screen control commands.

If,  though, the default terminal type is not a VT52, you will need

to take some action. You can do one of several things here:

1. If you are  connected  to  your  VAX  system  with  a  directly

connected  permanent line, you could ask your system manager to

set the terminal characteristics that VMS will use.  This  will

be permanent, and once it is done the default terminal type for

that line will be a VT52. This will not, though, be possible if

you  are  using  some  sort  of  network to link you to the VAX

system.

2. Whenever you start a terminal job, you could issue the command

$ SET TERMINAL/DEVICE=VT52

to tell VMS that you are using a VT52. You must do  this before

you  use  the  EDT editor (or anything else, such as MAIL, that

sends screen-control characters). You could include the command

in your LOGIN.COM file to save you having to remember to  issue

it every time: however, the terminal type will then be wrong if

you ever use a device that is not a VT52.

3. You could include in your LOGIN.COM file the command

$ SET TERM/INQUIRE

This  will  make VMS send an "inquiry sequence" to the terminal

when you log in: BBC KERMIT will  then  identify  itself  as  a

VT52,  and VMS will set itself up accordingly. This is probably

the preferable method, since it will adapt itself automatically

whatver device type you use. However, there are some drawbacks.

If you use a device  that  does  not  respond  to  the  inquiry

sequence  you will see some rubbish charaters appear, and login

120

BBC KERMIT User Guide

may be slowed down slightly: also, any commands you type  while

waiting  for  the login sequence to complete will be discarded.

There is also the problem that  some  peculiar  terminals  (ICL

terminals  are  apparently  bad here) may interpret the inquiry

sequence as something  else  entirely,  and  may  do  something

strange or even stop working altogether.

121

BBC KERMIT User Guide

A3.2 EDIT KEYPAD KEYS

As we saw in section 4.4, the models B, B+ and B+128 do not possess

keys  corresponding  to  the  VT52 keypad, so BBC KERMIT emulates them

using the red function keys. Section 4.4 specified the  actual  ESCAPE

sequences  that  the keys will send to the host computer in VT52 mode:

here we shall consider the use of the keys in terms of the EDT editor.

If you have a Master 128 you will be able to use the  keypad  keys  to

send the ESCAPE sequences.

A3.2.1 Models B, B+ and B+128

These  machines  do not have keypads, so you will need to use the

red function keys to send the ESCAPE sequences to EDT. Note  that

some editor functions are invoked by pressing a sequence of keys:

in the list below

F3

indicates that you should press key F3, and

SHIFT-F1/F3

indicates that you should press SHIFT-F1 (the GOLD function) then

F3.

BBC key sequence         VT52 keypad key     EDT function

SHIFT-F1                 PF1                 GOLD

SHIFT-F2                 PF2                 HELP

SHIFT-F3                 PF3                 DELETE LINE

SHIFT-F1/SHIFT-F3        GOLD/PF1            UNDELETE LINE

SHIFT-F7                 PERIOD              SELECT

SHIFT-F1/SHIFT-F7        GOLD/PERIOD         RESET

SHIFT-F8                 ENTER               ENTER

SHIFT-F1/SHIFT-F8        GOLD/ENTER          SUBS

F0                       0                   LINE

SHIFT-F1/F0              GOLD/O              OPEN LINE

F1                       1                   WORD

SHFT-F1/F1               GOLD/1              CHANGE CASE

F2                       2                   EOL

SHIFT-F1/F2              GOLD/2              DELETE EOL

F3                       3                   CUT

SHIFT-F1/F3              GOLD/3              PASTE

122

BBC KERMIT User Guide

F4                       4                   ADVANCE

SHIFT-F1/F4              GOLD/F1             BOTTOM

F5                       5                   BACKUP

SHIFT-F1/F5              GOLD/5              TOP

F6                       6                   DELETE CHAR

SHIFT-F1/F6              GOLD/F6             UNDELETE CHAR

F7                       7                   PAGE

SHIFT-F1/F7              GOLD/7              COMMAND

F8                       8                   FIND NEXT

SHIFT-F1/F8              GOLD/8              FIND

F9                       9                   DELETE WORD

SHIFT-F1/F9              GOLD/9              UNDELETE WORD

UP                       UP                  CURSOR-UP

SHIFT-F1/UP              GOLD/UP             REPLACE

DOWN                     DOWN                CURSOR-DOWN

SHIFT-F1/DOWN            GOLD/DOWN           SECT

RIGHT                    RIGHT               CURSOR-RIGHT

SHIFT-F1/RIGHT           GOLD/RIGHT          SPECINS

LEFT                     LEFT                CURSOR-LEFT

SHIFT-F1/LEFT            GOLD/LEFT           APPEND

A3.3.2 The Master 128

This  machine  possesses  a  keypad  to  the  right  of  the main

keyboard, and the arrangement of  these  keys  closely  parallels

what  you  would  find  on  a  real terminal. The key arrangement

differs most in that the cursor keys on a real VT52  are  in  the

keypad area: on the Master they are separate, and the keypad keys

that occupy these positions are not used.

The  keypad  sends  EDT  control sequences as follows. If the key

diagram shows  two  functions,  the  upper  one  is  selected  by

pressing  the  key  alone,  and  the  lower  is obtained by first

pressing GOLD, then pressing the key.

123

BBC KERMIT User Guide

---------------------------------

!       !       !       !       !

! GOLD  ! HELP  ! DEL L !       !

!       !       ! UND L !       !

!       !       !       !       !

---------------------------------

!       !       !       !       !

! PAGE  ! FNDNXT! DEL W !       !

! COMND ! FIND  ! UND W !       !

!       !       !       !       !

---------------------------------

!       !       !       !       !

! ADV   ! BACKUP! DEL C !       !

! BOTTOM! TOP   ! UND C !       !

!       !       !       !       !

---------------------------------

!       !       !       !       !

! WORD  ! EOL   ! CUT   !       !

! C CASE! D EOL ! PASTE !       !

!       !       !       !       !

---------------------------------

!       !       !               !

! LINE  ! SELECT!    ENTER      !

! OPN L ! RESET !    SUBS       !

!       !       !               !

---------------------------------

124