HH     HH  PPPPPPPP    8888888    6666666

HH     HH  PP     PP  88     88  66

HH     HH  PP     PP  88     88  66

HHHHHHHHH  PPPPPPPP    8888888   66666666

HH     HH  PP         88     88  66     66

HH     HH  PP         88     88  66     66

HH     HH  PP          8888888    6666666

KK    KKK  EEEEEEEE  RRRRRRRR    MMM     MMM  IIIIII  TTTTTTTTTT

KK   KKK   EE        RR     RR   MMMM   MMMM    II        TT

KK  KKK    EE        RR     RR   MM MM MM MM    II        TT

KKKKK      EEEEEEE   RRRRRRRR    MM  MMM  MM    II        TT

KKKKKK     EE        RR RRR      MM   M   MM    II        TT

KK  KKK    EE        RR   RRR    MM       MM    II        TT

KK    KKK  EEEEEEEE  RR     RRR  MM       MM  IIIIII      TT

====================================================================

IIIII N   N SSSSS TTTTT  AA  L    L     AA   TTTTT IIIII  OOO  N   N

I   NN  N S       T   A  A L    L    A  A    T     I   O   O NN  N

I   N N N SSSSS   T   AAAA L    L    AAAA    T     I   O   O N N N

I   N  NN     S   T   A  A L    L    A  A    T     I   O   O N  NN

IIIII N   N SSSSS   T   A  A LLLL LLLL A  A    T   IIIII  OOO  N   N

====================================================================

HP86 Kermit Installation Manual

Edition 1.00

This Manual applies to version 1.00

- 1 -

===============================

HP86 Kermit Installation Manual

===============================

Edition 1.00

This Manual applies to version 1.00

CONTENTS

--------

1. Introduction.

1.1. Software requirments.

1.2. File name convention.

2. Installation of HP86 Kermit version 1.00

2.1 Try again to find a Kermit disk.

2.2 Bootstrapping from the communication line.

2.2.1 Using an existing file capture system.

2.2.2 Type in your own bootstrap.

Appendix A: Program listings.

Appendix B: RS232 settings.

Appendix C: Modifying the program.

- 2 -

1. INTRODUCTION

------------

This manual describes what software is needed to run HP86 Kermit on

the  HP86  microcomputer  and  how  to  obtain  this  software.    Two

bootstrapping  programs,  for downloading Kermit from a mainframe, are

supplied for use if necessary. Also information is  given  on  how  to

modify the Kermit program.

The HP86 Kermit program was written by Martin Rootes of the Computer

Services Department, Sheffield City Polytechnic. The two bootstrapping

programs  (HP8BOO.BAS  &  HP8BOO.FOR)  are  adaptions of two programs,

written by Bill Catchings of Columbia University, for MS-DOS Kermit.

- 3 -

1.1 SOFTWARE REQUIREMENTS

-------------------------

To have a working version of Kermit for your HP86 you  will  need  a

disk with at least the following two files on it:

KERMIT   -   The actual Kermit BASIC program.

UTIL/1   -   A binary program containing various utilities.

Also if you wish to transfer BASIC programs you will need:

GETSAVE  -   Binary program  giving utilities to save BASIC programs

as DATA files, and load DATA files into memory as BASIC

programs (GET)

The two utility programs can be found on the HP86 DEMO disk supplied

with  each  machine.  The  rest of this document describes how you can

obtain a copy of the Kermit program, and gives information on  how  to

modify the program.

1.2 FILE NAME CONVENTIONS

-------------------------

The  file  names  of kermit files have a slightly different format

depending on whether they are on a mainframe  or  on  the  HP86.  when

referring  to  files  on a mainframe the following convention is used:

name.type , where name is 6 characters and type 3. When  referring  to

files on the HP86 the '.' separator is replaced by a space, thus :

HP8KER.BAS would be translated to HP8KER BAS

Also  all the files on the remote are stored as ASCII strings, so in

the case of BASIC programs once they are transferred to the HP86  they

should  be  translated  to  BASIC files, using GETSAVE. The files once

translated will be referred to by dropping the type part of the  name,

except HP8KER BAS which is renamed as KERMIT, for example:

HP8BOO BAS would after conversion become HP8BOO

- 4 -

2. INSTALLATION OF HP86-KERMIT VERSION 1.00

----------------------------------------

Modified from a document for MS-DOS Kermit by Bill Catchings, CUCCA

by M.J.Rootes, Sheffield City Polytechnic.

To obtain a copy of Kermit for your HP86 you will need to do one of

the following, borrow a floppy disk containing Kermit and copy it, use

a file capture program to download it from the mainframe at your site,

or use the following  instructions  to  'bootstrap'  Kermit  from  the

mainframe.

There are at least three methods of initially getting HP86 Kermit onto

your HP86:

1. Try again to find a copy on diskette.

2. Use another file capture facility to get it.

3. Type in and run a bootstrapping program.

2.1. Try Again To Find A Kermit Disk

------------------------------------

Before explaining how to bootstrap Kermit onto your HP86, a disclaimer

must be made. Although a fair amount of thought and time has gone into

these  procedures,  they  are  far  from  error  free.  If  they  were

foolproof, there would be no need for a protocol such as Kermit. There

are many places where things can go wrong, from something as simple as

a typing mistake to something as unavoidable and  probably  inevitable

as  a  communications line failure. By far the easiest and best way to

install Kermit is from a floppy disk. Before you embark on any of  the

following  procedures  it  is  a  good  idea to check once again for a

diskette to copy, even it it contains an old version  of  Kermit.  The

time you spend searching is likely to be far less frustrating than the

time  you  spend  trying  to bootstrap Kermit by the methods described

below.

2.2. Bootstrapping From the Communication Line

----------------------------------------------

If you can't find a disk with  Kermit  on  it,  there  are  two  other

methods  available  for  bootstrapping  HP86  Kermit onto your HP. The

first method is to use a file capture method or  other  file  transfer

protocol  to  transfer  the  file  to  your  HP86.   The second method

requires you to type in your own downloading program.

- 5 -

In either case, you must transmit the file from the  system  where  it

resides over a communication line and into your HP86.

The  HP8KER.BOO  file  contains  only  printable  ASCII characters, to

ensure that downloading can take place regardless of parity  or  other

peculiarities of the communication channel.

2.2.1. Use An Existing File Capture Facility

--------------------------------------------

Use  your  file  capture facility, whatever it may be, to get the file

HP8KER.BAS onto your HP's disk, but first make sure  you  have  enough

room  for  it  (approx  30k).  Once the file is on your disk, you must

convert it in to a BASIC program using the  GETSAVE  utility  program,

supplied  on  the HP86's demo disk. The utility program UTIL/1 must be

present in the HP86's memory before GETTING 'KERMIT BAS'.

LOADBIN "UTIL/1"     <- Load the utility program.

LOADBIN "GETSAVE"    <- Load the DATA - BASIC conversion utility.

GET "HP8KER BAS"     <- Load the DATA file 'HP8KER BAS' as a BASIC

program.

.....If successful, no lines should be displayed during GET.....

STORE "KERMIT"       <- Save the BASIC program on your disk.

2.2.2. Type In Your Own Bootstrap

---------------------------------

If you can't find some method for downloading the HP8KER.BOO file, the

second way of bootstrapping Kermit is to use the programs 'HP8BOO BAS'

and HP8BOO.FOR to download via your HP86's serial port from your  host

You  run  the program HP8BOO.FOR on your host and then run the program

'HP8BOO BAS' on your HP86.

The FORTRAN program sends the HP8KER.BOO file to  the  BASIC  program,

which  stores  it as string data on your current disk as 'KERMIT BOO'.

A very rudimentary form of error checking is done to  allow  obviously

corrupted records to be retransmitted. Follow this procedure:

1. First, you must establish a connection from your HP86 to the host

system.  A high speed connection is preferable; a "clean" line is

preferable to a noisy one. In fact, a clean line is essential for

this procedure. You must be able to log in  to  the  host  system

over  this  connection.  If  your  HP86  already  has  a terminal

emulation facility, use that. If not, you might need to put  your

HP86  next  to  a real terminal and use that for logging in, then

switch the connector to the HP86 at the critical moment.  If  you

are  using a terminal, make sure the terminal and HP86 have their

communication ports set to the same speed.

- 6 -

2. Ensure that the files HP8BOO.FOR and HP8KER.BOO  are  present  on

the host system.  HP8BOO.FOR is listed below, in case you need to

type it in.

3. Get  back  to  your HP86 and type in 'HP8BOO BAS' on your HP86; a

listing appears below. There is no need to type in  the  comments

(anything  following an exclamation mark); they are only there to

clarify what the program  is  doing.  Check  very  carefully  for

errors. You should check the RS232 settings in the program (lines

2000  -  2180)  to  see  that  it reflects the way your system is

actually set  up.  If  necessary,  change  the  control  register

settings  (see  appendix  B).    The  settings  are  correct  for

downloading from an IBM 4341.   For  half-duplex  systems,  leave

line  1000 as it is; otherwise, replace it by a RETURN statement.

If you type it in directly  to  BASIC  make  sure  you  save  the

program  before you run it, so you won't have to type it in again

in case of error.

4. Get back to your host system and compile HP8BOO.FOR, if it  needs

compiling.  Define  logical  unit  numbers  5  and  6  to  be the

controlling  terminal,  and  logical  unit  7  to  be  the   file

HP8KER.BOO. On VAX/VMS systems, for example, use these commands:

$assign sys$input for005

$assign sys$output for006

$assign hp8ker.boo for007

On a DECSYSTEM-20, do:

define 5: tty:

define 6: tty:

define 7: hp8ker.boo

On an IBM system under VM/CMS, do this:

.filedef 5 term ( lrecl 80 recfm v

.filedef 6 term ( lrecl 80 recfm f

.filedef 7 disk hp8ker boo ( lrecl 80 recfm f perm

5. Set your host system up for downloading:

- Ensure  that  your terminal does not automatically pause at the

end of a screenful of output. For instance,  on  a  DEC-20  you

would issue the command "terminal no pause end-of-page".

- Do  whatever you can to disable messages from appearing at your

terminal while these programs are running. This  would  include

messages from other users, mail notification, alarms or alerts,

system  messages,  and  so  forth. Such messages will interfere

with the procedure, and probably render the result useless.

- 7 -

- You should put your host terminal  in  "local  echo"  or  "half

duplex" mode, if possible.

6. Start the HP8BOO program on your host system.

7. Get  back  to the HP86. If you have been using a terminal, switch

the connector to the HP86.

8. Now run the BASIC program, 'HP8BOO BAS'. This procedure will take

at least twenty minutes and possibly  longer  depending  on  line

speed.   Watch your disk lights for reassurance that something is

happening.

9. The file 'KERMIT BOO' should now exist on the current  HP86  disk

this  should  be converted into a BASIC program using the GETSAVE

utility.

LOADBIN "UTIL/1"     <-  This must be loaded otherwise getsave

will not convert the file correctly.

LOADBIN "GETSAVE"    <-  Load the conversion utility.

GET "KERMIT BOO"     <-  Load the  data file  'KERMIT BOO' and

convert to a BASIC program.

.....When the conversion has completed successfully.........

STORE "KERMIT"       <-  Store the Kermit BASIC program.

10. Before finishing remember to log off from the  host  system,  you

may of course now use the HP86 Kermit to achieve this.

By  using  one  of  these  installation methods, you should now have a

working version of Kermit. If you experience any  problems  or  quirky

behavior  with  the  program,  it's  possible that some part of it was

corrupted during the downloading procedure. Perhaps enough usable code

remains to allow you to transfer HPKERMIT.BAS from the host.  If  not,

you will have to repeat the downloading procedure.

Once  you  have  HP86 Kermit  on  your  disk, you should make the disk

available to other users for copying, so that they can be  spared  the

tedium of this bootstrap procedure.

- 8 -

Appendix A: Program listings

----------------------------

Here  is  a  listing  of  'HP8BOO  BAS'. The DISP statements with line

numbers 140 & 350 may be included if you want incoming records  to  be

displayed on the screen. You don't need to include the comments.

1 ! Run this program on the HP86 in conjunction with a FORTRAN program

2 ! (HP8BOO.FOR) on the mainframe to download kermit to the HP86. This

3 ! program will run for about thirty minutes, depending on line speed

4 ! Original program, for use with a PC running MS-DOS, by:-

5 !   Bill Catchings, June 1984 (revised Sept 1984)

6 !   Columbia University Center for Computing Activities

7 !  Converted for use on HP86 by:-

8 !   M.J.Rootes, Sheffield City Polytechnic, Apr 1986.

9 !

10 INTEGER n,s

20 DIM x$[88],f$[20]

30 SETTIME 0,0

40 GOSUB 2000 ! Set RS232 interface

100 OUTPUT 10 ;"O ,2"

110 ENTER 10 ; f$

120 IF LEN (f$)<5 THEN 110 !   in case of echo repeat

130 ENTER 10 ; n

140 DISP f$&" "&VAL$ (n)

150 IF n>10 THEN 900 !         incorrect  filename

160 CREATE f$,110

165 ASSIGN# 1 TO  f$

170 DISP "Outputting to "&f$

180 GOTO 300

200 GOSUB 1000 !   Turnaround processing

210 OUTPUT 10 ;"N" ! Data incorrect

220 GOTO 320

300 GOSUB 1000 !  Turnaround

310 OUTPUT 10 ;"O" ! data Ok

320 ENTER 10 ; x$

330 IF LEN (x$)<5 THEN 320 ! in case of echo

340 ENTER 10 ; n

350 DISP x$&" "&VAL$ (n)

360 IF LEN (x$)#n THEN 200 ! length wrong - nak

370 IF x$="&&&&&&&&&&" THEN 800 ! end of file

400 PRINT# 1 ; x$

410 GOTO 300

800 DISP "Processing complete, file transferred"

805 DISP "Time taken "&VAL$ (TIME )&"secs"

810 ASSIGN# 1 TO  *

820 END

- 9 -

900 DISP "Filename incorrect"

910 GOTO 820

1000 STATUS 10,9 ; s ! return for full duplex

1010 IF s<128 THEN 1000 ! wait for transmitter to be enabled

1020 RETURN

2000 ! SET RS232 INTERFACE

2020 CONTROL 10,3 ; 6 ! Set baud rate 300 baud

2040 CONTROL 10,4 ; 42 ! Set parity to MARK

2060 CONTROL 10,5 ; 0 ! Set auto handshake (DTR/RTS) off

2080 CONTROL 10,9 ; 225 ! Strip Nulls, backspace & enable transmit &

receive

2100 CONTROL 10,11 ; 192 ! Set handshake/flow-control on

2105 CONTROL 10,14 ; 19 !  Set XOFF

2110 CONTROL 10,15 ; 17 !  Handshake character XON

2120 CONTROL 10,16 ; 67 !  EOL transmit disable & 3 char EOL count

2140 CONTROL 10,17 ; 13 ! CR

2160 CONTROL 10,18 ; 17 ! XON

2170 CONTROL 10,19 ; 46 ! '.'

2180 RETURN

- 10 -

Here  is a listing  of HP8BOO.FOR,  in case you can't  find it on your

host system:

C   This Fortran program should be run on the mainframe in conjunction

C   with a BASIC program ('HP8BOO BAS') on the HP86 to transfer

C   HP8KER.BOO to the HP86 and store it as 'KERMIT BOO'.  this

C   program uses a very rudimentary technique to try to insure that

C   the characters it sends arrive correctly.  It just sends a count

C   of the number of characters sent after each line.  In this way any

C   errors of character loss or insertion will be caught.  If a

C   character is just corrupted it will not be caught.  Hopefully if

C   this happens it will be in a non-critical part of the 'KERMIT BOO'

C   file.  The reason a simple checksum was not used was so that this

C   program could run on machines using either EBCDIC or ASCII

C   characters.  This program should take about thirty minutes to run.

C

C   This program assumes that 5 and 6 are directed to the terminal and

C   7 is directed to the file HP8KER.BOO.

C

C   *Original Program* for use with a PC running MS-DOS by:-

C   Bill Catchings,

C   Columbia University Center for Computing Activities

C   June 1984 (Revised September 1984)

C

C   *REVISED* at Sheffield City Polytechnic by R.L.Horton 22Aug85:-

C    1) FORTRAN G1 cannot accept "I-1" on a WRITE statement.

C    2) Insert a call to NULIN and to RETYPE.

C   *REVISED* at Sheffield City Polytechnic by R.L.Horton 10Apr86:-

C    1) NULIN has been rename to NULLIN.

C    2) For some unknown reason this program started issuing a double

C       read to the terminal at statement 100/3 (at start-up)!

C       The FORMAT statement 200 has been corrected to stop it.

C    3) The FILEDEF for the terminal has been changed to RECFM F80.

C

C   *CHANGED* for use with 'HP8BOO BAS' at Sheffield City Polytechnic

C     By M.J.Rootes 11Apr86:-

C      1) Accept lines up to 79 characters long

C      2) Reverse character count as HP86 BASIC lines contain spaces

C      3) Insert Linefeed between Line and character count

C

INTEGER LINE(79), ACK(4), CHECK, OK, SPACE, COMMA

CALL NULLIN

CALL RETYPE

WRITE(6,100)

100   FORMAT(' Ready to transfer, now run 'HP8BOO BAS' on the HP86.')

C     Get characters for constants (character constants are rough in

C     some FORTRANs).

- 11 -

READ (5,200) OK, SPACE, COMMA, ACK

200   FORMAT(A1,A1,A1,4A1)

C     The following statement has been changed from GO TO 30.

GO TO 20

C     Get terminal handshake.

10    READ (5,200)ACK

C     Did the other side like it?  (Did they send OK?)

IF (ACK(1) .NE. OK) GO TO 50

C     Yes, get new line from file.

20    READ (7,300,END=99)LINE

300   FORMAT(79A1)

C           Count the characters as some rudimentary check for noise.

I = 79

30    IF (LINE(I) .NE. SPACE) GO TO 50

I = I - 1

GO TO 30

C           Put in a comma followed by the count.

C     40    LINE(I) = COMMA

C     Write to TTY.

50    WRITE (6,400)LINE,I

400   FORMAT(79A1/I2)

GOTO 10

C     Send good-bye message.

99    WRITE (6,500)

500   FORMAT(10('&')/'10')

STOP

END

- 12 -

Appendix B: RS232 settings

--------------------------

The following is a list  of certain control registers  for the RS232

interface  and  there  effect  on  its  characteristics  for  use when

adapting 'HP8BOO BAS' to systems other than the IBM 4341 (Mark parity,

XON handshake,  on  300 baud line).  Please  read the  I/O programming

manual for a more detailed explanations of these registers.

CONTROL 10,3 ;  - Gives baud rate       110 - 2

300 - 6

1200 - 8

CONTROL 10,4 ;  - Parity               None  -  3

Odd   - 10

Even  - 26

Mark  - 42

Space - 58

CONTROL 10,5 ;  - DTR/RTS handshake    Off - 0

On  - 48

CONTROL 10,11 ; - XON/XOFF control     None     -   0

XON      - 128

XON/XOFF - 192

CONTROL 10,14 ; - XOFF character       XOFF - 19

CONTROL 10,15 ; - XON  character       XON  - 17

CONTROL 10,16 ; - End of line          No of characters - bits 0 - 4

Transmit disable - bit 5

Auto RTS enable  - bit 7

CONTROL 10,17 - CONTROL 10,23  -  Hold end of line character sequence

- 13 -

Appendix C: Modifying the program

---------------------------------

HP86  Kermit  version  1.00  is a very basic implementation of the

Kermit protocol, most of the advanced facilities having been left out.

Due to the basic nature of version 1.00 it is anticipated  that  other

users  may  want to upgrade this version to include facilities such as

server support. Also users may wish to  modify  the  program  to  give

different default values.

To enable easy modification a fully commented version is available

(HP8KRC.BAS), however this program is to large to  run  in  the  (64k)

memory  of  the  HP86.  To circumvent the memory problem the commented

program is also available split into  various  logical  sections,  the

relevant  section  can  then  be loaded over the non commented version

using the GET utility.  Once modified the new version of that  section

should  be  saved  (SAVE)  and then the comments stripped from the new

program.

To overlay a commented section of program use the following procedure

LOAD "KERMIT"          <- Load kermit program if not already done.

LOADBIN "UTIL/1"       <-  ''  utilities      ''  ''   ''     '' .

LOADBIN "GETSAVE"      <- Load the GETSAVE utility program.

GET "HP8xxx BAS"       <- Load the desired section.

When any modification is complete SAVE the program as it is,  delete

all  lines  except  those in the modified section and SAVE it. Now use

the 'HP8CSU BAS' comment strip utility to remove the comments from the

saved program.

LOADBIN "GETSAVE"      <- Load GETSAVE utility if not present.

SAVE "TEMP"            <- Save the complete program as a data file.

DELETE 1,nnnn          <- Delete lines before changed section.

DELETE mmmm,99999      <-   ''    ''   after    ''      ''   .

SAVE "HP8xxx BAS"      <- Save the desired section.

... If 'HP8CSU BAS' has not been converted to a BASIC file then

DELETE 1,99999         <- Remove any existing program.

GET "HP8CSU BAS"       <- Get DATA file and convert to BASIC.

... Else

LOAD "HP8CSU"          <- Load basic program.

RUN

'Enter old name, new name, size of file ?'

TEMP,HP8KER BAS,120    <- Enter names and size of the new file.

A new version of the commented program  can  be  produced  by  using

GETSAVE.    GET the old file 'HP8KER BAS', delete the section that has

been changed, then GET the new commented section, and finally save the

complete program.

- 14 -

LOADBIN "UTIL/1"       <- Load utilities if not present.

LOADBIN "GETSAVE"      <-  ''  GETSAVE   '' ''    ''   .

GET "HP8KRC BAS"       <- GET old version.

DELETE nnnn,mmmm       <- Delete the section that was changed.

GET "HP8xxx BAS"       <- GET the new version of that section.

SAVE "HP8KRC BAS"      <- SAVE the complete commented program.

HP86 Kermit version 1.00 : Program sections:-

Name        Line numbers  Description

----        ------------  -----------

HP8DEF BAS     1 -   499  Define arrays, variables & constants.

HP8CPR BAS   500 -   999  Command processor.

HP8TRM BAS  1000 -  1999  Terminal mode (CONNECT command)

HP8SND BAS  2000 -  3999  SEND command.

HP8REC BAS  4000 -  4999  RECEIVE command.

HP8SSC BAS  5000 -  6999  SET and SHOW commands.

HP8COM BAS 10000 - 12999  Subroutines for sending & receiving packets

HP8CDF BAS 20000 - 20999  Functions for coding & decoding packets.

HP8DSP BAS 30000 - 30999  Subroutines for displaying current state.

HP8FSS BAS 40000 - 42999  Filling system subroutines

HP8MSC BAS 50000 - 52999  Miscellaneous subroutines.

- 15 -

#