1

KERMIT

A SIMPLE FILE TRANSFER PROTOCOL FOR MICROCOMPUTERS AND MAINFRAMES

Frank da Cruz,  Bill Catchings

Columbia University Center for Computing Activities

New York, N.Y.  10027

May 1983

During  recent  years, the technical press has focused a lot of atten-

tion on developments in computer networking -- the IEEE  802  committee,

TCP/IP, SNA, the latest VLSI Ethernet interface, fibre optics, satellite

communications,  broadband  versus  baseband.   But little attention has

been given to the single mechanism that may be the most widely  used  in

the  real  world  for direct interprocessor communication: the so-called

``asynchronous protocol'', which is to be found in some form  at  almost

every  institution  where  there  is  a  need  to transfer files between

microcomputers and central computers.

Columbia University is such an institution.   Large  timesharing  com-

_______________

1

This  is  the  original  manuscript  of the article published in BYTE

Magazine as "Kermit: A File Transfer Protocol  for  Universities",  June

and  July  1984.    Some  minor editorial differences exist between this

manuscript and the article as  published.    The  Kermit  file  transfer

protocol  is  named after Kermit the Frog, star of the television series

THE MUPPET SHOW, used by permission of Henson Associates, Inc.

2

puters  at a central site are complemented by many smaller systems scat-

tered in the laboratories and departments.  The past few years have wit-

nessed the inexorable progress of diverse microcomputers,  word  proces-

sors, and professional workstations into offices and laboratories throu-

ghout  the University, and into the homes or dormitory rooms of faculty,

students, and staff.  As soon as these small machines began  to  appear,

their  users  asked  for  ways  to  exchange  files with the central and

departmental systems.

At the same time, student use of our central systems was growing at an

astonishing rate.  We could no longer afford to  provide  students  with

perpetual  online  disk  storage;  we began to issue IDs per course, per

term.  With the decreased longevity of the IDs came the  need  for  stu-

dents to economically archive their files.  Given a reliable way to from

the  central mainframes and back, microcomputers with floppy disks could

provide inexpensive removable media ideal for this purpose.

The situation called for a file transfer  mechanism  that  could  work

among  all  our  computers, large and small.  We knew of none that could

handle the required diversity.   Some  were  intended  for  use  between

microcomputers,  others  between  large computers, but none specifically

addressed the need for communication among the widest possible range  of

computers,  particularly  between micros and our IBM and DEC mainframes.

Most commercial packages served a limited set of systems, and their cost

would have been prohibitive when  multiplied  by  the  large  number  of

machines we needed to support.

3

So  we  embarked on our own project.  We were not well-versed in these

matters at the outset; we learned  as  we  proceeded,  and  we're  still

learning.   This article discusses some of the issues and tradeoffs that

came up in the design, and illustrates them in terms of our result,  the

KERMIT  protocol for point-to-point file transfer over telecommunication

lines.  Because commercial local area networking products are expensive,

not yet widely available, and unsuitable for one-shot or  long-haul  ap-

plications,  humble  asynchronous protocols such as KERMIT are likely to

be with us for a long time to come.

It is assumed  the  reader  is  familiar  with  common  computing  and

telecommunications  terminology,  and  with the ASCII alphabet, which is

listed at the end of this article for reference.

* The Communication Medium

The only communication medium  common  to  all  computers,  large  and

small,  is the asynchronous serial telecommunication line, used for con-

necting terminals to computers.  Standards for this  medium  are  almost

universally  followed -- connectors, voltages, and signals (EIA RS-232-C

[1]), character encoding (ASCII, ANSI X3.4-1977 [2]),  and bit transmis-

sion sequence (ANSI X3.15-1976 [3, 4]).  Serial connections can be  made

in  many  ways:  dedicated  local  lines (``null modem'' cables), leased

telephone circuits, dialup connections.  Dialup connections can  be  in-

itiated  manually  from the home or office using an inexpensive acoustic

4

coupler, or automatically from one computer to another using a programm-

able  dialout  mechanism.    The asynchronous serial line offers the or-

dinary user a high degree of convenience and control in establishing in-

tersystem connections, at relatively low cost.

Once two computers are connected with a serial line,  information  can

be  transferred  from one machine to the other, provided one side can be

instructed to send the information and the other to  receive  it.    But

right away, several important factors come into play:

1. Noise  --  It  is rarely safe to assume that there will be no

electrical interference on a line; any long or switched  data

communication  line  will  have  occasional  interference, or

noise, which typically results in garbled  or  extra  charac-

ters.  Noise corrupts data, perhaps in subtle ways that might

not be noticed until it's too late.

2. Synchronization  --  Data  must  not  come in faster than the

receiving machine can handle it.  Although line speeds at the

two ends of the connection may match, the  receiving  machine

might not be able to process a steady stream of input at that

speed.   Its central processor may be too slow or too heavily

loaded, or its buffers too full or too small.    The  typical

symptom  of  a  synchronization  problem  is  lost data; most

operating systems will simply discard incoming data they  are

not prepared to receive.

3. Line Outages -- A line may stop working for short periods be-

cause  of  a  faulty  connector,  loss  of  power, or similar

reason.  On dialup or switched connections, such intermittent

failures will cause carrier to drop and the connection to  be

closed, but for any connection in which the carrier signal is

not used, the symptom will be lost data.

Other   communication  media,  such  as  the  parallel  data  bus,  have

safeguards built in to prevent or minimize these effects.  For instance,

distances may be strictly limited, the environment  controlled;  special

5

signals  may be available for synchronization, and so forth.  The serial

telecommunication line provides no such safeguards, and we  must  there-

fore regard it as an intrinsically unreliable medium.

* Getting Reliable Communication over an Unreliable Medium

To  determine  whether  data has been transmitted between two machines

correctly and completely, the two machines can compare the  data  before

and  after transmission.  A scheme that is commonly used for file trans-

fer employs cooperating programs running simultaneously on each machine,

communicating in a well-defined, concise language.  The sending  program

divides outbound data into discrete pieces, adding to each piece special

information  describing  the data for the receiving program.  The result

is called a ``packet''.  The receiver separates the description from the

data and determines whether they still match.  If so, the packet is ack-

nowledged and the transfer proceeds.  If not, the packet is ``negatively

acknowledged'' and the sender retransmits it; this procedure repeats for

each packet until it is received correctly.

The process is called a communication protocol -- a set of  rules  for

forming  and  transmitting  packets, carried out by programs that embody

those rules.  Protocols vary in complexity; our  preference  was  for  a

simple  approach that could be realized in almost any language on almost

any computer by a programmer of moderate skill, allowing the protocol to

be adapted easily to new systems.

6

* Accommodating Diverse Systems

Most  systems agree how to communicate at the lowest levels -- the EIA

RS-232-C asynchronous communication line and  the  ASCII  character  set

--  but  there  is rarely agreement beyond that.  To avoid a design that

might lock out some kinds of systems, we must consider certain important

ways in which systems can differ.

. Mainframes vs Micros

A distinction must first be made between micros and mainframes.  These

terms are not used  perjoratively;  a  ``micro''  could  be  a  powerful

workstation, and a ``mainframe'' could be a small minicomputer.  For our

purposes,  a  micro  is  any single-user system in which the serial com-

munication port is strictly an external device.  A mainframe is any sys-

tem which is ``host'' to multiple simultaneous terminal users,  who  log

in  to  ``jobs'', and where a user's terminal is the job's ``controlling

terminal''.  Some mainframe systems allow users  to  ``assign''  another

terminal line on the same machine as an external input/output device.

Mainframe operating system terminal drivers usually treat a job's con-

trolling  terminal specially.  Full duplex systems echo incoming charac-

ters on the controlling terminal, but not on an assigned line.    System

command interpreters or user processes might take special action on cer-

tain  characters  on  the  controlling line, but not on an assigned line

(for instance, control-C under CP/M  or  most  DEC  operating  systems).

7

Messages  sent to a job's controlling terminal from other jobs could in-

terfere with transmission of data.  The ability of a system to test  for

the  availability  of input on a serial line might depend on whether the

line is the job's controlling terminal or an assigned device;  CP/M  and

IBM VM/370 are examples of such systems.  CP/M can test for data only at

the console, VM can test anywhere but the console.

Output  to  a  job's  controlling  terminal  may be reformatted by the

operating system: control characters  may  be  translated  to  printable

equivalents, lower case letters specially flagged or translated to upper

case  (or  vice  versa), tabs expanded to spaces.  In addition, based on

the terminal's declared ``width'' and ``length'', long  lines  might  be

``wrapped  around''  or  truncated,  formfeeds translated to a series of

linefeeds, and the system may want to pause at the end of each screenful

of output.  Input from a job's controlling terminal may also be  handled

specially:  lower  case letters may be converted to upper case, linefeed

may be supplied when carriage return is typed,  control  characters  may

invoke special functions like line editing or program interruption.  The

DECSYSTEM-20  is  an example of a computer where any of these might hap-

pen.

The moral here is that care must be taken to disable special  handling

of a mainframe job's controlling terminal when it is to be a vehicle for

interprocessor communication.  But some systems simply do not allow cer-

tain  of  these features to be disabled, so file transfer protocols must

be designed around them.

8

. Line Access

Line access can be either full or half duplex.  If full duplex, trans-

mission  can  occur in both directions at once.  If half duplex, the two

sides must take turns sending, each signaling the other when the line is

free; data sent out of turn is discarded, or it can  cause  a  break  in

synchronization.  On mainframes, the host echoes characters typed at the

terminal  in full duplex, but not in half duplex.  Naturally, echoing is

undesirable during file transfer.  Full duplex systems can  usually  ac-

commodate half duplex communication, but not vice versa.  IBM mainframes

are the most prevalent half duplex systems.

. Buffering and Flow Control

Some  systems  cannot  handle  sustained bursts of input on a telecom-

munications line; the input buffer can fill up faster  than  it  can  be

emptied, especially at high line speeds.  Some systems attempt to buffer

``typeahead''  (unrequested input), while others discard it.  Those that

buffer typeahead may or may not provide a mechanism to test or clear the

buffer.

Systems may try to regulate how fast characters come in using  a  flow

control  mechanism,  either in the data stream (XON/XOFF) or in parallel

to it (modem control signals) [5], but no two systems can be assumed  to

honor  the  same conventions for flow control, or to do it at all.  Even

when flow control is being done, the control signals themselves are sub-

9

ject to noise corruption.

Our  experiments  with several host computers revealed that a burst of

more than about a line's worth of characters (60-100 characters) into  a

terminal port at moderate speed could result in loss of data -- or worse

--  on  some  hosts.   For instance, the communications front end of the

DECSYSTEM-2060 is designed on the statistical assumption that  all  ter-

minal  input  comes  from  human fingers, and it cannot allocate buffers

fast enough when this assumption is violated by sending continuous  data

simultaneously from several microcomputers attached to terminal ports.

. Character Interpretation

Systems can differ in how they interpret characters that arrive at the

terminal  port.    A  host  can  accept  some characters as sent, ignore

others, translate others, take special action on others.  Communications

front ends or multiplexers might swallow certain  characters  (typically

DC1,  DC3)  for  flow  control, padding (NUL or DEL), or for transfer of

control (``escape'').  The characters that typically trigger special be-

havior are the ASCII control characters, 0-31 and 127.  For instance, of

these  33  control  characters,  17  invoke  special  functions  of  our

DECSYSTEM-20  command  processor.   However, all hosts and communication

processors we've encountered allow any  ``printable''  character  (ASCII

32-126)  to  reach an application program, even though the character may

be translated to a different encoding,  like  EBCDIC [6],  for  internal

10

use.

Some  operating systems allow an application to input a character at a

time, others delay  passing  the  characters  to  the  program  until  a

``logical  record''  has been detected, usually a sequence of characters

terminated by carriage return or linefeed.  Some record oriented systems

like IBM VM/370 discard the terminator, others keep it.  And  there  are

different  ways  of  keeping  it -- UNIX translates carriage return into

linefeed; most DEC operating systems keep the carriage return  but  also

add a linefeed.

. Timing Out

Hosts  may or may not have the ability to ``time out''.  When exchang-

ing messages with another computer, it is desirable to be able to  issue

an  input  request  without  waiting forever should the incoming data be

lost.  A lost message could result in a protocol ``deadlock''  in  which

one  system is waiting forever for the message while the other waits for

a response.  Some systems can set timer interrupts to allow escape  from

potentially  blocking operations; others, including many microcomputers,

can not do so.  When timeouts are not possible, they may be simulated by

sleep-and-test or loop-and-test operations, or deadlocked systems may be

awakened by manual intervention.

11

. File Organization

Some  computers  store  all files in a uniform way, such as the linear

stream of bytes that is a UNIX file.  Other computers may have more com-

plicated or diverse  file  organizations  and  access  methods:  record-

oriented  storage with its many variations, exemplified in IBM OS/360 or

DEC RMS.  Even simple  microcomputers  can  present  complications  when

files  are treated as uniform data to be transferred; for instance under

CP/M, the ends of binary and text files are determined differently.    A

major  question  in  any operating system is whether a file is specified

sufficiently by its contents and its name, or if additional external in-

formation is required to make the file valid.  A simple generalized file

transfer facility can be expected to transmit a  file's  name  and  con-

tents, but not every conceivable attribute a file might possess.

Designers  of  expensive  networks  have gone to great lengths to pass

file attributes along when transferring files  between  unlike  systems.

For  instance, the DECnet Data Access Protocol [7] supports 42 different

``generic system capabilities'' (like whether files can be preallocated,

appended to, accessed randomly, etc), 8 data types (ASCII,  EBCDIC,  ex-

ecutable, etc), 4 organizations (sequential, relative, indexed, hashed),

5 record formats (fixed, variable, etc), 8 record attributes (for format

control),  14  file allocation attributes (byte size, record size, block

size, etc), 28 access options (supersede, update, append, rewind,  etc),

26  device  characteristics  (terminal,  directory  structured,  shared,

12

spooled, etc), various access options (new, old, rename, password, etc),

in  addition  to  the  better  known file attributes like name, creation

date, protection code, and so on.  All this was  deemed  necessary  even

when  the  designers had only a small number of machines to worry about,

all from a single vendor.

The ARPA network, which attempts to provide  services  for  many  more

machines  from many vendors, makes some simplifying assumptions and sets

some restrictions in its File Transfer Protocol (FTP) [8].    All  files

are  forced  into  certain  categories  with respect to encoding (ASCII,

EBCDIC, image), record format control, byte size, file structure (record

or stream), and it is generally left to the host FTP  implementation  to

do  the  necessary transformations.  No particular provision is made, or

can be made, to ensure that such transformations are invertible.

DECnet is able to provide invertibility for operating systems like VMS

or RSX, which can store the necessary file  attributes  along  with  the

file.    But simpler file systems, like those of TOPS-10 or TOPS-20, can

lose vital information about incoming files.  For instance, if  VMS  re-

quires some type of file to have a specific blocksize, while TOPS-20 has

no  concept of block size, then the blocksize will be lost upon transfer

from VMS to TOPS-20 and cannot be restored automatically when  the  file

is sent back, leaving the result potentially unusable.

Invertibility  is  a  major  problem,  with  no simple solution.  For-

tunately, most file transfer between unlike systems involves  only  tex-

tual  information -- data, documents, program source -- which is sequen-

13

tial  in  organization,  and  for  which  any  required  transformations

(e.g. blocked to stream, EBCDIC to ASCII) are simple and  not  dependent

on any special file attributes.

In  fact, invertability can be achieved if that is the primary goal of

a file transfer protocol.  All the external attributes of a file can  be

encoded  and  included with the contents of the file to be stored on the

remote system.  For unlike systems, this can render the file  less  than

useful on the target system, but allows it to be restored correctly upon

return.   However, it is more commonly desired that textual files remain

intelligible when transferred to a foreign  system,  even  if  transfor-

mations  must  be  made.  To allow the necessary transformations to take

place on textual files between unlike systems, there must be a  standard

way of representing these files during transmission.

. Binary Files versus Parity

Each  ASCII character is represented by a string of 7 bits.  Printable

ASCII files can be transmitted in a straightforward fashion, because AS-

CII transmission is designed for them: a serial stream of 8-bit  charac-

ters,  7  bits  for data and 1 for parity, framed by start and stop bits

for the benefit of the hardware [3].  The parity bit is added as a check

on the integrity of a character; some systems  always  transmit  parity,

others insist on parity for incoming characters, still others ignore the

parity bit for communication purposes and pass it along to the software,

14

while  still  others  discard it altogether.  In addition, communication

front ends or common carriers might usurp the parity bit, regardless  of

what the system itself may do.

Computer  file  systems generally store an ASCII file as a sequence of

either 7-bit or 8-bit bytes.  8-bit bytes are more common, in which  the

8th  bit  of  each byte is superfluous.  Besides files composed of ASCII

characters, however, computers also  have  ``binary''  files,  in  which

every  bit is meaningful; examples include executable ``core images'' of

programs, numbers stored in ``internal format'', databases with imbedded

pointers.  Such binary data must  be  mapped  to  ASCII  characters  for

transmission  over  serial lines.  When two systems allow the user-level

software to control the parity bit, the  ANSI  standards [2, 3]  may  be

stretched to permit the transmission of 8 data bits per character, which

corresponds  to  the byte size of most machines.  But since not all com-

puters allow this flexibility, the ability to transfer  binary  data  in

this fashion cannot be assumed.

. Software

Finally,  systems  differ  in  the application software they have.  In

particular, no system can be assumed to have  a  particular  programming

language.  Even widespread languages like FORTRAN and BASIC may be lack-

ing  from some computers, either because they have not been implemented,

or because they are proprietary and have not been purchased.  Even  when

15

two  different  systems  support the same language, it is unrealistic to

expect the two implementations of the language to be totally compatible.

A general purpose file transfer protocol should not  be  geared  towards

the features any particular language.

* KERMIT

Our  protocol,  which  we call KERMIT, addresses the problems outlined

above by setting certain minimal standards for transmission, and provid-

ing a mapping between disk storage organization, machine word  and  byte

size, and the transmission medium.

KERMIT has the following characteristics:

- Communication takes place over ordinary terminal connections.

- Communication  is half duplex.  This allows both full and half

duplex systems to participate, and it eliminates  the  echoing

that  would  otherwise occur for characters arriving at a host

job's controlling terminal.

- The packet length is variable, but the maximum is  96  charac-

ters  so that most hosts can take packets in without buffering

problems.

- Packets are sent in alternate directions; a reply is  required

for each packet.  This is to allow half duplex systems to par-

ticipate,  and  to prevent buffer overruns that would occur on

some systems if packets were sent back to back.

- A timeout facility, when available, allows transmission to re-

sume after lost packets.

- All transmission is in ASCII.  Any non-ASCII hosts are respon-

sible for conversion.  ASCII control characters  are  prefixed

and then converted to printable characters during transmission

16

to  ensure  that  they arrive as sent.  A single ASCII control

character (normally SOH) is used to mark the  beginning  of  a

packet.

- Binary files can be transmitted by a similar prefix scheme, or

by use of the parity bit when both sides have control of it.

- Logical records (lines) in textual files are terminated during

transmission  with  quoted carriage-return/linefeed sequences,

which are transparent to the protocol and may appear  anywhere

in  a  packet.  Systems that delimit records in other ways are

responsible for conversion, if they desire the distinction be-

tween records to be preserved across unlike systems.

- Only a file's name and contents  are  transmitted  --  no  at-

tributes.    It  is  the user's responsibility to see that the

file is stored correctly on the target system.    Within  this

framework,  invertible  transfer of text files can be assured,

but invertible transfer  of  non-text  files  depends  on  the

capabilities  of  the particular implementations of KERMIT and

the host operating systems.

- KERMIT has no special knowledge of the host on the other side.

No attempt is made to ``integrate'' the two  sides.    Rather,

KERMIT  is designed to work more or less uniformly on all sys-

tems.

- KERMIT need not be written in any particular language.  It  is

not a portable program, but a portable protocol.

Thus  KERMIT  accommodates  itself  to many systems by conforming to a

common subset of their features.    But  the  resulting  simplicity  and

generality allow KERMIT on any machine to communicate with KERMIT on any

other   machine,   micro-to-mainframe,   micro-to-micro,   mainframe-to-

mainframe.  The back-and-forth exchange of packets keeps the  two  sides

synchronized;  the  protocol can be called ``asynchronous'' only because

the communication hardware itself operates asynchronously.

As far as the user is concerned, KERMIT is a do-it-yourself operation.

17

For  instance, to transfer files between your micro and a mainframe, you

would run KERMIT on your micro, put KERMIT into terminal emulation mode,

which ``connects'' you to the mainframe, log in and run  KERMIT  on  the

mainframe,  then  ``escape'' back to the micro and issue commands to the

micro's KERMIT to send or fetch the desired files.    Any  inconvenience

implicit  in  this  procedure is a consequence of the power it gives the

ordinary user to establish reliable connections between  computers  that

could not otherwise be connected.

* Packets

KERMIT  packets  need  to  contain the data that is being transferred,

plus minimum information to assure (with high probability) that the  ex-

pected  data  arrives  completely and correctly.  Several issues come up

when designing the packet layout: how to represent data, how to  delimit

fields  within  the packet, how to delimit the packet itself, how to ar-

range the fields within the packet.  Since the transmission  medium  it-

self  is  character-oriented, it is not feasible to transmit bit strings

of arbitrary length, as do the bit-oriented protocols like HDLC and SDLC

[5].   Therefore the smallest unit of information in a  packet  must  be

the  ASCII  character.    As we will see, this precludes some techniques

that are used with other communication media.

18

. Control Fields

Most  popular protocol definitions view the packet as layers of infor-

mation, which pass through a hierarchy of protocol  levels,  each  level

adding  its  own information at the ends of an outbound packet or strip-

ping its information from the ends of an incoming packet, and then pass-

ing the result along to the next level in the hierarchy.  The fields for

each layer must be arranged so that they can be found,  identified,  and

interpreted correctly at the appropriate level.

Since KERMIT packets are short, it is important to minimize the amount

of  control information per packet.  It would be convenient to limit the

control fields to one character  each.    Since  we  have  95  printable

characters  to work with (128 ASCII characters, less DEL and the 32 con-

trol characters), we can represent values from 0 to  94  with  a  single

character.

- The packet sequence number is used to detect missing or dupli-

cate  packets.   It is unlikely that a large number of packets

could be lost, especially since packet n is  acknowledged  be-

fore  packet  n+1  is  sent.   So the sequence number can be a

small quantity, which ``wraps around'' to  its  minimum  value

when it exceeds a specified maximum value.

- To  prevent  long  packets,  a small maximum length can be en-

forced by specifying the packet length with a  single  charac-

ter; since there are 95 printable ASCII characters, this would

be  the  maximum length, depending on how we count the control

fields.

- The checksum can be  of  fixed  length.    The  actual  length

depends  on the desired balance between and efficiency and er-

ror detection.

19

The packet length and checksum act together to detect corrupted, miss-

ing,  or extra characters.  These are the essential fields for promoting

error-free transmission.  But so far, we've only considered packets that

carry actual file data; we will also require  special  packets  composed

only  of  control  information, for instance to tell the remote host the

name of the file that is about to come, or to tell it that the transmis-

sion is complete.  This can be accomplished with a  packet  type  field.

The  number of functions we need to specify in this field is small, so a

single character can suffice here too.

. Packet Framing

We choose to mark the beginning of a packet with a distinguished start

character, SOH (Start Of Header, ASCII 1, Control-A).    This  character

cannot  appear anywhere else within the packet.  SOH was chosen because,

unlike most other control characters, it is generally accepted upon  in-

put  at a job's controlling terminal as a data character, rather than an

interrupt or break character on most mainframes.  This  is  probably  no

accident, since it was originally intended for this use by the designers

of  the  ASCII alphabet [9].  Should a system be incapable of sending or

receiving SOH, it is possible to redefine the start-of-packet  character

to  be  any other control character; the two sides need not use the same

one.

There are three principal options for recognizing the end of a packet:

20

a  fixed  length,  a  distinguished  packet-end  character, and a length

field.  There are arguments for and against each involving what  happens

when  characters,  particularly  a  length  or  terminator,  is  lost or

garbled, which will be mentioned later.  KERMIT uses a length field.

To take in a packet, a KERMIT program gets characters  from  the  line

until  it  encounters the SOH.  The next character is the length; KERMIT

reads and decodes the length and then reads that many subsequent charac-

ters to complete the packet.  If another SOH is encountered  before  the

count  is  exhausted,  the  current packet is forgotten and a new one is

started.   This  stratagy  allows  arbitrary  amounts  of  noise  to  be

generated  spontaneously  between  packets  without interfering with the

protocol.

. Encoding

When transmitting textual data, KERMIT terminates logical records with

carriage-return linefeed combinations (CRLFs).  On record oriented  sys-

tems,  trailing  blanks or length fields are removed and a CRLF appended

to outbound records, with the inverse operation  performed  on  incoming

records.    On stream oriented systems, incoming CRLFs may be translated

to some other terminator.  Files,  of  course,  need  not  have  logical

records,  in which case record processing can be skipped altogether, and

the file can be treated as a long string of bytes.   This  is  known  as

``image''  transfer,  and can also be used between like systems where no

21

transformations are necessary.

In  order  to  make  each  character  in  the packet printable, KERMIT

``quotes'' any unprintable character by transforming it to  a  printable

one and precedes it with a special prefix character.  The prefix is nor-

mally  ``#'';  the transformation is done by complementing bit 6 (adding

or subtracting  64  ,  modulo  64).    Thus  control-A  becomes  ``#A'',

10

control-Z  becomes ``#Z'', US (control-underscore on most terminals) be-

comes ``#_''.  The prefix  character  is  also  used  to  quote  itself:

``##''.  Upon input, the reverse transformation is performed.  Printable

characters  are not tranformed.  The assumption is that most files to be

transferred are printable, and printable files  contain  relatively  few

control  characters; when this is true, the character stream is not sig-

nificantly lengthened by quoting.  For binary files, the average quoting

overhead will be 26.6% if all bit patterns are equally likely, since the

characters that must be quoted (the control characters,  plus  DEL,  and

``#'' itself) comprise 26.6% of the ASCII alphabet.

KERMIT also provides a scheme for indicating the status of the 8th bit

when transferring binary files between systems that must use the 8th bit

for  parity.  A byte whose 8th bit is set is preceded by another special

quoting character, ``&''.  If the low-order 7 bits coincide with an  AS-

CII control character, then control-character quoting is also done.  For

instance, the byte 10000001  would be transmitted as ``&#A''.  The ``&''

2

character  itself  can  be  included as data by quoting it (#&), and the

control-quote character may have its 8th bit set (&##).  8th-bit quoting

22

is  only  done when necessary; if both sides can control the parity bit,

then its value is preserved during transmission.  If the 8th bit is  set

randomly  on  binary  files, then 8th-bit quoting will add 50% character

overhead.  For some kinds of binary data, it  could  be  less;  for  in-

stance,  positive  binary  numbers  in 2's complement notation do not to

have their high-order bits set, in which case at least one byte per word

will not be quoted.

A third kind of ``quoting'' implements rudimentary  data  compression.

At  low  speeds, the bottleneck in file transmission is likely to be the

line itself, so any measure that can cut down on use of the  line  would

be  welcome.  The special prefix character ``~'' indicates that the next

character is a repeat count (a single character, encoded printably)  and

that  the  character  after that (which may also have control or 8th-bit

prefixes) is repeated so many times.  For instance ``~}A''  indicates  a

series  of 93 letter A's; ``~H&#B'' indicates a series of 40 control-B's

with the parity bit set.  The repeat count prefix itself can be included

as text by quoting it with ``#''.

To keep the protocol simple, no other transformations are  done.    At

this point, however, it might be worth mentioning some things we did not

do to the data:

- Fancy  Data  compression.    If  the  data  is known to be (or

resemble) English text, a Huffman encoding [10, 11]  based  on

the  frequency of characters in English text could be used.  A

Huffman code resembles Morse code, which has  variable  length

characters  whose boundaries can always be distinguished.  The

more frequent the character, the shorter  the  bit  string  to

23

represent  it.    Of  course,  this scheme can backfire if the

character distribution of the data is very different from  the

one  assumed.  In any case, variable length characters and AS-

CII transmission don't mix well.

- Error Correcting Codes.  Techniques,  such  as  Hamming  codes

[12],  exist  for  detecting  and correcting errors on a per-

character basis.  These are expensive in resources and complex

to program.  KERMIT uses per-packet  block  check  techniques,

which are explained below.

- Nibble  Encoding.    To  circumvent  problems with control and

8-bit characters, it would have been possible to divide  every

character into two 4-bit ``nibbles'', sending each nibble as a

printable character (e.g. a hexadecimal digit).  The character

overhead  caused  by  this  scheme would would always be 100%.

But it would be an easy way to transfer binary files.

. Error Detection

Character parity and Hamming codes are forms of ``vertical  redundancy

checks''  (VRCs), formed by combining all the bits of a character in one

way or another.  The other kind  of  check  that  can  be  used  is  the

``longitudinal  redundancy check'' (LRC), which produces a ``block check

character'' formed by some combination of each character  within  a  se-

quence.  The sending side computes the LRC and sends it with the packet;

the  receiving  side  recomputes  it  for comparison.  There are various

forms of LRCs.  One form produces  a  ``column  parity''  character,  or

``logical  sum'',  whose bits are the exclusive-ORs of the corresponding

bits of the data characters.  Another is the ``checksum'' which  is  the

arithmetic  sum  of  all  the  characters  in  the sequence, interpreted

numerically.  Another is the ``Cyclic Redundancy Check'' (CRC) [13, 14],

24

which  passes  the  characters  through what amounts to a shift register

with imbedded feedback loops, producing a block check in which each  bit

is effected in many ways by the preceding characters.

All  of  these  techniques will catch single-bit errors.  They vary in

their ability to detect other kinds of errors.  For instance, a  double-

bit column error will always go undetected with column parity, since the

result  of  XORing  any  two  bits  together is the same as XORing their

complements, whereas half the possible double bit errors can  be  caught

by  addition  because of the carry into the next bit position.  CRC does

even better by rippling the effect of a data bit  multiply  through  the

block  check  character,  but  the method is complex, and a software im-

plementation of CRC can be inscrutable.

Standard, base-level  KERMIT  employs  a  single-character  arithmetic

checksum, which is simple to program, is low in overhead, and has proven

quite  adequate  in  practice.  The sum is formed by adding together the

ASCII values of each character in the packet  except  the  SOH  and  the

checksum  itself,  and including any quoting characters.  Even non-ASCII

hosts must do this calculation  in  ASCII.    The  result  can  approach

12,000   in the worst case.  The binary representation of this number is

10

10111011100000,  which  is  14  bits  long.   This is much more than one

character's worth of bits, but we can make the  observation  that  every

character  included  in the sum has contributed to the low order 7 bits,

so we can discard some high order bits and still have a viable  validity

check.

25

The KERMIT protocol also allows other block check options, including a

two-character  checksum  and  a  three-character  16-bit  CRC.  The two-

character checksum is simply the low order 12  bits  of  the  arithmetic

sum,  broken  into two printable characters.  The CRC sequence is formed

from the 16-bit quantity generated by the  CCITT-recommended  polynomial

16  12  5

X  +X  +X +1  which  is also used in some form with other popular trans-

mission techniques, like ISO HDLC and IBM SDLC [5].  The  high  order  4

bits  of  the  CRC  go  into  the first character, the middle 6 into the

second, and the low order 6 into the third.

Some care must be taken in the formation of the single-character block

check.  Since it must be expressed  as  a  single  printable  character,

values  of  the  high order data bits may be lost, which could result in

undetected errors, especially when transferring binary  files.    There-

fore,  we  extract  the 7th and 8th bits of the sum and add them back to

the low order bits; if the arithmetic sum of all the  characters  is  S,

then the value of the single-character KERMIT checksum is given by

(S + ((S AND 300)/100)) AND 77

(numbers  are in octal notation).  This ensures that the checksum, terse

though it is, reflects every bit from every character in the packet.

The probability that an error will not be caught by a correctly trans-

mitted arithmetic checksum is the ratio of the number of possible errors

that cancel each other out to the total number of possible errors, which

n

works out to be something like 1/2 , where n is the number  of  bits  in

26

the  checksum, assuming all errors are equally likely.  This is 1/64 for

the single character checksum, and 1/4096 for the 2-character  checksum.

But  the probability that errors will go undetected by this method under

real conditions cannot be easily derived, because all  kinds  of  errors

are  not equally likely.  A 16-bit CRC will detect all single and double

bit errors, all messages with an odd number of bits in error, all  error

bursts  shorter  than  16  bits, and better than 99.99% of longer bursts

[13].   These probabilities all assume, of course, that the block  check

has  been identified correctly, i.e. that the length field points to it,

and that no intervening characters have been lost or spuriously added.

A final note on parity -- a parity bit on each character combined with

a logical sum of all the characters (VRC and LRC) would allow  detection

and  correction of single-bit errors without retransmission by pinpoint-

ing the ``row'' and ``column'' of the bad  bit.    But  control  of  the

parity  bit cannot be achieved on every system, so we use the parity bit

for binary data when we  can,  or  surrender  it  to  the  communication

hardware if we must.  If we have use of the 8th bit for data, then it is

figured into the block check; if we do not, then it must be omitted from

the  block  check  in  case  it  has  been  changed by agents beyond the

knowledge or control of the KERMIT program.

27

. Packet Layout

KERMIT packets have the format:

+------+-----------+-----------+------+------------+-------+

| MARK | char(LEN) | char(SEQ) | TYPE |    DATA    | CHECK |

+------+-----------+-----------+------+------------+-------+

|                  |           |                   |       |

|                  |           +-- (application) --+       |

|                  |                               |       |

|                  +-------------- (session) ------+       |

|                                                          |

+--------------------------------- (data link) ------------+

where all fields consist of ASCII characters, and the char function con-

verts a number in the range 0-94   to a printable ASCII character by ad-

10

ding 32  .

10

In  terms  of  the  ISO  network reference model [15], 8-bit bytes are

presented to the KERMIT program by the  hardware  and  operating  system

software  comprising  the  physical link layer.  Correct transmission is

ensured by the packet-level routines that implement the data link  layer

using  the  outer  ``skin''  of  the  packet -- the MARK, LEN, and CHECK

fields.  The network and transport layers are moot, since  KERMIT  is  a

point-to-point  affair,  in  which the user personally makes all the re-

quired connections.  The session layer  is  responsible  for  requesting

retransmission  of  missing packets or ignoring redundant ones, based on

the SEQ field; the presentation layer is responsible for any  data  con-

versions (EBCDIC/ASCII, insertion or stripping of CRLFs, etc).  Finally,

the  remainder of the packet, the TYPE and DATA fields, are the province

28

of  the application layer; our application, of course, is file transfer.

In any particular  implementation,  however,  the  organization  of  the

program  may not strictly follow this model.  For instance, since trans-

mission is always in stream ASCII, IBM implementations must convert from

EBCDIC and insert CRLFs before checksum computation.

The fields of a KERMIT packet are as follows:

MARK      Start-of-packet character, normally SOH (ASCII 1).

LEN       The number of ASCII characters, including  quoting  characters

and the checksum, within the packet that follow this field, in

other words the packet length minus two.  Since this number is

expressed  as a single character via the char function, packet

character counts of 0 to 94   are permitted, and 96    is  the

10                      10

maximum total packet length.

SEQ       The packet sequence number, modulo 100 .  The sequence numbers

8

``wraps around'' to 0 after each group of 64   packets.

10

TYPE      The  packet  type,  a single printable ASCII character, one of

the following:

D   Data

Y   Acknowledge (ACK)

N   Negative Acknowledge (NAK)

S   Send Initiate (Send-Init)

R   Receive Initiate

B   Break Transmission (EOT)

F   File Header

Z   End of file (EOF)

E   Error

G   Generic Command.  A single character in  the  data  field,

possibly  followed  by operands, requests host-independent

remote execution the specified command:

L   Logout, Bye.

F   Finish, but don't logout.

29

D   Directory    query    (followed   by   optional   file

specification).

U   Disk usage query.

E   Erase (followed by file specification).

T   Type (followed by file specification).

Q   Query server status.

and others.

C   Host Command.  The data field contains a string to be  ex-

ecuted  as  a  system  dependent  (literal) command by the

host.

X   Text display header, to indicate the arrival of text to be

displayed on the screen, for instance as the result  of  a

generic or host command executed at the other end.  Opera-

tion is exactly like a file transfer.

DATA      The  ``contents''  of the packet, if any contents are required

in the given type of  packet,  interpreted  according  to  the

packet  type.  Nonprintable ASCII characters are prefixed with

quote characters and  then  ``uncontrollified''.    Characters

with  the  8th  bit  set  may also be prefixed, and a repeated

character can be prefixed by a count.  A prefixed sequence may

not be broken across packets.

CHECK     The block check sequence, based on all the characters  in  the

packet  between, but not including, the mark and the check it-

self, one, two, or three characters  in  length  as  described

above,  each  character  transformed  by  char.  Normally, the

single-character checksum is used.

The packet may be followed by any line terminator required by the  host,

carriage return (ASCII 15) by default.  Line terminators are not part of

the  packet, and are not included in the count or checksum.  Terminators

are not necessary to the protocol, and are invisible to it, as  are  any

characters  that may appear between packets.  If a host cannot do single

character input from a TTY line, then a terminator will be required  for

that host.

Here are some sample KERMIT data packets:

30

^AE"D   No celestial body has required J

^AE#Das much labor for the study of its#

^AE$D#M#Jmotion as the moon.  Since ClaA

^AE%Dirault (1747), who indicated a way7

^AE&D of#M#Jconstructing a theory conta5

The  ``^A''  represents  the SOH (or Control-A) character.  In the final

packet shown, ``E'' is the length.  The ASCII value of the ``E'' charac-

ter is 69  , less 32 (the unchar transformation, which is  the  opposite

10

of  char) gives a length of 37.  The next character tells the packet se-

quence number, in this case 6 (``&'' is ASCII 38).    The  next  is  the

packet  type ``D'' for Data.  The next characters, `` of#M#Jconstructing

a theory conta'', form the data; note the prefixed carriage  return  and

line feed.  The final character, ``5'' is the checksum, which represents

the number 21 (all numbers in this paragraph are in decimal).

. Effects of Packet Corruption

What  are  the  consequences  of  transmission  errors  in the various

fields?  If the SOH is garbled, the packet will  be  treated  as  inter-

packet  garbage,  and lost.  If any other character within the packet is

garbled into SOH, the current  packet  will  be  discarded,  and  a  new

(spurious)  packet  detected.    If the length is garbled into a smaller

number, then a character from the data field will be  misinterpreted  as

the  checksum;  if  larger,  then the program will probably become stuck

trying to input characters that will not be sent until one side  or  the

other  times  out and retransmits.  If the sequence number, type, any of

31

the  data  characters,  or  the checksum itself is garbled, the checksum

should be wrong.  If characters are lost, there will most  likely  be  a

timeout.   If noise characters are spontaneously generated, they will be

ignored if they are between packets, or will cause the  wrong  character

to  be  interpreted as the checksum if they come during packet transmis-

sion.

Most kinds of errors are caught by the checksum  comparison,  and  are

handled  by  immediate retransmission.  Timeouts are more costly because

the line sits idle for the timeout period.  The packet design  minimizes

the  necessity  for  timeouts  due to packet corruption: the only fields

that can be corrupted to cause a timeout are  the  SOH  and  the  packet

length,  and  the  latter only half the time.  Lost characters, however,

can produce the same effect (as they would  with  a  fixed-length  block

protocol).  Had a distinguished end-of-packet character been used rather

than  a  length  field,  then there would be a timeout every time it was

corrupted.  It is always better to retransmit immediately than  to  time

out.

32

* Protocol

The  KERMIT  protocol can be described as a set of states and a set of

transitions that define, for a given event, what action to take and what

new state to change to.  The inherent simplicity  of  the  design,  par-

ticularly the requirement that each packet must be acknowledged, reduces

the  number  of  states and actions, and the amount of state information

that must be maintained, to a minimum.

Here is a simplified version of a state diagram for KERMIT receiving a

file:

+--------------+

| Receive Init |

+--------------+

|

| (Get Send Init Packet)

|

(Get EOT Packet)         V

+--------------+

(Done) <------------------| Receive File |<---------------+

+--------------+                |

|                       |

| (Get File             | (Get EOF Packet)

|  Header Packet)       |

V                       |

+--------------+                |

+------>| Receive Data |----------------+

|       +--------------+

|               |

|               | (Get Data Packet)

|               |

+---------------+

For simplicity, some transitions are not shown in the diagram:

33

- If  in any state a bad packet is received or a timeout occurs,

a null transition back to the same state occurs,  with  a  NAK

for the expected packet.

- In any state an error may occur that can cause the transfer to

terminate.   For instance, the target disk might fill up.  The

side that encountered the error sends an  error  packet,  con-

taining an informative error message, and quits.  Upon receipt

of  the  error  packet, the other side displays the message on

the screen (if it is in control of the screen) and also quits.

- Actions that are taken on each transition, such as  opening  a

file  when a File Header packet is received, are not shown; in

particular each packet successfully received is ACK'd.

The receiver starts out in Receive Init state and waits for the  other

side  to  send  a  Send-Init  packet.    If  any other kind of packet is

received, or the Send-Init does not arrive within the timeout  interval,

a NAK is sent.  Timeouts or NAKs can occur up to a threshold which, when

exceeded for a particular packet, causes the protocol to assume that the

connection has become unusable, and to give up.  After the Send-Init ar-

rives,  the  state  becomes Receive File; KERMIT waits for a File Header

packet containing the name of the file which is to come.  When the  file

header arrives, KERMIT opens a new file using the name provided (perhaps

transformed  to  suit  local  naming  conventions,  or  to  avoid a name

collision), and switches to Receive Data state.   KERMIT  then  receives

the contents of the file, until an EOF (End Of File) packet arrives.  At

that  point KERMIT switches back to Receive File state.  If another file

is to be sent, another File Header packet will follow, otherwise an  EOT

(End  Of Transmission) packet will terminate the transfer.  The distinc-

tion between EOF and EOT, plus the File Header itself, allows  files  to

34

be  sent in groups.  EOF marks the end of a file, EOT marks the end of a

group.  This  distinction  also  allows  the  two  sides  to  disconnect

cleanly:  the  EOF must be ACK'd before the sender will believe the file

has been transmitted correctly; the EOT will  follow,  but  if  the  ACK

which is sent in response is lost, no harm will be done since both sides

are terminating anyway.

The  state  transitions  for  a  sending  KERMIT are similar.  In each

state, instead of waiting for particular packet types, KERMIT sends  the

appropriate  packet  and  waits  for an ACK.  If the ACK does not arrive

within the allotted time, or a NAK appears instead of an ACK,  the  same

packet  is  retransmitted.    A  send  operation begins with a Send-Init

packet, includes one or more files, each starting with  a  File  Header,

followed  by  one  or  more data packets, followed by EOF.  When all the

specified files have been sent, an EOT packet closes the connection  and

terminates the operation.

35

+-----------+

| Send Init |

+-----------+

|

| (Get Good ACK)

|

(No more files to send)          V

+----------+              +-----------+

| Send EOT |<-------------| Send File |<---------------+

+----------+              +-----------+                |

|                    |

| (Good ACK)         | (EOF)

|                    |

V                    |

+-----------+                |

+------>| Send Data |----------------+

|       +-----------+

|               |

|               | (Good ACK)

|               |

+---------------+

Base-level KERMIT provides that during any particular transaction, the

sender is the ``master'' and the receiver is the ``slave''.  These roles

may  be  reversed  in the next transaction; any KERMIT implementation is

capable of acting as either master or slave.  In addition, mainframe im-

plementations may  also  be  put  in  a  kind  of  permanent  slave,  or

``server'',  mode in which all commands come in command packets from the

master, or ``user'' KERMIT.

36

. Initial Connection

To allow a diverse group of computers to communicate with one another,

an  exchange  takes  places  during  initial connection in which the two

sides ``configure'' each other.    The  sending  KERMIT  includes  setup

parameters  in  its  Send-Init  packet and the receiving KERMIT responds

with an ACK packet containing the corresponding parameters as they apply

to itself.  The Data field of the Send-Init packet looks like this:

1      2      3      4      5      6      7      8      9      10

+------+------+------+------+------+------+------+------+------+-------------

| MAXL | TIME | NPAD | PADC | EOL  | QCTL | QBIN | CHKT | REPT |  CAPAS...

+------+------+------+------+------+------+------+------+------+-------------

The fields are as follows (the first and second person ``I'' and ``you''

are used to distinguish the two sides).  Fields  are  encoded  printably

using the char function (ASCII value + 32  ) unless indicated otherwise.

10

MAXL    The  maximum  length  packet  I  want to receive, a number up to

94  .You respond with the maximum you want me to send.  This al-

10

lows systems to adjust to each other's buffer sizes, or  to  the

condition of the transmission medium.

TIME    The  number  of  seconds  after  which I want you to time me out

while waiting for a packet from me.  You respond with the amount

of time I should wait for packets from you.  This allows the two

sides to accommodate to different line speeds or  other  factors

that could cause timing problems.

NPAD    The number of padding characters I want to precede each incoming

packet;  you  respond  in  kind.  Padding may be necessary for a

half duplex system that requires some time to change the  direc-

tion of transmission.

PADC    The control character I need for padding, if any, XOR'd with 100

octal  to make it printable.  You respond in kind.  Normally NUL

(ASCII 0), some systems use DEL (ASCII 177).  This field is  ig-

nored if the value NPAD is zero.

37

EOL     The  character  I  need to terminate an incoming packet, if any.

You respond in kind.  Most systems  that  require  a  line  ter-

minator  for terminal input accept carriage return for this pur-

pose.  (Can you see the Catch-22 here?)

QCTL    The printable ASCII  character  I  will  use  to  quote  control

characters  and  prefix characters, normally ``#''.  You respond

with the one you will use.

QBIN    The printable ASCII character I want to use to quote  characters

which  have  the 8th bit set, for transmitting binary files when

one or both systems cannot use the parity bit for data.    Since

this  kind  of quoting increases both processor and transmission

overhead, it is normally to be avoided.

CHKT    Check Type, the method for detecting errors.  ``1'' for  single-

character  checksum (the normal method), ``2'' for two-character

checksum, ``3'' for three-character CRC-CCITT.  If your response

agrees, the  designated  method  will  be  used;  otherwise  the

single-character  checksum  will be used.  Other check types may

also be added.

REPT    The prefix character I will use to indicate a  repeated  charac-

ter.    This  can  be  any  printable character other than blank

(which denotes no repeat count  prefix),  but  ``~''  is  recom-

mended.    If  you don't respond identically, repeat counts will

not be done.  Groups of 4 or more identical  characters  may  be

transmitted more efficiently using a repeat count, though an in-

dividual implementation may wish to set a higher threshhold.

CAPAS   An  extendable  bit  mask encoded printably, to indicate whether

certain advanced capabilities, such as  file-attribute  packets,

are supported.

Reserved Fields

The next four fields are reserved for future use.  Sites wishing

to  add  their own parameters to the initial connection exchange

should start at the fifth field after the capability mask in or-

der to remain compatible with other KERMIT programs.

Naturally, the three prefix characters must be distinct  and  should  be

chosen  to  be uncommonly used printable characters, to minimize further

overhead from having to prefix them when they are found in the data.

Trailing fields within the DATA field may be omitted,  in  which  case

they will assume appropriate defaults.  Defaults for intermediate fields

can be elected by setting those fields to blank.  Every parameter has an

38

appropriate  default, and in fact the entire data field of the Send Init

packet or its ACK may be left empty to accept all defaults.    The  more

exotic  paramaters  are at the end, and reflect more recent developments

in the KERMIT protocol; earlier implementations  can  still  communicate

with newer ones, since there will not be agreement to use these options.

The  Send-Init  mechanism preserves compatibility from the very earliest

KERMIT to the very newest.

There is no protracted negotiation; everything must be  settled  in  a

single  exchange.    Some  parameters, however, are outside the scope of

this exchange and must be set even before the very first packet is sent.

For instance, if the receiving computer can only  read  characters  with

odd  parity  but  the  sending computer sends them with even parity, the

Send-Init packet will never arrive successfully.  In  cases  like  this,

the  user  may  have to issue some preliminary commands to inform one or

both KERMITs about the vagaries of the other system.  Another example is

the packet terminator (EOL) mentioned above -- if the  receiving  KERMIT

requires  one  that the sending KERMIT doesn't know about, the Send-Init

will never get through.

For these reasons, most implementations of KERMIT provide SET commands

for all the parameters listed above, and some others as well.

39

. Rules and Heuristics

During  a  file transfer, one KERMIT sends information packets -- file

headers, data, and so forth, and the other KERMIT  sends  only  ACKs  or

NAKs in response.  The most important rule in the KERMIT protocol is

1. ``Wait for a response before sending the next packet.''

This  prevents  buffer overruns, and allows participation by half duplex

systems.  Of course, KERMIT should not wait forever;  a  timeout  should

occur  after a few seconds if the expected packet has not arrived.  Upon

timeout, a sending KERMIT retransmits the current  packet;  a  receiving

KERMIT re-ACKs the current packet or NAKs the expected one.

Some  interesting  heuristics are used in the KERMIT protocol to boost

efficiency and improve error recovery.  A number of important rules take

care of the cases when packets are lost in transmission.  The first  can

be stated as

2.  ``A NAK for the next packet implies an ACK for the current

packet.''

A NAK with packet number n+1 means the receiving KERMIT  got  packet  n,

sent an ACK that was never received, and not knowing that the ACK didn't

get  through (since we don't ACK an ACK), is now waiting for packet n+1,

which was never sent.  In this case, we simply send packet n+1.  An  im-

portant  exception is when the missing ACK is for a Send-Init packet; do

40

you see why?  The next rule,

3. ``ACK and discard redundant packets.''

handles  the situation where the same packet arrives again.  The sending

KERMIT timed out waiting for an  ACK  which  was  sent,  but  lost,  and

retransmitted  the packet.  The receiver must discard the redundant data

and ACK the packet again; to do otherwise  could  have  undesirable  ef-

fects,  like  adding  the  same data to a file twice or opening the same

file multiple times.  Note that the situation resulting from a lost  ACK

depends upon which side times out first.

KERMIT  must handle another situation arising from possible lost pack-

ets:

4. ``NAK the expected command.''

The potential problem occurs in either the Receive Init state or when  a

KERMIT  server  is  waiting  for a command.  In either case KERMIT won't

know whether communication has begun if the other side's initial  packet

was  lost.    KERMIT  can't  assume  the  other  side  will time out and

retransmit, so it must check periodically by sending a  NAK  for  packet

zero.    If  KERMIT gets no response it assumes nothing has happened yet

and goes back to sleep for a while.

But sending periodic NAKs opens the door  to  the  buffering  problem.

Some systems buffer input for a device; when a program isn't looking for

input  some  or  all  the  input is saved by the computer for future re-

41

quests.    This  can  cause  problems when talking to a KERMIT server or

sending a file to a KERMIT in receive wait.  If some  time  has  elapsed

since activating the remote KERMIT and escaping back and starting up the

local  KERMIT,  a  number  of  NAKs  may  have  accumulated in the local

KERMIT's input buffer, so:

5. ``Clear the input buffer at the beginning of a transfer.''

If the input buffer is not cleared, the  local  KERMIT  will  think  the

remote  side is having trouble receiving the first packet.  In an effort

to get the packet through, it will be sent again; this repeats for every

NAK waiting in the input buffer.  By the time the first ACK  is  finally

encountered  in  the  buffer, a number of duplicates of the first packet

will have been sent out.  If this number exceeds the NAK threshhold, the

connection will be broken.  If not, however, the second packet  will  be

retransmitted  once  for  each  of the extra ACKs the remote KERMIT cor-

rectly sent for the duplicate first packets.  This can  continue  throu-

ghout the file transfer, causing each packet to be sent many times.  So,

in addition,

6. ``Clear the input buffer after reading each packet.''

Any computer that buffers its input should clear its input buffer before

the transfer and after each packet that arrives successfully.  But since

not  all  systems  provide  a  clear-buffer function, we may add another

rule:

42

7. ``Discard redundant ACKs.''

In the situation just described, the first packet would be sent out mul-

tiple times, once for each buffered NAK.  Upon receipt of the first ACK,

the  second  packet would go out, but the next response would be another

ACK for the first packet; by rule 7, KERMIT would  simply  take  in  the

redundant  ACK,  ignore  it, and look for the next ACK, until it got the

desired one, in violation of the spirit of Rule 1.

If we allowed ourselves to violate Rule 1, we could add a final  rule,

``An  ACK  for  packet n also ACKs all previous packets,'' as is done in

network protocols like DDCMP [16], allowing data packets to be sent in a

continuous stream.  KERMIT cannot use this rule for  many  reasons:  se-

quence number wraparound, buffer overflows, locking out half duplex sys-

tems  (how  can  they  NAK a bad packet if they can't get control of the

line?).  Thus if we violate rule 1, it must be only in a very minor way.

. Example

Here is a sequence of packets from a real file transfer.  Each  packet

starts with Control-A, shown as ^A.

43

^A) SH( @-#^                                  Send Init

^A) YH( @-#%                                  ACK for Send Init

^A+!FMOON.DOC2                                File Header

^A#!Y?                                        ACK for File Header

^AE"D   No celestial body has required J      First Data packet

^A#"Y@                                        ACK for first Data packet

^AE#Das m%%%uch labor for the study of its#   Second Data packet, Bad

^A##N8                                        NAK for second Data packet

^AE#Das much labor for the study of its#      Second Data packet again

^A##YA                                        ACK for second Data packet

^AE$D#M#Jmotion as the moon.  Since ClaA      etc...

^A#$YB

(many packets omitted here)

^AD"Dout 300 terms are sufficient.#M#JU       Last Data packet

^A#"Y@                                        ACK for last Data

^A##ZB                                        EOF

^A##YA                                        ACK for EOF

^A#$B+                                        EOT

^A#$YB                                        ACK for EOT

In  the  first  packet, we see following the control-A the packet length

``)'' (41  , less 32, or 9), followed by the packet type, S (Send-Init),

10

followed by  the  appropriate  parameter  declarations:  maximum  packet

length  is  H  (72-32=40), timeout is ``('' (40-32=8), number of padding

characters is 0 (space=32-32=0), the padding character is 0, end-of-line

is  ``-''  (45-32=13,  the  ASCII  value  of   carriage   return),   the

control-quote  character is ``#'', and the remaining fields are omitted,

defaulting to appropriate values.  The  final  character  ``^''  is  the

single-character checksum, computed as follows (all numbers and computa-

tions in octal, and ``sp'' represents a space):

)    sp   S     H     (    sp   @     -    #

51 + 40 + 123 + 110 + 50 + 40 + 100 + 55 + 43 = 674

44

674 + (674/300) = 676

676 AND 77 = 76;

char(76) = 76+40 = 136 = "^"

The  receiver  ACKs  with  its own parameters, which are the same.  Then

comes the file header, the file, EOF, and EOT.  One data packet was cor-

rupted by a burst of ``%'' characters, NAK'd, and retransmitted.

* Performance

For text files (documents or program source), assuming an average line

length of 40 with lines separated by  a  carriage-return/linefeed  pair,

the  only  control  characters  normally  found in the text file, we see

about 5% overhead for prefixing of control characters.  Assuming no line

terminators for packets (although one or both sides may  require  them),

no  retransmissions or timeouts, and no time wasted for the line to turn

around between packet and response, then for average  packet  length  p,

using a single-character checksum, the KERMIT protocol overhead consists

of:

5 control field characters in the data packet

5 characters in the acknowledgement packet

+ 0.05p for control character quoting

This  gives  10/p  +  0.05 overhead.  E.g. if the packet length is 40,

there is 30% overhead.  If p is 96 (the maximum), there  is  about  15%.

45

These  figures  will vary with the average line length and the frequency

of other control characters (like tabs and formfeeds) in the  file,  and

will  go  up  with  immediate  retransmissions,  and way up with delayed

retransmissions.  For binary files, the quoting overhead will be higher.

But transmission overhead can also go down dramatically if prefix encod-

ing is used for repeated characters, depending on the nature of the data

(binary data containing many zeroes, highly indented or columnar data or

program text will tend to benefit).  Each  file  transfer  also  gets  a

fixed  overhead  for  the  preliminary (Send Init, File Header) and ter-

minating (EOF, EOT) packets.

If the mainframe end of a connection is heavily loaded,  it  may  take

considerable  time  to  digest  and  process  incoming characters before

replying.  On half duplex mainframes, there may be a pause between send-

ing and receiving, even if the load is light; this might be used to  ad-

vantage  by  preparing  the next packet in advance while waiting for the

current ACK.  Another problem may occur  on  heavily  loaded  mainframes

--  undesirable timeouts.  Timeouts are intended to detect lost packets.

A heavily loaded system may take longer than  the  timeout  interval  to

send  a  packet.  For this reason, mainframe KERMITs should take the re-

quested timeout interval only as a minimum, and  should  adjust  it  for

each  packet  based  on the current system load, up to a reasonable max-

imum.

On a noisy line, there is a greater likelihood  of  corrupted  packets

and  therefore  of  retransmission overhead.  Performance on noisy lines

46

can  be improved by reducing the packet length, and thus the probability

that any particular packet will be corrupted, and the amount of time re-

quired  to  retransmit  a  corrupted  packet.    A  KERMIT  program  can

unilaterally  adjust  the  packet  length  according  to  the  number of

retransmissions  that  are  occurring.    Short  packets  cut  down   on

retransmission overhead, long packets cut down on character overhead.

* ``User Interface''

KERMIT was designed from a mainframe perspective.  Like many mainframe

programs,  KERMIT  issues a prompt, the user types a command, KERMIT ex-

ecutes the command and issues another prompt, and so on until  the  user

exits  from  the  program.   Much care is devoted to the command parser,

even on microcomputer versions.  The goal  is  to  provide  English-like

commands  composed  of sequences of keywords or operands, with abbrevia-

tions possible for any keyword in any field down to the  minimum  unique

length,  and  with  ``?'' help available at any point in a command.  Not

all implementations need follow this model, but most do.

The basic commands are SEND and RECEIVE.  These allow most KERMITs  to

exchange  files.    Operands can be the name of a single file, or a file

group designator (e.g. with ``wildcards'') to transmit multiple files in

a single operation.  Although some systems may not provide wildcard file

processing, the KERMIT protocol allows it.

The CONNECT command provides the mechanism for logging in  and  typing

47

commands  at  the  remote host, which is necessary in order to start the

KERMIT on that side.  The CONNECT facility provides  character-at-a-time

transmission,  parity selection, remote or local eching, and the ability

to send any character, including the ``escape character'' that  must  be

used to get back to the local KERMIT.  However, there is no error detec-

tion  or  correction  during CONNECT, just as there normally is none be-

tween an ordinary terminal and a host.

When two systems are dissimilar, a SET command is  provided  to  allow

them  to accommodate each other's peculiarities, for instance SET PARITY

ODD to add odd parity to all outbound characters, or SET  LOCAL-ECHO  to

do  local  echoing when connected as a terminal to a half duplex system.

The SET command must sometimes be used to supply information to the tar-

get system on how to store an incoming file with respect to block  size,

byte size, record format, record length.

Most  KERMIT  implementations  take  special care to reassure the user

during file transfer.  The names of  the  files  being  transferred  are

shown, and a dynamic display is made of the packet traffic, showing suc-

cessful transmission of packets as well as timeouts and retransmissions.

Messages  are  issued when the user connects to the remote system or es-

capes back from it, and  KERMIT  prompts  identify  the  implementation.

Helpful  error  messages are displayed when necessary; these may emanate

from either the local or the remote system.  The  final  disposition  of

the transfer is clearly stated, complete or failed.

The  actions  required  of  the  KERMIT user depend upon the degree to

48

which  the  KERMIT programs involved have implemented the specification.

Minimal implementations require that the  user  connect  to  the  remote

host, start KERMIT there, issue a SEND (or RECEIVE) command, escape back

to the local machine, and issue the complementary RECEIVE (or SEND) com-

mand.  All this must be done for each transfer.  More advanced implemen-

tations allow the remote side to run as a ``server'' and to take all its

instructions  in special command packets from the local KERMIT; all that

is required of the user on the remote end is to connect initially in or-

der to start the server.  The server will even log itself out upon  com-

mand  from  the  local KERMIT.  A minimal server can process commands to

send files, receive files, and shut itself down.

Here is an example of a session in which the user of an  IBM  PC  gets

files  from  a DECSYSTEM-20.  The actions shown are required for minimal

KERMIT implementations.  The parts the user types are  underlined,  com-

ments  begin with ``!'' or appear in italics.  Everything else is system

typeout.

A>kermit              ! Run Kermit on the PC.

Kermit V1.20

Kermit-86>            ! This is the Kermit prompt for the PC.

Kermit-86>connect     ! Connect to the DEC-20.

[Connecting to host. Type CTRL-]C to return to PC.]

! You are now connected to the DEC-20.

Columbia University CU20B                   ! The system prints its herald.

@terminal vt52        ! Set your terminal type (optional).

@login my-id password ! Login using normal login method.

(The DEC-20 prints various messages.)

49

@kermit               ! Run Kermit on the DEC-20.

Kermit-20>            ! This is Kermit-20's prompt.

Kermit-20>send *.for  ! Send all FORTRAN files.

^]c                   ! Type the escape sequence to return to the PC.

[Back at PC.]

Kermit-86>receive     ! Tell the PC files are coming.

(The progress of the transfer is shown continuously on the screen.)

Transfer Complete.

Kermit-86>connect     ! Get back to the DEC-20.

[Connecting to host. Type CTRL-]C to return to PC.]

Kermit-20>exit        ! Get out of Kermit-20.

@logout               ! Logout from the DEC-20.

Logged out Job 55, User MY-ID, Account MY-ACCOUNT, TTY 146,

at 24-Apr-83 15:18:56,  Used 0:00:17 in 0:21:55

^]c                   ! Now "escape" back to the PC,

[Back at PC.]

Kermit-86>exit        ! and exit from the PC's Kermit.

The session is somewhat simpler when the remote KERMIT is being run as

a  server.  The user must still CONNECT, log in, and start KERMIT on the

remote end, but need never  again  CONNECT  to  issue  subsequent  SEND,

RECEIVE,  EXIT,  or  LOGOUT  commands, even though many transactions may

take place.  All actions can be initiated from the PC.

50

* Advanced Features

An  optional  feature  of  the  KERMIT  protocol  is  a special packet

designed to express the attributes of a file in a  compact  and  generic

manner.    The  receiver may either attempt to use the attributes to the

incoming file, or archive them for later use.   Attributes  include  not

only  file  characteristics  but also the intended disposition -- store,

print, submit for batch processing, send as mail, etc.   Other  optional

features  include  mechanisms  for gracefully interrupting, delaying, or

suspending operations in progress; alternate forms of packet data encod-

ing; filename conversion, local file management, raw  data  transmission

and capture; command macro definition, etc.

Although  KERMIT  was  never intended to fulfill the role of a general

purpose network server, its design has made it simple to add  new  func-

tions.    A  KERMIT server has the ability to accept commands in packets

from a remote KERMIT.  The basic commands are for  sending  or  fetching

files and for shutting down the server.  Other commands may require dis-

play  of  text  at the user's terminal, which is controlled by the local

KERMIT.  For example,  a  directory  listing  could  be  requested;  the

resulting  text  is  sent  back to the local KERMIT exactly as if a file

were being transferred, except the destination  is  the  user's  screen,

rather  than  a  disk  file.  (Or it could be disk file too.)  With this

ability in place, it is possible to implement all sorts of commands, for

instance to delete a file, show who's  logged  in,  inquire  about  disk

51

space,  verify  access to a directory, submit batch jobs, send messages,

and so forth.

The ability of the KERMIT server to perform host functions can be  ad-

ded  very  simply  under certain operating systems.  For instance, under

UNIX [17], KERMIT can ``fork'' a shell  with  commands  to  perform  any

function  possible under UNIX, redirecting the standard output through a

process (KERMIT itself) that encapsulates it  into  KERMIT  packets  and

sends it along.

A  server with these capabilities could provide convenient access to a

timesharing system by users at personal workstations, without  requiring

the  users  to  be  directly  involved with the host.  If, for instance,

workstations had dedicated connections to a host, and the host had dedi-

cated KERMIT servers for each such line, users could get access  to  and

manage their host files completely by commands typed at the workstation.

Taking this idea one step further, the workstation system software could

be  modified  to  make  the whole process transparent by incorporating a

KERMIT-like protocol in its file access logic -- fetching  and  updating

host  files  as  necessary  behind  the  user's back.  Since the demands

placed on a host by KERMIT are relatively modest, many more simultaneous

users could probably be serviced in this way.  This approach could be  a

relatively  painless  entree into the distributed, networked environment

of tomorrow.  When local area network protocols become  mature  and  the

hardware economical and widespread, KERMIT can be replaced by ``the real

thing''.   But for the ordinary computer user for whom dedicated connec-

52

tions  are impractical, ``do-it-yourself'' KERMIT, or some facility like

it, will be a valuable tool for years to come.

* Conclusion

The need for  a  cheap,  convenient  file  transfer  capability  among

diverse  systems  is  pressing,  and  there  are  certainly many efforts

similar to ours under way at many places.  We hope that this article may

contribute to those efforts; we don't claim to have the last word on any

of the issues raised here, and expect that this article may  flush  some

other  approaches  out  of  the  woodwork.    We have billed KERMIT as a

``simple'' protocol; anyone who has read this  far  will  begin  to  ap-

preciate  what  must go into the more complicated protocols used in real

networks, or when ``integration'' of micro and mainframe is a major goal

-- demand paging of remote files, remote database  queries,  distributed

editing and computation.

Meanwhile,  the  KERMIT  protocol has been proven successful, and con-

tinues to grow in popularity.  As of this writing, implementations exist

for more than 50 computer systems.  Some of the major ones include:

Machine                Operating System       Language

DECsystem-10           TOPS-10                MACRO-10

DECSYSTEM-20           TOPS-20                MACRO-20

IBM 370 Series         VM/CMS                 IBM Assembler

VAX-11                 VMS                    Bliss-32

VAX, SUN, PDP-11, etc  UNIX                   C

PDP-11                 RT-11, RSX, RSTS       MACRO-11

8080, 8085, or Z80     CP/M                   8080 ASM

8086, 8088             PC DOS, MS DOS         IBM PC Assembler

Apple II 6502          Apple DOS              DEC-10/20 CROSS

53

Some  of  these  have  been  contributed or enhanced by the institutions

listed in the acknowledgements, below.  No single implementation  neces-

sarily  includes all the features mentioned in this article, but all are

able to communicate at least at base level.  Additional  implementations

are in preparation, and present ones are being enhanced.

Columbia  University  is  willing  to  provide  all  KERMIT  programs,

sources, manuals, and other documentation to computing centers, academic

or corporate, in return for a modest  fee  to  cover  costs  for  media,

printing,  postage,  labor, and computing resources.  Only magnetic tape

and listings can be shipped.  We cannot produce floppy  disks;  instruc-

tions  are  included for bootstrapping the microcomputer implementations

from the mainframe computers.  Details  will  be  provided  on  request;

write to:

KERMIT Distribution

Columbia University Center for Computing Activities

7th Floor, Watson Laboratory

612 West 115th Street

New York, NY  10025

The protocol specification supplemented by examples of existing KERMIT

implementations  allows  new implementations to be created with relative

ease.  In the past, KERMIT implementors  have  shared  their  work  with

other  KERMIT  users  by contributing it to the Columbia KERMIT library.

We hope that this practice will continue until KERMIT has spread through-

out the known world.

54

* Acknowledgements

In  designing  the initial KERMIT protocol, we studied several models,

primarily  the  ANSI  recommendation [9].  other  include  the Stanford

University  DIALNET  project,  the  University  of  Utah  ``Small  FTP''

project,  and  the  Stanford  University  Medical Center TTYFTP project.

And we examined some real networks, like ARPANET and DECnet.

Acknowledgements also to the many sites that have contributed new KER-

MIT  implementations or enhanced old ones: Stevens Institute of Technol-

ogy, Digital Equipment Corporation, the National Institutes  of  Health,

Cornell  University,  the  University of Toronto, the University of Ten-

nessee, the University of Toledo, Cerritos College, and others.   Thanks

to Dr. Howard Eskin for help with this article.

55

* The ASCII Alphabet

Dec Oct Character  Dec Oct

000 000 NUL (^@)   064 100 @

001 001 SOH (^A)   065 101 A

002 002 STX (^B)   066 102 B

003 003 ETX (^C)   067 103 C

004 004 EOT (^D)   068 104 D

005 005 ENQ (^E)   069 105 E

006 006 ACK (^F)   070 106 F

007 007 BEL (^G)   071 107 G

008 010 BS  (^H)   072 110 H

009 011 HT  (^I)   073 111 I

010 012 LF  (^J)   074 112 J

011 013 VT  (^K)   075 113 K

012 014 FF  (^L)   076 114 L

013 015 CR  (^M)   077 115 M

014 016 SO  (^N)   078 116 N

015 017 SI  (^O)   079 117 O

016 020 DLE (^P)   080 120 P

017 021 DC1 (^Q)   081 121 Q

018 022 DC2 (^R)   082 122 R

019 023 DC3 (^S)   083 123 S

020 024 DC4 (^T)   084 124 T

021 025 NAK (^U)   085 125 U

022 026 SYN (^V)   086 126 V

023 027 ETB (^W)   087 127 W

024 030 CAN (^X)   088 130 X

025 031 EM  (^Y)   089 131 Y

026 032 SUB (^Z)   090 132 Z

027 033 ESC (^[)   091 133 [

028 034 FS  (^\)   092 134 \

029 035 GS  (^])   093 135 ]

030 036 RS  (^^)   094 136 ^

031 037 US  (^_)   095 137 _

032 040     (SP)   096 140 `

033 041 !          097 141 a

034 042 "          098 142 b

035 043 #          099 143 c

036 044 $          100 144 d

037 045 %          101 145 e

038 046 &          102 146 f

039 047 '          103 147 g

040 050 (          104 150 h

041 051 )          105 151 i

042 052 *          106 152 j

56

043 053 +          107 153 k

044 054 ,          108 154 l

045 055 -          109 155 m

046 056 .          110 156 n

047 057 /          111 157 o

048 060 0          112 160 p

049 061 1          113 161 q

050 062 2          114 162 r

051 063 3          115 163 s

052 064 4          116 164 t

053 065 5          117 165 u

054 066 6          118 166 v

055 067 7          119 167 w

056 070 8          120 170 x

057 071 9          121 171 y

058 072 :          122 172 z

059 073 ;          123 173 {

060 074 <          124 174 |

061 075 =          125 175 }

062 076 >          126 176 ~

063 077 ?          127 177 DEL

57

* References

The  following  publications  provided useful guidance or diversion in

the development of KERMIT.

[1]   EIA Standard RS-232-C

Electronic Industries Association, 2001 Eye Street N.W.,

Washington DC 20006, 1969, 1981.

[2]   ANSI X3.4-1977, Code for Information Interchange

American National Standards Institute, 1430 Broadway, NYC 10018,

1977.

[3]   ANSI X3.15-1976,

Bit Sequencing of ASCII in Serial-By-Bit Data Transmission

1976.

[4]   ANSI 3.16-1976, Character Structure and Character Parity Sense for

Serial-By-Bit Data Communication in ASCII

1976.

[5]   McNamara, J.E.

Technical Aspects of Data Communication.

Digital Press, 1982.

[6]   Mackenzie, C.E.

Coded-Character Sets: History and Development.

Addison-Wesley, 1980.

[7]   DNA Data Access Protocol (DAP) Functional Specification

Digital Equipment Corporation, 1980.

AA-K177A-TK.

[8]   Neigus, N.J.

File Transfer Protocol for the ARPA Network

Bolt Beranek and Newman, Inc., 1973.

RFC 542, NIC 17759.  Available in the ARPANET Protocol Handbook,

NTIS AD/A-052 594.

[9]   ANSI X3.28-1976, Procedures for the Use of Control Characters of

ASCII in Specified Data Communications Links

1976.

58

[10]  Pierce, J.R., and Posner, E.C.

Applications of Communications Theory:  Introduction to Communica-

tion Science and Systems.

Plenum Press, 1980.

[11]  Knuth, D.E.

The Art of Computer Programming.  Volume I:  Fundamental Al-

gorithms.

Addison-Wesley, 1973.

[12]  Hamming, R.W.

Error Detecting and Error Correcting Codes.

Bell System Technical Journal 29:147-160, April, 1950.

[13]  Martin, James.

Teleprocessing Network Organization.

Prentice-Hall, 1970.

[14]  Perez, Aram.

Byte-wise CRC Calculations.

IEEE MICRO 3(3):40-50, June, 1983.

[15]  ISO Reference Model for Open Systems Interconnection (OSI)

International Organization for Standardization (ISO), 1982.

Draft Proposal 7498.

[16]  DNA Digital Data Communications Message Protocol (DDCMP) Func-

tional Specification

Digital Equipment Corporation, 1980.

AA-K175A-TK.

[17]  Thomas, R. and Yates, J.

A User Guide to the UNIX System.

OSBORNE/McGraw-Hill, 1982.