KERMIT USER GUIDE

Seventh Edition

Christine Gianone, Editor

Columbia University Center for Computing Activities

New York, New York 10027

May 26, 1988

Copyright (C) 1981,1988

Trustees of Columbia University in the City of New York

Permission is granted to any individual or institution to use, copy,

or redistribute this document so long as it is not sold for profit, and

provided this copyright notice is retained.

. PREFACE                                                                Page 1

-------------------------------------------------------------------------------

PREFACE

Kermit  is  the  name of a protocol for transferring files from one computer to

another over ordinary asynchronous terminal connections.  Kermit programs  have

been  written  for  many  different computers, and in general any two computers

that have Kermit programs can exchange  sequential  files  correctly  and  com-

pletely.   This manual gives a brief and general overview of what Kermit is and

how to use it, but consists mostly of detailed instructions for use and instal-

lation of specific Kermit programs.

For  a  more  detailed  introduction  to  Kermit,  complete with illustrations,

diagrams, and tutorials, consult the book Kermit, A File Transfer Protocol,  by

Frank  da Cruz, Digital Press, Bedford MA (1987), ISBN 0-932376-88-6, DEC order

number EY-6705E-DP (phone 1-800-343-8321).  The Kermit book describes Kermit in

detail, from the points of view of the beginner, the user, the computer profes-

sional who must install Kermit programs or support their use, and the  program-

mer  who wishes to write new Kermit implementations.  Also included are general

introductions to computers, data communications, and file organization, plus  a

detailed troubleshooting guide, bootstrapping hints, and various appendices and

tables.  The latter half of the book is taken up by a complete  description  of

the Kermit file transfer protocol, with programming examples in the C language,

plus some analysis and comparisons of Kermit with other popular protocols  such

as Xmodem.

WHY IS THIS MANUAL SO THICK?

Don't  panic, Kermit isn't that complicated!  The Kermit User Guide begins with

a relatively short introduction to Kermit in general -- what it's for,  how  to

use  it,  what  the  commands are, etc -- in Chapters 1 through 4.  Each of the

remaining chapters describes a specific Kermit programs in detail:    one  each

for  MS-DOS, UNIX, IBM mainframes, VAX/VMS, DEC-20, DEC PDP-11, Apple II, Apple

Macintosh, and CP/M.  You only need to use the chapters that apply to the  sys-

tems you're actually using.

In  general,  the  longer  the chapter, the more powerful the version of Kermit

it's describing: MS-DOS, PDP-11, etc.  But remember, a major goal of Kermit  is

that  it  should  be  easy  for you to get started, so the bulk of the verbiage

covers the "odd cases" that many people won't ever encounter, and the  advanced

features that are available for those who want to learn about them.

The  increase  in  size from the sixth to the seventh edition of this manual is

mostly because of the growth of the MS-DOS, IBM mainframe,  and  several  other

chapters, plus the addition of the PDP-11 chapter.

HISTORY AND ACKNOWLEDGEMENTS

The  Kermit file transfer protocol was designed at the Columbia University Cen-

ter for Computing Activities (CUCCA) in 1981-82 by  Bill  Catchings  and  Frank

da Cruz.    Bill wrote the first two programs, one for the DECSYSTEM-20 and one

for a CP/M-80 microcomputer.

The initial objective was to allow users of our DEC-20 and IBM 370  timesharing

systems  to archive their files on microcomputer floppy disks.  The design owes

much to the ANSI and ISO/OSI models, and some ideas were borrowed from  similar

. PREFACE                                                                Page 2

-------------------------------------------------------------------------------

projects  at  Stanford University and the University of Utah.  The protocol was

designed to accommodate the "sensitive" communications front end of  the  full-

duplex  DEC-20 system as well as the peculiarities of half-duplex IBM mainframe

linemode communications.  The protocol was soon implemented successfully on our

IBM mainframe systems under VM/CMS by Daphne Tzoar of CUCCA.

Meanwhile  it  was  becoming apparent that Kermit was useful for more than just

file archiving; IBM PCs were beginning to appear in  the  offices  and  depart-

ments,  and there arose a general need for file transfer among all our systems,

as well as a need to use the IBM PCs as terminals.   Before  long,  Daphne  had

prepared an IBM PC implementation.

After  our  initial success with Kermit, we presented it at conferences of user

groups like DECUS and SHARE, and began to get requests for it from other sites.

Since we had written down a description of the protocol, some sites wrote their

own implementations for new computers, or adapted one of our implementations to

run  on  additional  systems, and sent back these new versions to us so that we

could share them with others.  In this way, Kermit has grown  to  support  over

300 different machines and operating systems; it has been sent on magnetic tape

or diskette from Columbia University to about ten thousand sites all  over  the

world, and has reached many thousands more through user groups and networks.

Thanks  to the hundreds of individuals and institutions who have contributed to

the Kermit storehouse over the years.

The Kermit protocol was named after Kermit the Frog,  star  of  the  television

series  THE  MUPPET  SHOW;  the name Kermit is used by permission of Henson As-

sociates, Inc., New York.

DISCLAIMER

Neither Columbia University, nor the editor, nor the authors of the  individual

chapters,  nor  any  individual  or institution contributing Kermit programs or

documentation to the Columbia University Kermit Distribution,  acknowledge  any

liability  for  any claims arising from use or misuse of Kermit programs or for

inaccuracies in the documentation or bugs in the programs.  Kermit programs are

produced on a voluntary basis and contributed freely for public use in the hope

that they will be useful, but without any kind of warranty or guarantee, or any

commitment  to  address  or  fix  problems.    In practice, Kermit programs and

documentation are contributed in good faith, and will be supported on  a  best-

effort basis, time and other commitments permitting.

CUSTOMIZING THIS MANUAL

Although  the  Kermit  User  Guide was produced at Columbia University, all at-

tempts have been made to keep it free of site-specific information.    However,

due  to  the  large  number of Kermit implementations, descriptions of each one

would make the manual prohibitively thick.  Therefore, the manual is sent  from

Columbia  with  specific  documentation  about a selection of systems.  Some of

these descriptions may not be of interest at your site, while others  that  are

may be lacking.

Each  site,  upon  receiving a Kermit tape, may decide which versions of Kermit

are important to it, and include the appropriate documentation in this  manual.

. PREFACE                                                                Page 3

-------------------------------------------------------------------------------

This is most conveniently done if your site has the Scribe text formatting sys-

tem (from UNILOGIC Ltd in Pittsburgh PA,  USA),  with  which  this  manual  was

produced.    Scribe  runs  on a wide variety of systems.  There are also Scribe

subsets, such as Perfect Writer and Final Word, that run on  various  microcom-

puters.   Many have asked why Scribe is used for Kermit manuals instead of TeX.

The answer is simply that TeX can only  produce  output  for  typesetters,  not

plain-text ASCII files, which are necessary for online documentation.

The system-specific parts of the Kermit User Guide are included with "@INCLUDE"

statements at the end of the Scribe source file for this manual, whose filename

is  KUSER.MSS.    You may add or delete @INCLUDE statements to suit your needs,

and run the result through the text formatter to produce a  customized  manual.

If  you  do  this,  you should include an indication on the title page that the

manual has been customized for your site.

Not all system-specific documentation is provided in .MSS (Scribe  input)  for-

mat, since some Kermit contributors do not have Scribe at their sites.  In that

case, you will either have to add Scribe formatting commands, or  else  enclose

the  whole  subfile  in @BEGIN(VERBATIM)...@END(VERBATIM) brackets (and replace

all atsigns (@) in the text with double atsigns (@@)).

If you do not have SCRIBE, you may still use an editor to delete  or  add  sec-

tions  to  the  finished  documentation file, though the results will not be as

satisfactory -- the table of contents, index, cross references, and  page  num-

bers will not be automatically adjusted.

If you are running a version of Kermit for which adequate documentation has not

been provided (after all, this is a  distributed,  volunteer  effort!),  please

feel free to write some, preferably in Scribe input format, and send it back to

Columbia so that others may benefit from it.  Likewise if you produce a new im-

plementation  of  Kermit.  If you don't know Scribe, you can use one of the ex-

isting chapters as a model.

. How To Get Kermit                                                      Page 4

-------------------------------------------------------------------------------

How To Get Kermit

The Kermit software is free and available to all, source code and documentation

included,  from  a variety of sources.  For example, most universities are con-

nected to academic computer networks from which the Kermit  files  at  Columbia

University  can be reached.  The Kermit files are also available from many user

groups, dialup information or bulletin board  services,  diskette  reproduction

services, and private volunteers.

Kermit  software is not in the public domain.  The Kermit manuals and most Ker-

mit programs bear copyright notices to  protect  Columbia  University  and  the

various  contributors  from  having  their  work  taken by others and sold as a

product, for profit.  This is not to say that the Kermit file transfer protocol

can  never be included as a feature of a commercial product; the conditions un-

der which this may be done are spelled out in a flyer POLICY ON COMMERCIAL  USE

AND DISTRIBUTION OF KERMIT.

Columbia  University  distributes Kermit programs by mail order on various mag-

netic  media  (primarily  9-track  reel-to-reel  tape  and  certain  kinds   of

diskettes),  charging  a  distribution fee to defray costs for media, printing,

postage, materials, labor, and computing resources.  This  is  not  a  software

license  fee;  no license is required.  To receive a current list of Kermit im-

plementations, the statement on commercial policy, and  a  Kermit  order  form,

write to:

Kermit Distribution

Columbia University Center for Computing Activities

612 West 115th Street

New York, NY  10025

Everyone  is  free  to copy and redistribute Kermit programs and documentation,

and is encouraged to do so, with the following  stipulations:  Kermit  programs

should not be sold for commercial gain; credit should be given where it is due;

and new material should be sent back to  Columbia  University  at  the  address

above  so that we can maintain a definitive and comprehensive set of Kermit im-

plementations for further distribution.

Since new Kermit programs are added -- and old ones improved -- so  frequently,

sites  that  use  Kermit heavily are encouraged to contact Columbia for updates

two or three times a year for news.

-- PLEASE USE KERMIT ONLY FOR PEACEFUL AND HUMANE PURPOSES --

. Organization of This Manual                                            Page 5

-------------------------------------------------------------------------------

Organization of This Manual

Chapter 2, How to Use Kermit, describes the basics of text file  transfer,  and

shows  some specific examples.  If you follow the examples but you can't make a

terminal connection or you can't transfer files successfully,  consult  Chapter

3, When Things Go Wrong.

If  you  expect to be a heavy user of Kermit, you should read Section 4, Kermit

Commands, which describes most of Kermit's features and commands.  You may find

that  familiarity with the material in this section will help you get past dif-

ficulties that can crop up when you are making  new  kinds  of  connections  or

transferring  unusual  kinds of files.  You will also find descriptions of some

advanced file management features that have been omitted from the earlier  sec-

tions.

The  subsequent  chapters  describe selected popular Kermit programs in detail.

You should read the appropriate section for each system you expect to use; each

section  describes  the  file naming conventions and other system features that

are important to Kermit users, and lists the Kermit commands  for  that  system

mainly  in terms of their differences from the "ideal" Kermit described in sec-

tion 4.

1. Introduction                                                          Page 6

-------------------------------------------------------------------------------

1. Introduction

There is an ever-increasing need to  move  information  from  one  computer  to

another.    Information can be exchanged using magnetic media -- tapes or disks

-- or over networks.  Networks are expensive, and when  your  computer  is  not

connected  to  one (or to the right one), you must find other means to transfer

information.  In the early days of computing, magnetic media formats were rela-

tively  standardized,  but  with  the  arrival  of  microcomputers  things have

changed: most microcomputer users have no access to tapes, and disk formats are

incompatible  between most microcomputer makes and models.  Even when disk for-

mats agree, the disk must be physically moved from one system to the  other  in

order  for  information  to  be  exchanged  -- the effort and delay can be sig-

nificant if the systems are widely separated.

The telecommunication line provides a cheap and widely available alternative to

networks and magnetic media.  Asynchronous telecommunication is the method used

by most terminals to connect to most computers.    When  dedicated  "hardwired"

connections,  such  as those found between a timesharing computer and its local

terminals, are not available, computer users can create their own  dialup  con-

nections with a telephone and a modem.

Most  computers  come equipped with asynchronous telecommunications interfaces,

or "serial ports", which allow them to act as, or communicate with,  terminals.

The  question is how to use the serial port to exchange data.  Fortunately, the

standards for connecting terminals to computers  are  almost  universally  fol-

lowed:  connector  configuration  (DB-25  or  DB-9),  transmission signals (EIA

RS-232), a commonly accepted set of transmission speeds  (baud  rates),  and  a

convention  for encoding characters in storage and during transmission (ASCII).

These standards provide the physical medium and the data format,  but  they  do

not specify a process for exchanging data.

1.1. Why Kermit?

When  data  is transmitted from one computer to another; the receiving computer

has to be instructed to take in the data and put  it  somewhere,  and  it  also

needs  a  way  of  ensuring  that the data has been received correctly and com-

pletely in spite of several factors that  will  tend  to  interfere  with  this

process:

1. Noise --  It is rarely safe to assume that there will be no electri-

cal interference on a line; any long or switched data  communication

line  will  have  occasional interference, or noise, which typically

results in garbled or extra characters.  Noise corrupts  data,  per-

haps in subtle ways that might not be noticed until it's too late.

2. Synchronization  --  Data must not come in faster than the receiving

machine can handle it.  Although line speeds at the two ends of  the

connection  must  match  before  communication  can  take place, the

receiving machine might not be able to process a  steady  stream  of

input  at  that speed.  Its central processor may be too slow or too

heavily loaded, its buffers too full or too small, or its  disk  too

slow.    The  typical symptom of a timing problem is lost data; most

operating systems will simply discard incoming  data  they  are  not

prepared to receive.

1.1. Why Kermit?                                                         Page 7

-------------------------------------------------------------------------------

To  prevent  corruption  of  data and to synchronize communication, cooperating

computers can send special messages to one another along with the data.  Inter-

mingling  of control information with data together requires a set of rules for

distinguishing messages from data, and specifying what the messages are and the

actions  associated  with  each  message.    Such  a  set  of rules is called a

protocol.

Kermit is a file transfer protocol.  It is specifically designed  for  transfer

of  sequential  files  over  ordinary  telecommunication  lines.  Kermit is not

necessarily better than other terminal-oriented file transfer protocols but  it

is  free,  it  is  well documented, and it has been implemented compatibly on a

wide variety of microcomputers, PCs, workstations,  minicomputers,  mainframes,

and supercomputers.

1.2. How Kermit Works

Kermit  transfers  data by encapsulating it in  packets of control information.

This information includes a synchronization marker, a packet sequence number to

allow detection of lost packets, a length indicator, and a "block check" to al-

low verification of the data, as shown in Figure 1-1.

-------------------------------------------------------------------------------

+------+------+------+------+--------- - - - -+-------+

| MARK | LEN  | SEQ  | TYPE | DATA            | CHECK |

+------+------+------+------+--------- - - - -+-------+

Figure 1-1:   A Kermit Packet

-------------------------------------------------------------------------------

The MARK (usually an ASCII Control-A character) appears at the beginning of the

packet.    The  next character is a length field (LEN), specifying how long the

rest of the packet is.  The sequence number (SEQ) is used  to  detect  lost  or

duplicated  packets; retransmission is requested for lost packets and duplicate

packets are discarded.  The TYPE field specifies whether  the  packet  contains

data  or  control information.  The CHECK field contains a quantity obtained by

combining all the other characters in the packet together  in  one  of  several

ways;  the  sender  computes this value and sends it.  The packet receiver also

computes the value and checks it against the value sent;  if  they  agree,  the

packet  is  accepted;  if they disagree, then the packet has been corrupted and

retransmission is requested.  The DATA field contains up to  90  characters  of

data.  All fields except the mark are encoded as printable ASCII characters, to

prevent host or network interference.  Figure 1-2  shows  how  a  typical  file

transfer  proceeds.   Figure 1-2 does not show how Kermit recovers from errors.

Very briefly, here's how it works:

- If a packet is corrupted in transit by noise or loss  of  characters,

the block check will be wrong.  A file receiver will NAK ("negatively

acknowledge")  a  corrupted  packet,  which  causes  the  sender   to

retransmit  the  same packet (or, alternatively, it will ACK the last

correctly received packet again).  A file sender only  receives  ACKs

and  NAKs  from  the  receiver;  a  corrupted ACK, or a NAK, from the

receiver causes the sender to retransmit its most recent packet.

1.2. How Kermit Works                                                    Page 8

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Sender                 Receiver

Send-Init ------------->         Sender and Receiver exchange greetings

<-------------------- ACK

File-Header ----------->         Sender sends first file name to Receiver

<-------------------- ACK   Receiver acknowledges

File-Data ------------->         Sender sends first file data packet

<-------------------- ACK

File-Data ------------->         Sender sends second file data packet

<-------------------- ACK

File-Data --xx~~~p'''-->         Third data packet is corrupted by noise

<-------------------- NAK    and Receiver negatively acknowledges it

File-Data ------------->         Sender retransmits third packet

<-------------------- ACK

File-Data packets are sent and acknowledged until the whole file is sent

End-Of-File ----------->         Sender indicates first file is complete

<-------------------- ACK

File-Header ----------->         Name second of file

<-------------------- ACK

File-Data ------------->         First data packet for second file

<-------------------- ACK

File-Data packets are sent and ack'd until the whole file is sent

End-Of-File ----------->         Sender indicates second file is complete

<-------------------- ACK

End-Of-Transaction ---->         Sender indicates no more files to come

<-------------------  ACK

Figure 1-2:   Kermit File Transfer

-------------------------------------------------------------------------------

- If the file sender does not receive  an  ACK  within  the  prescribed

timeout  interval,  it  retransmits  the  same  packet.   If the file

receiver does not receive an expected packet within the  timeout  in-

terval,  it  sends  a NAK for the expected packet (or another ACK for

the most recently correct packet).

Many encoding, compression, block check, timeout, and packet length options are

provided.    These  options  are  automatically  negotiated  by  the two Kermit

programs when they initially make contact, and the greatest common set of  fea-

tures is used.  For this reason, any two Kermit programs should be able to com-

municate successfully, from the oldest, most bare-bones version, to the newest,

1.2. How Kermit Works                                                    Page 9

-------------------------------------------------------------------------------

most  feature-laden version.  The protocol is described in detail in the Kermit

book.

2. How to Use Kermit                                                    Page 10

-------------------------------------------------------------------------------

2. How to Use Kermit

Kermit embodies a set of rules for transferring files reliably between two com-

puters.    In  general,  one computer is a large system (a host, for instance a

timesharing system with many terminals), and the other is a  personal  computer

(PC).  The host believes that the PC is an ordinary terminal.  In order for the

Kermit protocol to occur, a Kermit program must be running on each end  of  the

communication line -- one on the host, one on the PC.

Your  task  is  just  to get the two Kermits started.  You have to use a single

keyboard and screen to talk to two different computers, two different programs.

Let's  talk  about  a common case: you are sitting at a personal computer (PC),

which has a serial communication port.  The serial port is connected to a  host

computer using, say, a dialup modem.

Normally, when you use your PC, you are "talking" directly to it; your commands

are interpreted directly by the PC's  operating  system  (CP/M,  MS-DOS,  UNIX,

etc),  or  by  some  program  that runs on the PC (an editor, a text formatter,

space invaders...).  The version of Kermit on your PC is  a  program  like  any

other, but it has a special ability to either interpret your commands directly,

like other programs, or to pass everything  you  type  through  to  the  other,

remote computer.  When you tell Kermit to CONNECT, it sends every character you

type out the serial port, and it puts every character that comes in the  serial

port onto the screen.  This is called "terminal emulation" -- one computer acts

as if it were a terminal to the other.  You are now  "talking"  to  the  remote

computer, and the PC is (mostly) ignoring you.

Kermit,  like  most interactive programs, has a prompt.  The prompt is a string

of characters it types on the left margin to indicate that it is ready for  you

to  type  a  command.   Kermit's prompt is normally "Kermit-xx>".  The xx iden-

tifies the implementation of Kermit; the Kermit that runs on MS-DOS systems  is

called  "Kermit-MS" and its prompt is "Kermit-MS>"; the Kermit that runs on Z80

and  8080-based  microcomputers  is  called  "Kermit-80"  and  its  prompt   is

"Kermit-80>",  and so forth.  If you become confused about which Kermit you are

talking to, the prompt should provide a clue.  In addition, most Kermits  print

an informative message like

[Connecting to remote host, type CTRL-]C to return]

when you CONNECT, and type another message like

[Connection closed, back at PC]

when you return.

Having  "connected"  to  the host, there must be a way for you to "get back" to

the PC.  This is accomplished by an escape sequence.   As  Kermit  passes  your

characters  through  to  the  host, it checks each one to see if it's a special

predefined escape character.  When the PC sees this character, it stops  ignor-

ing you -- you are once again "talking" to the PC, not to the host.  The escape

character is normally chosen to be one that you will not  need  to  type  while

talking to the host, and one that is hard to type by accident -- it's usually a

control character, such as Control-], which is entered by holding down the  key

marked  CTRL  or  CONTROL  and  typing the indicated character (in this case, a

right bracket "]").  The CTRL key works just like a SHIFT key.  Control charac-

ters  are  written either as Ctrl-A or ^A, where A is the character to be typed

2. How to Use Kermit                                                    Page 11

-------------------------------------------------------------------------------

while holding down the Ctrl key.

2.1. Transferring a File

From system command level on your PC, first run the Kermit program.  Then  tell

Kermit to CONNECT you to the host.  Now you're talking to the remote host -- at

this point you must log in, and then run Kermit on the host.

Now you have a Kermit program on each end of the connection.  The next step  is

to  tell  each Kermit what to do.  Suppose you want to transfer a file from the

host to the PC; you would first tell the host Kermit to  SEND  the  file,  then

"escape"  back  to the PC Kermit and tell it to RECEIVE the file.  The transfer

begins -- you can sit back and watch, or go make yourself a sandwich.   The  PC

Kermit  will produce a running display on your screen as the transfer proceeds,

and will notify you when it is complete.

The desired file should now be on your PC's disk.  The Kermit protocol has  en-

sured  that  the  file arrived correctly and completely.  Now you must clean up

after yourself: CONNECT back to the host, exit from Kermit on the host, log out

from  the  host,  "escape"  back to PC Kermit and exit from it.  Now you can do

whatever you had planned for your file -- edit it, print it on your PC printer,

etc.    Transferring a file in the other direction works the same way, but with

the SEND and RECEIVE commands exchanged.

The Kermit protocol, and most Kermit programs, allow you  to  send  text  files

reliably  from  the host to the PC, from the PC to the host, from host to host,

or PC to PC, usually without any special regard for  the  nature  of  the  par-

ticular machines involved.  Most implementations also allow files to be sent in

groups, with a single command, such as "send *.*"  The  scenario  for  each  of

these  is  the same as above -- only the details of how to establish the actual

connection differ.

Kermit works best with "printable" files -- files  composed  only  of  letters,

digits,  punctuation marks, carriage returns, tabs, and so forth -- since these

can be represented on almost any kind of computer.   Kermit  is  also  able  to

transfer "binary" files -- files such as executable programs -- composed of ar-

bitrary bit patterns, but binary files normally are meaningful only to the kind

of computer on which they are generated.  Nevertheless, Kermit can usually move

such files from system A to system B (where they are not much use) and back  to

system  A  in their original condition, although in most cases special measures

must be taken to accomplish this.

Let's look at some more concrete examples.  First you need  to  know  what  the

basic Kermit commands are.

2.2. Basic Kermit Commands

These  are  generic  descriptions  of the most basic Kermit commands.  Detailed

descriptions will come later.  In these descriptions, local refers to the  sys-

tem  that  you are using directly, remote refers to the system to which you are

CONNECTed via Kermit.  Commands may take one or more operands on the same line,

and are terminated by a carriage return.

SEND filespec

2.2. Basic Kermit Commands                                              Page 12

-------------------------------------------------------------------------------

Send the file or file group specified by filespec from this Kermit  to  the

other.    The  name of each file is passed to the other Kermit in a special

control packet, so it can be stored there with the same name.  A file group

is  usually  specified  by  including "wildcard" characters like "*" in the

file specification.  Examples:

send foo.txt

send *.for

Some implementations of Kermit may not support  transfer  of  file  groups;

these  versions  would  require a separate SEND command for each file to be

transferred.

RECEIVE

Receive  a  file  or file group from the other Kermit.  If an incoming file

name is not legal, then attempt to transform it to a  similar  legal  name.

Options are be provided for handling filename collisions.

CONNECT

Make a terminal connection to the remote system.  To "escape" from the ter-

minal  connection,  type  Kermit's  escape character (e.g. CTRL-], control-

rightbracket), followed by the letter "C" for "Close Connection".

SET Establish various nonstandard settings, such as CONNECT  escape  character,

file characteristics, communication line number, speed (baud rate), parity,

or flow control.  All of these are explained later.

SHOW

(or STATUS) Display the values of SET options.

HELP

Type a summary of Kermit commands and what they do.

EXIT

Exit from Kermit back to the host operating system.

?   Typed almost anywhere within a Kermit command:  List the commands, options,

or operands that are possible at this point.  This command may or  may  not

require a carriage return, depending on the host operating system.

2.3. Real Examples

Kermit  can  be  used  in several ways: from a PC that is connected to a larger

host computer; from a host computer which is connected to  another  host;  from

one PC to another.

2.3.1. PC to Host

In this example, the user is sitting at an IBM Personal Computer (PC), which is

connected through its serial port to a DEC VAX/VMS host computer.  The  IBM  PC

is  local, the VAX is remote.  This example will also apply almost literally to

any other microcomputer implementation of Kermit.

You have started up your PC and have the Kermit program on your disk.  Begin by

2.3.1. PC to Host                                                       Page 13

-------------------------------------------------------------------------------

running Kermit on the PC.  Use Kermit's CONNECT command to become a terminal to

the VAX.  In fact, the PC emulates the popular DEC VT-102 (VT-100), so so it is

desirable  to  tell  the host that your terminal is of this type.  Login on the

VAX and run Kermit there.  Here is an example of this procedure  with  commands

you type underlined; the material lined up on the right is commentary, not sys-

tem typeout or part of a command:

A>Kermit                 Run Kermit on the PC.

Kermit-MS V2.30

IBM-PC Kermit-MS: V2.30  8 Jan 88

Type ? for help

Kermit-MS>               This is the Kermit prompt for the PC.

Kermit-MS>connect        Connect to the VAX.

[Connecting to host, type Control-] to return to PC]

You are now connected to the VAX.

Welcome to CUMIN, MicroVMS V4.6    i(The system prints its herald.)

Username: my-id          Type your user ID.

Password: my-password    Type your password.

(Various greeting or notice messages are displayed.)

$                        This is the VMS system prompt.

$ Kermit                 Run Kermit on the VAX.

VMS Kermit-32 version 3.3.111

Default terminal for transfers is: _TXA0:

Kermit-32>               This is VMS Kermit's prompt.

You are now ready to transfer files between the two machines.

The following example illustrates how to send files from the  VAX  to  the  PC.

Note the use of the "*" wildcard character to denote a file group.

Kermit-32>send *.for     Send all my FORTRAN files.

^]c                      Now return back to the PC by

typing the escape sequence, in this case

^]C (Control-] followed by "C")

[Back at PC.]

Kermit-MS>receive        Tell the PC that files are coming.

If  you  take  more than about 5 seconds to get back to Kermit-MS and issue the

RECEIVE command, the first packets from Kermit-32 may  arrive  prematurely  and

appear  on  your  screen,  but  no harm will be done because the packet will be

retransmitted by the VAX until the PC acknowledges it.

Once the connection is established, the PC will show you what is  happening  --

it  first  clears the screen and waits for incoming packets; as packets arrive,

the current file name and packet number will be continuously displayed  on  the

screen.   When the PC's "Kermit-MS>" prompt returns to your screen (with an ac-

companying beep to catch your attention) the transfer  is  done.    Notice  the

screen display; the status should be indicated as "complete".  If not, an error

has occurred and an appropriate message should be displayed to tell you why.

2.3.1. PC to Host                                                       Page 14

-------------------------------------------------------------------------------

After you're finished transferring files, you must  CONNECT  back  to  the  VAX

host,  EXIT  from  Kermit there, logout, and "escape back" to the PC as you did

previously:

Kermit-MS>connect        Get back to the VAX.

[Connecting to host. Type CTRL-]C to return to PC.]

Kermit-32>               Here we are.

Kermit-32>exit           Get out of VMS Kermit.

$ logout                 Logout from the VAX.

MY-ID   logged out at 25-JAN-1988 15:12:27.85

^]c                      Now "escape" back to the PC,

[Back at PC.]

Kermit-MS>exit           and exit from the PC's Kermit.

A>                       Now you see the DOS prompt again.

The files you transferred should now be on your PC disk.

To send files from the PC to the VAX, follow a similar procedure.  First follow

the  instructions  in the previous section to log in to the VAX through the PC.

Then in response to the host Kermit's "Kermit-32>" prompt  you  type    RECEIVE

rather  than SEND.  Now escape back to the PC and use the  SEND command to send

the local PC files to VAX.  The PC will show you the progress of the  transmis-

sion on its screen.

When  the  "Kermit-MS>"  prompt indicates that the transmission is complete you

should follow the procedure shown above to logout from  the  VAX  host,  except

that you may first wish to confirm that the files have been stored correctly in

your directory on the VAX.

2.3.2. Host to Host

A "host" is considered to be a large or multi-user system, whose distinguishing

characteristic  is  that it has multiple terminals.  Use of Kermit for host-to-

host file transfers differs from the PC-to-host case in that the line your ter-

minal  is connected to is not the same as the line over which the data is being

transferred, and that some special SET commands may have to be issued to  allow

one Kermit to conform to unusual requirements of the other host.

In  this  example,  you  are already logged in to a Unix system, and you use an

autodialer to connect to an IBM 370-series system running VM/CMS  through  Unix

device /dev/tty12.

% kermit

C-Kermit, 4E(070) 24 Jan 88, 4.2 BSD

Type ? for help

C-Kermit>set modem hayes

C-Kermit>set line /dev/tty12

C-Kermit>set speed 1200

C-Kermit>dial 7654321

Connected!

Other methods exist for connecting two hosts with a serial line.  For instance,

dedicated hookups can be made by running an RS-232 "null modem"  cable  between

2.3.2. Host to Host                                                     Page 15

-------------------------------------------------------------------------------

TTY  ports  on  the two systems (null modem cables, RS-232 signals, modems, and

other data communication apparati are described in detail in the Kermit  book).

The  following  procedure  would  be the same in any case, once a connection is

made.  The four "set" commands below  are  necessary  when  connecting  to  IBM

mainframes in "linemode" (as opposed to full-screen 3270 mode; if you don't use

IBM mainframes, you can ignore them for now).

C-Kermit>set duplex half The IBM mainframe is half duplex.

C-Kermit>set flow none   No full duplex XON/XOFF.

C-Kermit>set handshake xon                        Turn on line turnaround

C-Kermit>set parity mark The IBM mainframe uses mark parity.

C-Kermit>connect         Connect to the mainframe.

Connecting thru /dev/tty31, speed 1200.

The escape character is CTRL-\ (28).

Type the escape character followed by C to get back,

or followed by ? to see other options.

(Type carriage return here.)

VM/370 ONLINE            The IBM system prints its herald.

.login myuserid mypassword                        Login to IBM system.

LOGON AT 15:33:02 EST MONDAY 02/08/88

CUVMA CMS 3.1 8409 01/25/85

.

.Kermit

Kermit-CMS Version 4.0 (87/12/17)

Enter ? for a list of valid commands

Kermit-CMS>.send profile exec

^\c                      C-Kermit's escape sequence typed here.

[Back at Local System]

C-Kermit>receive         Tell Unix Kermit to RECEIVE.

The transfer takes place now; Unix Kermit will  print  the  names  of  incoming

files,  followed  by dots or percents to indicate the packet traffic (a dot for

every 4 packets successfully  transferred,  a  percent  for  every  timeout  or

retransmission).   The transfer is complete when when you see "[OK]", a beep is

sounded, and the C-Kermit prompt next appears.  At that point we  connect  back

to the remote IBM system, exit from the remote Kermit and log out.

.

profile.exec ..%%.[OK]

C-Kermit>connect         Get back to mainframe and clean up.

Kermit-CMS>.

Kermit-CMS>.exit

R;

.

SP/CMS

.logout

CONNECT= 00:03:01 VIRTCPU= 000:00.12 TOTCPU= 000:00.60

LOGOFF AT 15:40:13 EST MONDAY 02/08/88

^\c                      Type C-Kermit's escape sequence

[Back at Local System]

2.3.2. Host to Host                                                     Page 16

-------------------------------------------------------------------------------

C-Kermit>exit            All done with Kermit.

That's  the  whole  procedure.   The file is in your Unix directory, completely

readable, as profile.exec -- note  that  Kermit-CMS  translated  from  the  IBM

EBCDIC  character encoding into standard ASCII, and converted the space between

the file name and file type to a dot.

To send a file from the local host to the remote host,  we  would  merely  have

reversed the SEND and RECEIVE commands in the example above.

2.3.3. Micro to Micro

Kermit  also  works  between personal computers (microcomputers, workstations).

The difference here is that commands are typed on two keyboards, rather than  a

single one.  This is because a personal computer normally only accepts commands

from its own keyboard.  If one PC Kermit CONNECTs to another, there  will  nor-

mally be no program on the other side to listen.

You  can make the physical connection between two micros in two ways: direct or

dialup.  If the two units are in close proximity (say, 50 feet  or  less),  you

can connect their serial ports with a null modem cable.

Connections  at longer distances can be made via dialup, providing the required

modems are available (one side needs autoanswer capability), or using any  kind

of  dedicated or switched circuit that may be available -- CBX, port contention

unit, almost anything you can plug an EIA connector into.

In this example, a DEC VT180 "Robin" CP/M-80 microcomputer is  connected  to  a

Intertec  "SuperBrain"  CP/M-80  micro, using a female-to-male null modem cable

(these systems have nostalgia value, being among the  first  for  which  Kermit

programs were written).  The connection can be tested by running Kermit and is-

suing the CONNECT command on both ends: typein from each micro should appear on

the screen of the other.

Suppose  you  want  to  send  a  file FOO.HEX from the Robin to the SuperBrain.

Proceed as follows:

1. Run Kermit on the SuperBrain, and give the RECEIVE command:

A>Kermit

Intertec SuperBrain Kermit-80 - V4.09

Kermit-80>receive

2. Run Kermit on the Robin, and give the SEND command for FOO.HEX.

A>Kermit

DEC VT18X Kermit-80 - V4.09

Kermit-80>send foo.hex

Watch the packets fly.  When you get the next Kermit-80> prompt, the

transfer is done, and you can EXIT from both Kermits.

The key point is to start the receiving end first -- some microcomputer Kermits

do not include a timeout facility, and if the receiver is not ready to  receive

when the sender first sends, there will be a protocol deadlock.

2.4. Another Way -- The Kermit Server                                   Page 17

-------------------------------------------------------------------------------

2.4. Another Way -- The Kermit Server

So  far, we have been describing the bare-bones version of the Kermit protocol.

An optional extension to the protocol includes the concept of a Kermit  server.

A  Kermit  server  is a Kermit program that does not interact directly with the

user, but only with another Kermit program.  You do not type commands to a Ker-

mit server, you merely start it at one end of the connection, and then type all

further commands at the other end.

Not all implementations of Kermit can be servers, and not all know how to  talk

to  servers -- but most of the major ones can and do.  The server is run on the

remote computer, which would normally be a timesharing system, such as  an  IBM

mainframe,  a  Unix system, or VAX/VMS, but may be a minicomputer or even a PC.

It depends on whether the particular Kermit program  has  a  "server"  command.

You  must  still connect to the remote host to log in and start the server, but

you no longer have to tell one side to SEND and the other to RECEIVE, nor  must

you  connect  back to the remote side to clean up and log out when you're done.

Using the server, you can do as many send and receive operations  as  you  like

without  ever  having  to  connect  back to the remote host.  Some servers also

provide additional services, such as directory listings, file deletion, or disk

usage inquiries.

A  Kermit  server  is  just a Kermit program running in a special way.  It acts

much like ordinary Kermit does after you give it a RECEIVE command -- it  waits

for  a message from the other Kermit, but in this case the message is a command

saying what to do, normally to send or to receive a file  or  group  of  files.

After escaping back to the local system, you can give as many SEND and GET com-

mands as you like, and when you're finished transferring files,  you  can  give

the  BYE  command, which sends a message to the remote Kermit server to log it-

self out.  You don't have to connect back to the  remote  host  and  clean  up.

However,  if  you want to connect back to the host, you can use the FINISH com-

mand instead of BYE, to shut down the Kermit server on the remote host  without

logging it off, allowing you to CONNECT back to your job there.

Here's an example of the use of a Kermit server.  The user is sitting at an IBM

PC and a DECSYSTEM-20 is the remote host.

A>Kermit                 Run Kermit on the micro.

Kermit-MS V2.30

IBM-PC Kermit-MS: V2.30  8 Jan 88

Type ? for help

Kermit-MS>               This is the Kermit prompt for the PC.

Kermit-MS>connect        Connect to the VAX.

CU20B                    The DEC-20 prints its herald.

@login my-id password    Log in normally.

(The DEC-20 prints various login messages here.)

@Kermit                  Run Kermit-20 normally

Kermit-20>server         Tell it to be a server.

Kermit Server running on DEC-20 host.  Please type your escape

sequence to return to your local machine.  Shut down the server by

typing the Kermit BYE command on your local machine.

2.4. Another Way -- The Kermit Server                                   Page 18

-------------------------------------------------------------------------------

^]c                      Now escape back to the PC.

Kermit-MS>get *.pas      Get all my DEC-20 Pascal programs.

Kermit-MS>send foo.*     Send all the "foo" files from my PC.

Kermit-MS>exit           Exit from Kermit back to DOS.

A>

(Here you can do some work on the micro, edit files, whatever you like.)

A>Kermit                 Run Kermit-80 some more.

Kermit-MS>send file.pas  Send another file.

Kermit-MS>bye            That's all.  Shut down the Kermit server.

A>                       Back at DOS automatically.

This is much simpler.  Note that once you've started the Kermit Server  on  the

remote end, you can run Kermit as often as you like on the micro without having

to go back and forth any more; just make sure to  shut  the  server  down  when

you're done by typing the BYE command.

If  it's  so  much  simpler,  why  not  do  it this way all the time?  You can,

provided your remote Kermit has a "server" command.  But server operation, plus

the  special  commands  the  local  Kermit needs to communicate with the server

(GET, REMOTE, BYE,  FINISH)  are  optional  Kermit  features,  so  some  Kermit

programs might not have them.  All Kermit programs, however, should provide the

basic SEND/RECEIVE mode of operation.

Here are the basic commands available for talking to servers:

SEND filespec   Sends a file or file group from the local host  to  the  remote

host in the normal way.

GET filespec    Ask the remote host to send a file or file group.  Example:

get *.c

This  command is exactly equivalent to typing "send *.c" at the

remote host followed by "receive" on the local host.  Note that

the local Kermit does not attempt to validate the filespec.  If

the server cannot access the specified file(s),  it  will  send

back  an  appropriate  error message.  Please note that GET and

RECEIVE are not the same!  RECEIVE tells  Kermit  to  passively

wait  for  a  file.    GET actively sends a request to a Kermit

server to send the named file.

REMOTE command [argument]

Ask  the  server to perform the specified command, and send the

results to your screen.  Not all servers are  capable  of  per-

forming  REMOTE  commands; those that can most commonly provide

REMOTE DIRECTORY, REMOTE DELETE, REMOTE SPACE, and similar file

management services.

BYE             Shut  down  the  remote server and exit from Kermit.  This will

cause the job at the remote end to log itself out.    You  need

not  connect  back and clean up unless you get an error message

in response to this command.

2.4. Another Way -- The Kermit Server                                   Page 19

-------------------------------------------------------------------------------

FINISH          Shut down the server without having  it  log  itself  out,  and

don't  exit from Kermit.  A subsequent CONNECT command will put

you back at your job on the  remote  host,  at  system  command

level.

Server  operation is not limited to mainframes.  Some PC Kermit implementations

can also act as servers, notably MS-DOS and Unix.  For instance, an IBM  PC  at

the  office  with  an autoanswer modem can be left in server mode at the end of

the day, and then dialed up from home in the evening for file transfer.

3. When Things Go Wrong                                                 Page 20

-------------------------------------------------------------------------------

3. When Things Go Wrong

Connecting two computers can be a tricky  business,  and  many  things  can  go

wrong.  Before you can transfer files at all, you must first establish terminal

communication.  But successful terminal connection does  not  necessarily  mean

that  file  transfer  will  also work.  And even when file transfer seems to be

working, things can happen to ruin it.  The  following  sections  treat  a  few

basic problems.  See the troubleshooting section of the Kermit book for greater

detail.

3.1. Basic Connection Problems

If you have a version of Kermit on your microcomputer, but the CONNECT  command

doesn't seem to work at all, please:

- Make  sure  all  the required physical connections have been made and

have not wiggled loose.  If you are using a modem, make sure the car-

rier light is on.

- If  you have more than one connector on your micro, make sure you are

using the right one.

- Make sure that the port is set to the right communication  speed,  or

baud  rate.   Some versions of Kermit have a built-in SET BAUD or SET

SPEED command, others require that you set the baud rate using a sys-

tem  command or setup mode before you start the Kermit program.  Some

versions of Kermit have SHOW or STATUS commands that  will  tell  you

what the current baud rate is.

- Make  sure that the other communication line parameters, like parity,

bits per character, handshake, and flow control are set correctly.

You may have to consult the appropriate manuals for the systems  and  equipment

in question.

If  all  settings and connections appear to be correct, and communication still

does not take place, the fault may be in your modem.    Internal  modems  (i.e.

those  that  plug in to a slot inside the microcomputer chassis) are not recom-

mended for use with Kermit unless they totally mimic  the  asynchronous  serial

port  hardware  they purport to replace, or unless the Kermit program claims to

support the particular internal modem.  Many microcomputer Kermit programs  are

written  to  control the communication hardware explicitly; internal modems can

interfere with that control.

Even external modems can cause trouble -- the  "smarter"  they  are,  the  more

potential  danger of disagreement between the modem and the microcomputer about

settings of baud rate, character framing, echo, and so forth.  Make  sure  your

modem is set up correctly (consult your modem manual).

3.2. Terminal Connection Works But The Transfer Won't Start             Page 21

-------------------------------------------------------------------------------

3.2. Terminal Connection Works But The Transfer Won't Start

Once you've made a terminal connection to the remote system, you will generally

also be able to transfer files.  But not always.  If Kermit's  terminal  emula-

tion  seems  to work correctly, but a file transfer will not start at all, then

something in the communication path is probably  interfering  with  the  packet

data:

PARITY:

A device can impose parity upon the communication line.   This  means  that

the 8th bit of each character is used by the equipment to check for correct

transmission.  Use of parity will:

- Cause packet checksums to appear incorrect to  the  receiver  and

foil  any  attempt at file transfer.  In most cases, not even the

first packet will get through.

- Prevent the use of the 8th bit for binary file data.

If terminal connection works but file transfer does not, parity is the most

likely  culprit.    To  overcome  this impediment, you should find out what

parity is being used, and inform the Kermits one side or  both  (using  the

SET PARITY command) so that they can:

- Compose and interpret the checksums correctly.

- Employ a special encoding to allow 8-bit data to pass through the

7-bit communication channel.

Many packet-switched networks, such as GTE TELENET, require  parity  to  be

set, as do IBM mainframes and their front end processors.

ECHOING:

Some communication processors, typically  front  ends,  echo  their  input.

When this happens, every Kermit packet that is sent to it will bounce right

back, causing no end of confusion.  Some Kermit programs have been designed

to  ignore  echoed  packets,  but  others  have not.  If you encounter this

problem, there are several possible solutions:

- Disable the front end echoing by typing some special command,  if

such a command is provided by the system.

- Some front ends respond to certain escape or control sequences as

commands to turn off echoing, either from that point  onward,  or

else  on a per-line basis.  In this case, the appropriate control

sequence can be inserted between packets by Kermit  programs  in-

structed to do so, for instance using the SET PAD command.

- If  the  echoing cannot be disabled, then the two Kermit programs

should be instructed to use differing packet start markers, using

the  SET START-OF-PACKET command -- for instance, one Kermit uses

Control-A as usual, and the other uses Control-B.  This can  only

be done if both Kermits have this SET command.

3.3. Special Characters                                                 Page 22

-------------------------------------------------------------------------------

3.3. Special Characters

There  is one problem that can prevent a file transfer from starting at all, or

may crop up after the file transfer is underway.  For instance, during  a  file

transfer  operation  you  might  find your smart modem suddenly hanging up your

current connection and placing a call to Tasmania.   Or  you  might  find  that

packets  containing a certain character like "@" cannot be transmitted success-

fully.

This is the problem of "special characters".  Some device in the  communication

path  -- a front end, a port switcher, a multiplexer, a "smart" modem -- inter-

prets certain characters in the data stream as commands rather than as data  to

be passed them along to the other side.  Usually such equipment interferes only

with the transmission of ASCII control characters; so  long  as  Control-A  and

Carriage  Return  -- Kermit's normal packet start and end delimiters -- are not

molested, then Kermit can operate.  However, equipment may exist which swallows

even  printable  characters.    Since  Kermit  assumes that ALL printable ASCII

characters (ASCII 40 through 176, octal)  can  be  transmitted  without  inter-

ference  or  modification, such equipment will prevent Kermit file transfer un-

less its printable-character-swallowing features can be disabled.

3.4. The Transfer Starts But Then Gets Stuck

Once a Kermit file transfer has begun, there are certain conditions under which

it can become stuck.  Since many hosts are capable of generating timeout inter-

rupts when input doesn't appear within a reasonable interval, they  can  resend

unacknowledged  packets  or request that missing packets be retransmitted.  But

since not all Kermit programs are capable of timing out, a means for manual in-

tervention is provided in the local Kermit -- you can type a carriage return on

the keyboard of most micros to wake up and continue the transfer.

The following sections discuss various reasons why a transfer in progress could

become  stuck.   Before examining these, first make sure that you really have a

Kermit on the other end of the line, and you have issued the  appropriate  com-

mand:  SEND,  RECEIVE, or SERVER.  If the remote side is not a server, remember

that you must connect back between each  transfer  and  issue  a  new  SEND  or

RECEIVE command.

3.4.1. The Connection is Broken

Check  the  connection.   Make sure no connectors have wiggled loose from their

sockets.  If you're using a modem, make sure you still have a  carrier  signal.

Reestablish your connection if you have to.

If  upon  reconnection you get no response, maybe the remote host or the remote

Kermit program  crashed.  Get back to command level on  the  local  Kermit  (on

microcomputer  implementations, you may be able to do this by typing about five

RETURNs, or one or more Control-C's).  Issue the CONNECT command  so  that  you

can  see what happened.  If the remote system has crashed then you will have to

wait for it to come back, and restart whatever file that was being  transferred

at the time.

3.4.2. The Disk is Full                                                 Page 23

-------------------------------------------------------------------------------

3.4.2. The Disk is Full

If  your  local  floppy  disk  or  remote directory fills up, the Kermit on the

machine where this occurs will inform you and then terminate the transfer.  You

can  continue the transfer by repeating the whole procedure either with a fresh

floppy or after cleaning up your directory.  Some Kermits also have  a  feature

that  allows  you  to keep incompletely received files; this would allow you go

back to the sending system, extract the unsent portion of the  file,  and  send

it, and then append the two received portions together using an editor or other

system utility.  Kermit does not provide the ability to switch disks  during  a

file transfer.

3.4.3. Transmission Delays

Packet  transmission  can  be  delayed by various agents: congested timesharing

systems or networks, earth satellites, etc.  When  transmission  delay  exceeds

the  per-packet timeout interval for a significant length of time, the transfer

could fail.  Most Kermit programs provide commands that allow you to adjust the

timeout interval or the packet transmission retry threshhold in order to accom-

modate to severe transmission delays.

3.4.4. Noise Corruption

If your connection is extremely noisy, packets will become corrupted -- and re-

quire   retransmission   --  more  often.    The  probability  that  successive

retransmissions will fail because of corruption rises with the noise level  un-

til  it  exceeds  the retry threshhold, at which point the file transfer fails.

There are several recourses.  First, try to establish a  new  connection.    If

that  is  impractical,  then  use  SET  commands (when available) to reduce the

packet length and increase the retry threshhold.  Shorter  packets  reduce  the

probability  that  a particular packet will be corrupted and the retransmission

overhead when corruption  does  occur,  but  they  also  increase  the  overall

protocol  overhead.    In a noisy environment, you should also request a higher

level of error checking (SET BLOCK 2 or 3).

3.4.5. Host Errors

Various error conditions can occur on the remote host that  could  effect  file

transmission.    Whenever any such error occurs, the remote Kermit normally at-

tempts to send an informative error message to the local one, and  then  breaks

transmission, putting you back at Kermit command level on the local system.

3.5. File is Garbage

There  are  certain  conditions under which Kermit can believe it transferred a

file correctly when in fact, it did not.  The most likely cause has to do  with

the tricky business of file attributes, such as text vs binary, 7-bit vs 8-bit,

blocked vs stream, and so forth.  Each system has its  own  peculiarities,  and

each  Kermit  has special commands to allow you to specify how a file should be

sent or stored.  However, these difficulties usually crop up only when  sending

binary files.  Textual files should normally present no problem between any two

Kermit programs.

4. Kermit Commands                                                      Page 24

-------------------------------------------------------------------------------

4. Kermit Commands

An "ideal" Kermit program will be described here, which has most  of  the  fea-

tures  specified  in  the Kermit book.  Few Kermit programs will have all these

commands or support all these options.  The exact form of some of the  commands

may  differ  from version to version.  Some Kermit programs may support system-

dependent options not described here.  The intention of this description is  to

provide a base from which specific Kermit programs can be described in terms of

their differences from the "ideal."

4.1. Remote and Local Operation

In any connection between two Kermit programs, one Kermit  is  remote  and  the

other is local.  The remote Kermit is usually running on a mainframe, which you

have CONNECTed to through a PC or other computer.  When Kermit  runs  remotely,

all  file transfer is done over the job's controlling terminal line -- the same

line over which you logged in, and to which you  would  type  interactive  com-

mands.    What  the  system thinks is your terminal is really another computer,

usually a microcomputer, running its own copy of Kermit.

When Kermit is in "local mode", file transfer is done over an external  device,

such  as  a  microcomputer's serial communication port, or an assigned terminal

line on a mainframe.  The local Kermit is connected in some way (like a dialout

mechanism)  to another computer, again running its own copy of Kermit.  A local

Kermit is in control of the screen, a remote Kermit has no direct access to it.

Microcomputer  Kermits  are  run  in  local  mode  unless instructed otherwise;

mainframe Kermits run remotely unless some special command places them in local

mode.  Some commands make sense only for remote Kermits, others only for local,

still others can be used with either.  Local and remote operation of Kermit  is

shown  schematically here:  The Kermit program on the PC is a local Kermit.  It

can control the screen, the keyboard, and the port separately, thus it can  up-

date  the  screen with status information, watch for interrupt signals from the

keyboard, and transfer packets on the communications  port,  all  at  the  same

time.

The  Kermit program running on the mainframe is a remote Kermit.  The user logs

in to the mainframe through a terminal port.  The  host  computer  cannot  tell

that  the  user  is  really  coming  in through a microcomputer.  The keyboard,

screen, and port functions are all combined in user's mainframe terminal  line.

Therefore  a remote Kermit is cut off from your screen and keyboard during file

transfer.

4.2. The Command Dialog

Most Kermit programs communicate with  you  through  interactive  keyword-style

command  dialog (a prominent exception is Macintosh Kermit, which uses pulldown

menus that overlay the terminal  emulation  screen).    The  program  issues  a

prompt, indicating that it is waiting for you to type a command.  The prompt is

usually of the form

Kermit-xx>

where xx indicates the version of  Kermit  --  Kermit-MS>  for  MS-DOS  Kermit,

Kermit-86> for CP/M-86 Kermit, etc.

4.2. The Command Dialog                                                 Page 25

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

PC is Local, Mainframe is Remote:

Communication

Line                    (Packets)

+---------------/  /-----------------+  Other terminals

|                                    |  |  |  |

|                                    |  |  |  |

PC        |     LOCAL              Mainframe   |  |  |  |  REMOTE

+----------+----------+            +------------+--+--+--+--------+

|   Serial Port       |            |            |                 |

|                     |            |            |                 |

|                     |            |            |                 |

|  +---------------+  |            |            Your job's        |

|  | Packets: 724  |  |            |            terminal line     |

|  | Retries:   7  |  |            |                              |

|  | File: FOO.BAR |  |            |                              |

|  +---------------+  |            |                              |

|   Screen            |            |                              |

|                     |            |                              |

+---------------+-----+            +------------------------------+

|

| (Commands)

|

+------------+---------+

\       Keyboard       \

+----------------------+

You

Figure 4-1:   Local and Remote Kermits

-------------------------------------------------------------------------------

In  response  to  the  program's  prompt  you may type a keyword, such as SEND,

RECEIVE, or EXIT, possibly followed by additional keywords or operands, each of

which  is  called a field.  You can abbreviate keywords (but not file names) to

any length that makes them distinguishable from any  other  keyword  valid  for

that  field.  You can type a question mark at any time to get information about

what's expected or valid at that point.  The ESC and "?"  features work best on

full  duplex  systems,  where the program can "wake up" immediately and perform

the required function.  On half duplex or record-oriented systems, the ESC fea-

ture is not available, and the "?" requires a carriage return to follow.

In this example, the user types "set" and then a question mark to find out what

the SET options are.  The user then continues the command at  the  point  where

the question mark was typed, adding a "d" and another question mark to see what

set options start with "d".  The user then adds a "u" to select  "duplex"  (the

only SET option that starts with "du") followed by an ESC (shown here by a dol-

lar sign) to complete the current field, then another question mark to see what

the possibilities are for the next field, and so forth.  The command is finally

terminated by a carriage return.  Before carriage return is typed, however, the

command  can  be  edited  or erased using RUBOUT or other command editing keys.

Finally, the same command is entered again with a minimum of  keystrokes,  with

each  field  abbreviated  to  its  shortest unique length.  In the example, the

parts the user types are underlined; all the rest is system typeout:

4.2. The Command Dialog                                                 Page 26

-------------------------------------------------------------------------------

Kermit-20>set ? one of the following:

debugging        delay             duplex            escape

file             handshake         IBM               line

parity           receive           send

Kermit-20>set d? one of the following:

debugging   delay        duplex

Kermit-20>set du$plex (to) ? one of the following:

full   half

Kermit-20>set duplex (to) h$alf

Kermit-20>set du h

4.3. Notation

In the command descriptions, the following notation is used:

italics     A parameter - the symbol in italics is replaced by an  argument  of

the specified type (number, filename, etc).

[anything]  A  field  enclosed in square brackets is optional.  If omitted, the

field defaults to an appropriate value.  You don't type the  brack-

ets.

{x,y,z}     A list of alternatives is enclosed in curly braces; you type one of

the alternatives.

number      A whole number, entered in prevailing notation of the system.

character   A single character, entered literally, or as a number (perhaps  oc-

tal or hexadecimal) representing the ASCII value of the character.

floating-point-number

A "real" number, possibly containing a decimal point  and  a  frac-

tional part.

filespec    A file specification, i.e. the name of a file, possibly including a

search path, device or directory name, or other  qualifying  infor-

mation,  and  possibly  containing  "wildcard"  or pattern-matching

characters to denote a group of files.

^X          A control character may be written using "uparrow" or "caret" nota-

tion, since many systems display control characters this way.  Con-

trol characters are produced by holding down the key marked CTRL or

Control  and  typing  the  appropriate  character,  e.g. X. Control

characters may also be written Ctrl-X, CTRL-X, CTRL/X, etc.

Commands are shown in upper case, but can be entered in any combination of  up-

per and lower case.

4.3. Notation                                                           Page 27

-------------------------------------------------------------------------------

4.4. Summary of Kermit Commands

Here  is a brief list of Kermit commands as they are to be found in most Kermit

programs.  The following sections will describe these commands in detail.

For exchanging files:

SEND, RECEIVE, GET

For connecting to a remote host:

CONNECT, SET LINE, SET PARITY,  SET  DUPLEX,  SET  HANDSHAKE,  SET  ESCAPE,

SET FLOW-CONTROL, SET SPEED (or BAUD)

For acting as a server:

SERVER

For talking to a server:

BYE, FINISH, GET, SEND, REMOTE

Setting nonstandard transmission and file parameters:

SET BLOCK-CHECK, SET DEBUG,  SET  DELAY,  SET  FILE,  SET  INCOMPLETE,  SET

PARITY, SET RETRY;

SET  SEND  (or RECEIVE) END-OF-LINE, START-OF-PACKET, PACKET-LENGTH, PAUSE,

TIMEOUT, PADDING

For defining and executing "macros" of commands:

DEFINE, DO

For interrupting transmission:

Control-X, Control-Z, Control-C, Control-E

Getting information:

HELP, STATISTICS, SHOW

Executing command files:

TAKE

For recording the history of a file transfer operation:

LOG TRANSACTIONS

For non-protocol file capture or transmission:

LOG SESSION, TRANSMIT, INPUT, OUTPUT, PAUSE, CLEAR, SCRIPT

For closing log files:

CLOSE

Leaving the program:

EXIT, QUIT

If you have a file called KERMIT.INI in your default or home disk, Kermit  will

execute  an  automatic TAKE command on it upon initial startup.  KERMIT.INI may

contain any Kermit commands, for instance SET commands, or DEFINEs  for  macros

to  configure  Kermit  to various systems or communications media.  Note:  Your

particular implementation of Kermit may use a different  name  for  this  file,

like MSKERMIT.INI for MS-DOS Kermit, or VMSKERMIT.INI for VAX/VMS Kermit.

4.4. Summary of Kermit Commands                                         Page 28

-------------------------------------------------------------------------------

4.5. The SEND Command

Syntax:

Sending a single file:

SEND filespec1 [filespec2]

Sending multiple files:

SEND wild-filespec1

The  SEND  command  causes a file or file group to be sent to the other system.

There are two forms of the command, depending  on  whether  filespec1  contains

"wildcard" characters.  Use of wildcard characters is the most common method of

indicating a group of files in a single file specification.   For  instance  if

FOO.FOR  is  a  single file, a FORTRAN program named FOO, then *.FOR might be a

group of FORTRAN programs.

Sending a File Group --

If filespec1 contains wildcard characters then all matching files will be sent,

in directory-listing order by name.  If a file can't be opened for read access,

it will be skipped.

Sending a Single File --

If filespec1 does not contain any wildcard characters,  then  the  single  file

specified  by  filespec1  will  be  sent.  Optionally, filespec2 may be used to

specify the name under which  the  file  will  arrive  at  the  target  system;

filespec2  is  not parsed or validated locally in any way.  If filespec2 is not

specified, the file will be sent with its own name.

SEND Command General Operation --

Files will be sent with their filename and filetype (for instance  FOO.BAR,  no

device  or directory field, no generation number or attributes).  If communica-

tion line parity is being used (see SET PARITY), the sending  Kermit  will  re-

quest that the other Kermit accept a special kind of prefix notation for binary

files.  This is an advanced feature, and not all Kermits have it; if the  other

Kermit  does  not  agree  to use this feature, binary files cannot be sent cor-

rectly.

The sending Kermit will also ask the other Kermit whether it can handle a  spe-

cial  prefix encoding for repeated characters.  If it can, then files with long

strings of repeated characters will be transmitted very efficiently.   Columnar

data,  highly  indented  text,  and binary files are the major beneficiaries of

this technique.

4.5. The SEND Command                                                   Page 29

-------------------------------------------------------------------------------

SEND Remote Operation --

If you are running Kermit remotely (for instance, from  a  microcomputer),  you

should  "escape  back"  to your local Kermit within a reasonable amount of time

and give the RECEIVE command.  Don't take more than a minute or two to complete

the  switch, or Kermit may "time out" and give up (in that case, you'll have to

CONNECT back to the remote system and reissue the SEND command).

SEND Local Operation --

If you're running Kermit locally, for instance on a microcomputer,  you  should

have  already  run Kermit on the remote system and issued either a RECEIVE or a

SERVER command.

Once you give Kermit the SEND command, the name of each file will be printed on

your  screen  as  the transfer begins, and information will be displayed to in-

dicate the packet traffic.  When  the  specified  operation  is  complete,  the

program will sound a beep, and the status of the operation will be indicated by

a message like OK, Complete, Interrupted, or Failed.

If you see many packet retry indications, you are  probably  suffering  from  a

noisy  connection.  You may be able to cut down on the retransmissions by using

SET SEND PACKET-LENGTH to decrease the packet  length;  this  will  reduce  the

probability that a given packet will be corrupted by noise, and reduce the time

required to retransmit a corrupted packet.

If you notice a file being sent which you do not really want to send,  you  may

cancel  the  operation immediately by typing either Control-X or Control-Z.  If

your are sending a file group, Control-X will cause  the  current  file  to  be

skipped,  and Kermit will go on to the next file, whereas Control-Z will cancel

sending the entire group and return you to Kermit-20 command level.

4.6. The RECEIVE Command

Syntax:  RECEIVE [filespec]

The RECEIVE command tells Kermit to wait for the arrival a file or  file  group

sent  by  a  SEND  command  from  the  other system.  If only one file is being

received, you may include the optional filespec as the name to store the incom-

ing file under; otherwise, the name is taken from the incoming file header.  If

the name in the header is not a legal file name on  the  local  system,  Kermit

will attempt to transform it to a legal name.

If  an  incoming file has the same name as an existing file, Kermit will either

overwrite the old file or else try to create a new unique  name,  depending  on

the setting of FILE WARNING.

If you have SET PARITY, then 8th-bit prefixing will be requested.  If the other

side cannot do this, binary files cannot be transferred correctly.  The sending

Kermit may also request that repeated characters be compressed.

If  an incoming file does not arrive in its entirety, Kermit will normally dis-

card it; it will not appear in your directory.  You may change this behavior by

using  the command SET INCOMPLETE KEEP, which will cause as much of the file as

4.6. The RECEIVE Command                                                Page 30

-------------------------------------------------------------------------------

arrived to be saved in your directory.

RECEIVE Remote Operation --

If your are running Kermit remotely, you should escape back to your local  Ker-

mit and give the SEND command.  You should do this within about two minutes, or

the protocol may time out and give up; if this happens, you can CONNECT back to

the remote system and reissue the RECEIVE command.

RECEIVE Local Operation --

If  you  are running Kermit locally, you should already have issued a SEND com-

mand to the remote Kermit, and then escaped back to DEC-20 Kermit (you can  not

issue  a  RECEIVE  command to a Kermit server, you must use the GET command for

that).

As files arrive, their names will be shown on your screen, along  with  a  con-

tinuous display the packet traffic.

If a file begins to arrives that you don't really want, you can attempt to can-

cel it by typing Control-X; this sends a cancellation  request  to  the  remote

Kermit.  If the remote Kermit understands this request (not all implementations

of Kermit support this feature), it will comply; otherwise it will continue  to

send.   If a file group is being sent, you can request the entire group be can-

celled by typing Control-Z.

4.7. The GET Command

Syntax: GET [remote-filespec]

The GET command requests a Kermit  server  to  send  the  file  or  file  group

specified by remote-filespec.  Note the distinction between the RECEIVE and GET

commands: RECEIVE instructs the program to wait passively, whereas GET actively

sends a request to a server.

The  remote  filespec  is  any  character  sequence  that  can  be a legal file

specification for the remote system; it is not parsed or validated locally.  As

files  arrive,  their names will be displayed on your screen, along with a con-

tinuous indication of the packet traffic.  As in the RECEIVE command,  you  may

type  Control-X  to  request  that  the  current  incoming  file  be cancelled,

Control-Z to request that the entire incoming batch be cancelled.

Optional Syntax: If you are requesting a single file, you may type the GET com-

mand  without a filespec.  In that case, Kermit programs that implement the op-

tional GET syntax will prompt you for the remote  filespec  on  the  subsequent

line, and the name to store it under when it arrives on the line after that:

Kermit-MS>get

Remote Source File: aux.txt

Local Destination File: auxfile.txt

4.8. The SERVER Command                                                 Page 31

-------------------------------------------------------------------------------

4.8. The SERVER Command

Syntax: SERVER

The  SERVER command instructs Kermit to cease taking commands from the keyboard

and to receive all further instructions in the  form  of  Kermit  packets  from

another  system.    The  other Kermit must have commands for communicating with

remote servers; these include GET, SEND, FINISH, and BYE.

After issuing this command, return to the "client" system and issue SEND,  GET,

BYE,  FINISH, or other server-directed commands from there.  If your local Ker-

mit does not have a BYE command, then it does not have the full ability to com-

municate  with  a  Kermit  server and you should not put the remote Kermit into

SERVER mode.  If your local Kermit does have a BYE command, use it to shut down

and log out the Kermit server when you are done with it.

Any  nonstandard parameters should be selected with SET commands before putting

Kermit in server mode.

4.9. The BYE Command

Syntax: BYE

When running talking to a Kermit server, use the BYE command to shut  down  the

server  and,  if  the  server is on a timesharing system, also log out the job.

This will also close any open log files and exit from the local Kermit.

4.10. The FINISH Command

Syntax: FINISH

When running as a local Kermit talking to a remote Kermit server use the FINISH

command to shut down the server without logging out the remote job, so that you

can CONNECT back to it.

4.11. The REMOTE Command

Syntax: REMOTE command

When talking to a remote Kermit server, use the REMOTE command to request  spe-

cial  functions  of  the  remote server.  If the server does not understand the

command or offer the requested service (all of these commands and services  are

optional  features  of  the Kermit protocol), it will reply with a message like

"Unknown Kermit server command".  If it  does  understand,  it  will  send  the

results  back,  and  they will be displayed on the screen.  The REMOTE commands

include:

REMOTE CWD [directory]

Change  Working  Directory.    If no directory name is provided, the server

will change to the default directory.  Otherwise, you will be prompted  for

a  password,  and the server will attempt to change to the specified direc-

tory.  If access is not granted, the server will provide a message to  that

effect.    If  the  remote  system does not require a password for changing

4.11. The REMOTE Command                                                Page 32

-------------------------------------------------------------------------------

directories (UNIX is an example), then  you  can  simply  type  a  carriage

return in response to the password prompt.

REMOTE DELETE filespec

Delete the specified file or files.   The  names  of  the  files  that  are

deleted should be displayed on your screen.

REMOTE DIRECTORY [filespec]

The names of the files that match the given file specification will be dis-

played  on  your  screen,  possibly along with additional information about

file sizes and dates.  If no file specification is given,  all  files  from

the current directory will be listed.

REMOTE SPACE [directory]

Information about disk usage in the current remote directory -- quota, cur-

rent  storage,  or  amount  of remaining free space -- is displayed on your

screen.

REMOTE HELP

A list of available server functions is displayed.

REMOTE HOST [command]

The given command is passed to the server's host command processor, and the

resulting output is displayed on your screen.

REMOTE KERMIT [command]

The  given  command,  which  is  expressed  in  the  server  Kermit's   own

interactive-mode  command  syntax,  is  passed to the server for execution.

This is useful for changing settings, logging, and other functions.

REMOTE TYPE filespec

The contents of the specified file is displayed on your screen.

REMOTE WHO [username]

List users, or a specified user, logged in on the server's system.

4.12. Local Commands

Syntax: [LOCAL] command

Execute the specified command on the local system -- on the system where Kermit

to  which your are typing this command is running.  These commands provide some

local file management capability without having to leave  the  Kermit  program,

which  is  particularly  useful  on microcomputers.  On most systems, the LOCAL

prefix for these commands can be omitted.

CWD [directory]

"Change Working Directory" to the specified directory.

DELETE filespec

Delete the specified file or files.

DIRECTORY [filespec]

Provide a directory listing of the specified files.

4.12. Local Commands                                                    Page 33

-------------------------------------------------------------------------------

SPACE

Display local disk usage and/or free space.

RUN filespec [operands]

Run the indicated program with the supplied command-line operands.

PUSH  Invoke  the  local  system  command interpreter in such a way that it can

return (or "pop" or "exit") back to Kermit.

Some Kermit programs may provide commands for these or other functions  in  the

syntax  of their own system, when this would cause no confusion.  For instance,

CP/M Kermit may use ERA in place of LOCAL DELETE.

4.13. The CONNECT Command

LOCAL ONLY -- Syntax: CONNECT [terminal-designator]

Establish a terminal connection to the system at the  other  end  of  the  com-

munication  line.   On a microcomputer, this is normally the serial port.  On a

mainframe, you will have to specify a terminal line number or other identifier,

either  in  the  CONNECT command itself, or in a SET LINE command.  Get back to

the local Kermit by typing the escape character followed by a single  character

"command".  Several single-character commands are possible:

C   Close the connection and return to the local Kermit.

S   Show status of the connection.

B   Send a BREAK signal.

0   (zero) Send a NUL (0) character.

F   Copy the current screen into a disk file.

D   Drop the line, hangup the modem.

P   Push  to  the local system command processor without breaking the connec-

tion.

Q   Quit logging session transcript.

R   Resume logging session transcript.

?   List all the possible single-character arguments.

^] (or whatever you have set the escape character to be)

Typing  the  escape character twice sends one copy of it to the connected

host.

You can use the SET ESCAPE command to define a different escape character,  and

SET  PARITY, SET DUPLEX, SET FLOW-CONTROL, SET HANDSHAKE to establish or change

those parameters.

4.14. HELP

Syntax: The HELP Command

Typing HELP alone prints a brief summary of Kermit and its commands,  and  pos-

sibly instructions for obtaining more detailed help on particular topics.  Most

Kermit implementations also allow the use of "?" within a command to produce  a

short help message.

4.15. The TAKE Command                                                  Page 34

-------------------------------------------------------------------------------

4.15. The TAKE Command

Syntax: TAKE filespec

Execute  Kermit commands from the specified file.  The file may contain contain

any valid Kermit commands, including other TAKE commands.

4.16. The EXIT and QUIT Commands

Exit from Kermit.  QUIT is a synonym for EXIT.

4.17. The SET Command

Syntax: SET parameter [option] [value]

Establish or modify various parameters for file transfer  or  terminal  connec-

tion.

When  a  file transfer operation begins, the two Kermits automatically exchange

special initialization messages, in which each program provides the other  with

certain information about itself.  This information includes the maximum packet

size it wants to receive, the timeout interval it wants  the  other  Kermit  to

use,  the  number  and  type  of  padding  characters it needs, the end-of-line

character it needs to terminate each packet (if any), the block check type, the

desired  prefixes  for  control characters, characters with the "high bit" set,

and repeated characters.  Each Kermit program  has  its  own  preset  "default"

values  for  these  parameters, and you normally need not concern yourself with

them.  You can examine their values with the SHOW command; the SET  command  is

provided to allow you to change them in order to adapt to unusual conditions.

The following parameters may be SET:

BAUD  Set the speed of the current communications port

BLOCK-CHECK  Packet transmission error detection method

DEBUG  Mode or log file

DELAY  How long to wait before starting to send

DUPLEX  For terminal connection, full (remote echo) or half (local echo)

END  Packet termination character (normally CR)

ESCAPE  Character for terminal connection

FILE  For setting file parameters like name conversion and byte size

FLOW-CONTROL  Selecting flow control method, like XON/XOFF

HANDSHAKE  For turning around half duplex communication line

IBM  Set parameters for IBM mainframe linemode connection

INCOMPLETE  What to do with an incomplete file

KEY  Establish a key redefinition or keyboard macro

LOCAL-ECHO  Specify who echoes during terminal connection

LINE  Terminal line to use for terminal connection or file transfer

MODEM  Modem type or characteristics

PARITY  Character parity to use

PORT  For switching communication ports

PROMPT  For changing the program's command prompt

RECEIVE  Various parameters for receiving files

RETRY  How many times to retry a packet before giving up

SEND  Various parameters for sending files

4.17. The SET Command                                                   Page 35

-------------------------------------------------------------------------------

SPEED  Synomym for BAUD.

TERMINAL  Parameters for terminal emulation

TIMER  Enable/disable timeouts

WARNING  Filename collision protection

The  DEFINE  command  may be used to compose "macros" by combining SET and pos-

sibly other commands.  The SET commands are now described in detail.

SET BAUD

Set or change the baud rate (approximate  translation:  transmission  speed  in

bits per second) on the currently selected communications device.  Ten bits per

second is usually equivalent to one character per second; 300 baud  =  30  cps.

The  way  of  specifying  the  baud  rate varies from system to system; in most

cases, the actual number (such as 1200 or 9600) is typed.  Systems that do  not

provide  this  command  generally expect that the speed of the line has already

been set appropriately outside of Kermit.  Common values are 300,  1200,  2400,

4800, 9600, 19200.

SET BLOCK-CHECK {1, 2, 3}

Kermit  normally uses a 1-character block check, or "checksum", on each packet.

The sender of the packet computes the block check based on the other characters

in  the  packet,  and  the receiver recomputes it the same way.  If these quan-

tities agree, the packet is accepted and transmission proceeds.  If  they  dis-

agree, the packet is rejected and retransmission is requsted.

However,  the  block  check  is not a foolproof method of error detection.  The

normal single-character Kermit block check is only a 6-bit  quantity  (the  low

order  8 bits of the arithmetic sum folded upon itself).  With only six bits of

6

accuracy, the chances are one in 2  -- that is, 1/64 -- that an error can occur

which  will  not  be  detected  in  the  checksum, assuming that all errors are

equally likely (they aren't).

You can decrease the probability that an error can slip through, at the expense

of transmission efficiency, by using the SET BLOCK-CHECK command to select more

rigorous block check methods.  Note that all  three  methods  will  detect  any

single-bit error, or any error in an odd number of bits.  The options are:

1-CHARACTER-CHECKSUM:

The normal single-character 6-bit checksum.

2-CHARACTER-CHECKSUM:

A  2-character, 12-bit checksum.  Reduces the probability of an error going

undetected to 1/4096, but adds an extra character to each packet.

3-CHARACTER-CRC:

A  3-character,  16-bit Cyclic Redundancy Check, CCITT format.  In addition

to errors in any odd number of bits, this method detects double bit errors,

all error bursts of length 16 or less, and more than 99.99% of all possible

longer bursts.  Adds two extra characters to each packet.

The single character checksum has proven to be quite adequate in practice, much

4.17. The SET Command                                                   Page 36

-------------------------------------------------------------------------------

more  effective  than straightforward analysis would indicate, since all errors

are not equally likely, and a simple checksum is well suited  to  catching  the

kinds of errors that are typical of telecommunication lines.  The other methods

should be requested only when the connection is very noisy and/or when  sending

binary files, or when using "long packets" (see SET RECEIVE PACKET-LENGTH).

Note that the 2- and 3-character block checks are not available in all versions

of Kermit; if the other Kermit is not capable of performing  the  higher-preci-

sion  block  checks,  the  transfer will automatically use the standard single-

character method.

SET DEBUG {ON, OFF}

Syntax: SET DEBUG options

Record debugging information, either on your terminal or in a  file.    Options

are:

ON  Turn on debugging.

OFF Don't  display debugging information (this is the default).  If debugging

was in effect, turn it off and close any log file.

or possibly others, like STATES, PACKETS, SESSION, etc., to select  logging  of

different  phenomena.  Some Kermit programs may control debugging by use of the

LOG DEBUG command.

SET DELAY

Syntax: SET DELAY number

Specify how many seconds to wait before sending the first packet after  a  SEND

command.    Use  when remote and SENDing files back to your local Kermit.  This

gives you time to "escape" back and issue a RECEIVE command.  The normal  delay

is 5 seconds.  In local mode or server mode, Kermit does not delay before send-

ing the first packet.

SET DUPLEX

Syntax: SET DUPLEX {FULL, HALF}

For use when CONNECTed to a remote system.  The keyword choices  are  FULL  and

HALF.   FULL means the remote system echoes the characters you type, HALF means

the local system echoes them.  FULL is the default, and is used by most  hosts.

HALF  is  necessary  when  connecting  to  IBM mainframes.  Half duplex is also

called "local echo"; in some Kermits, use SET  LOCAL-ECHO  ON  instead  of  SET

DUPLEX HALF.

4.17. The SET Command                                                   Page 37

-------------------------------------------------------------------------------

SET ESCAPE

Syntax: SET ESCAPE character

Specify or change the character you want to use to "escape" from remote connec-

tions back to Kermit.  This would normally be a character you don't  expect  to

be  using on the remote system, perhaps a control character like ^\, ^], ^^, or

^_.  Most versions of Kermit use one of these by default.  After you  type  the

escape  character, you must follow it by a single-character "argument", such as

"C" for Close Connection.  The arguments are listed above, under  the  descrip-

tion of the CONNECT command.

SET FILE

Syntax: SET FILE parameter value

Establish  file-related  parameters.    Depending on the characteristics of the

system, it may be necessary to tell Kermit how to fetch an outbound  file  from

the  disk,  or  how  to  store an incoming file.  The actual parameters you can

specify in this command will vary from system to system, and you should consult

the  documentation  for your particular version of Kermit.  Some examples would

be file type (text or binary), byte size (PDP-10 architecture),  record  length

or  block  size  (record  oriented  systems),  end-of-file detection method (on

microcomputers), file naming conversion option.

This can be a very important command if you intend to  transfer  binary  files,

but is normally unecessary for transmitting textual files.

SET FLOW-CONTROL

Syntax: SET FLOW-CONTROL {XON/XOFF,NONE}

For  communicating  with full duplex systems.  System-level flow control is not

necessary to the Kermit protocol, but it can help to use it if the same  method

is  available  on  both  systems.  The most common type of flow control on full

duplex systems is XON/XOFF.  When a system's input buffer comes close to  being

full, it will send an XOFF character (Control-S) to request the other system to

stop sending.  When it has emptied sufficient characters from its input buffer,

it  signals the other system to resume sending by transmitting an XON character

(Control-Q).  This process operates in both directions simultaneously.  The op-

tions  for  the  Kermit SET FLOW command are usually restricted to XON/XOFF and

NONE, which is used to disable this feature.

SET HANDSHAKE

Syntax: SET HANDSHAKE option

For communicating with half duplex systems.  This lets  you  specify  the  line

turnaround  character sent by the half duplex host to indicate it has ended its

transmission and is granting you permission to transmit.  When a  handshake  is

set,  Kermit  will  not  send  a packet until the half duplex host has sent the

specified character (or a timeout has occurred).  The options may include:

4.17. The SET Command                                                   Page 38

-------------------------------------------------------------------------------

NONE    No handshake; undo the effect of any previous SET HANDSHAKE.

XOFF    Control-S.

XON     Control-Q.

BELL    Control-G.

CR      Carriage Return, Control-M.

LF      Linefeed, Control-J.

ESC     Escape, Control-[.

Some Kermit programs may require the option  to  be  specified  by  typing  the

character  literally or entering its numeric ASCII value.  If you use this com-

mand to enable handshaking, you should also SET FLOW OFF.

SET INCOMPLETE

Syntax: SET INCOMPLETE {KEEP, DISCARD}

Specify what to do when a file transfer fails before it is completed.  The  op-

tions are DISCARD (the default) and KEEP.  If you choose KEEP, then if a trans-

fer fails to complete successfully, you will be able  to  keep  the  incomplete

part that was received.

SET LINE

Syntax: SET LINE [terminal-designator]

Specify the terminal line to use for file transfer or CONNECT.  This command is

found on mainframe Kermits, which normally run in "remote mode" using their own

controlling  terminal  for  file transfer.  Specifying a separate line puts the

program in "local mode."  If no line is specified, revert  to  the  job's  con-

trolling terminal, i.e. go back to "remote mode."

SET PORT

Syntax: SET PORT terminal-designator

Specify  the communications port for file transfer or CONNECT.  This command is

found on microcomputer Kermits that run in "local" mode.   SET  PORT  does  not

change  the  remote/local  status but simply selects a different port for local

operation.

SET PARITY

Syntax: SET PARITY {EVEN, ODD, MARK, SPACE, NONE}

Parity is a technique used by communications equipment for detecting errors  on

a  per-character basis; the "8th bit" of each character acts as a check bit for

the other seven bits.  Kermit uses block checks to  detect  errors  on  a  per-

packet basis, and it does not use character parity.  However, some systems that

Kermit runs on, or equipment through which these systems  communicate,  may  be

using character parity.  If Kermit does not know about this, arriving data will

have been modified and the block check will appear to  be  wrong,  and  packets

will be rejected.

4.17. The SET Command                                                   Page 39

-------------------------------------------------------------------------------

If  parity  is  being used on the communication line, you must inform both Ker-

mits, so the desired parity can be added to outgoing characters,  and  stripped

from  incoming  ones.    SET PARITY should be used for communicating with hosts

that require character parity (IBM mainframes are typical examples) or  through

devices  or networks (like GTE TELENET) that add parity to characters that pass

through them.  Both Kermits should be set to the same parity.    The  specified

parity  is used both for terminal connection (CONNECT) and file transfer (SEND,

RECEIVE, GET).

The choices for SET PARITY are:

NONE  (the default) eight data bits and no parity bit.

MARK  seven data bits with the parity bit set to one.

SPACE seven data bits with the parity bit set to zero.

EVEN  seven data bits with the parity bit set  to  make  the  overall  parity

even.

ODD   seven data bits with the parity bit set to make the overall parity odd.

NONE  means no parity processing is done, and the 8th bit of each character can

be used for data when transmitting binary files.

If you have set parity to ODD, EVEN, MARK, or SPACE, then most versions of Ker-

mit  will request that binary files be transferred using 8th-bit-prefixing.  If

the Kermit on the other side knows how to do 8th-bit-prefixing (this is an  op-

tional  feature  of  the Kermit protocol, and not all implementations of Kermit

have it), then binary files can  be  transmitted  successfully.    If  NONE  is

specified, 8th-bit-prefixing will not be requested.

SET PROMPT

Syntax: SET PROMPT string

This allows you to change the program's prompt.  This is particularly useful if

you are using Kermit to transfer files between two systems of the same kind, in

which  case  you  can change the prompts of the Kermit programs involved to in-

clude appropriate distinguishing information.

SET SEND

SET SEND parameter value

Establish parameters for outgoing packets.  This command is generally  used  to

override negotiated values, or to establish before negotiation takes place.

END-OF-LINE character

The ASCII character to be used as a line terminator for outbound  pack-

ets,  if  one  is  required  by  the  other  system, carriage return by

default.  You will only have to use this command for systems  that  re-

quire a line terminator other than carriage return.

PACKET-LENGTH number

Maximum packet length to send between 10 and 94 (decimal).   Shortening

the packets might allow more of them to get through through without er-

ror on noisy communication lines.  Lengthening  the  packets  increases

4.17. The SET Command                                                   Page 40

-------------------------------------------------------------------------------

the  throughput  on clean lines.  This command can be used to specify a

shorter length than the one requested by the other Kermit,  but  not  a

longer one.

TIMEOUT number

How many seconds to wait for a packet before trying again.  A value  of

zero means don't time out -- wait forever.

PAUSE floating-point-number

How many seconds to pause before sending each  data  packet.    Setting

this  to  a  nonzero  value may allow a slow system enough time to con-

solidate  itself  before  the  next  packet  arrives.    Normally,   no

per-packet pausing is done.

PADDING number, PADCHAR character

How much padding to send before a packet, if the other side needs  pad-

ding,  and what character to use for padding.  Defaults are no padding,

and NUL (0) for the padding character.  This command is also handy  for

inserting  special  characters  that  may be required by communications

equipment.

QUOTE character

What  printable character to use for quoting of control characters, "#"

(43) by default.  There should be no reason to change this.

START-OF-PACKET character

The start-of-packet character is the only control character used "bare"

by the Kermit protocol.  It  is  Control-A  by  default.    If  a  bare

Control-A  causes problems for your communication hardware or software,

you can use this command to select a  different  control  character  to

mark the start of a packet.  You must also issue the reciprocal command

(SET RECEIVE  START-OF-PACKET)  to  the  Kermit  on  the  other  system

(providing it has such a command).

SET RECEIVE

Syntax: SET RECEIVE parameter value

Parameters to request or expect for incoming packets, as follows:

END-OF-LINE character

Carriage return (15) by default.

PACKET-LENGTH number

Maximum  length  packet for the other side to send, decimal number, be-

tween 10 and 94, decimal.  Some Kermit programs also  support  a  "long

packet"  protocol  extension for improved file transfer efficiency.  If

you specify a value greater than 94 (and normally  less  than  1000  or

2000),  then  the two Kermits will attempt to negotiate the use of long

packets in the receiver's direction.  If the negotiation is  unsuccess-

ful  (e.g.  because  the sending Kermit does not support long packets),

then ordinary packets will be used automatically.

TIMEOUT number

How  many  seconds the other Kermit should wait for a packet before as-

4.17. The SET Command                                                   Page 41

-------------------------------------------------------------------------------

king for retransmission.

PAUSE floating-point-number

How  many seconds to pause before acknowledging a packet.  Setting this

to a nonzero value will slow down the rate at which  data  packets  ar-

rive,  which  may  be necessary for systems that have "sensitive" front

ends and cannot accept input at a high rate.

PADDING number, PADCHAR character

How many padding characters to request before each incoming packet, and

what the padding character should be.  No Kermits  are  known  to  need

padding,  and  if  one  did, it would request it without your having to

tell it to do so.  This command would only be necessary, therefore, un-

der very unusual circumstances.

QUOTE character

What printable character to use for quoting of control characters,  "#"

(43) by default.  There should be no reason to change this.

START-OF-PACKET character

The control character to mark the beginning of incoming packets.   Nor-

mally SOH (Control-A, ASCII 1) (see SET SEND START-OF-PACKET, above).

SET RETRY

SET RETRY option number

Set the maximum number of retries allowed for:

INITIAL-CONNECTION

How many times to try establishing the initial protocol connection  be-

fore giving up, normally something like 15.

PACKETS How  many  times  to  try sending a particular packet before giving up,

normally 5.  If a line is very noisy, you might want to  increase  this

number.

SET WARNING

SET WARNING {ON, OFF}

Tell  Kermit whether to let incoming files overwrite existing files of the same

name.  If WARNING is ON, then Kermit will warn you of filename  collisions  and

will  attempt to construct a new, unique name for the arriving file, and inform

you what the new name is.  When OFF, Kermit silently overwrites existing  files

when  there's a name collision.  This command may also be called SET FILE WARN-

ING.

4.18. The DEFINE Command                                                Page 42

-------------------------------------------------------------------------------

4.18. The DEFINE Command

DEFINE macroname [command [, command [, ...] ] ]

Define a "macro" to allow convenient association of one or more Kermit commands

with  a  mnemonic  keyword of your choice.  The definition consists of a list a

list of one or more Kermit commands, separated by commas.  If you use Kermit to

communicate with several different kinds of systems, you may set up a macro for

each, for instance:

DEFINE IBM SET PARITY MARK, SET DUPLEX HALF, SET HANDSHAKE XON

DEFINE UNIX SET PARITY NONE, SET DUPLEX FULL, SET HANDSHAKE NONE

DEFINE TELENET SET PARITY MARK, SET RECEIVE TIMEOUT 20

You may then type DO IBM, DO  UNIX,  and  so  forth  to  set  all  the  desired

parameters  with  a  single command.  It is convenient to include these defini-

tions in your Kermit initialization file.

Another other handy use for macros would be for rapid adaptation  to  different

conditions of line noise:

DEFINE CLEAN SET BLOCK-CHECK 1, SET REC PACKET-LEN 94, SET RETRY 5

DEFINE NOISY SET BLOCK 2, SET SEND PACKET 60, SET RETRY 10

DEFINE VERY-NOISY SET BLOCK 3, SET SEND PACKET 40, SET RETRY 20

You  may  redefine an existing macro in the same manner as you defined it.  You

can undefine an existing macro by typing an empty DEFINE command  for  it,  for

instance:

DEFINE IBM

You  can  list  all your macros and their definitions with the SHOW MACROS com-

mand.  Syntax of SET and DO commands may vary among Kermit programs.

4.19. The SHOW Command

Syntax: SHOW [option]

The SHOW command displays the values of the parameters settable by the SET com-

mand.    If  a  particular  option is not requested, a complete display will be

provided.  Type "show ?" for a list of what can be shown.

4.20. The STATISTICS Command

Give statistics about the most recent file transfer, such as the  total  number

of characters transmitted, the effective baud rate, and so forth.  On some sys-

tems, this is SHOW STATISTICS.

4.21. The LOG Command                                                   Page 43

-------------------------------------------------------------------------------

4.21. The LOG Command

Syntax: LOG [option] [filespec]

Log the specified entity to the specified log file.

TRANSACTIONS    Direct Kermit to log transactions, such as  files  successfully

sent  or  received or files that could not be successfully sent

or received.  A transaction is useful recording the progress of

a long, unattended multifile transfer.

SESSION         Create  a transcript of a CONNECT session, when running a local

Kermit connected to a remote system,  in  the  specified  file.

The log is closed when connection is closed.  In some implemen-

tations, logging can be "toggled" by typing the connect  escape

character followed by Q (Quit logging) or R (Resume logging) or

similar single-character commands.  Session-logging  is  useful

for  recording  dialog  with  an  interactive  system,  and for

"capturing"  from systems that don't have Kermit.  No guarantee

can  be made that the file will arrive correctly or completely,

since no error checking takes place.

DEBUGGING       Record debugging information in the specified file.  There  may

be  several options to select the desired information -- entire

packets, state transitions,  internal  program  trace,  etc  --

available via the SET DEBUGGING command.

PACKETS         Record packets, and all communication line traffice during file

transfer, in the specified file.

4.22. The TRANSMIT Command

Syntax: TRANSMIT filespec

Send the contents of the specified file to the  other  system  "bare",  without

protocol,  packets,  error checking, or retransmission.  This command is useful

for sending files to systems that don't have Kermit.  No guarantee can be  made

that  the  target  system will receive the file correctly and completely.  When

receiving a file, the target system would normally be running a text editor  in

text collection mode.

The  current  communication  settings,  such as parity, flow control, and hand-

shake, are obeyed by most Kermit versions that have a TRANSMIT  command.    The

action  is  normally line-at-a-time.  Each line of the file is sent, terminated

by a carriage return (no linefeed), just as you would type it.    Kermit  waits

for  the  remote  system  to echo a linefeed (as it does when you type carriage

return) before it sends the next line.

Thus  TRANSMIT  provides  a  "raw  upload"  capability.    The  opposite,  "raw

download", may be accomplished by using the LOG SESSION command.

4.23. Login Scripts: The INPUT, OUTPUT, CLEAR, and PAUSE Commands       Page 44

-------------------------------------------------------------------------------

4.23. Login Scripts: The INPUT, OUTPUT, CLEAR, and PAUSE Commands

When  running  Kermit  in local mode, you may use the INPUT, OUTPUT, CLEAR, and

PAUSE commands to carry on a dialog with the remote system.  When combined into

a "script" in a Kermit TAKE command file, these commands provide the ability to

initially connect and log in to a remote system, and to  set  it  up  for  file

transfer.    While you may do this yourself manually using the CONNECT command,

the login script facility allows this often tedious task to be  automated,  and

more  important,  allows  it  to take place unattended -- perhaps late at night

when the phone rates are low and the system is faster.

THE CLEAR COMMAND

Syntax: CLEAR

Clear the input and output buffers of the currently selected line, and  attempt

to clear any XOFF deadlock.

THE PAUSE COMMAND

Syntax: PAUSE [interval]

Pause  the  specified number of seconds before executing the next command.  The

default interval is one second.

THE INPUT COMMAND

Syntax: INPUT [interval] [string]

On the currently selected communication line, look for the given string for the

specified  interval  of time, which is specified in seconds.  If no interval is

specified, then wait for the default interval, which may be  specified  by  SET

INPUT  DEFAULT-TIMEOUT, and is normally 5 seconds.  Specifying an interval of 0

(or less) means no timeout -- wait forever for the specified string.  An  INPUT

command  can  normally  be interrupted by typing one or more Control-C's, which

will return you to Kermit prompt level.

Characters coming in from the line will be scanned for the search  string,  and

when  a  match is found, the command will terminate successfully; if the string

is not found within the given interval, the command will  terminate  unsuccess-

fully.

The  search string may contain any printable characters.  Control or other spe-

cial characters that you could not normally type as part of a  command  may  be

included  by preceding their ASCII values with a backslash, for instance foo\13

is "foo" followed by a carriage return  (ASCII  13,  decimal).    (Some  Kermit

programs expect or allow other number bases, such as octal or hexadecimal.)

While  scanning, alphabetic case is ignored ("a" = "A") unless you have SET IN-

PUT CASE OBSERVE.  If no search string is given, then the  INPUT  command  will

simply  display all incoming characters on your screen until it times out or is

interrupted.

4.23. Login Scripts: The INPUT, OUTPUT, CLEAR, and PAUSE Commands       Page 45

-------------------------------------------------------------------------------

If the INPUT command finds the specified string within the  alloted  amount  of

time, it terminates immediately, without an error message, and the next command

will be executed.  If the INPUT command fails to find the requested string,  it

will  "fail"; failure is only significant if the command was issued from a TAKE

command, and INPUT TIMEOUT-ACTION is SET to QUIT.  When a timeout occurs  under

these  conditions,  the  command  file is immediately terminated and control is

returned to the invoking level, either Kermit prompt level or a  superior  com-

mand  file.    If INPUT TIMEOUT-ACTION is SET to PROCEED, then the next command

(if any) will be executed from the current command file.

In addition to otherwise untypable control characters (like Control-C), certain

printable  characters  in  the  search string may need to be "quoted" using the

backslash mechanism, including at-sign, question mark, or other characters that

are significat to Kermit's command processor.

THE OUTPUT COMMAND

Syntax: OUTPUT string

The  given  string  is sent out the currently selected communication line.  The

string is in the same form as the INPUT string; control or  special  characters

may  be  included  by prefacing their octal ASCII value with a backslash.  Note

that any terminating  carriage  return  must  be  included  explicitly  as  \13

(decimal).

HOW TO USE LOGIN SCRIPTS

Login  scripts  are  useful  on  computers  that  have autodialers or TTY ports

hardwired or otherwise connected to terminal ports on other systems.    Scripts

can  be  used to automate the task of connecting and logging in.  For instance,

suppose your PC is connected to a VAX Unix system through a hardwired  line  on

communication  port  2.  To send a file to the VAX, you must connect to the VAX

through the port, log in, run Unix Kermit, escape back to the PC, and issue the

appropriate  file  transfer  commands, then connect back to the VAX and logout.

This may all be automated by means of the following "script"  stored  in  a  PC

file invoked by the Kermit TAKE command:

set port 2

output \13

input login:

out myuserid\13

in 10 Password:

out mypassword\13

in 20 %

out kermit -r\13

send foo.bar

out \4

input

The  first line points Kermit at the communication line.  The next line sends a

carriage return, which makes Unix issue a "login:" prompt; the following  INPUT

command  waits  for  this  prompt  to  appear.    When  it does, Kermit outputs

"myuserid" followed by a carriage return.  Unix then prompts  for  a  password;

after  the prompt appears, Kermit supplies the password.  Then, Kermit waits up

4.23. Login Scripts: The INPUT, OUTPUT, CLEAR, and PAUSE Commands       Page 46

-------------------------------------------------------------------------------

to 20 seconds for the Unix shell's "%" prompt; this  allows  time  for  various

system  messages  to be displayed.  When the shell prompt appears, Kermit sends

the command "kermit -r", which tells Unix Kermit to receive a file; then a SEND

command  is  given  to the local Kermit.  After the file is successfully trans-

ferred, Kermit sends a logout command (Control-D, "\4") to Unix.  The final IN-

PUT  command  causes  Kermit  to  display  any  typeout  (in this case the Unix

system's logout message) that occurs up to the default timeout interval.

The INPUT command is very important, because it ensures synchronization.    One

might expect to be able to simply send all the characters out the communication

line at once, and let the remote host's typeahead and buffering facilities take

care  of the synchronization.  In rare or simple cases, this might work, but it

assumes that (a) the remote  host  allows  typeahead,  (b)  the  remote  host's

typeahead  buffers  are  big  enough to accommodate all the characters, and (c)

that the remote host never clears pending typeahead.  These  conditions  rarely

hold.  For instance, Unix clears its input buffer after issuing the "Password:"

prompt; any typeahead will be lost.  Interactive users as well as login  script

facilities  must wait for the prompt before entering the password.  This is the

function of the INPUT command.  On half duplex systems, this function is criti-

cal  --  these systems cannot accept any input in advance of a prompt; there is

no typeahead.

The Kermit script facility is not a programming  language;  there  is  no  con-

ditional execution of commands, no branching, no labels.  Nevertheless, the SET

INPUT command provides a degree of control.  If the Unix system  were  down  in

the  sample script above, Kermit would still proceed merrily through the entire

script, sending its output into the void and waiting the entire timeout  inter-

val  on  each  INPUT  command,  then attempting to send a file to a Kermit that

wasn't there.  It could take several minutes of timing  out  to  terminate  the

script.  This could be avoided by including the command

SET INPUT TIMEOUT-ACTION QUIT

at the top of the script.  When the "login:" prompt failed to appear within the

timeout interval, the rest of the script would be cancelled.

See the chapters on MS-DOS and DEC-20 Kermit  for  further  examples  of  login

scripts.

5. MS-DOS KERMIT                                                        Page 47

-------------------------------------------------------------------------------

5. MS-DOS KERMIT

Program:        Joe  R.  Doupnik (Utah State University), with contributions by

James  Harvey  (Indiana/Purdue  University),  James  Sturdevant

(A.C. Nielson Company), and many others (see History).

Language:       Microsoft Macro Assembler (MASM)

Version:        2.30

Released:       January 8, 1988

Documentation:  Christine Gianone, Frank da Cruz (Columbia University),

Joe R. Doupnik (Utah State University)

Dedicated To:   Peppi

Kermit-MS Capabilities At A Glance:

Local operation:                Yes

Remote operation:               Yes

Transfers text files:           Yes

Transfers binary files:         Yes

Wildcard send:                  Yes

File transfer interruption:     Yes

Filename collision avoidance:   Yes

Can time out:                   Yes

8th-bit prefixing:              Yes

Repeat count compression:       Yes

Alternate block check types:    Yes

Terminal emulation:             VT102, H19, VT52, Tektronix 4010

Communication settings:         Speed, Parity, Flow Control, Handshake, Echo

Transmit BREAK:                 Yes (and Long BREAK)

IBM mainframe communication:    Yes

Transaction logging:            No

Session logging (raw download): Yes

Raw upload:                     Yes

Act as server:                  Yes

Talk to server:                 Yes

Advanced server functions:      Yes

Advanced commands for servers:  Yes

Local file management:          Yes

Command/init files:             Yes

Command macros:                 Yes

Extended-length packets:        Yes

Local area networks:            Yes (NetBIOS support)

MS-Windows compatibility:       Yes

Attribute packets:              No

Sliding windows:                No

MS-DOS  Kermit, or "Kermit-MS" (or MS-Kermit), is a program that implements the

Kermit file transfer protocol for the entire IBM PC family, including the  PS/2

series,  IBM  compatibles,  and  several other machines based on the Intel 8086

processor series (8088, 80286, 80386, etc) and the DOS operating system  family

(PC-DOS  or  MS-DOS,  henceforth  referred  to collectively as MS-DOS or simply

5. MS-DOS KERMIT                                                        Page 48

-------------------------------------------------------------------------------

DOS).

It is assumed you are acquainted with your PC and with DOS, and  that  you  are

familiar with the general ideas of data communication and Kermit file transfer.

A very brief overview is given here, but for details consult the early chapters

of  the  Kermit  User  Guide (of which this document is a chapter), or the book

Kermit, A File Transfer Protocol, by Frank da Cruz, Digital Press (1987), order

number  EY-6705E-DP  (phone  1-800-343-8321),  which  also  includes background

tutorials on computers, file systems, and data communication (including modems,

cabling,  etc).    For further information about Kermit documentation, updates,

lists of current available versions, and ordering information, write to:

Kermit Distribution

Columbia University Center for Computing Activities

612 West 115th Street

New York, NY  10025  (USA)

5.1. System Requirements

Kermit-MS version  2.30  runs  in  as  little  as  70K  of  memory  (about  55K

contiguous),  but  will occupy up to 120K, if it can be found, for extra screen

rollback memory.  Versions not using screen rollback memory  will  not  require

the  additional  space.   It will also try to leave 24 Kbytes free for a second

copy of COMMAND.COM which is needed for execution of certain commands.

On the IBM PC family, Kermit-MS 2.30 performs almost complete emulation of  the

DEC  VT-102  and  Heath/Zenith-19  terminals  at  speeds  up  to 19,200 baud or

greater, lacking only the VT102's smooth scrolling and (on some display boards)

132 column features.  And as of version 2.30, Kermit-MS also performs Tektronix

4010 graphics terminal emulation on IBM PC family systems  equipped  with  CGA,

EGA, and Hercules graphics adapters, with either color or monochrome monitors.

Much  of  Kermit's speed is accomplished by direct writes to screen memory, but

this is done in a "TopView-aware" manner to allow successful operation in  win-

dowing  environments  like  MS-Windows, DesqView, and TopView itself.  Speed is

also due to direct access of the serial port 8250  UART (Universal Asynchronous

Receiver/Transmitter)  chip, with buffered, interrupt-driven receipt of charac-

ters and selectable XON/XOFF  flow control.  Full speed 9600 baud operation  is

possible  on 4.77Mhz systems without flow control, but flow control is required

on these systems for 19,200 baud or higher rates.  The IBM  PC  version  should

also  run  on near-clones like the DG/1 that differ from true PCs only in their

choice  of  UART;  non-8250  UARTs  are  detected  automatically,  and   slower

non-interrupt  driven Bios serial port i/o is used, in which case the top speed

is in the 1200 baud range.  On the IBM PC family, COM1 and COM2 are  supported,

and "hooks" are available for (inevitably nonstandard) COM3 and COM4 options.

Kermit-MS 2.30 runs on the entire IBM PC family (the PC, XT, AT, PCjr, Portable

PC, PC Convertible, PS/2) and compatibles (Compaq,  VAXmate,  Z150,  etc),  and

there are also specially tailored versions for non-IBM-compatibles like the DEC

Rainbow, NEC APC, Sanyo MBC, Victor 9000, HP-110, HP-150, HP Portable Plus, and

others, plus a "generic DOS" version that should run (slowly) on any 8086-based

MS-DOS machine.  This document concentrates on the IBM  version;  some  of  the

system-dependent capabilities described here may be lacking in the non-IBM ver-

sions.  See section 5.9 for features of different systems.

5.1. System Requirements                                                Page 49

-------------------------------------------------------------------------------

KERMIT.EXE for the IBM PC family occupies about 86K of disk storage (the figure

will  vary for other versions).  This can be reduced by about 15K if you run it

through EXEPACK.  MS-Kermit is not distributed in packed form, because problems

have  been  reported on certain systems when this is done.  So if you decide to

pack it, make sure to keep an unpacked version available to  fall  back  to  in

case of problems.

5.2. History

Over  the  years,  MS-Kermit has grown from a Kermit file transfer program that

embodied a simple terminal emulator into a complex and  powerful  communication

program that includes the Kermit file transfer protocol.  As a result, the bulk

of this manual is devoted to the communication  features,  rather  than  Kermit

protocol operation.  Skip ahead to the next section if you're not interested in

the history of MS-Kermit.

MS-DOS Kermit (like the Kermit file transfer protocol itself) is a  product  of

the  Systems  Group of the Columbia University Center for Computing Activities,

and it was one of the four original Kermit programs (with the CP/M, DEC-20, and

IBM  mainframe versions).  It was initially written for the IBM PC with DOS 1.1

by Daphne Tzoar in 1981-1982, based largely on Bill Catchings's  original  CP/M

8080  assembler version.  PC-Kermit (as it was called then) provided basic Ker-

mit file transfer and VT52 emulation.  Joellen Windsor  of  the  University  of

Arizona  added  conditional  assembly  support for the Heath/Zenith-100 shortly

thereafter, and soon after that Dave King of Carnegie-Mellon  University  added

Heath-19  terminal emulation, and some patches to let the program run under the

new DOS version, 2.0.  During this era, the program version numbers  went  from

1.0 to 1.20.

With  the  appearance  in the marketplace of many new MS-DOS machines that were

not compatible with the IBM PC, it became apparent that conditionally assembled

code  supporting  each of these machines within a single monolithic source file

was not the best way to organize the program.   Therefore  Daphne,  along  with

Jeff  Damens of Columbia, undertook to reorganize the program in a modular way,

isolating system dependencies into separate files.    The  result  was  version

2.26,  released  in  July  1984.   It included support for the DEC Rainbow, the

HP-150, the Wang PC, and generic MS-DOS, as well as for the IBM PC  family  and

the  H/Z-100.  It also included many new features, like 8th-bit prefixing (code

contributed by The Source  Telecomputing),  alternate  block  check  selection,

byte-count  compression,  server/client operation, access to local file and DOS

operations, command macros, initialization and command files, screen  rollback,

key  redefinition,  and  more.    For  the  2.26 release, the executable Kermit

programs were encoded printably as ".BOO" files, designed by Bill Catchings  as

part of this effort.

Release 2.27 was produced by Daphne and Jeff in December 1984.  Unlike 2.26, it

ran correctly on the new PC/AT under DOS 3.0, and included support for the  NEC

APC  from  Ron  Blanford  of  Seattle, WA, and Ian Gibbons of the University of

Hawaii, and for the TI Professional from Joe Smith of the  Colorado  School  of

Mines,  plus  some bug fixes and reorganization.  2.27 is the last version that

runs under pre-2.0 versions of DOS.

Version 2.28 (Daphne, Jeff, June  1985)  added  dynamic  memory  allocation  to

reduce  disk  storage for the .EXE file, and to allow the program to adjust it-

self to the PC's memory size, plus the inevitable bug fixes (many of them  con-

5.2. History                                                            Page 50

-------------------------------------------------------------------------------

tributed  by  Edgar  Butt  of the University of Maryland and Gregg Small of the

University of California at Berkeley).  During this  period,  support  for  ad-

ditional MS-DOS systems was added by various people.

In  December 1985, a tape showed up at Columbia sent by Prof. Joe R. Doupnik of

the Center for Atmospheric and Space Studies and EE Department  at  Utah  State

University.  This tape contained version 2.28 modified to fully support the DOS

2.0 file system, and to which many new features had  been  added,  notably  the

ability  of  the  MS-DOS Kermit server to process various REMOTE commands (DIR,

CWD, SPACE, etc).  And at about the same time, a tape arrived from James Harvey

of  Indiana/Purdue University, who had changed Kermit's CONNECT command to emu-

late the popular DEC VT100 terminal.  This material was sent to Joe,  who  then

laboriously  fitted  James's  work  into his own code, keeping the VT52 and H19

emulation alive as options, and upgrading the VT100 emulation to VT102  by  ad-

ding  features  such  as line and character insertion and deletion.  The result

was version 2.29, released in May 1986.

Soon after the release of 2.29, some disks were sent in by James Sturdevant  of

the A.C. Nielson Company, containing a full implementation of the Kermit script

facility, as described in the Kermit book.  This material was sent to Joe,  who

had by now become keeper of MS-DOS Kermit and had already begun work on version

2.30 by adding support for extended-length packets.  Joe had been  carrying  on

voluminous  network  correspondence  (Thanks,  BITNET!)  with Columbia and with

MS-DOS Kermit users and testers all over the world, giving birth  to  many  new

features,  including  Tektronix graphics terminal emulation, support for opera-

tion over local area networks, support for 8-bit ASCII terminal connections and

international  character  sets,  ANSI  printer  control, and a redesigned, more

powerful, more portable key redefinition mechanism.  Version 2.30 was  formally

released on January 1, 1988, after many "alpha" and "beta" tests.

Among the many contributors to this version are: Brian Holley and Joe Smith for

the Tektronix emulation, Robert Goeke for the NEC AP3 support,  Brian  Peterson

and  Andreas  Stumpf  for  the  Victor  9000, Bob Babcock and Joe White for the

Sanyos, Christopher Lent for the Wang PC, Jack Bryans for an  Intel  iRMX  ver-

sion,  Jim Noble for the Grid Compass, Geoff Mulligan and others for the Zenith

100, and David Knoell for the special Rainbow edition.  And thanks  to  Gisbert

Selke,  Jack  Bryans,  and  others for proofreading drafts of this manual.  And

apologies to anyone we neglected to mention.

Finally, please note that the program version number is not a whole number  and

a fraction; 2.30 is pronounced "two point thirty", and is not equal to 2.3.

5.3. Using MS-Kermit

MS-DOS  Kermit performs two major functions, terminal emulation and file trans-

fer.  File transfer can be done using either the Kermit file transfer protocol,

or  else  (without  error checking), ASCII or XON/XOFF capture and transmission

methods.  To use Kermit for "raw" uploading or downloading of  files,  see  the

descriptions of the LOG SESSION and TRANSMIT commands.

Before  you  can  transfer files with another system using Kermit protocol, you

must first connect to it as a terminal, login if necessary, and start up a Ker-

mit  program  there.   The following example shows this process; the other com-

puter is a Unix system, but the method is the same with most others.  The parts

you  type  are  underlined  (if this document was printed on a printer that can

5.3. Using MS-Kermit                                                    Page 51

-------------------------------------------------------------------------------

underline), and when you type a command,  you  terminate  it  with  a  carriage

return,  which  you  can't  see  in  the  example.    The  mysterious  "^]c" is

MS-Kermit's "escape sequence", which you enter  by  holding  down  the  Control

(Ctrl)  key and pressing "]" (right square bracket), and then typing the letter

C. The example assumes the MS-Kermit program is stored on disk as KERMIT.EXE.

Program Dialog:             Explanation:

A>kermit

IBM PC Kermit-MS V2.30 8 Jan 88

Type ? for help

Kermit-MS>set speed 1200    Set the right baud rate.

Kermit-MS>connect           Connect as a terminal.

(Connecting to host, type ^]C to return to PC.)

ATDT7654321                 Dial the modem if necessary.

CONNECT 1200                The modem tells you you're connected.

Now you're talking to the Unix system.

Type a carriage return to get its attention.

Login: christin             Login to the host.

password:                   (Passwords normally don't echo.)

% kermit                    Run Kermit on the host.

C-Kermit>receive            Tell it to receive a file.

^]c                         Escape back to the PC.

Kermit-MS>send autoexec.bat Send a file.

(The file is transferred...)

Kermit-MS>                  Transfer complete, prompt reappears.

In this example, the user types "kermit", and sees the program's herald and its

prompt, "Kermit-MS>".  Then she sets the appropriate communication speed ("baud

rate"), connects as a terminal, issues a dialing command to a Hayes-like  modem

(you would skip this step if you had a direct connection), logs in to her ID on

the Unix system which she has dialed, starts "C-Kermit"  on  the  Unix  system,

tells it to receive a file, escapes back to the PC, and tells MS-Kermit to send

a file.  After the file is transferred, the user would normally connect back to

the Unix system, exit from the Kermit program there, and log out:

Kermit-MS>connect        Connect again.

(Connecting to host, type ^]C to return.)

C-Kermit>exit

% ^D                     Logout from Unix by typing Ctrl-D.

^]c                      Escape back to the PC.

Kermit-MS>exit           Return to DOS.

To  transfer  a  file  in  the  other direction, simply exchange the "send" and

"receive" commands above.  That's the easiest and quickest way to  use  Kermit.

If  this simple scenario does not work for you, issue the MS-Kermit STATUS com-

mand and look for any obvious incorrect settings (speed, parity), fix them with

SET  commands  (described  in  Section  5.6.9),  and try again.  (IBM mainframe

linemode connections have so many "different" settings, there's a special  com-

mand  to  do  them  all  at  once,  "do ibm", which you would type as the first

Kermit-MS command above.)  If that doesn't help, read on.   Many  problems  can

5.3. Using MS-Kermit                                                    Page 52

-------------------------------------------------------------------------------

crop  up when you attempt to connect two unlike systems over a possibly hostile

communication medium.  And if you intend to be a frequent user of Kermit, there

are many options you can take advantage of to adapt MS-Kermit to different sys-

tems, improve its performance, and automate common tasks.

5.4. The MS-DOS File System

The features of the MS-DOS file system of greatest interest to Kermit users are

the form of the file specifications, and the formats of the files themselves.

5.4.1. File Specifications

MS-DOS file specifications (in version 2.0 or later of DOS) are of the form

DEVICE:\PATHNAME\NAME.TYPE

where  the  DEVICE  is  a  single character identifier (for instance, A for the

first floppy disk, C for the first fixed disk, D for a RAM disk emulator)  fol-

lowed by a colon (":"), PATHNAME is up to 63 characters of identifier(s) (up to

8 characters each) surrounded by backslashes ("\"), NAME is an identifier of up

to  8  characters,  and  TYPE is an identifier of up to 3 characters in length.

Device and pathname may be omitted.  The first backslash in the pathname may be

omitted  if  the  specified  path is relative to the current directory.  In the

path field, "." means the current directory, ".." means the  parent  directory.

Some  DOS implementations (like Wang) may use slash ("/") rather than backslash

as a directory separator.

Pathname is normally omitted, but can be specified in  all  Kermit-MS  commands

(as of version 2.29).  Device and directory pathnames, when omitted, default to

either the user's current disk and  directory,  or  to  the  current  directory

search path as specified in the DOS PATH environment variable, depending on the

context in which the file name appears.

When this document says that a file is searched  for  "in  the  current

path,"  it means that Kermit-MS looks on the current disk and directory

first, and if the file is not found, then the directories listed in the

PATH  environment variable are searched.  If the PATH environment vari-

able is empty, Kermit looks only at the current disk and directory.

NAME.TYPE is sufficient to specify a file on the current  disk  and  directory,

and only this information is sent along by Kermit-MS with an outgoing file.

The  device, path, name, and type fields may contain uppercase letters, digits,

and the special characters "-" (dash), "_" (underscore), "$" (dollar sign), "&"

(ampersand),  "#"  (number  sign),  "@" (at sign), "!"  (exclamation mark), "'"

(single  quote),  "()"  (parentheses),  "{}"  (curly  braces),  "^"  (caret  or

circumflex), "~" (tilde), and "`" (accent grave).  Normally, you should confine

your filenames to letters and digits for maximum  transportability  to  non-DOS

systems.    When  you  type  lowercase letters in filenames, they are converted

automatically to uppercase.  There are no imbedded or trailing spaces.    Other

characters  may  not be included; there is no mechanism for "quoting" otherwise

illegal characters in filenames.  The fields of the file specification are  set

off from one another by the punctuation indicated above.

5.4.1. File Specifications                                              Page 53

-------------------------------------------------------------------------------

The  name  field is the primary identifier for the file.  The type, also called

the extension or suffix, is an indicator which, by convention, tells what  kind

of  file  we have.  For instance FOO.BAS is the source of a BASIC program named

FOO; FOO.OBJ might be the  relocatable  object  module  produced  by  compiling

FOO.BAS;  FOO.EXE  could  be an executable program produced by loading FOO.OBJ,

and so forth.  .EXE and .COM are the normal suffixes for executable programs.

MS-DOS allows a group of files to be specified in a single  file  specification

by including the special "wildcard" characters, "*" and "?".  A "*" matches any

string of characters from the current position to the end of the field, includ-

ing  no  characters  at all; a "?" matches any single character.  Here are some

examples:

*.BAS   All files of type BAS (BASIC source files) in the current directory.

FOO.*   Files of all types with name FOO.

F*.*    All files whose names start with F.

*.?     All files whose types are exactly one character long, or have no type

at all.

Wildcard  notation  is used on many computer systems in similar ways, and it is

the mechanism most commonly used to instruct Kermit to send a group of files.

Users of Kermit-MS should bear in mind that other (non-MS-DOS) systems may  use

different wildcard characters.  For instance VMS and the DEC-20 use "%" instead

of "?" as the single character wildcard; when  using  Kermit-MS  to  request  a

wildcard  file  group  from a Kermit-20 server, the DOS "?" must be replaced by

the DEC-20 "%".

5.4.2. File Formats

MS-DOS systems store files as streams of 8-bit bytes, with no  particular  dis-

tinction  among text, program code, and binary files.  ASCII text files consist

of lines separated by carriage-return-linefeed sequences (CRLFs), and this con-

forms  exactly  to the way Kermit represents text files during transmission, so

Kermit-MS has no need for a SET FILE TYPE BINARY command.   But  since  a  non-

MS-DOS  receiving  system  might need to make distinctions as to file type, you

will probably have to issue SET FILE TYPE commands there if you are sending  it

non-text  files.   In transmitting files between Kermit-MS programs, regardless

of file contents, the receiving MS-DOS system is equally capable of  processing

text, code, and data, and in fact requires no knowledge of how the bytes in the

file are to be used.

MS-DOS (unlike CP/M) knows the exact end of a file  because  it  keeps  a  byte

count  in  the  directory,  so one would expect no particular confusion in this

regard.  However, certain MS-DOS programs continue to use the  CP/M  convention

of  terminating  a text file with a Control-Z character, and won't operate cor-

rectly unless this terminating byte is present.  Therefore, you should be aware

of  a  special  SET  EOF option for both incoming and outbound files, described

later.

Non-MS-DOS  systems  may  be  confused  by  nonstandard  ASCII  files  sent  by

Kermit-MS:

5.4.2. File Formats                                                     Page 54

-------------------------------------------------------------------------------

- Files  containing  any  of  the 8-bit "extended ASCII" characters may

need conversion (or translation) to 7-bit ASCII.

- Files produced by word processing programs like Word Perfect or  Word

Star may contain special binary formatting codes, and could need con-

version to conventional 7-bit ASCII  format  prior  to  transmission,

using commonly available "exporter" programs.

- Files  created  by  word processors that store formatting data at the

end of the file, after the Control-Z and before physical end, may re-

quire  special  processing  via SET EOF to strip the formatting data,

lest they confuse non-MS-DOS recipients.

- Spreadsheet or database files usually need special formatting  to  be

meaningful  to  non-MS-DOS recipients (though they can be transmitted

between MS-DOS systems with Kermit-MS).

- BASIC programs are normally saved in a binary "tokenized" form.   Use

BASIC's ",a" SAVE option to save them as regular ASCII text, as in

save"foofa",a

In  general,  when  attempting  to transfer non-text files between MS-DOS and a

different kind of system, consult the Kermit manual for that system.

5.5. Program Setup and Invocation

The MS-DOS Kermit program can be run from any disk without any special  instal-

lation  procedure.  On hard disk systems, it is convenient to store the program

in one of the directories listed in your DOS PATH, and it is often desirable to

customize  Kermit's  operation to your communications and computing environment

by creating an initialization file.

Kermit-MS can be run interactively, from a batch file, or as an "external"  DOS

command.   Commands consist of one or more fields, separated by "whitespace" --

one or more spaces or tabs.

Upon initial startup, the program executes  any  commands  found  in  the  file

MSKERMIT.INI in the current path.  This initialization file may contain command

macro definitions, communications settings for one or more ports, or any  other

Kermit-MS  commands,  and  you  may  create it using any text editor capable of

saving files in plain ASCII text format.  Here is a sample:

comment -- MSKERMIT.INI, MS-DOS Kermit initialization file

comment -- Don't overwrite my files!

set warning on

comment -- Define macros for the systems I use...

define unix set local-echo off,set par non,set flow xon,set timer off

def ibm set par odd,set loc on,set hands xon,set flo none,set tim on

def modem set port 2, set baud 1200

comment -- Define a macro for quickly adapting to noisy connections...

def noisy set block-check 3, set send packet-length 40, set retry 20

5.5. Program Setup and Invocation                                       Page 55

-------------------------------------------------------------------------------

comment -- I always start out by connecting to my UNIX system...

set port 1

set baud 4800

do unix

connect

A different file may be substituted for MSKERMIT.INI by using "-f filename"  on

the DOS command line, e.g.

kermit -f monday.ini

The  meanings  of these commands will emerge below.  For now, just note how you

can use command files (and "macro definitions") to easily  adapt  MS-Kermit  to

widely  differing  communication  environments.  A more advanced initialization

file is shown below in section 5.8.

Interactive Operation:

To run Kermit-MS interactively, invoke the program from DOS  command  level  by

typing  its name, normally "kermit" (this means the program should be stored in

your path with the name KERMIT.EXE).  When you see the program's prompt,

Kermit-MS>

you may type Kermit commands  repeatedly  until  you  are  ready  to  exit  the

program,  as  in  the following example (which assumes there's already a Kermit

"server" set up on the other end):

A>

A>kermit

IBM PC Kermit-MS V2.30  8 Jan 88

Type ? for help

Kermit-MS>set speed 19200

Kermit-MS>send foo.*

The files are sent.

Kermit-MS>get fot.*

The requested files are received.

Kermit-MS>exit

A>

Interactive commands are described in Section 5.6.

Command Line Invocation:

Kermit-MS may be invoked with command line arguments from  DOS  command  level,

for instance:

A>kermit send peter.amy

5.5. Program Setup and Invocation                                       Page 56

-------------------------------------------------------------------------------

or

A>kermit set port 1, set baud 9600, connect

In  this case, help and completion  are not available (because the program that

provides them won't start running until  after  you  type  the  entire  command

line),  and Kermit-MS will exit back to DOS after completing the specified com-

mand or commands.    Therefore,  when  invoked  with  command  line  arguments,

Kermit-MS  will  behave as if it were an external DOS command, like MODE.  Note

that several commands may be given on the command line,  separated  by  commas.

This can't be done interactively or from TAKE command files.

As  of  version  2.30,  two new Kermit commands can be given on the DOS command

line.  First is the keyword STAY which prevents Kermit from  exiting  naturally

when  the last command has completed (unless, of course, EXIT or QUIT was among

the commands).  The second command is

-F filename

This means use the indicated filename as the initialization  file  rather  than

MSKERMIT.INI.   The path will be searched for this file, if necessary.  A space

or tab must separate -F from the filename, and the F may be in upper  or  lower

case.  Example:

kermit -f tuesday.ini, set port 2, do ibm, stay

You can run Kermit with no initialization file at all by using the command

kermit -f nul

If -F is the only command line option, STAY is implied.

Batch Operation:

Like  other MS-DOS programs, Kermit-MS may be operated under batch with command

line arguments.  If you invoke it without command line arguments, it  will  run

interactively, reading commands from the keyboard and not the batch file.  When

it exits, batch processing will continue to the end of the batch file.

Kermit-MS returns the "errorlevel"  parameter  used  as  program  exit  status.

Present  values  are  in  the range 0 to 7 with three areas yielding success or

failure reports for the entire Kermit session. The errorlevel values are:

errorlevel   Kermit session status

0       entirely successful operation

1       a Send command completed unsuccessfully

2       a Receive or GET command completed unsuccessfully

4       a REMOTE command completed unsuccessfully

3,5,6,7    combinations (addition) of the above conditions

Note that failures are remembered for the whole session and are not canceled by

a  following successful operation of the same type. Thus, sending several files

individually yields an errorlevel of 0 only if all the files were sent success-

fully.  The "errorlevel" parameter also applies to script commands where OUTPUT

corresponds to SEND and INPUT to RECEIVE.  An example of  Batch  invocation  of

5.5. Program Setup and Invocation                                       Page 57

-------------------------------------------------------------------------------

Kermit is shown in Figure 5-3.

Remote Operation:

The MS-DOS CTTY command allows an MS-DOS system to be used from a terminal con-

nected to its communication port.  Such sessions must be conducted  with  great

care, since many programs assume that they are running on the real console, and

explicitly reference screen memory or keyboard scan codes.  Kermit can be  used

in this manner too, but before you give it any file transfer commands, you must

inform it that it is running in "remote mode" rather  than  its  normal  "local

mode."    Use  the  SET REMOTE ON command for this purpose, to prevent the file

transfer display from being sent out the port.

RAM Disk Operation:

If you invoke Kermit frequently, and you have sufficient memory on your PC, you

may find it convenient to copy Kermit and its initialization file to a RAM disk

when you start your system.  This allows Kermit to be started and used  quickly

and silently, with no disk operations.

For  instance, if you're using IBM's VDISK facility to create the RAM disk, you

might put statements like this in your CONFIG.SYS file:

DEVICE=VDISK.SYS 512 512 32 /e

This assumes you have 512K of extended (/e) memory installed and  VDISK.SYS  is

in  the  root directory of the boot disk.  It creates a 512K RAM disk with 512K

sector size and space for 32 directories in the extended memory,  assigning  it

the  disk letter of your first unused disk.  And then in your AUTOEXEC.BAT file

(assuming the RAM disk is disk D:)...

COPY KERMIT.EXE   D: >NUL

COPY MSKERMIT.INI D: >NUL

COPY COMMAND.COM  D: >NUL

SET COMSPEC=D:\COMMAND.COM

PATH D:\; ...

APPEND D:\; ...

The PATH and APPEND commands allow DOS to find KERMIT.EXE, and Kermit  to  find

MSKERMIT.INI  and  COMMAND.COM, on the RAM disk.  If you use Kermit to transfer

files to your RAM disk, remember to copy those files to a real disk before  you

turn off the system.

Use of MS-Kermit in Windowing Environments:

Kermit-MS  can  operate  within  windowing  environments  like such as TopView,

DESqview, and MS-Windows.  It runs in an active window  under  MS-Windows,  ac-

cepts  cut and paste material, talks with mice, and shrinks to an icon (a boxed

"KER").  An MS-Windows .PIF file  can  be  constructed  for  Kermit  using  the

PIFEDIT  program,  supplied with Windows.  Memory requirements should be listed

as 90 to 128KB.  It should be noted that Kermit does  not  modify  the  screen,

keyboard,  memory,  COM1,  or  COM2 (!).  Program switch and exchange should be

marked as Text, and Close Window on Exit should be checked.  This configuration

5.5. Program Setup and Invocation                                       Page 58

-------------------------------------------------------------------------------

will let you run Kermit with all the Windows features, but very slowly.  To run

at full speed under Windows, tell PIFEDIT  that  Kermit  modifies  the  screen.

Then  you lose the Windows features (cutting, pasting, running the clock at the

same time, etc), but you still get back to the Windows interface when you  EXIT

Kermit.

Local Area Network Operation:

MS-Kermit  2.30 is capable of using a serial port on another local area network

(LAN) node, so long as that node is running an asynchronous communication serv-

er  and  you  have  installed a device driver on your own PC that makes COM1 or

COM2 i/o use the network  server.    This  type  of  connection  works  because

MS-Kermit  2.30 (but not earlier releases) on IBM PCs checks the selected port,

COM1 or COM2, to see if it's a real 8250 UART chip, and  if  it  isn't,  Kermit

uses  only  Bios  calls for port i/o, and the network routes these through your

network device driver.  It may be necessary to turn off a  real  COM1  or  COM2

device  (with  a  switch  or jumper on the board) to convince Kermit to use the

Bios.  This style of operation should be transparent to Kermit, except that not

all asynchronous communications servers utilize this technique.

As  of version 2.30, the IBM PC version of Kermit can also communicate directly

with another PC on a local area network through the IBM NetBIOS  emulator  dis-

tributed  with  the  LAN.  In essence, the LAN substitutes for the serial port,

modem, and other wiring.  Kermit running on one user machine can transfer files

with  another  Kermit  also  on  the  network much as if they were connected by

modems, and Kermit can talk with some larger machines the same way.  The impor-

tant, and only, network command is

SET PORT NET nodename

which is described in the section on SET commands.  Also see the SERVER command

description, and (if you're interested) section 5.16.1 for a technical descrip-

tion.

Kermit  can  even  communicate with some other computers, such as Unix systems,

which accept logins via this remote pathway.  The initial startup is  the  same

as  calling a mainframe and logging in except the command SET PORT NET nodename

is used instead of SET PORT COM1.  A connection is established with  the  first

use  of the communications circuit, such as CONNECT, REMOTE DIR, SEND, or other

file transfer command, and terminated with the HANGUP command.

5.5. Program Setup and Invocation                                       Page 59

-------------------------------------------------------------------------------

5.6. Kermit-MS Commands

MS-DOS Kermit supplies most of the commands and  features  of  "ideal"  Kermit.

Here's a summary:

-F  specify alternate init file name on DOS command line.

BYE  to remote server, exit from MS-Kermit.

CLEAR  serial port buffer.

CLOSE  log files and stop logging remote session.

COMMENT  For including comments in command files.

CONNECT  as terminal to remote system.

CWD  change local working directory.

DEFINE  a macro of Kermit-MS commands.

DELETE  local files.

DIRECTORY  listing of local files.

DISABLE  server recognition of selected commands.

DO  a command macro.

ECHO  a line of text on the screen.

ENABLE  server recognition of selected commands.

EXIT  from Kermit-MS.

FINISH  Shut down remote server.

GET  remote files from server.

HANGUP  the phone or network connection.

HELP  about Kermit-MS.

INPUT  specified string from serial port, for scripts.

LOG  remote terminal session and/or packets.

LOGOUT  remote server, don't exit from Kermit-MS.

OUTPUT  string out serial port, for scripts.

PAUSE  between commands.

PUSH  to MS-DOS command level.

QUIT  from Kermit-MS (same as EXIT).

RECEIVE  files from remote Kermit.

REMOTE  Prefix for remote file management commands.

RUN  an MS-DOS program or command.

SEND  files to remote Kermit.

SERVER  mode of remote operation.

SET  various parameters.

SHOW  various parameters.

SPACE  inquiry (about disk space).

STATUS  inquiry (about settings).

STAY  stay within Kermit after DOS command line invocation.

TAKE  commands from a file.

TRANSMIT  a file "raw" (no error checking).

TYPE  a local file on the screen.

VERSION  display Kermit-MS program version number.

Not  all of these commands are necessarily available on all MS-DOS systems, and

some of the commands may work somewhat differently between DOS versions.

A command keyword, such as SEND, RECEIVE, HELP, etc,  may  be  abbreviated,  so

long  as  you  have  typed enough letters to distinguish it from other keywords

that are valid in that position.  For instance, you can type CLE for CLEAR  and

CLO  for CLOSE.  Several common commands also have special non-unique abbrevia-

tions, like C for CONNECT, S for SEND, and R for RECEIVE.

5.6. Kermit-MS Commands                                                 Page 60

-------------------------------------------------------------------------------

During interactive operation, you may edit the command you're currently  typing

using  BACKSPACE  to  erase the character most recently typed, Ctrl-W to delete

the most recent field, or Ctrl-U to delete the entire  command.    The  editing

characters  may be used in any combination until the command is finally entered

by typing RETURN (Carriage Return, Enter) or Ctrl-L.

You may use the help ("?") and keyword completion  (ESC) features freely  while

typing Kermit-MS commands.  A question mark typed at almost any point in a com-

mand produces a brief description, or "menu", of what is expected  or  possible

at  that point.  ESC typed at any point, except in a local filename, will cause

the current field to be filled out if what you have typed so far is  sufficient

to  identify  it,  and will leave you in position to type the next field (or to

type a "?" to find out what the next field is);  otherwise,  the  program  will

beep at you and wait for you to type more characters.

Kermit-MS  recognizes  only 7-bit ASCII characters when examining a Kermit com-

mand line. The SET KEY and SHOW KEY commands can prompt for keyboard input  and

understand 8-bit characters but only at their interactive prompt.  The SET KEY,

INPUT, and OUTPUT commands accept "backslash number format"  on the main Kermit

command  line.  Thus, national characters which are full 8-bit codes can be ex-

pressed on command lines in backslash number form (\ddd), provided  the  Kermit

command  itself  can  understand the form.  Presently, INPUT, OUTPUT, ECHO, SET

KEY, SET PROMPT, and DEFINE  commands  understand  this  notation.    To  enter

characters  in  backslash  number  format, type a backslash ("\") followed by a

number corresponding to the ASCII code for the character:

\123    a decimal number (decimal is the default number base)

\d249   a decimal number (also \D)

\o177   an octal (base 8) number (also \O)

\x0d    a hexadecimal (base 16) number (also \X)

Table 5-1 shows all of the 7-bit ASCII codes in decimal.  Most Kermit  commands

understand  backslash-ASCII  codes, both imbedded within character strings, and

alone, as when a single character or number is to be specified.

Some Kermit-MS commands like GET, SHOW KEY, and SET KEY,  may  prompt  for  ad-

ditional  information  on  subsequent  lines.  If you have reached one of these

prompts and then wish to cancel the command, you may type Control-C to get back

to the main Kermit-MS> prompt.

Summary of Kermit-MS command editing characters:

SPACE   Separates fields within the command.

TAB     Same  as  Space,  and echoes as Space.  You may also use Ctrl-I for

Tab.

BACKSPACE

Deletes the character most recently typed.  May be typed repeatedly

to delete all the way back to the prompt.  You may also use DELETE,

RUBOUT, Ctrl-H, or equivalent keys.

5.6. Kermit-MS Commands                                                 Page 61

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Dec Name Ctrl            Dec Char      Dec Char     Dec Char

0   NUL  ^@           |  32  SP    |   64   @   |   96  `

1   SOH  ^A           |  33   !    |   65   A   |   97  a

2   STX  ^B           |  34   "    |   66   B   |   98  b

3   ETX  ^C           |  35   #    |   67   C   |   99  c

4   EOT  ^D           |  36   $    |   68   D   |  100  d

5   ENQ  ^E           |  37   %    |   69   E   |  101  e

6   ACK  ^F           |  38   &    |   70   F   |  102  f

7   BEL  ^G beep      |  39   '    |   71   G   |  103  g

8   BS   ^H backspace |  40   (    |   72   H   |  104  h

9   HT   ^I tab       |  41   )    |   73   I   |  105  i

10   LF   ^J linefeed  |  42   *    |   74   J   |  106  j

11   VT   ^K           |  43   +    |   75   K   |  107  k

12   FF   ^L formfeed  |  44   ,    |   76   L   |  108  l

13   CR   ^M return    |  45   -    |   77   M   |  109  m

14   SO   ^N shift out |  46   .    |   78   N   |  110  n

15   SI   ^O shift in  |  47   /    |   79   O   |  111  o

16   DLE  ^P           |  48   0    |   80   P   |  112  p

17   DC1  ^Q XON       |  49   1    |   81   Q   |  113  q

18   DC2  ^R           |  50   2    |   82   R   |  114  r

19   DC3  ^S XOFF      |  51   3    |   83   S   |  115  s

20   DC4  ^T           |  52   4    |   84   T   |  116  t

21   NAK  ^U           |  53   5    |   85   U   |  117  u

23   ETB  ^W           |  54   6    |   86   V   |  118  v

22   SYN  ^V           |  55   7    |   87   W   |  119  w

24   CAN  ^X           |  56   8    |   88   X   |  120  x

25   EM   ^Y           |  57   9    |   89   Y   |  121  y

26   SUB  ^Z           |  58   :    |   90   Z   |  122  z

27   ESC  ^[ escape    |  59   ;    |   91   [   |  123  {

28   FS   ^\           |  60   <    |   92   \   |  124  |

29   GS   ^]           |  61   =    |   93   ]   |  125  }

30   RS   ^^           |  62   >    |   94   ^   |  126  ~

31   US   ^_           |  63   ?    |   95   _   |  127  RUBOUT,DELETE

Table 5-1:   The US ASCII Character Set (ANSI X3.4-1977)

-------------------------------------------------------------------------------

Ctrl-W  Deletes the most recent "word", or field, on the command line.  May

be typed repeatedly.

Ctrl-U  Deletes the entire command line, back to the prompt.

Ctrl-C  Cancels the current command and returns to the "Kermit-MS>" prompt.

Also, terminates execution of a TAKE command file.

ESC     If  enough  characters have been supplied in the current keyword to

identify it uniquely the remainder of the field is supplied and the

cursor  is positioned to the next field of the command.  Otherwise,

a beep is sounded.  ESC does not  provide  filename  completion  in

version 2.30.

5.6. Kermit-MS Commands                                                 Page 62

-------------------------------------------------------------------------------

?       Displays  a  brief message describing what may be typed in the cur-

rent command field.  Also,  wildcard  character  for  matching  any

single character in all but the first position of a filename.

#       Wildcard  character  for  matching  single characters in filenames.

Equivalent to MS-DOS "?", but used  in  the  first  position  of  a

filename only, so that "?" may be used to get help at the beginning

of a filename field.

RETURN  Enters the command.  On most keyboards, you may also use  ENTER  or

Ctrl-M.

Ctrl-L  Clears the screen and enters the command.

Liberal  use  of "?" allows you to feel your way through the commands and their

fields.  This feature is sometimes called "menu on  demand"  or  "context  sen-

sitive help" -- unlike systems that force you to negotiate menus at every turn,

menu-on-demand provides help only when it is needed.

Command parsing is done through DOS calls.  Kermit key  redefinition  does  not

apply  at  MS-Kermit  command  level.    But ANSI.SYS or other external console

drivers can be used for this purpose, for instance to assign ESC  to  the  PC's

backquote key (ANSI.SYS is the IBM-supplied extended screen and keyboard device

driver, described in the IBM DOS Technical Reference Manual).    Other  console

drivers available include ProKey, SuperKey, NANSI.SYS (a public-domain replace-

ment for ANSI.SYS), and FANSICONSOLE.

The notation used in command descriptions is as follows: Optional fields are in

[square  brackets],  lists  of alternatives are in {curly braces}, separated by

commas.  Parameters, such  as  numbers  or  filenames,  are  shown  in  italics

(providing  the printer is capable of printing italics), and in dialog examples

user typein is underlined (on printers that can show it) to distinguish it from

computer typeout.

The following sections describe MS-Kermit's commands.  Command descriptions may

contain references to other commands that haven't  been  explained  yet.    You

might find that this manual makes more sense on a second reading.

5.6.1. Program Management Commands

"Program  management" is a rubric for Kermit-MS commands like TAKE, EXIT, HELP,

COMMENT, ECHO, and VERSION, that don't fall into any other category.

HELP simply displays a short help message (the same  one,  in  fact,  that  you

would see if you typed a question mark in the same place).

VERSION displays the MS-Kermit program version number, which you should know in

case you are reporting bugs or seeking technical assistance.

Other program management commands require a bit more explanation.

5.6.1. Program Management Commands                                      Page 63

-------------------------------------------------------------------------------

The EXIT Command

Syntax: EXIT  or  QUIT

EXIT and QUIT are synonyms for each other.  They cause MS-Kermit to return con-

trol  to DOS or whatever program invoked MS-Kermit.  The specific actions taken

are:

- Close any open log or other files.

- Close any open network connection.

- Release all memory claimed by the program.

- Disable interrupts for the currently selected communication device.

- Terminate execution.

The serial port RS-232 signals are left alone upon EXIT, so that modem  connec-

tions  are  not broken.  Kermit-MS may be restarted with the connection intact.

Use HANGUP to explicitly break a modem connection.

The STAY Command

Syntax: STAY

The STAY command, if included among command line arguments, instructs MS-Kermit

not  to  exit upon completion but rather to enter interactive mode, unless EXIT

or QUIT was among the command arguments.  STAY has no effect when  entered  in-

teractively or from a TAKE file.

The PUSH Command

Syntax: PUSH

PUSH  is  similar  to  EXIT,  except  it leaves MS-Kermit intact by invoking an

MS-DOS command processor "under"  Kermit-MS,  either  COMMAND.COM  or  whatever

shell  you  have  specified with COMSPEC (or SHELL, depending on the system) in

your CONFIG.SYS file.  You can return to Kermit-MS by typing  the  MS-DOS  EXIT

command,  and you will find Kermit-MS as you left it, with all settings intact.

The same function is invoked by the CONNECT escape-level command P. Example:

Kermit-MS>push           Push to DOS.

Command v3.10            COMMAND.COM program herald.

C>diskcopy a: b:         Run a DOS program.

DISKCOPY dialog here...

C>dir b:                 More DOS commands...

DOS session continues...

C>exit                   When done, type DOS EXIT command.

Kermit-MS>               Back at Kermit.

5.6.1. Program Management Commands                                      Page 64

-------------------------------------------------------------------------------

The TAKE Command

Syntax: TAKE filespec

The TAKE command gives you way a to collect MS-Kermit commands  into  a  single

file,  so that you can execute many commands by typing a single (TAKE) command.

TAKE instructs MS-Kermit to execute commands from the file  that  you  specify.

The  current directory is searched for the file first, and then any directories

listed in the PATH environment variable.  The  command  file  may  include  any

valid  Kermit-MS  commands, including TAKE, but it cannot include characters to

be sent to a remote host  after  a  CONNECT  command  (use  scripts  for  that,

described  below).    Execution  of  a  TAKE  file  may  be cancelled by typing

Control-C at the keyboard.

An implicit TAKE command is executed upon the initialization file, MSKERMIT.INI

(or  another  file  specified  in the "-f" command-line argument), whenever you

start MS-Kermit.  The MSKERMIT.INI file contains any commands you  want  to  be

executed  each  time  you  run Kermit.  A sample is shown above, and a more am-

bitious example is shown in section 5.8.

Commands within TAKE files, unlike interactive commands, may  include  trailing

comments,  preceded  by semicolons (if a real semicolon is needed in a command,

express it as "\;" and it will not be mistaken for the start of a comment).

set port 2      ; Select the modem port.

set speed 1200  ; Set the baud rate for the modem.

connect         ; Conduct a terminal session.

hangup          ; Hang up the phone after escaping back.

Note the HANGUP command after CONNECT.  The HANGUP command is not executed  un-

til  after you escape back from your CONNECT session.  If this file were called

MODEM.CMD, the following TAKE command would execute it:

Kermit-MS>take modem.cmd

This directs MS-Kermit to find the MODEM.CMD file, open it,  execute  the  com-

mands  in  it,  close  it, and return to the MS-Kermit> prompt when done.  This

process can take a while on floppy-disk based systems.

Commands from the TAKE file will normally  not  be  displayed  on  your  screen

during  execution.   If you want to see them as they are executing, you can SET

TAKE-ECHO ON (for instance, at the beginning or end of your MSKERMIT.INI file).

With  the  echoing ON, comments are also displayed for reference, but the semi-

colon is not shown.

TAKE files may be nested to any reasonable level.  A command file that was  in-

voked by another command file returns to its invoking command file, rather than

to the MS-Kermit> prompt.

There is currently no parameter substitution mechanism for TAKE files.

Warnings:

- An explicit query mark ("?") in a TAKE file will cause a help message

to be displayed and the rest of the line will be read as another com-

mand.

5.6.1. Program Management Commands                                      Page 65

-------------------------------------------------------------------------------

- Since TAKE file processing discards all characters from a line begin-

ning with the first semicolon, it is normally not possible to include

semicolons as part of the commands themselves, e.g.

get dska:foo.bar;6

To get around this restriction, you may precede such semicolons  with

a backslash:

get dska:foo.bar\;6

The -F Command

Syntax: -F filespec

The  "-f"  command  is  effective  only  on the DOS command line.  It instructs

MS-Kermit to use the specified file as its  initialization  file,  rather  than

MSKERMIT.INI.    Unlike other command-line arguments, "-f" does not, of itself,

cause MS-Kermit to exit upon completion.  Example:

C>kermit -f sunday.ini

Kermit-MS>

The -F command line option allows different MS-Kermit initialization  files  to

coexist.  You can create batch commands to invoke Kermit in different ways, for

instance MONDAY.BAT might contain "kermit -f monday.ini",  TUESDAY.BAT  "kermit

-f tuesday.ini", etc.

The ECHO Command

Syntax: ECHO [string]

The  ECHO  command  writes  the string to the screen, without adding a carriage

return or line feed.  ECHO may be used to report progress during execution of a

TAKE command file, or to issue prompts during the execution of a script.

ECHO Part one completed...\13\10

The  numbers  at the end are "backslash codes" for ASCII control characters, in

this case carriage return (\13) and linefeed (\10).  Since the ECHO command in-

terprets   backslash  codes,  ANSI.SYS  and  similar  console  drivers  can  be

programmed through this command by embedding ANSI escape sequences (see section

5.15.3) in the echo string.

The COMMENT Command

Syntax: COMMENT text

The  COMMENT  command  lets  you add comments to a TAKE command file.  The word

COMMENT (or any unique prefix thereof) must appear as the  first  word  on  the

line.  The COMMENT command may also be entered interactively.  It has no effect

at all.  Comments may also be entered in TAKE files,  but  not  on  interactive

command lines, by using the semicolon (";") character.  Unlike the COMMENT com-

5.6.1. Program Management Commands                                      Page 66

-------------------------------------------------------------------------------

mand, semicolon comments may appear on the same line as a Kermit command.   Ex-

ample:

COMMENT - MS-Kermit command file to connect port 2 to an IBM mainframe

set port 2

set speed 4800   ; Baud rate is 4800

do ibm           ; Set parameters for IBM linemode

connect          ; Be a terminal

Question marks should not be included in comments, as they will invoke the help

function.

The DEFINE Command

Syntax: DEFINE macro-name [command [, command [, ...]]]

Command macros provide another way of collecting many commands  into  a  single

command.    The difference between a macro and a TAKE file is that Kermit keeps

all its macro definitions in memory, and can execute them as many times as  you

like,  without  having  to look them up on disk, whereas every time you issue a

TAKE command, Kermit has to access a disk.  But... you can have  as  many  TAKE

command  files  as  you  like,  and  they  can  be as long as you want, whereas

MS-Kermit's memory for storing macro definitions is limited.  You can put macro

definitions and DO commands for them in TAKE files (or for that matter, you can

put TAKE commands in macro definitions).

Kermit-MS command  macros  are  constructed  with  the  DEFINE  command.    Any

Kermit-MS commands may be included.  Example:

define telenet set parity mark, set baud 1200, connect

A macro can be undefined by typing an empty DEFINE command for it, like

define telenet

A  macro  definition  may be no longer than 128 characters.  Longer definitions

can be accomplished by "chaining."  Example:

define setup set port 1, set speed 19200, set par even, do setup2

define setup2 set flo no, set handsh xon, set local on, do setup3

define setup3 set timer on, set terminal color 1 31 45, do setup4

define setup4 set warning on, set incomplete keep, connect

DO SETUP will invoke all of these commands.  The commas  above  stand  for  the

carriage  returns  needed  by  individual  commands.    Commas  must be used to

separate commands in macro definitions; carriage returns (\13) cannot be  used.

When  control or other special characters are needed in a macro they may be ex-

pressed in backslash number form, \nnn.

The SHOW MACROS command displays the value of all currently defined macros, and

tells how much space is left for further definitions.

5.6.1. Program Management Commands                                      Page 67

-------------------------------------------------------------------------------

The DO Command

Syntax: DO macro-name

A  Kermit-MS  command  macro  is  invoked  using the DO command.  For instance,

Kermit-MS comes with a predefined macro  to  allow  convenient  setup  for  IBM

mainframe  line-mode  communications; to invoke it, you would type DO IBM.  The

IBM macro is defined as "set timer on, set  local-echo  on,  set  parity  mark,

handshake xon, set flow none".  You can use the DEFINE command to redefine this

macro or remove the definition altogether.

There is no automatic way to undo the effect of a macro.  If you  need  to  ac-

complish  this  effect,  you should define another macro for that purpose.  For

instance, to undo the effect of "do ibm" so that you could connect to,  say,  a

DEC VAX, you could:

def vax set par none,set hand none,set flo x,set tim off,set loc off

Then  you  can  "do  ibm" whenever you want to use the IBM system, and "do vax"

whenever you want to use the VAX.

If you wish to view the macro expansion whenever you issue a  DO  command,  you

can SET TAKE-ECHO ON.

Local File Management Commands

These  commands  are  executed  on your local PC, and generally invoke DOS ser-

vices.  This allows you to perform common DOS functions without leaving Kermit.

All  file specifications may include device and/or directory fields.  The local

file management commands are:

CWD path

Changes  the  current  working  directory  to  the  given  path.    All

references to local file names without explicit  paths  will  refer  to

that  path.  A drive letter may be included to also change disk drives.

This command affects Kermit and any inferior programs that you  RUN  or

PUSH  to,  but  your  previous disk and directory are restored when you

exit from Kermit.

DELETE filespec

Deletes  the  specified  file  or  files.   As in DOS, the names of the

deleted files are not listed, only the  message  "file(s)  deleted"  or

"file(s)  not  found",  and  if  you  give  the  command  "delete *.*",

Kermit-MS will prompt "Are you sure?" since DOS is doing the work.

DIRECTORY [filespec]

Lists  the  names,  sizes,  and  creation dates of files that match the

given file specification.  If no filespec  is  given,  the  command  is

equivalent to DIR *.*.  Normal DOS switches are effective.

SPACE   Performs  the MS-DOS CHKDSK function by running the CHKDSK program from

the current path.  CHKDSK summarizes your disk and memory usage.

5.6.1. Program Management Commands                                      Page 68

-------------------------------------------------------------------------------

RUN command

Passes  the  command  line to COMMAND.COM for execution.  Any legal DOS

operation is permitted: running a program (perhaps  with  command  line

arguments  or i/o redirection), executing a DOS command, or executing a

batch file.  The COMMAND.COM file should be in the current path.   Ker-

mit  is  suspended  while  the  command  is  executed and automatically

resumes afterward.  You may even nest RUN KERMIT commands several times

if  memory  is  available.    The  command will be executed directly by

COMMAND.COM so follow the rules of DOS.  Example:

Kermit-MS>run more < kim.txt

TYPE filespec

Displays  the  specified  local file on the screen.  Automatic pause is

not available at the end of a page (but see above example  for  how  to

accomplish this).  On most systems, Ctrl-S can be typed to stop scroll-

ing and Ctrl-Q to continue scrolling.

5.6.1. Program Management Commands                                      Page 69

-------------------------------------------------------------------------------

5.6.2. COMMANDS FOR TERMINAL CONNECTION

The CONNECT command connects your PC as a terminal to the remote system so that

you  may conduct a session there, and the HANGUP command may be used to discon-

nect your modem (if you have one) from the remote system.  There  is  presently

no  DIAL  command;  modems  may be dialed "manually" during CONNECT or by using

scripts.  Scripts are described in detail in subsequent sections.

For completeness, the descriptions below contain copious reference to  the  SET

commands,  which  let  you  modify  all  sorts  of  terminal  and communication

parameters (the SET commands are described in a later section).   MS-Kermit  is

initially  set up with the following parameters, so that you only need to issue

SET commands for those that need to be changed:

PORT                    1 (in most cases, e.g. COM1 on the IBM PC family)

TERMINAL                VT102(*) emulation (on the IBM PC  family  and  DEC

Rainbow)

SPEED                   Whatever the serial card is currently set to.

PARITY                  None

FLOW-CONTROL            XON/XOFF

HANDSHAKE               None

LOCAL-ECHO              Off

DISPLAY                 7-bit characters

INPUT TRANSLATION       Off

ESCAPE                  Control-Rightbracket

(*)  The  VT102  terminal  is compatible with the VT100, but includes a few ad-

ditional functions.

The CONNECT Command

Syntax: CONNECT -or- C

The CONNECT command establishes  an  interactive  terminal  connection  to  the

remote  system  using the currently selected communications port (SET PORT COM1

or COM2) with all settings currently in effect for  that  port,  emulating  the

currently selected type of terminal.

During  CONNECT,  the  characters you type are sent out the communication port,

and the characters that arrive at the port are displayed on the screen  or  in-

terpreted  by  the  selected  terminal  emulator.    If  you SET LOCAL-ECHO ON,

MS-Kermit will also display the characters you type on the screen.

Before you issue the CONNECT command, be sure to set the correct  communication

speed  (SET  SPEED)  and any other necessary communication parameters (e.g. SET

PARITY, SET LOCAL-ECHO).  If you have SET DEBUG ON, then (on  certain  systems,

particularly the IBM PC), terminal emulation will be disabled and received con-

trol characters will be displayed in special notation.

By default, 7-bit ASCII characters are displayed on the screen.    If  you  SET

DISPLAY  8, then 8-bit characters will be used (useful for "national" character

sets).  Character translation will be done according to any SET TRANSLATION IN-

PUT  and  SET  KEY  commands you have issued.  In addition, characters that are

sent to the screen will also be recorded in a disk file or on a printer if  you

have issued a LOG SESSION command.

5.6.2. COMMANDS FOR TERMINAL CONNECTION                                 Page 70

-------------------------------------------------------------------------------

To  get back to the PC, type the escape character followed by the letter C (for

"Close connection").  On most MS-DOS systems the  escape  character  is  Ctrl-]

(Control-Rightbracket).    That  means,  hold down the Ctrl key, press "]", and

then type the letter C.

Kermit-MS>connect        Connect to remote system.

Conduct terminal session here...

^]c                      Escape back to PC.

Kermit-MS>               Prompt reappears.

This is called "escaping back".  You can use the SET ESCAPE command  to  change

the escape character to something besides "^]", or you can assign the escaping-

back operation to a single key or key combination with SET KEY (on the  IBM  PC

the default for this is Alt-X).

You can include the CONNECT command in a TAKE command file, but not "bare" text

to be sent to the remote system during CONNECT (use scripts for that, see  Sec-

tion  5.7).    When a TAKE file includes a CONNECT command, no further commands

will be executed from the file until after you escape back.

When you CONNECT, the program attempts to raise the DTR and RTS RS-232 signals,

and  it  takes no specific action to lower them unless you explicitly issue the

HANGUP command; thus you can EXIT from Kermit-MS and restart it  without  drop-

ping  a  dialup connection.  While CONNECTed, you can communicate directly with

an autodialer or "smart modem" to control the communications line, hang it  up,

and the like, for instance, by typing AT commands to a Hayes-like modem.

Kermit-MS>set speed 2400 (See Section 5.6.9)

Kermit-MS>connect

AT                       Now you're talking to the modem.

OK                       Your modem responds

ATDT8765432              Type the modem's dialing command.

RINGING

CONNECT 2400

Welcome to ...           Now you're talking to the host computer.

Please login:

MS-Kermit  makes  no attempt to monitor the modem's Carrier Detect (CD) or Data

Set Ready (DSR) signals, and will take no notice if they drop.  Thus it is  not

possible  to  automatically  terminate  a  session if the connection is broken.

However, you may query  the  status  of  these  modem  signals  yourself  using

Kermit's SHOW MODEM command.

When  using Kermit to connect two PCs "back to back," SET LOCAL-ECHO ON so that

when you CONNECT to the other PC to send messages to its operator, you can  see

what  you are typing.  Depending on the system, you may have to type a carriage

return and a linefeed (Control-J) at the end of each line in order to make  the

display look right.

5.6.2. COMMANDS FOR TERMINAL CONNECTION                                 Page 71

-------------------------------------------------------------------------------

The HANGUP Command

On  serial  port  connections, the HANGUP command attempts to momentarily lower

the modem signals DTR and RTS.  It may be used to hang up the phone when dialed

up  through  a  modem, or to get the attention of port contention units or ter-

minal concentrators that operate in this manner.   On  direct  connections,  it

will  probably  have no effect.  On local area network connections, the network

session is fully terminated.

TERMINAL EMULATION

The IBM PC version of Kermit-MS emulates the DEC VT102 terminal by default, and

may  also  be  instructed  to  emulate  the  DEC VT52, the Heath/Zenith-19, the

Tektronix 4010 graphics terminal, or no terminal at all,  selectable  with  the

SET TERMINAL command.  Emulation of each of these terminals is nearly complete.

VT102 emulation lacks only smooth scroll and 132 column mode (some  EGA  boards

allow it).  Double-height, double-width characters are supported, but simulated

using ordinary characters.

The IBM PC's 40-column (large character) screen mode may be used during CONNECT

if  you SET TERMINAL NONE (but you may also have to inform the remote host that

your screen width is 40).  This can provide improved  readability  to  visually

impaired  persons,  but at the expense of terminal emulation.  To use 40-column

mode, enter the DOS command "MODE 40" (or CO40 or BW40).

On color monitors, the foreground and background colors may be  set  using  SET

TERMINAL  COLOR,  and  inverse/normal video display may also be selected, along

with many other terminal parameters.  A complete list of the commands,  default

key configurations, and escape sequences accepted by the IBM PC Kermit terminal

emulator is given in section 5.15.1.   Non-IBM-compatible  PCs  have  different

terminal emulation options.  See section 5.9.

Escape-Level Commands

The  escape  character,  normally Control-], is used to regain the attention of

Kermit-MS during CONNECT  (you  can  change  the  escape  character  using  SET

ESCAPE).  When you type the escape character, Kermit-MS waits for you to follow

it with a single character command.  For instance, the single character command

"?"  produces  a  list of available single character commands.  This command is

executed immediately; it may not be edited, and the program does not wait for a

carriage  return  to confirm it.  Table 5-2 shows CONNECT escape-level commands

available in Kermit-MS.  Typing any other  character  (except  the  space  bar,

which is the "null command") after the escape character will cause Kermit-MS to

beep, but will do no harm.  These actions are also Kermit action verbs and  can

be assigned to single keys.  See SET KEY for details.

The Mode Line

When  you  first  issue the CONNECT command, a message (on some systems, an in-

verse video "mode line") will display the most important facts about  the  con-

nection you've just established, so that you can quickly diagnose any problems.

Here's what the IBM PC mode line looks like:

5.6.2. COMMANDS FOR TERMINAL CONNECTION                                 Page 72

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

?   Help -- Lists the available single-character commands.

0   (the digit zero) Transmit a NUL (ASCII 0).

B   Transmit a BREAK signal.

L   Transmit a Long BREAK signal (on some systems).

C   Close the connection and return to Kermit-MS prompt level.

H   Hangup the phone by lowering DTR and CTS momentarily.

F   File the current screen in the screen dump file.

M   Toggle the mode line, i.e. turn it off if it is on or vice versa.

P   Push to DOS; get back to CONNECT by typing EXIT.

Q   Temporarily quit logging the remote session.

R   Resume logging the remote session.

S   Show the status of the connection.

^]  (or whatever you have set the escape character to be)

Typing the escape character twice sends one copy of it to the connected

host.

Table 5-2:   Kermit-MS Single-Character CONNECT Escape Commands

-------------------------------------------------------------------------------

Esc-chr:^] help:^]? port:1 speed:9600 parity:odd echo:rem VT102 .... PRN

This shows that the escape character is Ctrl-Rightbracket, that you would  type

Ctrl-rightbracket followed by question mark ("^]?") to get help during CONNECT,

that you are connected on port 1 at 9600 baud with odd parity and remote  echo,

and  that  a  VT102  terminal  is  being emulated.  The four dots represent the

VT102s LEDs (they turn into the digits 1,2,3,4 when "lit") and PRN will show up

if the printer is activated (e.g. by Ctrl-PrintScreen).

The  mode  line  may be turned on and off using SET MODE, or the CONNECT escape

character followed by the letter M.

Screen Rollback

On the IBM PC and some  other  systems  (see  Table  5-5),  Kermit-MS  provides

several  pages  of screen memory which let you recall earlier terminal screens.

These may be scrolled up and down using keys as shown in Table 5-6.    For  in-

stance,  the  IBM PC uses PgUp (previous screen), PgDn (next screen), Ctrl-PgUp

and Ctrl-PgDn (one line at a time).  Only lines that scroll off the top of  the

screen  are  saved.   When an application clears the screen, that screen is not

saved.  These functions and others may be assigned to different keys  with  the

SET KEY command.

If  you  have  rolled the screen back and a new character must be displayed, it

will normally appear at the current cursor position on the old screen.  This is

useful  when  you  are trying to copy something from a previous screen.  If you

wish new characters to appear in their proper place on the "newest" screen, you

can SET TERMINAL ROLL ON.

5.6.2. COMMANDS FOR TERMINAL CONNECTION                                 Page 73

-------------------------------------------------------------------------------

Screen Dump

The  screen  dump  feature  writes the contents of the current screen to a file

(KERMIT.SCN unless another file was selected by the SET DUMP command) when  the

CONNECT  escape-level  command F is typed.  The screen dump file is appended to

on each successive screen dump,  with  each  screen  separated  by  a  formfeed

(Ctrl-L).    This  feature may be used in conjunction with screen rollback -- a

handy way to recapture screenfuls of laboriously typed-in text after  a  remote

host  has  crashed  without saving your work.  The corresponding action verb is

"dump".  Screen dump does not function when in Tektronix graphics mode; instead

one of many graphics screen capture programs may be used independently commonly

via the DOS Shift PrtSc key combination or by LOGging the incoming byte stream.

A screen dump differs from a session log in two  ways.    First,  each  desired

screen  must  be  manually  filed,  and  second,  the screen dump file has been

stripped of any escape sequences, whereas the session log records them (see LOG

SESSION).

Printer Control

During  terminal emulation, a locally attached printer may be controlled in the

normal manner, on most systems.  Pushing the "Print  Screen"  key  (shifted  on

some  systems)  will  cause the current contents of the screen to be printed by

DOS; holding down Ctrl while depressing Print Screen will alternately start and

stop  the  spooling  of incoming characters to the printer.  On the IBM PC, the

mode line will show PRN when the printer is activated in this manner.  ^P or ^N

are  sent  to  the host during terminal emulation and do not toggle printing as

they do when you're talking directly to DOS.  CTRL-Print-Screen  can  be  simu-

lated  with  the  Kermit-MS  LOG  PRN  and  CLOSE commands.  VT102 (ANSI) style

host-controlled transparent printing is also supported on the IBM PC.  See sec-

tion 5.16.6 for technical information about MS-Kermit's printer control.

Graphics

MS-Kermit  on the IBM PC, compatibles, and several other systems, is capable of

emulating a Tektronix 4010 graphics terminal, for use with host-based  software

that can generate Tektronix control codes.  When you enter Tektronix emulation,

your cursor will disappear.  Don't be alarmed, this is how Tektronix  terminals

behave.

The  Tektronix  emulator  implements a mixture of Tek 4010 and 4014 features to

draw characters, lines, and dots in graphics mode.  These  Tektronix  terminals

have a graphics display 780 dots high by 1024 dots wide.  They use storage tube

technology whereby a dot stays illuminated until the  full  screen  is  erased.

They  also  lack  cursor  keys.  Kermit's Tek emulator maps the 1024 by 780 dot

display to the PC's current screen dimensions, say 640 across  by  200  or  350

dots high, and retains limited use of the cursor keys.  It automatically senses

the active  display  adapter  (EGA,  CGA,  Hercules,  Mono,  and  AT&T/Olivetti

640x400)  and retains screen coloring (EGA) and the current graphics image (EGA

and Hercules) if the adapter has sufficient memory.  Pure  monochrome  systems,

of  course,  lack  a  graphics capability; in this case Kermit approximates the

graphic image by writing dots as plus signs.

Tektronix graphics mode is entered two different ways, automatically and volun-

5.6.2. COMMANDS FOR TERMINAL CONNECTION                                 Page 74

-------------------------------------------------------------------------------

tarily:

1. Automatically  (which you can prevent via the Kermit command DISABLE

TEK).  While emulating a VT102, VT52, or Heath-19, reception of  the

byte pair ESCAPE Control-L causes the PC to change to graphics mode,

clear the screen, and obey new  input  as  Tektronix  commands.    A

second   automatic   entry  is  reception  of  the  escape  sequence

"ESC [ ? 3 8 h" which does the same as above except  the  screen  is

not  cleared.    Automatic  mode  is  exited  by either reception of

Control-X or "ESC [ ? 3 8 l" (lower case L), or by toggling the ter-

minal type (ALT minus, Kermit verb\KTermtype) to VT102, or something

other than TEK.  (These "ESC [ ? 3 8 h/l" sequences derive from  the

DEC VT340 terminal.)

2. Voluntary mode is when terminal type TEK4010 is selected by the Ker-

mit command SET TERMINAL TEK4010 or by toggling to it as above.   It

is  exited  by  SET  TERMINAL another-kind or by toggling to another

kind.  ENABLE or DISABLE TEK and the exit-Tek-mode escape  sequences

are not applicable to voluntary mode.

Here are several common questions about Tek mode, and their answers:

1. "How  do  I  escape  from  graphics  mode  back  to  being a regular

terminal?"  Within CONNECT mode, you can type  the  \KTermtype  key,

which  is  assigned  by  default to Alt-Minus.  Repeated pressing of

this key "toggles"  among  Kermit's  terminal  types,  VT102,  VT52,

Heath-19, and Tektronix.  You can also escape back to Kermit-MS com-

mand level and issue an explicit SET TERMINAL command to change  the

terminal type.

2. "How  can  I return to the graphics screen without erasing it?"  The

graphics screen is preserved if your graphics adapter has sufficient

memory  (see  Table 5-3).  In this case, both your text and graphics

screens will be preserved when you toggle back and forth  between  a

character terminal (e.g. VT102) and Tektronix.

3. "How do I erase the graphics screen?"  You can type the \KReset key,

which is normally assigned to Alt-=.  The screen also clears if  the

host sends a Control-L or ESC Control-L.

While acting as a Tek terminal Kermit uses the keyboard translation appropriate

to the VT102 terminal.  However, received escape sequences are  interpreted  by

the  Tek emulator and VT102 escape codes are inoperative.  The Tek emulator ab-

sorbs the ESCAPE and following character  and  treats  any  additional  unknown

items as ordinary text.

The  emulator  can  display text characters from a built-in 8-by-8 dot font for

characters Space through DELete (no control codes nor special characters).    A

backspace  operation  erases  the character under the final cursor location (an

extension beyond a real Tektronix 4010).  Tabs are converted to single  spaces.

Only the low 7 bits of the character are used.

While in Tek mode the emulator behaves as a simple TTY device for ordinary text

and as a line or dot drawing Tektronix device for commands listed in Table 5-8.

The  screen  resolution  is  governed by the kind of active display adapter and

monitor in the PC (Table 5-3).  Kermit senses this automatically when  graphics

5.6.2. COMMANDS FOR TERMINAL CONNECTION                                 Page 75

-------------------------------------------------------------------------------

mode  is  entered.  Graphics are saved on page 1 of screen memory.  Coloring is

determined by the current terminal status, either the default  screen  or  that

overridden by the command SET TERMINAL COLOR.

-------------------------------------------------------------------------------

Display Adapter               Display Mode

EGA w/256KB    Hi res color   16 dec  640x350, graphics saved, 16 colors.

Med res color  14      640x200, graphics saved, 8 colors.

Monochrome     15      640x350, graphics saved, b/w.

EGA w/64KB     Hi res color   16      640x350, graphics not saved,

4 colors of red, white, blue, black.

Med res color  14      640x200, graphics saved, 8 colors.

Monochrome     15      640x350, graphics not saved.

CGA            Color          6       640x200, graphics not saved, b/w.

Hercules       Monochrome     none    720x348, graphics saved if memory.

Monochrome     Monochrome     7       80 by 25 text, graphics not saved.

AT&T/Olivetti  any            72      640x400, grahics not saved, b/w.

Table 5-3:   Adapters Supported by IBM PC MS-Kermit for Tektronix Emulation

-------------------------------------------------------------------------------

The technical details of Tektronix emulation are presented in section 5.15.7.

5.6.2. COMMANDS FOR TERMINAL CONNECTION                                 Page 76

-------------------------------------------------------------------------------

5.6.3. COMMANDS FOR FILE TRANSFER

MS-Kermit's SEND, GET, and RECEIVE invoke the Kermit file transfer protocol for

error-checked transmission  of  files  between  MS-Kermit  and  another  Kermit

program  on the other end of the connection.  There are also commands for "raw"

transfer of files (no error checking)  with  systems  that  don't  have  Kermit

programs:  LOG  SESSION (for capturing text files on your PC) and TRANSMIT (for

uploading text files to the remote system).

During file transfer, MS-Kermit normally displays its progress on the screen as

shown  in  Figure  5-1.  The items in the right-hand column are updated more or

less at random.  In the current version, the percent done  is  filled  in  only

when sending files, not when receiving.  Several other display options are also

available; see SET DISPLAY.

-------------------------------------------------------------------------------

Kermit-MS: V2.30  8 Jan 88

File name: FOT.

KBytes transferred: 7

Percent transferred: 52%

Sending: In progress

Number of packets: 74

Packet length: 93

Number of retries: 2

Last error: None

Last warning: None

Figure 5-1:   MS-Kermit File Transfer Display Screen

-------------------------------------------------------------------------------

Although MS-Kermit makes no distinction between text  and  binary  files,  most

other  Kermit  programs  do.   Therefore, before you attempt to transfer binary

files with another type of system (say, a VAX, or an IBM mainframe), be sure to

give  the appropriate command -- usually something like SET FILE TYPE BINARY --

to the Kermit on the remote end.

File transfers involving floppy disks will be slow and noisy.  Hard  disks  are

much  faster  (and  quieter),  and RAM disks faster still (and totally silent).

But if you store files on a RAM disk, be sure to move them to a real  disk  be-

fore  turning  off your PC.  And before attempting to transfer files to the PC,

make sure you have enough room on the selected device.  Kermit does not provide

a way for you to change disks during a file transfer.

The SEND Command

Syntax:  SEND filespec1 [filespec2]

The  SEND  command causes a file or file group to be sent from the local MS-DOS

system to the Kermit on the remote system.  The remote Kermit may be running in

5.6.3. COMMANDS FOR FILE TRANSFER                                       Page 77

-------------------------------------------------------------------------------

either  server or interactive mode; in the latter case, you should already have

given it a RECEIVE command and escaped back  to  your  PC.    S  is  a  special

non-unique abbreviation for SEND.

filespec1  may  contain  the    wildcard  characters  "*" to match zero or more

characters within a field, and/or "#" (first position) or  "?"  (elsewhere)  to

match any single character.  If filespec1 contains wildcard characters then all

matching files will be sent, in the same order that MS-DOS would show them in a

directory  listing.    If  filespec1  specifies  a  single file, you may direct

Kermit-MS to send that file with a different name, given in filespec2.  For in-

stance, in the command

Kermit-MS>send foo.bar framus.widget

filespec2  begins  with  the  first nonblank character after filespec1 and ends

with the carriage return; thus it may contain blanks or other  unusual  charac-

ters  that  may  be  appropriate on the target machine.  The alphabetic case of

text in filespec2 is preserved in transmission, so if case matters on the  tar-

get system, be sure to type filespec2 appropriately.

If  the  SEND command is specified by itself on the command line, then you will

be prompted separately for the name of the file to send, and the name  to  send

it under:

Kermit-MS>send

Local Source File: c:\chris\xcom1.txt

Remote Destination File: com1.txt

If  a file can't be opened for read access, standard MS-DOS recovery procedures

will take place.  For example:

Not ready error reading drive A

Abort, Retry, Ignore?

Kermit remains active even if you select "Abort" (DOS's word, not ours).

Files will be sent with  their  MS-DOS  filename  and  filetype  (for  instance

FOO.TXT,  no  device or pathname).  Special characters in the file name are not

converted.  If there is no filetype, then only the name will be  sent,  without

the  terminating dot.  Each file is sent as is, with no conversions done on the

data, except for possibly stopping at a terminating  Control-Z  character  (see

the SET EOF command).

Once  you  give  Kermit-MS the SEND command, the name of each file will be dis-

played on your screen as the transfer begins.  Packet, retry, and other  counts

will be displayed along with informational messages during the transfer, in the

style specified by SET DISPLAY.  If the file is successfully  transferred,  you

will  see  "Complete",  otherwise  there  will  be  an error message.  When the

specified operation is done, the program will sound a beep.

Several single-character commands may be given while  a  file  transfer  is  in

progress:

^X  (Control-X)  Stop  sending the current file and go on to the next one, if

any.

5.6.3. COMMANDS FOR FILE TRANSFER                                       Page 78

-------------------------------------------------------------------------------

^Z  Stop sending this file, and don't send any further files.

^C  Return to Kermit-MS command level immediately without sending any kind of

notification to the remote system.  (^Z or even ^E is preferable.)

^E  Like  ^C,  but send an Error packet to the remote Kermit in an attempt to

bring it back to server or interactive command level.

CR  Simulate a timeout: resend the current packet, or NAK the expected one.

Control-X, Control-Z, and Control-E send the proper protocol  messages  to  the

remote  Kermit  to  bring it gracefully to the desired state.  Control-C leaves

the remote Kermit in whatever state it happens to be in,  possibly  retransmit-

ting  its  last  packet  over and over, up to its retry limit.  You should only

have to use Control-C in dire emergencies (the  remote  Kermit  is  stuck,  the

remote  system  crashed, etc), or at those times when you realize that you have

given a file transfer command to Kermit-MS without first having told the remote

Kermit about it.

The RECEIVE Command

Syntax: RECEIVE [filespec]

The  RECEIVE  command  tells Kermit-MS to receive a file or file group from the

other system.  The file is stored under the name it was transmitted  with,  ex-

cept  that  any  illegal characters are translated to X's.  Kermit-MS passively

waits for the file to arrive; this command is not to be used when talking to  a

Kermit  server  (use GET for that).  You should already have issued a SEND com-

mand to the remote Kermit and escaped back  to  Kermit-MS  before  issuing  the

RECEIVE command.  R is a special non-unique abbreviation for RECEIVE.

If  the optional filespec is provided, incoming files will be stored under that

name.  If the filespec is really just a path then files are  stored  where  the

path indicates.  If it is an actual filename the first incoming file is renamed

and any additional files either overwrite the first (if FILE WARNING is OFF) or

are renamed slightly from the filespec (digits are added to the end of the main

filename part before the  dot  and  extension)  if  FILE  WARNING  is  ON  (the

default). The filespec may include any combination of the following fields:

Device designator

Store the file on the designated device, in  the  current  directory  for

that  device.   If no device designator is given, store it on the current

default device.

Directory path

Store  the file in the designated directory.  If no path given, store the

file in the current directory.

File name

Store the file under the name given.  If no name is given, store it under

the name it was sent under, converted, if necessary, to suit DOS  conven-

tions,  and modified, if SET WARNING ON, to avoid overwriting any file of

the same name in the same directory.

If an incoming file does not arrive in its entirety,  Kermit-MS  will  normally

5.6.3. COMMANDS FOR FILE TRANSFER                                       Page 79

-------------------------------------------------------------------------------

discard  it  and it will not appear in your directory.  You may change this be-

havior by using the command SET INCOMPLETE KEEP, which will cause  as  much  of

the file as arrived to be saved on the disk.

The same single-character commands are available as during SEND:

^X  Request that the remote Kermit stop sending the current file, and proceed

to the next one immediately.  Since this is an optional  feature  of  the

Kermit protocol, the remote Kermit might not honor the request.

^Z  Request  that  the  remote  Kermit terminate the entire transfer; this is

also an optional feature that may or may not be supported by  the  remote

Kermit.

^C, ^E, and CR operate in the same way as they do during SEND.  In this case,

^E should always do what ^Z is supposed to do.

If WARNING is OFF and you type ^X or ^Z to interrupt the transfer,  you'll  ei-

ther get a partial new file, or else both the old and the new file of that name

will be lost, depending on SET INCOMPLETE.  In any case, when WARNING  is  off,

files with the same name as incoming files will not survive.

Caution: If an incoming file's name (the part before the dot) corresponds to an

MS-DOS device name, such as NUL, COM1, CON, AUX, or PRN, output will go to that

device, rather than to a file with that name.  This is a feature of MS-DOS.

5.6.4. Hints for Transferring Large Files

During  a  prolonged file transfer session, things can go wrong that are beyond

Kermit's control.  The longer the session, the greater the probability it  will

be fatally interrupted.  But you can take a few precautions:

- Make  sure  there  is sufficient disk space at the receiving end.  If

possible, first run a disk utility (such as CHKDSK on MS-DOS systems)

to clean out any bad disk blocks.

- If you are using a telephone connection, make sure your session won't

be interrupted by call waiting, people picking up  other  extensions,

etc.

- Don't  attempt  to  transfer  a  single file of many megabytes over a

telephone connection.  The longer the call, the greater the chance of

disconnection  (carrier  loss).   Although it's a bother, it may save

time in the long run to break the file up into smaller pieces, trans-

fer the pieces, and then recombine on the other end.

- SET  INCOMPLETE  KEEP  on  the receiving end, so that if the transfer

fails, then the partial file will be retained.  Then  chop  the  part

that wasn't transferred into a separate file, reconnect, and send it.

Then join the pieces together.

Consider moving truly massive amounts of data on magnetic media.  "Never under-

stimate the bandwidth of a station wagon full of magnetic tapes!"

5.6.5. Commands for Raw Uploading and Downloading                       Page 80

-------------------------------------------------------------------------------

5.6.5. Commands for Raw Uploading and Downloading

MS-Kermit  can  be used to send files to, or capture files from, remote systems

that do not have Kermit programs available.  No error checking or correction is

done,  so  the  results can very likely contain corrupted characters, spurts of

noise, gaps, or extraneous system messages or prompts.  The command for upload-

ing is TRANSMIT, and for downloading LOG SESSION.

To  minimize  loss  of  data  during  these  operations,  be  sure  to  SET the

FLOW-CONTROL and HANDSHAKE parameters to match the characteristics of the  sys-

tem on the other end.

The TRANSMIT Command

Syntax: TRANSMIT filespec [prompt-character]

The  TRANSMIT  command  provides  a  basic raw upload (export) facility to send

straight ASCII text files to the  host  without  packets,  error  checking,  or

retransmissions,  but using all the currently selected communication parameters

for flow control, parity, etc.  Information is read from the disk file  a  line

at a time, sent out the serial port, and the command waits for a single charac-

ter prompt (normally linefeed) from the host before sending the next file line.

A  disk  file line ends with carriage-return-linefeed (CRLF), but only the car-

riage return is sent, just as you only type carriage return at  the  end  of  a

line, not CR and LF.  Most remote systems will echo the CR and then also supply

a LF, which indicates that they have processed  the  line  and  are  ready  for

another one.  Setting the prompt to binary zero, \0, makes the TRANSMIT command

proceed without waiting for a prompt.  Pressing the local Return key  simulates

arrival of a prompt character.

Typically,  before  using this command to upload a file, you would start a text

editor (preferably a line-oriented, rather than  full-screen,  editor)  on  the

remote  host  and put it into text insertion mode.  When the file has been com-

pletely transmitted, you would manually enter the required sequence for getting

the  editor out of text insertion mode, and then make any necessary corrections

by hand.  Here's an example for VAX/VMS:

Kermit-MS>set flow xon/xoff Set flow control to match VAX/VMS.

Kermit-MS>connect           Connect to VAX.

$ edt foo.txt               Start the EDT editor.

*i                          Put it into "insert" mode.

^]c                         Escape back to Kermit-MS.

Kermit-MS>transmit foo.txt  Upload the file a line at a time.

...                        The lines are displayed on your screen.

Kermit-MS>connect           When done, connect back to the VAX.

^Z                          Type Ctrl-Z to exit EDT insert mode.

*exit                       Exit from EDT to save the file.

$

If transmission appears to be stuck, you can wake it up by  typing  a  carriage

return  on  the  keyboard.    You  can  cancel the TRANSMIT command by typing a

Control-C.  Control-Z's or other control characters in the file  may  have  ad-

verse  effects  on the host.  For this reason, you should use TRANSMIT only for

files that contain 7-bit printing  ASCII  characters,  spaces,  tabs,  carriage

returns, linefeeds, and possibly formfeeds.

5.6.5. Commands for Raw Uploading and Downloading                       Page 81

-------------------------------------------------------------------------------

The LOG SESSION Command

Syntax: LOG SESSION [filespec]

The LOG SESSION command lets you copy the characters that appear on your screen

during CONNECT into the specified file on the PC.  You can use this command  to

download files by displaying (usually with a command like TYPE) the file on the

remote system while logging is in effect.  Example:

Kermit-MS>set flow xon/xoff Set flow control to match VAX/VMS.

Kermit-MS>connect           Connect to the VAX.

$ type foo.bar              Give this command, but don't type CR yet.

^]c                         Escape back.

Kermit-MS>log sess foo.bar  Start logging.

Kermit-MS>connect           Connect back.

Now type the carriage return.

This is the file FOO.BAR.   The file is displayed on your screen

Blah blah ...               and captured into PC file FOO.BAR.

$                           The prompt is captured too.

^]c                         When done, escape back

Kermit-MS>close             and close the log file.

The PC file FOO.BAR now contains a (possibly  mutilated)  copy  of  the  remote

computer's  FOO.BAR  file.    It probably has the remote system's prompt at the

end, which you can edit out.  The session  log  can  also  be  used  to  record

typescripts,  editing  sessions, Tektronix graphics output, or any other output

from, or dialog with, the remote computer.  The  LOG  and  CLOSE  commands  are

described further below, in section 5.6.8.

5.6.6. Kermit Server Commands

Kermit-MS  can  act as a Kermit server, and can also interact with other Kermit

servers.  Normally, the remote Kermit is put into server mode.  Then the  local

Kermit  becomes a "client", and may issue repeated commands to the server with-

out having to connect and escape back repeatedly.  Servers can not only  trans-

fer  files,  but  can also provide a variety of file management functions.  The

SERVER command puts MS-Kermit into server mode, and the DISABLE and ENABLE com-

mands modify the behavior of the server.

Kermit  servers respond only to information sent as Kermit protocol packets and

not to ordinary CONNECT-mode commands.  When MS-Kermit is the client,  it  uses

the  SEND  command (described above) to send files to a server, the GET command

(not RECEIVE) to get files from a server, the REMOTE  commands  to  invoke  the

file  management  functions  of the server, and the BYE, FINISH, or LOGOUT com-

mands to shut down the server.  The MS-Kermit server can also  be  returned  to

interactive mode by typing Ctrl-C or Ctrl-Break on the PC's console keyboard.

The SERVER Command

Kermit-MS is capable of acting as a full-fledged Kermit server for users coming

in through one of the communication ports or a local  area  network.    To  put

Kermit-MS  into server mode, first issue any desired SET commands to select and

configure the desired port, then DISABLE any undesired functions, and then type

the  SERVER  command.    Kermit-MS will await all further instructions from the

5.6.6. Kermit Server Commands                                           Page 82

-------------------------------------------------------------------------------

client Kermit on the other end of the connection, which may  be  hardwired,  or

connected  through  a network or autoanswer modem.  In the following example, a

Kermit server is set up for dialing in:

Kermit-MS>set port 1

Kermit-MS>set baud 1200

Kermit-MS>connect

ATS0=1

OK

^]c

Kermit-MS>set timer on

Kermit-MS>set warning on

Kermit-MS>disable all

Kermit-MS>server

Before putting Kermit in server mode in this case, it was necessary to  connect

to  the  modem  (in  this  example, a Hayes) and put it into autoanswer mode by

typing the ATS0=1 command.

MS-Kermit 2.30 server mode supports the following requests:

SEND           REMOTE CWD          REMOTE SEND

GET            REMOTE DELETE       REMOTE SPACE

FINISH         REMOTE DIRECTORY    REMOTE TYPE

BYE            REMOTE HELP         REMOTE WHO

LOGO           REMOTE HOST

REMOTE CWD can be used to change both directories and devices.  The REMOTE SEND

command  accepts a one line message on the command line which will be displayed

on the operator's console.  An MS-Kermit  Server  can  DISABLE  recognition  of

selected REMOTE commands to help reduce accidents.

CAUTION: The method used for most of the REMOTE commands is to invoke a

task with the user's command line, redirect standard output to  a  tem-

porary  file,  $KERMIT$.TMP, send that file back to the remote end, and

then delete the file.  Sufficient space must be available to store this

file.  To service DOS commands or user tasks the boot drive must hold a

copy of COMMAND.COM.  PATH will not  be  searched  (this  can  be  dis-

asterous on a floppy disk based system).

FURTHER  CAUTION:  Any  of these DOS tasks or programs may encounter an

error, and in that case, DOS will generally put  the  familiar  "Abort,

Retry, Ignore?" message on the screen, and will wait for an answer from

the keyboard.  This will hang the server until a  human  comes  to  the

keyboard  and  gives  a  response.  The same thing will happen when any

program is invoked that interacts with the real console.  For instance,

REMOTE  SPACE  works  by  running  CHKDSK and if CHKDSK finds something

wrong with the disk while tallying up the space, it will  ask  (at  the

console) if you want to it to be fixed.  This, too, will hang the serv-

er.

MORAL: The MS-DOS Kermit server should probably not be used for  REMOTE

commands  unless  someone  is  around  to  take care of it when it gets

stuck.

For local network operation, the SET PORT NET command (with no node name)  must

5.6.6. Kermit Server Commands                                           Page 83

-------------------------------------------------------------------------------

be  issued  before  the  SERVER command.  MS-Kermit then becomes a network-wide

server, and other client Kermits can start a network session with it  by  using

the  name  of the Kermit Server, which is shown on the server's screen when SET

PORT NET is given.  The Kermit Server accepts connections from  other  Kermits,

but only one at a time.  There may be many Kermit Servers active on the network

simultaneously because each has a unique node name.  Operations are exactly the

same  as  with  serial port usage and the session (equivalent to a dialed phone

connection) is maintained between the pair until too many  timeouts  occur,  or

the  client  Kermit  issues a HANGUP command, exits to DOS, or SETs PORT NET to

another node.  In the latter cases, the server remains  available  for  use  by

other client Kermits.  If a client Kermit issues the BYE or FINISH command, the

network server is shut down (unless it was started with FIN disabled).

The DISABLE and ENABLE Commands

For security purposes, it may be desirable to leave your PC  in  Kermit  server

mode  so that it can be dialed in to, but with certain functions unavailable to

those who dial in.  The DISABLE and ENABLE commands provide this control.

The DISABLE and ENABLE commands affect the following functions, with the effect

of DISABLEs noted:

CWD     Changing of directories, disabled entirely.

DEL     Deletion of files confined to current directory.

DIR     Production of directory listings confined to current directory.

FIN     Shutting down the server (applies also to BYE) disabled entirely.

GET     Getting files from the server confined to current directory.

HOST    Execution of all REMOTE HOST (DOS) commands disabled entirely.

SEND    Forces files sent to server into current directory.

SPACE   Asking the server for a disk space report, disabled.

TYPE    REMOTE TYPE files confined to current directory.

ALL     All of the above.

TEK     Automatic  invocation  of Tektronix graphics mode by host commands.

This function is not related to server mode, and is not included in

the ALL term.

For  reasons  which  should  be  obvious,  the Kermit server does not provide a

REMOTE ENABLE command!

The GET Command

Syntax: GET remote-filespec

The GET command requests a remote Kermit server to send the file or file  group

specified by remote-filespec.  This command can be used only when Kermit-MS has

a Kermit server active on the other end of the connection.  This means that you

must  have  CONNECTed  to the other system, logged in, run Kermit there, issued

the SERVER command, and escaped back (e.g. "^]C") to the local  Kermit-MS  (or,

in  the case of LAN operation, a Kermit server must be running somewhere on the

network).  If the remote Kermit does not have a SERVER command, then you should

use SEND and RECEIVE as described above.

You  may  use  the GET command in a special way to specify a different name for

storing the incoming file.  Just type GET alone on a  line,  and  you  will  be

5.6.6. Kermit Server Commands                                           Page 84

-------------------------------------------------------------------------------

prompted separately for the remote filespec and the local filespec:

Kermit-MS>get

Remote Source File: com1 txt

Local Destination File: a:xcom1.txt

The  local  file name may contain a device field, and/or a directory specifica-

tion. Device and directory specifications in the local  destination  file  name

work the same way as in the RECEIVE command.

The  remote  filespec  is any string that can be a legal file specification for

the remote system; it is not parsed or  validated  locally.    It  can  contain

whatever wildcard or file-group notation is valid on the remote system.

Once the file transfer begins, the GET command behaves exactly like the RECEIVE

command.

Warning:  If the remote filespec is to contain a semicolon, and the GET command

is  being issued from a TAKE command file, you must prefix the semicolon with a

backslash.  Otherwise, all characters beginning with the semicolon will be  ig-

nored:

get me.home\;0

5.6.7. Commands for Controlling Remote Kermit Servers

The  BYE,  FINISH,  and  LOGOUT commands allow you to shut down a remote Kermit

server:

BYE     When communicating with a remote Kermit server, use the BYE command  to

shut  down the server, log out its job, and exit locally from Kermit-MS

to DOS.  On local area networks, BYE also terminates the  network  ses-

sion.

FINISH  Like  BYE,  FINISH  shuts down the remote server.  However, FINISH does

not log out the server's job.  You are left at Kermit-MS  prompt  level

so that you can connect back to the job on the remote system.  On local

area nets, FINISH shuts down the MS-Kermit server, but in  a  way  that

allows it to be restarted as if no interruption had occurred.

LOGOUT  The  LOGOUT  command  is  identical to the BYE command, except you will

remain at Kermit-MS prompt level, rather than exit to DOS, so that  you

can  establish  or  use  another  connection  without having to restart

MS-Kermit.

The REMOTE Commands

The REMOTE keyword is a prefix for a number of commands.  It indicates that the

command  is  to be performed by a remote Kermit server.  Not all Kermit servers

are capable of executing all of these commands, and some Kermit servers may  be

able to perform functions for which Kermit-MS does not yet have the correspond-

ing commands.  In case you send a command the server cannot  execute,  it  will

send  back  a message stating that the command is unknown to it.  If the remote

server can execute the command, it will send  the  results,  if  any,  to  your

5.6.7. Commands for Controlling Remote Kermit Servers                   Page 85

-------------------------------------------------------------------------------

screen.

Here are the REMOTE commands that Kermit-MS may issue:

REMOTE CWD [directory]

Ask the server to Change your Working Directory  on  the  remote  host,

that  is, the default source and destination area for file transfer and

management.  You will be prompted for a password, which will  not  echo

as  you type it.  If you do not supply a password (i.e. you type only a

carriage return), the server  will  attempt  to  access  the  specified

directory  without  a password.  If you do not supply a directory name,

your default or login directory on the remote system  will  be  assumed

and you will not be prompted for a password.

REMOTE DELETE filespec

Ask the server to delete the specified file or files on the remote sys-

tem.  In response, the server may display a list of the files that were

or were not successfully deleted.

REMOTE DIRECTORY [filespec]

Ask  the  server to display a directory listing of the specified files.

If no files are specified, then the list should include  all  files  in

the current working directory.

REMOTE HELP

Ask the server to list the services it provides.

REMOTE HOST [command]

Ask  the  server  to  send  the  command to the remote system's command

processor for execution.

REMOTE KERMIT command

Send  the  command  to the remote Kermit for interpretation as a Kermit

command in the remote Kermit server's own command syntax.  Most  Kermit

servers,  including  Kermit-MS, do not yet recognize REMOTE KERMIT com-

mands.

REMOTE SEND message

Send the one line message to be displayed on the Server's screen.

REMOTE SPACE [directory]

Ask the server to  provide  a  brief  summary  of  disk  usage  in  the

specified area on the remote host or, if none specified, the default or

current area.

REMOTE TYPE filespec

Ask  the server to display the contents of the specified remote file or

files on your screen.

REMOTE WHO [who-spec]

Ask  the  server  to  list  actively logged on users; optional who-spec

qualifies the list and uses the syntax of the server system.

5.6.8. The LOG and CLOSE Commands                                       Page 86

-------------------------------------------------------------------------------

5.6.8. The LOG and CLOSE Commands

Syntax: LOG {PACKET, SESSION} [filespec]

CLOSE

LOG SESSION specifies  that  your  terminal  session  during  CONNECT  will  be

recorded  in  the indicated file (whose name defaults to KERMIT.LOG in the cur-

rent directory), with any input character translations applied according to SET

TRANSLATION  INPUT.    The  filespec  may include a device specification and/or

directory path.  The LOG SESSION command allows you to "capture" files  from  a

remote   system  that  doesn't  have  Kermit,  as  well  as  to  record  remote

typescripts.  LOG PACKET causes the log file to record all packets in file  ex-

changes (DEBUG need not be set ON).  Both kinds of logging can be active simul-

taneously but only one filename can be specified -- everything goes to the same

log  file.  The log is closed when you EXIT from Kermit-MS or when you issue an

explicit CLOSE command.

If the log file already exists then new material is appended to it.  Try not to

delete an open log file (CLOSE it first) because DOS will become confused.

During  terminal emulation, the LOG command records all the characters that ar-

rive from the remote host in the specified file,  including  escape  sequences.

If  you  have  SET  LOCAL-ECHO ON, it will also record the characters you type.

Logging may be suspended and resumed within a terminal session with the CONNECT

escape-level  commands  Q  and  R. The log file will be composed of 7-bit ASCII

bytes if (a) PARITY is other than NONE, or (b) DISPLAY is SET to 7.  If DISPLAY

is  8  and  PARITY  is NONE, or if DEBUG is ON, then the log will contain 8-bit

bytes.

You may LOG PRN to cause the logging information to be printed directly on your

printer.  Any escape sequences that are sent to the screen are also sent to the

printer.

If you want to record information without imbedded escape  sequences,  use  the

screen  dump  feature,  invoked by the CONNECT escape-level command F, which is

described under the CONNECT command.

A session log cannot be played back directly on the PC from the log file.    To

relive the session, you must transfer it to the remote system and display it in

"binary mode" (e.g. cat in Unix) which CONNECTed.

5.6.8. The LOG and CLOSE Commands                                       Page 87

-------------------------------------------------------------------------------

5.6.9. The SET Command

Syntax: SET parameter value

-or- SET parameter parameter value

The SET command establishes or modifies parameters for file  transfer  or  ter-

minal  connection.   You can examine their values with the STATUS command.  The

following SET commands are available in Kermit-MS:

BAUD  Communications port line speed (synonym for SPEED)

BELL  Whether to beep at the end of a transaction

BLOCK-CHECK-TYPE  Level of error checking for file transfer

DEBUG  Display packet contents during file transfer

DEFAULT-DISK  Default disk drive for file i/o

DELAY  Wait number seconds before Sending a file

DESTINATION  Default destination device for incoming files

DISPLAY  For selecting the type of file transfer display

DUMP  Screen dump file (or device) name

END-OF-LINE  Packet termination character

EOF  Method for determining or marking end of file

ESCAPE  Escape character for CONNECT

FLOW-CONTROL  Enable or disable XON/XOFF

HANDSHAKE  Half-duplex line turnaround option

INCOMPLETE  What to do with an incompletely received file

INPUT  Behavior of INPUT command for scripts

KEY  Specify key redefinitions

LOCAL-ECHO  Specify which computer does the echoing during CONNECT

MODE-LINE  Whether to display a mode line during terminal emulation

PARITY  Character parity to use

PORT  Select a communications port

PROMPT  Change the "Kermit-MS>" prompt to something else

RECEIVE  Request remote Kermit to use specified parameters

REMOTE  For running Kermit-MS interactively from back port

RETRY  Packet retransmission threshold

SEND  Use the specified parameters during file transfer

SPEED  Communications port line speed (synonym for BAUD)

TAKE-ECHO  Control echoing of commands from TAKE files

TERMINAL  Emulation and parameters

TIMER  Enable/disable timeouts during file transfer

TRANSLATION  Enable/disable/specify conversion of arriving characters

WARNING  Specify how to handle filename collisions

The SET commands are now described in detail, in alphabetical order.

SET BAUD

Syntax: SET BAUD number

Set the speed (in bits per second,  commonly  called  baud)  of  the  currently

selected  terminal  communications  port  (COM1 by default) to 300, 1200, 1800,

2400, 4800, 9600, or other common speed, and  on  the  IBM  PC  family,  higher

speeds  including  19200, 38400, 57600, and 115200.  Both connected systems, as

well as any intervening communication equipment, must be able  to  support  the

specified baud rate, and both systems should be set to the same baud rate.

5.6.9. The SET Command                                                  Page 88

-------------------------------------------------------------------------------

Some implementations do not support the SET BAUD command.  But Kermit-MS leaves

the current communication port settings alone unless  you  issue  explicit  SET

commands to change them, so you may use MODE or other DOS programs to establish

the desired settings before running Kermit.

On certain systems, when you first run Kermit after powering the system up, you

may  get  a  message "Unrecognized baud rate".  This means that Kermit tried to

read the baud rate from the port and none was set.  Simply  use  SET  BAUD  (if

available) or the DOS MODE command to set the desired baud rate.

SET SPEED is a synomym for SET BAUD.

SET BELL

Syntax: SET BELL {ON, OFF}

Specifies  whether  the  bell  (beeper)  should sound upon completion of a file

transfer operation.  Normally ON.

SET BLOCK-CHECK-TYPE

Syntax: SET BLOCK-CHECK-TYPE {1, 2, 3}

Selects the error detection method: a 1-character 6-bit  checksum  (the  normal

case), a 2-character 12-bit checksum, or a 3-character 16-bit cyclic redundancy

check (CRC).  If the other Kermit program is not capable of type 2 or 3  check-

ing  methods,  automatic fallback to type 1 will occur.  The more secure type 2

and 3 block checks take essentially no more execution time than  the  simple  1

character  checksum.    SET  BLOCK 3 is a stronger check than SET BLOCK 2.  SET

BLOCK 2 or 3 is recommended for use with long packets (see below),  noisy  com-

munication lines, binary (8-bit data) files, and text files containing critical

data (budgets, grades, etc).

SET DEBUG

Syntax: SET DEBUG {PACKET, SESSION, ON, OFF}

With DEBUG PACKET, Kermit will display packet traffic  on  your  screen  during

file  transfer.   With the normal file transfer display, regular-length packets

sent  and  received  are  displayed  in  fixed-size  slots.    The  display  of

extended-length  packets,  however  (see  SET  RECEIVE PACKET-LENGTH), tends to

overlap.  If this bothers you, then also SET DISPLAY SERIAL, or LOG the packets

rather than displaying them.

With  DEBUG  SESSION,  during terminal emulation (on the IBM PC, Rainbow, and a

few others), control characters are displayed in  uparrow  ("^")  notation  and

characters  with the 8th bit set are preceded by the tilde ("~") sign, and your

session log (if any) will record 8-bit bytes, rather than 7-bit ASCII,  regard-

less  of  SET  DISPLAY  or  SET PARITY.  Character translation (SET TRANSLATION

INPUT) is not done during session debugging.  The effect of SET  DEBUG  SESSION

during  terminal connection can be disconcerting, but it gives you a convenient

line monitor equivalent to a specialized device  that  costs  several  thousand

dollars,  and  it  can  prove  very  handy for tracking down data communication

5.6.9. The SET Command                                                  Page 89

-------------------------------------------------------------------------------

problems.

SET DEBUG ON turns on both SESSION and PACKET  debugging,  and  SET  DEBUG  OFF

turns them both off.

SET DEFAULT-DISK

Syntax: SET DEFAULT-DISK x:

Specify  the  default  disk drive to use for file transfer, directory listings,

and so forth.  Equivalent to typing the DOS command for changing disks (A:, B:,

etc).    Affects Kermit and all inferior processes, but when you exit from Ker-

mit, you will still have the same default disk as when you entered.

SET DELAY

Syntax: SET DELAY number

Wait the specified number of seconds before starting a file transfer.  Intended

for  use  when  the  other side needs appreciable time to become ready, such as

rearranging cables, changing programs, etc.,  or  when  MS-DOS  Kermit  is  the

remote  Kermit  (e.g.  after  CTTY COM1, SET REMOTE ON).  The number is 0 to 63

seconds, normally 0.

SET DESTINATION

Syntax: SET DESTINATION {DISK, PRINTER, SCREEN}

SET DESTINATION PRINTER will cause incoming files to be sent  directly  to  the

printer;  SCREEN will send output normally destined for the disk to the screen.

The normal destination is DISK.  SET DESTINATION affects only files transferred

with SEND, GET, or RECEIVE; it cannot be used to reroute the output from REMOTE

server commands.

SET DISPLAY

Syntax: SET DISPLAY {QUIET, REGULAR, SERIAL, 7-BIT, 8-BIT}

During file transfer, MS-DOS Kermit's regular display  is  a  formatted  screen

whose  fields  are  randomly  updated  with  file  names, packet numbers, error

counts, percent done, error messages, and so forth.

If you wish to run Kermit-MS interactively through the back port, for  instance

after  the operator has done CTTY COM1, you must give the command SET REMOTE ON

(which, currently at least, is equivalent to  SET  DISPLAY  QUIET);  this  sup-

presses  the  file transfer display screen, so that the display won't interfere

with the file transfer itself.  You can also use this command to  suppress  the

display  in  local  mode,  in case you are using a system that allows you to do

other work while file transfer proceeds in the background.

If you have your PC connected to a  speaking  device  (a  common  practice  for

visually  impaired  people), or you are logging the display screen to a printer

5.6.9. The SET Command                                                  Page 90

-------------------------------------------------------------------------------

(using DOS ^P or kermit > prn), the random nature of the regular  display  will

make  the  results of little use.  SET DISPLAY SERIAL is provided for this pur-

pose; it causes the program to report progress "serially" on the  screen.    In

serial mode, error messages are preceeded with the word "Error" and repeat mes-

sages with the word "Retry".  Packets are numbered as dots with every tenth be-

ing  a  plus  sign.  The packet display is automatically broken across lines at

every 70th packet.  The serial display makes much more sense when  spoken  than

does the regular display.

The  serial display does not show the percent and kilobytes transferred.  It is

the default display style for generic MS-DOS Kermit; REGULAR is the default for

all others.

The  last  two parameters, 7-BIT and 8-BIT, control the size of characters sent

to the screen during terminal emulation.  7-BIT is the default and includes all

ASCII characters.  8-BIT is useful with national and line drawing characters.

SET DUMP

Syntax: SET DUMP filespec

On  those  systems that support this feature, change the file or device name of

the screen dump file.  The normal file name is KERMIT.SCN.  See the section  on

terminal  emulation  for details about screen dumps.  If the specified file al-

ready exists then new material is appended to old.

SET END-OF-LINE

Syntax: SET END-OF-LINE number

If the remote system needs packets to be terminated by anything other than car-

riage  return, specify the decimal value, 0-31, of the desired ASCII character.

Equivalent to SET SEND END-OF-LINE (SET END-OF-LINE is kept only for historical

reasons, and the parameter really should be called END-OF-PACKET anyway.)

SET EOF

Syntax: SET EOF {CTRL-Z, NOCTRL-Z}

Controls  how the end of file is handled.  CTRL-Z specifies a Control-Z charac-

ter should be appended to the end of an incoming file, unless it  already  ends

with  a  Control-Z.  Certain MS-DOS text editors and other applications require

files to be in this format.  For outbound files, treat the first  Control-Z  as

the  end  of  the  local file, and do not send it or any subsequent characters.

NOCTRL-Z is the default; incoming files are stored, and MS-DOS files are  sent,

exactly as is, in their entirety.

5.6.9. The SET Command                                                  Page 91

-------------------------------------------------------------------------------

SET ESCAPE

Syntax: SET ESCAPE character

Specify  the  control character you want to use to "escape" from remote connec-

tions back to Kermit-MS.   On  most  systems  the  default  is  "^]"  (Control-

Rightbracket),  which  was chosen because it is a character you would otherwise

rarely type.

The character is entered literally after SET ESCAPE or in backslash number form

(\29),  and  should be chosen from the ASCII control range.  It is not possible

to use non-ASCII characters (like function keys) for this purpose (but see  SET

KEY for a way around this restriction).

SET FLOW-CONTROL

Syntax: SET FLOW-CONTROL {XON/XOFF, NONE}

Specify the full duplex flow control to be done on the currently selected port.

The options are XON/XOFF and NONE.  The specified type of flow control will  be

done  during  both  terminal emulation and file transfer.  By default, XON/XOFF

flow control is selected.  XON/XOFF should not be used  on  half-duplex  (local

echo)  connections,  or with other systems that do not support it.  If XON/XOFF

is used, HANDSHAKE should be set to NONE.

SET HANDSHAKE

Syntax: SET HANDSHAKE {CODE number, BELL, CR, LF, NONE, XOFF, XON}

Specify any half-duplex line turnaround handshake character to be  used  during

file  transfer on the currently selected port.  The CODE number form allows any

ASCII character to be specified by its decimal ASCII code.  Handshake  is  NONE

by default; if set to other than NONE, then FLOW-CONTROL should be set to NONE.

In operation the handshake character is sought at  the  end  of  each  received

packet,  following  the  normal END-OF-LINE character, but is not sent for out-

going packets.

SET INCOMPLETE

Syntax: SET INCOMPLETE {DISCARD, KEEP}

Specifies what to do with files that arrive incompletely: discard them or  keep

them.  They are normally discarded.

SET INPUT

Syntax: SET INPUT {CASE, DEFAULT-TIMEOUT, ECHO, TIMEOUT-ACTION}

SET INPUT controls the behavior of the script INPUT command (see the section on

scripts for more information):

5.6.9. The SET Command                                                  Page 92

-------------------------------------------------------------------------------

SET INPUT CASE {IGNORE, OBSERVE}

Says  whether or not to distinguish upper and lower case letters when doing

a matchup in the INPUT command.  OBSERVE causes upper and lower  case  let-

ters to be distinguished.  The default is to IGNORE case distinctions.

SET INPUT DEFAULT-TIMEOUT seconds

Changes the waiting time from one second to this new value.

SET INPUT ECHO {ON, OFF}

Show  on  the screen characters read from the serial port during the script

operation, or not.  Default is ON, show them.

SET INPUT TIMEOUT-ACTION {PROCEED, QUIT}

Determines  whether or not the current TAKE command file is to be continued

or exited if a timeout occurs.  PROCEED  is  the  default  and  means  that

timeouts are ignored.  QUIT causes the current script file to be exited and

control passed to either the next higher level script  file  (if  there  is

one) or to Kermit's main prompt.

SET KEY

Syntax: SET KEY key-specifier [key-definition]

Also: SET KEY {ON, OFF, CLEAR}

WARNING: The format and functions of this command have changed substan-

tially since version 2.29B and earlier.  The changes were made in order

to  allow  key  redefinition  to work on a wider variety of systems and

keyboards without customization of the program  source  code  for  each

configuration.  See section 5.10 for further details.

The  SET  KEY command is one of the most useful -- and confusing -- features of

MS-Kermit.  Typical uses:

- You're used to having the ESC key in the upper  left  corner  of  the

keyboard,  but  your new PC keyboard has an accent grave ("`") there.

You can use SET KEY to make the accent key transmit an ESC,  and  you

can assign accent grave to some other key.

- You  send  a lot of electronic mail, and always sign it the same way.

You can put your "signature" on a single key to save yourself  a  lot

of repetitive typing.

- You  must  set  up  your PC's function keys or numeric keypad to work

properly with a host application.

The SET KEY command does these things and more, and SHOW KEY  gives  us  assis-

tance.  A key can be defined to:

- send a single character other than what it would normally send,

- send a string of multiple characters,

- invoke a CONNECT-mode Kermit action verb,

- send itself again.

SET KEY specifies that when the designated key is struck during terminal emula-

tion, the specified character or string is sent or the specified Kermit  action

5.6.9. The SET Command                                                  Page 93

-------------------------------------------------------------------------------

verb  is  performed.    Key  definitions  operate  only  during CONNECT, not at

Kermit-MS> or DOS command level.

The key-specifier is the identification of the key expressed in  system-depend-

ent  terms.    This can be a letter, such as Q for the key which emits an upper

case Q, or the numeric ASCII value of the letter in  backslash  notation  (e.g.

"\81"),  or  else the numerical "scan code" observed by the system when the key

is pressed (e.g. "\3856" for Ctrl-Alt-Q on an IBM PC).    Material  printed  on

keycaps  is  not necessarily a guide to what the key-specifier should be.  When

the word CLEAR is used in place of a key-specifier,  all  key  definitions  are

cleared and then any built-in definitions are restored.

A string definition is one or more characters, including 8-bit values expressed

in backslash form, such as

SET KEY \315 top \13 list\13    IBM F1 key sends "top <cr> list<cr>"

SET KEY S L                     S key sends upper case L

SET KEY T \27[m                 T key sends three bytes ESC [ m

The string begins with the first non-spacing character following the key  iden-

tification  and  continues  until  the  end  of line, exclusive of any trailing

spaces.  If a semicolon comment is used and the definition is given in  a  TAKE

file,  the  line  ends  at the last non-spacing character before the semicolon.

Curly braces, {...<}>, can be use to delimit the string in case  you  want  the

definition  to  include trailing spaces.  All text after the closing bracket is

ignored.

SET KEY \315 {login } this extra text is ignored

This manual does not contain a list of all the scan codes for all the  keys  on

all  the  keyboards  on  all  the PCs supported by MS-Kermit -- that would be a

manual in itself.  Rather, in order to obtain the key-specifier for the SET KEY

command, you must type a SHOW KEY command and press the desired key or key com-

bination.  This will report a scan code that you can use as the  key  specifier

in  a SET KEY command.  To do this for many keys is a laborious process, so you

should collect all your SET KEY commands into a file, which you  can  TAKE,  or

put them in your MSKERMIT.INI file.

If  you enter SET KEY by itself, with no key specifier, the command will prompt

you to press the selected key and again for the  definition  string.    Certain

characters,  like ESC and CR, may not be entered literally into the string, but

can be included by inserting escape codes of the form \nnn,  a  backslash  fol-

lowed by a 1- to 4-digit number corresponding to the ASCII value of the desired

character.  Where an ASCII digit follows directly  after  a  backslash  number,

confusion can be avoided by placing curly braces {} around the backslashed num-

ber; thus, \{27}5 represents the two ASCII characters ESC 5.

Here is an example of the use of SET KEY to assign ESC (ASCII 27) to the accent

grave key.  First the user gets the key-specifier for the key:

Kermit-MS>show key

Push key to be shown (? shows all): `

ASCII char: ` \96 decimal is defined as

Self, no translation.

Free space: 145 keys and 96 string definitions, 894 string characters.

5.6.9. The SET Command                                                  Page 94

-------------------------------------------------------------------------------

The free space report says that 145 more keys may be redefined, and up to 96 of

them may have multi-character strings assigned to them (as  opposed  to  single

characters),  and  that  there  are 894 bytes left for these strings, in total.

Confident that there is enough space left for a new key  definition,  the  user

proceeds:

Kermit-MS>set key

Push key to be defined: `

Enter new definition: \27

Once  a key definition is constructed and tested, it may be entered on a single

line in a command file (such as MSKERMIT.INI):

set key \96 \27

To prevent accidents, SET KEY shows the current definition before asking for  a

new one; enter a Control-C to keep the current definition, or a carriage return

to undefine the key, or a query mark (?) to see available choices.

A more exact definition of SET KEY follows these construction rules:

SET KEY [<key-ident><whitespace><definition>]

where <key-ident> is:

- a single ordinary printable ASCII character, or

- the numerical equivalent of an ASCII character (ESC is \27), or

- a scan code written as  a  backslashed  number  (e.g.  \3856  for

Ctrl-Alt-Q), or

- keyword  CLEAR which removes all current key definitions and then

restores any built-in definitions.

- ? (a question mark), which displays a help message.

Scan codes, non-printing ASCII characters, 8-bit characters, and  other  binary

codes are written in backslash form (see Table 5-1 for ASCII codes):

\123            a decimal number (decimal is the default number base)

\d789           a decimal number (also \D)

\o177           an octal number (also \O)

\x0d            a hexadecimal number (also \X)

\{b###}         braces  around  material  following  the  slash,  where the

numerical base letter b is d, o, x, or absent.

<whitespace> is one or more spaces and or tabs.

<definition> is:

missing altogether which "undefines" a key.

\number         (a number in backslash notation between 0 and 255  decimal)

- assigns a character with the corresponding 8-bit value.

\Kverb          for a Kermit action verb; upper or lower case K is OK.

\{Kverb}        Same.  Verb is the name of an action verb.

5.6.9. The SET Command                                                  Page 95

-------------------------------------------------------------------------------

text            a  string with allowed embedded whitespace and embedded bi-

nary chars as above.  This kind of string may not  commence

with sequences \K or \{K.  The string begins with the first

non-whitespace character and ends at the end  of  the  line

(exclusive of trailing spaces and comments).

{text}          string  confined  to  material  within  but  excluding  the

braces.  Note, where the number of opening  braces  exceeds

the number of closing braces the end of line terminates the

string:

{ab{}{{c}d ==> ab{}{{c}d

but

{ab}{{c}d ==> ab

?               Displays help message and lists all action verbs.

Control-C       Quits the Set Key command without making changes.

Curly braces can be used to separate backslash numbers from regular ASCII text,

such as when we want to send the two bytes ESC 8 (ESC is decimal 27):

SET KEY T \{27}8

SET KEY L {words \{27}8 more words} trailing junk to be ignored

If a string begins with a left curly brace then Kermit counts them and ends the

string on the matching right curly brace (or end of line); the SET  KEY  L  ex-

ample  above  relies on brace matching to omit the trailing comment.  The outer

curly brace delimiters are not sent but the inner ones are; for example:

SET KEY U {a {b}\{27}8}         sends the 7 bytes "a {b}ESC8"

SET KEY U a{ {b}\{27}8}         sends the 9 bytes "a{ {b}ESC8}"

SET KEY U a{ {b}\{{27}}8}       sends the 15 bytes "a{ {b}\{{27}}8}"

A definition is removed, that is a key is defined as itself, by giving an empty

definition:

SET KEY q                       q key sends q again

SET KEY \331                    IBM left arrow key is undefined.

The  keyboard  can  be restored to its startup state, that is all redefinitions

removed and all built-in defitions restored, by  using  the  keyword  CLEAR  in

place of the key identification:

SET KEY CLEAR

Undefined  keys  which do not send ASCII characters are trapped by the keyboard

translator and are rejected; a beep results from using an  undefined  non-ASCII

key.

SET  KEY  OFF directs MS-Kermit to read keycodes from DOS, rather than BIOS, so

that console drivers like ANSI.SYS that operate at the DOS level  may  be  used

during  Kermit CONNECT sessions.  This would also apply to any special keyboard

replacements that come with DOS-level drivers.  SET KEY ON turns key definition

5.6.9. The SET Command                                                  Page 96

-------------------------------------------------------------------------------

back on, and returns Kermit to processing keystrokes at the BIOS level.

Kermit Action Verbs:

An  action  verb is the shorthand expression for a named Kermit procedure, such

as "generate the proper sequence for a left  arrow,"  "show  status,"  "send  a

BREAK,"  and  others; verbs are complex actions and each verb has a name.  In a

key definition the verb name is preceeded by backslash K (\K)  to  avoid  being

confused with a string.  Verbs and strings cannot be used together on a key.

SET KEY \331 \Klfarr

SET KEY \2349 \Kexit

makes  the IBM keyboard left arrow key execute the verb named lfarr which sends

the proper escape sequence for a VT102 left arrow key (which changes  depending

on  the internal state of the VT102).  The leading \K identifies the definition

as a Kermit verb, so no string can start as \K or as \{K in upper or lower case

(use  \92K).  The second example has Alt-X invoking the Leave-Connect-Mode verb

"exit" (same as Kermit escape character "^]" followed by C).

Each system has its own list of verbs and predefined keys.    Table  5-4  shows

those available for the IBM PC family (there are also some additional verbs for

reassigning Heath or VT100 function keys, see section 5.15.2).    The  SET  KEY

command  shows  the list of available verbs when a query mark (?) is given as a

definition.  SHOW KEY displays  all  currently  defined  keys  or  individually

selected ones; SHOW KEY can be executed only interactively.

Some  systems  have  preset key definitions when Kermit first begins (those for

the IBM PC are shown in section 5.15.2).  You can find out  what  they  are  on

your  system  by typing SHOW KEY, and then question mark on the next line.  You

may supplement or change the predefined keys with SET KEY commands typed inter-

actively or in MSKERMIT.INI or other command files.

The  MS-Kermit  CONNECT command may be used in conjunction with certain console

drivers that do their  own  key  redefinitions.    Since  MS-Kermit  intercepts

keystrokes at the BIOS level, drivers like ANSI.SYS which work at the DOS level

will have no effect during CONNECT, even though they work at MS-Kermit  command

level.    Other  drivers, like SuperKey and ProKey, work at the BIOS level, and

their key assignments will remain effective during  Kermit  terminal  sessions,

and additional Kermit SET KEY assignments may be made "on top" of them.

SET LOCAL-ECHO

Syntax: SET LOCAL-ECHO {ON, OFF}

Specify  how  characters  are echoed during terminal emulation on the currently

selected port.  ON specifies that characters are  to  be  echoed  by  Kermit-MS

(because  neither the remote computer nor the communications circuitry has been

requested to echo), and is appropriate for half-duplex connections.  LOCAL-ECHO

is OFF by default, for full-duplex, remote echo operation.

5.6.9. The SET Command                                                  Page 97

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Verb           Meaning

\Kupscn        Roll up (back) to previous screen

\Kdnscn        Roll down (forward) to next screen

\Khomscn       Roll up to top of screen memory

\Kendscn       Roll down to end of screen memory (current position)

\Kupone        Roll screen up one line

\Kdnone        Roll screen down one line

\Kprtscn       Print the current screen

\Kdump         Append the current screen to dump file

\Klogoff       Turn off session logging

\Klogon        Turn on session logging

\Ktermtype     Toggle terminal type

\Kreset        Reset terminal emulator to initial state

\Kmodeline     Toggle modeline off/on

\Kbreak        Send a BREAK signal

\Klbreak       Send a "long BREAK" signal

\Khangup       Drop DTR so modem will hang up phone

\Knull         Send a null (ASCII 0)

\Kdos          "Push" to DOS

\Khelp         Display CONNECT help message

\Kstatus       Display STATUS message

\Kexit         Escape back from CONNECT mode

\Kgold,\Kpf1   VT102 keypad function key PF1

\Kpf2..\Kpf4   VT102 keypad function keys

\Kkp0..\Kkp9   VT102 keypad numeric keys

\Kkpdot,\Kkpminus,\Kkpcoma,\Kkpenter   Other VT102 keypad keys

\Kuparr,\Kdnarr,\Klfarr,\Krtarr   VT102 cursor (arrow) keys

Table 5-4:   Kermit-MS Verbs for the IBM PC Family

-------------------------------------------------------------------------------

SET MODE-LINE

Syntax: SET MODE-LINE {ON, OFF}

On  systems,  like the IBM PC family, which are capable of displaying a status,

or "mode" line on the 25th line during terminal connection, disable  or  enable

this  function.    This  command has no effect on systems that do not display a

mode line during connect.

The mode line shows several important facts about the  connection,  like  which

port  is  being  used,  the baud rate and parity, the current escape character,

etc.  When the mode line is enabled, it may be turned on and off using the CON-

NECT escape-level command M or the Kermit verb "modeline".

The  mode  line occupies the 25th line of those systems that have such a thing,

and is not affected by scrolling (on some systems that have large screens,  the

mode  line  should appear on whatever the bottom line is, e.g. the 43rd).  When

emulating a VT102 or Heath-19, Kermit will allow the host to address  the  25th

line  directly using cursor positioning commands.  If this happens, Kermit will

5.6.9. The SET Command                                                  Page 98

-------------------------------------------------------------------------------

remove its mode line and relinquish control of the 25th line to the host (as if

you had typed SET MODE OFF).  When no terminal is being emulated, the 25th line

(if any) is available for scrolling.  If the mode line is disabled  by  an  ap-

plication  or  by the command SET MODE OFF then the only way to revive Kermit's

mode line display is to give the command SET MODE ON.  On IBM systems  support-

ing  different  length  displays, the host needs to address the bottom line ex-

plicitly.  Also, note the line is saved  and  restored  when  CONNECT  mode  is

exited  and  reentered  so an old "fossil" mode line may reappear unexpectedly.

If this happens, you can escape back to the Kermit prompt,  say  SET  MODE  ON,

CONNECT, and toggle it off ("^]M") to clear the line.

SET PARITY

Syntax: SET PARITY {EVEN, ODD, MARK, SPACE, NONE}

Specify  the  character parity to be used on the currently selected port.  NONE

means no parity processing is done, and the 8th bit of each  character  can  be

used  for  data  when  transmitting binary files.  This is the normal case.  If

parity is other than none, then there will be 7 data bits (use of parity with 8

data bits is not supported).

If  you  have  set parity to ODD, EVEN, MARK, or SPACE, then Kermit-MS will re-

quest that binary files be transferred using 8th-bit-prefixing.  If  the  other

Kermit  knows  how  to do 8th-bit-prefixing (this is an optional feature of the

Kermit protocol, and some implementations of Kermit don't have it), then  8-bit

binary  files  can be transmitted successfully.  If NONE is specified, 8th-bit-

prefixing will not be requested.  Note that there  is  no  advantage  to  using

parity.    It  reduces  Kermit's file transfer efficiency without providing ad-

ditional error detection.  The SET PARITY command is  provided  only  to  allow

Kermit  to  adapt to conditions where parity is required, or 8-bit transmission

is otherwise thwarted.

You will need to SET PARITY to ODD, EVEN, MARK, or  possibly  SPACE  when  com-

municating  with a system, or over a network, or through modems, concentrators,

multiplexers, or front ends that require or impose character parity on the com-

munication  line.   For instance, most IBM mainframe computers use EVEN or MARK

parity; Telenet normally uses MARK parity.  If you neglect to SET  PARITY  when

the  communications  equipment  requires  it,  the symptom may be that terminal

emulation works (well or maybe only partially), but file transfer or script IN-

PUT commands do not work at all.

If parity is in use, then the display during terminal emulation, as well as any

session log, will be 7-bit ASCII, unless you have SET DEBUG ON (q.v.).

There may be situations in which you require 7-bit ASCII with no parity  during

terminal  emulation,  but  still  want  to  force 8th bit prefixing during file

transfer.  To accomplish this, SET PARITY SPACE.

The INPUT and TRANSMIT commands use 7 or 8 bits if parity is NONE, according to

the  SET DISPLAY command, and this may upset recognition of received characters

when the host unexpectedly sends them with its own parity.

5.6.9. The SET Command                                                  Page 99

-------------------------------------------------------------------------------

SET PORT

Syntax: SET PORT {number, COM1, COM2, NET [nodename]}

On machines with more than one communications port, select the port to use  for

file  transfer and CONNECT.  This command lets you use a different asynchronous

adapter, or switch between two or more  simultaneous  remote  sessions.    Sub-

sequent  SET  BAUD, PARITY, HANDSHAKE, FLOW, and LOCAL-ECHO commands will apply

to this port only -- each port remembers its own parameters, so  that  you  may

set  them for each port and then switch between ports conveniently with the SET

PORT command.

SET PORT 1 selects COM1, SET PORT 2 selects COM2.  All versions default to port

1,  except  for  the  IBM  PCjr, which uses port 2 if its internal modem is in-

stalled.  Additionally, COM3 and COM4 are supported for IBM PC's if the  system

is informed of their presence (as explained in Section 5.16.3).

In  "generic" MS-DOS Kermit, the following alternate forms allow you to experi-

ment with device names or numbers until you find the communication port:

SET PORT {DEVICE, FILE-HANDLE}

Just type a carriage return after either of these commands,  and  you  will  be

prompted for a device name or a numeric port-handle.  Keep trying till you find

one that works. File-handle 3, the system auxillary device, is conventional  on

many machines, as are device names COM1, COM2, and AUX.

MS-Kermit  for  the  IBM  PC family is able to operate over local area networks

through the NetBIOS interface.  The command

SET PORT NET [nodename]

redirects communications the LAN board installed in the local computer and  the

associated NetBIOS emulator software, if active, rather than the serial port or

the COM device driver.  It installs a unique Kermit node name in the local LAN,

so  that  other  nodes  can  refer to it when files are transferred or terminal

emulation is done.  This name is displayed when you give the SET PORT NET  com-

mand.    The server should use SET PORT NET, and the client should use SET PORT

NAME nodename, specifying the server's name,  e.g.  CHRIS.K.    Note  that  al-

phabetic case is significant in node names!

Both  the regular serial port and a network connection can be kept alive simul-

taneously; clearly, only one can be used at a time under MS-DOS.  MS-DOS 3.x is

not  required  for Kermit network usage, but most LANS do need DOS 3.1 or later

for conventional file server work.  Kermit needs only the NetBIOS emulator net-

work software.

SET PROMPT

Syntax: SET PROMPT [string]

This  command  allows  you  to  change the MS-DOS Kermit program's prompt.  The

string may be enclosed in curly braces.  Control characters like ESC can be in-

cluded as backslashed numbers like "\27".  ANSI.SYS and similar console drivers

can be programmed through this command.  The  string  must  be  less  than  128

5.6.9. The SET Command                                                 Page 100

-------------------------------------------------------------------------------

characters.    If  the  string is omitted (missing) Kermit's original prompt of

"Kermit-MS>" is restored.

SET RECEIVE

Syntax: SET RECEIVE parameter value

This command lets you modify the ways in which MS-Kermit asks the other  Kermit

to behave.  That is, it controls the file transfer protocol options for packets

sent to MS-Kermit by the other Kermit.  The parameters and values  you  specify

in the SET RECEIVE command are sent to the other Kermit during initial negotia-

tions.

END-OF-LINE number

ASCII value of packet terminator to look for on incoming packets.  Nor-

mally carriage return.  Use this command if the other  Kermit  is  ter-

minating its packets with some other control character.

PACKET-LENGTH number

Ask the remote Kermit to use the specified maximum length  for  packets

that  it  sends to Kermit-MS.  The normal length is 94 bytes.  Use this

command to shorten packets if the communication line is noisy  or  ter-

minal  buffers somewhere along the path are too small.  Shorter packets

decrease the probability that a particular packet  will  be  corrupted,

and will reduce the retransmission overhead when corruption occurs, but

will increase the protocol overhead.

If a length greater than 94 is  specified,  a  protocol  option  called

"long  packets"  will  be used, provided the other Kermit also supports

it.  Kermit-MS handles extended-length packets up to 1000  bytes  long.

Long Packets can improve efficiency by reducing the per-packet overhead

for a file, but they will not be used unless you  issue  this  command.

Before  using  this option, ensure that the equipment on the communica-

tions pathway can absorb a long packet,  and  that  the  connection  is

clean  (retransmission of long packets is expensive!).  You should also

SET BLOCK-CHECK 2 or 3 for more reliable error checking.

PADCHAR number

Ask  the remote Kermit to use the given control character (expressed as

a decimal number 0-31, or 127)  for  interpacket  padding.    Kermit-MS

should never require any padding.

PADDING number

Ask the remote Kermit to insert the given number of padding  characters

before  each  packet it sends.  MS-Kermit never needs padding, but this

mechanism might be required  to  keep  some  intervening  communication

equipment happy.

5.6.9. The SET Command                                                 Page 101

-------------------------------------------------------------------------------

START-OF-PACKET number

If the remote Kermit will be marking the beginning of  packets  with  a

control  character  other  than  Control-A,  use  this  command to tell

Kermit-MS about it (the number should be the decimal ASCII value  of  a

control  character).   This will be necessary only if the hosts or com-

munication equipment involved cannot pass a Control-A through as  data,

or  if some piece of communication equipment is echoing packets back at

you.

TIMEOUT number

Ask  the remote Kermit to time out after the given number of seconds if

a packet expected from Kermit-MS has not arrived.  Use this command  to

change the other Kermit's normal timeout interval.

SET REMOTE

Syntax: SET REMOTE {ON, OFF}

SET  REMOTE  ON removes the file transfer display (as if you had given the com-

mand SET DISPLAY QUIET).  It should be used when you are running  Kermit-MS  in

remote mode when coming in from another PC through the Kermit-MS's "back port",

to which the console has been reassigned using the DOS CTTY command, e.g.

CTTY COM1

It is necessary to issue the SET REMOTE ON command because (a) Kermit-MS has no

way  of  knowing that its console has been redirected, and (b) when the console

is the same as the port, the file transfer display will interfere with the file

transfer  itself.    SET  REMOTE  OFF  returns the file transfer display to its

preferred style (REGULAR or SERIAL).  When you SET REMOTE ON,  you  might  also

want  to  SET  DELAY 5 or thereabouts, to allow yourself time to escape back to

the local system before MS-Kermit starts sending packets.

WARNING: During CTTY console redirection, many programs  still  output  to  the

real  screen  and require input from the real keyboard and will hang the system

until keyboard requests are satisfied.  On the IBM PC, CTTY CON returns control

to  the  normal keyboard and screen (other systems may use other device names).

See section 5.16.4 for further details about remote operation.

SET RETRY

Syntax: SET RETRY number

Sets the number of times a packet is retransmitted before  the  protocol  gives

up.   The number of retries can be between 1 and 63, and is 5 by default.  This

is an especially useful parameter when the communications line is noisy or  the

remote host is very busy.  The initial packet of a file exchange is given three

times as many retries to allow both systems to become ready.

5.6.9. The SET Command                                                 Page 102

-------------------------------------------------------------------------------

SET SEND

Syntax: SET SEND parameter value

The SET SEND command is primarily used to override negotiated protocol options,

or to establish them before they are negotiated.

END-OF-LINE number

ASCII value of packet terminator to put on outbound packets.   Normally

carriage  return.  Use this command if the other Kermit needs its pack-

ets terminated with a nonstandard control character.

PACKET-LENGTH number

Use this as the maximum length for outbound packets, regardless of what

was negotiated with the other Kermit.  Normally,  you  would  use  this

command  only  to  send shorter packets than the other Kermit requests,

because you know something the other Kermit doesn't know, e.g.  there's

a device on the communication path with small buffers.

PADCHAR number

Use the specified control character  for  interpacket  padding.    Some

hosts  may require some padding characters (normally NUL or DEL) before

a packet, and certain front ends or other communication  equipment  may

need  certain  control  characters to put them in the right modes.  The

number is the ASCII decimal value of the padding character, (0 - 31, or

127).

PADDING number

How many copies of the pad character to send before each  packet,  nor-

mally zero.

PAUSE number

How many milliseconds to pause before sending each packet, 0-127,  nor-

mally  zero.    This helps half-duplex systems prepare for reception of

our packet.  Padding characters are sent only after the time limit  ex-

pires.

QUOTE number

Use the indicated printable character for prefixing  (quoting)  control

characters and other prefix characters.  The only reason to change this

would be for sending a very long  file  that  contains  very  many  "#"

characters (the normal control prefix) as data.

START-OF-PACKET number

Mark the beginning of outbound  packets  with  some  control  character

other than Control-A.  This will be necessary if the remote host or the

communication channel cannot accept a  Control-A  as  data,  or  if  it

echoes  back  your  packets.   The remote host must have been given the

corresponding SET RECEIVE START-OF-PACKET command.

TIMEOUT number

Change  Kermit-MS's  normal timeout interval; this command is effective

only if TIMER is set to be ON; it is normally ON so that  both  Kermits

can control timeouts.  When the timer is ON, the default interval is 13

seconds.

5.6.9. The SET Command                                                 Page 103

-------------------------------------------------------------------------------

SET SPEED

Syntax: SET SPEED rate

Same as SET BAUD, q.v.

SET TAKE-ECHO

Syntax: SET TAKE-ECHO {ON, OFF}

Specifies whether screen display should occur during implicit or explicit  TAKE

operations on MSKERMIT.INI or other Kermit-MS command files, and during evalua-

tion of macro definitions by the DO command.  Handy for finding errors in  TAKE

files or macro definitions.

SET TERMINAL

Syntax: SET TERMINAL parameter [value]

This  command  controls  most  aspects  of  terminal  emulation.    Most of the

parameters are only settable (or meaningful) on the  IBM  PC  family  and  com-

patibles.   (Programmers who are proficient on other MS-DOS systems are invited

to fill in these functions for those systems  and  send  the  results  back  to

Columbia.)   On other systems, built-in setup modes or DOS commands can be used

to accomplish the same functions.

The first group of parameters tells which kind of terminal to  emulate.    When

Kermit-MS uses its built-in software for emulation, incoming characters are ex-

amined for screen control commands (escape sequences)  specific  to  that  ter-

minal, and if encountered, the commands are executed on the PC screen.

NONE    Act  as  a  dumb terminal.  All incoming characters will be sent to the

screen "bare", as-is, through DOS.  If you have loaded a device  driver

into DOS for the CON device, such as ANSI.SYS, then that driver will be

able to interpret the codes itself.  Many non-IBM  systems  have  their

own  screen  control  code  interpreter  built into DOS or firmware, or

available as a loadable device driver.

VT52    The DEC VT-52 terminal.

HEATH   The Heath/Zenith-19 terminal (H19), which supports all  the  VT52  com-

mands, plus line and character insert/delete editing functions, an ANSI

mode, and a 25th line.

VT102   The DEC VT102 (ANSI) terminal, which is the same as a  VT100  but  also

supports   line/character  insert/delete  editing  functions  and  ANSI

printer controls.

TEK4010 A Tektronix 4010 graphics terminal.  Currently only available  on  IBM,

TI, and Victor PCs.  On the IBM family, Kermit automatically senses and

adapts to the CGA, EGA, or Hercules color board.

The specific escape sequences supported by Kermit for each  of  these  terminal

types  are  listed in section 5.15.1.  Note that when a Kermit program includes

5.6.9. The SET Command                                                 Page 104

-------------------------------------------------------------------------------

Tektronix emulation, this can be invoked automatically while in character  mode

(VT102, VT52, or Heath emulation) when the emulator receives certain escape se-

quences.  This can be turned off using the DISABLE TEK command.

The remaining SET TERMINAL commands specify setup options for the selected ter-

minal:

CHARACTER-SET {UK, US}

UK displays "#" (ASCII 35, number sign) as a pound  sterling  sign,  US

displays "#" as "#".  This command applies during VT100/102 emulation.

COLOR number [, number [, number]]

Several numbers, applied in left to right sequence, separated by commas

or spaces:

0   Reset  the colors to normal intensity white characters on a black

background and use the "no-snow" mode on the IBM  Color  Graphics

Adapter (CGA).

1   High intensity foreground

10  Request fast screen updating for use on the IBM Mono, EGA, or PGA

(usually sensed and set internally by Kermit), and  some  non-IBM

CGAs.

3x  Foreground color

4x  Background color

where  x is a single digit from 0 to 7, which is the sum of the desired

colors:

1   Red

2   Green

4   Blue

Example: 0, 1, 37, 44 on an IBM CGA would produce bold white characters

on  a blue field with no snow. The snow removal business has to do with

whether the program should synchronize with vertical retrace  when  up-

dating  screen  memory.   This is necessary with certain color adaptors

(like the CGA) and unnecessary for others (like the EGA).

CURSOR-STYLE {BLOCK, UNDERLINE}

Sets  the cursor rendition to your preference.  Note that on some early

IBM PCs and compatibles, the cursor may not be restored correctly after

escaping back from CONNECT because of a bug in the early IBM BIOS.

KEYCLICK {ON, OFF}

Turns electronic keyclick ON or OFF.  If your keyboard has a mechanical

clicker  (as IBM boards do), you may not notice the effect of this com-

mand.

MARGIN-BELL {ON, OFF}

Controls  whether  the  bell  should  be sounded when the cursor passes

column 72 near the right screen margin; wider displays set the  bell  8

columns from the right edge.

5.6.9. The SET Command                                                 Page 105

-------------------------------------------------------------------------------

NEWLINE-MODE {ON, OFF}

ON sends a carriage-return-linefeed combination (CRLF)  when  you  type

carriage  return  (CR)  during  terminal emulation.  OFF (default) just

sends a CR when you type CR.

ROLL {ON, OFF}

ON  unrolls  the screen to the bottom before adding new material if the

screen had been rolled back, e.g. by Ctrl-PgUp.  ROLL OFF (the default)

displays  new  material on the current screen, possibly overwriting old

material.

SCREEN-BACKGROUND {NORMAL, REVERSE}

NORMAL  means  dark  background, light characters.  REVERSE means light

background, dark characters.

TAB {AT n, CLEAR AT n, CLEAR ALL}

Sets  tab  stops or clears one or all tab stops; n is the numeric posi-

tion of the tab to be set or cleared.  By default,  tabs  are  every  8

spaces,  at positions 9, 17, 25, etc.  Only meaningful when emulating a

terminal that has settable  tabs  (the  VT52  doesn't  really  but  the

emulator  can set them anyway).  More than one tabstop may be specified

by separating column numbers with commas, spaces, or tabs.  132 columns

are supported.

WRAP {ON, OFF}

ON automatically breaks screen lines (by inserting a  CRLF)  when  they

reach  the  right  margin.    OFF disables wrapping -- if a line is too

long, the excess characters go off the screen.  WRAP is OFF by default,

since most hosts format lines to fit on your screen.

SET TIMER

Syntax: SET TIMER {ON, OFF}

This command enables or disables the timer that is used during file transfer to

break deadlocks that occur when expected packets do not arrive.    By  default,

the  timer  is  ON.   If the other Kermit is providing timeouts, you can safely

turn the timer OFF to avoid packet collisions that might occur when two  timers

go off simultaneously.

SET TRANSLATION

Syntax: SET TRANSLATION INPUT {ON, OFF, char1 char2}

This  command  provides  multi-language  support  (and  perhaps  other  special

effects) during CONNECT, and during execution of the INPUT, OUTPUT, PAUSE,  and

TRANSMIT  script commands, but not during file transfer or at MS-Kermit command

level.  A character that arrives at the  communication  port  (char1)  will  be

translated  to another character (char2) before display on the screen.  As many

as 256 characters may have translations specified concurrently.    But  to  see

characters  with  ASCII values higher than 127, you must also SET DISPLAY 8 and

SET PARITY NONE.

SET TRANSLATION INPUT ON enables translation (the keyword INPUT is required  to

5.6.9. The SET Command                                                 Page 106

-------------------------------------------------------------------------------

allow  future translation mechanisms).  OFF disables the translation and is the

default.  So even if you have set up a translation table, you must  SET  TRANS-

LATION  INPUT  ON  before  it will take effect.  SHOW TRANSLATION tells whether

translation is OFF or ON, and displays any current table entries.

Translation table entries are made by specifying byte pairs in ASCII or numeric

backslash form:

SET TRANS INPUT \3 \13

converts  incoming  ASCII  ETX characters (decimal 3) to ASCII CR (decimal 13).

8-bit values are allowed, and refer to characters in the "upper  half"  of  the

PC's character set, either the ROM characters supplied with the PC or else sub-

stitutions provided by a special device driver.

A more practical example shows how the user of a German PC could  use  the  SET

TRANSLATION  and  SET KEY commands to make the PC's umlaut-a key (key code 132)

send a left curly brace ("{", ASCII 123), and to display incoming curly  braces

as umlaut-a's:

SET KEY \d132 \d123

SET TRANS INP { \d132

(This  example  applies  to  the IBM PC German keyboard, and assumes the German

keyboard  driver,  KEYBGR,  has  been  loaded.    This  is  usually   done   in

AUTOEXEC.BAT.)

SET WARNING

Syntax: SET WARNING {ON, OFF}

Specify  what  to do when an incoming file is about to be stored under the same

name as an existing file in the target device and directory.    If  ON,  Kermit

will  warn you when an incoming file has the same name as an existing file, and

automatically rename the incoming file (as indicated in the warning message) so

as  not to destroy (overwrite) any existing one.  If OFF, the pre-existing file

is destroyed, even if the incoming file does not arrive completely.  WARNING is

ON by default as a safety measure.

The  new  name  is  formed by adding numbers to the part of the name before the

dot.    For  instance,  ABC.TXT  becomes  ABC00001.TXT,  ABC00001.TXT   becomes

ABC00002.TXT, etc.

The STATUS Command

Syntax: STATUS

The  STATUS  command displays the values of all the current SET options.  There

are currently no operands for the STATUS command.  It always displays  all  the

options, and the display fills the screen.

5.6.9. The SET Command                                                 Page 107

-------------------------------------------------------------------------------

The SHOW Command

Syntax: SHOW option

Most  parameters  that  may  be  altered with SET commands are displayed by the

STATUS command.  The SHOW command is used for displaying macro definitions, key

redefinitions, file transfer statistics, and translations.

SHOW MACROS

displays the definitions of all currently defined macros,  as  well  as

the amount of space left for new macro definitions.

SHOW KEY

allows you to determine a key's identification code and  what  it  will

send in CONNECT mode, most useful for obtaining the identification of a

key when SET KEY commands will be placed in a TAKE FILE.  This  command

can  be  done  only  interactively  (use  a ? to see all defined keys).

Refer to the terminal emulation section for examples.

SHOW MODEM

displays  the  status  of  the  modem signals DSR (dataset ready, modem

tells the PC that it is turned on and in  data  mode),  CTS  (clear  to

send,  modem  grants  the  PC permission to send data), and CD (carrier

detect, local modem tells the PC that it is  connected  to  the  remote

modem).  The results may be misleading if your asynchronous adapter, or

the connector or cable that is attached to it, is  strapped  to  supply

these modem signals itself.

SHOW STATISTICS

displays counts of characters sent and received during file  transfers,

for  both  the  most recent transfer and the entire session, and an es-

timate of the average baud rate while sending and listening.

SHOW TRANSLATION

displays  the  entries in the 256 byte input translation table.  Values

are expressed numerically to avoid  confusion  with  different  display

adapters, and the command shows only entries for which input and output

codes differ.

5.7. SCRIPTS

A script is a file containing Kermit commands to be executed.  It is  the  same

as a TAKE file, in Kermit terminology, but includes INPUT, OUTPUT, PAUSE, ECHO,

and CLEAR commands to automatically detect and respond to  information  flowing

though  the serial port, actions which otherwise would be performed by the user

during CONNECT.  The login sequence of a host computer is a classical example.

The script commands INPUT, OUTPUT, PAUSE, and ECHO may be terminated by  typing

Control-C at the keyboard.

5.7. SCRIPTS                                                           Page 108

-------------------------------------------------------------------------------

The CLEAR command

Syntax: CLEAR

The  CLEAR command flushes the buffers of the serial port to forget any earlier

material to help the INPUT command with a  clean  start.    (This  command  was

called  CLRINP  in 2.29B and earlier, and CLEAR was used to erase macro and key

definition memory).

The INPUT command

Syntax: INPUT [timeout] {search-string, @filespec}

INPUT is the most powerful of the script commands.  It  reads  characters  from

the  serial  port  continuously  until  one  of two things occurs: the received

characters match the search string or the time limit expires.  Matching strings

is the normal use, as in

Kermit-MS>input 5 Login please:

to  recognize  the  phrase "Login please:", or else time out after trying for 5

seconds.  A special binary character \255 or \o377 or \xFF stands for the  com-

bination  carriage return and a line feed, in either order, to simplify pattern

matching.

Beware of characters arriving with parity set because the pattern matching con-

siders  all 8 bits of a byte unless the local parity is other than NONE and SET

DISPLAY is 7-BITS.  Arriving characters are  modified  by  first  removing  the

parity  bit, if parity is other than NONE, then they are passed through the SET

TRANSLATION INPUT converter, the high bit is again suppressed if SET DISPLAY is

7-BITs, the result is logged and stored for pattern matching.

The  INPUT  and  OUTPUT  commands  have  a special syntax to replace the normal

string with text obtained from a file or device:

OUTPUT @filespec

INPUT @filespec

Both forms read one line of text from the file or device  and  use  it  as  the

desired  string.    A common use is to wait for a password prompt and then read

the password from the console  keyboard.    A  string  starts  with  the  first

non-spacing character and ends at either the end of line or, if executed within

a TAKE file, at a semicolon.  Indirectly obtained strings, the @filespec  form,

read  the  first line of the file up to but not including the explicit carriage

return.  Note if a trailing carriage return is  needed  it  must  be  expressed

numerically, such as \13 decimal.

The  behavior  of  the INPUT command can be controlled by SET INPUT parameters:

CASE, TIMEOUT-ACTION, DEFAULT-TIMEOUT, and ECHO.    If  the  TIMEOUT-ACTION  is

PROCEED  then failure to match strings is ignored and the script continues with

the next command.  Otherwise, QUIT causes the current script file to be  exited

immediately;  the  next  higher  level script file or the Kermit prompt is done

next.  CASE controls whether upper and lower case letters should be  considered

the same during matching.  DEFAULT-TIMEOUT supplies a different default for how

long to wait, in case an explicit value is not  given  in  the  INPUT  command.

5.7. SCRIPTS                                                           Page 109

-------------------------------------------------------------------------------

Finally,  ECHO  controls  displaying  of  serial  port data during matching and

during the other script commands.

When a script fails because an INPUT command  did  not  encounter  the  desired

string within the timeout interval, the message is "?Not confirmed".

The OUTPUT command

Syntax: OUTPUT {string, @filespec}

The  OUTPUT command writes the indicated character string to the serial port as

ordinary text.  The string may contain control or other special binary  charac-

ters  by  representing  them  in the backslash numerical form.  Carriage Return

(CR), for example, is \13 decimal, \o15 octal, or \x0D hexadecimal.  The string

may use 8-bit characters if the communications parity is type NONE.

The OUTPUT string may not contain Kermit CONNECT-mode action verbs, but it does

recognize a special code \b or \B, which causes a BREAK  signal  to  be  trans-

mitted.

The  string to be transmitted starts with the first non-spacing character after

the OUTPUT command and ends at either the end of line or, if executed within  a

TAKE file, at a semicolon (if you need to output a semicolon from within a TAKE

file, use backslash notation, e.g. "\59").  Indirectly  obtained  strings,  the

@filespec form, read the first line of the file up to but not including the ex-

plicit carriage return.

As a convenience, text arriving at the serial port during the OUTPUT command is

shown  on the screen if SET INPUT-ECHO is ON, and stored in a 128-byte internal

buffer for rereading by a following INPUT command.

The PAUSE command

Syntax: PAUSE [number]

PAUSE simply waits one or more seconds before Kermit executes the  next  script

command.   Pauses are frequently necessary to avoid overdriving the host and to

let a modem proceed through a dialing sequence without interruptions from  Ker-

mit.   The default waiting time is set by SET INPUT DEFAULT-TIMEOUT and is nor-

mally one second.  The optional integer number selects the number of seconds to

pause  for  this command.  An explicit value of zero produces a pause of just a

few milliseconds which can be useful in some situations.

Text arriving during the  PAUSE  interval  is  shown  on  the  screen,  if  SET

INPUT-ECHO  is  ON, and stored in a 128-byte internal buffer for rereading by a

following INPUT command.

Script Examples

Figure 5-2 shows a simple script file that dials a Hayes modem and logs in to a

computer, prompting the user for her password, and then connects as a terminal.

Notice  the  semicolons used to indicate comments in TAKE files.  If these same

5.7. SCRIPTS                                                           Page 110

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

CLEAR

SET SPEED 1200

OUTPUT AT\13                         ; Wakeup modem.

PAUSE 2                              ; Let it get ready.

INPUT OK                             ; Its response.

OUTPUT ATDT 9,555-1212\13            ; Dial the phone.

INPUT 30 CONNECT                     ;  Wait for connection.

PAUSE 2                              ; Breathing space for modems.

OUTPUT \13                           ; Send carriage returns to

PAUSE 2                              ;  awaken host, wait 2 secs

OUTPUT \13                           ;  between them.

INPUT 15 Please login:               ; Get expected prompt.

OUTPUT Sari\13                       ; Send username and CR.

SET INPUT ECHO OFF                   ; Privacy, please.

INPUT Enter password:                ; Quietly wait for this.

ECHO Type your password now.         ; Make our own prompt.

OUTPUT @CON                          ; Send console keystrokes.

OUTPUT \13                           ; Add a real carriage return

SET INPUT ECHO ON                    ; Go verbose again.

CONNECT                              ; Start terminal emulation

Figure 5-2:   MS-Kermit Script for Dialing Up and Logging In

-------------------------------------------------------------------------------

commands were typed by hand at the Kermit prompt the semicolon  material  would

be  considered  part  of  a string!  Typing a Control-C will interrupt and ter-

minate any of the commands.

The MS-Kermit script package maintains a small memory buffer to allow the INPUT

command to inspect characters received during the previous few script commands.

CLEAR removes them and any others in a lower level main serial port buffer.

As with any TAKE file, scripts can be nested by using the TAKE command within a

script.    If an INPUT command fails to obtain a match and uses the QUIT option

or if the script simply finishes normally the TAKE file is exited  and  control

resumes where it left off in the next higher TAKE file or at the Kermit prompt.

A slightly more complicated combination of DOS Batch and Kermit Script files is

shown in Figures 5-3 and 5-4 (see your DOS manual for  an  explanation  of  the

batch  file  syntax).  The purpose is to allow a user to say "SEND filename" at

the DOS prompt.  The DOS batch shell, SEND.BAT, and the login script,  KX,  are

combined  to  login  to  a  VAX through a data switch, run VMS Kermit in server

mode, transfer the file, submit it to VMS Mail, delete the disk file, shut down

the  server  and  logout  from the VAX, and report the overall transfer status.

The user is asked to provide a password interactively.  Scripts are illustrated

further in the next section.

5.8. Initialization Files Revisited                                    Page 111

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

File SEND.BAT, DOS batch program:

echo off

Rem Kermit, one-line file mailer, by Joe Doupnik.

Rem Logon to VAX, run Kermit, Send user's file,

Rem  post via MAIL, logout from VAX.

if ".%2" == "." goto usage

if exist %1 goto proceed

echo No file to send!

:usage

echo Usage is  SEND filename username

goto done

:proceed

echo Logging onto the Vax ...

kermit set disp q,take kx,send %1,pau,rem host mail %1 %2,pau 2,bye,

if errorlevel 3 goto badrem

if errorlevel 2 goto badrcv

if errorlevel 1 goto badsnd

echo File(s) "%1" has been mailed to %2.

goto done

:badrem

echo Mail did not cooperate!

:badrcv

echo Receive failed!

goto done

:badsnd

echo Send failed!

goto done

:done

echo on

Figure 5-3:   MS-DOS Batch File Invoking Kermit to Send VAX Mail

-------------------------------------------------------------------------------

5.8. Initialization Files Revisited

At  Columbia  University,  we  have  IBM  370-series mainframes running VM/CMS,

DECSYSTEM-20 mainframes running TOPS-20, a VAX 8700 running Unix.  All of these

systems  are  accessible  through a Gandalf PACX port contention unit.  The IBM

systems have two different kinds of front ends, a COMTEN 3695 (similar  to  IBM

3705) for linemode half-duplex connections, and various Series/1-style protocol

converters (including the 7171 and  4994)  for  full-screen,  full-duplex  3270

emulation,  all of which use various combinations of parity and other settings.

Figure 5-5 shows an MSKERMIT.INI file composed by Vace Kundakci of Columbia  to

automate  the  task  of  switching  his  PC/AT among all these systems.  It il-

lustrates the creative use of macros and scripts.  Numerous site-  and  system-

dependent key definitions have been omitted.

A  bit  of explanation might clarify some of this.  PACX is our port contention

unit.  Its output appears in even parity.  It prompts us to  "Enter  node  name

=>",  and  we  respond  with the name of one of our systems or front ends, like

5.8. Initialization Files Revisited                                    Page 112

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

File KX, Kermit script:

Comment Login script for VAXA via Micom data PBX Switch.

set input timeout quit

set input echo off

set display quiet

output \13

comment - "slowly." and "CLASS" are part of the switch's prompt.

input 10 slowly.

input 10 CLASS

pause

comment - Slowly tell switch "vaxa", wait for beep.

output v

output a

output x

output a

output \13

pause

input 5 \7

comment - Done with Switch, wake up the VAX and log in.

pause

output \13

pause

input 5 Username:

set input timeout proceed

output MYNAME\13

input 2 Password:

comment - Prompt ourselves, then get password from console.

echo Enter password:

output @con

comment - Send a carriage return at the end of the password.

output \13

comment - Expect ESC Z from the VAX's Set Term/Inquire...

comment - Respond ESC [ <query symbol> 6 ; 2 c   (say we are VT102).

comment - Note syntax for including question mark and semicolon!

input 15 \27Z

output \27[\{63}6\;2c

comment Look for VMS dollar sign prompt

input 15 $

comment Start VMS Kermit and place it in server mode

output kermit server\13

comment - allow server's message to finish, "machine." appears twice.

input 10 machine.

input 10 machine.

pause

Figure 5-4:   MS-Kermit Script for Logging into VAX and Sending Mail

-------------------------------------------------------------------------------

CU20B or CU20D (DEC-20s), SIMA or SIMB (7171s), CUVMA or CUVMB (IBM  mainframes

front  ended by COMTEN), or CUNIXC (a VAX).  To connect to a given system, Vace

types "do cu20b" or "do sima" to invoke a "connecting" macro.  Each  of  these,

5.8. Initialization Files Revisited                                    Page 113

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

; MSKERMIT.INI for IBM PC/AT Kermit 2.30, by Vace Kundakci

COMMENT - INPUT command defaults for scripts

set inp tim quit

set inp echo off

set inp case observe

COMMENT - Macros for connecting to PACX and selecting various systems

def cu20b do pacx,o cu20b\13,do 2065

def cu20d do pacx,o cu20d\13,do 2065

def sima do pacx,o sima\13,do 7171

def simb do pacx,o simb\13,do 4994

def cunixc do pacx,o cunixc\13,do 8700

def cuvma do pacx,o cuvm\13,do 3695,o vma\13,do 3083

def cuvmb do pacx,o cuvm\13,do 3695,o vmb\13,do 3083

COMMENT - Macros for logging in to various systems.

def pacx cle,set par e,o \13,i 5 Enter node name =>\32,pau

def 3695 i 5 SWITCHING CHARACTERS:\32\32

def 3083 i 5 ONLINE,o L\32,do pwd,do vml,c

def 8700 i 5 login:\32,do pwd,do dec,c

def 2065 i 5 \13\10\64,o ter vt102\13,do pwd,do dec,c

def 7171 pau,cle,o \13,i 5 TERMINAL TYPE:\32,o vt-100\13,do 3270

def 4994 pau,cle,o \13,i 5 terminal type:\32,pau,o vt100\13,do 3270

def 3270 pau,cle,o \13,o L\32,do pwd,do xed,c

COMMENT - Macros for communicating with various systems

def vml set par m,set k \270 \8,set k \3 \Kbreak,do tty

def xed set par e,set k \270 \8,set k \3,do def

def dec set par n,set k \270 \127,set k \3,do def

def def set tim of,set loc of,set ter wr of,set han non,set flo xon

def tty set tim on,set loc on,set ter wr on,set han xon,set flo non

COMMENT - Macro for obtaining user ID and password

def pwd echo user:,o @con,o \13, do pw2

def pw2 echo Password:,o @con,o \13

Figure 5-5:   An Advanced MS-Kermit Initialization File

-------------------------------------------------------------------------------

in  turn,  invokes  the  PACX macro to navigate through the PACX to the desired

system, and then invokes the appropriate macro (3695, 7171, etc)  to  get  past

any  associated  front  end  (e.g.  to  tell  the COMTEN which IBM mainframe is

wanted, or to tell the protocol converter what terminal to emulate),  and  then

to  login  on  the  desired  system,  prompting  on  the screen for user ID and

password.  Finally, a macro like "vml" (VM linemode),  "xed"  (XEDIT,  i.e.  VM

full  screen),  or  "dec"  (DEC-20 or VAX) is executed to set the communication

parameters for the system just logged in to.   The  key  definitions  that  are

shown  in  the "vml", "xed", and "dec" macros assign the host's character dele-

tion code (backspace or rubout) to the AT's backarrow key.

5.9. MS-Kermit Features for Different Systems                          Page 114

-------------------------------------------------------------------------------

5.9. MS-Kermit Features for Different Systems

As noted early on, MS-Kermit was designed primarily for the IBM PC family,  and

later  adapted  to various non-IBM-compatible MS-DOS (and even non-MS-DOS) sys-

tems.  Some of these adaptations provide all the features of the  IBM  PC  ver-

sion,  others  provide only a subset, and still others may include features not

available on the IBM family.  These features are all  of  the  system-dependent

variety; the Kermit file transfer protocol should be implemented identically on

all versions of MS-Kermit.  The most obvious differences are  in  the  terminal

emulation  options  and  the keyboards.  Table 5-5 shows the terminal emulation

options for the systems presently supported by Kermit-MS, and Table 5-6,  shows

which  keys  are  used  for screen rollback on the various systems supported by

MS-Kermit.

-------------------------------------------------------------------------------

System         EscChar   Capabilities Terminal Service

ACT Apricot      ^]            K      VT52 ???

DEC Rainbow      ^]      R   P K D    VT102 firmware

DECmate/DOS      ^]            K      VT100

Generic DOS      ^]            K      Depends on system

Grid Compass     ^]            K      ???

HP-110           ^]            K      Dumb terminal

HP-150           ^]      R     K      HP-2623 firmware

IBM PC family    ^]      R M P K D    H19,VT52,VT102,Tek emulation

Intel 3xx        ^]            K      Uses real terminal

NEC APC3         ^]      R M P K D    H19,VT52,VT102 emulation

NEC APC          ^]      R   P K      VT100, ADM3A firmware

Olivetti M24     ^]      R M P K D    Same as IBM PC

Sanyo MBC55x     ^]      R M P K D    H19,VT52,VT102 emulation

Wang PC          ^A            K      Wang firmware

TI Pro           ^]        M P K      VT100/Tektronix

Victor 9000      Alt-]     M P K D    H19,VT52,VT102 and/or Tek4010

Zenith Z100      ^]            K      Heath-19 emulation

R=Rollback, M=Modeline, P=Printer control, K=Key redefinition, D=screen Dump

Table 5-5:   Kermit-MS Terminal Emulation Options

-------------------------------------------------------------------------------

Another difference is the default communication port, the number of  communica-

tion  ports  supported,  and the names given to them.  For instance, the IBM PC

family supports COM1 and COM2, and uses COM1 by default.  MS-Kermit may be per-

suaded  to  support higher-numbered IBM ports using the method outlined in sec-

tion 5.16.3.  For remote operation, IBM's name for the console is  CON,  so  if

you CTTY COM1, you do CTTY CON to put the PC back to normal.

5.9. MS-Kermit Features for Different Systems                          Page 115

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

System    Screen Down   Line Down       Screen Up   Line Up

IBM PC    PgUp          Ctrl-PgUp       PgDn        Ctrl-PgDn

Rainbow   PrevScreen    Ctrl-PrevScreen NextScreen  Ctrl-NextScreen

HP-150    Prev          Shift-UpArrow   Next        Shift-DownArrow

NEC APC   Uparrow       Ctrl-UpArrow    DownArrow   Ctrl-DownArrow

NEC APC3  PgUp          Ctrl-PgUp       PgDn        Ctrl-PgDn

Sanyo 55x PgUp          Ctrl-RtArrow    PgDn        Ctrl-PgDn

The IBM PC also allows use of the Home key to get to the  top  of  its  display

memory and End key to get to the bottom, and the keypad minus (-) key to toggle

the mode line on and off.  The Rainbow uses Shift-Next-Screen  to  get  to  the

bottom  of  its  display memory, but provides no key for moving directly to the

top.

Table 5-6:   Kermit-MS Screen Scroll Keys

-------------------------------------------------------------------------------

The DEC Rainbow

The  DEC  Rainbow  version  of  MS-Kermit 2.30 uses the built-in VT102 terminal

firmware and setup modes, and can operate at speeds up to 9600 baud.  It has no

25th  screen  line,  and therefore no Kermit mode line during CONNECT.  It sup-

ports only the Rainbow's single communication port, and not the  printer  port,

so  SET  PORT for the Rainbow is not implemented (but of course the printer may

be used for printing.)  The Rainbow may be put in remote mode by CTTY AUX,  and

returned  to  normal with CTTY SCRN.  The Rainbow supports several SET TERMINAL

commands: VT102, VT52, and ROLL.

The keypad and cursor keys all work properly in VT102 and VT52 modes and in ap-

plication  as  well  as  native  states  (they never had in previous versions).

Newline mode is activated for received characters (LF ==> CR/LF).  Screen  roll

back  is  almost 11 screenfuls.  Table 5-7 shows the verb names and default key

assignments for the Rainbow.  On the main typewriter keyboard the shifted comma

and period are converted to special keys available for Set Key assignment with-

out impacting the normal unshifted ASCII actions; Shift Lock has no  effect  on

these keys.

The DECmate II

MS-Kermit for the DECmate II with the XPU option is somewhat similar to Rainbow

Kermit.  It uses built-in terminal VT100 firmware  and  setup  modes  and  baud

rates  up  to  9600  on the single communication port.  The printer port is not

available for communications in this version.  There is no mode line, but other

connect-mode escapes are supported, including sending BREAK.  Disks A through I

are supported, and the floppy disk format is compatible with the Rainbow.   DEC

utilities are available for file conversion between DOS and WPS-8 files.

5.9. MS-Kermit Features for Different Systems                          Page 116

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Rainbow Key           Verb Name      Operation

PF1                   \Kpf1,\Kgold   Keypad function key

PF2..PF4              \Kpf2..\Kpf4   Keypad function keys

keypad 0..9           \Kkp0..\Kkp9   Keypad digit keys

keypad -              \Kkpminus      Keypad minus key

keypad ,              \Kkpcoma       Keypad commma

keypad .              \Kkpdot        Keypad dot (period) key

keypad Enter          \Kkpenter      Keypad Enter key

up arrow              \Kuparr        Cursor keys

down arrow            \Kdnarr

left arrow            \Klfarr

right arrow           \Krtarr

Shift Prev Screen     \Khome         Rewind to start of screen buffer

Shift Next Screen     \Kend          Unwind to end of screen buffer

Ctrl Prev screen      \Kupone        Backup one screen line

Ctrl Next screen      \Kdnone        Advance one screen line

Prev screen           \Kupscn        Backup one screen

Next screen           \Kdnscn        Advance one screen

Print Screen          \Kprtscr       Copy screen to printer

Ctrl Print Screen     \Ktoggle_prn   Toggle echoing screen to printer

(printer failure resets toggle)

Do                    \Kdump         Copy screen to file (KERMIT.SCN)

Break                 \Kbreak        Send a BREAK

Shift Break           \Klbreak       Send a Long BREAK

Main Screen           \KDOS          Push to DOS

Help                  \Khelp         Show Connect mode help menu

Exit                  \Kexit         Exit Connect mode

*                     \Knull         send a null out the serial port

*                     \Khangup       hangup phone by dropping DTR, RTS

*                     \Klogon        resume logging, if active

*                     \Klogof        suspend logging

*                     \Kstatus       display status table

* (verbs not pre-assigned to keys)

Table 5-7:   Kermit-MS Verbs for the DEC Rainbow

-------------------------------------------------------------------------------

The NEC APC3

The NEC APC3 version of MS-Kermit assumes that the ANSI.SYS driver has been in-

stalled and that a color monitor is being used; the color  graphics  option  is

not  used  by  Kermit.  Although the display should be entirely sensible with a

monochrome system, it has not been tested.  Differences from the IBM PC version

include:

SET BAUD: The useful baud rates supported range from 300 to 9600.

5.9. MS-Kermit Features for Different Systems                          Page 117

-------------------------------------------------------------------------------

SET  PORT:  The  available  ports  are 1, 2, 3, or their equivalents AUX, AUX2,

AUX3.

SET TERMINAL COLOR: Instead of specifying colors by  number,  the  words  BLUE,

RED, MAGENTA, GREEN, CYAN, YELLOW, or WHITE are appropriate.  This is the color

of the text in connect mode; background colors are not available.    Monochrome

monitors will respond with display changing from most dim to most bright if the

colors are specified in the order given.

SET TERMINAL KEYCLICK: Not implemented in Kermit; use the NEC provided command.

SET TERMINAL SCREEN-BACKGROUND: Not implemented.

During terminal emulation, screen scroll is handled by the PgUp and PgDn  keys.

If  used  in combination with the Ctrl key, the display moves but one line.  If

used in combination with the Fnc key, the display scrolls to  the  end  of  the

buffer.    The  Fnc-INS  combination toggles the mode line on/off.  The Fnc-DEL

combination toggles the terminal emulation type.    The  Fnc-Break  combination

resets  the emulator.  The Help key pulls down the connect mode menu.  The ANSI

escape sequence for disable/enable cursor is implemented.

5.10. Compatibility with Older Versions of MS-DOS Kermit

The last monolithic (single source file) release of  MS-DOS  Kermit  was  1.20.

Meanwhile, implementations based on versions of that vintage will have at least

the following incompatibilies from the version described here:

- "RECEIVE filespec" is used instead of "GET filespec".   There  is  no

GET  command  in older versions, and no way to specify a new name for

an incoming file.

- No LOCAL or REMOTE commands.

- No 8th-bit prefixing, repeat counts, CRCs or 2-character checksums.

- No TAKE or initialization files.

- No command macros or command line arguments.

- No terminal session logging.

and others, depending on the specific version.

Incompatibilities between 2.29 and 2.30 include:

- LOCAL command has been removed from 2.30.

- CLEAR command now means clear serial port buffer rather than key  and

macro definitions.  Key and macro definition string space is now gar-

bage collected, so a CLEAR command for them is no longer necessary.

- CLRINP command is gone (replaced by CLEAR).

- Numbers of the form \nnn default to decimal rather than octal.

- Status of Default Disk is now shown as default disk and path.

- LOG  filespec  replaced  by  LOG  SESSION  filespec  and  LOG  PACKET

filespec.

- SET  KEY  and SHOW KEY commands use different key identifications and

syntax:

MS-Kermit no longer understands keycap names such as F1 and  BACKSPACE  because

the  codes are now highly dependent on individual keyboards, software, and com-

puters.  Also, not every key press  combination  is  supported  by  the  system

5.10. Compatibility with Older Versions of MS-DOS Kermit               Page 118

-------------------------------------------------------------------------------

software  and  key  codes do depend on the keyboard in use.  Thus, the SHOW KEY

command is normally used to obtain codes for keys on  your  system.    In  most

cases,  defining one key also redefines all other keys sending the same charac-

ter.  This is a side effect of  not  knowing  the  physical  details  of  every

keyboard.    However,  efforts  have been made to recognize many such "aliased"

keys and to generate unique identifications for each. Special keys, such as F1,

F2  and others which do not send an ASCII code are usually unique and are iden-

tified by scan codes.

Previous versions of MS Kermit used a different key coding  algorithm  and  not

all  old  codes  map to the expected keys.  However, Kermit does attempt to use

the older SET KEY syntax properly as much as possible.  The  older  syntax  re-

quired  the  keyword SCAN followed by a number WITHOUT the BACKSLASH.  The cur-

rent MS Kermit uses decimal as the default number base  and  previous  versions

used  octal  in  certain commands.  So, when Kermit senses an old style SET KEY

command it converts the number, displays the new format  and  gives  a  warning

message.  It is best to make a new style SET KEY file.

5.11. What's Missing

Kermit-MS  has  plenty of room for improvement.  Missing features (which may be

added in future releases) include:

- Sliding windows.

- Attribute packets.

- Login  script  elaboration  within  the  Kermit   command   structure

(variables, conditional branching, etc).

- Substitutible parameters for TAKE and DO commands.

- Default filetype for TAKE command files.

- DIAL  command,  telephone  directory,  explicit  support  for various

modems.

- A way to play back session logs directly from disk to screen.

- Trapping of carrier loss.

- Pause at end of screen during local TYPE.

- Piped operation a la UNIX (e.g. "compress foo.bar | kermit send").

- Transaction file logging.

- A way to accept default values for omitted trailing  fields  in  com-

mands.

- A better built-in help facility.

- Simple Control and Meta key reassignment.

- Separation  of  STATUS output into several screens for terminal, com-

munication, and protocol parameters.

- Specification of character sequences having special meaning  to  com-

munications "black boxes" which use ASCII characters for control pur-

poses.  Byte stuffing or character doubling may be required.

Future releases of MS-Kermit will probably have major portions of  the  program

(now  written  entirely  in assembler) replaced by C-language code.  This would

include the file transfer portions, the command parser, etc.

5.12. Installation of Kermit-MS                                        Page 119

-------------------------------------------------------------------------------

5.12. Installation of Kermit-MS

If you already have Kermit on your PC, you can use it to obtain new versions of

Kermit-MS  when  they appear on the central system at your site.  If you do not

have Kermit or any other reliable file capture facility on your PC, you can or-

der  a  Kermit  diskette  from Columbia (write to Kermit Distribution, Columbia

University Center for Computing Activities, 612 West 115th Street, New York, NY

10025,  USA,  for  information), or from any of a number of user groups or dis-

kette services.  If you absolutely can't get a Kermit diskette, but have access

to  another  computer  that has a copy of the MS-DOS Kermit program (usually in

".BOO" format, explained below), there are two recommended methods for  getting

it onto your PC:

1. Use another file capture facility to get it.

2. Type  in and run the "baby Kermit" program (72 lines) from chapter 7

of the Kermit book.

The first method involves either "raw capture" (no error checking), or else use

of  (gasp!)  another  protocol,  such as Xmodem, which, like Kermit, requires a

program to execute the same protocol on both ends of the connection.

Raw capture generally involves "typing" the file on the  other  computer,  with

your  PC  taking the place of the terminal, and rather than displaying the file

on the screen as it's being typed, your PC is storing it on the disk.  This  is

a  tricky  process, however, because data can easily be lost or corrupted.  For

instance, you could write a very short BASIC program to capture a file in  this

way,  but  it  could probably not keep up -- even at low baud rates -- with the

transmission speed unless you included the tricky serial port  BASIC  commands.

The  DOS  command COPY COM1 filename command has the same speed problem, and it

stops only when it receives a Control-Z character from the other computer.

If the other computer has Kermit on it -- which is likely, since this is  prob-

ably  the  reason you want to get Kermit onto your PC -- you should type in the

receive-only BASIC Kermit program listed on pp.186-188 of the Kermit book,  and

then  use  it  in  conjunction with the other computer's Kermit to transfer the

file.  Make sure to set a long enough delay on the other computer to give your-

self  time to escape back to the PC and start up the "baby Kermit" before pack-

ets start to arrive, otherwise you'll probably get fatal DOS i/o errors.

Note that  Kermit  programs  are  often  distributed  under  names  other  than

"Kermit".    The  Columbia  Kermit  program library contains hundreds of Kermit

programs, which must be given unique names.  MS-DOS Kermit for the IBM PC,  for

instance,  is  called MSVIBM.BOO.  Once you have this program in .EXE format on

your disk, you probably should rename it to KERMIT.EXE, because it's harder  to

remember (and type) the distribution name.

You  will  probably  also  want  to create an MS-Kermit initialization file.  A

sample is distributed with MS-Kermit as MSVIBM.INI.  This should be tailored to

your  requirements,  and  then renamed to MSKERMIT.INI, and stored where Kermit

can find it (according to the DOS PATH and APPEND commands).

Finally, if you run Kermit often, you might want to have your AUTOEXEC.BAT file

copy  Kermit, the initialization file, and a copy of COMMAND.COM to a RAM disk.

The procedure is explained in section 5.5.

5.12. Installation of Kermit-MS                                        Page 120

-------------------------------------------------------------------------------

".BOO Files"

MS-Kermit (and many other Kermit programs) are often distributed using  a  spe-

cial encoding called "boo" (short for "bootstrap") format, developed especially

for distribution of MS-Kermit over networks and communication lines.  MS-Kermit

has  grown  to  have  so  many features that the binary program image (the .EXE

file) has become quite large.  But binary files are  generally  not  compatible

with  the  common  labeled  tape formats (e.g. ANSI D), electronic mail, or raw

downloading -- the methods most commonly used for Kermit distribution.

A common practice is to encode .EXE  and  other  binary  files  into  printable

characters,  such  as hexadecimal digits, for transportability.  A simple "hex"

encoding results in two characters per 8-bit  binary  byte,  plus  CRLFs  added

every  80  (or  less)  hex  characters  to  allow  the  file  to  pass  through

card-oriented links.  A hex file is therefore more than twice as large  as  the

original binary file.

A  .BOO file is a more compact, but somewhat more complicated, encoding.  Every

three binary bytes (24 bits) are split up into four 6-bit bytes with 48  (ASCII

character  "0")  added to each, resulting in four ASCII characters ranging from

"0" (ASCII 48) to "o" (ASCII 111), with CRLFs added at  or  near  "column  76".

The  resulting file size would therefore be about 4/3 the .EXE file size.  This

is still quite large, so .BOO  files  also  compress  consecutive  null  (zero)

bytes.    Up to 78 consecutive nulls are compressed into two characters.  Tilde

("~") is the null-compression lead-in, and the  following  character  indicates

how many nulls are represented (subtract 48 from this character's ASCII value).

For instance "~A" means 17 consecutive nulls; "~~" means 78 of them.   Repeated

nulls are very common in .EXE files.

4-for-3 encoding combined with null compression reduces the size of the encoded

file to approximately the same size as the original .EXE  file,  and  sometimes

even smaller.  The first line of a .BOO file is the name (in plain text) of the

original file.  Here's what the first few lines of a  typical  .BOO  file  look

like:

MSVIBM.EXE

CEYP0Id05@0P~3oomo2Y01FWeP8@007P000040HB4001`W~28bL005\W~2JBP00722V0ZHPYP:

\8:H2]R2V0[`PYP:68>H2S23V0YHPiP:Xg800;Qd~2UWD006Yg~2Ogl009]o~2L8000;20~~~~

~~~~~~~:R2H008TV?P761T410<H6@P40j4l6RRH0083l17@PP?`1M@?YSP20o0Ee0nUD0h3l

1WD3jO@3]0VjW03=8L?X4`N0o01h1\H6~20l>0i7n0o1]e7[@2\PO=8LH60@00Raj>04^97Xh0

Programs for Handling .BOO Files

Kermit Distribution includes several useful .BOO-file programs:

MSBPCT.BAS      This  Microsoft BASIC program can be used on any PC to decode a

.BOO file into an .EXE file.  It's about 50 lines line,  so  it

can be typed in.

MSBPCT.BOO      BASIC  programs  run  rather slowly, so .BOO-file decoders have

also  been  written  in  high-level  languages  like   C.   The

MSBPCT.EXE  file that was produced by compiling MSBPCT.C is en-

coded  into  MSBPCT.BOO,  which  can  be  decoded   back   into

MSBPCT.EXE  using MSBPCT.BAS.  Once you've done that, you don't

need to run the slow BASIC version any more, which is a  bless-

5.12. Installation of Kermit-MS                                        Page 121

-------------------------------------------------------------------------------

ing,  because  the  MS-Kermit .BOO file takes more than half an

hour to decode using the BASIC version, but only seconds  using

MSBPCT.EXE.

MSBPCT.*        There  are  .BOO-file  decoders written in other languages too,

like assembler, Turbo Pascal, etc.  Take your pick.   They  all

do the same thing.

MSBMKB.*        This is the program for encoding an .EXE file into a .BOO file.

It is written in C, compiled, and translated (by  itself)  into

.BOO  format,  suitable for decoding back into .EXE form by any

of the MSBPCT programs.

MSBHEX.* are programs for producing and decoding straight hex files.

5.13. Program Organization

Kermit-MS version 2 is composed of separate assembler source  files,  assembled

separately, and linked together.  The modules are:

System/Device Independent:

MSSKER.ASM      Main program

MSSSEN.ASM      File sender

MSSRCV.ASM      File receiver

MSSSER.ASM      Server operation

MSSFIL.ASM      File i/o

MSSCMD.ASM      Command parser

MSSTER.ASM      CONNECT command

MSSCOM.ASM      Packet reader and sender

MSSSET.ASM      SET, SHOW, and STATUS commands

MSSSCP.ASM      Script CLEAR, ECHO, INPUT, OUTPUT, PAUSE, TRANSMIT commands

MSSFIN.ASM      Dummy  module  to  find  the  end  of the data segment; must be

linked LAST.

MSSDEF.H        Data structure definitions and equates

System/Device Dependent:

MSGxxx.ASM      System-dependent graphics terminal for system xxx

MSUxxx.ASM      System-dependent keyboard translator for system xxx

MSXxxx.ASM      System-dependent code for system xxx

MSYxxx.ASM      Terminal emulation for system xxx

MSZxxx.ASM      More terminal emulation for system xxx

The xxx is replaced by a 3-letter code for the particular system, e.g. IBM  for

the IBM PC family, RB1 for the Rainbow-100, etc.

The  modular  organization  allows  easier modification of the program, quicker

transfer of modified portions from system-to-system.  The modules are  designed

to  be  well-defined and self-contained, such that they can be easily replaced.

For instance, someone who prefers windows and mice to typing commands should be

able  to  replace  the command parsing module without having to worry about the

effect on the other modules.

To assemble any of the Kermit modules, file MSSDEF.H must  be  on  the  default

5.13. Program Organization                                             Page 122

-------------------------------------------------------------------------------

disk.

All  the  Kermit  implementations  require  the modules MSSCMD, MSSCOM, MSSFIL,

MSSKER, MSSRCV, MSSSCP, MSSSEN, MSSSER, MSSSET, MSSTER, MSSFIN.  MSSFIN must be

linked last.

Each  particular  implementation requires at least an MSXxxx module, usually an

MSUxxx module, and, if it is doing terminal  emulation  in  software,  also  an

MSYxxx and possible also an MSZxxx module, and for graphics terminal emulation,

also an MSGxxx module.  See the batch or make files from the  source  distribu-

tion  for  details  of  exactly which modules are required for a particular im-

plementation.

Once all the required object modules exist, they  may  be  linked  together  to

produce a Kermit program.  For example, on the IBM PC:

A>link

Microsoft Object Linker V2.00

(C) Copyright 1982 by Microsoft Inc.

Object Modules [.OBJ]: msscmd+msscom+mssfil+mssker+mssrcv+mssscp+msssen+

mssser+mssset+msster+msgibm+msuibm+msxibm+msyibm+mszibm+mssfin

Run File [MSSCMD.EXE]: kermit

List File [NUL.MAP]:;

A>

5.14. Bringing Kermit to New Systems

You  can bring Kermit-MS to MS-DOS systems that are not explicitly supported in

one of two ways -- attempt to run the "generic" MS-DOS Kermit on it, or add ex-

plicit code to support your system.

To  get  started with Kermit on a new system, try running "generic" MS-DOS Ker-

mit; in many cases, it will run as is.  The generic  version  accomplishes  all

its port and console i/o through DOS calls, and during terminal connection does

not attempt to emulate any particular kind of terminal.   In  some  cases,  the

generic version may still require some fiddling to run on a new system; for in-

stance, different systems refer to their communication ports in different  ways

--  COM1,  J1,  AUX,  etc.  The SET PORT command allows you to specify the port

using any of these device names, or using DOS file handles -- keep trying until

you find the one that works.  Generic MS-DOS Kermit will probably run no faster

than 1200 baud, and it only works with DOS 2.0 or later.

If you want to write code to explicitly support a new  system,  first  call  or

write Kermit Distribution at Columbia to make sure no one else is already doing

the same work.  If you're the first, then begin by reading the file MSXAAA.DOC,

provided  with the MS-DOS Kermit sources in the Kermit distribution, which is a

guide to the system dependent modules of Kermit-MS.  Then create new MSUxxx.ASM

and  MSXxxx.ASM  modules, and, if your version is also doing terminal emulation

in software, also an MSY and possibly an MSZ module patterned after those  that

have been written for other systems.

5.14. Bringing Kermit to New Systems                                   Page 123

-------------------------------------------------------------------------------

5.15. Kermit-MS VT102 Terminal Emulator Technical Summary

5.15.1. Treatment of Inbound Characters During Terminal Emulation

Many  things  can  happen to a character that arrives at the communication port

before you see it, depending on all the factors mentioned above.    The  actual

sequence of events is:

1. Obtain character from serial port.

2. Remove high bit if parity is other than none.

3. Detect and remove xon/xoff if FLOW is XON/XOFF.

4. If DEBUG is active (ON or SESSION) then put character to debug style

display, otherwise:

5. If transparent printing is active (for VT102 emulators)  then  print

the character but do not show it on the display, otherwise:

6. Remove high-order bit if DISPLAY is 7-bit

7. Translate  if TRANSLATE INPUT is ON, but skip this step if an escape

sequence is in progress

8. If LOG SESSION is active then copy character to the log file

9. Pass the character to the terminal emulator  for  interpretation  or

display.

The  following  sections  summarize the Kermit-MS keyboard and screen operation

during emulation of H19, VT52, and VT102 terminals, prinicipally for the IBM PC

but  also  used by the NEC APC3, Victor 9000, and Sanyo 55x systems.  Note that

spaces shown between characters of escape sequences are there for ease of read-

ing.  The actual sequences contain no spaces.

5.15.1. Treatment of Inbound Characters During Terminal Emulation      Page 124

-------------------------------------------------------------------------------

5.15.2. Keyboard Layout and Characters Sent

Here  is  how  the  keypad  functions are assigned to the IBM keyboard function

keys.  You may change them by using the SET KEY command to define a desired key

as  the appropriate Kermit action verb; use SET KEY without a definition to un-

define a key.  Names of appropriate verbs are also shown for use in the Set Key

command, such as

Set Key \2352 \Kbreak           (IBM Alt-B assigned to verb BREAK)

Verb  names  are  system  dependent, use ? in the Set Key definition part for a

list of local verbs.  IBM PC verbs are listed in Table 5-4; IBM key values  are

either  straight  ASCII or the IBM Bios scan code, plus 256, plus 512 for Shift

key held down, plus 1024 for Control key held down, plus 2048 for Alt key  held

down; non-ASCII keys are always 256 decimal or greater.  Keys particular to the

Enhanced Keyboard have 4096 added to the result.

---------------------------------------------------------------------------

Heath-19 and VT52 Keypads                 VT102 keypad

IBM Keys                                 IBM keys

+------+------+-------+----------+   +------+------+------+------+

| Blue |  Red |  Grey | up arrow |   |  PF1 |  PF2 |  PF3 |  PF4 |

|  F1  |  F2  |  F3   | up arrow |   |  F1  |  F2  |  F3  |  F4  |

+------+------+-------+----------+   +------+------+------+------+

|  7   |  8   |  9    |down arrow|   |  7   |  8   |  9   |  -   |

|  F5  |  F6  |  F7   |down arrow|   |  F5  |  F6  |  F7  |  F8  |

+------+------+-------+----------+   +------+------+------+------+

|  4   |  5   |  6    | rgt arrow|   |  4   |  5   |  6   |  ,   |

|  F9  |  F10 |  SF1  | rgt arrow|   |  F9  |  F10 |  SF1 |  SF2 |

+------+------+-------+----------+   +------+------+------+------+

|  1   |  2   |  3    |left arrow|   |  1   |  2   |  3   |  E   |

|  SF3 |  SF4 |  SF5  |left arrow|   |  SF3 |  SF4 |  SF5 |  n  S|

+------+------+-------+----------+   +------+------+------+  t  F|

|  0------0   |  .    |  Enter   |   |  0------0   |  .   |  e  6|

|  SF7        |  SF8  |  SF6     |   |   SF7       |  SF8 |  r   |

+-------------+-------+----------+   +-------------+------+------+

SF1 means push Shift and F1 keys simultaneously

---------------------------------------------------------------------------

CURSOR KEYS:

H-19 & VT52           VT102

VT52/H19 key   IBM Verb  IBM key       All Modes      Numeric   Application

up arrow       UPARR    up arrow       ESC A          ESC [ A   ESC O A

down arrow     DNARR    down arrow     ESC B          ESC [ B   ESC O B

right arrow    RTARR    right arrow    ESC C          ESC [ C   ESC O C

left arrow     LFARR    left arrow     ESC D          ESC [ D   ESC O D

AUXILIARY KEYPAD:

Heath-19 & VT52         VT102

VT52/H19 key   IBM Verb  IBM key     Numeric Applic.   Numeric   Applic.

PF1/HF7/Blue   GOLD,PF1   F1         ESC P   ESC P     ESC O P   ESC O P

PF2/HF8/Red    PF2        F2         ESC Q   ESC Q     ESC O Q   ESC O Q

5.15.2. Keyboard Layout and Characters Sent                            Page 125

-------------------------------------------------------------------------------

PF3/HF9/Grey   PF3        F3         ESC R   ESC R     ESC O R   ESC O R

PF4/HF1        PF4        F4         ESC S   ESC S     ESC O S   ESC O S

0              KP0        SF7        0       ESC ? p   0         ESC O p

1              KP1        SF3        1       ESC ? q   1         ESC O q

2              KP2        SF4        2       ESC ? r   2         ESC O r

3              KP3        SF5        3       ESC ? s   3         ESC O s

4              KP4        F9         4       ESC ? t   4         ESC O t

5              KP5        F10        5       ESC ? u   5         ESC O u

6              KP6        SF1        6       ESC ? v   6         ESC O v

7              KP7        F5         7       ESC ? w   7         ESC O w

8              KP8        F6         8       ESC ? x   8         ESC O x

9              KP9        F7         9       ESC ? y   9         ESC O y

comma  (,)     KPCOMA     SF2        ,       ESC ? l   ,         ESC O l

minus  (-)     KPMINUS    F8         -       ESC ? m   -         ESC O m

period (.)     KPDOT      SF8        .       ESC ? n   .         ESC O n

Enter          KPENTER    SF6        ^M(cr)  ESC ? M   ^M        ESC O M

(SFn means hold down Shift key while pressing Function key n.)

An often confusing item is knowing the mode of the auxillary keypad: numeric or

application.    Digital  Equipment  Corporation designed the terminal to change

modes only under command from the remote computer  and  not  at  all  from  the

keyboard.  So the startup state is numeric/cursor mode, and reception of escape

sequences "ESC [ ? 1 h" or "l" changes the mode.  Kermit verbs for  the  keypad

and  cursor  keys generate the correct escape sequences appropriate to the cur-

rent mode and terminal type.

A best attempt is made to safely test for the 101/102 key Enhanced keyboard and

use it if present.  If it is present then the keyboard translator separates the

individual arrow keys from those on the numeric keypad and also  separates  the

asterisk  and  forward  slash  keys  on  the  keypad  from those on the regular

typewriter keyboard.  These special Enhanced keyboard keys are reported as scan

codes with 4096 added to the base scan code.

OTHER IBM KEYS OPERATIONAL IN CONNECT MODE:

IBM key         IBM Verb      Action

Keypad Del                    Send ASCII Del code (rubout) \127

Backspace (<-)                Send ASCII Del code (rubout) \127  (BS is \8)

Keypad -        MODELINE      Toggle mode line on/off (only if Mode Line is

enabled and not used by the host).

Alt -           TERMTYPE      Toggle among H-19, VT52, and VT100 emulations.

Alt =           RESET         Clear screen and reset terminal emulator to

starting (setup) state.

Alt B           BREAK         Send a BREAK signal

Alt H           HELP          Show drop down help menu (detailed below)

Alt S           STATUS        Show settings

Alt X           EXIT          Exit Connect mode, back to Kermit prompt

Home            HOMSCN        Roll screen up (text down) to beginning of

storage.

End             ENDSCN        Roll screen down (text up) to end of storage.

PgUp            UPSCN         Roll screen up (back, earlier) one screen.

PgDn            DNSCN         Roll screen down (forward, later) one screen.

Ctrl-PgUp       UPONE         Roll screen up one line.

5.15.2. Keyboard Layout and Characters Sent                            Page 126

-------------------------------------------------------------------------------

Ctrl-PdDn       DNONE         Roll screen down one line.

Control PrtSc   PRTSCN        Toggle  on/off copying of received text to

printer, "PRN" shows on far right of mode

line when activated.

Control-End     DUMP          Dump image of screen to a disk file or device.

Default filename is KERMIT.SCN in the current

directory. Use command SET DUMP to change the

filename.  Screen images are appended to the

file, separated by formfeeds.

Shift-PrtSc     Standard DOS Print-screen, dump screen image to printer.

"Alt  -"  means  hold  down  Alt  and  type  minus on the upper key rank.  This

switches among the various kinds of emulation but does not change most  operat-

ing parameters of the emulator.

CONNECT ESCAPE COMMANDS:

Type the Kermit escape character (normally "^]"), then one of the keys below:

(equivalent IBM Verb)

?   display this short list.                                HELP

0   send a null character.                                  NULL

B   send a BREAK signal.                                    BREAK

C   close connect session & return to Kermit prompt.        EXIT

F   dump screen to filespec, default is KERMIT.SCN.         DUMP

H   hangup the phone or network connection                  HANGUP

L   send a Long BREAK signal                                LBREAK

M   toggle mode line on/off.                                MODELINE

P   push to DOS.                                            DOS

Q   quit (suspend) logging.                                 LOGOFF

R   resume logging.                                         LOGON

S   show status.                                            STATUS

Kermit escape character itself: send it to the host.

5.15.3. Responses To Characters Received By the Terminal Emulator

Unknown  escape  sequences  of the form "ESC char" are absorbed by the emulator

without further effect; longer unknown escape sequences echo the extra  charac-

ters.

DEC  VT102 functions while in ANSI (VT102) mode, unsupported features marked by

an asterisk (*):

Escape Seq     Mnemonic       Description of Action

ESC D          IND            Index, moves cursor down one line, can scroll

ESC E          NEL            Move cursor to start of line below, can scroll

ESC H          HTS            Set one horizontal tab at current position

ESC M          RI             Reverse Index, cursor up one line, can scroll

ESC Z          DECID          Identify terminal (response is ESC [ ? 6 c)

ESC c          RIS            Reset terminal to initial state

ESC =          DECKPAM        Enter keypad application mode

5.15.3. Responses To Characters Received By the Terminal Emulator      Page 127

-------------------------------------------------------------------------------

ESC >          DECKNPNM       Enter keypad numeric mode

ESC 7          DECSC          Save cursor position and attributes

ESC 8          DECRC          Restore cursor from previously saved position

ESC # 3        DECDHL         Double height and width line, top half

ESC # 4        DECDHL         Double height and width line, bottom half

ESC # 5        DECSWL         Single height and width line

ESC # 6        DECDWL         Double width single height line

ESC # 8        DECALN         Test screen alignment, fill screen with E's

ESC [ Pn @     ICH            ANSI insert Pn spaces at and after cursor

ESC [ Pn A     CUU            Cursor up Pn lines, does not scroll

ESC [ Pn B     CUD            Cursor down Pn lines, does not scroll

ESC [ Pn C     CUF            Cursor forward, stays on same line

ESC [ Pn D     CUB            Cursor backward, stays on same line

ESC [ Pn; Pn H CUP            Set cursor to row, column (same as HVP)

ESC [ Ps J     ED             Erase in display:

0 = cursor to end of screen, inclusive

1 = start of screen to cursor, inclusive

2 = entire screen, reset lines to single

width, cursor does not move.

ESC [ Ps K     EL             Erase in line:

0 = cursor to end of line, inclusive

1 = start of line to cursor, inclusive

2 = entire line, cursor does not move

ESC [ Pn L     IL             Insert Pn lines preceding current line.

ESC [ Pn M     DL             Delete Pn lines from current downward, incl.

ESC [ Pn P     DCH            Delete Pn chars from cursor to left, incl.

ESC [ Pn; Pn R CPR            Cursor report (row, column), sent by terminal

Example: home position yields ESC [ 1; 1 R

ESC [ Pn c     DA             Device attributes (reports ESC [ ? 6 ; 2 c)

ESC [ Pn; Pn f HVP            Set cursor to row, column (same as CUP)

ESC [ Ps g     TBC            Tabs clear, 0 = at this position, 3 = all

ESC [ 4 h      IRM            Insert mode on

ESC [ 20 h     LNM            Set newline mode (cr => cr/lf)

ESC [ 4 l      IRM            Replacement mode on

ESC [ 20 l     LNM            Reset newline mode (cr => cr)

ESC [ ? Ps;...;Ps h  SM       Set mode, see table below

ESC [ ? Ps;...;Ps l  RM       Reset mode, see table below

Ps     Mnemonic       Mode           Set (h)        Reset (l)

0                     error (ignored)

1      DECCKM         cursor keys    application    cursor/numeric

2      DECANM         ANSI/VT52      ANSI/VT102     VT52

3      DECCOLM        Columns        +132 col       80 col

4      DECSCLM       *Scrolling      smooth         jump

5      DECSCNM        Screen         reverse video  normal

6      DECOM          Origin         relative       absolute

7      DECAWM         Autowrap       on             off

8      DECARM        *Autorepeat     on             off

9      DECINLM       *Interlace      on             off

18     DECPFF         Printer termination character, use FF if set

19     DECPEX         Printer extent,set=screen,off=scrolling region

38     n/a            Graphics (Tek) ++graphics     text

+ See comments on EGA boards.

++ Ignored if DISABLE TEK has been given.

ESC [ Pn i     MC             Printer controls (Media Copy)

0                      Print whole Screen

4                      Exit printer controller (transparent print)

5.15.3. Responses To Characters Received By the Terminal Emulator      Page 128

-------------------------------------------------------------------------------

5                      Enter printer controller (transparent print)

ESC [ ? Pn i   MC             Printer controls (Media Copy)

1                      Print line containing cursor

4                      Exit auto print (stop echoing to printer)

5                      Enter autoprint (echo screen chars to printer)

ESC [ Ps;...;Ps m  SGR        Select graphic rendition

0 = all attributes off (#'s 1, 4, 5, 7)

1 = bold, intensify foreground

4 = underscore (reverse video on IBM CGA)

5 = blink

7 = reverse video

non-DEC extensions: 30-37 = foreground color = 30 + colors

40-47 = background color = 40 + colors

colors: 1 = red, 2 = green, 4 = blue

ESC [ Ps n     DSR            Device Status Report.

Response from VT100: 0=ready, 3=malfunction.

Command to VT100: 5=report status with DSR,

6=report cursor position using CPR sequence.

ESC [ Ps;...;Ps q  DECLL      Load LEDs, Ps = 0 means clear LED #1-4

Ps = 1,2,3,4 sets LED # 1,2,3,4 on status line.

ESC [ Pn; Pn r DECSTBM        Set top and bottom scrolling margins, resp.

ESC [ r resets margin to full screen.

ESC [ sol x    DECREQTPARM    Request terminal parameters, see table below

ESC [ sol; par; nbits; xspeed; rspeed; clkmul; flags x

DECREPTPARM    Reports terminal parameters

sol = 0  request; terminal can send unsolicited

reports - supported as sol = 1 below.

sol = 1, request; term reports only on request

sol = 2, this is a report (DECREPTPARM)

sol = 3, terminal reporting only on request

par = 1 none, 2 space, 3 mark, 4 odd, 5 even

nbits = 1 (8 bits/char), 2 (7 bits/char)

xspeed,rspeed = transmit & receive speed index

0,8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128 correspond to speeds of

50,75,110,134.5,150,200,300,600,1200,1800,2000,2400,3600,4800,9600,19200,

and 38400 baud.              clkmul = 1 (clock rate multiplier is 16)

flags = 0-15 (Setup Block #5), always 0 here

ESC [ 2; Ps y  DECST          *Confidence tests - not supported

SCS            Select character sets.

ESC ( A        SCS            G0 points to UK symbols

ESC ) A        SCS            G1 points to UK symbols

ESC ( B        SCS            G0 points to ASCII symbols

ESC ) B        SCS            G1 points to ASCII symbols

ESC ( 0        SCS            G0 points to special (line drawing) graphics

ESC ) 0        SCS            G1 points to special (line drawing) graphics

ESC ( 1        SCS            G0 points to alt char ROM - UK symbols

ESC ) 1        SCS            G1 points to alt char ROM - UK symbols

ESC ( 2        SCS            G0 points to alt graphics ROM - as ESC ( 0

ESC ) 2        SCS            G1 points to alt graphics ROM - as ESC ) 0

(Separate graphics used for DEC and Heath)

^E             ENQ           *Answerback message (not supported)

^G             BELL           Sound VT102 style beep

^H             BS             Backspace, move cursor left one character

^I             HT             Horizontal tab, move cursor to next tabstop

^J             LF             Linefeed, move cursor down one line

^K             VT             Vertical Tab, treated as a line feed

5.15.3. Responses To Characters Received By the Terminal Emulator      Page 129

-------------------------------------------------------------------------------

^L             FF             Formfeed, treated as a line feed

^M             CR             Carriage return, move cursor to col 1

^N             SO             Select usage of G1 character set

^O             SI             Select usage of G0 character set

^X             CAN            Cancel escape sequence in progress

^Z             SUB            Treated as a CAN

Other extensions:

ESC [ 25; Pc f                VT52/VT100 move cursor to 25th line.

ESC [ 25; Pc H                VT52/VT100 move cursor to 25th line.

(These will disable Kermit's own status line.)

ESC * char                    VT200 series graphics command, ignored.

5.15.4. DEC VT102 Functions While in VT52 Mode

Escape sequence               Description of action

ESC A                         Cursor up

ESC B                         Cursor down

ESC C                         Cursor right

ESC D                         Cursor left

ESC F                         Enter graphics mode

ESC G                         Exit graphics mode

ESC H                         Cursor home

ESC I                         Reverse line feed

ESC J                         Erase to end of screen

ESC K                         Erase to end of line

ESC V                         Print cursor line

ESC X                         Exit Printer Controller mode, transparent print

ESC Y row column              Direct cursor address, offset from space

ESC W                         Enter Printer Controller mode,transparent print

ESC Z                         Identify (response is ESC / Z)

ESC ^ (caret)                 Enter autoprint mode (printer echoes screen)

ESC _ (underscore)            Exit autoprint mode

ESC ]                         Print Screen

ESC =                         Enter alternate keypad mode

ESC >                         Exit alternate keypad mode

ESC <                         Enter ANSI mode (changes to VT102)

5.15.5. Heath-19 Functions While in Non-ANSI Mode

Escape seq     Mnemonic       Description of action

ESC A          HCUU           Cursor Up

ESC B          HCUD           Cursor Down

ESC C          HCUF           Cursor Forward, stays on same line

ESC D          HCUB           Cursor Backward, stays on same line

ESC E          HCD            Clear display

ESC F          HEGM           Enter Graphics mode

ESC G          HXGM           Exit Graphic mode

ESC H          HCUH           Cursor Home

ESC I          HRI            Reverse Index

ESC J          HEOP           Erase to end of page

ESC K          HEOL           Erase to end of line

5.15.5. Heath-19 Functions While in Non-ANSI Mode                      Page 130

-------------------------------------------------------------------------------

ESC L          HIL            Insert line

ESC M          HDL            Delete line

ESC N          HDCH           Delete character

ESC O          HERM           Exit Insert Char mode

ESC Y row col  HDCA           Direct cursor addressing, offset from space

ESC Z          HID            Identify (response is ESC / K  which is a VT52)

ESC b          HBD            Erase Beginning of display

ESC j          HSCP           Save cursor position

ESC k          HRCP           Set cursor to saved position

ESC l          HEL            Erase entire line

ESC n          HCPR           Cursor Position Report request

ESC o          HEBL           Erase beginning of line

ESC p          HERV           Enter Reverse Video mode

ESC q          HXRV           Exit Reverse Video mode

ESC r Bn       HMBR           *Modify baud rate - not supported

ESC t          HEKS           *Enter Keypad shifted mode, not supported

ESC u          HXKS           *Exit Keypad shifted mode, not supported

ESC v          HEWA           Wrap around at end of line

ESC w          HXWA           Discard at end of line

ESC x Ps       HSM            Set Mode. See table below

ESC y Ps       HRM            Reset Mode. See table below

Ps     Mnemonic       Mode            Set (x)        Reset (y)

1      HSM/HRM        25th line       enabled       +disabled

2                    *keyclick        off            on

3                    *holdscreen      enabled        disabled

4                     cursor type     block          underline

5                     cursor on/off   on             off

6                    *keypad-shifted  shifted        unshifted

7                     alt app keypad  enabled        disabled

8                    *linefeed        lf=>cr/lf      lf=>lf

9                     newline mode    cr=>cr/lf      cr=>cr

+ disabling the 25th line also clears it

ESC z          HRAM           Reset to power-up configuration

ESC =          HAKM           Enter Alternate Keypad mode

ESC >          HXAM           Exit Alternate Keypad mode

ESC <          HEAM           Enter ANSI mode (ESC [ stuff)

ESC @          HEIM           Enter Insert Char mode

ESC [          HEHS          *Enter Hold Screen mode, not supported

ESC \          HXHS          *Exit Hold Screen mode, not supported

ESC { and }    HEK, HDK      *Keyboard enable/disable, not supported

ESC ]          HX25          *Transmit 25th line, not supported

ESC #          HXMP          *Transmit page, not supported

5.15.6. Heath-19 Functions While in ANSI Mode

Escape Seq     Mnenonic       Description of Action

ESC [ s        PSCP           Save cursor position & attributes

ESC [ u        PRCP           Restore cursor position & attributes

ESC [ z        PRAM           Reset to power-up configuration

ESC [ 2 J      ED             Erase entire screen but do not move cursor;

regular Heath-19 moves cursor to Home.

ESC [ ? 2 h    PEHM           Revert to normal Heath-19 non-ANSI mode

5.15.6. Heath-19 Functions While in ANSI Mode                          Page 131

-------------------------------------------------------------------------------

ESC [ > Ps h   SM             Same as ESC x Ps

ESC [ > Ps l   RM             Same as ESC y Ps

Plus most of the ANSI escape sequences listed for the VT102.

5.15.7. Tektronix 4010/4014 Graphics Terminal Functions

MS-Kermit's Tektronix 4010 emulator responds to ordinary text, several  special

control  codes  (for  drawing lines and dots), and several escape sequences, as

shown in Table 5-8.  The commands SET DEBUG and SET TRANSLATION INPUT  are  ef-

fective in Tek mode.

-------------------------------------------------------------------------------

Control Code                  Action

FS,  Control-\ Backslash      draw dots

GS,  Control-] Right square bracket   draw lines

RS,  Control-^ Caret          Draw dots incrementally

US,  Control-_ Underscore     Display text

BEL, Control-G                Beep, make a noise

BS,  Control-H                Backspace, destructive delete

HT,  Control-I                Tab, convert to single space

LF,  Control-J                Line feed, go down one line

VT,  Control-K                Move up one text line

FF,  Control-L                Clears the screen

CR,  Control-M                Carriage return, start of line

CAN, Control-X                Exit Tek sub-mode, or ignore

DEL, RUBOUT                   Delete code, same as BS

Escape Sequence                       Action

ESC Control-E                 Send a status report

ESC Control-L                 Clear the screen

ESC Control-Z                 Activate crosshairs (GIN mode)

ESC Z                         Send terminal identification

ESC ` (accent grave)                  Use solid lines in drawing

ESC a through ESC e                   Use dashed line patterns:

a=fine dots, b=short dashes

c=dash dot,  d=long dash dot

e=dash dot dot.

ESC [ ? 3 8 l                 Exit Tek mode (become text terminal, VT102 etc)

ESC [ ? 3 8 h                 Enter Tek mode (from VT102 mode)

Table 5-8:   Response of MS-Kermit Tektronix Emulator to Received Characters

-------------------------------------------------------------------------------

In  the  table,  US  is the name for the ASCII character Control-Underscore, 31

decimal.  Text is written starting with the last drawn point  being  the  lower

left  corner  of  the first 8 by 8 character cell.  The drawing position is up-

dated by 8 dots to the right for each character, and lines wrap  at  column  80

(column  90  for Hercules boards).  If text extends "below the screen" the sign

"More >" is shown at the bottom right corner and the user needs to press a  key

to  continue.    Then the screen will be cleared and the new text will start at

5.15.7. Tektronix 4010/4014 Graphics Terminal Functions                Page 132

-------------------------------------------------------------------------------

the top of the screen (no scrolling is done in graphics mode).  A real Tek 4010

begins  new text at column 40 and will overwrite dots from older material.  The

high resolution EGA screen and the Hercules screen will hold 43 lines, the  CGA

and  Monochome screens hold 25 lines, and the AT&T screen holds 50 lines.  Her-

cules screens are 90  characters  wide  and  others  are  80  characters  wide.

Monochrome  systems  lack graphics so the text is the normal hardware character

font placed at the nearest  normal  80x25  location  (similarly,  "drawing"  on

Monochrome  systems  is  achieved  by  using a text plus ("+") sign where a dot

would appear).  Text mode is interrupted  by  the  drawing  commands  discussed

below.

Drawing commands GS, FS, RS:

1. Draw a line or move to a point: GS <xy xy ... xy>

GS  is  the  name for ASCII character Control-] (right square bracket), decimal

29.  <xy> stands for an encoded x,y coordinate as explained below.  One or more

x,y  coordinates may follow GS and line segments are drawn from point to point.

The first point is reached without drawing so that GS and the initial <xy> is a

simple  "move-to"  command  rather than a "draw-to" command.  Lines may be con-

structed from six dash patterns described in Table 5-8.  <xy>  coordinates  are

encoded by separating the 10 bit value of x and of y into 5 bit components, ad-

ding an ASCII space (32) to each to form a printable character, and then adding

two high bits to each to identify which component is being represented: high-y,

low-y, high-x, or low-x.  They are transmitted in that order,  with  the  low-x

byte  always  sent  last.   In fact, bytes may be omitted if they do not change

from point to point, provided that low-x is always sent.    These  bytes  range

from  ASCII  space  (32 decimal) to ASCII DEL (127 decimal).  Details are given

below, and summarized in Table 5-10.  This mode completes when a new command or

CRLF  (carriage  return,  line  feed)  arrives;  escape sequences are processed

transparently but other control codes are ignored.  The interrupting  character

is accepted and processed next.

2. Draw dots at given locations: FS <xy xy ... xy>

FS is the name for the ASCII character Control-\ (backslash), decimal 28.  <xy>

is in the same form as above.  A dot is drawn at each x,y  point.    This  mode

completes  when a new command or a CRLF character arrives; escape sequences are

processed transparently but other control codes are ignored.  The  interrupting

character is accepted and processed next.

3.  Draw  dots  from the current location: RS <pen> <direction> <direction> ...

<direction>

RS is the name for the ASCII character Control-^ (caret), decimal 30.   pen  is

the  character  Space (32 decimal) to move without drawing or P (80 decimal) to

draw while moving.  <direction> is one of the letters A, E, D, F, B, J, H, I as

shown in Table 5-9.

Example:  RS  P  J J J (no spaces here, naturally) means draw three dots in the

southwest direction, stepping to each in turn.  This mode completes when a  new

command  or  a non-<pen> or non-<direction> character arrives; the interrupting

character is accepted and processed next.

5.15.7. Tektronix 4010/4014 Graphics Terminal Functions                Page 133

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

<direction>  Move One Tek Dot This Way

A       East (right)

E       East and North         F   D   E

D       North (up)

F       North and West         B   *   A     (* is current location)

B       West

J       South and West         J   H   I

H       South

I       South and East

Table 5-9:   Tektronix Dot-Drawing Commands

-------------------------------------------------------------------------------

Graphics INput (GIN) mode:

Graphics input mode is entered when ESC Control-Z is received.  A crosshair  is

drawn  on  the screen and may be moved by the numeric keypad arrows (fine scale

motion) or the Shift key and these arrows (coarse scale motion).   Pressing  an

ASCII-producing key sends the position of the crosshairs to the host as the se-

quence of: pressed key, X  coordinate,  Y  coordinate,  carriage  return,  then

removes the crosshairs, and then returns to text mode.  The coordinates are en-

coded as for a GS command, but with the Kind bits omitted (see Table 5-10).  If

the host echos the report Kermit tries to absorb it without comment; otherwise,

the text might be shown starting at the crosshairs.  One may  prematurely  exit

GIN  mode  by typing Control-C or Control-Break.  Shift-PrtSc (DOS screen dump)

remains active, however.

The six byte report string might be echoed by the host so the emulator attempts

to  read  the  next  six  incoming bytes and suppress them while they match the

report. Normally, the host is prepared to receive the report and it should turn

off echoing during the process.

Status or Position Report:

ESCAPE  Control-E  requests  a  status report from the emulator.  Tek terminals

have many sub-fields.  Kermit-MS sends a byte of 24 hex for being in text  mode

or  20  hex  otherwise, followed by the encoded X then Y coordinates and a car-

riage return.  Coordinates are encoded 5 bits at a time similar to the GS  com-

mand except the Kind bits are omitted (see Table 5-10).

Identification Report:

ESCAPE  Z  requests  terminal identification, as for VT52 and VT102.  Currently

this report is the 10 character sequence IBM_TEK ESCAPE / Z (no spaces).

5.15.7. Tektronix 4010/4014 Graphics Terminal Functions                Page 134

-------------------------------------------------------------------------------

Screen Capturing:

Kermit does not implement a graphics screen capture facility.  There  are  many

such  Terminate-and-Stay-Resident  (TSR)  programs  in  circulation,  as either

public domain offerings or parts of commercial  packages  (Paint  programs  and

even GRAPHICS.COM from DOS).  High resolution EGA screens require more than the

GRAPHICS.COM program.  MS Windows tells the program (Kermit-MS) the  system  is

using a pure text-only monochrome adapter so dots are shown as plus signs.

Although  Kermit  cannot  save  graphics  screens  directly  (e.g.  via the ^]F

connect-mode command), the received Tektronix escape  sequences  can  still  be

logged to a PC file using the LOG SESSION command.  The resulting log cannot be

"played back" directly on the PC, but it can be transferred to the host and run

through  Kermit's Tek emulator again, just like a character-mode Kermit session

log.

VGA Modes:

Considerable  effort  went  into  ensuring  the  graphics  display  would  work

automatically and not damage monitors.  Thus, Kermit-MS safely tests the active

display adapter for its kind and capabilities before  starting  graphics  mode.

Recent  EGA+  display  boards  are capable of the 640 by 480 scan-line 16-color

"VGA" mode which is now available on IBM PS/2 computers.  The Tek emulator will

happily  run  with  480  scan lines, but: there is no easy and safe way to test

systems if they fully support it, and the normal 256KB of video memory  is  in-

sufficient  to  save a full graphics image.  So activating these higher resolu-

tion modes is left to individuals to  modify  three  simple  constants  in  the

source  code  of  file  MSGIBM.ASM and experiment with their own system.  Other

systems can be added by individuals in the same manner.

Coordinate Encoding:

Coordinate 0,0 is the lower left corner and the X axis is the  horizontal  one.

Tektronix  positions  are mapped into the typically 640 dots wide by 200 or 350

dots high PC screen and thus adjacent Tek  positions  may  yield  the  same  PC

screen dot.

4010-like  devices use positions from 0 to 1023 for both X and Y, although only

0 to 779 are visible for Y due to screen geometry.  The Tek screen is 10.24  by

7.80 inches and coordinates are sent as 1-4 characters.

4014-like devices use positions 0 to 4095, but each movement is a multiple of 4

positions unless the high-resolution LSBXY are sent.  This makes it  compatible

with the 4010 in that a full sized plot fills the screen.  The emulator accepts

the LSBXY components but does not use them.

The various modes are summarized in Table 5-10, in which the following notation

is used:

HIX, HIY = High order 5 bits of a 10 or 12 bit position.

LOX, LOY = Middle order 5 bits of position (low order of Tek 4010).

LSBXY = Low order 2 bits of X + low order 2 bits of Y (4014 mode),

recognized by the Tek emulator but not used to calculate position.

5.15.7. Tektronix 4010/4014 Graphics Terminal Functions                Page 135

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Hi Y    Lo Y   Hi X    LSBXY  Characters Sent (Lo-X Always Sent)

Same    Same   Same    Same                          Lo-X

Same    Same   Same    Diff         LSB, Lo-Y,       Lo-X   4014

Same    Same   Diff    Same              Lo-Y, Hi-X, Lo-X

Same    Same   Diff    Diff         LSB, Lo-Y, Hi-X, Lo-X   4014

Same    Diff   Same    Same              Lo-Y,       Lo-X

Same    Diff   Same    Diff         LSB, Lo-Y,       Lo-X   4014

Same    Diff   Diff    Same              Lo-Y, Hi-X, Lo-X

Same    Diff   Diff    Diff         LSB, Lo-Y, Hi-X, Lo-X   4014

Diff    Same   Same    Same   Hi-Y,                  Lo-X

Diff    Same   Same    Diff   Hi-Y, LSB, Lo-Y,       Lo-X   4014

Diff    Same   Diff    Same   Hi-Y,      Lo-Y, Hi-X, Lo-X

Diff    Same   Diff    Diff   Hi-Y, LSB, Lo-Y, Hi-X, Lo-X   4014

Diff    Diff   Same    Same   Hi-Y,      Lo-Y,       Lo-X

Diff    Diff   Same    Diff   Hi-Y, LSB, Lo-Y,       Lo-X   4014

Diff    Diff   Diff    Same   Hi-y,      Lo-Y, Hi-X, Lo-X

Diff    Diff   Diff    Diff   Hi-y, LSB, Lo-Y, Hi-X, Lo-X   4014

Kind code for byte:           20h   60h  60h   20h   40h

(transmitted left to right)

Table 5-10:   MS-Kermit Tektronix Coordinate Interpretation

-------------------------------------------------------------------------------

Note that LO-Y must be sent if HI-X has changed so that the Tektronix knows the

HI-X byte (in the range of 20h-3Fh) is HI-X and not HI-Y.  LO-Y  must  also  be

sent  if LSBXY has changed, so that the 4010 will ignore LSBXY and accept LO-Y.

The LSBXY byte is

60h + (MARGIN * 10h) + (LSBY * 4) + LSBX

MARGIN is 0 here and refers to splitting the screen left  and  right  for  text

rollover, which the Kermit Tek emulator does not do.

Tek 4010 Example:

Suppose  <xy>  is  point y = 300, x = 500 in Tektronix coordinates.  Split each

10-bit coordinate into 5-bit groups, add ASCII Space (32) and add the Kind code

to each.  Send the X part last.

HI-Y     LO-Y                   HI-X     LO-X

Y=300d=012Ch=  01001    01100   X=500d=01F4h=  01111    10100

+32d=+20h   +10000   +10000     +32d=+20h   +10000  +100100

+Kind code +100000 +1100000     +kind code +100000 +1000000

Binary        111001  1111100                 111111  1100100

ASCII            9       |                       ?       d

So  <xy> = (500,300) is sent or received in a GS command as "9|?d".  An example

in C (program fragments):

5.15.7. Tektronix 4010/4014 Graphics Terminal Functions                Page 136

-------------------------------------------------------------------------------

#define GS   29

#define US   31

FILE *fp;                      /* File descriptor for terminal */

. . .

fputc( GS, fp); coord( 75, 65);       /* Move to 75,65      */

fputc( US, fp); fputs("A House", fp); /* Annotate at 75,65  */

fputc( GS, fp);                       /* Now draw lines...  */

coord( 50, 50); coord(300, 50);       /* Bottom side        */

coord(300,200); coord( 50,200);       /* Right wall, top    */

coord(175,250); coord(300,200);       /* Roof               */

fputc( GS, fp);                       /* Start a new line   */

coord( 50, 50); coord( 50,200);       /* Left wall at 50,50 */

. . .

coord(x, y) int x, y; {        /* Send x,y coordinates to Tek 4010 */

fputc((y / 32) + 32, fp);             /* High y */

fputc((y % 32) + 96, fp);             /* Low  y */

fputc((x / 32) + 32, fp);             /* High x */

fputc((x % 32) + 64, fp);             /* Low  x */

}

5.16. IBM PC Kermit Technical Summaries

Under  normal  circumstances,  MS-Kermit  takes  advantage  of  the  computer's

hardware,  and often bypasses DOS (sometimes even BIOS) to achieve high perfor-

mance, to exercise special machine features, or to produce an attractive screen

display.  Thus, it is not in all respects a "well behaved" DOS program.

MS-Kermit  redirects interrupts 0BH (COM2/4) or 0CH (COM1/3), 16H, 23H, 24H and

returns them when done.  It uses the BIOS for keyboard, video display, and sys-

tem  information  interrupts.   It examines segment 40H for EGA operating modes

and it does direct screen reads and writes.  Memory for the  screen  roll  back

buffer is negotiated with DOS to leave room for a second copy of COMMAND.COM to

run tasks within Kermit; about 90KB to 128KB is needed for the entire  program.

Video page zero is normally used, but page one is employed to save screens with

non-standard dimensions.  Hercules and other graphics mode  displays  are  sup-

ported only in Tektronix terminal mode.  Kermit's timing delays are dynamically

adjusted each time the serial port is started to accomodate  machines  of  dif-

ferent speeds; duration of the normal software timing loop is measured with the

hardware timer chip and looping is adjusted to produce uniform delays  on  8088

through 80386 machines.

5.16.1. Kermit-MS/IBM on Local Area Networks

The IBM version of Kermit-MS has support for the IBM Local Area Network NetBIOS

(and emulators) interface, Interrupt 5CH, with additional support for  selected

vendor  specific  features  (presently just AT&T STARLAN), activated by the SET

PORT NET command, described above.  Communications across a LAN  occur  through

the  NetBIOS interface using virtual circuits (Sessions), named nodes, and con-

ventional NetBIOS packets.  Kermit-MS  does  not  use  LAN  terminal  interface

packages nor the Redirector or similar functions.

Kermit  LAN  operations  are  harmonious  with normal network activity and many

5.16.1. Kermit-MS/IBM on Local Area Networks                           Page 137

-------------------------------------------------------------------------------

pairs of Kermits can communicate simultaneously. Kermit does  not  use  conven-

tional LAN File Server functions.

Kermit uses the standard NetBIOS interrupt 5CH interface, for those who need to

know such things, which means it will run on most LANS including  IBM  PC  Net,

IBM Token Ring, AT&T STARLAN, and many others, and will run with Novell NetWare

software.  Presently, Kermit knows some details of STARLAN and is able to  send

a  BREAK  across  the  net and can use ISN node names with long path parts.  If

STARLAN is not operating these features are not available.   As  more  detailed

information  becomes  available  special  features  of  other  networks  can be

built-in.

The sequence of operations is similar for a client or server Kermit.   The  SET

PORT  NET command is issued by both. This command causes Kermit to validate the

presence of the Interrupt 5CH interface, test for vendor additions, test for  a

session  already underway, establish and display a unique Kermit node name, but

not make a network session.  The node name of the remote server machine follows

the  word  NET;  this  is  not  to be confused with our own node name discussed

below.

If an earlier LAN session is still active then the  current  remote  node  name

field  of  the  command  is examined for presence of a name. If a name is given

then Kermit asks the user whether to RESUME the session or  start  a  NEW  one.

Starting a new one results in Kermit hanging up the old session (HANGUP) before

proceeding; resuming an old one requires no further work at this point.

When Kermit attaches to the network for the first time it  needs  to  select  a

unique  local  node  name so that two systems can form a Session by using these

names as addresses. Kermit uses a simple algorithm to  make  the  name.  Kermit

probes the network adapter board/software for the name of the local system.  If

the name is present Kermit makes its own name by appending a dot K (.K) to  the

local name. If the local name is absent then Kermit first tries a standard name

of "mskermit.K"; should the network report that the name is not unique (another

node  is  using the name) then the user is asked to choose a name. This process

continues until a unique name is obtained or the user decides  to  quit.    The

final  Kermit  node name is reported on the screen; client Kermits will need to

know the name of the server Kermit.

Communication across the LAN begins differently for client and server  Kermits.

The  server  must  be started first, by simply placing a Kermit in server mode.

This results in a network Listen request being posted so that arriving  packets

with  the  correct  node  name  can  be delivered to the server Kermit. Next, a

client Kermit tries to connect to the server by issuing a Kermit server command

to  the  proper node name (as given in the client's SET PORT NET node command);

REMOTE WHO is a satisfactory choice. The client machine actually issues a  net-

work  Call  to  the server's node name to make a connection and then follows it

with data packets holding the Kermit server request. The  initial  exchange  of

packets  establishes a particular virtual circuit between the two nodes. If the

connection cannot be started then the client Kermit reports this  fact  to  the

user. The most common causes of a failure at this point are:

1. The  client  Kermit  did  not specify the correct server Kermit node

name (spelling errors, wrong case for letters, missing dot K),

2. One or both machines are using a network adapter board which is  not

the first in the machine; Kermit uses only the first board,

5.16.1. Kermit-MS/IBM on Local Area Networks                           Page 138

-------------------------------------------------------------------------------

3. The LAN NetBIOS emulator does not fully support IBM standard virtual

circuits,

4. The server machine was not started on the network before the client.

A virtual circuit will be broken if a sender or receiver gets no response to  a

request  within  a  short  time  interval  set  by  the  LAN hardware/software.

However, the LAN procedures within Kermit automatically reestablish the circuit

transparently  to  the user when new information is communicated; the last used

remote node name is remembered internally for this purpose.   This  also  means

the  server Kermit will respond to a connection from a new client Kermit if the

first client is idle for say a minute or so. A session can be terminated by the

user  by issuing the HANGUP command or by exiting Kermit. A session will not be

broken this way if the user on the client Kermit changes to  a  regular  serial

port.

Finally,  when Kermit returns control to DOS, but not via the PUSH command, its

unique Kermit node name is removed from the network adapter board.

During network communications Kermit uses network packets holding 256 bytes  of

data.  If both Kermits are given the command

SET RECEIVE PACKET 1000

then  the  network  and Kermit will be used to best efficiency.  Experience has

shown that the client Kermit should have its TIMER OFF because the  server  may

be  asked  to do an operation via DOS which does not complete before the client

side would timeout. An observation of some  token  passing  networks  indicates

that  Kermit  packets slightly longer than 256, 512, etc bytes result in marked

slowing down because the remaining small piece is not sent until  a  net  timer

expires.    Carrier  sense (Ethernet, STARLAN) boards seem to be more agressive

and export small packets immediately.

Kermit can access files on the LAN file server via DOS even while using the LAN

as  a communications medium.  Network administrators should note this point be-

cause a user operating Kermit in Server mode can allow his or her  file  server

directories to be available to other network users also running Kermit, without

additional security checking of the other users.  The network drives visible to

the Server Kermit can become devices available for Kermit-to-Kermit file trans-

fers, etc, unless the DISABLE command is used to confine access to the  current

disk  and  directory.  A corollary is when files are accessible to DOS commands

they can become public.

5.16.2. Use of Kermit-MS with External Device Drivers

It is often desirable to supplement or modify the behavior of a DOS program  by

loading  it with special external device drivers.  These drivers may operate at

either the DOS or BIOS level.   When  Kermit-MS  accesses  the  BIOS  directly,

DOS-level drivers are ineffective.  When Kermit accesses the hardware directly,

both the DOS and the BIOS level drivers are locked  out.    Kermit-MS  provides

several mechanisms to allow these external drivers to operate as intended.

Here are a few examples:

- IBM's  ANSI.SYS  console driver operates at the DOS level.  It allows

5.16.2. Use of Kermit-MS with External Device Drivers                  Page 139

-------------------------------------------------------------------------------

the major IBM PC keys to be redefined, and also  interprets  selected

ANSI-format  escape  sequences  for screen control.  It works fine at

Kermit-MS command level, except SHOW KEY does not  recognize  strings

assigned  to  keys  via  ANSI.SYS, and fine at CONNECT level.  To use

ANSI.SYS at CONNECT level, issue the Kermit-MS commands SET  KEY  OFF

(to  read  keys via DOS) and SET TERMINAL NONE (to display characters

through DOS).

- Blind people often have speaking  or  Braille  machines  attached  to

their  PCs.   DOS-level device drivers are generally used to redirect

screen output to these devices, which works OK at  DOS  or  MS-Kermit

command level.  SET TERMINAL NONE will allow this redirection to take

place during CONNECT.  But  these  devices  also  need  to  have  the

computer's  output  appear  as  a  coherent  stream of text, so users

should also take care to inform the remote host to format its  output

for  a  "dumb"  or hardcopy terminal.  In addition, Kermit-MS' normal

file transfer display does not mesh well with these devices, but that

can be remedied using SET DISPLAY SERIAL.

- People  with motor impairments may be using special keyboard replace-

ments supported by DOS-level  device  drivers.    As  with  ANSI.SYS,

Kermit-MS  may be directed to use such keyboard drivers with the com-

mand SET KEY OFF.

- Other keyboard drivers are available that work,  like  Kermit-MS,  at

BIOS level.  Examples include ProKey and SuperKey.  These may be used

at DOS or Kermit-MS command level as well as during CONNECT.

- Conceivably, drivers exist that allow DOS communication  programs  to

emulate  terminals  other than ANSI.  You should be able to use them,

if they exist, in conjunction with Kermit-MS by telling Kermit to SET

TERMINAL NONE, but the speed may not be high because of the interven-

ing DOS call.

5.16.3. Kermit-MS/IBM Serial Port Information

Kermit-MS for IBM PC's and compatibles does testing of serial ports before use.

This section describes those tests so users may understand what Kermit does.

When a serial port is selected by the SET PORT COMx command Kermit looks at low

memory addresses in segment 40H assigned to hold the base address of each  COMx

port;  COM1 is in word 40:0H, COM2 is in word 40:2H, and so on. If the value in

the appropriate word is binary zero then Kermit declares the  port  to  be  un-

available.  Otherwise, Kermit runs read-only (i.e., safe) tests at the base ad-

dress to validate the presence of an official 8250 UART chip. If the tests fail

Kermit  indicates  it will do i/o through the slow Bios pathway; some PC clones

need to work this way even though the Bios has  speed  problems  even  at  1200

baud.  Otherwise, interrupt driven i/o will be done through the 8250 UART (that

is very fast).

There is a special case when a communications board is present, set  for  COM2,

but  a normal COM1 serial port is not.  Kermit detects this situation, issues a

short notice and uses the available COM2 board.

Many machines now have more than two serial ports, yet  there  is  no  standard

5.16.3. Kermit-MS/IBM Serial Port Information                          Page 140

-------------------------------------------------------------------------------

about addresses for COM3 and COM4.  PC DOS 3.30 does not assign them either be-

cause it is really a problem of the system ROM Bios  boot  code  run  when  the

power  is turned on.  However, Kermit will use COM3 and/or COM4 if the base ad-

dress of a port is placed in low memory words 40:4H (COM3) or 40:6H (COM4); the

tests  described above are then carried out. One restriction is that the Inter-

rupt ReQuest number (IRQ in the serial port board manual) must be IRQ4 for COM3

(and  for  COM1)  and  IRQ3  for  COM4  (and for COM2). Check the board and its

manual.  DOS utility DEBUG can be used to create a short program to insert  the

board's  addresses  into  the segment 40H memory locations; a sample program is

given below.

-------------------------------------------------------------------------------

Serial Port       Base Address IRQ           Conventions

COM1              03F8H        4             IBM standard

COM2              02F8H        3             IBM standard

COM3              ?            4             Board, Kermit wants IRQ4

COM4              ?            3             Board, Kermit wants IRQ3

Table 5-11:   IBM PC/XT/AT Serial Port Numbers

-------------------------------------------------------------------------------

The addresses shown as query marks are to be found  in  the  board's  reference

manual;  values  such  as  2E8H and 2E0H would be common.  However, there is no

standard for anything to do with COM3 and COM4.

Assuming that you have selected an address in harmony with the rest of the sys-

tem  (good  luck  on  that  part), set the board's switches or jumpers, and use

DEBUG to insert the address(es) in segment  40H  memory.    The  example  below

creates  a small program named SETCOM3.COM to put address 02E8H into the memory

word 40:04H for COM3 and writes the program to drive  A.  (Disregard  the  xxxx

items below):

A> DEBUG                        don't type these comments

-n a:setcom3.com                sets name of output file

-a                              assemble command

xxxx:100 mov ax,40              value 40h

xxxx:103 mov es,ax              put it into register es

xxxx:105 mov ah,02              the 02 part of 02E8H

xxxx:107 mov al,e8              the E8 part of same

xxxx:109 es:

xxxx:10A mov [4],ax             store in 40:4 for com3 ([6] for com4)>

xxxx:10D int 20                 return to DOS

xxxx:10F                        blank line to end assemble mode

-r cx                           show contents of register cx

CX 0000

: 0f                            set register cx to write 0fh bytes

-w                              write material to the disk file

-q                              quit debug

A> DEBUG setcom3.com

-u                              unassemble to see if all is well

-q                              quit debug

5.16.3. Kermit-MS/IBM Serial Port Information                          Page 141

-------------------------------------------------------------------------------

Note,  for  COM4,  use  [6]  above  rather  than [4], and of course employ your

board's port address in place of 02E8H (check the manual).  Finally, try it:

A> setcom3                      run the program

A> DEBUG                        now see what's down there

-d 40:00                        display bytes in seg 40H

( Shows many bytes.  See yours?  Good. )

-q

A>

A small side effect noted in practice is the first time the extra port is  used

there  may be garbage from it.  Just return to the Kermit prompt and try again,

if necessary SET PORT to the other COM lines momentarily, all  should  be  well

the second time.

More technical comments, for those with an interest.  When Kermit finishes with

a port it disables interrupts for that serial port and returns the  IRQ  signal

line  to  its  state found when Kermit started since many devices can share the

same Interrupt ReQuest line but only one device at a time can be active  on  an

IRQ.    If  you find that transmissions are good but there is no reception then

another device has stolen the IRQ; disable it or find a guru.  Kermit will work

with  non-standard  addresses for COM1 and COM2 but the IRQ's must be as in the

table above.  Accessing a non-existent port produces a  message  and  all  com-

munications are discarded safely in the bit bucket.

5.16.4. CTTY COMx for IBM Machines

The  DOS  command  CTTY  COMx  redirects the standard input and output from the

keyboard and screen, respectively, to the indicated communications channel.  If

a  Kermit  Server  is operated this way, "through the back port", then both DOS

and Kermit can access the port hardware simultaneously; a deadlock develops  on

IBM machines.  The items below refer to only the IBM version of Kermit-MS.

Kermit-MS/IBM  version 2.30 successfully resolves the deadlock in the following

manner.  When Kermit requires the serial port it also attaches itself to Inter-

rupt  16H, the Bios RS232 serial port routine.  Code within Kermit receives the

DOS serial port requests via Interrupt 16H and either passes the request to the

Bios  if  the COM line is not that used by Kermit or it handles the request in-

ternally for conflicting situations.  When the same port is used  by  both  DOS

and  Kermit, Kermit discards DOS output material (typically a prompt, but could

be the dreaded Abort, Retry, Ignore message) and returns a success code to DOS,

it  returns an ascii Backspace code to DOS read requests (this is a key item to

keep DOS complacent while  Kermit  communicates),  and  it  returns  reasonable

status  for  modem  status.    The interception ceases when Kermit releases the

port, such as when the Kermit prompt is displayed, and this lets  DOS  converse

out the serial port.

It  is  worth  restating  that a large number of programs bypass DOS to achieve

higher performance.  When such programs are started through the back door  they

may  still  require input from the real keyboard and will hang, waiting for it.

There is nothing to do about this situation except a) don't let it  happen,  b)

contact  the  local  operator  to  push some keys.  Kermit-MS is not a Bulletin

Board Server.

5.16.5. Screen Sizes and the EGA Board, IBM Versions                   Page 142

-------------------------------------------------------------------------------

5.16.5. Screen Sizes and the EGA Board, IBM Versions

Support has been included for Enhanced Graphics  Adapter  (EGA)  video  display

boards  which  can  be  configured for other than the standard 80 columns by 25

lines, say 132 columns or 43 lines or other.  Three boards, the Tseng Labs  EVA

board  with  the  132 column kit installed, the ATI EGA Wonder, and the Video 7

Deluxe, can be controlled directly by Kermit for 80/132 column changes.   Other

boards  need to be placed in the desired display mode by the user.  Kermit then

adapts to the settings if the board obeys standard rules for using the Bios EGA

memory  areas in segment 40H.  The Video-7 Vega Deluxe board has been used suc-

cessfully in all screen sizes, including 132 columns by 43 lines, with  an  NEC

Multisync monitor.

The  IBM EGA board has several noteworthy bugs which are now standards.  One is

the cursor dots are not always on the correct scan lines  when  the  number  of

screen  lines  is other than 25.  Kermit-MS attempts to compensate for this at-

tribute.  Screen roll back space is fixed in size so there are fewer pages  for

more  dense  screens;  standard  screens  use  an internal buffer, non-standard

screens use a buffer plus video page 1. ANSI.SYS is hard coded for 25 line dis-

plays  so all DOS i/o will eventually overwrite itself on line 25; the emulator

does not use DOS i/o.  Commercial replacements for ANSI.SYS should be  able  to

use all screen lines.

Screen  dumps work correctly if done with Kermit commands.  DOS PrintScreen may

or may not, depending on your EGA board.

When the VT102 receives escape sequences to change between 80  and  132  column

modes the screen is reset and the Tseng Labs, ATI, or Video 7 board is asked to

change modes (but only if that board is present); other  display  adapters  are

left  in their current state.  The right margin is enforced strongly so a board

in 132 column mode will not display material to the right of column 80  if  the

emulator  is  in 80 column mode.  Similarly, material to the right of column 80

is not preserved in the emulator if the display  adapter  is  operating  in  80

column  mode;  real  VT102s  keep  that  invisible  material in hardware memory

whereas the emulator does not.

Reference is made to line 25 in the emulator; this is normally the  status/mode

line  in  Kermit.    Real  VT102's  have only 24 line displays.  If the display

adapter is set for a different number of lines per screen then the 25th line is

interpreted  to  mean the bottom display adapter line, such as line 43.  Should

the host access the status/mode line then the line is declared to  be  disabled

(same  as  SET MODE OFF) so that Kermit's own status information does not over-

write the host's when the screen is restored.  Toggling a  disabled  mode  line

has  no  effect;  only SET MODE ON will enable it again.  The Heath-19 terminal

has the unusual feature that disabling the mode line (ESC y 1) also clears it.

5.16.6. Kermit-MS/IBM Printer Control

The IBM PC MS-Kermit VT102 terminal emulator  also  supports  full  transparent

printing  of  8-bit  binary  bytes.    The escape sequence "ESC [ 5 i" turns on

transparent printing, in which all further 8-bit characters are  sent  directly

to  the  printer, bypassing the SET TRANSLATION INPUT filter, and are not shown

on the screen.  Escape sequence "ESC [ 4 i" turns off transparent printing  and

the  escape  sequence  is not sent to the printer.  Non-transparent printing is

controlled by the "ESC [ ? 5 i" and "ESC [ ? 4 i"  sequences.    Such  printing

5.16.6. Kermit-MS/IBM Printer Control                                  Page 143

-------------------------------------------------------------------------------

simply  duplicates  text  intended  for the screen, excluding escape sequences.

The text also appears on the screen.

Kermit-MS accesses the system printer through DOS calls several  ways;  neither

the  Bios  nor the hardware are used.  Files directed to the printer by the SET

DESTINATION PRINTER command are written by opening a file  with  the  name  PRN

(DOS's  name  for  the  system printer) and writing to it the same as to a disk

file; DOS provides limited buffering.  LOGging to device  PRN  works  the  same

way,  as  can be noticed by the last line or so not being printed until the log

file is CLOSED.  DOS is used again while emulating a terminal in CONNECT  mode.

If the VT102 emulator found in the IBM PC is used for transparent or Controller

printing (via arcane escape sequences sent from the host) single characters are

written  to DOS file handle 4, the DOS standard print device.  If the screen is

echoed to the printer via the typical Control PrtSc key combination, or  equiv-

alent,  single  characters  are  written by the DOS function 05H Printer Output

call.  In both cases of terminal emulation the printer's ready status is  found

by the DOS IOCTL 44H call.  Only the Control PrtSc case results in the PRN mes-

sage being displayed on the status line.  Finally, the classical IBM  PC  Shift

PrtSc  command to copy the whole screen to the printer is unknown to Kermit be-

cause the system Bios traps the key combination and does not tell Kermit  about

it.    If the Control P command is given to DOS before Kermit starts then again

characters are echoed by the  system  Bios  without  Kermit's  knowledge;  this

situation can result in lost characters.

Print spoolers generally operate by being told an existing filename and then in

the background they steal cpu cycles  to  read  from  disk  and  write  to  the

printer.    The  DOS  PRINT command invokes such a spooler.  Although an active

Kermit does not feed these software programs directly the  spooler  and  Kermit

can  compete  for cpu cycles and characters can be lost.  If a non-DOS resident

program intercepts characters destined for the printer device and  spools  them

Kermit does not know about it and similar competion can occur.

During  file  transfers  printing is carefully sequenced to occur only when the

local Kermit is in control of the communications line so  that  a  small  pause

will  not  result in missing characters arriving at the serial port.  When ter-

minal emulation is active then printing competes for cpu time with  the  serial

port  routines.    Generally, the serial port wins such contests if the port is

interrupt driven (Generic Kermit is not interrupt driven, so beware).  However,

the printing itself can use enough cpu cycles to delay processing of characters

to the screen and eventually the receive buffer of the serial port fills to the

high  water  mark  and  an  XOFF  flow control character is sent to the host to

suspend further transmissions until we send an XON.  If FLOW is NONE  then  ex-

pect  lost  characters  at  the serial port.  Experience with ordinary IBM PC's

through 80386 machines at very high baud rates indicates no characters are lost

when  FLOW  is  XON/XOFF.    However,  it  is possible on some machines for the

printer to have priority over the serial port, and hence to have  lost  charac-

ters,  especially  if  a  Terminate Stay Resident program intercepts characters

destined for the printer and keeps interrupts turned off too long.

6. UNIX KERMIT                                                         Page 144

-------------------------------------------------------------------------------

6. UNIX KERMIT

Program:    Frank da Cruz, Bill Catchings, Jeff  Damens,  Columbia  University;

Herm Fischer, Encino CA; contributions by many others.

Language:   C

Documentation:

Christine Gianone, Frank da Cruz

Version:    4E(070)

Date:       January 24, 1988

C-Kermit is an implementation of Kermit, written modularly and transportably in

C.  The protocol state transition table is written in wart, a (non-proprietary)

lex-like preprocessor for C. System-dependent primitive functions are  isolated

into  separately compiled modules so that the program should be easily portable

among Unix systems and also to non-Unix systems that have C compilers, such  as

VAX/VMS,  Data  General AOS/VS, Apollo Aegis, the Apple Macintosh, and the Com-

modore Amiga.  This document applies to Unix implementations of  C-Kermit,  and

in most ways also to the VMS, Data General, and other implementations.

Unix Kermit Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Login scripts:                     Yes (UUCP style)

Transfer text files:               Yes

Transfer binary files:             Yes

Wildcard send:                     Yes

File transfer interruption:        Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                Yes

Communication settings:            Yes

Transmit BREAK:                    Yes (most versions)

Support for dialout modems:        Yes

IBM mainframe communication:       Yes

Transaction logging:               Yes

Session logging:                   Yes

Debug logging:                     Yes

Packet logging:                    Yes

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         Yes

Local file management:             Yes

Command/Init files:                Yes

UUCP and multiuser line locking:   Yes

Long packets:                      Yes

Sliding Windows:                   No

File attributes packets:           No

Command macros:                    No

Raw file transmit:                 No

All numbers in the C-Kermit documentation are decimal unless noted otherwise.

6. UNIX KERMIT                                                         Page 145

-------------------------------------------------------------------------------

C-Kermit  provides  traditional  Unix  command line operation as well as inter-

active command prompting and execution.  The command line options  provide  ac-

cess  to a basic subset of C-Kermit's capabilities; the interactive command set

is far richer.

On systems with dialout modems, C-Kermit's  command  file,  DIAL  command,  and

login  script  facilities  provide a counterpart to UUCP for file transfer with

non-UNIX operating systems, including the use of scheduled  (e.g.  late  night)

unattended operation.

6.1. The Unix File System

Consult  your  Unix manual for details about the file system under your version

of Unix.  In general, Unix files have lowercase names, possibly containing  one

or  more  dots  or  other special characters.  Unix directories are tree-struc-

tured.  Directory levels are separated by slash ("/") characters.  For example,

/usr/foo/bar

denotes the file bar in the directory /usr/foo.  Alphabetic case is significant

in Unix file and directory names, i.e. "a" is a different file  (or  directory)

from "A".  Wildcard or "meta" characters allow groups of files to be specified.

"*" matches any string; "?" matches any single character.

When C-Kermit is invoked with file arguments  specified  on  the  Unix  command

line,  the  Unix  shell  (Bourne Shell, C-Shell, K-Shell, etc) expands the meta

characters itself, and in this case a wider variety is available.  For example,

kermit -s ~/ck[uvm]*.{upd,bwr}]

is  expanded by the Berkeley C-Shell into a list of all the files in the user's

home directory (~/) that start with the characters "ck", followed by  a  single

character  "u", "v", or "m", followed by zero or more characters, followed by a

dot, followed by one of the strings "upd" or "bwr".  Internally,  the  C-Kermit

program itself expands only the "*" and "?" meta characters.

Unix  files are linear (sequential) streams of 8-bit bytes.  Text files consist

of 7-bit ASCII characters, with the high-order bit off (0), and lines separated

by  the Unix newline character, which is linefeed (LF, ASCII 10).  This distin-

guishes Unix text files from those on most other ASCII systems, in which  lines

are  separated by a carriage-return linefeed sequence (CRLF, ASCII 13, followed

by linefeed, ASCII 10).  Binary files are likely to contain data  in  the  high

bits of the file bytes, and have no particular line or record structure.

When  transferring  files,  C-Kermit  will convert between upper and lower case

filenames and between LF and CRLF line terminators automatically,  unless  told

to  do  otherwise.   When binary files must be transferred, the program must be

instructed not to perform LF/CRLF conversion (-i on the command  line  or  "set

file type binary" interactively; see below).

6.2. File Transfer                                                     Page 146

-------------------------------------------------------------------------------

6.2. File Transfer

If  C-Kermit  is  in  local mode, the screen (stdout) is continously updated to

show the progress of the file transer.  A dot is printed for  every  four  data

packets, other packets are shown by type:

I Exchange Parameter Information

R Receive Initiate

S Send Initiate

F File Header

G Generic Server Command

C Remote Host Command

N Negative Acknowledgement (NAK)

E Fatal Error

T Indicates a timeout occurred

Q Indicates a damaged, undesired, or illegal packet was received

% Indicates a packet was retransmitted

You may type certain "interrupt" commands during file transfer:

Control-F:  Interrupt the current File, and go on to the next (if any).

Control-B:  Interrupt the entire Batch of files, terminate the transaction.

Control-R:  Resend the current packet

Control-A:  Display a status report for the current transaction.

These  interrupt characters differ from the ones used in other Kermit implemen-

tations to avoid conflict with commonly used Unix shell  interrupt  characters.

With  Version  7,  System  III, and System V implementations of Unix, interrupt

commands must be preceeded by the 'connect' escape character (e.g. normally-\).

Ctrl-F  and  Ctrl-B are effective only during the transfer of data (D) packets,

and cannot be used to interrupt a transfer that has not yet reached that stage.

CAUTION:  If Control-F or Control-B is used to cancel an incoming file,

and  a file of the same name previously existed, and the "file warning"

feature is not enabled, then the previous copy of the  file  will  dis-

appear.

EMERGENCY EXIT:  When running Unix Kermit in remote mode, if you have started a

protocol operation (sending or receiving a file, server command wait, etc), you

will  not  be able to communicate with the terminal in the normal way.  In par-

ticular, you cannot stop the protocol  by  typing  the  normal  Unix  interrupt

characters,  since  the  terminal  has  been put in "raw mode".  If you need to

regain control quickly -- for instance, because the protocol is  stuck  --  you

can  type  two Control-C's directly to the Unix Kermit program ("connect" first

if necessary):

Control-C Control-C

This will cause the program to exit and restore the terminal to normal.

6.3. Command Line Operation                                            Page 147

-------------------------------------------------------------------------------

6.3. Command Line Operation

The C-Kermit command line syntax conforms to the Proposed Syntax Standards  for

Unix  System  Commands  put forth by Kathy Hemenway and Helene Armitage of AT&T

Bell Laboratories in Unix/World, Vol.1, No.3, 1984.  The rules that apply are:

- Command names must be between 2 and 9 characters ("kermit" is 6).

- Command names must include lower case letters and digits only.

- An option name is a single character.

- Options are delimited by '-'.

- Options with  no  arguments  may  be  grouped  (bundled)  behind  one

delimiter.

- Option-arguments cannot be optional.

- Arguments immediately follow options, separated by whitespace.

- The order of options does not matter.

- '-' preceded and followed by whitespace means standard input.

A group of bundled options may end with an option that has an argument.

The following notation is used in command descriptions:

fn      A  Unix  file specification, possibly containing the "wildcard" charac-

ters `*' or `?' (`*' matches all character strings, `?'    matches  any

single character).

fn1     A Unix file specification which may not contain `*' or `?'.

rfn     A  remote  file  specification in the remote system's own syntax, which

may denote a single file or a group of files.

rfn1    A remote file specification which should denote only a single file.

n       A decimal number between 0 and 94.

c       A decimal number between 0 and 127 representing the value of  an  ASCII

character.

cc      A  decimal  number  between 0 and 31, or else exactly 127, representing

the value of an ASCII control character.

[ ]     Any field in square braces is optional.

{x,y,z} Alternatives are listed in curly braces.

C-Kermit command line options may specify any combination of actions  and  set-

tings.    If C-Kermit is invoked with a command line that specifies no actions,

then it will issue a prompt and  begin  interactive  dialog.    Action  options

specify either protocol transactions or terminal connection.

-s fn   Send  the  specified  file  or  files.   If fn contains wildcard (meta)

characters, the Unix shell expands it into a list.  If fn is  '-'  then

kermit sends from standard input, which may come from a file:

kermit -s - < foo.bar

or a parallel process:

6.3. Command Line Operation                                            Page 148

-------------------------------------------------------------------------------

ls -l | grep christin | kermit -s -

You  cannot use this mechanism to send terminal typein.  If you want to

send a file whose actual name is "-" you can precede  it  with  a  path

name, as in

kermit -s ./-

-r      Receive a file or files.  Wait passively for files to arrive.

-k      Receive  (passively)  a file or files, sending them to standard output.

This option can be used in several ways:

kermit -k

Displays  the  incoming  files  on  your screen; to be used only in

"local mode" (see below).

kermit -k > fn1

Sends  the  incoming file or files to the named file, fn1.  If more

than one file arrives,  all  are  concatenated  together  into  the

single file fn1.

kermit -k | command

Pipes the incoming data (single or multiple files) to the indicated

command, as in

kermit -k | sort > sorted.stuff

-a fn1  If you have specified a file transfer option, you may give an alternate

name for a single file with the -a ("as") option.  For example,

kermit -s foo -a bar

sends the file foo telling the receiver that its name is bar.  If  more

than  one  file  arrives or is sent, only the first file is affected by

the -a option:

kermit -ra baz

stores the first incoming file under the name baz.

-x      Begin server operation.  May be used in either local or remote mode.

Before proceeding, a few words about remote and local operation are  necessary.

C-Kermit  is  "local"  if it is running on PC or workstation that you are using

directly, or if it is running on a multiuser system and transferring files over

an  external  communication line -- not your job's controlling terminal or con-

sole.  C-Kermit is remote if it is running on a multiuser system and  transfer-

ring  files  over  its  own controlling terminal's communication line (normally

/dev/tty), connected to your PC or workstation.

If you are running C-Kermit on a PC, it is normally used in  local  mode,  with

the  "back  port" designated for file transfer and terminal connection.  If you

are running C-Kermit on a multiuser (timesharing) system, it is in remote  mode

unless  you  explicitly  point it at an external line for file transfer or ter-

minal connection.  The following command sets C-Kermit's "mode":

6.3. Command Line Operation                                            Page 149

-------------------------------------------------------------------------------

-l dev  Line -- Specify a terminal line to use for file transfer  and  terminal

connection, as in

kermit -l /dev/ttyi5

When an external line is being used, you will also need some additional options

for successful communication with the remote system:

-b n    Baud -- Specify the baud rate for the line given in the -l  option,  as

in

kermit -l /dev/ttyi5 -b 9600

This  option  should  always  be included with the -l option, since the

speed of an external line is not necessarily what you expect.

-p x    Parity -- e,o,m,s,n (even, odd, mark, space, or none).   If  parity  is

other  than none, then the 8th-bit prefixing mechanism will be used for

transferring 8-bit binary data, provided the  opposite  Kermit  agrees.

The default parity is none.

-t      Specifies  half  duplex,  line  turnaround  with  XON  as the handshake

character.

The following commands may be used only with a C-Kermit which is  local  either

by default or else because the -l option has been specified.

-g rfn  Actively  request  a remote server to send the named file or files; rfn

is a file specification in the remote host's own syntax.  If fn happens

to  contain  any  special  shell characters, like space, '*', '[', etc,

these must be quoted, as in

kermit -g x\*.\?

or

kermit -g "profile exec"

-f      Send a 'finish' command to a remote server.

-c      Establish a terminal connection over  the  specified  or  default  com-

munication line, before any protocol transaction takes place.  Get back

to  the  local  system  by  typing  the  escape   character   (normally

Control-Backslash) followed by the letter 'c'.

-n      Like  -c,  but  after a protocol transaction takes place; -c and -n may

both be used in the same command.  The use of -n and -c is  illustrated

below.

If the other Kermit is on a remote system, the -l and -b options should also be

included with the -r, -k, or -s options.

Several other command-line options are provided:

-i      Specifies that files should be sent or received exactly "as is" with no

conversions.    This option is necessary for transmitting binary files.

6.3. Command Line Operation                                            Page 150

-------------------------------------------------------------------------------

It may also be used in Unix-to-Unix transfers (it must be given to both

Unix Kermit programs), where it will improve performance by circumvent-

ing the normal text-file conversions, and will allow  mixture  of  text

and binary files in a single file group.

-w      Write-Protect -- Avoid filename collisions for incoming files.

-e n    Extended  packet  length -- Specify that C-Kermit is allowed to receive

packets up to length n, where n may be between 10 and some  large  num-

ber,  like  1000,  depending on the system.  The default maximum length

for received packets is 90.  Packets longer than 94 will be  used  only

if  the  other  Kermit  supports,  and agrees to use, the "long packet"

protocol extension.

-q      Quiet -- Suppress screen update during file transfer, for  instance  to

allow a file transfer to proceed in the background.

-d      Debug -- Record debugging information in the file debug.log in the cur-

rent directory.  Use this option if you believe  the  program  is  mis-

behaving, and show the resulting log to your local Kermit maintainer.

-h      Help -- Display a brief synopsis of the command line options.

The command line may contain no more than one protocol action option.

Files  are  sent with their own names, except that lowercase letters are raised

to upper, pathnames are stripped off, certain special characters like (`~') and

(`#')  are changed to `X', and if the file name begins with a period, an `X' is

inserted before it.  Incoming files are stored under  their  own  names  except

that  uppercase  letters  are  lowered, and, if -w was specified, a "generation

number" is appended to the name if it has the same name  as  an  existing  file

which  would  otherwise be overwritten.  If the -a option is included, then the

same rules apply to its argument.  The file transfer display shows  any  trans-

formations performed upon filenames.

During transmission, files are encoded as follows:

- Control characters are converted to prefixed printables.

- Sequences  of repeated characters are collapsed via repeat counts, if

the other Kermit is also capable of repeated-character compression.

- If parity is being used on the communication  line,  data  characters

with  the  8th  (parity)  bit on are specially prefixed, provided the

other Kermit is capable of 8th-bit prefixing; if  not,  8-bit  binary

files cannot be successfully transferred.

- Conversion is done between Unix newlines and carriage-return-linefeed

sequences unless the -i option was specified.

6.3. Command Line Operation                                            Page 151

-------------------------------------------------------------------------------

Command Line Examples:

kermit -l /dev/ttyi5 -b 1200 -cn -r

This command connects you to the system on the other end of ttyi5 at 1200 baud,

where  you  presumably  log in and run Kermit with a 'send' command.  After you

escape back, C-Kermit waits for a file (or files) to arrive.    When  the  file

transfer is completed, you are reconnected to the remote system so that you can

logout.

kermit -l /dev/ttyi4 -b 1800 -cntp m -r -a foo

This command is like the preceding one, except the remote system in  this  case

uses  half  duplex communication with mark parity.  The first file that arrives

is stored under the name foo.

kermit -l /dev/ttyi6 -b 9600 -c | tek

This example uses Kermit to connect your terminal to the system  at  the  other

end of ttyi6.  The C-Kermit terminal connection does not provide any particular

terminal  emulation,  so  C-Kermit's  standard   i/o   is   piped   through   a

(hypothetical) program called tek, which performs (say) Tektronix emulation.

kermit -l /dev/ttyi6 -b 9600 -nf

This command would be used to shut down a remote server and then connect to the

remote system, in order to log out or to make further use of it.  The -n option

is invoked after -f (-c would have been invoked before).

kermit -l /dev/ttyi6 -b 9600 -qg foo.\* &

This  command  causes C-Kermit to be invoked in the background, getting a group

of files from a remote server (note the quoting of the `*' character).  No dis-

play  occurs  on  the  screen, and the keyboard is not sampled for interruption

commands.  This allows other work to be done while file  transfers  proceed  in

the background.

kermit -l /dev/ttyi6 -b 9600 -g foo.\* > foo.log < /dev/null &

This  command  is  like  the previous one, except the file transfer display has

been redirected to the file foo.log.  Standard input  is  also  redirected,  to

prevent C-Kermit from sampling it for interruption commands.

kermit -iwx

This  command  starts  up  C-Kermit as a server.  Files are transmitted with no

newline/carriage-return-linefeed conversion; the -i option is necessary for bi-

nary  file transfer and recommended for Unix-to-Unix transfers.  Incoming files

that have the same names as existing files are given new, unique names.

6.3. Command Line Operation                                            Page 152

-------------------------------------------------------------------------------

kermit -l /dev/ttyi6 -b 9600

This command sets the communication  line  and  speed.    Since  no  action  is

specified,  C-Kermit issues a prompt and enters an interactive dialog with you.

Any settings given on the command line remain in force during the  dialog,  un-

less explicitly changed.

kermit

This command starts up Kermit interactively with all default settings.

The  next  example shows how Unix Kermit might be used to send an entire direc-

tory tree from one Unix system to another, using the tar  program  as  Kermit's

standard  input and output.  On the orginating system, in this case the remote,

type (for instance):

tar cf - /usr/fdc | kermit -is -

This causes tar to send the directory /usr/fdc (and all its files and  all  its

subdirectories and all their files...) to standard output instead of to a tape;

kermit receives this as standard input and sends it as a binary file.   On  the

receiving system, in this case the local one, type (for instance):

kermit -il /dev/ttyi5 -b 9600 -k | tar xf -

Kermit  receives  the  tar archive, and sends it via standard output to its own

copy of tar, which extracts from it a replica of the original directory tree.

A final example shows how a Unix compression utility might be used to speed  up

Kermit file transfers:

compress file | kermit -is -     (sender)

kermit -ik | uncompress          (receiver)

Exit Status Codes:

Unix  Kermit  returns  an exit status of zero, except when a fatal error is en-

countered, where the exit status is set to  one.    With  background  operation

(e.g., `&' at end of invoking command line) driven by scripted interactive com-

mands (redirected standard input and/or take  files),  any  failed  interactive

command (such as failed dial or script attempt) causes the fatal error exit.

6.4. Interactive Operation

C-Kermit's  interactive  command  prompt  is  "C-Kermit>".  In response to this

prompt, you may type any valid interactive C-Kermit command.  C-Kermit executes

the  command  and  then prompts you for another command.  The process continues

until you instruct the program to terminate.

Commands begin with a keyword, normally an English verb, such as "send".    You

may  omit  trailing  characters  from  any keyword, so long as you specify suf-

6.4. Interactive Operation                                             Page 153

-------------------------------------------------------------------------------

ficient characters to distinguish it from  any  other  keyword  valid  in  that

field.    Certain commonly-used keywords (such as "send", "receive", "connect")

also have special non-unique abbreviations ("s" for "send", "r" for  "receive",

"c" for "connect").

Certain  characters  have  special  functions during typein of interactive com-

mands:

?   Question mark, typed at any point in a command, will produce a  message

explaining  what  is  possible or expected at that point.  Depending on

the context, the message may be a brief phrase, a menu of keywords,  or

a list of files.

ESC (The  Escape  or  Altmode  key)  --  Request  completion of the current

keyword or filename, or insertion of a default value.  The result  will

be a beep if the requested operation fails.

DEL (The  Delete  or  Rubout key) -- Delete the previous character from the

command.  You may also use BS (Backspace, Control-H) for this function.

^W  (Control-W) -- Erase the rightmost word from the command line.

^U  (Control-U) -- Erase the entire command.

^R  (Control-R) -- Redisplay the current command.

SP  (Space) -- Delimits fields (keywords, filenames, numbers) within a com-

mand.  HT (Horizontal Tab) may also be used for this purpose.

CR  (Carriage  Return)  -- Enters the command for execution.  LF (Linefeed)

or FF (formfeed) may also be used for this purpose.

\   (Backslash) -- Enter any of the  above  characters  into  the  command,

literally.    To enter a backslash, type two backslashes in a row (\\).

A backslash at the end of a command line causes the  next  line  to  be

treated  as a continuation line; this is useful for readability in com-

mand files, especially in the 'script' command.

^Z  (Control-Z) -- On systems (like Berkeley Unix, Ultrix)  with  job  con-

trol,  suspend  Kermit,  i.e. put it into the  background in such a way

that it can be brought back into the  foreground  (e.g.  with  an  'fg'

shell command) with all its settings intact.

You  may  type  the editing characters (DEL, ^W, etc) repeatedly, to delete all

the way back to the prompt.  No action will be performed until the  command  is

entered by typing carriage return, linefeed, or formfeed.  If you make any mis-

takes, you will receive an informative error message and a new prompt  --  make

liberal  use  of `?' and ESC to feel your way through the commands.  One impor-

tant command is "help" -- you should use it the first time you run C-Kermit.

A command line beginning with a percent sign "%" is ignored.  Such lines may be

used to include illustrative commentary in Kermit command dialogs.

Interactive  C-Kermit accepts commands from files as well as from the keyboard.

When you start C-Kermit, the program looks for the file .kermrc in your home or

current  directory  (first  it looks in the home directory, then in the current

6.4. Interactive Operation                                             Page 154

-------------------------------------------------------------------------------

one) and executes any commands it finds there.  These commands must be  in  in-

teractive  format,  not  Unix  command-line  format.   A "take" command is also

provided  for  use  at  any  time  during  an  interactive  session,  to  allow

interactive-format  commands  to  be executed from a file; command files may be

nested to any reasonable depth.

Here is a brief list of C-Kermit interactive commands:

%  Comment

!  Execute a Unix shell command, or start a shell.

bye  Terminate and log out a remote Kermit server.

close  Close a log file.

connect  Establish a terminal connection to a remote system.

cwd  Change Working Directory.

dial  Dial a telephone number.

directory  Display a directory listing.

echo  Display arguments literally.

exit  Exit from the program, closing any open files.

finish  Instruct a remote Kermit server to exit, but not log out.

get  Get files from a remote Kermit server.

help  Display a help message for a given command.

log  Open a log file -- debugging, packet, session, transaction.

quit  Same as 'exit'.

receive  Passively wait for files to arrive.

remote  Issue file management commands to a remote Kermit server.

script  Execute a login script with a remote system.

send  Send files.

server  Begin server operation.

set  Set various parameters.

show  Display values of 'set' parameters.

space  Display current disk space usage.

statistics  Display statistics about most recent transaction.

take  Execute commands from a file.

The 'set' parameters are:

block-check  Level of packet error detection.

delay  How long to wait before sending first packet.

duplex  Specify which side echoes during 'connect'.

escape-character  Prefix for "escape commands" during 'connect'.

file  Set various file parameters.

flow-control  Communication line full-duplex flow control.

handshake  Communication line half-duplex turnaround character.

incomplete  Disposition for incompletely received files.

line  Communication line device name.

modem-dialer  Type of modem-dialer on communication line.

parity  Communication line character parity.

prompt  The C-Kermit program's interactive command prompt.

receive  Parameters for inbound packets.

retry  Packet retransmission limit.

send  Parameters for outbound packets.

speed  Communication line speed.

terminal  Terminal parameters.

The 'remote' commands are:

6.4. Interactive Operation                                             Page 155

-------------------------------------------------------------------------------

cwd  Change remote working directory.

delete  Delete remote files.

directory  Display a listing of remote file names.

help  Request help from a remote server.

host  A command to the remote host in its own command language.

space  Display current disk space usage on remote system.

type  Display a remote file on your screen.

who  Display who's logged in, or get information about a user.

Most of these commands are described adequately in the Kermit User Guide or the

Kermit  book.    Special  aspects of certain Unix Kermit commands are described

below.

THE 'SEND' COMMAND

Syntax:  send fn  - or -  send fn1 rfn1

Send the file or files denoted by fn to the other Kermit, which should be  run-

ning as a server, or which should be given the 'receive' command.  Each file is

sent under its own name (as described above, or as specified by the  'set  file

names'  command).  If the second form of the 'send' command is used, i.e.  with

fn1 denoting a single Unix file, rfn1 may be specified as a name to send it un-

der.    The  'send' command may be abbreviated to 's', even though 's' is not a

unique abbreviation for a top-level C-Kermit command.

The wildcard (meta) characters `*' and `?' are accepted in fn.  If `?' is to be

included, it must be prefixed by `\' to override its normal function of provid-

ing help.  `*' matches any string, `?' matches any  single  character.    Other

notations  for  file  groups,  like `[a-z]og', are not available in interactive

commands (though of course they are available on the command line).    When  fn

contains  `*'  or  `?' characters, there is a limit to the number of files that

can be matched, which varies from system to system.  If  you  get  the  message

"Too many files match" then you'll have to make a more judicious selection.  If

fn was of the form

usr/longname/anotherlongname/*

then C-Kermit's string space will fill up rapidly  --  try  doing  a  cwd  (see

below) to the path in question and reissuing the command.

Note  --  C-Kermit sends only from the current or specified directory.  It does

not traverse directory trees.  If the source directory contains subdirectories,

they  will be skipped.  By the same token, C-Kermit does not create directories

when receiving files.  If you have a need to do this, you can pipe tar  through

C-Kermit,  as  shown in the example on page 152, or under System III/V Unix you

can use cpio.

Another Note -- The 'send' command does not skip over  "invisible"  files  that

match  the  file  specification;  Unix  systems usually treat files whose names

start with a dot (like .login, .cshrc, and .kermrc) as  invisible.    Similarly

for "temporary" files whose names start with "#".

6.4. Interactive Operation                                             Page 156

-------------------------------------------------------------------------------

THE 'RECEIVE' COMMAND

Syntax:  receive  - or -  receive fn1

Passively  wait  for files to arrive from the other Kermit, which must be given

the 'send' command -- the 'receive' command does not work in conjunction with a

server  (use  'get'  for  that).  If fn1 is specified, store the first incoming

file under that name.  The 'receive' command may be abbreviated to 'r'.

THE 'GET' COMMAND:

Syntax:  get rfn

or: get

rfn

fn1

Request a remote Kermit server to send the named file or files.  Since a remote

file  specification  (or  list)  might  contain  spaces, which normally delimit

fields of a C-Kermit command, an alternate form of the command is  provided  to

allow  the inbound file to be given a new name: type 'get' alone on a line, and

you will be prompted separately for the remote and local  file  specifications,

for example

C-Kermit>get

Remote file specification: profile exec

Local name to store it under: profile.exec

As  with 'receive', if more than one file arrives as a result of the 'get' com-

mand, only the first will be stored under the alternate name given by fn1;  the

remaining  files will be stored under their own names if possible.  If a `?' is

to be included in the remote file specification, you must prefix it with `\' to

suppress its normal function of providing help.

If  you  have started a multiline 'get' command, you may escape from its lower-

level prompts by typing a carriage return in response to the prompt, e.g.

C-Kermit>get

Remote file specification: foo

Local name to store it under: (Type a carriage return here)

(cancelled)

C-Kermit>

THE 'SERVER' COMMAND:

The 'server' command places C-Kermit in "server mode" on the currently selected

communication  line.   All further commands must arrive as valid Kermit packets

from the Kermit on the other end of the line.    The  Unix  Kermit  server  can

respond to the following commands:

Command                Server Response

get                    Sends files

send                   Receives files

bye                    Attempts to log itself out

6.4. Interactive Operation                                             Page 157

-------------------------------------------------------------------------------

finish                 Exits to level from which it was invoked

remote directory       Sends directory lising

remote delete          Removes files

remote cwd             Changes working directory

remote type            Sends files to your screen

remote space           Reports about its disk usage

remote who             Shows who's logged in

remote host            Executes a Unix shell command

remote help            Lists these capabilities

The  Unix  Kermit  server  cannot always respond properly to a BYE command.  It

will attempt to do so using "kill()", but this will not work on all systems  or

under  all conditions because of the complicated process structures that can be

set up under Unix.

If the Kermit server is directed at an external line  (i.e.  it  is  in  "local

mode")  then  the console may be used for other work if you have 'set file dis-

play off'; normally the program expects the console to be used to observe  file

transfers  and  enter  status queries or interruption commands.  The way to get

C-Kermit into background operation from interactive command level  varies  from

system  to system (e.g. on Berkeley Unix you would halt the program with ^Z and

then use the C-Shell 'bg' command to continue it in the background).  The  more

common method is to invoke the program with the desired command line arguments,

including "-q", and with a terminating "&".

When the Unix Kermit server is given a 'remote host' command,  it  executes  it

using  the  shell  invoked  upon  login,  e.g. the Bourne shell or the Berkeley

C-Shell.

THE 'REMOTE', 'BYE', AND 'FINISH' COMMANDS:

C-Kermit may itself request services from a remote Kermit server.  In  addition

to 'send' and 'get', the following commands may also be sent from C-Kermit to a

Kermit server:

remote cwd [directory]

If the optional remote directory specification is included, you will be

prompted on a separate line for a password, which will not echo as  you

type  it.    If  the remote system does not require a password for this

operation, just type a carriage return.

remote delete rfn       delete remote file or files.

remote directory [rfn]  directory listing of remote files.

remote host command     command in remote host's own command language.

remote space            disk usage report from remote host.

remote type [rfn]       display remote file or files on the screen.

remote who [user]       display information about who's logged in.

remote help             display remote server's capabilities.

bye and finish:

When  connected  to  a  remote  Kermit server, these commands cause the

remote server to terminate; 'finish' returns it  to  Kermit  or  system

command  level  (depending on the implementation or how the program was

invoked); 'bye' also requests it to log itself out.

6.4. Interactive Operation                                             Page 158

-------------------------------------------------------------------------------

THE 'LOG' AND 'CLOSE' COMMANDS:

Syntax: log {debugging, packets, session, transactions} [ fn1 ]

C-Kermit's progress may be logged in various ways.  The 'log' command  opens  a

log,  the  'close' command closes it.  In addition, all open logs are closed by

the 'exit' and 'quit' commands.  A name may be specified for a log file; if the

name is omitted, the file is created with a default name as shown below.

log debugging

This produces a voluminous log of the internal workings of C-Kermit, of use

to  Kermit developers or maintainers in tracking down suspected bugs in the

C-Kermit program.  Use of this feature dramatically slows down  the  Kermit

protocol.  Default name: debug.log.

log packets

This produces a record of all the packets that go in and out  of  the  com-

munication port.  This log is of use to Kermit maintainers who are tracking

down protocol problems in either C-Kermit or any Kermit  that  C-Kermit  is

connected to.  Default name:  packet.log.

log session

This log will contain a copy of everything you see on  your  screen  during

the  'connect' command, except for local messages or interaction with local

escape commands.  Default name:  session.log.

log transactions

The transaction log is a record of all the files that were sent or received

while transaction logging was in effect.    It  includes  time  stamps  and

statistics,  filename  transformations,  and records of any errors that may

have occurred.  The transaction log allows you to have long unattended file

transfer  sessions  without  fear  of  missing  some  vital screen message.

Default name:  transact.log.

The 'close' command explicitly closes a log, e.g. 'close debug'.

Note:  Debug and Transaction logs are a compile-time option;  C-Kermit  may  be

compiled  without these logs, in which case it will run faster, it will take up

less space on the disk, and the commands relating to them will not be present.

LOCAL FILE MANAGEMENT COMMANDS:

Unix Kermit allows some degree of local file management from  interactive  com-

mand level:

directory [fn]

Displays a listing of the names, modes, sizes, and dates of files  matching

fn (which defaults to `*').  Equivalent to `ls -l'.

cwd [directory-name]

Changes Kermit's working directory to the one  given,  or  to  the  default

directory  if the directory name is omitted.  This command affects only the

Kermit process and any processes it may subsequently create.

space

6.4. Interactive Operation                                             Page 159

-------------------------------------------------------------------------------

Display  information about disk space and/or quota in the current directory

and device.

! [command]

The  command  is  executed  by the Unix shell.  If no command is specified,

then an interactive shell is started;  exiting  from  the  shell,  e.g.  by

typing Control-D or 'exit', will return you to C-Kermit command level.  Use

the `!' command to provide file  management  or  other  functions  not  ex-

plicitly  provided  by  C-Kermit  commands.    The  `!' command has certain

peculiarities:

- C-Kermit attempts to use your preferred, customary (login) shell.

- At least one space must separate the '!' from the shell command.

- A 'cd' (change directory) command executed in  this  manner  will

have no effect -- use the C-Kermit 'cwd' command instead.

THE 'SET' AND 'SHOW' COMMANDS:

Since  Kermit  is designed to allow diverse systems to communicate, it is often

necessary to issue special instructions  to  allow  the  program  to  adapt  to

peculiarities  of the another system or the communication path.  These instruc-

tions are accomplished by the 'set' command.  The 'show' command may be used to

display  current  settings.   Here is a brief synopsis of settings available in

the current release of C-Kermit:

block-check {1, 2, 3}

Determines  the  level  of  per-packet  error  detection.  "1" is a single-

character 6-bit checksum, folded to include the values  of  all  bits  from

each  character.    "2"  is  a  2-character,  12-bit  checksum.    "3" is a

3-character, 16-bit cyclic redundancy check (CRC).  The  higher  the  block

check,  the  better  the  error detection and correction and the higher the

resulting overhead.  Type 1 is most commonly used; it is supported  by  all

Kermit  implementations,  and it has proven adequate in most circumstances.

Types 2 or 3 would be used to  advantage  when  transferring  8-bit  binary

files over noisy lines.

delay n

How many seconds to wait before sending the first  packet  after  a  'send'

command.  Used in remote mode to give you time to escape back to your local

Kermit and issue a 'receive' command.  Normally 5 seconds.

duplex {full, half}

For  use  during  'connect'.    Specifies  which side is doing the echoing;

'full' means the other side, 'half' means C-Kermit must echo typein itself.

escape-character cc

For use during 'connect' to get C-Kermit's attention.  The escape character

acts as a prefix to an 'escape command', for instance to close the  connec-

tion  and  return  to  C-Kermit  or  Unix command level.  The normal escape

character is Control-Backslash (28).  The escape character is also used  in

System  III/V  implementations  to  prefix  interrupt  commands during file

transfers.

file {display, names, type, warning}

Establish various file-related parameters:

6.4. Interactive Operation                                             Page 160

-------------------------------------------------------------------------------

display {on, off}

Normally 'on'; when in local mode, display progress of  file  transfers

on  the  screen (stdout), and listen to the keyboard (stdin) for inter-

ruptions.  If off (-q on command line) none of this is  done,  and  the

file transfer may proceed in the background oblivious to any other work

concurrently done at the console terminal.

names {converted, literal}

Normally  converted,  which  means  that  outbound  filenames have path

specifications stripped, lowercase letters raised to upper, tildes  and

extra  periods  changed  to X's, and an X inserted in front of any name

that starts  with  period.    Incoming  files  have  uppercase  letters

lowered.  Literal means that none of these conversions are done; there-

fore, any directory path appearing in  a  received  file  specification

must exist and be write-accessible.  When literal naming is being used,

the sender should not use path names in the file  specification  unless

the same path exists on the target system and is writable.

type {binary, text} [{7, 8}]

The file type is normally text, which means that conversion is done be-

tween  Unix  newline  characters  and  the carriage-return/linefeed se-

quences required by the canonical Kermit file transmission format,  and

in  common use on non-Unix systems.  Binary means to transmit file con-

tents without conversion.  Binary (`-i' in command  line  notation)  is

necessary  for  binary  files, and desirable in all Unix-to-Unix trans-

actions to cut down on overhead.

The optional trailing parameter tells the bytesize for  file  transfer.

It  is  8  by  default.    If you specify 7, the high order bit will be

stripped from each byte of sent and received files.  This is useful for

transferring text files that may have extraneous high order bits set in

their disk representation (e.g.  Wordstar  or  similar  word  processor

files).

warning {on, off}

Normally off, which means that incoming files will  silently  overwrite

existing files of the same name.  When on (`-w' on command line) Kermit

will check if an arriving file would overwrite an existing file; if so,

it  will construct a new name for the arriving file, of the form foo~n,

where foo is the name they share and n is a "generation number"; if foo

exists,  then  the new file will be called foo~1.  If foo and foo~1 ex-

ist, the new file will be foo~2, and so on.  If the new name  would  be

longer than the maximum length for a filename, then characters would be

deleted from the end first, for instance, thelongestname  on  a  system

with a limit of 14 characters would become thelongestn~1.

CAUTION:  If Control-F or Control-B is used to cancel an incom-

ing file, and a file of the same name previously  existed,  and

the  "file  warning"  feature is not enabled, then the previous

copy of the file will disappear.

flow-control {none, xon/xoff}

Normally xon/xoff for full duplex flow control.  Should be set to 'none' if

the other system cannot do xon/xoff flow control, or if you have  issued  a

'set  handshake' command.  If set to xon/xoff, then handshake should be set

to none.  This setting applies during both  terminal  connection  and  file

6.4. Interactive Operation                                             Page 161

-------------------------------------------------------------------------------

transfer.    Warning:  This command may have no effect on certain Unix sys-

tems, where Kermit puts the communication line into 'rawmode', and  rawmode

precludes flow control.

incomplete {discard, keep}

Disposition for incompletely received files.  If an incoming file is inter-

rupted  or  an  error occurs during transfer, the part that was received so

far is normally discarded.  If you "set incomplete  keep"  then  such  file

fragments will be kept.

handshake {xon, xoff, cr, lf, bell, esc, none}

Normally none.  Otherwise, half-duplex communication line turnaround  hand-

shaking  is  done, which means Unix Kermit will not reply to a packet until

it has received the indicated handshake character or has timed out  waiting

for  it;  the  handshake setting applies only during file transfer.  If you

set handshake to other than none, then flow should be set to none.

line [device-name]

The device name for the communication line to be used for file transfer and

terminal connection, e.g. /dev/ttyi3.  If you specify a device name, Kermit

will be in local mode, and you should remember to issue any other necessary

'set' commands, such as 'set speed'.  If you omit the device  name,  Kermit

will  revert  to  its  default mode of operation.  If you specify /dev/tty,

Kermit will enter remote mode (useful when  logged  in  through  the  "back

port"  of  a  system normally used as a local-mode workstation).  When Unix

Kermit enters local mode, it attempts to synchronize  with  other  programs

(like  uucp)  that  use  external  communication lines so as to prevent two

programs using the same  line  at  once;  before  attempting  to  lock  the

specified  line,  it  will  close  and  unlock  any  external line that was

previously in use.  The method used for locking is the  "uucp  lock  file",

explained in more detail later.

modem-dialer {direct, hayes, racalvadic, ventel, ...}

The type of modem dialer on the communication line.  "Direct" indicates ei-

ther there is no dialout modem, or that if the line requires carrier detec-

tion to open, then 'set line' will  hang  waiting  for  an  incoming  call.

"Hayes",  "Ventel",  and  the  others  indicate  that 'set line' (or the -l

argument) will prepare for  a  subsequent  'dial'  command  for  the  given

dialer.    Support for new dialers is added from time to time, so type 'set

modem ?' for a list of those supported in your copy of  Kermit.    See  the

description of the 'dial' command

parity {even, odd, mark, space, none}

Specify character parity for use in packets and terminal  connection,  nor-

mally  none.    If  other  than none, C-Kermit will seek to use the 8th-bit

prefixing mechanism for transferring 8-bit binary data, which can  be  used

successfully  only  if  the  other Kermit agrees; if not, 8-bit binary data

cannot be successfully transferred.

prompt [string]

The  given  string  will  be  substituted for "C-Kermit>" as this program's

prompt.  If the string is omitted, the prompt will revert  to  "C-Kermit>".

If  the string is enclosed in doublequotes, the quotes will be stripped and

any leading and trailing blanks will be retained.

send parameter

6.4. Interactive Operation                                             Page 162

-------------------------------------------------------------------------------

Establish  parameters to use when sending packets.  These will be in effect

only for the initial packet sent, since the other Kermit may override these

parameters during the protocol parameter exchange (unless noted below).

end-of-packet cc

Specifies the control character needed by the other Kermit to recognize

the  end of a packet.  C-Kermit sends this character at the end of each

packet.  Normally 13 (carriage return),  which  most  Kermit  implemen-

tations  require.    Other  Kermits require no terminator at all, still

others may require a different terminator, like linefeed (10).

packet-length n

Specify  the  maximum  packet  length  to  send.  Normally 90.  Shorter

packet lengths can be useful on noisy lines, or with systems  or  front

ends  or networks that have small buffers.  The shorter the packet, the

higher the overhead, but the lower the chance of a  packet  being  cor-

rupted  by  noise,  and  the less time to retransmit corrupted packets.

This command overrides the value requested by the other  Kermit  during

protocol initiation unless the other Kermit requests a shorter length.

pad-character cc

Designate a character to send before each packet.   Normally,  none  is

sent.    Outbound padding is sometimes necessary for communicating with

slow half duplex systems that provide no other means of line turnaround

control.   It can also be used to send special characters to communica-

tions equipment that needs to be put  in  "transparent"  or  "no  echo"

mode,  when this can be accomplished in by feeding it a certain control

character.

padding n

How many pad characters to send, normally 0.

start-of-packet cc

The normal Kermit packet prefix is Control-A (1); this command  changes

the  prefix  C-Kermit  puts on outbound packets.  The only reasons this

should ever be changed would be: Some piece of equipment somewhere  be-

tween  the  two  Kermit programs will not pass through a Control-A; or,

some piece of of equipment similarly placed is echoing its input.    In

the  latter  case,  the recipient of such an echo can change the packet

prefix for outbound packets to be different from that of arriving pack-

ets,  so  that the echoed packets will be ignored.  The opposite Kermit

must also be told to change the prefix for its inbound packets.

timeout n

Specifies the number of seconds you want the other Kermit to wait for a

packet before timing it out and requesting retransmission.

receive parameter

Establish  parameters to request the other Kermit to use when sending pack-

ets.

end-of-packet cc

Requests  the  other Kermit to terminate its packets with the specified

character.

packet-length n

6.4. Interactive Operation                                             Page 163

-------------------------------------------------------------------------------

Specify  the maximum packet length to that you want the other Kermit to

send, normally 90.  If you specify a length of 95 or greater,  then  it

will  be used if the other Kermit supports, and agrees to use, the Ker-

mit protocol extension for long packets.  In  this  case,  the  maximum

length  depends  upon the systems involved, but there would normally be

no reason for packets to be more than about 1000 characters in  length.

The  'show  parameters' command displays C-Kermit's current and maximum

packet lengths.

pad-character cc

C-Kermit  normally does not need to have incoming packets preceded with

pad characters.  This command allows C-Kermit to request the other Ker-

mit to use cc as a pad character.  Default cc is NUL, ASCII 0.

padding n

How many pad characters to ask for, normally 0.

start-of-packet cc

Change  the  prefix C-Kermit looks for on inbound packets to correspond

with what the other Kermit is sending.

timeout n

Normally, each Kermit partner sets its packet timeout interval based on

what the opposite Kermit requests.  This command allows you to override

the  normal procedure and specify a timeout interval for Unix Kermit to

use when waiting for packets from the other Kermit.  If you specify  0,

then  no timeouts will occur, and Unix Kermit will wait forever for ex-

pected packets to arrive.

speed {0, 110, 150, 300, 600, 1200, 1800, 2400, 4800, 9600}

The  baud rate for the external communication line.  This command cannot be

used to change the speed of your own console terminal.  Many  Unix  systems

are set up in such a way that you must give this command after a 'set line'

command before you can use the line.  'set baud'  is  a  synomym  for  'set

speed'.

terminal

Used for specifying terminal parameters.  Currently, 'bytesize' is the only

parameter provided, and it can be set to 7 or 8.  It's 7 by default.

THE 'SHOW' COMMAND:

Syntax: show {parameters, versions}

The  "show"  command  with  the  default  argument of "parameters" displays the

values of all the  'set'  parameters  described  above.    If  you  type  "show

versions",  then C-Kermit will display the version numbers and dates of all its

internal modules.  You should use the "show versions" command to ascertain  the

vintage of your Kermit program before reporting problems to Kermit maintainers.

6.4. Interactive Operation                                             Page 164

-------------------------------------------------------------------------------

THE 'STATISTICS' COMMAND:

The  statistics  command  displays  information  about  the  most recent Kermit

protocol transaction, including file and communication line i/o, timing and ef-

ficiency,  as  well  as  what  encoding options were in effect (such as 8th-bit

prefixing, repeat-count compression).

THE 'TAKE' AND 'ECHO' COMMANDS:

Syntax: take fn1

echo [text to be echoed]

The 'take' command instructs C-Kermit to execute commands from the named  file.

The  file may contain any interactive C-Kermit commands, including 'take'; com-

mand files may be nested to any reasonable depth, but it may not  contain  text

to  be sent to a remote system during the 'connect' command.  This means that a

command file like this:

set line /dev/tty17

set speed 9600

connect

login myuserid

mypassword

etc

will not send "login myserid" or any of the following text to the  remote  sys-

tem.  To carry on a canned dialog, use the 'script' command, described later.

The '%' command is useful for including comments in take-command files.  It may

only be used at the beginning of a line.

The 'echo' command may be used within command files  to  issue  greetings,  an-

nounce progress, ring the terminal bell, etc.  The 'echo' command should not be

confused with the Unix 'echo' command, which can  be  used  to  show  how  meta

characters would be expanded.  The Kermit echo command simply displays its text

argument (almost) literally at the terminal; the argument may contain octal es-

capes  of the form "\ooo", where o is an octal digit (0-7), and there may be 1,

2, or 3 such digits, whose value specify an ASCII character, such as "\007" (or

"\07"  or  just "\7") for beep, "\012" for newline, etc.  Of course, each back-

slash must be must be entered twice in order for it to be passed along  to  the

echo command by the Kermit command parser.

Take-command  files  are  in  exactly  the same syntax as interactive commands.

Note that this implies that if you want  to  include  special  characters  like

question  mark  or  backslash  that you would have to quote with backslash when

typing interactive commands, you must quote these characters the  same  way  in

command  files.  Long lines may be continued by ending them with a single back-

slash.

Command files may be used in lieu of command macros, which have not been imple-

mented in this version of C-Kermit.  For instance, if you commonly connect to a

system called 'B' that is connected to ttyh7 at 4800 baud, you could  create  a

file called b containing the commands

% C-Kermit command file to connect to System B thru /dev/ttyh7

6.4. Interactive Operation                                             Page 165

-------------------------------------------------------------------------------

set line /dev/ttyh7

set speed 4800

% Beep and give message

echo \\007Connecting to System B...

connect

and  then simply type 'take b' (or 't b' since no other commands begin with the

letter 't') whenever you wish to connect to system B. Note  the  comment  lines

and the beep inserted into the 'echo' command.

For  connecting  to  IBM  mainframes,  a number of 'set' commands are required;

these, too, can be conveniently collected into a 'take' file like this one:

% Sample C-Kermit command file to set up current line

% for IBM mainframe communication

%

set parity mark

set handshake xon

set flow-control none

set duplex half

Note that no single command is available to wipe out all of these settings  and

return  C-Kermit  to its default startup state; to do that, you can either res-

tart the program, or else make a command file that executes the necessary 'set'

commands:

% Sample C-Kermit command file to restore normal settings

%

set parity none

set handshake none

set flow-control xon/xoff

set duplex full

An  implicit  'take'  command  is executed upon your .kermrc file when C-Kermit

starts up, upon either interactive or command-line  invocation.    The  .kermrc

file  should  contain  'set'  or other commands you want to be in effect at all

times.  For instance, you might want override the default action when  incoming

files have the same names as existing files -- in that case, put the command

set file warning on

in  your  .kermrc  file.    On some non-Unix systems that run C-Kermit, the in-

itialization file might have a different name, such as kermit.ini.

Errors encountered during execution of take files (such as failure to  complete

dial  or script operations) cause termination of the current take file, popping

to the level that invoked it (take file,  interactive  level,  or  the  shell).

When  kermit  is  executed in the background, errors during execution of a take

file are fatal.

Under Unix, you may  also  use  the  shell's  redirection  mechanism  to  cause

C-Kermit to execute commands from a file:

kermit < cmdfile

or you can even pipe commands in from another process:

6.4. Interactive Operation                                             Page 166

-------------------------------------------------------------------------------

cmdprocess | kermit

THE 'CONNECT' COMMAND:

The  'connect'  command  ('c'  is  an  acceptable  non-unique  abbreviation for

'connect') links your terminal to another computer as if it were a  local  ter-

minal  to  that  computer, through the device specified in the most recent 'set

line' command, or through the default device if your system is a PC or worksta-

tion.   All characters you type at your keyboard are sent out the communication

line (and if you have 'set duplex half', also displayed on  your  screen),  and

all  characters arriving at the communication port are displayed on the screen.

Current settings of speed, parity, duplex, and flow-control  are  honored,  and

the  data connection is 7 bits wide unless you have given the command 'set ter-

minal bytesize 8'.  If you have issued a 'log session' command, everything  you

see  on your screen will also be recorded to your session log.  This provides a

way to "capture" files from remote systems  that  don't  have  Kermit  programs

available.

To  get  back  to your own system, you must type the escape character, which is

Control-Backslash (^\) unless you have changed it with the  'set  escape'  com-

mand,   followed  by  a  single-character  command,  such  as  'c'  for  "close

connection".  Single-character commands include:

c     Close the connection

b     Send a BREAK signal

0     (zero) send a null

s     Give a status report about the connection

h     Hangup the phone

^\    Send Control-Backslash itself (whatever you  have  defined  the  escape

character to be, typed twice in a row sends one copy of it).

Uppercase  and  control  equivalents  for  (most of) these letters are also ac-

cepted.  A space typed after the  escape  character  is  ignored.    Any  other

character will produce a beep.

The  connect  command simply displays incoming characters on the screen.  It is

assumed any screen control sequences sent by the host will be  handled  by  the

firmware  or  emulation  software  in your terminal or PC.  If special terminal

emulation is desired, then the 'connect' command can invoked from the Unix com-

mand line (-c or -n), piped through a terminal emulation filter, e.g.

kermit -l /dev/acu -b 1200 -c | tek

THE 'DIAL' COMMAND:

Syntax: dial telephone-number-string

This  command  controls  dialout  modems; you should have already issued a "set

line" and "set speed" command to identify  the  terminal  device,  and  a  "set

modem"  command  to  identify the type of modem to be used for dialing.  In the

"dial" command, you supply the phone number and the Kermit program feeds it  to

the modem in the appropriate format and then interprets dialer return codes and

modem signals to inform you whether the call was  completed.    The  telephone-

number-string  may  contain  imbedded  modem-dialer commands, such as comma for

6.4. Interactive Operation                                             Page 167

-------------------------------------------------------------------------------

Hayes pause, or `&' for Ventel dialtone wait and `%' for Ventel pause  (consult

your modem manual for details).

At the time of this writing, support is included for the following modems:

- AT&T 7300 Internal Modem

- Cermetek Info-Mate 212A

- Concord Condor CDS 220

- DEC DF03-AC

- DEC DF100 Series

- DEC DF200 Series

- General DataComm 212A/ED

- Hayes Smartmodem 1200 and compatibles

- Penril

- Racal Vadic

- US Robotics 212A

- Ventel

Support for new modems is added to the program from time to time; you can check

the current list by typing "set modem ?".

The device used for dialing out is the one selected in  the  most  recent  "set

line"  command  (or on a workstation, the default line if no "set line" command

was given).  The "dial" command calls locks the path (see the section  on  line

locking  below) and establishes a call on an exclusive basis.  If it is desired

to dial a call and then return to the shell (such as to  do  kermit  activities

depending  on standard in/out redirection), it is necessary to place the dialed

call under one device name (say, "/dev/cua0") and  then  escape  to  the  shell

within  Kermit  on a linked device which is separate from the dialed line (say,

"/dev/cul0").  This is the same technique used by uucp (to allow  locks  to  be

placed separately for dialing and conversing).

Because  modem  dialers have strict requirements to override the carrier-detect

signal most Unix implementations expect, the sequence for dialing is more rigid

than most other C-Kermit procedures.

Example one:

kermit -l /dev/cul0 -b 1200

C-Kermit>set modem-dialer hayes    hint: abbreviate set m h

C-Kermit>dial 9,5551212

Connected!

C-Kermit>connect                   hint: abbreviate c

logon, request remote server, etc.

^\c                                escape back

C-Kermit> ...

C-Kermit>quit                      hint: abbreviate q

this disconnects modem, and unlocks line.

Example two:

kermit

C-Kermit>set modem-dialer ventel

C-Kermit>set line /dev/cul0

C-Kermit>dial 9&5551212%

6.4. Interactive Operation                                             Page 168

-------------------------------------------------------------------------------

Connected!

C-Kermit> ...

Example three:

kermit

C-Kermit>take my-dial-procedure

Connected!

file my-dial-procedure:

set modem hayes

set line /dev/tty99

dial 5551212

connect

In  general, C-Kermit requires that the modem provide the "carrier detect" (CD)

signal when a call is in progress, and remove that signal when  the  call  com-

pletes  or the line drops.  If a modem switch setting is available to force CD,

it should normally not be in that setting.  C-Kermit  also  requires  (on  most

systems)  that  the modem track the computer's "data terminal ready" (DTR) sig-

nal.  If a switch setting is available to  simulate  DTR  asserted  within  the

modem,  then  it  should  normally not be in that setting.  Otherwise the modem

will be unable to hang up at the end of a call or when interrupts are  received

by Kermit.

For  Hayes    dialers,  two important switch settings are #1 and #6.  Switch #1

should be normally be UP so that the modem can act according to your computer's

DTR signal.  But if your computer, or particular implementation of Kermit, can-

not control DTR, then switch 1 should be DOWN.  Switch #6 should normally be UP

so  carrier-detect functions properly (but put it DOWN if you have trouble with

the UP position).  Switches #2 (English  versus  digit  result  codes)  and  #4

(Hayes echoes modem commands) may be in either position.

If  you want to interrupt a dial command in progress (for instance, because you

just realize that you gave it the wrong number), type a Control-C to  get  back

to command level.

THE 'SCRIPT' COMMAND:

Syntax: script expect send [expect send] . . .

"expect" has the syntax: expect[-send-expect[-send-expect[...]]]

The  'script'  command  carries  on  a "canned dialog" with a remote system, in

which data is sent according to the remote system's responses.  The typical use

is for logging in to a remote system automatically.

C-Kermit's  script  facility operates in a manner similar to that commonly used

by the Unix UUCP system's "L.sys" file entries.  A login script is  a  sequence

of the form:

expect send [expect send] . . .

where  expect  is a prompt or message to be issued by the remote site, and send

is the string (names, numbers, etc) to return, and expects are  separated  from

6.4. Interactive Operation                                             Page 169

-------------------------------------------------------------------------------

sends  by  spaces.  The send may also be the keyword EOT, to send Control-D, or

BREAK, to send a break signal.  Letters in sends may be prefixed by `~' to send

special characters, including:

~b  backspace

~s  space

~q  `?'(trapped by Kermit's command interpreter)

~n  linefeed

~r  carriage return

~t  tab

~'  single quote

~~  tilde

~"  double quote

~x  XON (Control-Q)

~c  don't append a carriage return

~o[o[o]]  an octal character

~d  delay approx 1/3 second during send

~w[d[d]]  wait specified interval during expect, then time out

As  with  some UUCP systems, sent strings are followed by ~r unless they have a

~c.

Only the last 7 characters in each expect are matched.  A null expect, e.g.  ~0

or two adjacent dashes, causes a short delay before proceeding to the next send

sequence.  A null expect always succeeds.

As with UUCP, if the expect string does not arrive, the script  attempt  fails.

If  you  expect that a sequence might not arrive, as with UUCP, conditional se-

quences may be expressed in the form:

-send-expect[-send-expect[...]]

where dashed sequences are followed as long as previous expects fail.  Timeouts

for expects can be specified using ~w; ~w with no arguments waits 15 seconds.

Expect/send  transactions  can  be  easily be debugged by logging transactions.

This records all exchanges, both expected and actual.    The  script  execution

will also be logged in the session log, if that is activated.

Note that `\' characters in login scripts, as in any other C-Kermit interactive

commands, must be doubled up.  A line may be ended with a single `\'  for  con-

tinuation.

Example one:

Using  a  modem,  dial  a  UNIX  host site.  Expect "login" (...gin), and if it

doesn't come, simply send a null string with a ~r.  (Some Unixes require either

an  EOT  or  a  BREAK instead of the null sequence, depending on the particular

site's "logger" program.)  After providing user id and password, respond "x" to

a  question-mark prompt, expect the Bourne shell "$" prompt (and send return if

it doesn't arrive).  Then cd to directory kermit, and run  the  program  called

"wermit", entering the interactive connect state after wermit is loaded.

set modem ventel

set line /dev/tty77

set baud 1200

6.4. Interactive Operation                                             Page 170

-------------------------------------------------------------------------------

dial 9&5551212

script gin:--gin:--gin: smith ssword: mysecret ~q x $--$ \

cd~skermit $ wermit

connect

Note  that  'set  line'  is  issued after 'set modem', but before 'set baud' or

other line-related parameters.

Example two:

Using a modem, dial the Telenet network.  This network  expects  three  returns

with  slight  delays between them.  These are sent following null expects.  The

single return is here sent as a null string, with a return appended by default.

Four  returns  are  sent  to  be  safe before looking for the prompt.  Then the

Telenet id and password are entered.  Then Telenet is instructed to connect  to

a  host  site  (c 12345).  The host has a data switch that asks "which system";

the script responds "myhost" (if the "which system" prompt doesn't appear,  the

Telenet  connect command is reissued).  The script waits for an "@" prompt from

the host, then sends the user ID ("joe") and  password  ("secret"),  looks  for

another  "@" prompt, runs Kermit, and in response to the Kermit's prompt (which

ends in ">"), gives the commands "set parity even" and  "server".    Files  are

then  exchanged.  The commands are in a take file; note the continuation of the

'script' command onto several lines using the `\' terminator.

set modem hayes

set line /dev/acu

set speed 1200

set parity mark

dial 9,5551212

script ~0 ~0 ~0 ~0 ~0 ~0 ~0 ~0 @--@--@ id~saa001122 = 002211 @ \

c~s12345 ystem-c~s12345-ystem myhost @ joe~ssecret @ kermit \

> set~sparity~seven > server

send some.stuff

get some.otherstuff

bye

quit

Since these commands may be executed totally in the background, they  can  also

be  scheduled.  A typical shell script, which might be scheduled by cron, would

be as follows (csh used for this example):

#

#keep trying to dial and log onto remote host and exchange files

#wait 10 minutes before retrying if dial or script fail.

#

cd someplace

while ( 1 )

kermit < /tonight.cmd >> nightly.log &

if ( ! $status ) break

sleep 600

end

File tonight.cmd might have two takes in it, for example, one to  take  a  file

with  the set modem, set line, set baud, dial, and script, and a second take of

a file with send/get commands for  the  remote  server.    The  last  lines  of

tonight.cmd should be a bye and a quit.

6.4. Interactive Operation                                             Page 171

-------------------------------------------------------------------------------

THE 'HELP' COMMAND:

Syntax: help

or: help keyword

or: help {set, remote} keyword

Brief  help messages or menus are always available at interactive command level

by typing a question mark at any point.  A slightly more verbose form  of  help

is  available through the 'help' command.  The 'help' command with no arguments

prints a brief summary of how to enter commands and how to  get  further  help.

'help'  may be followed by one of the top-level C-Kermit command keywords, such

as 'send', to request information about a command.  Commands such as 'set'  and

'remote'  have  a further level of help.  Thus you may type 'help', 'help set',

or 'help set parity'; each will provide a successively more detailed  level  of

help.

THE 'EXIT' AND 'QUIT' COMMANDS:

These two commands are identical.  Both of them do the following:

- Attempt to insure that the terminal is returned to normal.

- Relinquish access to any communication line assigned via 'set line'.

- Relinquish any uucp and multiuser locks on the communications line.

- Hang  up the modem, if the communications line supports data terminal

ready.

- Close any open logs or other files.

After exit from C-Kermit, your default directory will be the same as  when  you

started the program.  The 'exit' command is issued implicitly whenever C-Kermit

halts normally, e.g. after a command line invocation, or after certain kinds of

interruptions.

6.5. UUCP Lock Files

Unix  has  no  standard  way  of obtaining exclusive access to an external com-

munication line.  When you issue the 'set line' command to  Unix  Kermit,  Unix

would  normally  grant  you  access  to  the line even if some other process is

making use of it.  The method adopted by  most  Unix  systems  to  handle  this

situation  is  the  "UUCP  lock  file".    UUCP, the Unix-to-Unix Copy program,

creates a file in its  directory  (usually  /usr/spool/uucp,  on  some  systems

/etc/locks)  with a name like LCK..name, where name is the device name, for in-

stance tty07.

Unix Kermit uses UUCP lock files in order to avoid conflicts with UUCP, tip, or

other  programs that follow this convention.  Whenever you attempt to access an

external line using the 'set line' command or `-l' on the command line,  Kermit

looks  in the UUCP directory for a lock file corresponding to that device.  For

instance, if you 'set line /dev/ttyi6' then Kermit looks for the file

/usr/spool/uucp/LCK..ttyi6

If it finds this file, it gives you an error message and a directory listing of

the file so that you can see who is using it, e.g.

6.5. UUCP Lock Files                                                   Page 172

-------------------------------------------------------------------------------

-r--r--r--  1 fdc        4 May  7 13:02 /usr/spool/uucp/LCK..ttyi6

In this case, you would look up user fdc to find out how soon the line will be-

come free.

This convention requires that the  uucp  directory  be  publicly  readable  and

writable.  If it is not, the program will issue an appropriate warning message,

but will allow you to proceed at your own risk (and the risk of anyone else who

might also be using the same line).

If  no lock file is found, Unix Kermit will attempt create one, thus preventing

anyone who subsequently tries to run Kermit, UUCP, tip, or similar programs  on

the  same line from gaining access until you release the line.  If Kermit could

not create the lock file (for instance because the  uucp  directory  is  write-

protected),  then  you  will  receive  a warning message but will be allowed to

proceed at your -- and everyone else's -- risk.  When  Kermit  terminates  nor-

mally, your lock file is removed.

Even  when  the  lock directory is writable and readable, the locking mechanism

depends upon all users using the same name for the same device.   If  a  device

has  more  than one path associated with it, then a lock can be circumvented by

using an alias.

When a lock-creating program abruptly terminates, e.g. because it crashes or is

killed  via  shell  command,  the  lock  file  remains  in  the uucp directory,

spuriously indicating that the line is in use.  If the lock file  is  owned  by

yourself,  you  may  remove it.  Otherwise, you'll have to get the owner or the

system manager to remove it, or else wait for a system task to do so; uucp sup-

ports  a function (uuclean) which removes these files after a predetermined age

-- uucp sites tend to run this function periodically via crontab.

Locking is not needed, or used, if communications occur over the  user's  login

terminal line (normally /dev/tty).

It may be seen that line locking is fraught with peril.  It is included in Unix

Kermit only because other Unix communication programs rely on it.  While it  is

naturally  desirable  to  assure  exclusive  access  to  a line, it is also un-

desirable to refuse access to a vacant line only because  of  a  spurious  lock

file, or because the uucp directory is not appropriately protected.

6.6. C-Kermit under Berkeley or System III/V Unix:

C-Kermit  may be interrupted at command level or during file transfer by typing

Control-C.  The program will perform its normal exit  function,  restoring  the

terminal and releasing any lock.  If a protocol transaction was in progress, an

error packet will be sent to the opposite  Kermit  so  that  it  can  terminate

cleanly.

C-Kermit  may  be invoked in the background ("&" on shell commmand line).  If a

background process is "killed", the user will have to manually remove any  lock

file  and  may  need  to  restore  the  modem.  This is because the kill signal

(kill(x,9)) cannot be trapped by Kermit.

During execution of a system command ('directory', 'cwd', or `!'), C-Kermit can

often  be returned to command level by typing a single Control-C.  (With System

6.6. C-Kermit under Berkeley or System III/V Unix:                     Page 173

-------------------------------------------------------------------------------

III/V, the usual  interrupt  function  (often  the  DEL  key)  is  replaced  by

Control-C.)

Under  Berkeley  Unix  only:  C-Kermit may also be interrupted by ^Z to put the

process in the background.  In this case the terminal is  not  restored.    You

will  have  to type Control-J followed by "reset" followed by another Control-J

to get your terminal back to normal.

Control-C, Control-Z, and Control-\ lose their normal functions during terminal

connection and also during file transfer when the controlling tty line is being

used for packet i/o.

If you are running C-Kermit in "quiet mode" in the foreground, then  interrupt-

ing  the  program  with a console interrupt like Control-C will not restore the

terminal to normal conversational operation.  This is because the  system  call

to  enable console interrupt traps will cause the program to block if it's run-

ning in the background, and the primary reason for quiet mode is to  allow  the

program  to  run  in  the background without blocking, so that you can do other

work in the foreground.

If C-Kermit is run in the background ("&" on shell commmand line), then the in-

terrupt  signal  (Control-C)  (and System III/V quit signal) are ignored.  This

prevents an interrupt signal intended for a foreground job (say a  compilation)

from being trapped by a background Kermit session.

6.7. C-Kermit on the DEC Pro-3xx with Pro/Venix Version 1

The  DEC Professional 300 series are PDP-11/23 based personal computers.  Venix

Version 1 is a Unix v7 derivative.  It should not be confused with  Venix  Ver-

sion 2, which is based on ATT System V; these comments apply to Venix Version 1

only.  C-Kermit runs in local mode on the Pro-3xx when invoked  from  the  con-

sole;  the default device is /dev/com1.dout.  When connected to a remote system

(using C-Kermit's 'connect' command), Pro/Venix itself  (not  Kermit)  provides

VT52  terminal  emulation.   Terminal operation at high speeds (like 9600 baud)

requires xon/xoff flow control, which unfortunately  interferes  with  applica-

tions such as the EMACS that use Control-Q and Control-S as commands.

When  logging  in to a Pro-3xx (or any workstation) through the "back port", it

may be necessary to give the command  "set  line  /dev/tty"  in  order  to  get

C-Kermit to function correctly in remote mode (on a system in which it normally

expects to be operating in local mode).

6.8. C-Kermit under VAX/VMS

C-Kermit can be built using VAX-11 C to run under VMS.  Most  of  the  descrip-

tions  in this manual hold true, but it should be noted that as of this writing

the VMS support is not thoroughly tested, and no explicit  support  exists  for

the various types of VMS files and their attributes.

The C-Kermit init file for VMS is called KERMIT.INI.

6.9. C-Kermit on the Macintosh and other Systems                       Page 174

-------------------------------------------------------------------------------

6.9. C-Kermit on the Macintosh and other Systems

The  "protocol kernel" of C-Kermit is also used by Columbia's Macintosh Kermit.

The user and system interface is  entirely  different,  and  is  covered  in  a

separate document.

There  is  also  a Kermit for the Commodore Amiga based on C-Kermit, as well as

versions for MS-DOS, Data General operating systems, etc.

6.10. C-Kermit Restrictions and Known Bugs

1. Editing characters:  The program's  interactive  command  interrupt,

delete,  and  kill  characters are Control-C, Delete (or Backspace),

and Control-U, respectively.  There is currently no  way  to  change

them  to  suit your taste or match those used by your shell, in case

those are different.

2. Flow control:  C-Kermit attempts to use XON/XOFF flow control during

protocol  operations,  but  it also puts the communication line into

"rawmode".  On many systems, rawmode disables flow control, so  even

though  you  may  have  "set flow xon/xoff", no flow control will be

done.  This is highly system and Unix-version dependent.

3. High baud rates:  There's no way to specify baud rates  higher  than

9600  baud.  Most Unix systems don't supply symbols for them (unless

you use EXTA, EXTB), and even when they do, the program has  no  way

of  knowing whether a specific port's serial i/o controller supports

those rates.

4. Modem controls:  If a connection is made over a  communication  line

(rather  than  on  the controlling terminal line), and that line has

modem controls, (e.g. data  terminal  ready  and  carrier  detection

implementation),  returning  to  the shell level will disconnect the

conversation.  In that case, one should use  interactive  mode  com-

mands,  and  avoid use of piped shell-level operation (also see 'set

modem-dialer' and 'dial' commands.)

5. Login Scripts:  The present login scripts implementation follows the

Unix conventions of uucp's "L.sys" file, rather than the normal Ker-

mit "INPUT/OUTPUT" style.

6. Dial-out vs dial-in  communications  lines:    C-Kermit  requires  a

dial-out or dedicated line for the "set line" or "-l" options.  Most

systems have some lines dedicated  to  dial-in,  which  they  enable

"loggers"  on,  and  some  lines  available  for  dial-out.   Recent

releases of Unix (ATT & Berkeley) have mechanisms for  changing  the

directionality of a line.

7. Using  C-Kermit  on  Local Area Networks:  C-Kermit can successfully

operate at speeds up to 9600 baud over LANs,  provided  the  network

buffers are big enough to accommodate Kermit packets.

When  computers are connected to LAN's through asynchronous terminal

interfaces, then the connection should be configured to do  XON/XOFF

flow  control between the network interface and the computer, rather

6.10. C-Kermit Restrictions and Known Bugs                             Page 175

-------------------------------------------------------------------------------

than passing these signals through transparently.    This  can  help

prevent  Kermit from overrunning the LAN's buffers if they are small

(or if the LAN is congested), and will can also prevent the LAN from

overrunning a slow Kermit's buffers.

If  the network hardware cannot accept 100 characters at a time, and

flow control cannot be done between the network  and  the  computer,

then  Kermit's  "set send/receive packet-length" command can be used

to shorten the packets.

8. Resetting terminal after abnormal termination or kill: When C-Kermit

terminates abnormally (say, for example, by a kill command issued by

the operator) the user may need to reset the  terminal  state.    If

commands  do  not  seem  to  be  accepted  at  the shell prompt, try

Control-J "stty sane" Control-J  (use  "reset"  on  Berkeley  Unix).

That  should  take  the  terminal  out of "raw mode" if it was stuck

there.

9. Remote host commands  may  time-out  on  lengthy  activity:    Using

"remote  host"  to instruct the C-Kermit server to invoke Unix func-

tions (like "make") that might take a long time  to  produce  output

can cause timeout conditions.

10. XOFF  deadlocks:    When  connecting back to C-Kermit after a trans-

action, or after finishing the server, it may be necessary to type a

Control-Q  to  clear  up  an  XOFF  deadlock.   There's not much the

program can do about this...

6.11. How to Build C-Kermit for a Unix System

The C-Kermit files, as distributed from Columbia, all  begin  with  the  prefix

"ck".    You  should  make  a  directory  for these files and then cd to it.  A

makefile is provided to build C-Kermit for  various  Unix  systems  (there  are

separate  makefiles  for  VMS and the Macintosh).  As distributed, the makefile

has the name "ckuker.mak".  You should rename it to "makefile"  and  then  type

"make xxx", where xxx is the symbol for your system, for instance "make bsd" to

make C-Kermit for 4.x BSD Unix.  The result will be a program called  "wermit".

You  should test this to make sure it works; if it does, then you can rename it

to "kermit" and install it for general use.  See the makefile for a list of the

systems supported and the corresponding "make" arguments.

6.12. Adapting C-Kermit to Other Systems

C-Kermit is designed for portability.  The level of portability is indicated in

parentheses after the module name: "C" means any system that has a  C  compiler

that conforms to the description in "The C Programming Language" by Kernighan &

Ritchie (Prentice-Hall, 1978).  "Cf" is like "C", but also requires  "standard"

features like printf and fprintf, argument passing via argv/argc, and so on, as

described in Kernighan & Ritchie.  "Unix" means the module should be useful un-

der  any  Unix  implementation;  it requires features such as fork() and pipes.

Anything else means that the module is  particular  to  the  indicated  system.

C-Kermit file names are of the form:

ck<system><what>.<type>

6.12. Adapting C-Kermit to Other Systems                               Page 176

-------------------------------------------------------------------------------

where the part before the dot is no more than 6 characters long, the part after

the dot no more than 3 characters long, and:

<type> is the file type:

c:    C language source

h:    Header file for C language source

w:    Wart preprocessor source, converted by Wart (or Lex) to a C program

nr:   Nroff/Troff text formatter source

mss:  Scribe text formatter source

doc:  Documentation

hlp:  Help text

bld:  Instructions for building the program

bwr:  A "beware" file - list of known bugs

upd:  Program update log

mak:  Makefile

<system> is a single character to tell what system the file applies to:

a:    Descriptive material, documentation

c:    All systems with C compilers

d:    Data General

h:    Harris computers (reserved)

i:    Commodore Amiga (Intuition)

m:    Macintosh

p:    IBM PC, PC-DOS (reserved)

u:    Unix

v:    VAX/VMS

w:    Wart

<what> is mnemonic (up to 3 characters) for what's in the file:

aaa:  A "read-me" file, like this one

cmd:  Command parsing

con:  Connect command

deb:  Debug/Transaction Log formats, Typedefs

dia:  Modem/Dialer control

fio:  System-depdendent File I/O

fns:  Protocol support functions

fn2:  More protocol support functions

ker:  General C-Kermit definitions, information, documentation

mai:  Main program

pro:  Protocol

scr:  Script command

tio:  System-dependent terminal i/o & control and interrupt handing

usr:  User interface

us2:  More user interface

us3:  Still more user interface

Examples:

ckufio.c        File i/o for Unix

ckmtio.c        Terminal i/o for Macintosh

ckuker.mss      Scribe source for for Kermit User Guide chapter

ckuker.nr       Nroff source file for Unix C-Kermit man page

6.12. Adapting C-Kermit to Other Systems                               Page 177

-------------------------------------------------------------------------------

The following material discusses each of the C-Kermit modules briefly.

ckcmai.c, ckcker.h, ckcdeb.h (Cf):

This  is  the  main program.  It contains declarations for global variables

and a small amount of code to initialize some variables and invoke the com-

mand  parser.    In  its distributed form, it assumes that command line ar-

guments are passed to it via argc and argv.  Since this portion of code  is

only several lines long, it should be easy to replace for systems that have

different styles of user interaction.  The header files define symbols  and

macros  used  by  the  various  modules  of C-Kermit.  ckcdeb.h is the only

header file that is included by all the C-Kermit modules,  so  it  contains

not  only  the  debug  format  definitions, but also any compiler-dependent

typedefs.

ckwart.c (Cf), ckcpro.w (C):

The  ckcpro module embodies the Kermit protocol state table and the code to

accomplish state switching.  It is written in "wart", a language which  may

be regarded as a subset of the Unix "lex" lexical analyzer generator.  Wart

implements enough of lex to allow the ckprot module to function.   Lex  it-

self  was not used because it is proprietary.  The protocol module ckcpro.w

is read by wart, and a system-independent C program is produced.  The  syn-

tax  of  a  Wart  program  is  illustrated by ckcpro.w, and is described in

ckwart.doc.

ckcfns.c (C):

The  module  contains  all  the Kermit protocol support functions -- packet

formation, encoding, decoding, block check calculation, filename  and  data

conversion, protocol parameter negotiation, and high-level interaction with

the communication line and file system.  To accommodate small systems, this

module has been split into two -- ckcfns.c and ckcfn2.c.

ckutio.c:

This module contains the system-dependent primitives for communication line

i/o,  timers, and interrupts for the various versions of Unix.  Certain im-

portant variables are defined  in  this  module,  which  determine  whether

C-Kermit  is  by  default  remote  or local, what the default communication

device is, and so forth.  The tio module maintains its own private database

of  file descriptors and modes for the console terminal and the file trans-

fer communication line so that other modules (like ckcfns or  the  terminal

connect module) need not be concerned with them.  The variations among Unix

implementations with respect to terminal  control  and  timers  are  accom-

modated via conditional compilation.

ckufio.c:

This module contains system-dependent primitives  for  file  i/o,  wildcard

(meta  character) expansion, file existence and access checking, and system

command execution for the various versions of Unix.  It maintains an inter-

nal  database  of i/o "channels" (file pointers in this case) for the files

C-Kermit cares about -- the input file (the file which is being sent),  the

output file (the file being received), the various logs, the screen, and so

forth.  This module varies little among Unix implementations except for the

wildcard  expansion  code;  the  directory structure of 4.2bsd Unix is dif-

ferent from that of other Unix systems.  Again, variation among  Unix  sys-

tems is selected using conditional compilation.

ckuusr.h, ckuusr.c, ckuus2.c, ckuus3.c (Unix):

6.12. Adapting C-Kermit to Other Systems                               Page 178

-------------------------------------------------------------------------------

This is the "user interface" for C-Kermit.  It includes the command parser,

the screen output functions, and console input functions.  The command par-

ser comes in two pieces -- the traditional Unix command line decoder (which

is  quite  small and compact), and the interactive keyword parser (which is

rather large).  This module is fully replacable; its interface to the other

modules  is  very  simple,  and is explained at the beginning of the source

file.  The ckuusr  module  also  includes  code  to  execute  any  commands

directly  which don't require the Kermit protocol -- local file management,

etc.  The module is rated "Unix" because it makes  occasional  use  of  the

system() function.

Note  that  while ckuusr is logically one module, it has been split up into

three C source files, plus a header file for the symbols they share in com-

mon.   This is to accommodate small systems that cannot handle big modules.

ckuusr.c has the command line and  top-level  interactive  command  parser;

ckuus2.c  has  the  help command and strings; ckuus3 has the set and remote

commands along with the logging, screen, and "interrupt" functions.

ckucmd.c, ckucmd.h (Cf):

This  is an interactive command parsing package developed for C-Kermit.  It

is written portably enough to be usable on any system that has a C compiler

that  supports  functions  like  printf.    The file name parsing functions

depend upon primitives defined in the fio module; if these primitives  can-

not  be  supplied for a certain system, then the filename parsing functions

can be deleted, and the package will still be useful for parsing  keywords,

numbers, arbitrary text strings, and so forth.  The style of interaction is

the same as that found on the DECSYSTEM-20.

ckucon.c (Unix):

This is the connect module.  As supplied, it should operate in any Unix en-

vironment, or any C-based environment that provides  the  fork()  function.

The  module  requires  access  to global variables that specify line speed,

parity, duplex, flow control, etc,  and  invokes  functions  from  the  tio

module  to  accomplish the desired settings and input/output, and functions

from the fio module to perform session logging.  No terminal  emulation  is

performed,  but  since  standard  i/o  is used for the console, this may be

piped through a terminal emulation filter.  The ckucon function may be  en-

tirely replaced, so long as the global settings are honored by its replace-

ment.  PC implementations of C-Kermit may require the ck?con module  to  do

screen  control,  escape sequence interpretation, etc, and may also wish to

write special code to get the best possible performance.

ckudia.c (Unix):

This  is the dialer module.  As supplied, it handles Hayes, Ventel, Penril,

Racal-Vadic, and several other modems.

ckuscr.c (Unix):

This  is  the  login  script  module.    As supplied, it handles uucp-style

scripts.

Moving C-Kermit to a new system entails:

1. Creating a new ck?tio module in C, assembler, or  whatever  language

is  most  appropriate  for system programming on the new system.  If

the system is Unix-like,  then  support  may  be  added  within  the

ckutio.c module itself using conditional compilation.

6.12. Adapting C-Kermit to Other Systems                               Page 179

-------------------------------------------------------------------------------

2. Creating a new ck?fio module, as above.

3. If  the system is not Unix-like, then a new ckuusr module may be re-

quired, as well as a different invocation of it from ckcmai.

4. If the distributed connect module doesn't work or  performs  poorly,

then  it may be replaced.  For instance, interrupt-driven i/o may be

required, especially if the system doesn't have forks.

Those who favor  a  different  style  of  user/program  interaction  from  that

provided  in ckuusr.c may replace the entire module, for instance with one that

provides a mouse/window/icon environment, a menu/function-key environment, etc.

A few guidelines should be followed to maintain portability:

- Keep variable and function names to 6 characters or less.  Don't  use

identifiers  that  are  distinguished  from  one  another only by al-

phabetic case.

- Keep modules small.  For instance, on a PDP-11  it  is  necessary  to

keep  the  code segment of each module below 8K in order to allow the

segment mapping to occur which is necessary to  run  programs  larger

than 64K on a non-I-and-D-space machine.

- Keep  strings short; many compilers have restrictive maximum lengths;

128 is the smallest maximum string constant length we've  encountered

so far.

- Keep  (f,s)printf  formats  short.  If these exceed some compiler de-

pendent maximum (say, 128) memory will be overwritten and the program

will probably core dump.

- Do not introduce system dependencies into ckcpro.w or ckcfn*.c.

- If  a  variable  is a character, declare as CHAR, not int, to prevent

the various sign extension and byte swapping foulups that occur  when

characters are placed in integer variables.

- Remember  that  different  systems may use different length words for

different things.  Don't assume an integer can be used as a  pointer,

etc.

- Don't  declare  static  functions; these can wreak havoc with systems

that do segment mapping.

- In conditional compilations expressions, use #ifdef and  #ifndef  and

not  #if,  which is not supported by some compilers.  Also, don't use

any operators in these expressions; many compilers will fail  to  un-

derstand expressions like #ifdef FOO | BAR.  Also, don't put trailing

tokens on #else's or #endif's (use /* comments */).

- Don't define multiline macros.

In general, remember that this program will have to be compilable by  old  com-

pilers and runnable on small systems.

7. IBM 370 KERMIT                                                      Page 180

-------------------------------------------------------------------------------

7. IBM 370 KERMIT

Program:      John Chandler (Harvard/Smithsonian Center for Astrophysics); con-

tributions from Vace Kundakci and Daphne Tzoar (Columbia U),  Bob

Shields  (U.  Maryland),  Victor Lee (Queens U.), Gary Bjerke (U.

Texas  at  Austin),  Greg  Small  (UC  Berkeley),  Clark  Frazier

(Harvard  Bus. Sch.), Bob Bolch and Steve Blankinship (Triangle),

Ron Rusnak (U. Chicago), Roger Fajman and Dale White (NIH), Andre

Pirard (U. Liege)

Language:     IBM 370 Assembler

Documentation:

John Chandler (CfA)

Version:      4.0

Date:         1988 February

Kermit-370 is a family  of  programs  implementing  the  KERMIT  file  transfer

protocol for IBM 370-series mainframes (System/370, 303x, 43xx, 308x, etc.) un-

der a variety of operating systems.  Kermit-370 operates over asynchronous  AS-

CII communication lines attached to 3705-style front ends ("TTY" devices), to a

Series/1 or 4994 running the Yale ASCII Terminal Communication  System  or  the

IBM  7171  ASCII  Device Control Unit or a 9370 with ASCII subsystem ("SERIES1"

devices), or to SNA-type front-ends with graphics pass-through  mode,  such  as

the MICOM 7400, Datastream/Leedata 8010, and PCI 1076 ("GRAPHICS" devices), or,

in some configurations, to an IBM 3708 ("FULLSCREEN" device).  As of this writ-

ing,  the  pending  implementation  of  full  7171 compatibility in the program

product SIM3278 has not been completed.  For more details  on  front-ends,  see

the section SET CONTROLLER.

The  source is coded in IBM 370 assembly language and is divided into sections,

some generic and some specific to an individual operating system.    While  the

details of file-system and supervisor interaction vary widely among the systems

available for IBM 370's, the basic features are essentially  the  same.    This

chapter  will describe the features common to all variants of Kermit-370, and a

separate chapter will deal with the system-specific details for each variant.

IBM 370 systems have some peculiarities that users should be aware of.   First,

they  are  essentially  half-duplex  systems; the communication line must "turn

around" before any data can be sent to it.   The  "TTY"  devices  are  strictly

half-duplex, and even the "SERIES1" and "GRAPHICS" devices, although they main-

tain full-duplex communication with the terminal, must transmit a  block  at  a

time  to  the mainframe.  The fact that a packet has been received from the IBM

system through a "TTY" device is no guarantee that it is  ready  for  a  reply;

generally,  the  true  indicator  of readiness is the line turnaround character

(XON), which the operating system sends immediately before issuing a  read  re-

quest.  On some systems, however, it is possible for Kermit to do away with the

system-supplied turnaround and schedule read  requests  immediately  after  the

corresponding  writes.    It is up to the user to tell the other Kermit that it

must conform to the requirements of the IBM mainframe.

Second, disk files are encoded using the EBCDIC character set.    Consequently,

there are three layers of character translation on packets exchanged on a "TTY"

device.  For an incoming packet, the outer layer is provided by  the  operating

system,  which translates all characters from ASCII to EBCDIC.  Kermit-370 must

then translate the packets back to ASCII (the middle layer) in  order  to  cal-

culate  and  verify  the  checksum.    Data  arriving  through  a  "SERIES1" or

"GRAPHICS" device are still in ASCII and therefore bypass the two outer layers.

7. IBM 370 KERMIT                                                      Page 181

-------------------------------------------------------------------------------

In  any  case,  Kermit-370 translates everything finally into EBCDIC (the inner

layer) before storing on disk (except BINARY files).  When Kermit-370  sends  a

file,  the  opposite translations occur.  In translation, EBCDIC characters not

representable in ASCII are replaced by nulls.  In some  cases,  several  EBCDIC

characters  are  mapped  into  a single ASCII character, but no two 7-bit ASCII

characters are translated into the same EBCDIC  character.    The  middle-layer

tables  used  by  Kermit  must be the inverses of the corresponding outer-layer

ones used by the host operating system if file transfers are to  work  at  all.

If   necessary,   the   system   programmer  should  add  the  appropriate  SET

TATOE/TETOA/TTABLE subcommands (q.v.)  to the global "INIT" file (see the  next

section).    Indeed,  it  is usually a good idea to set TTABLE ON in the global

"INIT" file to force using different built-in sets of tables for the inner  and

middle   layers,   whenever  the  system  has  "TTY"  devices.    The  standard

ASCII-to-EBCDIC translations can be found in the Appendix or the IBM System/370

Reference Card.

Another  distinction  of  IBM  370's  is  that they store and retrieve files as

records rather than byte streams.  Records may be either fixed-length with some

sort  of  padding  (as  needed)  or varying-length with some sort of (generally

hidden) delimiters.  Thus, Kermit-370 must assemble incoming data packets  into

records  by  stripping  off carriage return-linefeed pairs (CRLF's) and padding

with blanks or truncating as needed and must strip trailing blanks  and  append

CRLF's  to  outgoing  records.   Further, disk files typically have the records

combined into blocks for efficiency.  One consequence of this form  of  storage

is  that files have attributes describing the component records: maximum record

length (LRECL), record format (RECFM), and sometimes block size (BLKSIZE).

As mentioned before, Kermit-370 is a family of programs.  At present, only  the

CMS  and  TSO  versions  are operational.  Versions for DOS/VSE and MTS have at

least reached the "drawing board,"  but  no  others  have  even  been  started.

Volunteers  are always welcome to port Kermit-370 to other operating systems or

add new features to the existing family.  Anyone interested should first get in

touch  with  the Center for Computing Activities at Columbia University to find

out what projects of a similar nature are already pending (and thereby  prevent

unnecessary duplication of effort).

7.1. Program Operation

Kermit-370  can  be  invoked  directly  or from a command procedure.  In either

case, it reads and executes subcommands sequentially until directed to quit and

then returns.  A subcommand consists of one or more fields (words) separated by

spaces.

Upon initial startup, the  program  looks  for  two  (optional)  initialization

files,  one system-wide and a second specific to the user.  Both filespecs are,

of course, system-dependent.  The purpose of these files is to allow Kermit  to

be  customized for a particular system and for a user's specific settings with-

out changing the source code.  The system-wide file, which is maintained  by  a

systems programmer, should contain Kermit subcommands that all users would need

to issue in order for Kermit to run on  the  system,  such  as  subcommands  to

modify  the ASCII/EBCDIC tables used by Kermit-370.  The user-specific file, if

any, should contain subcommands that the user generally issues every time  Ker-

mit is run.  Kermit-370 executes any subcommands found in these files as though

they were typed at the terminal.  Here is a sample "INIT" file:

7.1. Program Operation                                                 Page 182

-------------------------------------------------------------------------------

* Asterisk in column one is a comment.

set debug on

set warning on

set block 3

During interactive execution, you may  use  the  built-in  help  feature  while

typing Kermit-370 subcommands.  A question mark ("?") typed at almost any point

in a subcommand, followed by a carriage return, produces a brief description of

what  is  expected or possible at that point.  Moreover, mistyping a subcommand

will generally produce a helpful error message or a list of possible options at

the   point  of  error.    Keywords  in  such  lists  are  displayed  with  the

minimum-length abbreviation in upper case and the remainder, if any,  in  lower

case.  In entering Kermit subcommands, any keyword may be shortened to any sub-

string that contains the minimum abbreviation.

7.2. Kermit-370 Subcommands

The following is a brief summary of Kermit subcommands.   The  starred  subcom-

mands can be issued as remote Kermit commands to Kermit-370 when it is in serv-

er mode.  System-specific subcommands are omitted from this list.

BYE      logs out other Kermit server.

CWD*     establishes a new working directory.

DIRECTORY*     displays all or part of the disk directory.

ECHO      a line back to the user.

EXIT      from Kermit-370.

FINISH      other Kermit server.

GET      file(s) from a Kermit server.

HELP      about Kermit-370.

HOST*     executes a system command.

KERMIT*     executes a Kermit subcommand.

QUIT      from Kermit-370.

RECEIVE      file(s) from other Kermit.

SEND      file(s) to other Kermit.

SERVER      mode of remote operation.

SET*     various parameters.

SHOW*     various parameters.

SPACE*     displays disk storage allocation.

STATUS*     inquiry.

TAKE*     subcommands from file.

TDUMP*     dumps the contents of a table.

TYPE*     a file.

XECHO      echoes a line (transparently).

XTYPE      displays a file (transparently).

Although Kermit-370 is generally a remote Kermit, it has the capability of com-

municating  with another Kermit in server mode.  In that situation, the subcom-

mand prefixes REMOTE and LOCAL refer  to  the  Kermit  server  and  Kermit-370,

respectively,  even  when  Kermit-370 is, strictly speaking, the remote Kermit.

Any replies from the Kermit server are added to a disk file (whose filespec is,

of  course,  system-dependent).  Such a transaction can be carried out, for ex-

ample, under control of a TAKE file if Kermit-370 is not operating locally.  If

the  local  Kermit  has a "magic" character sequence that switches it from ter-

minal emulation to server mode, then an entire session could be controlled from

the mainframe, possibly in response to a single command issued by a naive user.

7.2. Kermit-370 Subcommands                                            Page 183

-------------------------------------------------------------------------------

For example,

grab

Kermit-370 is invoked and executes the following TAKE file

ECHO Serve Me!        the local Kermit switches to server mode

GET file.a            the server uploads file.a

FINISH                the server switches back to terminal mode

The remainder of this section concentrates on the subcommands that have special

form or meaning for Kermit-370, but neglects those with highly system-dependent

syntax or use.

THE SEND SUBCOMMAND

Syntax: SEND [filespec [foreign-filespec]]

The SEND subcommand tells Kermit-370 to send a file or file group to the  other

Kermit.    If  no such file exists, Kermit-370 simply displays an error message

and issues another prompt.  If one or more files  are  sent,  their  names  are

recorded  in  memory and may be viewed later via the TDUMP subcommand (but with

no indication of whether any of them were rejected or cancelled  by  the  other

Kermit).    If this subcommand is issued without any arguments, Kermit-370 will

prompt the user for both native and foreign filespecs (and will insist on  get-

ting the former, but will do without the latter).

When Kermit-370 sends files using long packets (longer than 96), the throughput

is especially sensitive to the level of noise on the line because  retries  are

so  time-consuming.   Therefore, Kermit-370 adds an extra, heuristic size limit

for packets when retries have been found necessary.  When  that  is  the  case,

after  every 20 packets, Kermit computes the packet size for maximum throughput

assuming that the transmission errors were due to  sparse,  Poisson-distributed

noise  bursts.  The result of this calculation is then used as another limit on

the size of outgoing packets besides the one specified by the other Kermit.  If

no  retries  are required, then Kermit-370 assumes the line to be noiseless and

sends packets of the maximum length the other Kermit allows.  For more  details

on  the  SEND  subcommand  syntax and operation, see the chapter on the desired

system-specific version of Kermit-370.

THE RECEIVE SUBCOMMAND

Syntax: RECEIVE [filespec]

The RECEIVE subcommand tells Kermit-370 to accept a file or file  group.    The

user  must  issue the corresponding SEND subcommand to the other Kermit.  Under

some circumstances, the records of the received file(s) may be truncated;  when

this  happens,  Kermit  does  not  stop, but notes the fact as an error (unless

something more serious happens later).  For more details on the RECEIVE subcom-

mand  syntax and operation, see the chapter on the desired system-specific ver-

sion of Kermit-370.

7.2. Kermit-370 Subcommands                                            Page 184

-------------------------------------------------------------------------------

THE GET SUBCOMMAND

Syntax: GET [foreign-filespec [filespec]]

The GET subcommand tells Kermit to request a file or file group from the  other

system,  which  must  have a Kermit running in server mode.  Provided the other

Kermit complies, the effect is the same as if SEND  foreign-filespec  had  been

issued  directly  to  the  other  Kermit  and RECEIVE [filespec] to Kermit-370.

other Kermit).  If this subcommand is issued without any arguments,  Kermit-370

will  prompt the user for both foreign and native filespecs (and will insist on

getting the former, but will do without the latter).  See the  respective  SEND

and RECEIVE subcommands for a description of the each filespec.

THE TAKE SUBCOMMAND

Syntax: TAKE filespec

Execute Kermit subcommands from the specified file, usually called a TAKE file.

The TAKE file may in turn include TAKE subcommands to a nesting depth  of  ten.

If  a  TAKE  file  includes  the  subcommand SERVER, however, the nesting count

starts over again in server mode.  The user has the option of seeing  the  sub-

commands  echoed from the TAKE file as they are executed and also the option of

automatically exiting from a TAKE file on error.  See the subcommand  SET  TAKE

for details.

THE SERVER SUBCOMMAND

Kermit-370  is  capable  of acting as a server.  In server mode, Kermit-370 can

send and receive files, execute host commands, execute a restricted set of Ker-

mit  subcommands,  and perform a variety of generic Kermit functions.  The fol-

lowing list shows the typical local Kermit commands along with the server func-

tions  they  elicit.    When Kermit-370 is talking to another Kermit running in

server mode, these same subcommands may be used in the other direction.

BYE                 log out the Kermit server.

FINISH              server mode.

GET                 a file or files from the server.

REMOTE

COPY              a file or files.

CWD               set new working directory.

DIRECTORY         display file attributes.

ERASE             a file or files.

HELP              display this command summary.

HOST              execute a system command.

KERMIT            execute a Kermit-370 subcommand.

RENAME            a file or files.

SPACE             display disk space.

TYPE              a file.

SEND                a file or files to the server.

If your local Kermit does not support the REMOTE KERMIT command, you  may  need

to  issue  SET  subcommands  to select various options before typing the SERVER

subcommand.  Once in server mode, Kermit-370 will await  all  further  instruc-

tions from the user Kermit on the other end of the connection until a FINISH or

7.2. Kermit-370 Subcommands                                            Page 185

-------------------------------------------------------------------------------

BYE command is given.

Command execution in server mode is different  in  some  respects  from  normal

operation.  First of all, some Kermit subcommands are not allowed (see the list

at the beginning of this section).  Moreover, command errors  always  terminate

any  active  TAKE file.  Also, all commands will be run in the special environ-

ment that Kermit sets up during protocol transfers.  Among other things, Kermit

intercepts  all  terminal  I/O  (if  possible)  in this environment in order to

transmit the data to the local Kermit as text packets.

Note that some operations can be requested by several different commands.    If

for  example,  the  IBM 370 system has a command "PRT" for displaying a file, a

user interacting with a Kermit-370 server can choose to display a file  by  is-

suing  any  of  the commands: REMOTE TYPE, REMOTE HOST PRT, REMOTE KERMIT TYPE,

REMOTE KERMIT HOST PRT, or (if SYSCMD has been set ON) REMOTE KERMIT PRT.   The

first  form  simply transfers the requested file as text, but the others invoke

the "PRT" command with any specified options, intercept  the  terminal  output,

and return the results to the local Kermit.

THE SET SUBCOMMAND

Syntax: SET parameter [value]

The  SET subcommand establishes or modifies various parameters controlling file

transfers.  The values can, in turn, be  examined  with  the  SHOW  subcommand.

Some  parameters  have two levels.  In particular, there are two matching lists

of SEND and RECEIVE sub-parameters corresponding to  the  values  exchanged  by

Kermits  in  the  Send-Init/ACK sequence.  For each of these SEND/RECEIVE pairs

one element is encoded in outgoing parameter packets, and the other is  decoded

from incoming ones.  Setting the latter by hand may be needed to establish con-

tact and also has the effect of redefining the default value for decoding  from

subsequent  parameter  packets.    Generally,  the distinction between SEND and

RECEIVE parameters is unambiguous, the only  exception  being  TIMEOUT  (q.v.).

The following SET subcommands are available in Kermit-370:

APPEND              Append if file name collision.

ATOE                Modify the Kermit-370 ASCII-to-EBCDIC table.

BLOCK-CHECK         Level of error checking for file transfer.

CONTROLLER          Indicate type of terminal connection.

DEBUG               Log packet traffic during file transfer.

DELAY               Length of pause before a SEND subcommand.

EOF                 Text file truncation at CTRL-Z.

ETOA                Modify the Kermit-370 EBCDIC-to-ASCII table.

FILE                Attributes for incoming or outgoing files...

TYPE              ... text or binary.

other             ... system-specific attributes.

FOREIGN             Strings added to outgoing filespec...

PREFIX

SUFFIX

INCOMPLETE          Determine the action on an aborted file transfer.

LINE                Specify alternate communication line.

MARGIN              for sending files...

LEFT

RIGHT

PARITY              Indicate if 7-bit or 8-bit data.

7.2. Kermit-370 Subcommands                                            Page 186

-------------------------------------------------------------------------------

RETRY               Maximum retry count...

INIT              ... for initial packet exchange.

PACKET            ... per packet for ongoing transfer.

SYSCMD              Try apparently invalid Kermit subcommands on host system.

TABS-EXPAND         Determine tab-to-space conversion on reception.

TAKE

ECHO              Echo subcommands read from TAKE files.

ERROR-ACTION      Exit from TAKE file on command error.

TEST                Facilitate testing of Kermit.

TATOE               Modify the Kermit-370 ASCII-to-EBCDIC table.

TETOA               Modify the Kermit-370 EBCDIC-to-ASCII table.

TTABLE              Determine which tables undo the terminal translation.

WARNING             Rename if filename collision.

8-BIT-QUOTE         Determine state of 8th-bit prefixing.

SEND or RECEIVE

END-OF-LINE       Packet terminator.

PACKET-SIZE       Maximum packet size.

PAD-CHAR          Character to insert before each packet.

PADDING           Number of pad characters to insert.

QUOTE             Use to quote control characters in packets.

START-OF-PACKET   Packet beginning marker.

TIMEOUT           Time limit for response.

SET APPEND

Syntax: SET APPEND ON or OFF

ON      If  an incoming file has the same name as an existing one, the new file

is appended to the old one.  This option supersedes SET WARNING.

OFF     Filename collision is  handled  according  to  the  WARNING  parameter.

(Default.)

SET ATOE etc.

Syntax: SET table [num1 num2]

This  modifies  one  of  the ASCII/EBCDIC translation tables used by Kermit-370

(for example, to conform to your system).  The  valid  table  names  are  ATOE,

ETOA, TATOE, and TETOA.  The arguments are, respectively, the offset within the

named table and the new value for that offset.  If the arguments  are  omitted,

the table is restored to its initial arrangement.  Both num1 and num2 should be

in the range 0-255 (decimal).  For example, in ATOE or TATOE, the offset is the

ASCII  character  code,  and  the new value is the new EBCDIC result code.  In-

itially, ATOE and TATOE each contain two identical copies of  the  7-bit  ASCII

character table.  Note: the meaning of the tables depends on the TTABLE setting

-- if TTABLE is OFF, the TATOE and TETOA tables are not used.

7.2. Kermit-370 Subcommands                                            Page 187

-------------------------------------------------------------------------------

SET BLOCK-CHECK

Syntax: SET BLOCK-CHECK number

This determines the type of block check used during file transfer, provided the

other  Kermit  agrees.    Valid  options for number are: 1 (for a one-character

checksum), 2 (for a two-character checksum) and 3 (for a three-character  CRC).

This  is one of only two Send-Init parameters that cannot be SET separately for

SEND and RECEIVE.

SET CONTROLLER

Syntax: SET CONTROLLER type

The type may be TTY, SERIES1, GRAPHICS, or FULLSCREEN.    Kermit-370  automati-

cally  determines  whether you are connected via a Series/1 (or similar) emula-

tion controller or a TTY line.  This subcommand is provided, though,  to  allow

that choice to be superseded, and because Kermit may not be able to distinguish

between Series/1-type and other 3270-emulation controllers.  When CONTROLLER is

set  to SERIES1 or GRAPHICS, Kermit disables the 3270 protocol conversion func-

tion by putting the terminal controller into "transparent mode",  which  allows

Kermit packets to pass through intact.

SET DEBUG

Syntax: SET DEBUG ON or RAW or OFF

ON      Keep  a journal of all packets sent and received in a log file on disk.

If the file already exists, it is erased and overwritten.  The filespec

of  the log is, of course, system-dependent.  All packets are logged in

EBCDIC for legibility, even  when  CONTROLLER  is  set  to  SERIES1  or

GRAPHICS.    In  addition to the packets themselves, which are labelled

"S" or "R" for packets sent or  received,  the  log  includes  any  ad-

ditional status information (labelled "A"), such as the AID returned by

a SERIES1 device.

RAW     The same as ON, but packets are logged in the form that is passed to or

from  the  operating  system, i.e., EBCDIC for TTY terminals, and ASCII

for SERIES1 and GRAPHICS terminals.

OFF     Stop logging packets and close the the log file.  (Default.)

SET DELAY

Syntax: SET DELAY number

Normally, Kermit-370 waits 10 seconds after the SEND subcommand before starting

the  transfer,  but this delay may be SET to any non-negative value.  Two DELAY

values have special meaning.  When DELAY is 1, the usual two-line greeting dis-

played during protocol mode is abbreviated to a short message (the default Ker-

mit prompt with three dots...), and when DELAY is 0, the greeting is suppressed

entirely, along with the extra one-second pause for the RECEIVE and SERVER sub-

commands.

7.2. Kermit-370 Subcommands                                            Page 188

-------------------------------------------------------------------------------

SET EOF

Syntax: SET EOF ON or OFF

ON      Scan each incoming TEXT file for the first occurrence of CTRL-Z and ig-

nore  the remainder of the file (but continue decoding up to the actual

end of the file).  BINARY files are not affected.

OFF     Accept incoming files in their entirety.  (Default.)

SET FILE TYPE

Syntax: SET FILE TYPE TEXT or BINARY or V-BINARY or D-BINARY

TEXT       Specifies ordinary text.  ASCII-to-EBCDIC or EBCDIC-to-ASCII  trans-

lation  is  performed on the data.  Trailing blanks are removed, and

CRLF's are appended to outgoing records. CRLF's are used,  in  turn,

to  determine  the  end  of  incoming records, which are padded with

blanks if necessary to fill buffers.  (Default.)

BINARY     Specifies bit-stream data.  No translation is performed,  no  CRLF's

are  added  to  outgoing  records,  and blanks are neither added nor

removed.  Incoming bytes  are  added  successively  to  the  current

record  buffer,  which  is  written  out  when  the current LRECL is

reached.  Padding, if necessary, is done with nulls.

V-BINARY   Specifies varying-length-record binary data.  This type is like  BI-

NARY, except that a two-byte binary prefix is added to each outgoing

record giving the number of data bytes, and incoming records are set

off by (and stripped of) their prefixes on receipt.

D-BINARY   Is  like  V-BINARY  except  that  the  length prefixes are five-byte

ASCII-encoded decimal (right-justified with leading zeroes).

SET FOREIGN

Syntax: SET FOREIGN PREFIX string

This defines a prefix string to be added to the outgoing filespec generated  by

the  SEND  subcommand.  For example, the string might be set to "B:" to specify

output to the B disk drive on the other Kermit's system.  The default is a null

string.  There is also a FOREIGN SUFFIX handled in the same manner.

SET HANDSHAKE

Syntax: SET HANDSHAKE number

This defines the character, if any, that Kermit-370 should send (or cause to be

sent) immediately before reading each packet.  The character is  given  as  the

decimal  of  an  ASCII  control  character, or as zero if no handshake is to be

sent.  The default is 17 (XON), and any value in the range 0-31 is  valid,  but

13 (CR) should not be used because it is generally the end-of-packet character.

When Kermit-370  is  running  through  a  full-duplex  connection  (such  as  a

7.2. Kermit-370 Subcommands                                            Page 189

-------------------------------------------------------------------------------

"SERIES1"),  the  traditional  IBM  handshaking is not necessary, and HANDSHAKE

should be set to 0 (as long as the other Kermit can be instructed not to expect

a  handshake).  Note the distinction between SET HANDSHAKE in Kermit-370 (where

it defines a character to be sent) and in many micro Kermits (where it  defines

a character to be expected).

SET INCOMPLETE

Syntax: SET INCOMPLETE DISCARD or KEEP

DISCARD   Specifies that incomplete files (that is, files partially received in

a transfer cancelled by the other Kermit) are to be erased.  This  is

the default.  Note that when APPEND is ON, incomplete files are never

erased, lest pre-existing data be lost.

KEEP      Specifies that incomplete files are to be kept.

SET LINE

Syntax: SET LINE [name]

This specifies an alternate communication line for file transfers.  If the name

is omitted, the default line (the user's terminal) is used.  The format of name

is, of course, system-dependent, and some versions of Kermit-370 do not support

any alternate lines.  No version currently allows Kermit-370 to CONNECT over an

alternate line.

SET MARGIN

Syntax: SET MARGIN side column

When Kermit-370 sends a text file, each line may be truncated on  the  left  or

right (or both) at fixed column numbers.  Only the text from the left margin to

the right margin (inclusive) will be sent, and any trailing blanks in the trun-

cated lines will be stripped.  A value of zero for either margin disables trun-

cation on that side.

SET PARITY

Syntax: SET PARITY MARK or NONE

Transparent-mode ASCII data received from a "SERIES1" or "GRAPHICS" device will

typically have either all Mark parity (seven data bits with the eighth bit set)

or no parity (eight data bits).  Kermit-370 must know which kind of  parity  to

expect in order to calculate checksums properly.  Since Kermit-370 does not ac-

tually verify parity, the other possible variants (ODD, EVEN,  and  SPACE)  are

lumped  together  with  MARK  parity  for the purpose of this subcommand, which

merely chooses between 7-bit and 8-bit data transfer.  The default is MARK.

7.2. Kermit-370 Subcommands                                            Page 190

-------------------------------------------------------------------------------

SET PROMPT

Syntax: SET PROMPT string

This defines the character string that Kermit-370 displays when  asking  for  a

subcommand.   The prompt may be any string of up to 20 characters.  The default

is the name of the system-specific version of  Kermit-370  followed  by  a  ">"

sign, e.g., Kermit-CMS>.

SET RETRY

Syntax: SET RETRY INITIAL or PACKETS number

Kermit-370  resends its last packet after receiving a NAK or bad packet, but it

eventually gives up after repeated failures or the same packet.  The  limit  on

retries  can  be  set  separately for the initial packet exchange (Send-Init or

server-mode command) and for ordinary packets.  The default for INITIAL  is  16

and for PACKETS, 5.  Either limit can be set to any positive value.

SET SYSCMD

Syntax: SET SYSCMD ON or OFF

ON      If the user enters a command string which is not a valid Kermit subcom-

mand, Kermit-370 will pass the string along to the host operating  sys-

tem  for  execution.   If the string is rejected by the system as well,

Kermit will report it as an invalid Kermit subcommand.  Otherwise, Ker-

mit  will  assume  the  string  was intended as a host command and will

simply report the completion code if non-zero.

OFF     Invalid Kermit subcommands are simply rejected as such.    System  com-

mands  may  be  executed, of course, but only by specifying the generic

prefix "HOST" or the appropriate system-specific prefix, such as CMS or

TSO.  (Default.)

SET TABS-EXPAND

Syntax: SET TABS-EXPAND ON [list] or OFF

ON      Tab  characters  in  incoming  TEXT  files  are replaced by one or more

blanks to bring the record size up to the next higher multiple of eight

for  each  tab.   If tab settings other than columns 1, 9, 17, etc. are

desired, they may be specified  explicitly  in  a  list  following  the

keyword "ON".  Items in the list may be separated by spaces or commas.

OFF     Incoming tabs are retained.  (Default.)

7.2. Kermit-370 Subcommands                                            Page 191

-------------------------------------------------------------------------------

SET TAKE ECHO

Syntax: SET TAKE ECHO ON or OFF

ON      Subcommands are echoed to the terminal as they are executed from a TAKE

file.

OFF     Subcommands from a TAKE file are executed "silently."  (Default.)

SET TAKE ERROR-ACTION

Syntax: SET TAKE ERROR-ACTION CONTINUE or HALT

CONTINUE   Execution continues in a TAKE file regardless of  illegal  commands,

except in server mode.  This is the default.

HALT       A  command error in a TAKE file causes immediate exit to Kermit sub-

command level.

SET TEST

Syntax: SET TEST ON or OFF

ON      Allow setting the START-OF-PACKET and other special characters  to  any

value, and suppress checksum testing on received packets.

OFF     Normal operation.  (Default.)

SET TTABLE

Syntax: SET TTABLE ON or OFF

ON      The translation that undoes the terminal controller's ASCII/EBCDIC con-

version comes from the TATOE and TETOA tables, rather than the ATOE and

ETOA tables (which are used only for translating disk files).  This op-

tion has no effect when there is no translation  built  into  the  con-

troller, i.e., with SERIES1 and GRAPHICS connections.

OFF     The  ATOE  and ETOA tables are used for all translations by Kermit-370.

(Default.)

SET WARNING

Syntax: SET WARNING ON or OFF

ON      If an incoming file has the same filespec as an existing file on  disk,

Kermit  will  attempt  to rename the incoming file so as not to destroy

(overwrite) the pre-existing one.

OFF     Upon filename collision, the existing file will be erased and  replaced

by the incoming file.  (Default.)

7.2. Kermit-370 Subcommands                                            Page 192

-------------------------------------------------------------------------------

SET 8-BIT-QUOTE

Syntax: SET 8-BIT-QUOTE char or ON or OFF

This  controls  whether eighth-bit prefixing is done and can be used to specify

the character to be used.  This is one of only two  Send-Init  parameters  that

cannot be SET separately for SEND and RECEIVE.

char    Eighth-bit prefixing will be done using char, provided the other Kermit

agrees.

ON      Eighth-bit prefixing will be done, provided the other Kermit explicitly

requests it (and specifies the character).

OFF     Eighth-bit prefixing will not be done.  (Default.)

SET SEND/RECEIVE

The  following  parameters  can be set either as SEND or RECEIVE options.  As a

rule, in each pair, one is the operational value, and  the  other  is  used  to

change  the default for Send-Init packets received from the other Kermit and to

set up parameter values as if the other Kermit had specified them on  the  pre-

vious  exchange.    When both values are described, the operational one will be

first.  After a transfer, the operational values will  be  unchanged,  but  the

others  (as  displayed  by  SHOW)  will reflect the parameters specified by the

other Kermit.  The underlying defaults established by previous SET  subcommands

will still be in effect.  In the syntax descriptions, mode is SEND or RECEIVE.

END-OF-LINE

Syntax: SET mode END-OF-LINE number

RECEIVE should not be changed.

SEND  may be needed to establish contact.  If the other system needs packets to

be terminated by anything other than carriage return, specify the decimal value

of  the  desired  ASCII character.  number must be in the range 0-31 (decimal).

The default is 13 (CR).

PACKET-SIZE

Syntax: SET mode PACKET-SIZE number

RECEIVE defines number as the maximum length for incoming packets.   The  valid

range  is  26-9024,  but 94 is the limit for normal short-packet protocol.  The

default is 80.  In practice, the size may be limited by hardware  and  program-

ming considerations.  See the system-specific chapters for details.

SEND  might  be  needed  for  sending  files  to  a minimal Kermit that neither

specifies a buffer size in the Send-Init sequence nor can  accept  the  default

(80).  This parameter has no other function and is meaningful only in the range

26-94.

7.2. Kermit-370 Subcommands                                            Page 193

-------------------------------------------------------------------------------

PAD-CHAR

Syntax: SET mode PAD-CHAR number

RECEIVE defines number as the character to be used by the other Kermit for pad-

ding  packets.   The character must be an ASCII control character (in the range

0-31).  The default is 0 (NULL).  This option is seldom useful.

SEND may be needed to establish contact if the other Kermit (or  the  transmis-

sion line) needs padded packets.

PADDING

Syntax: SET mode PADDING number

RECEIVE  defines  the  number  of pad characters to be used for padding packets

from the other Kermit.  This number may be anywhere from 0 to 94.  The  default

is 0.  This option is seldom useful.

SEND  may  be needed to establish contact if the other Kermit (or the transmis-

sion line) needs padded packets.

QUOTE

Syntax: SET mode QUOTE char

SEND indicates a printable character for prefixing (quoting) control characters

and  other prefix characters.  The only good reason to change this would be for

sending a file that contains many "#" characters (the normal control prefix) as

data.    It  must  be  a  single  character  with  ASCII  value 33-62 or 96-126

(decimal).

RECEIVE would be needed only for talking to  a  crippled  Kermit  that  uses  a

non-standard quoting character, but does not admit it.

START-OF-PACKET

Syntax: SET mode START-OF-PACKET number

RECEIVE  defines  number  as  the character to be expected to mark the start of

packets from the other Kermit.  The character must be an ASCII control  charac-

ter (in the range 0-31).  The default is 1 (SOH).  This may be needed to estab-

lish contact.

SEND may also be needed to establish contact.  It defines number as the charac-

ter to be used to mark outgoing packets.

7.2. Kermit-370 Subcommands                                            Page 194

-------------------------------------------------------------------------------

TIMEOUT

Syntax: SET mode TIMEOUT time

RECEIVE  defines the time in seconds the other Kermit is to wait for a response

from Kermit-370 before resending a packet.  The default is 5.   A  value  of  0

means the other Kermit should wait indefinitely.

SEND  defines the time in seconds Kermit-370 is to wait for a response from the

other Kermit before resending a packet.  The default is 0.

THE SHOW SUBCOMMAND

Syntax: SHOW [option]

The SHOW subcommand displays the values of all parameters that can  be  changed

with  the  SET  subcommand, except for ATOE, ETOA, TATOE, and TETOA (for those,

see the TDUMP subcommand).  If specified, option can be a particular  parameter

or the keyword "ALL" (the default).  Groups of parameters, such as SEND, can be

displayed by requesting the group name, or  individual  sub-parameters  can  be

displayed by specifying the complete name.  For example,

SHOW RECEIVE EOL

will  display  the  decimal value of the packet terminator that Kermit-370 cur-

rently expects, i.e., 13.  Similarly,

SHOW FOREIGN

will display the character strings currently in use for prefix  and  suffix  on

each outgoing filespec.

THE STATUS SUBCOMMAND

Syntax: STATUS

This  subcommand displays information about the previously executed subcommand.

The response will include either the appropriate error message or  the  message

"No  errors".    The  initial status is "No file transfers yet".  If the status

reflects an error condition, the name of the last  file  used  (excluding  TAKE

files)  will be displayed as well.  If the error was detected by the other Ker-

mit, the message will be "Micro aborted" followed by the text  from  the  Error

packet.    Conversely, if Kermit-370 detected the error, the text of the status

message will have constituted the error packet sent out.  In any case,  if  the

last  file transfer was cancelled (by virtue of an attribute mismatch or manual

intervention), the reason for cancellation is displayed.  Also,  if  the  error

occurred in disk I/O, any available explanatory information is displayed.  Nor-

mally, the error status is altered only when a  transfer-initiating  subcommand

(SEND  or RECEIVE) is executed, but in server mode every subcommand is received

through a transfer from the other Kermit and may affect the status (except  the

STATUS subcommand itself, of course).  When Kermit-370 has been forced to trun-

cate one or more records in a RECEIVE operation (because of the current maximum

record  length),  the number of records truncated is reported.  The status dis-

play also includes throughput statistics for the last transfer: number of files

7.2. Kermit-370 Subcommands                                            Page 195

-------------------------------------------------------------------------------

sent,  duration,  number  of  packets,  number  of  retries,  and  averages  of

bytes/packet and bytes/second.    These  last  two  quantities  are  calculated

separately for bytes sent and received on the communication (including padding,

if any), and the last quantity is also calculated on the basis of the number of

bytes read from or written to disk.

Finally, if retries were necessary, Kermit-370 computes the optimum packet size

assuming the retries to have been due to sparse, Poisson-distributed bursts  of

noise.  This is the same heuristic optimum that Kermit-370 computes and uses as

an alternative packet-size limit when sending long packets.

THE TDUMP SUBCOMMAND

Syntax: TDUMP table-name or NAMES

This displays the contents of table-name.  The same table can be modified using

the  SET  subcommand.  The ATOE, ETOA, TATOE, and TETOA tables can presently be

displayed and changed.  Alternatively, the filespec of each file  sent  in  the

last transfer can be displayed.

THE GIVE SUBCOMMAND

Syntax: GIVE table-name filespec

This compares the named table with its default values and saves the differences

in the form of a TAKE file consisting of SET subcommands that would convert the

default  into  the  current  arrangement.  ATOE, ETOA, TATOE, and TETOA are the

available tables.  The details of the filespec are system-dependent, but  those

details will, in general, be the same as for the TAKE subcommand.

THE HOST SUBCOMMAND

Syntax: HOST text of command

This  issues  a  command  to the host operating system from Kermit-370.  When a

command returns a  non-zero  completion  code,  the  code  will  be  displayed.

Generally,  the  name of the system (e.g., CMS) is treated as a synonym for the

HOST subcommand.

THE KERMIT SUBCOMMAND

Syntax: KERMIT text of subcommand

This is provided for redundancy as the  counterpart  of  the  HOST  subcommand.

Kermit-370  executes  the  specified text as a Kermit subcommand just as if the

LOCAL prefix had been entered.

7.2. Kermit-370 Subcommands                                            Page 196

-------------------------------------------------------------------------------

THE ECHO AND XECHO SUBCOMMANDS

Syntax: [X]ECHO line

These subcommands type the line back at the user.  The line may contain control

characters  or  any desired text, including upper or lower case.  These subcom-

mands may be used, for example, to test the ASCII/EBCDIC translate tables or to

issue coded commands to the user's terminal.  XECHO differs from ECHO primarily

in that it uses transparent mode if CONTROLLER is SERIES1 or GRAPHICS.  It also

offers  its  own brand of control-character quoting, using the "^" character to

indicate that only the five low-order bits of the ASCII codes are to  be  used.

Thus,  "^a",  "^A", and "^!" are all translated to SOH (CTRL-A), while "^[" be-

comes ESC.  However, there must be one exception for "^" itself: "^>" and  "^~"

are both translated to RS (CTRL-^), but "^^" becomes just "^".

THE TYPE AND XTYPE SUBCOMMANDS

Syntax: [X]TYPE filespec

These  subcommands  type  the named file.  XTYPE differs from TYPE primarily in

that it uses transparent mode if CONTROLLER is SERIES1 or GRAPHICS,  and  sends

the data in bursts no larger than the current SEND PACKET-SIZE.  TYPE is effec-

tively a synonym for (and allows the same options as) the host  system  command

for listing files, but XTYPE merely sends the file "raw".

7.3. Before Connecting to the Mainframe

Several  flags  must be set on the micro version of Kermit before connecting to

an IBM 370 system as a "TTY" device.  You should set the LOCAL-ECHO flag to  ON

(to  indicate  half-duplex).  This is the norm but not true in absolutely every

case; if each  character  appears  twice  on  your  terminal  screen,  set  the

LOCAL-ECHO  flag  OFF.  FLOW-CONTROL should be set to NONE, and on some systems

HANDSHAKE should be set to XON.  The parity should  be  set  according  to  the

system's specifications.  On some micro versions of Kermit, all of the above is

done in one step using the DO IBM macro (or SET IBM ON).  Set the baud rate  to

correspond to the line speed.

Connecting  through a "SERIES1" or "GRAPHICS" device also requires that certain

flags be set on the micro version of Kermit.  You  should  set  the  LOCAL-ECHO

flag to OFF (to indicate full-duplex).  FLOW-CONTROL should be set to XON/XOFF,

and HANDSHAKE should be set to OFF.  For many systems, the PARITY should be set

to EVEN.  Set the baud rate to correspond to the line speed.

One  exception  to these rules is the case where the micro Kermit is attempting

automated  file  transfer,  e.g.,  downloading  several  separate  files   from

Kermit-370  running  in server mode.  In fact, under those circumstances, hand-

shaking is necessary even with "SERIES1" connections, and the two Kermits  must

be  instructed to adopt a common handshake character (e.g., by SET HANDSHAKE 10

to Kermit-370 and SET HANDSHAKE LF to the micro).

In any case, you should make sure that either the micro  Kermit  or  Kermit-370

will  provide  timeouts  during file transfers (if not both).  Some versions of

Kermit-370 (notably CMS) cannot provide timeouts, and you may need to  set  the

TIMER to ON in the micro.

7.4. After Returning from Kermit-370                                   Page 197

-------------------------------------------------------------------------------

7.4. After Returning from Kermit-370

When  Kermit-370  receives a QUIT or EXIT subcommand or finishes the subcommand

or subcommands specified in the original command string  that  invoked  Kermit,

control is returned to the caller.  Before returning, Kermit-370 closes any ac-

tive TAKE files (the EXIT or QUIT subcommand may be issued from a  TAKE  file).

On  return,  the completion code is set from the current error status according

to the codes in Table 7-0.

-------------------------------------------------------------------------------

Code  Symbol  Error Message

0    NOE     No errors

1    NFT     No file transfers yet

2    TRC     Transfer cancelled

3    USC     Invalid server command

4    TIE     Terminal I/O error

5    BPC     Bad packet count or chksum

6    IPS     Invalid packet syntax

7    IPT     Invalid packet type

8    MIS     Lost a packet

9    NAK     Micro sent a NAK

10    ABO     Micro aborted

11    FNE     Invalid file name

12    FNF     File not found

13    FUL     Disk or file is full

14    DIE     Disk I/O error

15    MOP     Missing operand

16    SYS     Illegal system command

17    KCE     Kermit command error

18    TIM     No packet received

19    RTR     Records truncated

20    COM     Bad communication line

Table 7-1:   Error messages and codes for Kermit-370

-------------------------------------------------------------------------------

7.5. What's New

Below is a list of the additions in Version 4.0 of Kermit-370:

1. Code reorganization into generic 370 and system-specific sections.

2. Optional separate translation tables for  counteracting  the  system

conversion of terminal I/O.

3. New GIVE subcommand for saving a modified translation table.

4. A  new,  RAW debug mode for recording the packet traffic as actually

sent and received on "GRAPHICS" and "SERIES1" devices.

7.5. What's New                                                        Page 198

-------------------------------------------------------------------------------

5. Preservation of the case of subcommands  as  typed,  with  uppercase

conversion of only those words that must be uppercase.

6. New  SET  MARGIN  subcommand  for limiting the width of a file to be

sent.

7. Settable tab  stops  for  Kermit's  conversion  of  tabs  to  spaces

(alternative to the default 1, 9, 17, etc.).

8. Replace SET SERIES1 subcommand with new SET CONTROLLER.  Support for

multiple terminal controller types.

9. New DIRECTORY and HOST subcommands following Kermit standard.

10. Combination of file-attribute SET subcommands (FILE-TYPE, LRECL, and

RECFM) into a new group SET FILE.

11. Separate retry limits for initial and subsequent packet exchanges.

12. Pad binary records on disk with nulls, rather than blanks.

13. Automatically tune packet length when sending long packets according

to heuristic optimum based on sparse  Poisson  statistics,  provided

that transmission errors do occur.

14. Expand  STATUS  report  to  include  the number of files in the last

transfer, throughput statistics,  heuristic  optimum  packet  length

(when  long packets are enabled), and the reason for any file rejec-

tion based on A-packets.

15. New subcommand TDUMP NAMES to display the list of files sent in  the

last transfer.

16. Add file creation date to A-packet repertoire.

17. REMOTE COPY and REMOTE RENAME commands to a server at the other end.

18. Allow long packets through a 7171 with VTAM.

19. New  type D-BINARY for binary files with undelimited variable-length

records.

20. SET 8-BIT-QUOTE.  Allow 8-bit data where possible via SET PARITY.

21. SET SYSCMD, so that Kermit can be told to try "illegal"  subcommands

as host system commands instead of just rejecting them.

22. SET PROMPT subcommand.

23. Do not forget parameters specified by the other Kermit in I-packets.

24. Keep  track  of  truncated  records  during  a RECEIVE operation and

report the count in STATUS; also  call  truncation  an  error  after

everything is received.

25. SET  HANDSHAKE  subcommand  to alter or suppress handshake character

7.5. What's New                                                        Page 199

-------------------------------------------------------------------------------

Kermit-370 sends out after each packet.

Both SEND and GET prompt the user for native and foreign filespecs  if  no  ar-

guments are entered.

7.6. What's Missing

Work  on  Kermit-370 will continue.  Features that need to be improved or added

include:

- Detect file properties from Attribute packets  and  allow  overriding

current parameter settings.  Also implement file archiving.

- Add SET REPEAT subcommand.

- Improve Kermit-370 operation as a local Kermit.

- Recover  from sudden line degradation by retransmitting partial pack-

ets.

- System-specific upgrades; see the respective chapters for details.

8. IBM VM/CMS KERMIT                                                   Page 200

-------------------------------------------------------------------------------

8. IBM VM/CMS KERMIT

Program:      John Chandler (Harvard/Smithsonian Center for Astrophysics); con-

tributions  from Vace Kundakci and Daphne Tzoar (Columbia U), Bob

Shields (U. Maryland), Victor Lee (Queens U.),  Gary  Bjerke  (U.

Texas  at  Austin),  Greg  Small  (UC  Berkeley),  Clark  Frazier

(Harvard Bus. Sch.), Bob Bolch and Steve Blankinship  (Triangle),

Ron Rusnak (U. Chicago), Andre Pirard (U. Liege)

Language:     IBM/370 Assembler

Documentation:

John Chandler (CfA)

Version:      4.0 (88/1/31)

Date:         1988 February

Kermit-CMS Capabilities At A Glance:

Local operation:                   No

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Z interruption:                Yes (through micro)

Filename collision avoidance:      Yes

Can time out:                      No

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                No

Communication settings:            No

Transmit BREAK:                    No

Transaction logging:               Yes

Session logging:                   No

Raw transmit:                      Yes (no prompts)

Sliding window:                    No

Long packets:                      Yes

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         Yes

Advanced commands for servers:     Yes

Local file management:             Yes

Handle Attribute Packets:          Yes

Command/init files:                Yes

Command macros:                    No

Kermit-CMS is a member of the generic Kermit-370 family and shares most of  the

features and capabilities of the group.  As its name implies, Kermit-CMS is the

version of Kermit-370 that runs under the VM/CMS operating system.  The primary

documentation  for  Kermit-CMS  is  actually  the  chapter on Kermit-370, which

describes general  properties;  the  present  chapter  assumes  the  reader  is

familiar  with  that material.  Only the details specific to CMS operation will

be discussed here, e.g., command syntax relating to the CMS file system or com-

mands not offered in general by Kermit-370.

8. IBM VM/CMS KERMIT                                                   Page 201

-------------------------------------------------------------------------------

CMS Specifics of Kermit-370:

Global INIT file:                  SYSTEM KERMINI *

User INIT file:                    <userid> KERMINI *

Debug packet log:                  KER LOG A1

Server reply log:                  KER REPLY A1

Maximum packet size:               1913

Maximum disk LRECL:                65535

8.1. The VM/CMS File System

The  features  of  the CMS file system of greatest interest to Kermit users are

the format of file specifications (or filespecs) and the  concept  of  records.

The latter is described in the Kermit-370 chapter.

The VM/CMS filespec takes the form

filename filetype filemode

(often  abbreviated  FN  FT  FM).    The filename and filetype are one to eight

characters each.  The name field is the primary identifier for  the  file,  and

the  type  is an indicator which, by convention, tells what kind of file it is.

For instance, TEST FORTRAN is the source  of  a  Fortran  program  named  TEST.

MODULE  is  the filetype for executable programs (as distinct from object code,

which has a filetype of TEXT!).  Although some operating systems  consider  the

filetype optional, VM/CMS requires a type for each file.  Therefore, Kermit-CMS

supplies a default type of "$" for any received file if no type is provided  by

the  remote  system.   The same default is used for a missing filename.  At the

same time, the FN and FT are forced to conform to CMS rules in other  respects.

The  FN  and  FT  may contain, in any order, uppercase letters, digits, and the

special characters "$" (dollar sign), "#" (pound  sign),  "@"  (at  sign),  "+"

(plus),  "-" (hyphen), ":" (colon), and "_" (underscore).  Other characters may

be not be included.  If an invalid character is found in the FN or FT field, it

is  replaced  by  an underscore (or converted to uppercase if it is a lowercase

letter).  Also, both FN and FT are truncated, if necessary,  to  eight  charac-

ters.

The  filemode,  which consists of a letter and a number, is similar to a device

specification on microcomputer systems: FN FT FM would translate to FM:FN.FT in

CP/M  or MS-DOS if the filemode number is ignored.  Indeed, the filemode number

is more properly an attribute of a file than part of its name -- no  two  files

can  co-exist  with  names  that  match  all but the filemode number.  Even the

filemode letter is not a fixed part of the filespec because the same  mini-disk

could  be  accessed  under a different mode letter.  In some ways, the filemode

letter is also like a disk directory designator, since many such mini-disks may

reside  on  the  same  disk  drive.  For this reason, the Kermit concept of the

"working directory" is  equated  with  a  particular  disk  mode  letter  under

Kermit-CMS.    The  current  "working  directory" is, thus, the "home" filemode

(normally "A", which is the primary user mini-disk under CMS), and file  trans-

fers take place preferentially to and from the "home" disk.  If the filemode is

omitted from a filespec when sending, the "home" disk  is  normally  used,  but

there  is  an  option  for  using  a default of "*" instead.  In this case, the

user's disks are scanned according to the search order and the first occurrence

of  the  file  is  the  one  that  is  sent.  If the filemode is omitted from a

filespec when receiving, the "home" disk is used with a filemode number of "1".

8.1. The VM/CMS File System                                            Page 202

-------------------------------------------------------------------------------

To provide compatibility with other operating systems, when Kermit-CMS sends  a

file,  it  ordinarily  makes a file header with only the filename and filetype.

It also converts the intervening blank to a period.  On the other  hand,  extra

information may be added by way of the SET FOREIGN subcommand.

VM/CMS  allows a group of files to be specified in a single filespec by includ-

ing the special "wildcard" characters "*" and "%".  A "*" matches any string of

characters  (even  a  null  string) from the current position to the end of the

field; a "%" matches any single character.  Here are some examples:

* COBOL A   All files of type COBOL (all COBOL source files) on the A disk.

F* * *      All files whose names start with F.

% * B       All B-disk files with one-character FN's.

CMS files, like those in other IBM 370 systems, are  record-oriented  (see  the

introduction  to the Kermit-370 chapter).  In particular, CMS files are charac-

terized by record format (RECFM), which may be fixed-length or  varying-length,

and by maximum record length (LRECL).  The size of record blocks is irrelevant,

however, because CMS performs the blocking and deblocking operations  automati-

cally  and  transparently, including the spanning of records across block boun-

daries.  Records in CMS files may be up to 65535 bytes long.

Another file system feature of occasional interest is the  means  of  reporting

errors.    When Kermit-CMS encounters a disk error, it records the function and

error code for inclusion in the STATUS report.  The explanations can  be  found

in  the  CMS  reference  manual under the FSREAD and FSWRITE macros (which cor-

respond to the RDBUF and WRBUF functions).

8.2. Program Operation

At startup time, Kermit-CMS looks for two initialization files, SYSTEM  KERMINI

and  <userid>  KERMINI  (where  <userid> is the user's logon ID).  If either of

these files exists on more than one disk, it will be read and executed from the

first  copy in the search order.  The file SYSTEM KERMINI should be placed on a

publicly accessible disk by a systems  programmer,  preferably  the  same  disk

where  the  Kermit executable module is kept.  The file <userid> KERMINI can be

maintained by the user on any convenient disk.

One important distinction between  Kermit-CMS  and  other  Kermits  is  that  a

program  running  under  VM/CMS is unable to interrupt a read on its "console".

This means that the CMS version of Kermit  cannot  time  out  after  sending  a

packet.    The  only way to time out is from the other side:  typing a carriage

return to the local Kermit causing it to retransmit  its  last  packet,  or  an

automatic timeout as provided by most other Kermits.

Five  CP  SET  parameters  (MSG, IMSG, WNG, ACNT, and TIMER) are set OFF during

protocol mode (and restored afterwards) to prevent CP from interrupting any I/O

in  progress, and RUN is set ON to ensure that Kermit can recover from acciden-

tal attention interrupts.  Also, on a TTY line, the TERMINAL  LINESIZE  is  set

OFF  to  prevent CP from inserting carriage return-linefeed pairs into packets,

TERMINAL SCROLL is set to CONT to prevent CP pauses, LINEDIT is set OFF to  en-

sure  that all characters are taken literally, and the CMS user terminal trans-

lation tables (established via the CMS SET INPUT and OUTPUT commands) are  tem-

8.2. Program Operation                                                 Page 203

-------------------------------------------------------------------------------

porarily  suppressed for both short and long packet protocols.  The settings in

effect when Kermit starts up are saved as a sort of  "normal"  status  snapshot

(as  opposed  to the "protocol" status just described).  The protocol status is

selected whenever Kermit enters protocol mode and also after Kermit executes  a

CP  command  in  server mode.  Similarly, normal status is selected when Kermit

leaves protocol mode and before Kermit executes a CP command  in  server  mode.

Note:  if  Kermit  is interrupted in the midst of a transfer or while in server

mode, these parameters  will  be  left  with  peculiar  settings  (namely,  the

protocol status), and they may need to be restored by hand.

CMS is different from some other IBM mainframe systems in that allows a program

to take control of prompting and synchronization on "TTY"  lines.    Kermit-CMS

takes  advantage of this option, and it is not, in general, necessary to enable

handshaking on the micro Kermit before connecting to  CMS.    In  other  words,

handshaking  should  be  suppressed  for  both "TTY" and "SERIES1" devices (the

micro Kermit should have HANDSHAKE set OFF, and Kermit-CMS  should  have  HAND-

SHAKE  set  to  0).    Since  the generic Kermit-370 default handshake (XON) is

retained in Kermit-CMS, the subcommand "SET HANDSHAKE 0" is  a  good  candidate

for inclusion in SYSTEM KERMINI.

Interactive Operation:

To  run Kermit-CMS interactively, invoke the program from CMS by typing KERMIT.

When you see the prompt,

Kermit-CMS>

you may type a Kermit subcommand.  When the subcommand completes, Kermit issues

another  prompt.    The cycle repeats until you exit from the program.  For ex-

ample:

KERMIT

Kermit-CMS Version 4.0 (88/1/31)

Enter ? for a list of valid commands

Kermit-CMS>send foo *

Files with fn FOO are sent

Kermit-CMS>receive test spss

File is received and called TEST SPSS A1

Kermit-CMS>exit

The prompt string under CMS is truly interactive.  In other words,  the  string

(without  carriage  return or linefeed) appears only when fresh input is needed

from the terminal.  If, for example, Kermit is invoked  after  several  subcom-

mands  have  been  stacked  up, the stack is read and executed before the first

prompt appears.

8.2. Program Operation                                                 Page 204

-------------------------------------------------------------------------------

Command Line Invocation:

Kermit-CMS may also be invoked with command line arguments from CMS.   The  ar-

guments  are  interpreted  as  one or more subcommands to be executed by Kermit

after completion of the initialization.  For instance:

KERMIT send test fortran

or

KERMIT set debug on # set file binary # server

Kermit will exit and return to CMS after completing the specified subcommand or

subcommands.    Note  that several commands may be given on the command line as

long as they are separated by the LINEND character, which is pound sign in this

case.  Note that the LINEND is a concept of CP, rather than Kermit, and applies

only to commands entered from the terminal and only when LINEDIT is on.  A com-

mand line may contain up to 130 characters.

EXEC Operation:

Like  other  CMS  programs, Kermit-CMS may be invoked from a CMS EXEC.  Subcom-

mands can be passed to Kermit using the program stack and/or command  line  ar-

guments.   For example, to start up Kermit-CMS and have it act as a server, in-

clude the line:

KERMIT server

To pass more than one subcommand, they must be stacked in the  order  in  which

they  are to be executed.  To start up a Kermit-CMS server with a three charac-

ter CRC, include:

&STACK set block 3

&STACK server

KERMIT

Another way of setting up multiple subcommands would be to collect the  subcom-

mands  into a TAKE file and then issue the TAKE subcommand via the command line

or program stack.  Of course,  EXEC's  may  be  executed  from  Kermit,  either

directly  or  from  a TAKE file, and Kermit subcommands, in turn, may be issued

from EXEC's as long as Kermit is active.  See  the  TAKE  subcommand  for  more

details.

Server mode:

Command  execution  in server mode is different in several respects from normal

operation.  First of all, some Kermit subcommands are not allowed (see the list

of  subcommands  in  the  Kermit-370 chapter).  Moreover, command errors always

terminate any active TAKE file.  Also, commands other than CP commands run in a

special  environment  with RUN ON, TIMER OFF, and so forth.  Another difference

is that Kermit intercepts all SVC instructions in order to  catch  console  I/O

and  transmit  the data to the local Kermit as text packets.  Since Kermit does

not emulate the substitution functions of the LINEDIT macro, some messages will

appear  rather  cryptic.   A more serious problem with this redirection is that

8.2. Program Operation                                                 Page 205

-------------------------------------------------------------------------------

some VM/CMS system commands may issue console I/O directly to CP, so that  some

messages  never  appear  to the local Kermit (except, perhaps, as bad packets).

For non-TTY terminals, such messages are stacked up in the console output queue

and appear all at once when Kermit returns from server mode.

8.3. Kermit-CMS Subcommands

Kermit-CMS  supports all the subcommands described in the corresponding section

of the Kermit-370 chapter.  In addition, there are two more, both of which  can

be  issued  as  remote  Kermit commands when Kermit-CMS is in server mode.  The

first is CMS, which is just a synonym for the generic  HOST  subcommand.    The

second  is  CP,  which  specifically  issues  a  command  to  CP.  In most cir-

cumstances, the latter is not needed, since CMS will pass along CP commands  to

CP.

The remainder of this section concentrates on the subcommands that have special

form or meaning for Kermit-CMS.  See also the chapter on Kermit-370 for further

details.

THE SEND SUBCOMMAND

Syntax: SEND filespec [foreign-filespec]

The SEND subcommand causes a file or file group to be sent from CMS to the Ker-

mit on the other system.  filespec takes the form:

filename filetype [filemode]

but the filemode if optional only if the foreign-filespec is omitted.

filespec may contain the wildcard characters "*" or "%".  If filespec  contains

wildcard  characters then all matching files will be sent.  If, however, a file

exists by the same name on more than one disk, only the  first  one  Kermit-CMS

encounters, according to the disk search order, is sent.  See also the CWD sub-

command.

The foreign-filespec, if any, is used for the file header of the outgoing file,

replacing  the  usual  filename.filetype  copied from the CMS filespec.  It may

take one of two forms:

filename filetype

or

arbitrary-string

Normally, this form of the SEND subcommand is used only for  single  files  be-

cause  the  foreign-filespec  is  used  only  for  the  first  file  of a group

(subsequent files having default headers).  However, in the two-token  form  of

the  foreign-filespec either the name or type may be an Equals sign "=" to sig-

nify that the corresponding CMS name or type is to  be  retained  in  the  file

header.   In that case, the partial renaming carries through an entire group of

files.  It is the user's responsibility to prevent such partial  renaming  from

sending duplicate file headers within a file group.

8.3. Kermit-CMS Subcommands                                            Page 206

-------------------------------------------------------------------------------

Although the file transfer cannot be cancelled from the CMS side, Kermit-CMS is

capable of responding to "cancel file" or "cancel batch" signals from the local

Kermit;  these  are typically entered by typing Control-X or Control-Z, respec-

tively.

THE RECEIVE SUBCOMMAND

Syntax: RECEIVE [filespec]

The RECEIVE subcommand tells Kermit to receive a file or file  group  from  the

other system.  You should then issue a SEND subcommand to the other Kermit.

The format of filespec is:

filename filetype [filemode]

If  the  optional filespec is omitted, Kermit-CMS will use the name(s) provided

by the other Kermit.  If that name is not a legal  CMS  file  name,  Kermit-CMS

will  delete  excess  characters  and  will change illegal characters to under-

scores.  A filespec in the subcommand indicates what  name  the  incoming  file

should be given.  The filespec may include a filemode to designate the destina-

tion disk.  If none is provided, the file will be saved on the "home" disk with

filemode  number  "1".    If  you  want  to  use  the same name but a different

filemode, specify "= = FM".  Wildcards may not be used.

If the optional filespec is provided, but more than one file arrives, the first

file  will be stored under the given filespec, and the remainder will be stored

under their own names on the "home" disk.  If, however, "= = FM" is  used,  all

files will be placed onto the specified disk.

When  the  record  format  is  "F", any received record longer than the logical

record length (LRECL) will be truncated, and shorter records  will  be  padded.

The  padding  character  is a blank for text files and a null for binary files.

Received binary (but not V-binary  or  D-binary)  files  are  treated  as  byte

streams  and  broken up into records all of the logical record length.  See the

SET FILE TYPE, SET FILE LRECL, and SET FILE RECFM subcommands.

If an error occurs during the file  transfer,  as  much  of  the  file  as  was

received  is  saved on disk.  If the sending of a file is cancelled by the user

of the foreign system, Kermit-CMS will discard whatever had arrived, unless AP-

PEND is ON or INCOMPLETE is KEEP.

If the incoming file has the same name as an existing file, and WARNING is OFF,

the original file will  be  overwritten.    If  WARNING  is  set  ON,  however,

Kermit-CMS will change the incoming name so as not to obliterate the pre-exist-

ing file.  It attempts to find a unique  name  by  successively  modifying  the

original  and checking for the existence of such a file at each step.  The pro-

cedure begins by truncating the filetype to six characters  if  necessary,  and

then  appending  "$0".  If a file by that name exists, Kermit then replaces the

"0" with a "1".  It continues in this manner up to "9", and if an  unused  name

cannot be found, the transfer fails.

8.3. Kermit-CMS Subcommands                                            Page 207

-------------------------------------------------------------------------------

THE GET SUBCOMMAND

Syntax: GET foreign-filespec [filespec]

The  GET subcommand tells Kermit to request a file or file group from the other

system, which must have a Kermit running in server mode.  The syntax is compli-

cated  by  the  allowance of two forms for the foreign-filespec, just as in the

SEND subcommand.    Here  the  parsing  is  based  on  the  number  of  "words"

(blank-delimited  strings)  in  the  subcommand argument, which can be anything

from one to five.  If the number is anything but four,  the  interpretation  is

unambiguous,  but when there are four words, the first word plays the key role.

If it has more than eight characters or contains a "." or "/", it is assumed to

be  the whole foreign-filespec; otherwise, it is assumed to be the first of two

words that, when joined by a ".", make up the filespec on the other system.

THE TAKE SUBCOMMAND

Syntax: TAKE filespec

Execute Kermit subcommands from the specified file, where filespec has the for-

mat  fn [ft [fm]].  The default filetype is "TAKE", and the default filemode is

"*".

Kermit subcommands may also be executed from CMS EXEC's, so that the TAKE  sub-

command  is,  in a sense, superfluous under VM/CMS.  In CMS terminology, Kermit

establishes a Kermit subcommand environment, and EXEC's written in  EXEC  2  or

REXX  may  invoke subcommands within that environment.  For example, to display

the current packet checksum type, an EXEC 2 would issue

&SUBCOMMAND KERMIT SHOW BLOCK-CHECK

and a REXX macro would issue

Address KERMIT 'SHOW BLOCK-CHECK'

There is one important difference between executing a TAKE file  and  an  EXEC:

the  former may issue a QUIT or EXIT subcommand, but the latter may not.  Also,

a Kermit subcommand issued from an EXEC returns a completion code according  to

the  current  error status (see the table under "After Kermit Completes" in the

Kermit-370 chapter).  An EXEC could therefore be set up to react  appropriately

to file transmission errors or other unpredictable events.

THE SET SUBCOMMAND

Syntax: SET parameter [value]

The  SET subcommand establishes or modifies various parameters controlling file

transfers.  The following SET parameters are available in Kermit-CMS,  but  not

in Kermit-370 in general:

DESTINATION         "Home" disk.

FILE

LRECL             Logical Record length for incoming file.

RECFM             Record format for incoming files.

8.3. Kermit-CMS Subcommands                                            Page 208

-------------------------------------------------------------------------------

SEARCH-ALL          Determine the default disk search scope.

SET DESTINATION

Syntax: SET DESTINATION letter

This subcommand is equivalent to the CWD subcommand (q.v.).

SET FILE LRECL

Syntax: SET FILE LRECL number

This  sets  the  logical record length for incoming files to a number from 1 to

65535 (64K-1).  This variable is used only for fixed format and  binary  files.

The default is 80.

SET FILE RECFM

Syntax: SET FILE RECFM option

This  sets  the  record  format  to  use for incoming files.  Valid options are

"Fixed" and "Variable" (the default).  Fixed-format records are padded or trun-

cated, as needed, to the current LRECL.

SET SEARCH-ALL

Syntax: SET SEARCH-ALL ON or OFF

ON      If the user omits the filemode from a SEND subcommand (or a GET request

to the other Kermit), Kermit-CMS will search all accessed disks for the

named file or files.

OFF     If  the  filemode  is  not  specified,  only  the  "home"  disk and its

read-only extensions will be searched for matching files.  (Default.)

THE CWD SUBCOMMAND

Syntax: CWD letter

The CWD  (Change  Working  Directory)  subcommand  establishes  a  new  default

("home")  CMS  disk.  letter may be the mode letter of any accessed disk.  Sub-

sequent file transfers take place preferentially to and from the default  disk.

The initial home disk is "A".  Note: setting the home disk in Kermit has no ef-

fect on the CMS search order.

8.3. Kermit-CMS Subcommands                                            Page 209

-------------------------------------------------------------------------------

THE DIRECTORY SUBCOMMAND

Syntax: DIRECTORY [filespec]

Under Kermit-CMS, the DIRECTORY subcommand is identical  to  the  CMS  LISTFILE

command.

THE SPACE SUBCOMMAND

Syntax: SPACE [letter]

This  subcommand displays the storage allocation on the specified CMS disk.  If

letter is omitted, the default disk specified by the  CWD  subcommand  is  dis-

played.    Aside  from this default, the subcommand is identical with CMS QUERY

DISK.

THE GIVE SUBCOMMAND

Syntax: GIVE table-name filespec

This subcommand compares the named table with its default values and saves  the

differences  in  a  TAKE file named filespec.  The format of filespec is fn [ft

[fm]].  The default filetype is "TAKE", and the default filemode is that of the

"home" disk.  See the CWD subcommand.

THE CP AND CMS SUBCOMMANDS

Syntax: CP or CMS text of command

Although  Kermit-CMS does not have a full set of its own subcommands for manag-

ing local files, it provides those services through the operating system.   You

can issue any CP or CMS command, but if Kermit-CMS has been invoked as a normal

user-area program, rather than as a high-memory "resident" program  or  nucleus

extension,  other  user-area CMS commands (such as COPYFILE) are illegal.  Even

then, you can list, type, rename or delete files, send  messages,  and  so  on.

The CMS subcommand under Kermit is synonymous with the HOST subcommand.

8.4. How to build an executable version of Kermit-CMS

Before  attempting  to  build Kermit-CMS, look in the Kermit distribution under

both IK0KER and IKCKER for an installation document, as well as "beware", help,

and  update files, and read them first.  They will probably contain information

that is more current than what you see here.

Kermit-CMS consists at present of a large assembly and a  small  optional  one.

The large assembly (KERMIT ASSEMBLE) contains the Kermit program, and the small

one (KERMBOOT ASSEMBLE) is a bootstrap program for  loading  Kermit  into  high

memory and running it.  Although KERMBOOT is all in one file in the Kermit dis-

tribution, the source for Kermit itself is in many  pieces,  some  generic  for

Kermit-370 and some specific to CMS.  All the necessary pieces are sequenced in

columns 73-80 so that the numbers form a strictly increasing sequence when  the

pieces  are  correctly  "pasted"  together.    It  is important to preserve the

8.4. How to build an executable version of Kermit-CMS                  Page 210

-------------------------------------------------------------------------------

original sequence numbers so that updates,  if  any,  can  be  applied  to  the

source.

To create a runnable version:

1. Combine  the following "ASM" files from the Kermit distribution into

a single file with RECFM F and LRECL  80:  IK0DOC,  IK0MAC,  IKCMAC,

IK0DEF,  IK0MAI,  IK0CMD, IK0COM, IKCUTL, and IK0PRO.  The resulting

file is the composite source for Kermit-CMS, called KERMIT ASSEMBLE.

This  source  must  retain  the original sequence numbers in columns

73-80 (in other words, be sure not to resequence the source acciden-

tally by using the editor!)

2. Copy  or rename IKCBOO ASM from the Kermit distribution (if desired)

to a file called KERMBOOT ASSEMBLE with RECFM F and LRECL 80.

3. GLOBAL the necessary MACLIBs.  Under VM/SP, these are DMSSP, CMSLIB,

OSMACRO, and TSOMAC.

4. Assemble the source file(s).

5. Load one file into memory via: LOAD KERMIT or LOAD KERMBOOT.  In the

former case, the entire Kermit program is now loaded; in the latter,

only  a bootstrap program which expects to find the object file KER-

MIT TEXT at run time.  Under CMS/SP Release 4 and above, there is  a

third and better option, namely, LOAD KERMIT (RLDSAVE).

6. Create  the executable called KERMIT MODULE via: GENMOD KERMIT.  Al-

ternatively (under CMS/SP Release 3 and below), create  both  KERMIT

and  KERMBOOT  modules  to  give  the  user a choice of user-area or

high-memory execution.  Since Kermit-CMS is  serially  reusable,  it

can  be  reinvoked  in the user area with the START command, but the

high-memory version must be reloaded each time.  If Kermit is loaded

using  the  RLDSAVE option (Release 4 and above), the module can, in

fact, be run either way; the command NUCXLOAD KERMIT will load  Ker-

mit  "permanently"  into  high memory as a nucleus extension for in-

vocation at need.  Note: the nucleus extension can be removed by the

command NUCXDROP KERMIT.

7. If your site's ASCII/EBCDIC translation table for TTY lines does not

conform to the one listed in the appendix (which in turn conforms to

the  one  given in the IBM System/370 Reference Summary), then enter

the appropriate SET ATOE/ETOA/TATOE/TETOA subcommands in the  SYSTEM

KERMINI  file, which should reside on the same disk as KERMIT MODULE

(and KERMIT TEXT).  NOTE: If the ASCII/EBCDIC translation is not in-

vertible, Kermit will not and cannot work.

8.5. What's New

Below  is  a list of the more important CMS-specific features in Version 4.0 of

Kermit-CMS added since the previous release, Version 3.1,  in  September  1986.

For the list of generic additions, see the chapter on Kermit-370.

1. System  commands  issued  through Kermit via the CMS or HOST subcom-

mands are automatically passed on to CP if (a) CMS rejects them  and

8.5. What's New                                                        Page 211

-------------------------------------------------------------------------------

(b) IMPCP is set ON.

2. Kermit subcommands may be executed directly from CMS EXEC's.

3. Reject  files  known  (via  A-packets)  to  be too big for available

storage.

4. Bypass user translation tables and  set  TERMINAL  SCROLL  CONT  for

protocol mode on TTY lines.

5. KERMBOOT  avoids  the  loading  problem  (VIRTUAL  STORAGE  CAPACITY

EXCEEDED) due to large GLOBAL TXTLIB's and preserves the untokenized

command line so that Kermit may be given mixed-case or long words as

part of the initial command.

8.6. What's Missing

Work on Kermit-CMS will continue.  Features that need to be improved  or  added

include:

- Allow  timeouts  so Kermit-CMS does not wait forever if a packet does

not arrive in a timely fashion.  This is not possible  under  CMS  at

present.

- Detect  file  properties  from Attribute packets and allow overriding

current parameter settings.  Also implement file archiving.

- Add a SET REPEAT subcommand.

- Finish SET LINE, so that Kermit-CMS can be used as  a  local  Kermit,

connecting  to  a  remote  host over an alternate communication port.

Add a CONNECT subcommand.

- Intercept CP messages during protocol mode,  rather  than  just  sup-

pressing  them.    Display  the messages later or log them or send in

packets as appropriate.

- Define EXEC variables from Kermit by analogy with the  XEDIT  EXTRACT

subcommand.

9. IBM MVS/TSO KERMIT                                                  Page 212

-------------------------------------------------------------------------------

9. IBM MVS/TSO KERMIT

Program:      John Chandler (Harvard/Smithsonian Center for Astrophysics); con-

tributions from Vace Kundakci and Daphne Tzoar (Columbia U),  Bob

Shields  (U.  Maryland),  Victor Lee (Queens U.), Gary Bjerke (U.

Texas  at  Austin),  Greg  Small  (UC  Berkeley),  Clark  Frazier

(Harvard  Bus. Sch.), Bob Bolch and Steve Blankinship (Triangle),

Ron Rusnak (U. Chicago), Roger Fajman and Dale White (NIH), Andre

Pirard (U. Liege)

Language:     IBM/370 Assembler

Documentation:

John Chandler (CfA)

Version:      4.0 (88/1/31)

Date:         1988 February

Kermit-TSO Capabilities At A Glance:

Local operation:                   No

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Z interruption:                Yes (through micro)

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                No

Communication settings:            No

Transmit BREAK:                    No

Transaction logging:               Yes

Session logging:                   No

Raw transmit:                      Yes (no prompts)

Sliding window:                    No

Long packets:                      Yes

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         Yes

Advanced commands for servers:     Yes

Local file management:             Yes

Handle Attribute Packets:          Yes

Command/init files:                Yes

Command macros:                    No

Kermit-TSO is a member of the generic Kermit-370 family and shares most of  the

features and capabilities of the group.  As its name implies, Kermit-TSO is the

version of Kermit-370 that runs  under  the  MVS/TSO  operating  system.    The

primary  documentation  for  Kermit-TSO  is actually the chapter on Kermit-370,

which describes general properties; the present chapter assumes the  reader  is

familiar  with  that material.  Only the details specific to TSO operation will

be discussed here, e.g., command syntax relating to the TSO file system or com-

mands not offered in general by Kermit-370.

9. IBM MVS/TSO KERMIT                                                  Page 213

-------------------------------------------------------------------------------

TSO Specifics of Kermit-370:

Global INIT file:                  'SYS1.KERMINI'

User INIT file:                    KERMINI

Debug packet log:                  KER.LOG

Server reply log:                  KER.REPLY

Maximum packet size:               1913

Maximum disk LRECL:                32756

9.1. The MVS/TSO File System

The  features  of  the TSO file system of greatest interest to Kermit users are

the format of file specifications (filespecs) and the concept of records.   The

latter is described in the Kermit-370 chapter.

The MVS/TSO filespec (called the data set name or DSN) takes the form of tokens

(known as qualifiers) of up to 8 alphanumeric  characters  each,  separated  by

periods.    Each qualifier must begin with an alphabetic or national character.

The total length must be no more than 44 characters, including periods.  To  be

precise,  a  DSN may contain uppercase letters, digits, and the special charac-

ters "$" (dollar sign), "#" (pound sign), "@" (at sign), and  "-"  (hyphen)  in

addition to the separating periods.  Other characters may be not be included.

There  is  another, structural restriction on data set names from the fact that

TSO data sets are all cataloged.  In a standard MVS catalog, each qualifier  of

each  DSN is associated with an index of the same name.  The index is a hierar-

chical catalog component which points downward either to a list  of  next-level

indices  or to a data set, but never to both.  Consequently, a cataloged DSN is

a chain of indices corresponding one-for-one with DSN qualifers, and  the  last

index  is  a  pointer  to the data set itself.  While there may be many indices

with the same name, no two such duplicates may both  be  chained  to  the  same

next-higher-level  index,  so  that,  if  DSN's  are  considered  as strings of

qualifiers (not of characters), no DSN can be a major substring of  any  other.

For  example,  if  the  name  'A.BB.C'  exists  in the catalog, then 'A.BB' and

'A.BB.C.X' are illegal, but 'A.B', 'A.BB.Y', 'A.BBB',  and  'A.BB.BB'  are  all

legal.

A  DSN  given  in  its entirety (as in the foregoing examples) is called "fully

qualified" and must be enclosed in single quotes when entered in TSO.  However,

by  convention (and by definition) the first qualifier of each data set belong-

ing to a given user must be the user's logon ID, and, by default,  that  ID  is

the  assumed  prefix in TSO when a DSN is not enclosed in quotes.  In practice,

then, names are abbreviated by omitting the quotes and the prefix.    The  most

common  type  of name, in fact, consists of the prefix plus two more qualifiers

giving the data set's name and type, respectively,  so  that  many  files  have

DSN's  that  correspond  exactly  to  the  canonical Kermit representation of a

filespec.  For instance, TEST.FORT is the source of  a  Fortran  program  named

TEST, and its fully qualified DSN would be '<userid>.TEST.FORT', where <userid>

is the owner's logon ID.

While this description is complete as far as it goes,  it  omits  an  important

feature  that is widely used in organizing files under MVS and TSO, namely, the

partitioned data set (PDS).  A PDS is a data set like any other, except that it

has  members,  each of which can usually be treated as a file in its own right.

In fact, with QSAM (the file access method employed by Kermit  and  many  other

9.1. The MVS/TSO File System                                           Page 214

-------------------------------------------------------------------------------

applications),  only  members,  and  not the whole PDS, may be read or written.

Thus, the term "file", as used in this chapter and the Kermit-370 chapter,  may

refer  to  either  a PDS member or an ordinary data set, but not to a PDS.  The

notation for a PDS member consists of the member name enclosed  in  parentheses

and  appended  to the DSN (and the whole enclosed in quotes if the DSN is fully

qualified).  For example, if the Fortran program TEST were copied  into  a  PDS

called DEBUG.FORT, it would then be known as

DEBUG.FORT(TEST)

Although the member name is written together with the DSN, it and the surround-

ing parentheses are not really part of the DSN and are not counted  toward  the

44-character  limit.   Still, a member name must conform to the rules for a DSN

qualifier and, in addition, must not contain any hyphens.

All these properties of DSN's come into play when a file is being  received  by

Kermit-TSO because a valid DSN must be generated for the new data set.  For ex-

ample, any invalid character in the supplied filespec is replaced  by  a  pound

sign  (or  converted  to  uppercase  if  it is a lowercase letter).  Also, each

qualifier (and the member name, if any) is prefixed with a  pound  sign  if  it

does  not already begin with an alphabetic or national character and then shor-

tened, if necessary,  to  eight  characters.    If  no  filespec  is  supplied,

Kermit-TSO  creates  a  default DSN of "$.$".  The DSN is expanded to its fully

qualified form and then truncated at 44 characters, if need be.

The DSN prefix, which defaults to the user's logon ID, is similar to  a  device

specification on microcomputer systems: it selects an area of disk storage, and

it usually need not be specified.  In some ways, the prefix is also like a disk

directory  designator,  since  the  file system structure is hierarchical.  For

this reason, the Kermit concept of the "working directory" is  equated  with  a

particular DSN prefix under Kermit-TSO.  The current "working directory" is the

collection of all data sets whose names begin with the current prefix, and file

transfers take place preferentially to and from that area (i.e., unless a fully

qualified DSN is given).

To provide compatibility with other operating systems, when Kermit-TSO sends  a

file,  it  ordinarily  makes a file header with only the last two qualifiers of

the full DSN (or only the member name in the case of a PDS  member).    On  the

other  hand,  extra  information may be added by way of the SET FOREIGN subcom-

mand.

Kermit-TSO allows a group of files to be specified in a single filespec by  in-

cluding  the  special  "wildcard"  character  "*",  which matches any string of

characters (even a null string) starting at a new DSN qualifier.   Only  one  *

may  be used in a filespec.  Also, the * may not be used for the member name of

a PDS (nor for any part of a PDS name).  Here are some examples:

*.COBOL   All files of type COBOL (all COBOL source  files)  in  the  current

working directory.

BATCH*H.FORT

All files in the current directory which begin with  the  qualifier

BATCH   and  which  end  with  H.FORT.    This  would  not  include

BATCH.FORT, however.

TSO files, like those in other IBM 370 systems, are  record-oriented  (see  the

9.1. The MVS/TSO File System                                           Page 215

-------------------------------------------------------------------------------

introduction  to the Kermit-370 chapter).  In particular, TSO files are charac-

terized by record format (RECFM), which may be fixed-length, varying-length, or

undefined-length;  by  maximum record length (LRECL); and by maximum block size

(BLKSIZE).  Fixed-length and varying-length may be (and, under  Kermit,  always

are)  combined  into  blocks, but undefined-length records may not.  Indeed, by

convention, they have no logical record length,  only  a  maximum  block  size.

Records  in TSO files may be up to 32760 bytes long, but varying-length records

use four bytes to specify the length and have an effective limit of 32756.

Another file system feature of occasional interest is the  means  of  reporting

errors.  When Kermit-TSO encounters a disk error, it attempts to prepare an ex-

planatory message for inclusion in the STATUS report.  The  primary  method  is

the standard SYNADAF macro.

9.2. Program Operation

At  startup time, Kermit-TSO looks for two initialization files, 'SYS1.KERMINI'

and '<userid>.KERMINI' (where, as before, <userid> is  the  user's  logon  ID).

The  latter  file would also be known as just KERMINI.  The file 'SYS1.KERMINI'

would be maintained by a systems programmer, but KERMINI would be maintained by

the user.

Two parameters in the user's profile (the character delete and line delete) are

disabled during protocol mode (and restored afterwards) to prevent any conflict

in  case either of these characters has been defined to be printable.  The set-

tings in effect when Kermit starts up are saved as a sort  of  "normal"  status

snapshot  (as  opposed  to the "protocol" status just described).  The protocol

status is selected whenever Kermit enters protocol mode, and the normal  status

is  selected  when Kermit leaves protocol mode.  Note: if Kermit is interrupted

in the midst of a transfer or while in server mode, these  parameters  will  be

left with peculiar settings (namely, the protocol status), and they may need to

be restored by hand.

Although TSO does not allow an application program to take control of  terminal

synchronization  on  "TTY"  lines, the various full-screen emulation front ends

are quite a different matter.  The standard IBM handshake (XON) is unnecessary,

for  example,  with  a 7171 or 4994 because the front end itself turns the line

around with essentially no delay in transparent mode.  Thus, handshaking should

be suppressed for "SERIES1" devices (the micro Kermit should have HANDSHAKE set

OFF, and Kermit-TSO should have  HANDSHAKE  set  to  0).    Since  the  generic

Kermit-370  default  handshake  (XON) is retained in Kermit-TSO, the subcommand

"SET HANDSHAKE 0" is a good candidate for inclusion the  KERMINI  file  of  any

user who habitually uses "SERIES1" lines.

Interactive Operation:

To  run Kermit-TSO interactively, invoke the program from TSO by typing KERMIT.

When you see the prompt,

Kermit-TSO>

you may type a Kermit subcommand.  When the subcommand completes, Kermit issues

another  prompt.    The cycle repeats until you exit from the program.  For ex-

ample:

9.2. Program Operation                                                 Page 216

-------------------------------------------------------------------------------

KERMIT

Kermit-TSO Version 4.0 (88/1/31)

Enter ? for a list of valid commands

Kermit-TSO>send foo.*

Files beginning with FOO are sent

Kermit-TSO>receive test.spss

File is received and called TEST.SPSS

Kermit-TSO>exit

Command Line Invocation:

Kermit-TSO may also be invoked with command line arguments from TSO.   The  ar-

guments  are interpreted as a subcommand to be executed by Kermit after comple-

tion of the initialization.  For instance:

KERMIT send test.fort

Kermit will exit and return to TSO after completing the specified subcommand.

CLIST Operation:

Like other TSO programs, Kermit-TSO may be invoked from a CLIST.    Subcommands

can  be  passed to Kermit using the program input stack and/or command line ar-

guments.  For example, to start up Kermit-TSO and have it act as a server,  in-

clude the line:

KERMIT server

To  pass  more  than one subcommand, they must be stacked in the order in which

they are to be executed.  To start up a Kermit-TSO server with a three  charac-

ter CRC, create and stack a file with the following:

set block 3

server

and  then  invoke  Kermit.    Like  many  utility programs, Kermit-TSO uses the

GETLINE/PUTLINE service routines for terminal I/O,  and  the  nominally  inter-

active  subcommands can thus be supplied under program control.  Another way of

setting up multiple subcommands would be to collect the subcommands into a TAKE

file  and  then  issue  the  TAKE  subcommand via the command line.  Of course,

CLIST's may be executed from Kermit, either directly or from a TAKE  file,  and

CLIST's  in turn may freely issue Kermit subcommands.  The subcommand KERMIT is

especially useful in this context for distinguishing  Kermit  subcommands  from

TSO commands.

9.2. Program Operation                                                 Page 217

-------------------------------------------------------------------------------

Server mode:

Command  execution  in server mode is different in several respects from normal

operation.  First of all, some Kermit subcommands are not allowed (see the list

of  subcommands  in  the  Kermit-370 chapter).  Moreover, command errors always

terminate any active TAKE file.  Also, commands run in  a  special  environment

with  the User Profile temporarily modified.  Another difference is that Kermit

intercepts terminal I/O as much as possible and transmits the data to the local

Kermit as text packets.  The problem with this redirection is that some MVS/TSO

commands issue terminal I/O directly, so that some messages never appear to the

local Kermit (except, perhaps, as bad packets).

9.3. Kermit-TSO Subcommands

Kermit-TSO  supports all the subcommands described in the corresponding section

of the Kermit-370 chapter.  In addition, there is the  system-specific  subcom-

mand  TSO, which is just a synonym for the generic HOST subcommand.  TSO can be

issued as a remote Kermit command when Kermit-TSO is in server mode.  Also, the

END subcommand is available as a synonym for EXIT and QUIT.

The remainder of this section concentrates on the subcommands that have special

form or meaning for Kermit-TSO.  See also the chapter on Kermit-370 for further

details.

THE SEND SUBCOMMAND

Syntax: SEND filespec [foreign-filespec]

The SEND subcommand causes a file or file group to be sent from TSO to the Ker-

mit on the other system.  DSN prefixing is done on the filespec  in  the  usual

way (see also the CWD subcommand).

filespec  may contain a wildcard "*".  If it does, then all matching files will

be sent, up to 711 files in all.

The foreign-filespec, if any, is used for the file header of the outgoing file,

replacing  the  usual  name.type  derived from the MVS/TSO filespec.  Normally,

this form of the SEND subcommand is used only  for  single  files  because  the

foreign-filespec  is  used only for the first file of a group (subsequent files

having default headers).

Although the file transfer cannot be cancelled from the TSO side, Kermit-TSO is

capable of responding to "cancel file" or "cancel batch" signals from the local

Kermit; these are typically entered by typing Control-X or  Control-Z,  respec-

tively.

9.3. Kermit-TSO Subcommands                                            Page 218

-------------------------------------------------------------------------------

THE RECEIVE SUBCOMMAND

Syntax: RECEIVE [filespec]

The  RECEIVE  subcommand  tells Kermit to receive a file or file group from the

other system.  You should then issue a SEND subcommand to the other Kermit.

A filespec in the subcommand indicates what name the incoming  file  should  be

given.  Wildcards may not be used.  If the filespec is invalid, Kermit-TSO will

suppress the transfer.  If the optional filespec is omitted (and, in any  case,

for  all  files  after  the  first  in a group) Kermit-TSO will use the name(s)

provided by the other Kermit.  If a name is not a legal  DSN,  Kermit-TSO  will

delete  excess characters, change illegal characters to pound signs, and so on,

to create a legal name.

When the record format is "F", any received  record  longer  than  the  logical

record  length  (LRECL)  will be truncated, and shorter records will be padded.

The padding character is a blank for text files and a null  for  binary  files.

Received  binary  (but  not  V-binary  or  D-binary)  files are treated as byte

streams and broken up into records all of the logical record length.   See  the

SET  FILE  TYPE,  SET  FILE LRECL, SET FILE BLKSIZE, and SET FILE RECFM subcom-

mands.

If an error occurs during the file  transfer,  as  much  of  the  file  as  was

received  is  saved on disk.  If the sending of a file is cancelled by the user

of the foreign system, Kermit-TSO will discard whatever had arrived, unless AP-

PEND is ON or INCOMPLETE is KEEP.

If  the  incoming file has the same name as an existing file (either a data set

or a PDS member), and WARNING is OFF, the original file  will  be  overwritten.

If WARNING is set ON, however, Kermit-TSO will protect the existing file in one

of two ways.  If the filespec was entered  with  the  subcommand,  Kermit  will

prompt  the  user  for  permission to overwrite the file.  If the filespec came

from the foreign Kermit, Kermit-TSO will change the incoming name so as not  to

obliterate the pre-existing file.  It attempts to find a unique name by succes-

sively modifying the original and checking for the existence of such a file  at

each  step.  The procedure operates on the second qualifier of the full DSN (or

the member name in the case of a PDS member) and begins  by  truncating  it  to

seven  characters,  if necessary, and then appends "0".  If a file by that name

exists, Kermit then replaces the "0" with a "1".  It continues in  this  manner

up to "9", and if an unused name cannot be found, the transfer fails.

THE SET SUBCOMMAND

Syntax: SET parameter [value]

The  SET subcommand establishes or modifies various parameters controlling file

transfers.  The following SET parameters are available in Kermit-TSO,  but  not

in Kermit-370 in general:

FILE

BLKSIZE           Block size for incoming file.

LRECL             Logical Record length for incoming file.

RECFM             Record format for incoming files.

SPACE             Allocation unit (in tracks) for incoming files.

9.3. Kermit-TSO Subcommands                                            Page 219

-------------------------------------------------------------------------------

UNIT              Device type for incoming files.

VOLUME            Disk pack for incoming files.

PREFIX              Default disk area.

TIMER               Determine whether Kermit-TSO should time out.

SET FILE BLKSIZE

Syntax: SET FILE BLKSIZE number

This  sets  the  block size for incoming files to a number from 1 to 32760.  In

the case of fixed-format files, this number is just an upper bound; the  actual

block  size  is  taken  to  be the largest multiple of the LRECL which does not

exceed this limit.  The default is 6233.

SET FILE LRECL

Syntax: SET FILE LRECL number

This sets the effective logical record length for incoming files  to  a  number

from 1 to 32756.  This parameter is not used for files of undefined record for-

mat.  Moreover, it is not exactly the same as the MVS/TSO LRECL, which is  four

more  than  the  actual  maximum  data  length for varying-length records.  The

default is 80.

SET FILE RECFM

Syntax: SET FILE RECFM option

This sets the record format to use for  incoming  files.    Valid  options  are

"Fixed",  "Varying"  (the  default),  and  "Undefined".  This parameter is thus

limited to a subset of the range of possibilities for the MVS/TSO  RECFM.    In

Kermit-TSO,  all  incoming  files  of fixed or varying format are automatically

blocked according to the current block size.  Fixed-format records  are  padded

or truncated, as needed, to the current LRECL.

SET FILE SPACE

Syntax: SET FILE SPACE number

This  sets  the  track allocation unit for incoming files to a number from 1 to

32760.  The default is 5.  Since data sets are allowed as many as  15  extents,

this default provides for files up to 75 tracks.

SET FILE UNIT

Syntax: SET FILE UNIT type

This  sets  the  device  type  or  group for incoming files.  Valid options are

installation-dependent.  The default is SYSDA, which is universally  available,

but  not necessarily desirable, since many installations restrict TSO data sets

to a particular set of disk volumes.  In such cases, there is  usually  a  unit

9.3. Kermit-TSO Subcommands                                            Page 220

-------------------------------------------------------------------------------

name  which  refers  only  to  those  TSO  volumes,  and  the  global INIT file

'SYS1.KERMINI' should set the file unit parameter to that name.

SET FILE VOLUME

Syntax: SET FILE VOLUME name

This  sets  the  disk  volume  for  incoming   files.      Valid   namess   are

installation-dependent,  but  are,  in  any case, no more than six alphanumeric

characters.  The default is blank (none); in that case, the system chooses  one

of the available volumes of the specified UNIT type.

SET PREFIX

Syntax: SET PREFIX [string]

This subcommand is equivalent to the CWD subcommand (q.v.).

SET TIMER

Syntax: SET TIMER ON or OFF

This specifies whether Kermit-TSO is to maintain a timer for each packet it ex-

pects to read.  The default is ON.  If the timer is enabled,  its  duration  is

set  by the SET SEND TIMEOUT subcommand initially and then set according to the

request of the other Kermit.

THE CWD SUBCOMMAND

Syntax: CWD [string or PDSname()]

The CWD (Change Working Directory) subcommand establishes  a  new  default  DSN

prefix  or turns prefixing off.  This facility is similar to, but not quite the

same as, the prefix defined in the User Profile.   The  string,  if  specified,

must  consist  of  one or more DSN qualifiers, and the first must already be an

index in the disk catalog.  Subsequent file transfers take place  to  and  from

the  corresponding  disk  area  whenever a fully qualified DSN (one enclosed in

quotes) is not given.  The initial prefix is the user's  logon  ID,  i.e.,  the

same  as the default prefix in the User Profile.  If no prefix is given in this

subcommand, then prefixing is no longer performed.  The user must be careful to

remember  the  distinction  between  the prefix defined for Kermit and that for

TSO.  Pure Kermit subcommands (like SEND and TAKE) always use the  former,  but

TSO commands (and the TSO-related subcommand TYPE) use the latter.

An  alternative  form  of  the  CWD  subcommand allows specifying the full (but

unquoted) name of a PDS followed by paired parentheses.  When such  a  "working

directory"  is  in use, a filespec other than a fully qualified DSN is taken to

be a member name within the PDS.  For that reason, this  form  should  be  used

cautiously,  since the Kermit-TSO log filespecs (such as KER.LOG and KER.REPLY)

would be treated the same way.  In particular, it is advisable to turn on debug

mode  only  when  the Kermit prefix is a partially qualified DSN (once started,

the log continues to the same data  set  regardless  of  what  happens  to  the

9.3. Kermit-TSO Subcommands                                            Page 221

-------------------------------------------------------------------------------

prefix).

THE DIRECTORY SUBCOMMAND

Syntax: DIRECTORY [filespec]

The  DIRECTORY  subcommand  uses the TSO LISTCAT command to display part of the

data set catalog, i.e., all data sets whose names begin with the qualifiers  in

the Kermit prefix (if any) concatenated with the given filespec (if any).

THE TSO SUBCOMMAND

Syntax: TSO text of command

Although  Kermit-TSO does not have a full set of its own subcommands for manag-

ing TSO files, it provides those services through the operating  system.    You

can  issue  any  TSO command, e.g., to list, type, rename or delete files, send

messages, and so on.  The TSO subcommand under Kermit is  synonymous  with  the

HOST subcommand.

9.4. How to build an executable version of Kermit-TSO

Before  attempting  to  build Kermit-TSO, look in the Kermit distribution under

both IK0KER and IKTKER for an installation document, as well as "beware", help,

and  update files, and read them first.  They will probably contain information

that is more current than what you see here.

Kermit-TSO consists at present of a large assembly (KERMIT.ASM, containing  the

Kermit  program)  and  a small one (DYNALC.ASM, containing a subroutine for al-

locating data sets).  Although DYNALC is a single file in the Kermit  distribu-

tion,  the  source  for  Kermit  itself  is  in  many  pieces, some generic for

Kermit-370 and some specific to TSO.  All the necessary pieces are sequenced in

columns  73-80 so that the numbers form a strictly increasing sequence when the

pieces are correctly "pasted" together.    It  is  important  to  preserve  the

original  sequence  numbers  so  that  updates,  if  any, can be applied to the

source.

To create a runnable version:

1. Combine the following "ASM" files from the Kermit distribution  into

a  single file with RECFM=F(B) and LRECL=80: IK0DOC, IK0MAC, IKTMAC,

IK0DEF, IK0MAI, IK0CMD, IK0COM, IKTUTL, and IK0PRO.   The  resulting

file  is  the  composite  source  for Kermit-TSO, called KERMIT.ASM.

This source must retain the original  sequence  numbers  in  columns

73-80 (in other words, be sure not to resequence the source acciden-

tally by using the editor!)

2. Copy or rename IKTDYN.ASM from the Kermit  distribution  to  a  file

called DYNALC.ASM with RECFM=F(B) and LRECL=80.

3. Assemble the source file(s).

4. Create  the  executable load module KERMIT using the linkage editor.

9.4. How to build an executable version of Kermit-TSO                  Page 222

-------------------------------------------------------------------------------

Kermit is designed to run as a command processor, and so it must  be

placed  in  SYS1.CMDLIB or in a PDS concatenated to SYS1.CMDLIB (for

example, via the STEPLIB command).

5. If your site's ASCII/EBCDIC translation table for TTY lines does not

conform to the one listed in the appendix (which in turn conforms to

the one given in the IBM System/370 Reference Summary),  then  enter

the   appropriate   SET   ATOE/ETOA/TATOE/TETOA   subcommands   into

'SYS1.KERMINI'.  NOTE: If the ASCII/EBCDIC translation is not inver-

tible, Kermit will not and cannot work.

In order to verify the operation of a new version of Kermit-TSO, you may run it

under TEST using the CP parameter.

9.5. What's New

Below is a list of the more important features in  Version  4.0  of  Kermit-TSO

added  since  the  release  of  TSO/3708 in May 1987.  Since Version 4.0 is the

first release of Kermit-370 for TSO, some of the "new"  features  are  actually

new only to the Columbia TSO distribution.

1. Suppression of LINE and CHAR delete functions during protocol mode.

2. Advanced  server  functions  and  subcommands for talking to another

Kermit running in server mode.

3. Long packet protocol.

4. TYPE, ECHO,  XTYPE,  and  XECHO  subcommands  (the  last  two  being

Series/1 analogs of the first two.)

5. REMOTE  KERMIT  commands honored by TSO server, including SET, SHOW,

TAKE, TDUMP, STATUS, HOST, TSO, CWD, DIR, and TYPE.

6. TEST mode for debugging.

7. Multi-column, two-level, selective SHOW display.

8. Send and acknowledge attribute packets.

9. Optionally append to, rather than replace, old data sets with dupli-

cate names.

10. Automatic detection of terminal controller type (TTY or SERIES1).

11. SYNADAF message in cases of disk I/O error.

9.6. What's Missing                                                    Page 223

-------------------------------------------------------------------------------

9.6. What's Missing

Work  on  Kermit-TSO will continue.  Features that need to be improved or added

include:

- Detect file properties from Attribute packets  and  allow  overriding

current parameter settings.  Also implement file archiving.

- Add a SET REPEAT subcommand.

- Finish  SET  LINE,  so that Kermit-TSO can be used as a local Kermit,

connecting to a remote host over  an  alternate  communication  port.

Add a CONNECT subcommand.

- Compute file size for outgoing A-packets and implement the SPACE sub-

command.

- Reject files known (via  A-packets)  to  be  too  big  for  available

storage.

- Intercept all terminal output during protocol mode.

- Allow wildcard notation for PDS members.

10. VAX/VMS KERMIT                                                     Page 224

-------------------------------------------------------------------------------

10. VAX/VMS KERMIT

Authors:    Robert C. McQueen, Nick Bush, Stevens Institute of Technology

Language:   BLISS-32

Documentation:

R. McQueen, N. Bush, C. Gianone

Version:    3.3.111

Date:       April 2, 1987

VAX/VMS Kermit-32 Capabilities At a Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Timeouts:                          Yes

8th-bit prefixing:                 Yes

Repeat character compression:      Yes

Alternate block check types:       Yes

Communication settings:            Yes

Transmit BREAK:                    No

IBM mainframe communication:       Yes

Transaction logging:               Yes

Session logging (raw capture):     Yes

Debug logging:                     Yes

Raw transmit:                      Yes

Login scripts:                     No

Act as server:                     Yes

Talk to server:                    Yes

Advanced commands for servers:     Yes

Local file management:             Yes

Command/init files:                Yes

Long packets:                      No

Sliding windows:                   No

Attribute packets:                 No

Kermit-32 is a program that implements the Kermit file  transfer  protocol  for

the  Digital  Equipment  Corporation  VAX  series  computers  under the VAX/VMS

operating system. It is written in BLISS-32 and MACRO-32, with sources for  all

BLISS  modules also available as MACRO-32 sources.  Kermit-32 should run on any

VAX/VMS system from version 4.0 on (Version 3.1 of Kermit-32 is the  last  ver-

sion that runs under pre-4.0 releases of VMS).

The  first  section  of  this chapter will describe the things you need to know

about the VAX/VMS file system and how Kermit-32 uses it.   The  second  section

describes the special features of Kermit-32.  The final section contains infor-

mation of interest to those who need to install Kermit-32 on a system.

10.1. The VAX/VMS File System                                          Page 225

-------------------------------------------------------------------------------

10.1. The VAX/VMS File System

The two main items of interest of the VAX/VMS file system (for the Kermit user)

are the format of file specifications and the types of files and file data.

VAX/VMS File Specifications

VAX/VMS file specifications are of the form

NODE::DEVICE:[DIRECTORY]NAME.TYPE;VERSION

Under  version  3.x of VMS, NAME may be up to 9 characters, TYPE may be up to 3

characters and each item in DIRECTORY may be up to 9 character long.  Only  al-

phanumeric characters may be used in DIRECTORY, NAME and TYPE.

Under version 4.0 (and later) of VMS, NAME, TYPE and each item in DIRECTORY may

be up to 39 characters long, and may contain alphanumeric characters  plus  un-

derscore.

VERSION  is  a  decimal number indicating the version of the file (generation).

DEVICE may be either a physical or logical device name.  If  it  is  a  logical

name,  it  may be up to 63 characters long and may contain alphanumeric charac-

ters plus dollar signs and underscores.  NODE may  be  either  a  logical  name

which  translates to a DECnet node name or a physical DECnet node name.  In ei-

ther case, access information can be included (see the DECnet-VMS User's  guide

for more information).  The node name is not normally present, since most files

are accessed on the same node where the user's job is  running.    The  version

number  is  not  normally  given (in fact, should not normally be given).  When

device and/or directory are not supplied, they default to  the  user's  current

default  device  and  directory.   Therefore, NAME.TYPE is normally all that is

needed to specify a file on the user's default device and directory.   This  is

also  all  the  Kermit-32 will normally send as the name of a file being trans-

ferred.

The node field specifies the name (and access information) for the DECnet  node

where  the  file is located.  Kermit-32 does not transmit the node field to the

target system, but will attempt to honor a node field in an incoming file name.

The device field specifies a physical or "logical" device upon which  the  file

is  resident.    The  directory field indicates the area on the device, for in-

stance the area belonging to the owner of the file.  Kermit-32  does  not  nor-

mally  transmit  the  device or directory fields to the target system, but will

attempt to honor device or directory fields that may appear  in  incoming  file

names.  It  will not create new directories, however, so any directory must al-

ready exist.

The name is the primary identifier for the file.  The  type,  also  called  the

"extension",  is  an indicator which, by convention, tells what kind of file we

have.  For instance FOO.FOR is the source  of  a  Fortran  program  named  FOO;

FOO.OBJ  might  be the relocatable object module produced by compiling FOO.FOR;

FOO.EXE could an executable program produced by LINKing FOO.REL, and so forth.

VAX/VMS allows a group of files to be specified in a single file  specification

by including the special "wildcard" characters, "*" and "%".  A "*" matches any

string of characters, including no characters at all; a "%" matches any  single

10.1. The VAX/VMS File System                                          Page 226

-------------------------------------------------------------------------------

character.  Here are some examples:

*.FOR   All  files of type FOR (all Fortran source files) in the default direc-

tory.

FOO.*   Files of all types with name FOO.

F*.*    All files whose names start with F.

F*X*.*  All files whose names start with F and contain at least one X.

%.*     All files whose names are exactly one character long.

*.%%*   All files whose types are at least two characters long.

Wildcard notation is used on many computer systems in similar ways, and  it  is

the mechanism most commonly used to instruct Kermit to send a group of files.

TEXT FILES AND BINARY FILES

The file system used by VAX/VMS provides for a large number of attributes to be

associated with each file.  These attributes provide some indication of whether

the file is a text file, or is some other type of non-text data.  The two major

attributes that affect Pro/Kermit are the record  type  and  record  attribute.

The  record type describes how logical records are stored in the file.  Records

may be of some fixed length  (specified  by  another  attribute),  or  variable

length  (specified  within  each  record),  or  stream (implying no real record

divisions).  The record attributes describe how the breaks between records  are

to  be  treated.    For  example, a record attribute of implied carriage return

means that any program reading the file with  intentions  of  printing  it  out

should add a carriage return/line feed sequence between each record.  Other at-

tributes include FORTRAN carriage control and print file format.

The "standard" method of storing text in a file under VAX/VMS is to  store  one

line  of text per record (variable length records), with a carriage return/line

feed sequence implied by the end of the record (implied carriage return).  This

is  the  method  Kermit-32 uses to store files it receives when using FILE TYPE

TEXT.  Note that there are other formats which are used  to  store  text  under

VAX/VMS,  however,  the  one used be Kermit-32 is the only one which is handled

correctly by all known utility programs under VAX/VMS.    Also,  most  programs

which  work  with  text files (the editor EDT, for example) place some limit on

the length of the  lines  which  can  be  handled.    Typically  this  is  255.

Kermit-32  can  write files with up to 4095 characters on a line, which means a

text file from another system  may  be  transferred  and  stored  correctly  by

Kermit-32, but may still be unusable by certain VAX/VMS programs.

There  is  no  standard format for storing binary files.  Basically, any record

format with no record attributes are used for binary  files.    Since  programs

which  work  with binary files under VAX/VMS expect to see some particular for-

mat, more information is needed for transfer of binary files than for  transfer

of text files.  The current version of Kermit-32 is not capable of transferring

all types of binary files which were created on a  VAX/VMS  system  to  another

system  and  retrieving  them  intact, nor is it capable of transferring all of

types binary files created on a VAX/VMS system to  another  VAX/VMS,  P/OS,  or

RSX-11M/M+  system  intact.    However,  certain formats of binary files can be

10.1. The VAX/VMS File System                                          Page 227

-------------------------------------------------------------------------------

transferred, and binary files from some other systems may be transferred  to  a

VAX and recovered intact.

Binary  files which are created on a VAX (or other Files-11 systems) with fixed

512 byte records (a fairly common format) can be transferred  using  Kermit-32.

The  only  required  action is to set the file type to "fixed" in the receiving

Kermit-32.

Using two programs supplied with Kermit-32, it is possible to  transfer  almost

any  type  of  sequential  file  between  VAXen, or between a VAX and a P/OS or

RSX-11M/M+ system.  These two programs (VMSHEX and VMSDEH) will convert the bi-

nary files to text (using a variation on Intel hex format).  The resulting text

file can be transferred like any other, and  finally  "dehexified"  reproducing

the  original  file, with the major attributes intact.  Unfortunately, the text

files tend to be about twice the size of the  original  binary  files,  so  the

transfers  take  a  bit  longer than regular text files.  On the plus side, the

text versions of the files can be transferred to any system with a  Kermit  and

still  retrieved  intact.    They can also be transferred over 7-bit data paths

without any problems.  The bootstrap procedure (described below), makes use  of

hexified versions of the binary file which makes up Kermit-32.

USING THE VAX TO ARCHIVE MICROCOMPUTER FILES

You can use Kermit to send textual files from a microcomputer or any 8-bit sys-

tem to a VAX/VMS system with no special provisions, since Kermit-32 stores  in-

coming  files  as  text  files  (variable  length records with implied carriage

returns) unless it  is  explicitly  told  otherwise.    But  Kermit-32  has  no

automatic  way  of distinguishing an incoming binary file from an incoming text

file.  It turns out that because of the method used by  Kermit-32  for  storing

text files, a binary file can be stored like a text file so long as it does not

contain a string of more than 4095 characters  between  carriage  return,  line

feed  sequences,  and ends with a carriage return line feed.  Since most binary

files do not have these characteristics, you must inform Kermit-32 that a  file

it  is  about  to receive is to be stored as a binary file.  This is done using

the SET FILE TYPE BINARY command.  This instructs Kermit-32 to store  the  data

it  receives  in  the  file without checking for carriage return, line feed se-

quences.  The file it creates will be variable record length,  with  no  record

attributes.  Each record will contain 510 bytes of data, except the last, which

will contain whatever amount is left before the end of the file.   This  allows

Kermit-32  to  correctly  return  exactly the data it was sent when the file is

returned to the original system.

Note that because of the need to use a different file type for binary files, it

is  not possible to use a "wildcard send" command to send a mixture of text and

binary files to a VAX system unless the text files are not intended for use  on

the VAX; rather, you must send all text files with Kermit-32's file type set to

text, and all binary files with the file type set to binary.

Once you get the foreign file into the VAX system, stored with the correct file

type,  you  need  take  no  special  measures  to send it back to its system of

origin.  This is because Kermit-32 honors the record type and attributes of the

file  as  it  is stored on the VAX.  In fact, SET FILE TYPE BINARY or TEXT only

affects how Kermit-32 receives files - it does not affect how Kermit-32  trans-

mits files.

10.1. The VAX/VMS File System                                          Page 228

-------------------------------------------------------------------------------

FILES KERMIT-32 CANNOT HANDLE

The  Kermit  protocol  can only accommodate transfer of sequential files, files

which are a linear sequence of bytes (or words).

Some files on a VAX/VMS system are not sequential, and cannot  be  successfully

sent  or  received  by Kermit-32.  These are mainly indexed data files, but can

also include other files which require more than just the data in the  file  to

be  completely reconstructed.  External control information and file attributes

are not transmitted.  However, any VMS file can be transferred with  Kermit  if

it has been "hexified" with VMSHEX.

10.2. Program Operation

Kermit-32's  prompt  is normally "Kermit-32>".  If a foreign command is defined

to run Kermit-32 (eg. KERMIT := $KERMIT), Kermit-32 will accept a  single  com-

mand on the command line, like this:

$

$ Kermit send foo.bar

the file is sent

$

$ mcr kermit send foo.bar

the file is sent

$

You can run the program interactively to issue several commands, like this:

$

$ run sys$system:kermit

VMS Kermit-32 version 3.3.111

Default terminal for transfers is: _TTA1:

Kermit-32>send foo.*

files are sent

Kermit-32>statistics

performance statistics are printed

Kermit-32>receive

files are received

Kermit-32>exit

$

Upon initial startup, Kermit-32 executes commands from its initialization file,

VMSKERMIT.INI, if any, in your directory.

10.2. Program Operation                                                Page 229

-------------------------------------------------------------------------------

Command keywords may be abbreviated to their shortest  prefix  that  sets  them

apart from any other keyword valid in that field.

Kermit-32 provides most of the commands possible for an "ideal" Kermit program,

as described in the main part of the Kermit User Guide.  The following sections

will concentrate on system-dependent aspects of Kermit-32.

10.3. Conditioning Your Job for Kermit

Kermit-32  does as much as it can to condition your line for file transfer.  It

saves all your terminal settings, and restores them after use.  However,  there

are  some sources of interference over which Kermit-32 has no control.  In par-

ticular, messages issued by other processes in your job  could  become  mingled

with  Kermit  packets  and  slow  things down or stop them entirely.  This is a

fairly rare occurence and can be  easily  avoided  by  not  running  any  other

process  which  wishes  to  perform  I/O to your terminal while you are running

Kermit-32.

Normally, when Kermit-32 is run, it assumes you wish to use it in  remote  mode

and  perform  file  transfers  over  the terminal line which controls your job.

This can be overridden, however, by defining a logical name  which  equates  to

some  other  terminal line in the system.  The default terminal line to be used

for file transfers is determined by the first of the  following  logical  names

which translates to a terminal line which is available for use by your process:

KER$COMM, SYS$INPUT, SYS$OUTPUT, and SYS$COMMAND.  If  none  of  these  logical

names  translate  to  an  available terminal line, there is no default terminal

line and a SET LINE command must be used before any transfer  command  is  per-

formed.  Note that this is the typical case in a batch job.

Kermit-32  will also default the type of parity to be used on the communication

line to that which is set on its default terminal  line  when  it  is  started.

This  means  that  if  all  communication at a site is normally done using even

parity (for example), Kermit-32 will also use even parity.

There are two things to keep in mind when using Kermit-32 in local mode  (where

the  file transfers are done over a different terminal line from where commands

are typed):

- Under VAX/VMS, every terminal line has an owner  UIC  and  protection

code  associated  with it.  This UIC and protection is used to deter-

mine who can allocate (and therefore use) the terminal line when they

are not logged in on that line.  Therefore, in order for Kermit-32 to

be able to perform file transfers over a terminal line other than the

one  on which you are logged in, the field of the protection code for

the terminal which applies to your job (based on  your  UIC  and  the

owner  UIC  of  the  terminal) must allow your job access to the ter-

minal.  You may need to request your system  manager  to  change  the

protection  for a terminal line to allow you to use it with Kermit-32

in local mode.  See the section on Installation for details.

- Terminal lines which have been declared as modem control  lines  will

have  the  phone  "hung  up"  whenever the terminal line becomes free

(deallocated).  This means that if you do not use  the  DCL  ALLOCATE

command  to  allocate  the  terminal line to your job before entering

Kermit-32, exiting Kermit-32 will cause the terminal  line  to  "hang

10.3. Conditioning Your Job for Kermit                                 Page 230

-------------------------------------------------------------------------------

up"  the  modem.    If  you  do  wish to get to DCL after having used

Kermit-32 to connect a modem control line which you do not  have  al-

located,  you  can use the PUSH command to spawn a subprocess running

DCL.

10.4. Kermit-32 Commands

This section describes the Kermit-32 commands -- in detail  where  they  differ

from  the  "ideal" Kermit, briefly where they coincide.  Kermit-32 has the fol-

lowing commands:

@  synonym for "take".

BYE  to remote server.

CONNECT  as terminal to remote system.

EXIT  from Kermit-32.

FINISH  Shut down remote server.

GET  remote files from server.

HELP  with Kermit-32.

LOCAL  prefix for local file management commands.

LOG  remote terminal session.

LOGOUT  remote server.

PUSH  to DCL command level.

QUIT  from Kermit-32.

RECEIVE  files from remote Kermit.

REMOTE  prefix for remote file management commands.

SEND  files to remote Kermit.

SERVER  mode of remote operation.

SET  various parameters.

SHOW  various parameters.

STATUS  about most recent file transfer.

TRANSMIT  Transmit (upload) a file with no error checking.

TAKE  Kermit-32 commands from a file.

10.4.1. Commands for File Transfer

Kermit-32 provides the standard SEND, RECEIVE, and GET commands  for  transfer-

ring files using the Kermit protocol.

THE SEND COMMAND

Syntax:

Sending a file or files:

SEND filespec

The  SEND  command  causes  a file or file group to be sent from the VAX to the

other system.  If filespec contains wildcard characters then all matching files

will be sent, in alphabetical order (according to the ASCII collating sequence)

by name.  If filespec does not contain any wildcard characters, then the single

file specified by filespec will be sent.

10.4.1. Commands for File Transfer                                     Page 231

-------------------------------------------------------------------------------

SEND Command General Operation:

Files will be sent with at least their VAX/VMS file name and type (for instance

FOO.BAR).  If a SET FILE NAMING FULL command has  been  given,  Kermit-32  will

also  send  the  device  name,  directory name and version number (for instance

USER$DISK:[JOE]FOO.BAR;25).  If a SET FILE NAMING UNTRANSLATED command has been

given, Kermit-32 will send the file name, type and version number (for instance

FOO.BAR;25).  If a SET FILE NAMING NORMAL_FORM command has been given (this  is

the initial default), Kermit-32 will only send the file name and type.

Each  file will be sent according to the record type and attributes recorded in

its file descriptor.  Kermit-32 attempts to translate all formats of text  file

(including  those with FORTRAN or print carriage control) to a format usable on

any system.  Note that there is no need to set  the  FILE  TYPE  parameter  for

sending  files,  since  Kermit-32  always  uses  the  information from the file

descriptor to determine how to send the file.

If communication line parity is being used (see SET PARITY), Kermit-32 will re-

quest that the other Kermit accept a special kind of prefix notation for binary

files.  This is an advanced feature, and not all Kermits have it; if the  other

Kermit  does  not  agree  to use this feature, binary files cannot be sent cor-

rectly.  This includes executable programs (like .EXE files, CP/M .COM  files),

relocatable  object  modules  (.OBJ files), as well as any text file containing

characters with the eighth bit on.

Kermit-32 will also ask the other Kermit whether it can handle a special prefix

encoding  for  repeated characters.  If it can, then files with long strings of

repeated characters will be  transmitted  very  efficiently.    Columnar  data,

highly  indented  text,  and  binary  files are the major beneficiaries of this

technique.

If you're running Kermit-32 locally, for instance dialing out  from  a  VAX  to

another  system  using an autodialer, you should have already run Kermit on the

remote system and issued either a RECEIVE or a SERVER command.  Once  you  give

Kermit-32  the  SEND  command,  the name of each file will be displayed on your

screen as the transfer begins.  If the file is  successfully  transferred,  you

will see "[OK]", otherwise there will be an error message.

During local operation, you can type Control-A at any point during the transfer

to get a brief status report.   You may also type Control-X or Control-Z to in-

terrupt the current file or file group.

THE RECEIVE COMMAND

Syntax:  RECEIVE [filespec]

The  RECEIVE  command  tells Kermit-32 to receive a file or file group from the

other system.  If only one file is being received, you may include the optional

filespec  as  the name to store the incoming file under; otherwise, the name is

taken from the incoming file header.  If the name in the header is not a  legal

VAX/VMS  file name, Kermit-32 will normally replace the illegal characters with

"X" (see SET FILE NAMING NORMAL_FORM).

If an incoming file has the same name  as  an  existing  file,  Kermit-32  just

creates  a  new  version  of  the  same  name and type, for instance FOO.BAR;3,

10.4.1. Commands for File Transfer                                     Page 232

-------------------------------------------------------------------------------

FOO.BAR;4.

Incoming files will all be stored with  the  prevailing  file  type,  ASCII  by

default,  which is appropriate for text files.   If you are asking Kermit-32 to

receive binary files from a microcomputer or other 8-bit system, you must first

type  SET  FILE  TYPE BINARY.  Otherwise, an error may occur when receiving the

file, or a carriage return, line feed will be added to the end of the file  and

the file will be useless when sent back to the system of origin.

If parity is being used on the communications line, then 8th-bit prefixing will

be requested.  If the other side cannot do this, binary files cannot be  trans-

ferred correctly.

If  an  incoming  file does not arrive in its entirety, Kermit-32 will normally

discard it; it will not appear in your directory.  You may change this behavior

by  using the command SET INCOMPLETE KEEP, which will cause as much of the file

as arrived to be saved in your directory.

If you are running Kermit-32 locally, you should already  have  issued  a  SEND

1

command   to  the  remote Kermit, and then escaped back to Kermit-32.  As files

arrive, their names will be displayed on your screen.  You can  type  Control-A

during the transfer for a brief status report.

If  a  file arrives that you don't really want, you can attempt to cancel it by

typing Control-X; this sends a cancellation request to the remote Kermit.    If

the  remote  Kermit understands this request (not all implementations of Kermit

support this feature), it will comply; otherwise it will continue to send.   If

a  file  group  is being sent, you can request the entire group be cancelled by

typing Control-Z.

THE GET COMMAND

Syntax: GET [remote-filespec]

The GET command requests a remote Kermit server to send the file or file  group

specified  by remote-filespec.  This command can be used only when Kermit-32 is

local, with a Kermit server on the other end of the line specified by SET LINE.

This  means  that  you  must have CONNECTed to the other system, logged in, run

Kermit there, issued the SERVER command, and escaped back to the VAX.

The remote filespec is any string that can be a legal  file  specification  for

the  remote  system; it is not parsed or validated locally.  Any leading spaces

before the remote filespec are stripped, and lower case characters  are  raised

to upper case.

As  files  arrive,  their  names  will  be displayed on your screen.  As in the

RECEIVE command, you may type Control-A to get a brief status report, ^X to re-

quest  that  the current incoming file be cancelled, ^Z to request that the en-

tire incoming batch be cancelled.

_______________

1

not SERVER -- use the GET command to receive files from a Kermit server.

10.4.1. Commands for File Transfer                                     Page 233

-------------------------------------------------------------------------------

If the remote Kermit is not capable of server functions, then you will probably

get  an  error  message back from it like "Illegal packet type".  In this case,

you must connect to the other Kermit, give a SEND  command,  escape  back,  and

give a RECEIVE command.

THE STATUS COMMAND

Give statistics about the most recent file transfer.

THE PUSH COMMAND

Syntax: TAKE

Invoke  an  inferior DCL command processor, to which you may issue any DCL com-

mands.  Type LOGOUT to return to Kermit-32.

THE TAKE COMMAND

Syntax: TAKE file-spec [ /DISPLAY ]

Where 'file-spec' is any normal VAX/VMS file specification.  If file-spec  does

not  specify  a file-type Kermit-32 will supply a default of .COM. The /DISPLAY

option causes the commands read from the file to be  displayed  on  the  user's

terminal.

The  TAKE  command tells Kermit-32 to execute commands from the specified file.

You may also use the VAX/VMS notation "@" instead of Take to specify a  command

file.

The file VMSKERMIT.INI is automatically taken upon program startup.

10.4.2. Server Operation

THE SERVER COMMAND

The  SERVER  command  puts  a  remote  Kermit-32  in  "server mode", so that it

receives all further commands in packets from the local Kermit.  The  Kermit-32

server is capable (as of this writing) of executing the following remote server

commands:  SEND, GET, FINISH, BYE, REMOTE DIRECTORY, REMOTE CWD, REMOTE  SPACE,

REMOTE  DELETE,  REMOTE  TYPE,  REMOTE HELP, REMOTE COPY, REMOTE RENAME, REMOTE

SEND_MESSAGE, REMOTE WHO, and REMOTE HOST.

Any nonstandard parameters should be selected with SET commands before  putting

Kermit-32  into  server mode, in particular the file type. The Kermit-32 server

can send all files in the correct manner automatically.  However, if  you  need

to  ask  Kermit-32 to receive binary files you must issue the SET FILE TYPE BI-

NARY command before putting it into server mode, and then you  must  only  send

binary  files.   You cannot send a mixture of text files and 8-bit binary files

to a Kermit-32 server unless the files are not for use on the VAX.

10.4.2. Server Operation                                               Page 234

-------------------------------------------------------------------------------

COMMANDS FOR SERVERS

When running in local mode, Kermit-32 allows you to give a wide range  of  com-

mands  to  a remote Kermit server, with no guarantee the that the remote server

can process them, since they are all optional features of the protocol.    Com-

mands  for  servers include the standard SEND, GET, BYE, LOGOUT and FINISH com-

mands, as well as the REMOTE command.

Syntax: REMOTE command

Send the specified command to the remote server.  If the server does not under-

stand  the  command  (all of these commands are optional features of the Kermit

protocol), it will reply with a message like "Unknown Kermit  server  command".

If  does  understand, it will send the results back, and they will be displayed

on the screen.  The REMOTE commands are:

COPY filespec   Copy file.  The server is asked to make a copy of the specified

file.    Kermit-32  will  prompt  for  the  new  file name on a

separate line.  Both filespecs must be in  the  correct  format

for  the  remote  system.  Kermit-32 does not parse or validate

the file specifications.  Any leading spaces will  be  stripped

and  lower  case characters converted to upper case.  Note that

this command simply provides for  copying  a  file  within  the

server's system - it does not cause a file to be transferred.

CWD [directory] Change  Working  Directory.   If no directory name is provided,

the server will  change  to  the  default  or  home  directory.

Otherwise,  you will be prompted for a password, and the server

will attempt  to  change  to  the  specified  directory.    The

password  is  entered  on a separate line, and does not echo as

you type it.  If access is not granted, the server will provide

a  message to that effect.  Note that while not all server Ker-

mits require (or accept)  a  password  to  change  the  working

directory,  Kermit-32  will always ask for one when a directory

name is provided.

DELETE filespec Delete the specified file or files.  The  names  of  the  files

that are deleted will appear on your screen.

DIRECTORY [filespec]

The names of the files that match the given file  specification

will  be  displayed on your screen, perhaps along with size and

date information for each file.  If no  file  specification  is

given, all files from the current directory will be listed.

DISK_USAGE [directory]

Display information about disk usage in the given directory (or

by  the  given  user).  If no directory is provided, disk usage

information is provided for the current working  directory  (or

user).  This is the same as the REMOTE SPACE command.

EXIT            Requests  the  server to leave Kermit, allowing the terminal to

be used for normal commands.

FINISH          Requests the server to return to the  Kermit  prompt,  allowing

statistics to be obtained about the transfers.

10.4.2. Server Operation                                               Page 235

-------------------------------------------------------------------------------

HELP [topic]    Provide  information  about  the  given  topic.  If no topic is

given, provide a list of the functions that are available  from

the  server.  Some servers may ignore the topic and always dis-

play the same information.

HOST [command]  Pass the given command to the server's host command  processor,

and display the resulting output on your screen.

LOGIN user-id   Supply  information  to  the  server  Kermit  to  indicate what

user-id, account and password are to be used.  The server  Ker-

mit may use this to validate the user's access to the system as

well as for billing purposes. It may also use this  information

to provide the user with access to files on its system.

LOGOUT          Request  the  server  to  exit  Kermit  and  logout its job (or

process).  This command is identical to the LOGOUT command.

RENAME filespec Change the name on the specified file (or  files).    Kermit-32

will  prompt  for  the new file specification on the next line.

Both file specifications must be valid for the server's system.

SEND_MESSAGE destination-address

Request  the  server  to  send  a  single  line  message to the

specified destination address (which might be a  user-id,  ter-

minal designator, or some other item, depending upon the server

Kermit).  Kermit-32 will prompt for the single line message  on

the next line.

SPACE [directory]

Display information about disk usage in the given directory (or

by  the  given  user).  If no directory is provided, disk usage

information is provided for the current working  directory  (or

user).  This is the same as the REMOTE DISK_USAGE command.

STATUS          Display information about the status of the server.

TYPE filespec   Display the contents of the specified file on your screen.

WHO [user-id]   Display  information  about  the  given user.  If no user-id is

given, display information about the  currently  active  users.

Kermit-32  will  prompt  for  options for selecting what infor-

mation to display and/or formatting parameters.  The format  of

both  the user-id and the options are dependent upon the server

Kermit.

10.4.3. Commands for Local File Management

Syntax: LOCAL [command]

Execute the specified command on the local system  --  on  the  VAX/VMS  system

where  Kermit-32 is running.  These commands provide some local file management

capability without having to leave the Kermit-32 program.  These  commands  are

very  similar  to the REMOTE commands in function and syntax.  They are all ex-

ecuted locally, and are available when Kermit-32 is  either  local  or  remote.

The arguments to these commands are the same as the arguments expected from the

10.4.3. Commands for Local File Management                             Page 236

-------------------------------------------------------------------------------

user Kermit when Kermit-32 is processing a command in server mode.

COPY filespec   Make a copy of the given  file  (or  files).    Kermit-32  will

prompt for the new file specification.  The command is actually

performed by using the  DCL  COPY  command  (COPY/LOG  old-file

new-file), and any options which are valid on the DCL COPY com-

mand may be included.

CWD [directory] Change working directory, or, in  VAX/VMS  terminology,  change

the  default device/directory.  This command takes the same ar-

guments as the DCL SET DEFAULT  command  (i.e.,  a  device  and

directory, only a directory, or only a device).  If no argument

is given, the default device and directory are reset to that in

effect  when  Kermit-32  was  run.   The new default device and

directory will be typed out.

DELETE filespec Delete the specified file or files.  This command is  performed

by  using the DCL DELETE command (DELETE/LOG filespec).  There-

fore, any options which are valid on the DCL DELETE command may

be included.

DIRECTORY [filespec]

Provide a directory listing of the specified files.  This  com-

mand is performed by using the DCL DIRECTORY command (DIRECTORY

filespec), so any options valid for the DCL  DIRECTORY  command

may be included.

DISK_USAGE [uic]

Display disk usage information for the given UIC.  If no UIC is

given,  display  disk  usage  information  for the process UIC.

This command is performed by using the DCL SHOW  QUOTA  command

(SHOW QUOTA or SHOW QUOTA/USER=uic).

HELP            Display  the  help message describing the server commands which

are available.

HOST DCL command

Perform  the given DCL command.  The command should not perform

any action which will require more input.  Any output resulting

from the command will be typed on the terminal.

RENAME filespec Change  the  name of the specified file.  Kermit-32 will prompt

for the new name on the next line.  This command  is  performed

by using the DCL RENAME command (RENAME/LOG old-file new-file),

so any options which are valid on the DCL RENAME command may be

included.

SEND_MESSAGE terminal-name

Send a single line message to the given  terminal.    Kermit-32

will  prompt for the message on the next line.  Since this com-

mand is performed using the DCL REPLY command

REPLY/TERMINAL=terminal-name "message"

OPER privileges are needed to perform it.

10.4.3. Commands for Local File Management                             Page 237

-------------------------------------------------------------------------------

TYPE filespec   Display the contents of the specified file  or  files  at  your

terminal.    Each  file  will  be preceded by its name in angle

brackets.

10.4.4. The CONNECT Command

Syntax: CONNECT [terminal-name]

Establish a terminal connection to the system connected to  the  terminal  line

specified  here or in the most recent SET LINE command, using full duplex echo-

ing and no parity unless otherwise specified in previous  SET  commands.    Get

back  to Kermit-32 by typing the escape character followed by the letter C. The

escape character is Control-Close-Square-Bracket (^]) by  default.    When  you

type the escape character, several single-character commands are possible:

C   Close the connection and return to Kermit-32.

Q   If a session log is active, temporarily Quit logging.

R   Resume logging to the session log.

S   Show status of the connection.

0   Send a null character.

?   List all the possible single-character arguments.

^] (or whatever you have set the escape character to be):

Typing the escape character twice sends one copy of it to  the  connected

host.

You  can use the SET ESCAPE command to define a different escape character, and

SET PARITY, and SET  LOCAL_ECHO  to  change  those  communication-line-oriented

parameters.  Type the SHOW LINE command for information about your current com-

munication settings.

Kermit-32 does not have any special autodialer interface.  It assumes that  the

connection has already been made and the line assigned.

10.4.5. The SET and SHOW Commands

THE SET COMMAND

Syntax: SET parameter [option [value]]

Establish  or  modify  various parameters for file transfer or terminal connec-

tion.  You can examine their values with  the  SHOW  command.    The  following

parameters may be SET:

BLOCK_CHECK  Packet transmission error detection method

DEBUGGING  Record or display state transitions or packets

DELAY  How long to wait before starting to send

ESCAPE  Character for terminal connection

FILE  For setting file parameters like file type

HANDSHAKE  For establishing half duplex line turnaround handshake

IBM_MODE  For communicating with an IBM mainframe

INCOMPLETE_FILE  What to do with an incomplete file

LINE  Terminal line to use for file transfer or CONNECT

LOCAL_ECHO  For terminal connection, ON or OFF

10.4.5. The SET and SHOW Commands                                      Page 238

-------------------------------------------------------------------------------

MESSAGE  The type of typeout to be done during transfers

PARITY  Character parity to use

PROMPT  Change the program's command prompt

RECEIVE  Various parameters for receiving files

REPEAT_QUOTE Character to use for repeat compression

RETRY  How many times to retry a packet before giving up

SEND  Various parameters for sending files

TRANSMIT  Control TRANSMIT command echo and delay

Those  SET  commands  which differ from the "ideal" Kermit are now described in

detail.

SET DEBUGGING

Syntax: SET DEBUGGING options

Record the packet traffic, either on your terminal or in a file.  Some  reasons

for  doing  this would be to debug a version of Kermit that you are working on,

to record a transaction in which an error occurred for evidence when  reporting

bugs,  or  simply  to  vary the display you get when running Kermit-32 in local

mode.  Options are:

ON              Display each incoming and outgoing packet (lengthy).

OFF             Don't display or record debugging information (this is the nor-

mal mode).  If debugging was in effect, turn it off.

The  debugging information is recorded in the file specified by the most recent

LOG DEBUGGING command.

SET ESCAPE

SET ESCAPE octal-number

Specify the control character you want to use to "escape" from  remote  connec-

tions back to Kermit-32.  The default is 35 (Control-]).  The number is the oc-

tal value of the ASCII control character, 1 to 37 (or 177), for instance  2  is

Control-B.  After you type the escape character, you must follow it by a one of

the single-character "arguments" described under the CONNECT command, above.

SET FILE

Syntax: SET FILE parameter keyword

Establish file-related parameters:

TYPE keyword

Type  of  file for VAX/VMS file output.  The choices are ASCII, BINARY,

or FIXED.

ASCII   Store the file as a standard  VAX/VMS  text  file.    Any  file

received  is  stored  as  variable length records with carriage

return, line feed sequences implied between records.   This  is

10.4.5. The SET and SHOW Commands                                      Page 239

-------------------------------------------------------------------------------

the  format  preferred  by most utility programs under VAX/VMS.

An error will occur if any line is more  than  4096  characters

long.   Note that lines are only terminated by carriage return,

line feed sequences.  A carriage return that is not followed by

a  line  feed or a line feed that is not preceded by a carriage

return is not considered the end of a  line,  and  is  included

within the body of a record.

BINARY  Store  the  file as a binary file.  Any file received is stored

as  variable  length  records  with   no   record   attributes.

Kermit-32  actually  will write 510 bytes in each record except

the last.  This makes each record take up one disk  block  (510

data  bytes  plus two bytes of record length).  The last record

is written containing only as much data is left to the  end  of

the  file.    Any  file  which is just a stream of bytes can be

stored as a BINARY file, and recovered intact later.   This  is

the preferred file type for use in archiving files.

FIXED   Store  the  file  as  a  fixed  length  binary  file.  Any file

received is stored as fixed length 512  byte  records  with  no

record  attributes.    This is the format used for binary files

such as VAX/VMS "EXE" files and RSX-11M/M+ "TSK" files.   Since

even  the  last  record  of  the file is written with 512 bytes

(even if it is not filled), this format  does  not  necessarily

maintain the correct length of a file.  It should normally only

be used for files which are coming from a VAX/VMS system  which

are currently stored in fixed 512 byte records.

NAMING keyword

Determine the form of names to be sent with outgoing files  and  deter-

mine the translation performed on incoming file names.  The choices are

FULL, NORMAL_FORM and UNTRANSLATED.

FULL    Kermit-32 will send full file names (including  device,  direc-

tory, file name, file type and version number).  When receiving

a file, Kermit-32 will perform no translation of the file  name

(which must therefore be a legal VAX/VMS file specification).

NORMAL_FORM

Kermit-32 will send only the file  name  and  file  type.  When

receiving a file, Kermit-32 will convert the file specification

received to contain only uppercase letters, digits, and at most

one  period.    Any other characters will be translated to "X".

There will be at most 9 characters before the period (if  any),

and at most 3 characters afterwards.  This forces the file name

to be a valid VAX/VMS file specification.  Note  that  standard

VAX/VMS  file  names  and  types  are  already normal form, and

therefore do not need translation.  This is the default.

UNTRANSLATED

Kermit-32  will  send  only  the  file name and file type. When

receiving a file, Kermit-32 will not perform any conversions on

the file specification, which therefore must be a legal VAX/VMS

file specification.

10.4.5. The SET and SHOW Commands                                      Page 240

-------------------------------------------------------------------------------

SET HANDSHAKE

Syntax: SET HANDSHAKE o

Sets the half duplex line turnaround handshake character to the ASCII character

whose  octal  value is o.  Normally required for communication with half duplex

systems like IBM mainframes.

SET IBM_MODE

Syntax: SET IBM_MODE ON or OFF

When IBM_MODE is set to ON, Kermit-32 will override the parity and  local  echo

settings and use odd parity, local echo on, and also enable a handshake charac-

ter of XON (control-Q, ASCII 021 octal).  This feature allows Kermit-32 to talk

with certain systems (notably some IBM mainframes), which require waiting for a

XON before sending data.

The various features selected by this command can be overridden subsequently by

SET PARITY, SET LOCAL_ECHO, and SET HANDSHAKE commands.

SET LINE

Syntax: SET LINE [terminal-name]

Specify   the   terminal  name  to  use  for  file  transfer  or  CONNECT;  the

terminal-name can be up to 16 characters long.  If you issue this command using

other  than  your  job's  controlling  terminal,  you will be running Kermit-32

locally, and you must log in to the remote system and run Kermit on  that  side

in order to transfer a file.  If you don't issue this command, Kermit-32 deter-

mines whether it is to run locally or remotely based on  the  default  terminal

line  found  when Kermit-32 is started.  Kermit-32 uses a list of logical names

to determine which terminal should be the default terminal line.  The first  of

these  names  which  translates to a terminal which is available (i.e., not al-

located by some other process) is used.  The logical names Kermit-32 tries  are

KER$COMM,  SYS$INPUT, SYS$$OUTPUT, and SYS$COMMAND.  If none of these translate

to an available terminal, Kermit-32 is running detached, and a terminal must be

specified  by  the  SET LINE command before any actions can be performed.  If a

terminal is found, Kermit-32 is running locally if this  is  a  terminal  other

than  the one controlling the job (i.e., different from SYS$COMMAND), otherwise

Kermit-32 is running remotely.  You can also select the line  directly  in  the

CONNECT command; the command

CONNECT TTA0

is equivalent to

SET LINE TTA0

CONNECT

If  you type SET LINE with no argument, you will deassign any previous assigned

line and revert to remote mode.

10.4.5. The SET and SHOW Commands                                      Page 241

-------------------------------------------------------------------------------

SET SERVER_TIMEOUT

Syntax: SET SERVER_TIMEOUT number

This specifies the number of seconds between  timeouts  during  server  command

wait,  0  specifies  that  no timeouts should occur during server command wait.

When a Kermit server times out, it sends a NAK packet.    Some  systems  cannot

clear  piled-up NAKs from their input buffers; if you're using such a system to

communicate with a Kermit-32 server, and you expect to be  leaving  the  server

idle  for  long periods of time, you should use this command to turn off server

command-wait timeouts.

SET TRANSMIT

Syntax: SET TRANSMIT DELAY integer, SET TRANSMIT ECHO ON/OFF

It is possible to set a few parameters associated with the raw TRANSMIT command

that  vary  both  what  the user sees on the screen as well as the speed of the

transmit.

SET TRANSMIT DELAY

This parameter is the amount of time to delay after  each  carriage  return  is

transmitted.   Valid delay values range between 0 (the default) and 9 tenths of

a second.  The format of the command is: SET TRANSMIT DELAY  d  Where  d  is  a

single decimal digit representing tenths of a second.

Some  remote  hosts  may  not  be  able  to  receive  the characters as fast as

Kermit-32 can send them.  The TRANSMIT DELAY can be used to slow up the  trans-

fer by adding a slight delay after each line is sent.

The transfer also runs slower if the transmit echo is on, and the remote system

is echoing the characters as it receives them.  If the transmit delay is set to

9  tenths  of  a  second, the remote system is echoing characters, the transmit

echo is on, and the remote system still cannot keep  up,  then  the  connection

should be made at a slower baud rate.

Conversely, the file transfer speed can be increased by: setting the delay to 0

and the echo off, stopping the remote system from  echoing  the  characters  it

receives, and connecting at higher baud rates.

SET TRANSMIT ECHO

This  command  controls what the user sees on the screen during the file trans-

fer.  The format of the command is SET TRANSMIT ECHO ON or OFF.

By default, the transmit echo is left off and the user sees the number of  each

line  after  it  has  been  transmitted.   With transmit echo on, the user sees

whatever the remote system would normally echo back to him while he  is  typing

in  a  file.    Note that turning the echo on typically slows the file transfer

down.

10.4.5. The SET and SHOW Commands                                      Page 242

-------------------------------------------------------------------------------

THE SHOW COMMAND

Syntax: SHOW [option]

The SHOW command displays various information:

ALL             All parameters.

BLOCK_CHECK_TYPE

The block check type being requested.

COMMUNICATIONS  Parameters  affecting  the  terminal  line  being used for com-

munication.

DEBUGGING       Debugging mode in effect, if any.

DELAY           The number of seconds Kermit-32 will delay  before  starting  a

SEND or RECEIVE command when in remote mode.

ESCAPE          The current escape character for the CONNECT processing.

FILE_PARAMETERS File type, file naming, and incomplete file disposition.

INCOMPLETE_FILE_DISPOSITION

The action to take when a transfer is aborted.

LINE            Terminal line in use.

LOCAL_ECHO      Whether characters should be echoed locally when CONNECTed.

PACKET          For incoming and outbound packets.

PARITY          The parity type in use.

RECEIVE         For inbound packets.

RETRY           The number of retries to be done on bad packets.

SEND            For outbound packets.

TRANSMIT        Parameters for TRANSMIT command.

VERSION         The program version number of Kermit-32.

10.4.6. Program Management Commands

THE HELP COMMAND

Syntax: HELP [topic {subtopic}]

Typing HELP alone prints a brief summary of Kermit-20 and its  commands.    You

can also type

HELP command

10.4.6. Program Management Commands                                    Page 243

-------------------------------------------------------------------------------

for  any  Kermit-20  command, e.g. "help send" or "help set parity" to get more

detailed information about a specific command.

THE EXIT AND QUIT COMMANDS

Syntax: EXIT

Exit from Kermit-32.  You can also exit from the Kermit-32 when it  is  waiting

for  a  command by typing a control-Z.  When Kermit-32 is running remotely, two

control-Y's will abort the transfer, bringing Kermit-32 back to  command  mode.

The two control-Y's must be typed together, as if a timeout occurs between them

the first is ignored.  When Kermit-32 is running locally, two control-Y's  will

stop  Kermit-32  and return you to DCL.  You will be able to CONTINUE if you do

not perform any command which runs  a  program.    However,  after  continuing,

control-A, control-X and control-Z will no longer be accepted as commands.

QUIT is a synonym for EXIT.

THE LOG COMMAND

Syntax: LOG [option [filespec]]

Log the specified option to the specified file:

SESSION         During  CONNECT log all characters that appear on the screen to

the specified file.  During CONNECT, the  session  log  can  be

temporarily  turned off during the remote session by typing the

escape character followed by Q (for Quit logging),  and  turned

on  again by typing the escape character followed by R (for Re-

sume logging).

TRANSACTIONS    During file transfer, log the progress of each  file.    Trans-

action  logging  is  recommended  for  long  or unattended file

transfers, so that you don't have to watch the screen.  The log

may  be  inspected  after  the transfer is complete to see what

files were transferred and what errors may have occurred.

DEBUGGING       Log debugging info to the specified file.  If no SET  DEBUGGING

command  was  previously issued, the file will be opened and no

information written.  If DEBUGGING is turned on (either via the

SET  DEBUGGING  command  or  by  typed control-D during a local

transfer), the packet debugging information will be written  to

the  file.    Packet format is described in the Kermit Protocol

Manual.

Any log files are closed when you EXIT or QUIT from Kermit.  You may explicitly

close  a log file and terminate logging by using the LOG command without a file

specification.

10.4.6. Program Management Commands                                    Page 244

-------------------------------------------------------------------------------

THE STATUS COMMAND

Syntax: STATUS

The current status of Kermit-32 will be displayed.  This includes the number of

characters  that  have  been sent and received from the remote Kermit. Also in-

cluded is an estimate of the effective baud rate of the transfer.  This  number

is not intended to be exact, but only an indication of what range of throughput

has been provided.

10.5. Raw Upload and Download

THE TRANSMIT COMMAND

Syntax: TRANSMIT file-spec

The TRANSMIT command allows you to upload files "raw"  to  systems  that  don't

have a Kermit program available.  Note that there is no error checking or pack-

ets involved in this method of file transfer.

This command does a raw transmit of an ASCII file, one  character  at  a  time,

with carriage returns (no line-feeds) at the end of each line.  It is used with

Kermit-32 in local mode.  The user  must  first  prepare  the  remote  host  to

receive  the file by starting an edit session in input mode.  Then the user can

escape back to Kermit-32 and issue the TRANSMIT command.  After the transmit is

finished,  the  user  then  CONNECTs back to the remote host again and ends the

edit session.

During a file transmit, the following control characters can be used to  affect

the transfer in progress:

CTRL-C          Abort the transmit

CTRL-X          Abort the file currently being transmitted

CTRL-Z          Abort the file group currently being transmitted

See SET TRANSMIT for information about controlling echo and delays.

THE LOG SESSION COMMAND

Syntax: LOG SESSION file-spec

"Raw  Download"  is  the term commonly used to describe the capture of a remote

file on the local system, without any kind of error  detection  or  correction.

This  allows  you  to obtain files from remote systems that do not have Kermit,

but with the risk of loss or corruption of data.

Kermit-32 provides raw downloading via the LOG SESSION command  during  CONNECT

to  a  remote system.  The session log is described above.  To use session log-

ging to capture a file:

1. Run Kermit on the VAX/VMS system.

10.5. Raw Upload and Download                                          Page 245

-------------------------------------------------------------------------------

2. SET LINE to the terminal line through which you will be connected to

the remote system.

3. Perform any required SET commands to condition Kermit for communica-

tion with the remote system.

4. CONNECT to the remote system and log in.

5. Condition your job on the remote system not to pause at the end of a

screenful  of  text,  and give whatever commands may be necessary to

achieve a "clean" terminal listing -- for instance, disable messages

from the system or other users.

6. Type  the  appropriate command to have the desired file displayed at

the terminal, but do not type the terminating carriage return.    On

most systems, the command would be "type", on Unix it's "cat".

7. Escape  back  to Kermit-32 and give the LOG SESSION command with the

file specification where you wish to store the data.

8. CONNECT back to the remote system and type a carriage return.    The

file  will  be  displayed on your screen and recorded in the session

log file.

9. Escape back to Kermit-32 and give the LOG SESSION command without  a

file specification.

The  file  you specified will contain everything that was typed on your screen.

You will probably  find  that  some  editing  necessary  to  remove  extraneous

prompts, messages, padding characters, or terminal escape sequences, or to fill

in lost or garbled characters.

Use the TRANSMIT command for raw uploading.

10.6. Installation of Kermit-32

Kermit-32 may be installed either by rebuilding the  entire  program  from  the

sources,  or  by making use of the distributed copy of the program.  When being

built from sources, it may be built using either a BLISS-32  compiler  or  just

the MACRO-32 assembler.

FILES

Kermit-32  is  built from a number of BLISS-32 sources and one MACRO-32 source.

In order to make it possible for sites  without  BLISS-32  to  build,  MACRO-32

sources  generated  by BLISS-32 are also included for all of the BLISS modules.

In the normal distribution of Kermit-32, all of the files start with the prefix

"VMS".    This  will  need to be changed to "KER" in order to build the program

properly.  The following files are distributed as part of Kermit-32:

VMSTT.BLI       Common BLISS source for the terminal text output support.

VMSGLB.BLI      Common BLISS source for the global storage for VMSMSG.BLI.

10.6. Installation of Kermit-32                                        Page 246

-------------------------------------------------------------------------------

VMSMSG.BLI      Common BLISS source for the protocol handling module.

VMSCOM.REQ      Common  BLISS  require  file  which  defines   various   common

parameters.  This is required by VMSMSG.BLI.  This file must be

renamed to KERCOM.REQ.

VMSMIT.BWR      "Beware File" for Kermit-32 (read it!).

VMSMIT.BLI      BLISS-32 source for the command parser, and some basic  support

routines.

VMSFIL.BLI      BLISS-32 source for the file I/O.

VMSTRM.BLI      BLISS-32  source for the terminal processing.  This handles the

driving of the terminal line for the transfers and the  connect

command processing.

VMSSYS.BLI      System  interface  routines  for  the  Kermit  generic  command

processing.

VMSGEN.MAR      Macro-32 source file that contains the REMOTE command text that

is  given  to  VMS.  Sites desiring to change what DCL commands

are used to process the various  generic  server  commands  can

make those changes in this source.  This also contains the text

of the help message returned in response to the server  generic

help command.

VMSERR.MSG      MESSAGE source for error messages used by VAX/VMS Kermit.

VMSERR.REQ      BLISS-32  require  file  which defines the error codes. This is

REQUIREd by the BLISS-32 sources.

VMSMIT.MSS      SCRIBE source file for VMSMIT.DOC (this document).

VMSMIT.RNH      RUNOFF source for the help files for VAX/VMS Kermit.  When this

is  run through RUNOFF with /VARIANT=SYSTEM, it produces a .HLP

(VMSSYS.HLP) file suitable for inserting into the  system  help

library  (SYS$HELP:HELPLIB.HLB)  to  provide a KERMIT topic for

the system HELP command.  When run through RUNOFF  without  the

/VARIANT=SYSTEM,  it  produces  a  .HLP file (VMSUSR.HLP) to be

stored on SYS$HELP: for use by the Kermit HELP command.

VMSSYS.HLP      RUNOFF output file for system wide Kermit HELP.

VMSUSR.HLP      RUNOFF output file for Kermit's HELP command.

VMSINS.COM      Command file to build and install VAX/VMS Kermit.

VMSMIT.EXE      The executable (binary) file of VAX/VMS Kermit.  Note that this

is  only  included  when  it is possible.  This may be either a

copy of the .EXE file which was transferred  to  a  TOPS-10  or

TOPS-20  system using Kermit with Kermit-10 (or -20) using FILE

BYTE-SIZE 8-BIT, or a direct copy of the .EXE file (if the file

is  either on a native VAX/VMS tape or is residing on a VAX/VMS

system).

10.6. Installation of Kermit-32                                        Page 247

-------------------------------------------------------------------------------

VMSMIT.HEX      A hexified version of .EXE file for VMS Kermit.  This file  can

be  dehexified  using  the  supplied  program.  In the hexified

form, the file should be transferable  over  any  medium  which

handles normal text.  This is the most reliable copy of the ex-

ecutable version of VMS Kermit.

VMSHEX.MAR      Source for the hexification program.  This is the program which

was used to produce VMSMIT.HEX.  It can also be used to produce

hexified version of any (or at least almost any) Files-11 file.

The  dehexification  program should then be able to reproduce a

copy of the original file with the  file  parameters  correctly

set.  Note that the format used for the hexified files is basi-

cally Intel hex format.  There are some additional records used

to  store the record format, etc.  Also, the file name as typed

to the prompt from VMSHEX is stored in the hexified version  of

the file for use by the dehexification program.  By doing this,

it is possible to store more than one binary file with a single

hexified file.

VMSDEH.MAR      Source for the dehexification program.

VMSV31.*Version 3.1 of VMS Kermit, the last version that will run under release

3.x of VMS.  Versions 3.2 and later require VMS release 4.0  or

later.

VMSV3x.MEM      Documentation  on the changes between releases 3.1 and 3.1, and

3.2 and 3.3 of Kermit-32, and  additional  installation  infor-

mation.

As  distributed,  Kermit-32  should work on any VAX/VMS system (version 4.0 and

later).  Customization  is  possible  with  or  without  a  BLISS-32  compiler.

Default parameter values may be changed by changing the appropriate LITERALs in

the BLISS-32 source for VMSMSG, or the actual values which are  stored  in  the

routine MSG_INIT in the MACRO-32 source for VMSMSG.

Sites  can  also  easily  change the commands which are used for processing the

generic server functions (REMOTE commands when running as a server).  The  text

which makes up these commands is in the file VMSGEN.MAR, along with the text of

the REMOTE HELP message.  This allows a site to make use of local programs  for

performing  some  of the commands (perhaps using FINGER to perform the WHO com-

mand, etc.).

BUILDING KERMIT-32 FROM SOURCES

A command file is included which will build Kermit-32 from either the  BLISS-32

or  MACRO-32 sources and optionally install Kermit-32 on the system.  This file

(VMSINS.COM) has not been extensively tested, however it should  work  on  most

systems.    It  is also a good example of what needs to be done to compile each

file and load the entire program.  It also contains the commands  necessary  to

install  a  version of Kermit-32 on the system once the .EXE and .HLP files are

generated.

10.6. Installation of Kermit-32                                        Page 248

-------------------------------------------------------------------------------

USING KERMIT-32 TO INSTALL A NEW VERSION OF ITSELF

If you already have a version of Kermit-32 running on a VAX/VMS system, you can

use it to transfer a new version of itself from another system.  If you have no

need to build Kermit-32 from sources (because you have no local modifications),

you can just transfer the new VMSMIT.EXE (or VMSMIT.HEX), and the new help file

VMSMIT.RNH.

If you have access to a system which has a copy of VMSMIT.EXE, you can transfer

it simply by setting the FILE TYPE to FIXED on the receiving Kermit.  Make sure

that the sending Kermit is willing to send the file as a binary file.   If  the

sending Kermit is another copy of Kermit-32, you don't need to do anything.  If

the sending Kermit is Kermit-10 or Kermit-20, make sure the file will  be  sent

as  an  eight-bit binary file by using SET FILE BYTE-SIZE EIGHT.  If some other

Kermit is sending the file, make sure to give it whatever command is  necessary

to  ensure  that  it  sends  eight-bit binary data from the file.  Also, if the

originating system is not a VAX/VMS, TOPS-10 or TOPS-20 system, make sure  that

the  file was stored on that system correctly to start out with.  Normally, the

file VMSMIT.EXE will only be available from a VAX/VMS, TOPS-10 or TOPS-20  sys-

tem.

If you have access to a copy of VMSMIT.HEX, you will need to transfer that file

(as a normal ASCII text file).  You will also need a copy of VMSDEH.MAR.  After

you  have  obtained both files, you can produce a copy of the .EXE file as fol-

lows.  First compile VMSDEH by using the command MACRO VMSDEH.  Then load it by

LINK VMSDEH.  Now run VMSDEH and when it asks for a file name, type VMSMIT.HEX.

The program will run for a short time and produce the .EXE file.

The system wide and user help files are produced from VMSMIT.RNH by RUNOFF.  To

produce the user help file (the one used by Kermit-32's HELP command), type:

$ RUNOFF VMSMIT.RNH/OUTPUT=KERUSR.HLP

$ LIBRARY/CREATE/HELP SYS$HELP:KERMIT.HLB KERUSR.HLP

To  produce the system wide help file and install it in the system help library

type:

$ RUNOFF VMSMIT.RNH/OUTPUT=KERSYS.HLP/VARIANT=SYSTEM

$ LIBRARY/REPLACE/HELP SYS$HELP:HELPLIB.HLB KERSYS.HLP

This allows the DCL HELP command to provide information on Kermit-32.

If you want to allow your users to assign external terminal lines for  connect-

ing  to remote systems from the VAX, e.g. by dialing out, you will have to con-

figure those lines to allow the desired access.  Otherwise, user's will  get  a

message  like  "No  privilege  for attempted operation" when they do a SET LINE

command.  Sample commands for terminal TXA0: might include:

$ SET PROTECTION=(W:R) TXA0:/DEVICE

or

$ SET PROTECTION=(W:RWLP)/DEVICE/OWNER=[1,4] TXA0:

or

10.6. Installation of Kermit-32                                        Page 249

-------------------------------------------------------------------------------

$ SET ACL/OBJECT=DEVICE/ACL=(IDENTIFIER=INTERACTIVE,OPTIONS=NONE,-

ACCESS=READ+WRITE) TXA0:

Consult your VAX/VMS system manager's manual for the ramifications  (especially

on security) of each of these commands.

11. DECSYSTEM-20 KERMIT                                                Page 250

-------------------------------------------------------------------------------

11. DECSYSTEM-20 KERMIT

Authors:    Frank da Cruz, Bill Catchings, Columbia University

Language:   MACRO-20

Version:    4.2(262)

Date:       January 1988

Kermit-20 Capabilities At a Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Timeouts:                          Yes

8th-bit prefixing:                 Yes

Repeat character compression:      Yes

Alternate block check types:       Yes

Communication settings:            Yes

Transmit BREAK:                    Yes

IBM mainframe communication:       Yes

Transaction logging:               Yes

Session logging:                   Yes

Debug logging:                     Yes

Raw transmit:                      Yes

Login scripts:                     Yes

Act as server:                     Yes

Talk to server:                    Yes

Advanced commands for servers:     Yes

Local file management:             Yes

Command/init files:                Yes

Long packets:                      No

Sliding windows:                   No

Handle file attributes:            No

Kermit-20  is  a  program that implements the Kermit file transfer protocol for

the Digital Equipment Corporation DECSYSTEM-20 mainframe computer.  It is writ-

ten in MACRO-20 assembly language and should run on any DEC-20 system with ver-

sion 4 of TOPS-20 or later.

The Kermit-20 section will describe the things you should know about the DEC-20

file system in order to make effective use of Kermit, and then it will describe

the special features of the Kermit-20 program.

11.1. The DEC-20 File System

The features of the DEC-20 file system of greatest interest to Kermit users are

the  form of the file specifications, and the distinctions between text and bi-

nary files.

11.1. The DEC-20 File System                                           Page 251

-------------------------------------------------------------------------------

DEC-20 File Specifications

DEC-20 file specifications are of the form

DEVICE:<DIRECTORY>NAME.TYPE.GEN;ATTRIBUTES

where the DIRECTORY, NAME, and TYPE may each be up to 39 characters in  length,

GEN  is  a  generation  (version  number),  and various attributes are possible

(protection code, account, temporary, etc).  Generation and attributes are nor-

mally  omitted.   Device and directory, when omitted, default to the user's own

(or "connected") disk and directory.  Thus NAME.TYPE is normally sufficient  to

specify  a  file,  and only this information is sent along by Kermit-20 with an

outgoing file.

The device, directory, name, and type fields  may  contain  uppercase  letters,

digits,  and  the  special  characters  "-"  (dash),  "_" (underscore), and "$"

(dollar sign).  There are no imbedded or trailing spaces.  Other characters may

be  included by prefixing them (each) with a Control-V.  The fields of the file

specification are set off from one another by the punctuation indicated above.

The device field specifies a physical or "logical" device upon which  the  file

is  resident.    The  directory field indicates the area on the device, for in-

stance the area belonging to the owner of the file.  Kermit-20 does not  trans-

mit  the  device or directory fields to the target system, and does not attempt

to honor device or directory fields that may appear in incoming file names; for

instance, it will not create new directories.

The  name  is  the  primary identifier for the file.  The type, also called the

"extension", is an indicator which, by convention, tells what kind of  file  we

have.    For  instance  FOO.FOR  is  the source of a Fortran program named FOO;

FOO.REL might be the relocatable object module produced by  compiling  FOO.FOR;

FOO.EXE could an executable program produced by LOADing and SAVing FOO.REL, and

so forth.

The DEC-20 allows a group of files to be specified in a single file  specifica-

tion  by  including  the  special  "wildcard"  characters,  "*" and "%".  A "*"

matches any string of characters, including no characters at all; a "%" matches

any single character.  Here are some examples:

*.FOR   All  files  of  type  FOR  (all  Fortran source files) in the connected

directory.

FOO.*   Files of all types with name FOO.

F*.*    All files whose names start with F.

F*X*.*  All files whose names start with F and contain at least one X.

%.*     All files whose names are exactly one character long.

*.%%%*  All files whose types are at least three characters long.

Wildcard notation is used on many computer systems in similar ways, and  it  is

the mechanism most commonly used to instruct Kermit to send a group of files.

11.1. The DEC-20 File System                                           Page 252

-------------------------------------------------------------------------------

TEXT FILES AND BINARY FILES

The  DEC-20, like most computers, has a file system with its own peculiarities.

Like many other systems, the DEC-20 makes a distinction between text files  and

binary files.  Text files are generally those composed only of printing charac-

ters (letters, digits,  and  punctuation)  and  "carriage  control"  characters

(carriage  return,  line  feed, form feed, tab).  Text files are designed to be

read by people.  Binary files are designed to be read by  a  computer  program,

and  may  have any contents at all.  If you use the DEC-20 TYPE command to dis-

play a text file on your terminal, the result will be  intelligible.    If  you

type  a  binary  file on your terminal, you will probably see mainly gibberish.

You can not always tell a text file from a binary file by its name or directory

information,  though in general files with types like .TXT, .DOC, .HLP are tex-

tual (as are "source files" for computer programs like text formatters and pro-

gramming  language  compilers),  and files with types like .EXE, .REL, .BIN are

binary.

The DEC-20 has an unusual word size, 36 bits.  It differs from most other  sys-

tems by storing text in 7-bit, rather than 8-bit, bytes.  Since text is encoded

in the 7-bit ASCII character set, this allows more efficient  use  of  storage.

However,  the  word size is not a multiple of the normal byte size.  The DEC-20

therefore stores five 7-bit characters per word, with one bit left over.

It is also possible to store files with other byte sizes.  The  common  layouts

of bytes within a word are shown in Figure 11-1.

The  minimum unit of disk allocation on the DEC-20 is a page, 512 36-bit words,

or 2560 7-bit characters, or 2048 8-bit bytes.  Any file that contains at least

one  bit  of information occupies at least a full page on the disk.  The direc-

tory information for a file includes the number of pages occupied on the  disk,

the bytesize of the file, and the number of bytes of that size which are in the

file.  This information can be seen by using the DEC-20 VDIRECTORY command, for

instance

@vdir foo.*

PS:<MY-DIRECTORY>

Name     Protection    Pages Bytes(Size) Creation

FOO.COM.1;P774242          1 384(8)     27-Dec-83

MAC.1;P774242           1 152(7)     27-Dec-83

.REL.1;P774242           1 39(36)     27-Dec-83

.EXE.1;P774242           2 1024(36)   27-Dec-83

Total of 5 pages in 4 files

In  this  example, FOO.MAC occupies 1 page, and is composed of 152 7-bit bytes.

This file is textual (program source for the MACRO assembler),  152  characters

long.  Programs which read text files (such as text editors, program compilers,

the TYPE command, etc) determine  the  end  of  a  file  from  the  byte  count

specified  in  the directory.  Kermit-20 determines the end of file in the same

way, so although FOO.MAC occupies an entire 2560-byte page of storage, only the

first  152  characters  are transmitted.  Binary files, such as FOO.EXE (an ex-

ecutable DEC-20 program), tend to  occupy  full  pages.    In  this  case  too,

Kermit-20 uses the byte count to determine the end of file.

Why  do  you  need  to know all this?  In most cases, you don't.  It depends on

11.1. The DEC-20 File System                                           Page 253

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

7: Text Files: Five 7-bit bytes per word.

+------+------+------+------+------++

|      |      |      |      |      ||

+------+------+------+------+------++

0      7      14     21     28     35

Normally,  bit 35 is unused and set to zero.  However, in EDIT (or SOS,

or OTTO) line-numbered files, bit 35 is set to 1 when the word contains

a line number.

8: "Foreign" binary files:  Four 8-bit bytes per word.

+-------+-------+-------+-------+---+

|       |       |       |       |   |

+-------+-------+-------+-------+---+

0       8       16      24      32  35

Bits 32-35 are unused.

36: "Native" binary files:  One 36-bit byte per word.

+-----------------------------------+

|                                   |

+-----------------------------------+

0                                   35

All bits are used.

Figure 11-1:   DECSYSTEM-20 Word/Byte Organization

-------------------------------------------------------------------------------

whether you are using the DEC-20 as your "home base".

USING A MICROCOMPUTER TO ARCHIVE DEC-20 FILES

Most computers (other than the DEC-10 and DEC-20)  store  characters  in  8-bit

bytes.    Let's call any such system an 8-bit-byte system.  Microcomputers that

run CP/M or MS-DOS or PC-DOS, and any computers  than  run  Unix,  store  these

8-bit  bytes in a linear sequence.  Certain other 8-bit-byte systems (PDP-11 or

VAX systems with FILES-11, IBM mainframes)  have  more  complex  file  formats.

This  discussion  applies  to  all  linear  8-bit-byte  systems, including most

popular microcomputers.

Kermit can send any "native" DEC-20 sequential file,  text  or  binary,  to  an

8-bit-byte  system  and  bring  it  back to the DEC-20 restored to its original

form.  If you are using a microcomputer to archive your DEC-20 files, you  need

never  concern  yourself  with  details  of byte size or file format.  The same

11.1. The DEC-20 File System                                           Page 254

-------------------------------------------------------------------------------

holds true between two DEC-20s, or a DEC-10 and a DEC-20.

There is, however, one special complication of which you should be aware.  Cer-

tain  microcomputer  operating  systems,  notably CP/M, do not have an entirely

satisfactory way of indicating the end of file.  The file length is recorded in

blocks  rather  than bytes.  For text files, the end of file is marked within a

block by inserting a Control-Z after the last data character.    Binary  files,

however,  might easily contain Control-Z characters as data.  Therefore, in or-

der not to lose data, these systems must  transmit  binary  files  in  complete

blocks.  If the binary file is of foreign origin (for instance, from a DEC-20),

and it did not happen to fill up the last block when it was transferred to  the

micro,  then  when  that  file  is sent back to the system of origin in "binary

mode," junk will appear at the end (if it is sent back in "text mode," it could

be  truncated at the first data byte that happened to correspond to Control-Z).

For DEC-20 programs in .EXE format, this generally has no effect  on  the  run-

nability  or behavior of the program.  But for other binary files, particularly

internal format numerical data or relocatable program object (.REL) files,  the

junk  could  have  bad  effects.  For instance, extraneous data at the end of a

.REL file will generally cause LINK to fail to load the file.

Most microcomputer Kermit programs have commands to control end-of-file  detec-

tion -- commands like SET FILE TEXT, SET FILE BINARY, SET EOF CTRLZ.

USING THE DEC-20 TO ARCHIVE MICROCOMPUTER FILES

You can use Kermit to send textual files from a microcomputer or any 8-bit sys-

tem to the DEC-20 with no special provisions, since Kermit-20  stores  incoming

characters  in 7-bit bytes as text unless you explicitly instruct it otherwise.

But Kermit-20 has no automatic way of distinguishing an  incoming  binary  file

2

from  an  incoming  text  file.  Binary files from 8-bit-byte systems generally

contain significant data in the 8th bit, which would be lost  if  the  incoming

characters  were  stored  in  7-bit bytes, rendering the file useless when sent

back to the original system.  Thus if you want to use Kermit to  store  foreign

8-bit  binary  data  on the DEC-20, you must tell it to store such files with a

bytesize of 8 rather than 7.  This can be the source of much confusion and  in-

convenience.  In particular, you cannot use a "wildcard send" command to send a

mixture of text and binary files from  an  8-bit-byte  system  to  the  DEC-20;

rather,  you  must send all text files with Kermit-20's file bytesize set to 7,

and all 8-bit binary files with the bytesize set to 8.

Once you get the foreign binary file into the DEC-20, stored with  the  correct

bytesize  (as FOO.COM is stored in the example above), you need take no special

measures to send it back to its system of origin.  This  is  because  Kermit-20

honors  the  bytesize  and byte count from the directory.  For instance, if you

told Kermit-20 to SEND FOO.*, every file in the example above would  be  trans-

mitted in the correct manner, automatically.

The  previous discussion assumes you want to store text files in usable form on

the DEC-20.  However, if you are using the DEC-20 purely as  a  repository  for

_______________

2

Unless the incoming file has an "ITS Binary Header"; see below.

11.1. The DEC-20 File System                                           Page 255

-------------------------------------------------------------------------------

your  microcomputer  files, and you have no desire to display or share the con-

tents of those files on the DEC-20, you can SET FILE BYTESIZE 8 for all  incom-

ing  files,  both text and binary.  When the files are sent back to a microcom-

puter, they will be stored correctly.

FILES KERMIT-20 CANNOT HANDLE

The Kermit protocol can only accommodate transfer of  sequential  files,  files

which are a linear sequence of bytes (or words).

Some files on the DEC-20 are not sequential, and cannot be successfully sent or

received by Kermit-20.    These  include  directory  files,  files  with  holes

(missing  pages),  ISAM files, and RMS files.  These files require external in-

formation (kept in the DEC-20's file descriptor block and/or  index  table)  in

order  to  be  reconstructed; when sending files, Kermit-20 presently transmits

only the file name and the contents of the file.  External control  information

and file attributes are not transmitted.

11.2. Program Operation

Kermit-20's  prompt is "Kermit-20>".  Kermit-20 will accept a single command on

the Exec command line, like this:

@

@Kermit send foo.bar

the file is sent

@

or you can run the program interactively to issue several commands, like this:

@

@Kermit

TOPS-20 Kermit version 4.2(262)

Kermit-20>send foo.*

files are sent

Kermit-20>statistics

performance statistics are printed

Kermit-20>receive

files are received

Kermit-20>exit

@

During  interactive  operation,  you  may  use  the  TOPS-20  help  ("?")   and

recognition (ESC) features freely while typing commands.  A question mark typed

11.2. Program Operation                                                Page 256

-------------------------------------------------------------------------------

at any point in a command displays the options available at that point;  typing

an  ESC  character  causes  the current keyword or filename to be completed (or

default value to be supplied), and a "guide word" in parentheses to  be  typed,

prompting  you for the next field.  If you have not typed sufficient characters

to uniquely specify the keyword or filename (or if there is no  default  value)

then a beep will be sounded and you may continue typing.

Command  keywords  may  be  abbreviated to their shortest prefix that sets them

apart from any other keyword valid in that field.

If you have a file called KERMIT.INI in your login  directory,  Kermit-20  will

execute  an  automatic TAKE command on it upon initial startup.  KERMIT.INI may

contain any Kermit-20 commands, for instance SET commands, or DEFINEs  for  SET

macros to configure Kermit-20 to various systems or communications media.

Kermit-20 provides most of the commands possible for an "ideal" Kermit program,

as described in the main part of the Kermit User Guide.  The following sections

will concentrate on system-dependent aspects of Kermit-20.

11.3. Remote and Local Operation

Kermit-20  normally  runs  in  remote mode, with the user sitting at a PC.  But

Kermit-20 can also run in local mode.  Local operation of Kermit-20  is  useful

if the DEC-20 has an autodialer, or a hardwired connection to another computer.

When in local mode, file transfer takes place over an assigned  TTY  line,  and

Kermit-20  is free to update your screen with status information, and to listen

to your keyboard for interrupt characters.

Kermit-20 enters local mode when you issue a SET LINE n command, where n is the

octal TTY number of any line other than your own controlling terminal.

11.4. Conditioning Your Job for Kermit

Kermit-20  does as much as it can to condition your line for file transfer.  It

saves all your terminal  and  link  settings,  and  restores  them  after  use.

However,  there  are some sources of interference over which Kermit-20 can have

no control.  In particular, messages issued by superior or parellel forks could

become  mingled with Kermit packets and slow things down or stop them entirely.

For this reason, before using Kermit-20 for any extended period, you should:

- Type the Exec commands SET NO MAIL-WATCH and SET NO ALERTS

- Make sure you don't have any print or batch jobs  pending  that  were

submitted with the /NOTIFY option.

- Make sure you don't have any superior or parallel forks that have en-

abled terminal interrupts on Control-A; these  could  prevent  Kermit

packets (which start with Control-A) from getting through.

After  running Kermit, you can restore your mail-watch and alerts by hand.  Al-

ternatively, you could have an Exec command file for invoking Kermit like this:

set no alerts

set no mail-watch

11.4. Conditioning Your Job for Kermit                                 Page 257

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Local Operation of Kermit-20:

DECSYSTEM-20

+---------------------------------------+

|                                       |

|   +--------------------+              |

|   | Your Job           |              |

|   |                    |              |

|   |   +------------+   | <--Commands  | Your Job's

|   |   | Kermit-20  +---+--------------+----------------- (-: You

|   |   |            |   | Display--->  | Controlling TTY

|   |   |            |   |              |

|   |   |            |   |              |

|   |   |            |   | <--Packets   | Kermit's

|   |   |            +---+--------------+-----------------> Remote

|   |   +------------+   | Packets-->   | Assigned TTY      System

|   |                    |              |

|   +--------------------+              |

|                                       |

+---------------------------------------+

Figure 11-2:   DEC-20 Kermit Local Operation

-------------------------------------------------------------------------------

kermit

set mail-watch

set alert  1:00PM Go to lunch

set alert  6:00PM Go to dinner

set alert 11:30PM Go to sleep

11.5. Kermit-20 Commands

This section describes the Kermit-20 commands -- in detail  where  they  differ

from  the  "ideal" Kermit, briefly where they coincide.  Kermit-20 has the fol-

lowing commands:

BYE  to remote server.

CLEAR  a stuck connection

CLOSE  log file and stop logging remote session.

CONNECT  as terminal to remote system.

CWD  change local working directory.

DEFINE  macros of Kermit-20 commands.

DELETE  local files.

DIRECTORY  listing of local files.

ECHO  a line of text.

EXIT  from Kermit-20.

FINISH  Shut down remote server.

GET  remote files from server.

HELP  about Kermit-20.

INPUT  characters from communication line.

11.5. Kermit-20 Commands                                               Page 258

-------------------------------------------------------------------------------

LOCAL  prefix for local file management commands.

LOG  remote terminal session.

OUTPUT  characters to communication line.

PAUSE  between commands.

PUSH  to TOPS-20 command level.

QUIT  from Kermit-20

RECEIVE  files from remote Kermit.

REMOTE  prefix for remote file management commands.

RUN  a DEC-20 program.

SEND  files to remote Kermit.

SERVER  mode of remote operation.

SET  various parameters.

SHOW  various parameters.

SPACE  inquiry.

STATISTICS  about most recent file transfer.

TAKE  commands from a file.

TRANSMIT  a file "raw".

TYPE  a local file.

11.5.1. Commands for File Transfer

Kermit-20 provides the standard SEND, RECEIVE, and GET commands  for  transfer-

ring files using the Kermit protocol.

THE SEND COMMAND

Syntax:

Sending a single file:

SEND nonwild-filespec1 (AS) [filespec2]

Sending multiple files:

SEND wild-filespec1 (INITIAL) [filespec2]

The  SEND command causes a file or file group to be sent from the DEC-20 to the

other system.  There are  two  forms  of  the  command,  depending  on  whether

filespec1  contains  wildcard characters ("*" or "%").  Kermit-20 automatically

recognizes the two cases  and  issues  the  appropriate  guide  word,  (AS)  or

(INITIAL), depending on the form of filespec1.

Sending a File Group

If filespec1 contains wildcard characters then all matching files will be sent,

in alphabetical order (according to the ASCII collating sequence) by name.   If

a  file  can't be opened for read access, it will be skipped.  The initial file

in a wildcard group can be specified with the optional filespec2.  This  allows

a previously interrupted wildcard transfer from where it left off, or it can be

used to skip some files that would be transmitted first.

11.5.1. Commands for File Transfer                                     Page 259

-------------------------------------------------------------------------------

Sending a Single File

If filespec1 does not contain any wildcard characters,  then  the  single  file

specified  by  filespec1  will  be  sent.  Optionally, filespec2 may be used to

specify the name under which  the  file  will  arrive  at  the  target  system;

filespec2  is  not  parsed or validated in any way by Kermit-20, but lower case

letters are raised to upper case, and leading "whitespace"  (blanks  and  tabs)

are  discarded.    If  filespec2 is not specified, Kermit-20 will send the file

3

with its own name.

SEND Command General Operation:

Files  will  be  sent  with  their  DEC-20  filename and filetype (for instance

FOO.BAR, no device or directory field, no generation number or attributes).  If

you expect to be sending files whose names contain characters that would be il-

legal in filenames on the target system, and you know that the  Kermit  on  the

target  system does not have the ability to convert incoming filenames, you can

issue the SET FILE NAMING NORMAL-FORM command to have Kermit-20 replace suspect

characters by X's.

Each file will be sent according to its bytesize and byte count from the direc-

tory unless you specify otherwise using SET FILE BYTESIZE, or unless  the  file

has  an  "ITS Binary" header.  If the bytesize is 8, then four 8-bit bytes will

be sent from each DEC-20 36-bit word, and the  low  order  four  bits  will  be

skipped.    If other than 8, then five 7-bit bytes will be sent from each word,

with the 8th bit of the 5th character set to the value  of  the  remaining  bit

4

("bit 35") from the word.

If communication line parity is being used (see SET PARITY), Kermit-20 will re-

quest that the other Kermit accept a special kind of prefix notation for binary

files.  This is an advanced feature, and not all Kermits have it; if the  other

Kermit  does  not  agree  to use this feature, binary files cannot be sent cor-

rectly.  This includes executable programs (like DEC-20 .EXE files,  CP/M  .COM

files),  relocatable  object  modules  (.REL files), as well as text files with

line sequence numbers.

Kermit-20 will also ask the other Kermit whether it can handle a special prefix

encoding  for  repeated characters.  If it can, then files with long strings of

repeated characters will be  transmitted  very  efficiently.    Columnar  data,

highly  indented  text,  and  binary  files are the major beneficiaries of this

technique.

_______________

3

Control-V's, which are used to quote otherwise illegal characters in  DEC-20

file specifications, are stripped.

4

This  is  the  same  method  used  by  the  DEC-20  to encode 36-bit data on

"ANSI-ASCII" tapes.  It allows not only DEC-20 binary files, but also the line-

sequence-numbered  files  produced  by  EDIT, SOS, or OTTO, which use bit 35 to

distinguish line numbers from text, to be sent and retrieved correctly.

11.5.1. Commands for File Transfer                                     Page 260

-------------------------------------------------------------------------------

If you're running Kermit-20 locally, for instance dialing out from  the  DEC-20

to  another  system  using an autodialer, you should have already run Kermit on

the remote system and issued either a RECEIVE or a SERVER command.    Once  you

give  Kermit-20  the  SEND  command, the name of each file will be displayed on

your screen as the transfer begins; a "." will be displayed for  every  5  data

packets  sucessfully  sent,  and a "%" for every retransmission or timeout that

occurs (you may also elect other typeout options with the SET  DEBUG  command).

If  the  file is successfully transferred, you will see "[OK]", otherwise there

will be an error message.   When  the  specified  operation  is  complete,  the

program  will  sound  a beep.  If you see many "%" characters, you are probably

suffering from a noisy connection.   You  may  be  able  to  cut  down  on  the

retransmissions  by using SET SEND PACKET-LENGTH to decrease the packet length;

this will reduce the probability that a  given  packet  will  be  corrupted  by

noise, and reduce the time required to retransmit a corrupted packet.

During local operation, you can type Control-A at any point during the transfer

to get a brief status report.   You may also type Control-X or Control-Z to in-

terrupt the current file or file group.

THE RECEIVE COMMAND

Syntax:  RECEIVE [filespec]

The  RECEIVE  command  tells Kermit-20 to receive a file or file group from the

other system.  If only one file is being received, you may include the optional

filespec  as  the name to store the incoming file under; otherwise, the name is

taken from the incoming file header.  Even if the name in the header is  not  a

legal TOPS-20 file name, Kermit-20 will store it under that name, in which case

you can refer to it later only by quoting each illegal character (spaces,  con-

trol  characters, etc) with Control-V.  If for some reason an incoming filename

simply  cannot  be  converted  to  legal  form,  the  file  will  be  saved  as

-UNTRANSLATABLE-FILENAME-.KERMIT  (new  generation).  You may also use SET FILE

NAMING NORMAL-FORM to have Kermit-20 choose more conventional names for  incom-

ing files.

If  an  incoming  file  has  the  same name as an existing file, Kermit-20 just

creates a new generation of the same name and  type,  for  instance  FOO.BAR.3,

FOO.BAR.4.    The  oldest generation will be automatically deleted, but you can

still UNDELETE it.

Incoming files will all be stored with the prevailing bytesize, 7  by  default,

which  is  appropriate for text files.   If you are asking Kermit-20 to receive

binary files from a microcomputer or other 8-bit system, you  must  first  type

SET  FILE BYTESIZE 8.  Otherwise, the 8th bit of each byte will be lost and the

file will be useless when sent back to the system of origin.

If you have SET PARITY, then 8th-bit prefixing will be requested.  If the other

side  cannot  do  this,  binary  files cannot be transferred correctly.  In all

cases, Kermit-20 will request the other Kermit to compress repeated characters;

if  the  other  side can do this (not all Kermits know how) there may be a sig-

nificant improvement in transmission speed.

If an incoming file does not arrive in its entirety,  Kermit-20  will  normally

discard it; it will not appear in your directory.  You may change this behavior

by using the command SET INCOMPLETE KEEP, which will cause as much of the  file

11.5.1. Commands for File Transfer                                     Page 261

-------------------------------------------------------------------------------

as arrived to be saved in your directory.

If  you  are  running  Kermit-20 locally, you should already have issued a SEND

5

command  to the remote Kermit, and then escaped back  to  DEC-20  Kermit.    As

files  arrive, their names will be displayed on your screen, along with "." and

"%" characters to indicate the packet traffic; you can  type  Control-A  during

the transfer for a brief status report.

If  a  file arrives that you don't really want, you can attempt to cancel it by

typing Control-X; this sends a cancellation request to the remote Kermit.    If

the  remote  Kermit understands this request (not all implementations of Kermit

support this feature), it will comply; otherwise it will continue to send.   If

a  file  group  is being sent, you can request the entire group be cancelled by

typing Control-Z.

THE GET COMMAND

Syntax: GET [remote-filespec]

The GET command requests a remote Kermit server to send the file or file  group

specified  by  remote-filespec.   This command can be used only when there is a

Kermit server on the other end of the line.  This means that you must have CON-

NECTed to the other system, logged in, run Kermit there, issued the SERVER com-

mand, and escaped back to the DEC-20, or else you Kermit-20 is in remote  mode,

TAKEing commands from a file, and interacting with a local Kermit server.

The  remote  filespec  is any string that can be a legal file specification for

the remote system; it is not parsed or validated locally.  You should not put a

trailing  comment  on  the  GET command, since this will be sent as part of the

remote filespec.

If you need to include otherwise illegal characters such as  "!"  or  ";"  (the

normal  command comment delimeters), "?" (the command help character), "@" (the

indirect command file indicator),  or  certain  control  characters,  then  you

should  precede  each  such  character  by a Control-V.  Kermit-20 will discard

these Control-V quoting prefixes before sending the file specification  to  the

remote host.

If  you  want  to  store  the incoming file name with a different name than the

remote host sends it with, just type GET alone on a line; Kermit-20 will prompt

you  separately for the source (remote) and destination (local) file specifica-

tion.  If more than one file arrives, only the first one will be  stored  under

the  name  given;  the  rest will be stored under the names they are sent with.

Example:

Kermit-20>get

Remote Source File: profile exec a1

Local Destination File: profile.exec

_______________

5

not SERVER -- use the GET command to receive files from a Kermit server.

11.5.1. Commands for File Transfer                                     Page 262

-------------------------------------------------------------------------------

As files arrive, their names will be displayed on your screen, along  with  "."

and  "%" characters to indicate the packet traffic.  As in the RECEIVE command,

you may type Control-A to get a brief status report, ^X  to  request  that  the

current  incoming  file  be  cancelled,  ^Z to request that the entire incoming

batch be cancelled.

If the remote Kermit is not capable of server functions, then you will probably

get  an  error  message back from it like "Illegal packet type".  In this case,

you must connect to the other Kermit, give a SEND  command,  escape  back,  and

give a RECEIVE command.

THE STATISTICS COMMAND

Give statistics about the most recent file transfer.  For instance, here's what

Kermit-20 displayed after transmitting a short  binary  file,  using  repeated-

character compression:

Maximum number of characters in packet:  80 received; 80 sent

Number of characters transmitted in 2 seconds

Sent:      34           Overhead:       34

Received:  107          Overhead:       -408

Total received:  141          Overhead:       -374

Total characters transmitted per second:       70

Effective data rate:   2570 baud

Efficiency:            214.1667 per cent

Interpacket pause in effect: 0 sec

Timeouts: 0

NAKs:     0

Note  that  the  data compression allowed the effective baud rate to exceed the

actual speed of the communication line, which in this case happened to be  1200

baud.  The efficiency is displayed only if the actual baud rate is known.

11.5.2. Server Operation

THE SERVER COMMAND

The  SERVER  command  puts  a  remote  Kermit-20  in  "server mode", so that it

receives all further commands in packets from the local Kermit.  The  Kermit-20

server is capable (as of this writing) of executing the following remote server

commands:  SEND, GET, FINISH, BYE, REMOTE DIRECTORY, REMOTE CWD, REMOTE  SPACE,

REMOTE DELETE, REMOTE TYPE, REMOTE HELP.

Any  nonstandard parameters should be selected with SET commands before putting

Kermit-20 into server mode, in particular the file bytesize.  The DEC-20 Kermit

server  can send most files in the correct manner automatically, by recognizing

the DEC-20 file bytesize.  However, if you need to ask the DEC-20 Kermit server

to  receive  binary  files  from an 8-bit-byte system (that is, from almost any

system that's not a DEC-10 or DEC-20) you must issue the SET  FILE  BYTESIZE  8

command  before  putting it into server mode, and then you must only send 8-bit

binary files.  You cannot send a mixture of text files and 8-bit  binary  files

to a Kermit-20 server.

11.5.2. Server Operation                                               Page 263

-------------------------------------------------------------------------------

COMMANDS FOR SERVERS

When  running  in local mode, Kermit-20 allows you to give a wide range of com-

mands to a remote Kermit server, with no guarantee the that the  remote  server

can  process  them, since they are all optional features of the protocol.  Com-

mands for servers include the standard SEND, GET, BYE, and FINISH commands,  as

well as the REMOTE command.

These  commands  are generally issued when Kermit-20 is in local mode, i.e. you

have already connected to another system, run Kermit  there  and  put  it  into

server mode, and escaped back to Kermit-20.  However, Kermit-20 also allows you

to operate in the opposite direction, i.e. Kermit-20 is the remote Kermit,  and

the local Kermit is in server mode.  This is handy when, for instance, you want

to transfer a disparate collection of files that can't be readily specified  by

a wildcard group, all in a single, unattended operation.  In this case, you can

create a TAKE command file for Kermit-20 that SENDs  and/or  GETs  the  desired

files, and then shuts down local server when done, e.g.:

set delay 0             ; No need to pause before sending

; Connect to own directory, leave a blank line for password.

cwd me:

log transactions        ; Keep a log

; Change directories on the PC.

remote cwd \kermit

send ker:mskerm.doc     ; Send the MS-DOS Kermit manual

send ker:mskerm.bwr     ; Send the MS-DOS Kermit "beware file"

; Now change to the MS-DOS binaries area

remote cwd \bin

send kb:msvibm.exe      ; Send the executable DOS Kermit program

; Put DOS back in default directory

remote cwd \chris

; Connect back to default directory on the DEC-20

cwd me:

close transactions      ; Close transaction log

send transaction.log    ; Send it

finish                  ; Shut down DOS Kermit server

Commands  to  servers  (GET,  BYE,  FINISH, REMOTE) can be issued from a remote

Kermit-20 only by means of a TAKE file.  When Kermit-20 is  local  (i.e.  after

SET LINE), you can issue these commands interactively as well.

The REMOTE Command

Send the specified command to the remote server.  If the server does not under-

stand the command (all of these commands are optional features  of  the  Kermit

protocol),  it  will reply with a message like "Unknown Kermit server command".

If does understand, it will send the results back, and they will  be  displayed

on the screen.  The REMOTE commands are:

CWD [directory] Change  Working  Directory.   If no directory name is provided,

11.5.2. Server Operation                                               Page 264

-------------------------------------------------------------------------------

the server will  change  to  the  default  or  home  directory.

Otherwise,  you will be prompted for a password, and the server

will attempt  to  change  to  the  specified  directory.    The

password  is  entered  on a separate line, and does not echo as

you type it.  If access is not granted, the server will provide

a message to that effect.  Do not put trailing comments after a

REMOTE CWD command, or after the password.

DELETE filespec Delete the specified file or files.  The  names  of  the  files

that are deleted will appear on your screen.

DIRECTORY [filespec]

The names of the files that match the given file  specification

will  be  displayed on your screen, perhaps along with size and

date information for each file.  If no  file  specification  is

given, all files from the current directory will be listed.

HELP            Provide  a  list  of  the functions that are available from the

server.

HOST [command]  Pass the given command to the server's host command  processor,

and display the resulting output on your screen.

SPACE           Provide  information about disk usage in the current directory,

such as the quota, the current storage, the amount of remaining

free space.

TYPE filespec   Display the contents of the specified file on your screen.

11.5.3. Commands for Local File Management

Syntax: LOCAL [command]

Execute  the  specified  command  on  the  local  system -- on the DEC-20 where

Kermit-20 is running.   These  commands  provide  some  local  file  management

capability without having to leave the Kermit-20 program.

CWD [directory] Change working directory, or, in DEC-20 terminology, CONNECT to

the specified directory.  If a password is required,  you  will

be  prompted  for one.  Do not include a trailing comment after

the password.

DELETE filespec Delete the specified file or files, but  do  not  expunge  them

(unless you have SET EXPUNGE ON).

DIRECTORY [filespec]

Provide a directory listing of the specified files.

RUN [filespec]  Attempts to run the specified file, which  must  be  in  ".EXE"

format  (.EXE  is  the  default filetype), in an inferior fork.

Control returns to Kermit-20 when the program terminates.  Once

you have used this command, you can restart the same program by

issuing  a  RUN  command  with  no  arguments.    If  you   RUN

SYSTEM:EXEC,  then  you  will be able to issue TOPS-20 commands

without leaving Kermit; you can get back  to  Kermit  from  the

11.5.3. Commands for Local File Management                             Page 265

-------------------------------------------------------------------------------

EXEC by typing the EXEC POP command.

SPACE           Show how much space is used and remaining in the current direc-

tory.

TYPE            Display the contents of the specified file  or  files  at  your

terminal.  This works like the DEC-20 TYPE command, except that

if a file has a bytesize of 8, Kermit-20 will  do  8-bit  input

from  it rather than 7-bit.  Also, the DEC-20 Control-O command

discards output only from the file currently  being  displayed;

if multiple files are being typed, then output will resume with

the next file.

The LOCAL commands may also be used without the "LOCAL" prefix.

11.5.4. The CONNECT Command

Syntax: CONNECT [number]

Establish a terminal connection to the system connected to the octal TTY number

specified  here or in the most recent SET LINE command, using full duplex echo-

ing and no parity unless otherwise specified in previous  SET  commands.    Get

back  to Kermit-20 by typing the escape character followed by the letter C. The

escape character is Control-Backslash (^\) by default.  When you type  the  es-

cape character, several single-character commands are possible:

C   Close the connection and return to Kermit-20.

S   Show status of the connection; equivalent to SHOW LINE.

P   Push to a new Exec.  POP from the Exec to get back to the connection.

Q   If a session log is active, temporarily Quit logging.

R   Resume logging to the session log.

B   Send a simulated BREAK signal.

?   List all the possible single-character arguments.

^\ (or whatever you have set the escape character to be):

Typing the escape character twice sends one copy of it to  the  connected

host.

You  can use the SET ESCAPE command to define a different escape character, and

SET PARITY, SET DUPLEX, SET HANDSHAKE, SET FLOW, and SET SPEED to change  those

communication-line-oriented  parameters.  In order for the simulated BREAK sig-

nal to work, TOPS-20 must know the speed of the terminal.  If it does not,  you

may  use  the  SET  SPEED  command.  Type the SHOW LINE command for information

about your current communication settings.

Kermit-20 does not have any special autodialer interface.  It assumes that  the

connection has already been made and the line assigned.

11.5.5. The SET, SHOW, and DEFINE Commands                             Page 266

-------------------------------------------------------------------------------

11.5.5. The SET, SHOW, and DEFINE Commands

SET  is  used  for  establishing  or changing parameters, DEFINE lets you group

several SET commands together into a single "macro" command, and SHOW lets  you

examine current settings or macro definitions.

THE SET COMMAND

Syntax: SET parameter [option [value]]

Establish  or  modify  various parameters for file transfer or terminal connec-

tion.  You can examine their values with  the  SHOW  command.    The  following

parameters may be SET:

BREAK  Adjust the BREAK simulation parameter

BLOCK-CHECK  Packet transmission error detection method

DEBUGGING  Record or display state transitions or packets

DELAY  How long to wait before starting to send

DUPLEX  For terminal connection, FULL or HALF

ESCAPE  Character for terminal connection

FILE  For setting file parameters like byte size

FLOW-CONTROL  For enabling or disabling XON/XOFF flow control

HANDSHAKE  For turning around half duplex communication line

IBM  For communicating with an IBM mainframe

INCOMPLETE  What to do with an incomplete file

INPUT  For specifying behavior of the INPUT command

ITS-BINARY  For recognizing a special 8-bit binary file format

LINE  TTY line to use for file transfer or CONNECT

PARITY  Character parity to use

PROMPT  Change the program's command prompt

RECEIVE  Various parameters for receiving files

RETRY  How many times to retry a packet before giving up

SEND  Various parameters for sending files

SPEED  Baud rate of communication line

TVT-BINARY  For negotiating binary mode on ARPANET

The  DEFINE  command may be used to compose "macros" by combining SET commands.

Those SET commands which differ from the "ideal" Kermit are  now  described  in

detail.

SET BREAK

Syntax:  SET BREAK n Specify the number of nulls to be sent at 50 baud to simu-

late a BREAK signal when connected to a remote host via SET LINE and CONNECT.

SET DEBUG

Syntax: SET DEBUG options

Record the packet traffic, either on your terminal or in a file.  Some  reasons

for  doing  this would be to debug a version of Kermit that you are working on,

to record a transaction in which an error occurred for evidence when  reporting

bugs,  or  simply  to  vary the display you get when running Kermit-20 in local

11.5.5. The SET, SHOW, and DEFINE Commands                             Page 267

-------------------------------------------------------------------------------

mode.  Options are:

STATES          Show Kermit state transitions and packet numbers (brief).

PACKETS         Display each incoming and outgoing packet (lengthy).

OFF             Don't display or record debugging information (this is the nor-

mal  mode).   If debugging was in effect, turn it off and close

any log file.

The debugging information is recorded in the file specified by the most  recent

LOG DEBUGGING command, DEBUGGING.LOG by default.

SET ESCAPE

SET ESCAPE octal-number

Specify  the  control character you want to use to "escape" from remote connec-

tions back to Kermit-20.  The default is 34 (Control-\).  The number is the oc-

tal  value  of the ASCII control character, 1 to 37 (or 177), for instance 2 is

Control-B.  After you type the escape character, you must follow it by a one of

the single-character "arguments" described under the CONNECT command, above.

SET EXPUNGE

SET EXPUNGE ON or OFF

Tell  whether  you  want a DELETE command (either the LOCAL DELETE command or a

REMOTE DELETE command sent to a  Kermit-20  server)  to  expunge  files  as  it

deletes  them.    On the DEC-20, a deleted file continues to take up space, and

may be "undeleted" at a later time in the same session.  To expunge  a  deleted

file  means to remove it completely and irrevocably, freeing its space for fur-

ther use.  EXPUNGE is OFF by default; deleted files are not  automatically  ex-

punged.    SET  EXPUNGE  applies  only  to files that are deleted explicitly by

Kermit-20, and not to files that are implicitly deleted when new generations of

existing files are created.

SET FILE

Syntax: SET FILE parameter keyword

Establish file-related parameters:

BYTESIZE keyword or number

Byte size for DEC-20 file input/output.  The  choices  are  SEVEN  (7),

EIGHT (8), and AUTO.

SEVEN   (or  7)  Always  store  or  retrieve five 7-bit bytes per word.

When sending a file, ignore the file bytesize and do 7-bit  in-

put from the file.  There would be no reason to use this option

except to explicitly force an 8-bit file to  be  treated  as  a

7-bit file.

11.5.5. The SET, SHOW, and DEFINE Commands                             Page 268

-------------------------------------------------------------------------------

EIGHT   (or  8)  Always  store  or  retrieve four 8-bit bytes per word.

When sending a file, ignore the file bytesize and do 8-bit  in-

put  from  the  file.  This command is necessary when receiving

binary files from 8-bit-byte systems, such  as  most  microcom-

puters.

AUTO    Equivalent  to SEVEN for incoming files, and for outgoing files

means to use EIGHT if the DEC-20 file bytesize (as shown by the

Exec  VDIR  command) is 8, otherwise use SEVEN.  The default is

AUTO.

The DEC-20 can send any mixture  of  file  types  in  the  correct  way

automatically, but you must set the file bytesize to 8 for any incoming

8-bit binary files, and to AUTO (i.e. 7) for any incoming text files or

DEC-20 binary files.

NAMING UNTRANSLATED or NORMAL-FORM

If NORMAL-FORM the names of incoming or outgoing  files  will  be  con-

verted  to  contain  only  uppercase  letters,  digits, and at most one

period; any other characters will be translated to "X".    If  UNTRANS-

LATED,  filenames will be sent and used literally.  UNTRANSLATED is the

default.

SET IBM

Syntax: SET IBM ON or OFF

SET IBM is really a predefined SET macro rather than a "hardwired" SET command;

it  can  be  redefined or undefined (see DEFINE); as distributed from Columbia,

Kermit-20 defines IBM to be "parity mark, handshake XON, duplex half".

SET IBM should be used when running Kermit-20 in local mode,  connected  to  an

IBM  or  similar mainframe.  If you have redefined the SET IBM macro, then your

parameters will be used instead.

SET ITS-BINARY

Syntax: SET ITS-BINARY ON or OFF

Specify whether ITS-Binary file headers are to be recognized or  ignored.    By

default,  they  are recognized.  ITS binary format is a way (devised at MIT) of

storing foreign 8-bit binary data on a 36-bit machine to allow automatic recog-

nition  of  these  files when sending them out again, so that you don't have to

depend on the file byte size, or to issue explicit SET FILE  BYTESIZE  commands

to Kermit.

An  ITS  format binary file contains the sixbit characters "DSK8" left-adjusted

in the first 36-bit word.  If ITS-BINARY is ON, then Kermit-20  will  send  any

file  starting  with this "header word" using 8-bit input from the file even if

the file bytesize is  not  8,  and  will  not  send  the  header  word  itself.

Kermit-20  will  also store any incoming file that begins with that header word

using 8-bit bytesize, again discarding the header word itself.   If  ITS-BINARY

is OFF, then the header word, if any, will be sent or kept, and i/o will be ac-

cording to the setting of FILE BYTESIZE.

11.5.5. The SET, SHOW, and DEFINE Commands                             Page 269

-------------------------------------------------------------------------------

This facility is provided for compatibility with the file formats used on  cer-

tain public-access CP/M libraries.

SET INPUT

Syntax: SET INPUT parameter value

The  INPUT  command is used in TAKE command files or DEC-20 Batch control files

as part of the login script facility, which  is  explained  in  greater  detail

later.    SET INPUT controls the behavior of the INPUT command.  The parameters

are as follows:

SET INPUT DEFAULT-TIMEOUT n

n  is  the number of seconds for an INPUT command to time out after not

receiving the requested input, when no interval is explicitly given  in

the INPUT command.  For instance, if the default timeout interval is 10

seconds, then the command

INPUT login:

will look for the "login:" prompt for 10 seconds.  The default  may  be

overriden by including an explicit interval in the INPUT command:

INPUT 15 login:

The default timeout interval is 5 seconds.

SET INPUT TIMEOUT-ACTION PROCEED or QUIT

If the INPUT command comes from a  Kermit-20  command  file  (see  TAKE

command)  or  a  TOPS-20  Batch  control file, then use this command to

specify whether processing of the command file should proceed  or  quit

after  a  timeout  occurs.  For TAKE files, the current command file is

terminated and control returns to the invoking level (Kermit-20  prompt

level, or a superior TAKE file).  The default action is PROCEED.

SET INPUT CASE IGNORE or OBSERVE

Specify whether alphabetic case should be ignored ("a" matches "A")  or

observed  ("a"  does  not  match  "A")  when scanning the input for the

specified search string.  By default, aphabetic case is ignored.

SET INPUT commands are "global"; the settings are  not  "pushed"  and  "popped"

when entering or leaving TAKE command files.

SET LINE

Syntax: SET LINE [octal-number]

Specify the octal TTY number to use for file transfer or CONNECT.  If you issue

this command, you will be running Kermit-20 locally, and you must log in to the

remote  system and run Kermit on that side in order to transfer a file.  If you

don't issue this command, Kermit-20 assumes it is running  remotely,  and  does

file  transfer  over  its job's controlling terminal line.  You can also select

the line directly in the CONNECT command; the command

11.5.5. The SET, SHOW, and DEFINE Commands                             Page 270

-------------------------------------------------------------------------------

CONNECT 12

is equivalent to

SET LINE 12

CONNECT

If you type SET LINE with no number argument, you will  deassign  any  previous

assigned line and revert to remote mode.

The  SHOW  LINE  command will display the currently selected communication line

and its charactistics, including parity, duplex, handshake, flow  control,  the

speed  if known, whether carrier is present (if it is a modem-controlled line),

and whether Kermit-20 is in local or remote mode.

SET RECEIVE

In addition to the full complement of SET RECEIVE  commands  described  in  the

main  part of the Kermit User Guide, you may also SET RECEIVE SERVER-TIMEOUT to

a value between 0 and 94.    This  specifies  the  number  of  seconds  between

timeouts  during server command wait, 0 specifies that no timeouts should occur

during server command wait.  When a Kermit server times out,  it  sends  a  NAK

packet.    Some systems cannot clear piled-up NAKs from their input buffers; if

you're using such a system to communicate with a Kermit-20 server, and you  ex-

pect  to  be  leaving  the server idle for long periods of time, you should use

this command to turn off server command-wait timeouts.

SET SPEED

Syntax: SET SPEED n

Set the baud rate of the currently selected communication  to  n,  the  decimal

baud  rate, for instance 300, 1200, 4800.  When operating in local mode, it may

be necessary to issue this command in order to enable BREAK simulation.

SET TVT-BINARY

Syntax: SET TVT-BINARY ON or OFF

Only for users running Kermit-20 on an ARPANET DEC-20, signed on to an  ARPANET

virtual terminal (TVT) from another host or through an ARPANET TAC.  SET TVT ON

causes Kermit-20 to negotiate binary mode (8-bit) communication  with  the  AR-

PANET  during file transfer.  Without this command, file transfer through a TVT

would not work in most cases.

TVT-BINARY is OFF by default.  If you normally use Kermit-20  through  the  AR-

PAnet, you can put the command SET TVT-BINARY ON into your KERMIT.INI file.

CAUTION:  This  facility requires certain features in the Release 5 TOPS-20 AR-

PANET monitor, which may not be present in releases distributed by  DEC.    See

the Kermit-20 source code for details.

11.5.5. The SET, SHOW, and DEFINE Commands                             Page 271

-------------------------------------------------------------------------------

THE DEFINE COMMAND

Syntax: DEFINE macroname [set-option [, set-option [...]]]

The  DEFINE command is available in Kermit-20 for building "macros" of SET com-

mands.  The macro name can be any keyword-style character string, and  the  set

options are anything you would type after SET in a SET command; several set op-

tions may be strung together, separated by commas.  Example:

define notimeout send timeout 0, receive timeout 0, receive server 0

Macro definitions may not include macro names.  You can list  all  your  macros

and  their definitions with the SHOW MACROS command.  You can list a particular

macro definition with HELP SET macroname.

THE SHOW COMMAND

Syntax: SHOW [option]

The SHOW command displays various information:

DAYTIME         Current date, time, phase of moon.

DEBUGGING       Debugging mode in effect, if any.

FILE-INFO       Byte size for DEC-20 file i/o, incomplete file disposition.

INPUT           INPUT command parameters.

LINE            TTY line,  parity,  duplex,  flow  control,  handshake,  escape

character,  speed  (if  known), and session loggin information.

Note that before release 6.0 of TOPS-20, the  DEC-20  does  not

keep  a record of the actual baud rate of a modem-controlled or

"remote" TTY line.

MACROS          Definitions for SET macros.

PACKET-INFO     For incoming and outbound packets.  Items under RECEIVE  column

show parameters for packets Kermit-20 expects to receive, under

SEND shows parameters for outgoing packets.

TIMING-INFO     Delays, retries, server NAK intervals.

VERSION         Program version of Kermit-20.   This  is  also  displayed  when

Kermit-20 is initially started.

ALL             (default) All of the above.

11.5.6. Program Management Commands                                    Page 272

-------------------------------------------------------------------------------

11.5.6. Program Management Commands

THE TAKE COMMAND

Syntax: TAKE filespec

Execute  Kermit-20 commands from the specified file.  The file may contain con-

tain any valid Kermit-20 commands, including other TAKE commands; command files

may  be  nested up to a depth of 20.  Default file type for the command file is

.CMD.  Most commands may have trailing comments, beginning  by  semicolon,  but

these  should  be  avoided  in REMOTE commands, GET commands, and the passwords

that are prompted for after CWD and REMOTE CWD commands.

THE ECHO COMMAND

Syntax: ECHO line of text

The line of text is echoed at the terminal.  This is useful  when  issued  from

within TAKE command files, to report progress or issue instructions.

THE HELP COMMAND

Syntax: HELP [topic [subtopic]]

Typing  HELP  alone  prints a brief summary of Kermit-20 and its commands.  You

can also type

HELP command

for any Kermit-20 command, e.g. "help send" or "help set parity"  to  get  more

detailed information about a specific command.  Type

HELP ?

to see a list of the available help commands.

THE EXIT AND QUIT COMMANDS

Syntax: EXIT

Exit  from  Kermit-20.    You  can  CONTINUE the program from the TOPS-20 Exec,

provided you haven't run another program on top of it.  You can also exit  from

Kermit-20  by  typing  one  or  more control-C's, even if it's in the middle of

transferring a file.  Kermit-20  will  always  restore  your  terminal  to  its

original condition, and you will be able to CONTINUE the program to get back to

"Kermit-20>" command level with current settings intact.

QUIT is a synonym for EXIT.

11.5.6. Program Management Commands                                    Page 273

-------------------------------------------------------------------------------

THE LOG COMMAND

Syntax: LOG [option [filespec]]

Log the specified option to the specified file:

SESSION         During CONNECT or execution of a login script, log all  charac-

ters  that  appear on the screen to the specified file.  During

CONNECT, the session log can be temporarily turned  off  during

the remote session by typing the escape character followed by Q

(for Quit logging), and turned on again by  typing  the  escape

character  followed  by R (for Resume logging).  Default log is

SESSION.LOG in the current directory.

TRANSACTIONS    During file transfer, log the  progress  of  each  file.    The

DEC-20 transaction log file looks like this:

Kermit-20 Transaction Log File, Monday 27-Feb-1984

18:40:13: Opened Log: PS:<TIMREK>SAMPLE.LOG.1

18:40:31: -- Send Begins --

8th bit prefixing: Off

Block check type: 1

18:40:31: Opened File: PS:<SY.FDC>LOGIN.CMD.6

Sending As "LOGIN.CMD"

Sent: 547 7-bit bytes

18:40:34: Closed PS:<SY.FDC>LOGIN.CMD.6

18:40:34: Send Complete

18:40:50: -- Receive Begins --

8th bit prefixing: Off

Block check type: 1

18:40:50: Opened: PS:<TIMREK>AUTOEXEC.BAT.1

Written: 186 7-bit bytes

18:40:51: Closed: PS:<TIMREK>AUTOEXEC.BAT.1

18:40:56: Closed Transaction Log

Transaction  logging is recommended for long or unattended file

transfers, so that you don't have to watch the screen.  The log

may  be  inspected  after  the transfer is complete to see what

files were transferred  and  what  errors  may  have  occurred.

Default log is TRANSACTION.LOG in the current directory.

DEBUGGING       Log  STATES  or  PACKETS,  as  specified in the most recent SET

DEBUGGING command, to the specified file.    If  log  file  not

specified,  then use TTY if local, or DEBUGGING.LOG in the cur-

rent directory if remote.  If  no  SET  DEBUGGING  command  was

previously  issued, log STATES to the specified file.  Also al-

low specification of bytesize for  the  log  file,  7  (normal,

default),  or 8 (for debugging binary transfers when the parity

bit is being used for data), for instance

LOG DEBUGGING BINARY.LOG 8

A 7-bit log file can be typed, printed, or examined with a text

editor or searching program.  An 8-bit log file can only be ex-

amined with a system utility like FILDDT.  When  logging  pack-

11.5.6. Program Management Commands                                    Page 274

-------------------------------------------------------------------------------

ets,  each  packet  is  preceded  by  a  timestamp, the current

timeout interval (preceded by a slash), and "R:" or "S:" to in-

dicate data being received and sent, respectively.  Packet for-

mat is described in the Kermit Protocol Manual.

SESSION is the default option.  Thus the command "LOG" alone will cause CONNECT

sessions  to  be logged in SESSION.LOG in the current directory.  Any log files

are closed when you EXIT or QUIT from Kermit, and are reactivated if  you  CON-

TINUE  the  program.  You may explicitly close a log file and terminate logging

with the CLOSE command.

THE CLOSE COMMAND

Syntax: CLOSE option

Close the specified log file, SESSION, TRANSACTION, or DEBUGGING, and terminate

logging to that file.

11.6. Login Scripts: The INPUT, OUTPUT, CLEAR, and PAUSE Commands

When  running  Kermit-20  in  local mode, connecting from the DEC-20 to another

system via an external TTY line (for instance, through an autodialer), you  may

use the Kermit-20 INPUT, OUTPUT, CLEAR, and PAUSE commands to carry on a dialog

with the remote system.  When combined into a "script" in a Kermit-20 TAKE com-

mand  file,  or  included  in  a Batch control file, these commands provide the

ability to initially connect and log in to a remote system, and to  set  it  up

for  file  transfer.    During script execution, session logging may be used to

record the dialog.

THE CLEAR COMMAND

Syntax: CLEAR

Clear the input and output buffers of the currently selected line, and  attempt

to clear any XOFF deadlock.

THE PAUSE COMMAND

Syntax: PAUSE [interval]

Pause  the  specified number of seconds before executing the next command.  The

default interval is one second.

THE INPUT COMMAND

Syntax: INPUT [interval] [string]

On the currently selected communication line, look for the given string for the

specified  interval  of time, which is specified in seconds.  If no interval is

specified, then wait for the default interval, which may be  specified  by  SET

INPUT  DEFAULT-TIMEOUT, and is normally 5 seconds.  Specifying an interval of 0

11.6. Login Scripts: The INPUT, OUTPUT, CLEAR, and PAUSE Commands      Page 275

-------------------------------------------------------------------------------

(or less) means no timeout -- wait forever for the specified string.  An  INPUT

command can by interrupted by typing one or more Control-C's, which will return

you to Kermit-20> prompt level.

Characters coming in from the line will be scanned for the search  string,  and

when  a  match is found, the command will terminate successfully; if the string

is not found within the given interval, the command will  terminate  unsuccess-

fully.   While the INPUT command is active, all incoming characters will appear

on your screen.

The search string may contain any printable characters.  Control or other  spe-

cial  characters  that  you could not normally type as part of a command may be

included by preceding their octal ASCII values with a backslash,  for  instance

foo\15  is  "foo" followed by a carriage return (ASCII 15, octal).  A backslash

alone will be taken as is, unless it is followed by an octal  digit  (0-7);  if

you  want to actually specify a backslash in this context, double the backslash

(\\5 will be taken as \5).

The behavior of the INPUT command is governed by the SET INPUT CASE, SET  INPUT

DEFAULT-TIMEOUT,  and  SET  INPUT  TIMEOUT-ACTION commands, as described in the

Kermit Commands section of the User Guide, or in the Kermit book.

In addition to normal use, Kermit-20 scripts can also be used in  DEC-20  batch

control  files.   Failure to match an input string in the timeout interval will

result in a message starting with "?", which signals the  Batch  controller  to

detect  an error.  If INPUT TIMEOUT-ACTION is SET to PROCEED, any timeout error

messages will be issued starting with a "%", which does not signal an error  to

Batch.

In addition to otherwise untypable control characters (like Control-C), certain

printable characters in the search string may need to  be  "quoted"  using  the

backslash mechanism:

@ (ASCII 100)   If  it  is  the  first  character  in  the string, atsign tells

TOPS-20 that the following characters will be the  name  of  an

indirect command file, for instance

input 10 @foo.txt

tells  Kermit  to  spend  10 seconds scanning the communication

line input for the  string  which  is  contained  in  the  file

FOO.TXT.  If you need to specify a string that starts with "@",

use \100 instead.

? (ASCII 77)    A question mark tells TOPS-20 to provide a brief  help  message

about this part of the command; use \77 instead.

! (ASCII 41)    If  it  is  the  first  character in the string, an exclamation

point will cause TOPS-20 to ignore the rest of the string, i.e.

treat it as a comment, use \41.

; (ASCII 73)    Same as exclamation mark, use \73.

( (ASCII 50)    In  first position, TOPS-20 will think this marks the beginning

of a "guide word"; use \50.

11.6. Login Scripts: The INPUT, OUTPUT, CLEAR, and PAUSE Commands      Page 276

-------------------------------------------------------------------------------

- (ASCII 55)    In final position, a dash tells TOPS-20 that the  command  line

is  to be continued, concatenated with the following line.  Use

\55 instead of a final dash.   For  instance,  to  specify  the

string "More?--", use "More\77-\55".

THE OUTPUT COMMAND

Syntax: OUTPUT string

The  given  string  is sent out the currently selected communication line.  The

string is in the same form as the INPUT string; control or  special  characters

may  be  included  by prefacing their octal ASCII value with a backslash.  Note

that any terminating carriage return must be included explicitly as \15.    The

string will also be echoed at your terminal.

LOGIN SCRIPT HINTS

It  is  not  a  good  idea  to  store  passwords  in plain text in a file.  The

facilities of the TOPS-20 command parser make this unnecessary, so long as  you

are  sitting  at  your  terminal.  Suppose you have a script that looks for the

string "Password: " and then outputs your password using a command like

out mypassword\15

If you change this line to

out @tty:

you may enter the password from your terminal as follows:

login: myuserid

Password: mypassword\15^Z

That is, you type the password, a backslash-encoded carriage return,  and  then

Control-Z.    This  may  be done even when executing commands from a TAKE file;

after the ^Z, control returns to the TAKE file.  In the OUTPUT command,  "@TTY:

designates  TTY:  (your  job's  controlling terminal) to be an indirect command

file; the ^Z is the "end of file" for a terminal.  This  same  technique  could

have been used in the first script example to allow you to supply from the ter-

minal the name of the file to be sent.  It might be a good idea to for  you  to

include  an  ECHO command in your script file to remind you to do this, for in-

stance:

input password:

echo ^GType your password, followed by "\15" and then a CTRL-Z

output @tty:

The ^G is a Control-G, which should get your attention by sounding  a  beep  at

your terminal.

One  might  expect  to be able to use the same indirect file mechanism with the

OUTPUT command to provide a crude "raw upload" facility, as in

output @foo.bar

11.6. Login Scripts: The INPUT, OUTPUT, CLEAR, and PAUSE Commands      Page 277

-------------------------------------------------------------------------------

to send the contents of  the  file  FOO.BAR  to  the  remote  system,  with  no

synchronization  or error checking.  Unfortunately, there are two problems with

this approach: first, TOPS-20 converts all carriage return /  linefeeds  in  an

indirect  command  file  to  spaces,  and  second, only very short files may be

treated this way, because they must fit within TOPS-20's command "atom" buffer.

The Kermit-20 TRANSMIT command provides a synchronized raw uploading of files.

11.7. Raw Download and Upload

"Raw  Download"  is  the term commonly used to describe the capture of a remote

file on the local system, without any kind of error  detection  or  correction.

This  allows  you  to obtain files from remote systems that do not have Kermit,

but with the risk of loss or corruption of data.

Kermit-20 provides raw downloading via the LOG SESSION command  during  CONNECT

to  a  remote system.  The session log is described above.  To use session log-

ging to capture a file:

1. Run Kermit on the DEC-20.

2. SET LINE to the TTY number through which you will  be  connected  to

the remote system.

3. Perform any required SET commands to condition Kermit for communica-

tion with the remote system.  You may need SET PARITY,  SET  DUPLEX,

SET  FLOW,  SET HANDSHAKE, etc., depending on the characteristics of

the remote system and the communication medium.

4. CONNECT to the remote system and log in.

5. Condition your job on the remote system not to pause at the end of a

screenful  of  text,  and give whatever commands may be necessary to

achieve a "clean" terminal listing -- for instance, disable messages

from the system or other users.

6. Type  the  appropriate command to have the desired file displayed at

the terminal, but do not type the terminating carriage return.    On

most systems, the command would be "type", on Unix it's "cat".

7. Escape  back  to  Kermit to the DEC-20 and give the LOG SESSION com-

mand.

8. CONNECT back to the remote system and type a carriage return.    The

file  will  be  displayed on your screen and recorded in the session

log file.

9. Escape back to Kermit on the DEC-20 and give the CLOSE SESSION  com-

mand.

The  file  will  be in SESSION.LOG in your connected directory, unless you gave

another name for it in your LOG SESSION command.  You will probably  find  that

some  editing necessary to remove extraneous prompts, messages, padding charac-

ters, or terminal escape sequences, or to fill in lost or  garbled  characters.

Here's an example showing how to capture a file foo.bar from a remote Unix sys-

tem:

11.7. Raw Download and Upload                                          Page 278

-------------------------------------------------------------------------------

@kermit

Kermit-20>set line 23

Kermit-20>connect

[KERMIT-20: Connecting to remote host over TTY23:,

type <CTRL-\>C to return.]

4.2 BSD UNIX

login: myuserid

Password: mypassword

% cat foo.bar^\C

[KERMIT-20: Connection Closed]

Kermit-20>log session foo.bar

Kermit-20>connect

[KERMIT-20: Connecting to remote host over TTY23:,

type <CTRL-\>C to return.]

[KERMIT-20: Logging to File FOO.BAR.1]

(Type carriage return now.)

This is the file foo.bar.

It has three lines.

This is the last line.

% ^\

[KERMIT-20: Closing Log File FOO.BAR.1>

[KERMIT-20: Connection Closed]

Kermit-20>close session

Note that in this case, the Unix "% " prompt at the end of the text  will  have

to be edited out.

RAW UPLOAD

"Raw  Upload" means sending a file from the local system to a remote one, again

without error detection or correction.  This allows you to send files from  the

DEC-20 to remote systems that don't have Kermit.  Kermit-20 provides the TRANS-

MIT command for this purpose.

Syntax: TRANSMIT filespec [prompt]

For use in local mode only.  Sends the specified text file a line  at  a  time,

"raw" (as is, without using Kermit protocol), to the remote system, waiting for

the specified prompt for each line.  Only a single file may be  sent  with  the

TRANSMIT  command;  wildcards are not allowed in the filespec.  The file should

be a text file, not a binary file.  Since protocol is not being  used,  no  as-

surance  can be given that the file will arrive at the destination correctly or

completely.

The prompt is any string, for instance the prompt of a line editor in text  in-

sertion  mode.    The prompt string may include special characters by preceding

their octal ASCII values with a backslash, e.g. \12 for linefeed, \21  for  XON

(^Q).    The  syntax of the prompt string is explained in greater detail above,

with the INPUT command.

If a prompt string is supplied, alphabetic case will be  ignored  in  searching

for  it unless you SET INPUT CASE OBSERVE.  If a prompt string is not supplied,

then linefeed will be used by default, unless you have performed  a  SET  HAND-

SHAKE  command, in which case the current handshake character will be used.  If

11.7. Raw Download and Upload                                          Page 279

-------------------------------------------------------------------------------

you really want to send the  entire  file  without  waiting  for  any  prompts,

specify a prompt of "\0" (ASCII zero, null) (this is not advised).

The  file  will be sent using the current settings for duplex, parity, and flow

control.  There are no timeouts on input, as there are with the INPUT  command.

The  TRANSMIT  command waits forever for the prompt to appear.  However, if you

observe that the transfer is stuck, there are three things you can do:

- Type a Carriage Return to transmit the next line.

- Type a Control-P to retransmit the line that was just transmitted.

- Type two Control-C's to cancel the TRANSMIT command and get  back  to

Kermit-20> command level.

TRANSMIT  should  be  used as follows: CONNECT to the remote system, login, and

start up some kind of process on the remote system to store input from the ter-

minal into a file.  On a DEC-20 (that doesn't have Kermit), you could do

copy tty: foo.bar

or  you could start a line editor like EDIT or OTTO and put it into text inser-

tion mode.  On a Unix system, you could

cat /dev/tty > foo.bar

or you could run "ed" and give it the "a" command.

The Kermit-20 TRANSMIT command will  send  the  first  line  of  the  file  im-

mediately.    Then  it  will  wait for a "prompt" from the remote system before

sending the next line.  When performing a copy operation from the terminal to a

file,  the  "prompt"  will  probably  be a linefeed, "\12" which is the default

prompt -- most full duplex systems expect you to type a line of text terminated

by  a  carriage  return;  they  echo  the characters you type and then output a

linefeed.  Half duplex systems, on the other hand, use some kind of  line  tur-

naround  handshake  character,  like XON (Control-Q), to let you know when they

are ready for the next line of input.  Line editors like EDIT and  OTTO  prompt

you  with  a  line number followed by a tab; in that case your prompt character

would be "\11" (be careful -- if the remote DEC-20 doesn't think your  terminal

has  hardware  tabs, it will simulate them by outputting spaces).  In any case,

to assure synchronization, it is your responsibility to set up the target  sys-

tem  to  accept line-at-a-time textual input and to determine what the system's

prompt will be when it is ready for the next line.

Each line is sent with a terminating carriage return; linefeeds are  not  sent,

since  these are supplied by the receiving system if it needs them.  The TRANS-

MIT command continues to send all the lines of the file in this manner until it

reaches the end, or until you interrupt the operation by typing Control-C's.

If  you  cannot  make the TRANSMIT command work automatically, for instance be-

cause the remote system's prompt  changes  for  each  line,  you  may  TRANSMIT

manually  by specifying a prompt string that will not appear, and then typing a

carriage return at your keyboard for each line you want to send.

If the TRANSMIT command completes successfully (you'll get a message to the ef-

fect  that  the  transmission  is  complete), then you must connect back to the

11.7. Raw Download and Upload                                          Page 280

-------------------------------------------------------------------------------

remote system and type whatever command it needs in orderto save  and/or  close

the file there.

11.8. Kermit-20 Examples

Here  are  a few examples of the use of Kermit-20.  Text entered by the user is

underlined.

REMOTE OPERATION

The following example shows use of Kermit-20 as a server from an IBM  PC.    In

this  example,  the  user  runs  Kermit  on the PC, connects to the DEC-20, and

starts Kermit-20 in server mode.  From that point on, the user need never  con-

nect  to  the  DEC-20  again.    In this example, the user gets a file from the

DEC-20, works on it locally at the PC, and then sends the results back  to  the

DEC-20.   Note that the user can leave and restart Kermit on the PC as often as

desired.

A>Kermit

Kermit-MS>connect

@

@Kermit

TOPS-20 Kermit version 4.2(262)

Kermit-20>server

Kermit Server running on DEC-20 host.  Please type your escape

sequence to return to your local machine.  Shut down the server by

typing the Kermit BYE command on your local machine.

^[C

Kermit-MS>get foo.txt

The transfer takes place.

Kermit-MS>exit

A>

A>edit foo.txt ; (or whatever...)

A>

A>Kermit

Kermit-MS>send foo.txt

The transfer takes place.

Kermit-MS>bye

A>

The next example shows the special procedure you would have to use in order  to

send  a mixture of text and binary files from a PC (or an 8-bit-byte system) to

the DEC-20.  Note that in this case, it's more convenient to avoid server mode.

Kermit-MS>connect

@

@Kermit

TOPS-20 Kermit version 4.2(262)

Kermit-20>receive

11.8. Kermit-20 Examples                                               Page 281

-------------------------------------------------------------------------------

^]C

Kermit-MS>send *.txt

Textual files are sent.

Kermit-MS>connect

Kermit-20>set file bytesize 8

Kermit-20>receive

^]C

Kermit-MS>send *.exe

Binary files are sent.

Kermit-MS>connect

Kermit-20>exit

@logout

^]C

Kermit-86>exit

A>

LOCAL OPERATION

In this example, a program DIAL is used to direct an autodialer to call another

computer (a DECsystem-10); once the connection is made, DIAL starts Kermit with

an implicit CONNECT command for the appropriate communication line.    DIAL  is

not  part of Kermit; if your system has an autodialer, there will be some site-

specific procedure for using it.

@dial

Dial>dial stevens

STEVENS, 1-(201) 555-1234, baud:1200

[confirm]

Dialing your number, please hold...

Your party is waiting on TTY11:.

@

@Kermit

TOPS-20 Kermit version 4.2(262)

Kermit-20>connect 11

[Kermit-20: Connecting over TTY11:, type <CTRL-\>C to return]

CONNECTING TO HOST SYSTEM.

Stevens T/S 7.01A(10) 20:20:04 TTY41 system 1282

Connected to Node DN87S1(101) Line # 57

Please LOGIN or ATTACH

.log 10,35

JOB 51 Stevens T/S 7.01A(10) TTY41

Password:

20:20   15-Dec-83       Thur

.r new:Kermit

TOPS-10 Kermit version 2(106)

Kermit-10>server

11.8. Kermit-20 Examples                                               Page 282

-------------------------------------------------------------------------------

[Kermit Server running on the DEC host.  Please type your escape

sequence to return to your local machine.  Shut down the server by

typing the Kermit BYE command on your local machine.]

^YC

[Kermit-20: Connection Closed.  Back at DEC-20.]

Kermit-20>set file bytesize 8

Kermit-20>get setdtr.cmd

^A for status report, ^X to cancel file, ^Z to cancel batch.

SETDTR.CMD.7 ^A

Receiving SETDTR.CMD.7, file bytesize 8

(repeated character compression)

At page 1

Files: 0, packets: 1, chars: 66

NAKs: 0, timeouts: 0

.[OK]

Kermit-20>bye

Job 51  User F DA CRUZ [10,35]

Logged-off TTY41  at 20:22:58  on 15-Dec-83

Runtime: 0:00:01, KCS:33, Connect time: 0:02:39

Disk Reads:72, Writes:4, Blocks saved:160

....

Hangup? y

Click. Call duration was 193 seconds to area 201.

Dial>exit

Note the use of Control-A to get a status report during the transfer.

11.9. Installation of Kermit-20

Kermit-20 is built from a single MACRO-20 source file, K20MIT.MAC.  It requires

the standard DEC-distributed tools MONSYM, MACSYM, and CMD; the following files

should be in SYS: -- MONSYM.UNV, MACSYM.UNV, MACREL.REL, CMD.UNV, and  CMD.REL.

The  CMD  package is also included with the Kermit distribution as K20CMD.*, in

case you can't find it on your system.

The program should work on all TOPS-20 systems as distributed,  but  many  cus-

tomizations  are possible.  The site manager may wish to change various default

parameters on a site-wide basis; this  may  be  done  simply  by  changing  the

definitions  of  the  desired symbols, under "subttl Definitions", and reassem-

bling.

The most notable site dependency is the definition  of  "SET  IBM".    As  dis-

tributed  from  Columbia,  Kermit-20  defines "SET IBM" in a built-in SET macro

definition as "parity mark, duplex half, handshake xon".  This  definition  may

be  found  at  MACTAB+1,  near  the  end of the impure data section.  It may be

changed or deleted, and the program reassembled.

Sites that do not have ARPANET may wish to delete the TVT-BINARY  entries  from

SET command tables, SETABL and SETHLP.

12. PDP-11 Kermit                                                      Page 283

-------------------------------------------------------------------------------

12. PDP-11 Kermit

Author:             Brian Nelson, University of Toledo, Ohio

Documentation:      Brian Nelson

Language:           Macro-11

Version:            3.58

Date:               September, 1987

Systems Supported:  RSTS/E, RSX-11M/M+, P/OS, Micro-RSX, RT-11 and TSX+

Kermit-11 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfer text files:               Yes

Transfer binary files:             Yes

Wildcard send:                     Yes

File transfer interruption:        Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

LONG Packet protocol support:      Yes

Sliding Windows protocol support:  No

Terminal emulation:                Yes

Communication settings:            Yes

Transmit BREAK:                    Yes (depends on system)

IBM mainframe communication:       Yes

Transaction logging:               Yes

Session logging:                   Yes

Debug logging:                     Yes

Packet logging:                    Yes

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         Yes

Local file management:             Yes

Command/Init files:                Yes

File attributes packets:           Yes

Command macros:                    No

Raw file transmit:                 Yes

12. PDP-11 Kermit                                                      Page 284

-------------------------------------------------------------------------------

12.1. File Systems on the PDP-11

12.1.1. File Specifications

The general format of a file name is:

NODE::DEVICE:[DIRECTORY]NAME.TYPE;VERSION

'Node'  refers  to  the  DECNET node name, for example, FUBAR::, if applicable.

'Device', if present, refers to the physical device or logical name  where  the

file resides.

For  RSTS/E, 'device' can be a physical device, such as DB0: or DU1:, or it can

be a user or system logical name which may include both a physical device  name

and  a directory name.  If the device name is a logical name, is it composed of

1 to 9 alphanumeric characters, including '$', as in DISK$ONE:, LB: and so  on.

For instance, the DCL system command

$ ASS/SYS DB1:[200,210] SRC$DIR

would  associate  both  the  device DB1: and directory [200,210] with SRC$DIR:.

Explicitly given directories override directory names  imbedded  in  a  logical

name.  Names longer than nine characters are truncated by the executive.

In  the  case of RSX-11M/M+ and RT-11, the device name can be either a physical

name, such as DU0:, or a logical name which will translate to a physical device

name, such as LB:.

On  RSTS/E  and RSX-11M/M+, the [directory] is a UIC (user identification code)

or PPN (project,programmer) number of the format [NNN,MMM].  All users are  as-

signed a UIC (or PPN) when accounts are created, this is the number you give to

LOGIN to log into the system.  It is also your default UIC (or PPN).  Micro-Rsx

and  P/OS  may have directories in either UIC format or named directory format,

such as [1,2] or [KERMIT].  For P/OS, the  default  directory  is  [USERFILES].

Directories are not used in RT-11.

The  NAME field is the primary identifier for the file.  The name can be one to

nine characters for RSX-11M/M+ and P/OS, and one to six characters for  RSTS/E,

RT-11  and  TSX+.    The TYPE field is usually used to group files according to

some convention.  For example, XXX.FTN refers  to  a  Fortran-77  source  file,

FOO.C to a 'C' source file, and K11POS.TSK refers to a task image.

The  version field is applicable ONLY to RSX type systems.  The default version

is always the highest version number.

All  systems  mentioned  support  some  sort  of  filename   wildcarding,   the

flexibility  of  which  varies  by  executive.    All support the use of '*' to

represent either a fully wildcarded NAME or TYPE.  RSTS/E supports the  use  of

'?'  to  match  any single character, whereas the others use a '%' to match any

single character.  The RSTS/E Kermit server will translate '%'  to  '?'  inter-

nally  for the GET and REMOTE DIR commands (see the section on Kermit-11 server

operation).

Examples of wildcarded filenames:

12.1.1. File Specifications                                            Page 285

-------------------------------------------------------------------------------

*.B2S           Match any file with a TYPE of B2S.

K11%%%.MAC      match any file starting with K11,  followed  by  one  to  three

characters, with a TYPE of MAC.

K11???.MAC      Same as above, but for RSTS/E only.

XYZ.*;*         All  versions  of  files  with  a  NAME  of  XYZ  with any TYPE

(RSX-11M/M+ and P/OS only).

12.1.2. File Formats (Binary and Text)

12.1.2.1. RT-11 and TSX+

RT-11 treats all files as a contiguous stream of characters.  There is  no  in-

formation  stored  in the directory to tell the system (or program) that a file

is readable text (source program, runoff document,...)  or consists  of  binary

data  (executable program, object file, .SYS file,...).  An application program

like Kermit-11 needs to know what type of file to expect, thus the presence  of

the  SET FILE TYPE command (discussed later).  The only real convention is that

text files are streams of seven bit data with each record terminated by a  car-

riage return/line feed character sequence and that binary files normally follow

a filename TYPE convention.  The TYPE (.SAV, .SYS, ...) is what Kermit-11  will

look at to decide if a file should be sent as a text or binary file.

12.1.2.2. RSTS/E, P/OS and RSX-11M/M+

These  systems  can provide for a large number of file attributes for each file

by using either FCS11 (RSX-11M/M+) or RMS11 (all).   Text  files  are  normally

considered  to  be  either STREAM format (FB$STM) or VARIABLE with implied car-

riage control (FB$VAR and FB$CR).  RSTS/E has historically defaulted to STREAM,

whereas  the  RSX based systems use VARIABLE.  Kermit-11 follows those defaults

when creating files unless told to do so otherwise by the presence of attribute

data.    The  conversion of the internal data representation to one that can be

transmitted to another Kermit is transparent for these types of  files.    Both

the  file  attributes and the filename TYPE are examined by Kermit-11 to deter-

mine if a file needs to be sent as a text file (default) or a binary file.  Ad-

ditionally,  on  RSTS/E  Kermit  checks the file protection code, as one of the

bits in it is used to flag an executable file (bit 6).

In all cases, unless (at this time) Kermit-11 is talking to another  Kermit-11,

or  if  Kermit-11  can't tell if a file is consists of binary data, the command

SET FILE TYPE FIXED must be used to force  Kermit  to  either  send  or  get  a

non-text file correctly.  When Kermit-11 is running in binary mode, all data is

read from (or written to) the file without any translation or  internal  record

control  information.   Any attribute information in the file's directory entry

is ignored and the data read (or written) in 512 byte unformatted blocks.  Thus

it  is indeed possible to transfer files like task images and object libraries.

Since Kermit-11 supports a subset of a protocol  feature  called  'attributes',

two Kermit-11's connected together can also correctly transfer files other than

simple text and unformatted binary files,  such  as  RMS  indexed  or  relative

files.

12.1.3. Saving Files on the PDP-11 From Your Microcomputer             Page 286

-------------------------------------------------------------------------------

12.1.3. Saving Files on the PDP-11 From Your Microcomputer

You  can  send textual files to Kermit-11 without any special considerations as

Kermit-11 defaults to creating normal text files.  However, if you are  sending

a  binary file (perhaps an .EXE) from say, your Rainbow under MS-DOS, you would

need to tell Kermit-11 to expect binary data.  This is done with the  Kermit-11

command  SET  FILE TYPE FIXED.  This will force Kermit-11 to write the data out

exactly as it comes, in 512 byte unformatted records.  Sending  the  same  file

back  to the Rainbow would not require any special action since the file, as it

sits on the PDP-11, has the proper information in the directory entry  to  tell

Kermit-11  that the file is binary.  As a note, for RT-11 you would need to use

a filetype that is normally considered 'binary' like .SAV or  .OBJ  (see  above

notes for RT-11).

Never  try  to  do  a wildcarded send with mixed binary and text files with the

file type set to FIXED.  The result could be unusable as not all systems  store

text  data  in  the  same internal format.  For example, if Kermit-11 is forced

into binary mode (via SET FIL TYP FIX) and is requested to send a file with im-

plied  carriage control (normal for RSX text files), it will actually send, for

each line, two bytes representing the record length, followed by the  data  and

then  followed by a ASCII NUL to pad the record to an even length.  That is not

incorrect, rather, it is EXACTLY how the data was stored on disk.

In general, avoid sending anything other than unformatted binary files and text

file  to  unlike  systems.  For example, requesting a RMS indexed file from the

PDP-11 to be sent to a PC would case Kermit-11 to send it as a binary file, but

the  file  attributes  would  be  lost.  Sending such a file back to the PDP-11

would result in an unusable file unless you could reconstruct the attribute in-

formation.

12.1.4. Program Operation

Kermit-11's  prompt  is  normally "Kermit-11>".  This can be changed if need be

via the SET PROMPT command.  Invoking Kermit-11 is very site dependent.

12.1.4.1. RSTS/E

If Kermit-11 has a ccl definition, it would  likely  be  invoked  as  "KER"  or

"KERMIT".  If not, try "RUN $KERMIT", as this is a likely place where Kermit-11

may have been put.  Otherwise consult your local support staff.

12.1.4.2. RSX-11M/M+

If Kermit-11 has been installed, it most likely will have a task name of ...KER

which  means that typing "KER" should get things running.  If not, consult your

local support staff.

12.1.4.3. RT-11/TSX+                                                   Page 287

-------------------------------------------------------------------------------

12.1.4.3. RT-11/TSX+

On version 5 of RT-11, programs can be  run  simply  by  typing  the  filename.

Thus,  if  there is a file SY:KERMIT.SAV, simply type "KERMIT".  If this fails,

contact your local support staff for assistance.

12.1.4.4. P/OS

Kermit-11 is generally run from DCL on P/OS.  The program is  invoked  via  the

DCL RUN command, as in RUN K11POS or RUN KERMIT, depending on what the task im-

age name is.

Note that for the case where Kermit is installed (for  RSTS/E  and  RSX-11M/M+)

that Kermit-11 can get command line arguments, as in:

$ KER SERV               Kermit starts as a server.

> KER send fubar.txt     Kermit sends the file.

Otherwise, the program is run interactively from the Kermit-11> prompt:

$ KERMIT

Kermit-11 V3.54

Kermit-11>SET BLO 3      Changes checksum type.

Kermit-11>SER            Enter Kermit server.

Note  that  whenever  Kermit-11  starts  up,  it will always try to find a file

called KERMIT.INI in your current directory.  This file can contain  any  valid

Kermit  command, though the usual use of this is to place various Kermit-11 SET

commands in it.  If this file does NOT  exist,  it  will  try  to  find  it  in

LB:[1,2]KERMIT.INI  (excluding  RT-11).  In addition to the .INI file, commands

may be placed in a file and then executed via the Kermit-11 TAKE  (or  @)  com-

mand.

12.2. Local and Remote Operation

Kermit-11  by  default assumes that all file transfers will occur over the ter-

minal line that you are currently logged in on (TI:, TT:, KB:).  This is  known

as  REMOTE  mode  (the PDP-11 is the remote system).  This would be the desired

case if you are running Kermit on a microcomputer such as  a  Rainbow  and  are

currently  logged into the PDP-11 through the micro.  However, if you wanted to

dial out, say by an autodial modem, from the PDP-11 to another system, you need

to  tell Kermit-11 to use some other terminal line.  This would be called LOCAL

mode (the PDP-11 is the local system).  The line can be altered  with  the  SET

LINE  command (see section on SET and CONNECT).  A SET LINE command is done im-

plicitly if Kermit-11 finds itself running on a  PRO/350,  under  either  P/OS,

RT-11 or TSX+.

Since  support  of  parity  varies by both interface type (DL11 vs DZ11) and by

operating system, Kermit-11 makes NO attempt  to  find  out  what  the  current

parity  of it's line is.  Kermit-11 generates it's own parity which is set with

the SET PARITY command.

There are a couple of things to point out regarding Kermit-11  and  LOCAL  mode

(you did a SET LINE command):

12.2. Local and Remote Operation                                       Page 288

-------------------------------------------------------------------------------

- The  system  manager may have lines other than your own protected (or

owned by the system).  On RSTS/E lines are  often  made  unaccessible

unless   your   account   possesses  the  needed  privilege(s).    On

RSX-11M/M+, privilege is required to alter settings on any other ter-

minal  line.   You may have to talk to your system manager to get ac-

cess to an outgoing terminal line.

- Once connected to a modem through another line, a  means  must  exist

for the connection to be broken (if the host you are calling won't do

it).  Given that your line has full or partial modem control  (DZV11,

DZ11,  DH11,  DHU/V11)  the RSX, RT-11/TSX+ and RSTS/E Kermits have a

HANGUP (or DISCONNECT) command, which instructs the system to discon-

nect  the modem.  Unless this is done, you never get disconnected and

could run up a tidy phone bill.

Kermit-11 has, as of v3.53, a rudimentary command line editor.  You can  recall

previous commands with the UP-Arrow key, and exit the command with the LEFT and

RIGHT arrow keys.  The RUBOUT key, of course,  deletes  characters,  while  the

Control-R  key  retypes  the  line.  Control-E moves to the end of the line and

Control-H moves to the start of the line.

12.3. Kermit-11 Commands

Kermit-11 has the following commands available:

@             Synonym for TAKE

BYE           Logout a remote server

CONNECT       Connect to a remote system

COPY          Local copy of a file(s)

CWD           Set new working directory

DELETE        Local delete of a file(s)

DIAL          Have a connected modem dial a number

DIRECT        Local directory display

DISCONNECT    Hangup a remote line

DISPLAY       Internal debugging

ERASE         Local delete of a file(s)

EXIT          Exit to system

FINISH        Stop a remote server without logging out

GET           Get a file(s) from a remote server

HANGUP        Hangup a remote line

HOST          Execute system command locally (where applicable)

LOCAL         Force interpretation of command to the local system

LOGFILE       Create a log file

QUIT          Same as EXIT

PRINT         Print a file locally (where applicable)

RECEIVE       Receive a file(s) from a remote kermit

REMOTE        Prefix for file management commands to a server

RENAME        Local rename of filename(s)

SEND          Send a file(s) to a remote Kermit

SERVER        start a Kermit server

SET           Change Kermit parameters

SHOW          Display Kermit parameters

TAKE          Execute indirect command file

TYPE          Local display of file on terminal

WHO           Local display of logged in users (RSTS/E only)

12.4. Commands for File Transfer                                       Page 289

-------------------------------------------------------------------------------

12.4. Commands for File Transfer

Kermit-11 includes the standard repertoire of Kermit  file  transfer  commands,

including SEND, RECEIVE, and GET.

THE SEND COMMAND

Syntax:  SEND filespec

The  SEND command causes a file or file group to be sent from the PDP-11 to the

other system.  If filespec contains wildcard characters then all matching files

will be sent, in alphabetical order (according to the ASCII collating sequence)

by name.  If filespec does not contain any wildcard characters, then the single

file specified by filespec will be sent.

SEND Command General Operation

:

Files will be sent with their PDP-11 file name and type (for instance FOO.BAR).

Each file will be sent according to the record type and attributes recorded  in

its  file descriptor.  Kermit-11 attempts to translate all formats of text file

to a format usable on any system.  Note that there is no need to set  the  FILE

TYPE  parameter  for sending files, since Kermit-11 always uses the information

from the file directory entry and the filetype (extension) to determine how  to

send the file.

If communication line parity is being used (see SET PARITY), Kermit-11 will re-

quest that the other Kermit use a special kind of prefix  notation  for  binary

files.   This is an advanced feature, and not all Kermits have it; if the other

Kermit does not agree to use this feature, binary files  cannot  be  sent  cor-

rectly.   This includes executable programs (like .EXE files, CP/M .COM files),

relocatable object modules (.OBJ files), as well as any  text  file  containing

characters with the eighth bit on.

Kermit-11 will also ask the other Kermit whether it can handle a special prefix

encoding for repeated characters.  If it can, then files with long  strings  of

repeated  characters  will  be  transmitted  very  efficiently.  Columnar data,

highly indented text, and binary files are  the  major  beneficiaries  of  this

technique.

If  you're running Kermit-11 locally, for instance dialing out from a PDP-11 to

another system using an autodialer, you should have already run Kermit  on  the

remote  system  and issued either a RECEIVE or a SERVER command.  Once you give

Kermit-11 the SEND command, the name of each file will  be  displayed  on  your

screen as the transfer begins.  As the transfer continues, you will get a small

display of the packet count along with the number of packets rejected.  See the

SET  TERMINAL  and SET UPDATE commands for more information.  You may also type

Control-X or Control-Z to interrupt the current file or file group.   Control-E

will also abort the transfer by sending an 'error' packet to the other Kermit.

12.4. Commands for File Transfer                                       Page 290

-------------------------------------------------------------------------------

THE RECEIVE COMMAND

Syntax:  RECEIVE [filespec]

The  RECEIVE  command  tells Kermit-11 to receive a file or file group from the

other system.  The name is taken from the incoming file header.  If an incoming

file  has the same name as an existing file, Kermit-11 will by default create a

new file.  On RT-11 and RSTS/E, the old file will be deleted by the  executive.

On  RSX-11M/M+  and  P/OS,  a  new  file  with  a higher version number will be

created.  To avoid files being superceded, see the SET FILE [NO]SUPERCEDE  com-

mand.

Incoming  files  will  all  be  stored  with the prevailing file type, ASCII by

default, which is appropriate for text files.  If you are asking  Kermit-11  to

receive binary files from a microcomputer or other 8-bit system, you must first

type SET FILE TYPE FIXED.  Otherwise, an error may  occur  when  receiving  the

file.  Please note that this does NOT apply to two Kermit-11 programs connected

to each other.  In that case the sending  Kermit-11  will  tell  the  receiving

Kermit-11 to switch to binary mode if need be.

If parity is being used on the communications line, then 8th-bit prefixing will

be requested.  If the other side cannot do this, binary files cannot be  trans-

ferred correctly.

If  you  are  running  Kermit-11 locally, you should already have issued a SEND

command to the remote Kermit, and then escaped back to Kermit-11.  As files ar-

rive, their names will be displayed on your screen.

If  a  file arrives that you don't really want, you can attempt to cancel it by

typing Control-X; this sends a cancellation request to the remote Kermit.    If

the  remote  Kermit understands this request (not all implementations of Kermit

support this feature), it will comply; otherwise it will continue to send.   If

a  file  group  is being sent, you can request the entire group be cancelled by

typing Control-Z.

Normally, one runs the remote Kermit as a SERVER, thus the RECEIVE  command  is

never used, rather, the GET command, described next, is used.

THE GET COMMAND

Syntax:  GET [remote-filespec]

The  GET command requests a remote Kermit server to send the file or file group

specified by remote-filespec.  This command can be used only when Kermit-11  is

local, with a Kermit server on the other end of the line specified by SET LINE.

This means that you must have CONNECTed to the other  system,  logged  in,  run

Kermit there, issued the SERVER command, and escaped back to the PDP-11.

The  remote  filespec  is any string that can be a legal file specification for

the remote system; it is not parsed or validated locally.  Any  leading  spaces

before  the  remote filespec are stripped, and lower case characters are raised

to upper case.

As files arrive, their names will be displayed on  your  screen.    As  in  the

RECEIVE  command,  Control-X  (^X) to request that the current incoming file be

12.4. Commands for File Transfer                                       Page 291

-------------------------------------------------------------------------------

ancelled, ^Z to request that the entire incoming batch be cancelled.

If the remote Kermit is not capable of server functions, then you will probably

get  an  error  message back from it like "Illegal packet type".  In this case,

you must connect to the other Kermit, give a SEND  command,  escape  back,  and

give a RECEIVE command.

12.4.1. Server Operation

The  SERVER  command  puts  a  remote  Kermit-11  in  "server mode", so that it

receives all further commands in packets from the local Kermit.  The  Kermit-11

server is capable (as of this writing) of executing the following remote server

commands: SEND, GET, FINISH, BYE, REMOTE DIRECTORY, REMOTE CWD,  REMOTE  SPACE,

REMOTE  DELETE,  REMOTE  TYPE,  REMOTE HELP, REMOTE COPY, REMOTE RENAME, REMOTE

WHO, REMOTE LOGIN and REMOTE HOST.

Any nonstandard parameters should be selected with SET commands before  putting

Kermit-11  into server mode, in particular the file type.  The Kermit-11 server

can send all files in the correct manner automatically.  As noted before, if  a

Kermit-11  is  talking  to  another Kermit-11, they will negotiate any 'binary'

parameters automatically.  However, if this is NOT the case and you need to ask

Kermit-11  to receive binary files you must issue the SET FILE TYPE FIX command

before putting it into server mode, and then you must only send  binary  files.

You  cannot  send a mixture of text files and 8-bit binary files to a Kermit-11

server unless the files are not for use on the PDP-11.

12.4.2. Commands for Servers

When running in local mode, Kermit-11 allows you to give a wide range  of  com-

mands  to  a remote Kermit server, with no guarantee the that the remote server

can process them, since they are all optional features of the protocol.    Com-

mands for servers include the standard SEND, GET, BYE, FINISH commands, as well

as the REMOTE command.

THE BYE COMMAND

The BYE command tells a remote server to log out of the remote system.  In  ad-

dition,  some remote systems will also disconnect the line for you.  If this is

not the case, the DISCONNECT command will (depending on your  interface)  cause

the line to be dropped.  See DISCONNECT.

THE FINISH COMMAND

The  FINISH  command tells the remote Kermit server to exit without logging out

of the remote system.  You can then CONNECT back to the Server  operation  sys-

tem.

12.4.2. Commands for Servers                                           Page 292

-------------------------------------------------------------------------------

THE REMOTE COMMAND

Send the specified command to the remote server.  If the server does not under-

stand the command (all of these commands are optional features  of  the  Kermit

protocol),  it  will reply with a message like "Unknown Kermit server command".

If does understand, it will send the results back, and they will  be  displayed

on the screen.  The REMOTE commands are:

REMOTE COPY filespec newfilespec

Copy file.  The server is asked to make a copy of the specified

file.    Both  filespecs  must be in the correct format for the

remote system.  Kermit-11 does not parse or validate  the  file

specifications.   Any leading spaces will be stripped and lower

case characters converted to upper case.  Note that  this  com-

mand  simply  provides  for  copying a file within the server's

system - it does not cause a file to be transferred.

REMOTE CWD directory

Change  Working  Directory.   If no directory name is provided,

the server will  change  to  the  default  or  home  directory.

Kermit-11 currently does not ask for a password.

REMOTE DELETE filespec

Delete the specified file or files.  The  names  of  the  files

that are deleted will appear on your screen.

REMOTE DIRECTORY [filespec]

The names of the files that match the given file  specification

will  be  displayed on your screen, perhaps along with size and

date information for each file.  If no  file  specification  is

given, all files from the current directory will be listed.

REMOTE HELP     The remote server will send back a list of server commands that

it can execute.

REMOTE HOST command

Pass  the given command to the server's host command processor,

and display the resulting output on your screen.  Not all  Ker-

mit  servers  can  do this function.  In the case of Kermit-11,

only the RSTS/E Kermit-11 server can execute  the  REMOTE  HOST

command.

REMOTE LOGIN user password

Ask a remote server to log into a different  account  or  user-

name.    The  support for this command is rarely implemented as

many systems layer login/logout support over the executive.   A

Kermit-11  server  can only support this on RSTS/E, and at that

only for version 9.0 or later.    Of  the  various  DEC  PDP-11

operating  systems,  only RSTS/E has the support for logging in

and out built into the executive  and  accessible  with  direc-

tives.

REMOTE RENAME oldfile newfile

Change the name on the specified file (or files).    Both  file

specifications must be valid for the server's system.

12.4.2. Commands for Servers                                           Page 293

-------------------------------------------------------------------------------

REMOTE SPACE    Display information about disk usage in the current directory.

REMOTE TYPE filespec

Display the contents of the specified file on your screen.

REMOTE WHO      Display current status of user's logged in.

12.5. Commands for Local File Management

These commands provide some local file management capability without having  to

leave  the  Kermit-11  program.   These commands are very similar to the REMOTE

commands in function and syntax.  They are all executed locally, and are avail-

able when Kermit-11 is either local or remote.  The arguments to these commands

are the same as the arguments expected from the user Kermit when  Kermit-11  is

processing  a  command  in  server  mode.   Additionally, these commands can be

prefixed by the LOCAL keyword.

COPY filespec newfilespec

CWD directory

DELETE filespec

DIRECTORY [filespec]

HELP

HOST command

RENAME oldfile newfile

SPACE

TYPE filespec

WHO

12.5.1. The CONNECT Command

The CONNECT command will allow you to connect in as a terminal  over  the  line

that  was specified by the SET LINE command.  (Using the CONNECT command before

using the SET LINE command will result in an error message.)  The terminal line

must be one which is accessible to the user.

The  distributed RSX-11M/M+ task has been built with the /PR:0 switch to enable

the task to change other terminal settings.  Additionally, for RSX-11M/M+,  the

MCR command SET /SLAVE=TTnn: should be done before entering Kermit-11.

If  you  are  running K11POS.TSK on a PRO/350, Kermit will set the line to XK0:

and the speed to 9600 by default.

Please note that Kermit-11 CAN NOT change the speed of a DL11  type  interface,

nor can it change the speed of a PDT-150 modem port (use SPEED.SAV).

The  following  is an example of using a Racal-Vadic VA212 autodialing modem to

log into a remote TOPS-20 system.  There is one point  at  which  there  is  no

echoing  of  the  user input, this is following the typing of the local 'escape

sequence', which by default is Control-\ followed by a 'c'.  The  control-back-

slash  informs  the terminal emulator that the next character is a command.  In

this case, the command was 'C', which means to return to the local PDP-11  sys-

tem.    Control-\  followed  by ? would print a help message.  All the commands

prior to the DIAL command were contained in the INI file, KERMIT.INI.

12.5.1. The CONNECT Command                                            Page 294

-------------------------------------------------------------------------------

$ kermit

Kermit-11 V3.46 Last edit: 21-Feb-1986

Kermit-11>set modem vadic

Kermit-11>set pho num cu 9K12121234567

Kermit-11>set logfile 20.log

Kermit-11>set deb console

Kermit-11>set lin tt58:

Link: TT58: Speed: 9600, DTR not present

Kermit-11>set dtr

Kermit-11>set spe 1200

Kermit-11>dial cu

Using: 9K12121234567

Connection established, type CONNECT to access remote

Kermit-11>connect

enter class 4

class 004 start

CU20B

@log xx.abcdef

CU20B, TOPS-20 Monitor 5.1(5101)-2

Job 28, TTY32, 2-Apr-84 4:15:24PM

Previous login was 2-Apr-84 4:10:16PM

.

.

@logout

[Confirm]

Logged out Job 28, User XX.ABCDEF , TTY 32,

at  2-Apr-84 16:19:34,  Used 0:00:11 in 0:04:10

Kermit-11>disc

KERMIT link TT58: disconnected

Kermit-11>exit

$ logout

12.6. The SET Command

Syntax: SET parameter keyword

The SET command is used to set various parameters in Kermit.  The format of the

SET command is:

SET ATTRIBUTES

Syntax: SET ATTRIBUTES {ON, OFF}

Part  of the Kermit protocol is the support of file attributes.  Connected Ker-

mits that support this can send information to  each  other  about  file  size,

time/date of creation, RMS file headers and other useful things.  Due to poten-

tial problems with incompatible implementations this feature can  be  disabled.

In  this  case,  the  sending Kermit-11 will never try to send file attributes,

even though the receiver may have indicated that it supports this.

12.6. The SET Command                                                  Page 295

-------------------------------------------------------------------------------

SET BAUD

This is the same as SET SPEED.  See HELP SET SPEED

SET BINARY-TYPE

Kermit-11 has a default list of filetypes that are scanned to decide if a  file

should  be sent in binary mode in addition to checking file attributes for RSX,

P/OS and RSTS/E.  The user can, however, override this list with the this  com-

mand.    The default list is fairly inclusive, with types such as .SAV and .TSK

forcing Kermit-11 into binary transmission.  See HELP SET FIL for  the  default

list.  Examples:

Kermit-11> set binary-type .sav

Kermit-11> set bin .exe

SET BLOCK-CHECK

Syntax: SET BLOCK_CHECK {1, 2, 3}

The  SET BLOCKCHECK command is used to determine the block check sequence which

will be used during transmission.  The block check sequence is used  to  detect

transmission  errors.    There are three types of block check available.  These

are the single character checksum (default), the two  character  checksum,  and

the  three  character CRC (cyclic redundancy check).  This command does not en-

sure that the desired type of block check will be used, since both Kermit's in-

volved  in the transfer must agree on the block check type.  Kermit-11 will re-

quest that the type of block check set by this command be used for a  transfer.

If  the other Kermit has also had the same block check type requested, then the

desired block check type will be used.  Otherwise, the single character  check-

sum will be used.  The command should be given to BOTH Kermits since Kermit-11,

when in server mode, has no say about what kind of checksum it  wants  to  use.

(See Kermit protocol manual for more information.)

SET CONSOLE

Syntax: SET CONSOLE {7, 8}

The SET CONSOLE command is used under P/OS to control the passing of 8 bit data

to the terminal during the connect command.  If you are  getting  multinational

characters  being  printed, this is a very useful thing to set.  The default is

SET CON 7.

SET DEBUG

Syntax: SET DEBUG {ALL, CONSOLE, CONNECT, FILE, PACKET, STATE}

The SET DEBUG command is used to specify the type and level of debugging  to  a

disk file .  This disk file must have been created by the SET LOGFILE command.

12.6. The SET Command                                                  Page 296

-------------------------------------------------------------------------------

SET DEBUG ALL

SET DEBUG ALL will turn on logging for CONSOLE,CONNECT,FILE,PACKET and STATE to

the disk file specified by SET LOGFILE.  This command is the same as SET  DEBUG

ON.  The command format is:

SET DEBUG CONSOLE

SET  DEBUG  CONSOLE will turn on logging for all i/o during a remote connect to

the disk file specified by SET LOGFILE.  This command is the same as SET  DEBUG

CONNECT.

SET DEBUG CONNECT

SET  DEBUG  CONNECT will turn on logging for all i/o during a remote connect to

the disk file specified by SET LOGFILE.  This command is the same as SET  DEBUG

CONSOLE.

SET DEBUG FILE

SET DEBUG FILE will log all file 'opens' and 'creates' to the file specified by

SET LOGFILE.

SET DEBUG HELP

SET DEBUG HELP gives the user a list of all qualifiers which can be  used  with

SET DEBUG.

SET DEBUG NONE

SET  DEBUG  NONE  'turns off' all debugging.  This is the same as the SET DEBUG

OFF command.

SET DEBUG OFF

SET DEBUG OFF 'turns off' all debugging.  This is the same  as  the  SET  DEBUG

NONE command.

SET DEBUG ON

SET  DEBUG  ON will 'turn on' logging for CONSOLE,CONNECT,FILE,PACKET and STATE

to the disk file specified by SET LOGFILE.  This command is  the  same  as  SET

DEBUG ALL.

12.6. The SET Command                                                  Page 297

-------------------------------------------------------------------------------

SET DEBUG PACKET

SET  DEBUG PACKET will 'turn on' logging of all receive and transmit packets to

the disk file specified by SET LOGFILE.

SET DEBUG STATE

SET DEBUG STATE will turn on logging of  all  internal  Kermit-11  state  tran-

sitions.

SET DELAY

Syntax: SET DELAY seconds

The  DELAY parameter is the number of seconds to wait before sending data after

a SEND command is given.  This is used when Kermit-11 is running in remote mode

to  allow  the  user time to escape back to the other Kermit and give a RECEIVE

command.

SET DEFAULT

Syntax: SET DEFAULT device

The DEFAULT parameter allows you to specify a device and UIC (or PPN)  for  all

subsequent  file  opens  (for SENDING) and file creates (for RECEIVING).  It is

disabled by typing SET HOME.  Example:

Kermit-11>set default db2:[200,201]

This is quite useful for Kermit-11 running on a DECNET link, as you can set the

default for file operations to include node names and passwords as in:

Kermit-11>set def orion::sys$system:[fubar]

SET DIAL

Kermit-11  has  knowledge built in to it of a number of the more common 'smart'

autodial modems.  To find out if your modem is directly supported try the  com-

mand SET MODEM ?.  If your modem is not in this list then you need the SET DIAL

command to generate the data base used by Kermit to control the modem.   Kermit

uses  this  information  to implement the DIAL command.  A command such as DIAL

can only be done when Kermit knows both how to format commands  to  the  modem,

and  what  kind  of text the modem will send back to it in response.  As an ex-

ample, the VADIC VA212PA modem is awakened from an idle state by the  character

sequence 05 015 (in octal), which is a Control-E followed by a carriage return.

In response to this two-character string, the modem responds with:

HELLO: I'M READY

*

Thus Kermit has to know that when it sends the wakeup sequence it needs to wait

for  the asterisk to be sent back by the modem.  At this point Kermit will know

12.6. The SET Command                                                  Page 298

-------------------------------------------------------------------------------

that the modem is in a state awaiting further commands, such as that to dial  a

phone number.

It  is  not  possible for Kermit to have knowledge of all makes of modems.  In-

stead Kermit supports a command, SET MODEM USER_DEFINED, which then allows  you

to  use the SET DIAL command to inform Kermit how the modem works.  Once Kermit

knows how to control the modem, you can use the DIAL command to initiate a call

from Kermit.

The SET DIAL commands are:

SET DIAL WAKEUP               Define the wakeup string

SET DIAL PROMPT               Define the prompt the modem uses

SET DIAL INITIATE             Define a string to start dialing

SET DIAL CONFIRM              Define the string to confirm number

SET DIAL FORMAT               Define the number formatting string

SET DIAL SUCCESS              Define string(s) for call complete

SET DIAL INFO                 Define string(s) for informative text

SET DIAL FAILURE              Define string(s) for call failure

SET DIAL CONFIRM              Define string for number confirmation

SET DIAL WAKE_RATE            Set pause time between wakeup characters

SET DIAL DIAL_RATE            Set pause time between number digits

SET DIAL DIAL_PAUSE           Define string for dial tone pause

Suppose  we  had  to tell Kermit about the Racal Vadic VA212PA modem (though in

reality Kermit already knows about that kind).  In checking the  owners  manual

for it, we find that:

- To  wake  the  modem  up,  we type a control E followed by a carriage

return.

- To dial a number, we type the letter D followed by a carriage return.

At  this  point, the modem prints a NUMBER?  prompt, we then type the

desired number in.  It reprints the number and then waits for a  car-

riage  return  from  us  to confirm that its really the correct phone

number.

- When it completes dialing, it will print 'ON LINE' or 'ONLINE' for  a

successful  call,  otherwise  it  may display on the terminal 'BUSY',

'FAILED CALL', 'NO DIAL', 'VOICE' or 'TIME OUT'.  While it is waiting

for  its call to be answered, it may print the line 'RINGING' several

times in order to tell you that it is working on it.

The Kermit commands required would be:

Kermit-11>SET MODEM USER_DEFINED

Kermit-11>SET DIAL WAKEUP \05\015

Kermit-11>SET DIAL PROMPT *

Kermit-11>SET DIAL INITIATE D\015

Kermit-11>SET DIAL FORMAT %P%S\015

Kermit-11>SET DIAL CONFIRM \015

Kermit-11>SET DIAL SUCCESS ONLINE

Kermit-11>SET DIAL SUCCESS ON LINE

Kermit-11>SET DIAL INFO RINGING

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE FAILED CALL

12.6. The SET Command                                                  Page 299

-------------------------------------------------------------------------------

Kermit-11>SET DIAL FAILURE NO DIAL

Kermit-11>SET DIAL FAILURE VOICE

Kermit-11>SET DIAL FAILURE TIME OUT

Kermit-11>SET DIAL DIAL_PAUSE 9K

Kermit-11>DIAL 14195551212

The notation "\05\015" indicates the Control E followed by a  carriage  return;

05  is  octal  for  control  E, 015 is octal for carriage return.  An alternate

notation for octal numbers can be used by  placing  the  value  inside  of  in-

equality  characters,  as  in  SET  DIAL  WAKE  <05><015>  though the former is

preferred.

The notation "%P%S\015" indicates to Kermit that the phone number from the dial

command  is to be followed by a carriage return; the %S is simply a placeholder

for the phone number.  The presence of the %P is to indicate  where  to  insert

the  dial  pause  string,  in this case we need to dial 9 and wait for a second

dial tone.  The "K" is the Racal Vadic code to get the modem to pause.  If  you

are  dialing  on a direct line, the DIAL_PAUSE command is unneeded.  If for any

reason you need to pass a "\" or "<" to your modem, simply prefix the character

with another "\", as in "\\".

Many  modems  require  only the WAKEUP, PROMPT, FORMAT and result strings.  The

Digital DF112 is an example of this; its definition would look like:

Kermit-11>SET MODEM USER_DEFINED

Kermit-11>SET DIAL WAKEUP \02

Kermit-11>SET DIAL PROMPT READY

Kermit-11>SET DIAL FORMAT %S#

Kermit-11>SET DIAL SUCCESS ATTACHED

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE DISCONNECTED

Kermit-11>SET DIAL FAILURE ERROR

Kermit-11>SET DIAL FAILURE NO ANSWER

Some modems may be unable to accept data at the line speed;  in  this  case  we

would  need  to  use  the SET DIAL WAKE_RATE and SET DIAL DIAL_RATE.  These two

commands accept a delay time in milliseconds; the  actual  delay  will  not  be

precise  as  the PDP-11 line clock interrupts sixty times per second.  Further-

more, on RSTS/E the finest granularity for timing is one second;  thus  setting

delays would result in delays of one second increments.

In  general, not all of the result fields need be specified except for the call

completed strings; Kermit will time out after a  while  if  it  can't  match  a

response with any definitions.

Further information can be found in the sections on SET MODEM, DIAL, REDIAL and

SET PHONE.

12.6. The SET Command                                                  Page 300

-------------------------------------------------------------------------------

SET DTR

The SET DTR command is very similar to the DISCONNECT (or HANGUP) command.  SET

DTR,  where  supported,  raises DTR for a predetermined amount of time, whereas

the DISCONNECT (or HANGUP) command drops DTR.  The SET DTR is  only  functional

on  RSTS/E,  which by default keeps DTR low until either RING INDICATOR or CAR-

RIER DETECT goes high.  This is opposite of the behavior on RT11 and RSX11M/M+,

both of which normally assert DTR.  The SET DTR command raises DTR for at least

30 seconds (depending on the version of RSTS/E) and is useful for  making  con-

nections  to  front end switches (such as MICOM and GANDALF).  On RT11, SET DTR

is identical to the HANGUP command; it simply drops DTR for two  seconds.    In

this  case  (RT11 and TSX+) this command is only supported on RT11 5.2 and TSX+

6.0 with the XL/XC and CL drivers, respectively.  This command is  a  no-op  on

RSX11M/M+  and  P/OS.   For further information on modem support, see the later

section regarding such.

SET DUPLEX

Syntax: SET DUPLEX {FULL, HALF}

The DUPLEX parameter controls whether an outgoing link (set via  the  SET  LINE

command)  is  a  full  duplex link (the default) or a half duplex link.  All it

does for half duplex is to cause all characters typed after the CONNECT command

to be echoed locally.

SET END-OF-LINE

Syntax: SET END-OF-LINE <octal ASCII value>

The END-OF-LINE parameter sets the ASCII character which will be used as a line

terminator for all packets SENT to the other KERMIT.    This  is  normally  not

needed for most versions of KERMIT.

SET ESCAPE

Syntax SET ESCAPE <octal ASCII value>)

This  command  will  set  the escape character for the CONNECT processing.  The

command will take the octal value of the character to use as the escape charac-

ter.    This is the character which is used to "escape" back to Kermit-11 after

using the CONNECT command.  It defaults to control (octal 34).  It is usually a

good idea to set this character to something which is not used (or at least not

used very much) on the system being to which Kermit-11 is CONNECTing.

SET FILE

Syntax: SET FILE {NOSUPERCEDE, SUPERCEDE, TYPE file-type}

The SET FILE command allows you to set various file related parameters.

12.6. The SET Command                                                  Page 301

-------------------------------------------------------------------------------

SET FILE TYPE ASCII

File type ASCII is for text files.  SET FILE TYPE TEXT is the same.

SET FILE TYPE AUTO

Kermit-11 will normally try to decide if a file must be  sent  in  binary  mode

based  on  the  file  attributes and filetype.  If, for instance, the directory

entry for FUBAR.TXT showed  it  to  be  RMS  (or  FCS)  fixed  length  records,

Kermit-11  will  switch  to binary mode and send it verbatim.  If the receiving

Kermit is Kermit-11, then the sending Kermit  will  send  attribute  data  over

also.   The file types shown in Table 12-1 also will normally be sent as binary

files unless you use the SET FILE TYPE NOAUTO command.

-------------------------------------------------------------------------------

*.TSK    RSX, IAS, and RSTS tasks

*.SAV    RT11 and RSTS save images

*.OBJ    Compiler and macro-11 output

*.STB    TKB and LINK symbol tables

*.CRF    TKB and LINK cross reference files

*.TSD    'Time shared DIBOL' for RT11

*.BAC    RSTS Basic-plus 'compiled' files

*.OLB    RSX, IAS, and RSTS object libraries

*.MLB    RSX, IAS, and RSTS macro libraries

*.RTS    RSTS/E run time systems

*.EXE    VMS executable

Table 12-1:   Kermit-11 File Types

-------------------------------------------------------------------------------

SET FILE TYPE BINARY

File type BINARY is for non-text files.   Note  that  binary  files  which  are

generated on a PDP-11 system cannot be transferred to another (non PDP-11) sys-

tem without losing file attributes.  This means that (for  example),  an  RSM11

indexed  file  cannot  be transmitted with Kermit-11 at this time.  You can not

have parity set to anything but NONE to use binary file transfer (see HELP  SET

PARITY) unless the other Kermit can process eight bit quoting.  Two Kermit-11's

connected to each other will use binary transmission automatically via the Ker-

mit  attribute  packets,  preserving file attributes where it makes sense (i.e.

RSTS/E and RSX only).

SET FILE TYPE DECMULTINATIONAL

PDP-11 Kermit normally strips the high bit of every character on both transmis-

sion and reception of files (unless the SET FILE TYPE FIXED command was given).

The SET FIL DEC command will cause Kermit-11 to leave all data intact but still

obey  the host file system when reading or writing files.  In other words, Ker-

mit will write sequential implied carriage control files with eight bit data if

12.6. The SET Command                                                  Page 302

-------------------------------------------------------------------------------

this command is used.

SET FILE TYPE FIXED

This is the same as SET FILE TYPE BINARY.

SET FILE TYPE NOAUTO

SET FILE NOAUTO disables Kermit-11 from trying to base binary transmission mode

on file attributes or filetype.

SET FILE SUPERCEDE

Syntax: SET FILE {SUPERCEDE, NOSUPERCEDE}

SET FILE [NO]SUPERCEDE allows Kermit-11 to  accept  or  reject  files  received

(from  either the RECEIVE or GET commands) on a per file basis.  The default is

SUPERCEDE.  By doing SET FILE NOSUPERCEDE Kermit-11 will always check to see if

the  file  to  be  created is already there (independent of version number) and

reject it to the sending server if it exists.  This presumes  that  the  Kermit

sending  the  file  understands the protocol to reject one file of a (possibly)

wildcarded group of files.  The main use of this is to resume getting  a  group

of  files,  as in GET KER:K11*.* or GET KER:MS????.* having lost the connection

after transferring some of the files.  If this is set, then any  files  already

transferred will not be transferred again.

SET HOME

SET  HOME  resets  the default device and UIC (or PPN) to nothing, ie, all file

opens and creates use your default disk (SY:) and your UIC (or PPN).

SET IBM-MODE

Syntax: SET IBM {ON, OFF}

The SET IBM ON (or OFF) will instruct Kermit-11 to wait for  an  XON  following

each  packet  sent  to an IBM host in linemode.  Since the default for IBM mode

may not always be appropriate for your IBM compatible system,  you  can  always

use  the  SET HANDSHAKE XON and SET DUPLEX HALF to avoid the parity setting im-

plied by using IBM mode.

SET LINE

Syntax: SET LINE device-designator

The SET LINE command sets the terminal name up for use with  the  connect  com-

mand.    To use this you must have access to that device.  On many systems ter-

minal lines other than your own are protected from access, and may require spe-

cial  procedures  to access them.  The form of the device name is TTnnn:, where

'nnn' is a decimal number for RSTS and an octal number  for  RSX-11M/M+.    For

12.6. The SET Command                                                  Page 303

-------------------------------------------------------------------------------

RT-11,  the  device name is simply the MT unit number shown by the SHO TER com-

mand, as in '5' for DZ11 unit 0 line 4.  If the system is running RT-11 version

5  you can do a SET LIN XL:.  At worst case, Kermit-11 can use the console port

on RT-11.  For more information see the notes later on for  RT-11  If  you  are

running K11POS.TSK for P/OS on the PRO/350, Kermit-11 will set the line to XK0:

and the speed to 9600 baud when Kermit starts.  To override the line or  speed,

set HELP SET LINE and HELP SET SPEED.  Examples:

Kermit-11>SET LINE TT55:        (for RSTS and RSX-11M/M+)

Kermit-11>SET LINE 5            (for RT-11 and MT service)

Kermit-11>SET LINE XK0:         (for P/OS, done implicitly)

Kermit-11>SET LINE XL:          (for RT-11 and XL handler)

See  HELP  CONNECT,  HELP  SET  DUPLEX and HELP SET SPEED for more information.

Also, for TSX+, see notes regarding TSX later in these notes.    The  RT-11  XL

handler has notes later on also.

SET LOGFILE

Syntax: SET LOGFILE filespec

The SET LOGFILE command creates a debug dump file for you.  It must be used BE-

FORE any SET DEBUG commands can be used.  See HELP  DEBUG  for  further  infor-

mation about debugging modes.

SET MODEM

The SET MODEM command defines the type of MODEM use for dialing out on the line

set with the SET LINE command, or, in the case of the PRO/350,  the  XC  or  XK

port.  There are only a few modems defined at this time, they are:

VADIC           Generic RACAL-VADIC autodial

VA212PA         Stand alone VADIC VA212

VA212PAR        Rack mounted VADIC VA212

VA4224          Rack mounted VADIC VA4224 .v22bis

HAYES           Hayes smartmodem

DF100           DEC DF112

DF200           DEC DF224

DF03            DEC DF03

MICROCOM

The  DIAL  command is then used after the SET MODEM command.  For example, on a

PRO/350 running P/OS:

Kermit-11>set prompt PRO>

PRO>set modem va212pa

PRO>dial 5374411

Modem in command mode

Modem dialing

Connection made, type CONNECT to access remote

PRO>con

Enter class ? VX785A

Class start

Username: BRIAN

12.6. The SET Command                                                  Page 304

-------------------------------------------------------------------------------

Password:

...and so on

SET PACKET-LENGTH

Syntax: SET PACKET-LENGTH length

You can alter the default transmitted packet length with the SET  PACKET-LENGTH

command.   This should not normally be needed unless the line is very noisy, at

which time you should probably give up anyway.

SET PARITY

Syntax: SET PARITY {EVEN, ODD, MARK, NONE, SPACE}

This is used with the SET LINE and CONNECT commands  to  specify  the  type  of

parity  for  the remote link.  It defaults to NONE and can be any of ODD, EVEN,

MARK or SPACE.

All parity generation is done via software, no special hardware is used.    The

use  of  software  parity  generation  is  restricted to 8 bit links only.  The

character format, if parity is set to anything but NONE, will be 7 bits of data

followed  with  high  bit  set  or  cleared to indicate the parity.  If you set

parity to anything but NONE (the default), Kermit-11 will be forced to  request

8bit  prefixing from the other Kermit-11, which is a method by which Kermit can

'prefix' eight bit characters with a shift code.  You MUST use parity (even  if

MARK  or  SPACE)  when  using  Kermit-11 with the IBM CMS Series/1 or 7171 3270

emulator, or in linemode through a 3705 front end.

SET PAUSE

Syntax: SET PAUSE seconds

PAUSE tells Kermit to wait the specified number of seconds between each  packet

being  sent  to the other Kermit.  This may be useful under situations of heavy

system load.  This may be automatically  computer  by  Kermit-11  in  a  future

release as a function of line speed.

SET PHONE

Syntax: SET PHONE {NUMBER, TONE, PULSE, BLIND}

The SET PHONE NUMBER command allows you to associate a phone number with a sym-

bolic name for later use with the DIAL command.   These  definitions  could  be

placed in your KERMIT.INI file, and then referenced later.  Example:

Kermit-11>set pho num work 5374411

Kermit-11>set pho num market 16174671234

Kermit-11>dial work

The  other  two  SET PHONE options, SET PHONE [TONE][PULSE] and SET PHONE BLIND

12.6. The SET Command                                                  Page 305

-------------------------------------------------------------------------------

are not useful unless the appropiate dial formatting string and  character  se-

quences for selecting PULSE or TONE, and BLIND dialing are present in the modem

definition macros in K11DIA.MAC.  The format effector for TONE/PULSE is %M  and

the  effector for BLIND is %B.  Currently (in 3.54) only the VA4224 has entries

for these options.

SET POS

Syntax: SET POS {DTE, NODTE}

The SET POS command allows options SPECIFIC to P/OS to be altered.    The  most

useful  option  is  the  SET POS [NO]DTE command.  This allows Kermit-11 to use

PRO/Communications version 2 for terminal emulation, if this product  has  been

installed  on  the  PRO/350.    Of course, if this option is chosen, control is

returned to the PRO with the EXIT key (F10) rather than with Control \C.

SET PROMPT

Syntax: SET PROMPT prompt

The SET PROMPT command is useful if you are using two Kermit-11's  to  talk  to

each  other.    By using the SET PROMPT command, you can change the prompt from

'Kermit-11>' on either (or both) Kermit to something that would indicate  which

system you are currently connected to.  Examples:

Kermit-11>set prompt Kermit-11/1170>

Kermit-11>set prompt Fubar>

Kermit-11>set prompt ProKermit-11>

SET RECEIVE

Currently  the  SET  RECEIVE  and SET SEND basically work the same in that they

only alter the END-OF-LINE character and the START-OF-PACKET value, as in:

Kermit-11>set rec start 2

Kermit-11>set rec end 12

The command SET RECEIVE PACKET-LENGTH command is discussed below.

SET RECEIVE END-OF-LINE

This instructs Kermit-11 to expect something other than  the  default  carriage

return  (octal  15)  at the end of a packet.  Kermit-11 will ignore packet ter-

minators.  The SET SEND END command is of more  use  in  conditioning  outgoing

packets.

12.6. The SET Command                                                  Page 306

-------------------------------------------------------------------------------

SET RECEIVE START-OF-PACKET

The  normal  Kermit  packet prefix is Control-A (ASCII 1); this command changes

the prefix Kermit-11 expects on incoming packets.  The only reasons this should

ever  be  changed  would  be: Some piece of equipment somewhere between the two

Kermit programs will not pass through a Control-A; or, some piece of of  equip-

ment  similarly placed is echoing its input.  In the latter case, the recipient

of such an echo can change the packet prefix for outbound packets  to  be  dif-

ferent  from  that  of  arriving packets so that the echoed packets will be ig-

nored.  The opposite Kermit must also be told to change the prefix for its  in-

bound packets and the prefix it uses on outgoing packets.

SET RECEIVE PACKET-LENGTH

Syntax: SET RECEIVE PACKET-LENGTH length

This command has two functions.  The first, and normal one, is to reduce incom-

ing packet lengths in the event that normal sized Kermit  packets  can  not  be

passed  through  the  communications  circuit.    There could be, perhaps, some

'black box' somewhere in the link that has a very small buffer size; this  com-

mand could be used to reduce the size that the SENDING Kermit will use.

The  other  use  is  to  enable  a  protocol  extension  to Kermit called 'Long

Packets'.  The actual protocol is documented elsewhere,  let's  just  say  that

this  is a way for two Kermit's to use packet sizes far greater than the normal

('Classic') packet size if 90 characters or so.  The main use of  this  feature

is in file transfer over links that introduce considerable delay, it is not un-

common for packets to incur an one to two second delay.  The net  result  is  a

VERY  slow  running  Kermit  with an effective speed of perhaps 300 to 600 baud

rather than 1200 or 2400 baud.  By making the packets longer, we raise the  ef-

fective  speed  of  such  a  circuit.   The main restriction on the packet size

chosen is the link, a given circuit may not pass 500 character packets.   Also,

BOTH  Kermits  must  support  this  extension to the protocol, they will always

negotiate it before any file transfer.  See the notes at the end of this  docu-

ment for more information.

It  is HIGHLY recommended that you use the CRC block check, as the default type

one checksum could be inadequate for such long packets, as in:

Kermit-11>SET BLO 3

SET RECORD-FORMAT

Syntax: SET RECORD-FORMAT {STREAM, VARIABLE}

Kermit will, by default, create RMS11 variable length implied carriage  control

records  for  text files.  You can override this and change it to create stream

ascii records with the SET RECORD-FORMAT STREAM command.  This  is  useful  for

RSTS/E  systems  if  you need file compatability with BASIC Plus.  This command

would be most useful in a KERMIT.INI file, which is  executed  by  KERMIT  when

Kermit starts.

12.6. The SET Command                                                  Page 307

-------------------------------------------------------------------------------

SET RETRY

Syntax: (SET RETRY )number

SET  RETRY  value tells Kermit to try that many times on a NAK'ed packet before

giving up.  This should only be needed if the line is extremely  noisy  or  the

PDP-11 host is running very slowly due to the system load.

SET RSX

The SET RSX command is intended to deal with the peculiarities often found with

RSX systems.  There are currently three SET RSX commands, as in:

Kermit-11>SET RSX FASTIO        Default for packet reading,

waits for <CR>.

Kermit-11>SET RSX CHARIO        Read one char at a time for

packet reading.

Kermit-11>SET RSX TC.DLU n      Alters  the TC.DLU setting.

Kermit-11>SET RSX CONNECT ALT   Uses a new  (v2.33) connect

driver which bypasses TTDRV

flow control.

Kermit-11>SET RSX CONNECT DEF   Use old connect code (2.32)

The SET RSX command is subject to change and the above options may  be  removed

in  the  future.    Note  the the SET RSX CHARIO may be needed when transfering

files with parity enabled.  This command alters the method by which a packet is

read; instead of waiting for a carriage return, Kermit reads the typeahead byte

count and then issues a read for that many characters.  This is the same method

Kermit-11 ALWAYS uses under P/OS.

SET RT-11 CREATE-SIZE

Syntax: (SET RT-11 CREATE-SIZE )number

The  SET  RT-11 CREATE value command was added to assist those RT-11 users with

very small disks to be able to get files with sizes greater that  half  of  the

available  contiguous  space available.  While this is NOT a problem going from

one Kermit-11 to another Kermit-11 since the PDP-11 Kermit supports a subset of

the  protocol known as 'ATTRIBUTES', other Kermits may not support the exchange

of file sizes (most do not).  Thus if your  largest  contiguous  space  is  300

blocks and you want to get a 250 block file, the command:

Kermit-11>set rt-11 cre 250

would be needed, as RT-11 by default only allocates 50 percent of the available

space.

12.6. The SET Command                                                  Page 308

-------------------------------------------------------------------------------

SET RT-11 FLOW-CONTROL

Syntax: SET RT-11 {FLOW-CONTROL, NOFLOW}

Note that for the connect command under RT-11 you will most likely need xon/off

flow  control to be generated by Kermit-11.  This is enabled with the SET RT-11

FLOW command.  This is by default NOFLOW since the modem  the  author  uses,  a

Vadic  212PA,  can't handle XONs and XOFFs while in command mode.  The solution

here is to escape back to Kermit command mode after the remote system has  been

logged into, and then type SET RT-11 FLOW.

The effect of SET RT-11 FLOW is for Kermit-11, when in connect mode, to send an

XOFF to the host every eight characters.  When the loop in the  connect  module

finds  no  more  data  in the input buffer, it sends up to 2 XON characters (in

case the first XON got lost) to tell the remote system to start sending  again.

The  reason  for  doing  so is that the RT-11 multiple terminal service is very

slow about handling input interrupts and does not do any of it's own flow  con-

trol  when  it's  internal ring buffer gets full.  This has been tested at line

speeds up to 4800 baud without losing data.  This setting should not be  needed

for use with the XC/XL handlers.

SET  RT-11  FLOW  has NO effect on packet transmission, since the Kermit packet

size is never mode than 96 characters,  and  the  RT-11  input  buffer  is  134

characters in size.

The SET RT-11 [NO]FLOW command replaces the older SET RTFLOW [ON][OFF].

SET RT-11 VOLUME-VERIFY

Syntax: SET RT-11 {VOLUME-VERIFY, NOVOLUME}

Normally  RT-11  Kermit-11  will check the directory header of a disk to verify

that it most likely contains a valid RT-11 file structure before trying to read

the directory.  If for some reason your disk does not contain the standard data

at offset 760 in the header, Kermit-11 will reject the disk.    The  SET  RT-11

NOVOL command will instruct Kermit-11 to bypass that check.

SET SEND

The SET SEND command controls what Kermit-11 will be doing for outgoing packets

in that you may want to alter the packet terminator and/or the start of  packet

character (by default, 15 octal and 1 octal respectively.  See HELP SET RECEIVE

for more information.

The only extra option for SET SEND is SET SEND [NO]XON.   If  the  command  SET

SEND  XON  is give, then every packet sent will be prefixed with an XON charac-

ter.  This could be useful in situations where flow control is  erratic.    The

actual  intent  of  this  option was to try to circumvent a firmware bug in the

DHV11 when used under RSTS/E.

12.6. The SET Command                                                  Page 309

-------------------------------------------------------------------------------

SET SPEED

Syntax: SET SPEED speed

SET SPEED value sets the line speed for the device specified via the  SET  LINE

command,  and  used  for the CONNECT command.  Changing the speed of a terminal

line requires privilege for RSTS and RSX-11M/M+.  The SET  SPEED  command  will

only function with a DH11, DHV11, DZ11 or DZV11 multiline interface.  Example:

Kermit-11>set speed 1200

1200 Baud would be a normal speed to use with a VA212PA or a DF03.

Please  note  that Kermit-11 CAN NOT change the speed of a DL11 type interface,

nor can it change the speed of a PDT-150 modem port.  For a PDT-150 modem port,

use  a  command  of  /M/S:nnnn  to  change  the speed to nnnn for the SPEED.SAV

program.

SET TIMEOUT

Syntax: SET TIMEOUT seconds

The timeout value tells Kermit how long to wait to get a packet from the  other

Kermit.   If system loads are high, it may be desirable to increase this beyond

the default of 10 seconds.

SET TERMINAL

Syntax: SET TERMINAL {TTY, VT100}

The SET TERMINAL command simply controls the way which Kermit-11 prints  packet

counts while send or receiving a file (or group of files).  The simplest way is

the default, SET TER TTY.  Using SET TER VT100 will  cause  Kermit  to  display

headers  for  the  numbers  printed,  at a possible cost in packet speed due to

screen control overhead.  On the PRO/350, VT100 is assumed.  On RSTS/E v9.0 and

later, the executive is queried for the terminal type.

SET UPDATE

Syntax: SET {UPDATE number, NOUPDATE}

The SET UPDATE command controls the frequency at which the packet count display

is updated.  The default is 1, displaying each packet.  A SET UPD 0  will  dis-

able all packet count logs, whereas a SET UPD N will update the display every N

packets.  The SET NOUPDATE command is the same as SET UPDATE 0.

12.6.1. The DIAL Command                                               Page 310

-------------------------------------------------------------------------------

12.6.1. The DIAL Command

The DIAL command is new for version 3.29 of Kermit-11.   The  DIAL  command  is

used  to  dial a number on an attached modem of known type (see SET MODEM).  To

find out the current known modems, use the SET MODEM ?  command.  The following

example  shows  a  RACAL-VADIC VA212 modem connect to the XK: port on a PRO/350

running P/OS version 2.

Kermit-11>set prompt PRO>

PRO>set modem va212pa

PRO>dial 5374401

Modem in command modem

Modem dialing

Connection failed, !BUSY

PRO>dial 5374411

Modem in command modem

Modem dialing

Connection made, type CONNECT to access remote

PRO>con

Enter class ? VX785A

Class start

Username: BRIAN

Password: ......................

See SET MODEM for more information.

12.7. System Manager's Notes

12.7.1. Odds and Ends

There are a few odds and ends that should be made aware to the  system  manager

of  any  PDP-11  system  regarding  Kermit-11.  They are as follows, grouped by

operating system.  Please note that installation instructions are in K11INS.DOC

and that additional information may be in Kermit-11's online help command.

RESTRICTIONS

Prior  to  version  2.21,  Kermit-11 did not support 8-bit prefixing.  Prior to

version 2.23, Kermit-11 did not support repeat character encoding.

The PRO/RT-11 version of Kermit-11 will request 8-bit prefixing due to the fact

that  the  XC handler does not support 8BIT data.  For most Kermits this should

not be a problem.  The XC handler always strips bit 7 from the character  being

sent,  so  the  PRO/RT-11 version of Kermit will request prefixing of such.  It

does so internally by setting PARITY to SPACE (always clear the high  bit,  bit

seven).

Note  that this implies that a SET PARITY SPACE command will force Kermit-11 to

request '8bit' prefixing in order to transfer binary files across a  seven  bit

link.

12.7.1. Odds and Ends                                                  Page 311

-------------------------------------------------------------------------------

P/OS

Kermit-11  will run on under P/OS on the Pro/350, the executable file is called

K11POS.TSK.  It does NOT run from a menu, the normal way to run it is  via  the

RUN  command  in  DCL.  It will support the Kermit-11 attribute packets, thus a

PRO/350 connected to a PDP-11 host can transparently handle  binary  and  other

types  of  files.   The P/OS Kermit-11 can be run either as a local Kermit or a

Kermit server.  This has been tested under P/OS version 2 connected to  both  a

PDP-11/23+ and PDP-11/70 RSTS/E host.

When  Kermit-11 is started on the PRO, it will automatically do a SET LINE XK0:

and a SET SPEED 9600.  You can, of course, change the  speed  to  whatever  you

need with the SET SPEED command.  The line should be left as XK0:.

The  top  row  function  keys are mapped internally.  Kermit-11 maps F5 (break)

into a true BREAK (a space of 275 ms), F6  (interrupt)  to  Control-C,  F10  to

Control-Z,  F11  to escape (octal 33) and F12 to backspace (octal 10).  The in-

coming escape sequence DECID is intercepted to allow Kermit-11 to send  back  a

device response of VT100.

RSTS/E

Kermit-11  runs  on  version 7.2 or later of RSTS/E.  Due to options present in

version 8, binary file transfers will not be  possible  under  version  7.2  of

RSTS/E.    This  is due to the use of 8 bit mode for the terminal link to allow

all characters to be passed.  The so called '8BIT' terminal setting was new  as

of version 8.0-06 of RSTS/E.

Any  RSTS/E  system  running Kermit-11 will need the sysgen option for multiple

private delimiters in the terminal driver.  This special mode is  needed  since

the  'normal'  RSTS/E binary terminal mode has a 'feature' that disables binary

mode whenever the terminal times out on a read.  Since timeouts  are  essential

to Kermit error recovery, binary mode can not be used for i/o.

Certain  functions  of Kermit-11 require that the system manager install Kermit

with temporary privileges, these commands are the SYSTEM, WHO and  REMOTE  HOST

commands.  Kermit-11 does NOT need these to operate correctly.

Kermit-11 can only be built (from source, not from HEX files) under RSTS/E ver-

sion 8.0 or later due to the use of RMS11 v2.0 and new assembler directives.

Support for the server remote login is  only  available  under  RSTS/E  9.0  or

later.    Also,  a REMOTE LOGIN command to a RSTS/E server will fail unless the

user has the WACNT privilege.  While the LOGIN program will skip  the  password

lookup if WACNT is present, Kermit will require a password.

RSX-11M/M+

Kermit-11 can not be installed non-checkpointable due to an apparent RMS11 bug.

In other words, don't try to install the task '/CKP=NO'.

To use the connect command effectively, typeahead support is needed in the ter-

minal  driver.    For  RSX-11M+,  set the typeahead buffer size high, as in SET

/TYPEAHEAD=TT22:200.  Also, if your connect line is TT22: (as above),  use  the

12.7.1. Odds and Ends                                                  Page 312

-------------------------------------------------------------------------------

mcr command SET/SLAVE=TT22:

Kermit-11  can  only  be  built  under  RSX-11M  version 4.1 or later, or under

RSX-11M Plus version 2.1 or later due to the use of  RMS11  v2.0  and  new  as-

sembler directives.

There is a SET RSX command, see HELP SET RSX for further information.

As  a  side  issue,  please note that the file K11POS.TSK is quite usable under

RSX, the difference being that K11RSX.TSK has DECNET support and  RMS-11  over-

layed  in  the task image (besides which, due to the lack author's systems run-

ning RSX may not be up to date) linked into it, whereas K11POS  has  NO  Decnet

support  but IS linked to the RMS11 library RMSRES (v2), thus K11POS saves disk

space as well as supporting named directories, ala VMS style.

RT-11

Kermit-11, as of version 2.20, has been tested under RT-11  version  5.0  under

the  FB  and  XM monitors using a DZ11 line for the link, and also on a PDT-150

using the modem port for  the  link.    It  has  additionally  been  run  under

Micro-11's and the PRO/350 using the XL and XC handlers respectively.

Kermit-11  requires .TWAIT support as well as multiple terminal support (unless

the XL/XC handler is used).   The  use  of  multiple  terminal  support  allows

Kermit-11  to use any type of interface sysgened, including the DZ11 and DZV11.

It is possible under version 5 of RT-11 to use the XL: handler instead  of  the

multiple  terminal  support.    The  use  of the XL: driver will result in much

faster file transfer at high baud rates.  Note that XL: must be set up at  sys-

tem startup or at some time later to set the proper speed, CSR and vector.

For  those  users who do not have multiple terminal support and do not have the

XL handler, Kermit-11 will force the use of the  console  for  data  transfers.

This  will  require  that  Kermit-11 request eight bit prefixing from any other

Kermit wishing to send binary data files.  Additionally, you can force  console

mode by doing a SET LINE TT:

Please  note that the device name syntax for terminal lines follows the MT unit

numbers, thus if a SHO TER gave unit 5 for DZ11 line  0  the  the  device  name

would  be  SET  LINE 5.  If you use the XL handler, you would say SET LINE XL:.

To force the console to be used, you would SET LINE TT:.

Additionally, Kermit-11 for RT-11 looks for its help file, K11HLP.HLP,  on  DK:

first and then on SY: if the first one fails.

Full  wildcarding  is  supported for RT-11, in the form *.type, name.*, *.* and

the % character to match any single character.

Kermit-11 can only be built on RT-11 version 5.0 or later due to the use of new

assembler directives.

Please note that for the connect command under RT-11 and the use of the MT ser-

vice, you will most likely  need  xon/off  flow  control  to  be  generated  by

Kermit-11.  This is enabled with the SET RTFLOW ON command.  This is by default

OFF since the modem the author uses, a Vadic 212P, can't handle XONs and  XOFFs

while  in  command mode.  The solution here is to escape back to Kermit command

12.7.1. Odds and Ends                                                  Page 313

-------------------------------------------------------------------------------

mode after the remote system has been logged into, and then type SET RTFLOW ON.

Due to overlaying constraints, the RT-11 Kermit-11 will  not  accept  wildcards

for the RENAME and DELETE commands and the REMOTE server equivalents.

The  executable  files  are K11XM.SAV for the XM system and PRO/350, and K11RT4

for the FB system.

As a final (I hope) RT-11 note, see the RT-11 v5.1 Release Notes page  9-2  and

chapter  12.    The discussion relevant here regards the use of the XL/XC hand-

lers.

Note that the default XL: handler vector (DL-11, DLV-11) is 300 and the CSR  is

176500.   For the Micro-11, PDP-11 and LSI-11, when the DL11/DLV11 interface is

installed the field service representative will inform you  what  the  CSR  and

VECTOR  are.    If they are NOT 176500 and 300, then to use the XL: handler you

will need, prior to running Kermit-11, to set them.  Suppose the DL  vector  is

400  and  the CSR is 176510.  Then the following DCL commands would set the ad-

dresses for RT-11:

.SET XL CSR=176510

.SET XL VECTOR=400

You SHOULD NOT ever alter these settings for XC: on the PRO/3xx.  The ONLY set-

tings  you  can  alter  for  the PRO/3xx is the speed, as in DCL command SET XC

SPEED=nnnn.  Kermit-11 CAN NOT alter the XC: speed itself.  As noted previously

in  this  document,  Kermit-11  executes  the Kermit-11 command SET LIN XC: im-

plicitly if it finds itself running on a PRO/3xx system.

Note that if your modem requires DTR to be present, you must use either an  in-

terface that asserts it (as does the PDT and PRO communications port), force it

high internally to the modem, or build a cable to force  it  high.    See  HELP

MODEM for more information.

TSX+

While  most  of  the above notes for RT-11 apply for TSX+, there are a few dif-

ferences of note.  The first, in that TSX+ is a timesharing system, allows  the

Kermit  user  to  log  in  normally from another system running Kermit (as in a

Rainbow) and give the TSX+ Kermit the SERVER command and commence file transfer

operations  from the other system (ie, the Rainbow).  If you are dialing INTO a

TSX+ system, you need to give the TSX command:

.SET TT 8BIT

to be able to transfer data to your local (PC, other PDP-11,...)  system  with-

out incurring the overhead of the Kermit protocol known as eight bit prefixing.

If this is not possible, due to your local system  requiring  parity,  or  some

other  intervening  device adds parity, then you should give Kermit the command

SET PARITY SPACE, to let Kermit know that it can't send binary data as-is.

To use Kermit-11 to dial out from the TSX+ system, the following  commands  are

needed.   Note that TSX+ commands will be preceeded by the normal RT-11 prompt,

the ever present DOT ('.'), whereas Kermit-11 commands will be prefixed by  the

default Kermit-11 prompt, 'Kermit-11>':

12.7.1. Odds and Ends                                                  Page 314

-------------------------------------------------------------------------------

.SET CL LINE=n          Where 'n' is the unit number

.SET CL NOLFOUT

.SET CL SPEED=n         Where 'n' is the speed for that unit

.ASS CL XL

Kermit-11>SET LIN XL:

Kermit-11>CONNECT

As  of Kermit-11 version 3.44, you may use CL directly in the SET LINE command,

as in:

.SET CL3 LINE=3

.R K11XM

Kermit-11>SET LIN CL3

Kermit-11>SET SPEED 1200

Kermit-11>CONNECT

A sample command file in actual use is:

SET CL3 LINE=3

SET CL3 NOLFOUT

SET CL3 TAB

SET CL3 FORM

SET CL3 SPEED=2400

ALLOCATE CL3:

R K11XM

DEALLOC CL3

SET CL3 LFOUT

SET CL3 LINE=0

SH CL

If you are running PRO/TSX+, then Kermit will make the assignment of LINE 3  to

either  CL0 or CL1 if you are running Kermit from the console, ie, LINE 1.  The

speed will default to the last SET SPEED or the speed set at system boot.

Lastly, TSX+ needs PLAS support to use K11XM.SAV, see  the  installation  notes

for further data.

RSTS/E VERSION 9.X

RSTS/E does not control modems signals in the manner that RSX or VMS does.  VMS

always asserts DTR whereas RSTS/E will not assert DTR until the terminal driver

can  see  RCD  (also known as DCD) which is pin 8 (eight) for the RS232 connec-

tion.  To connect directly to a modem (like a VADIC 212, sorry, no  DEC  modems

here) we must do one of two things:

1. Force  the  modem  (via strapping options or whatever) to assert RCD

(DCD) pin 8, thus RSTS/E will see carrier and raise DTR (pin 20  for

RS232)

2. Set   the   terminal   to   LOCAL   (RSTS/E   V9   syntax  'SET  TER

TTxx:/NODIAL/PERM') and break pin 20 (DTR) and connect pin 20  to  8

on the modem side.  This will cause the modem to be able to dial out

and allow RSTS/E to connect to it.  You will also need to  have  the

modem  assert  RCD,  pin 8.  Keep in mind that the Kermit-11 command

DISCONNECT (or HANGUP) will not function if a line is set to  NODIAL

12.7.1. Odds and Ends                                                  Page 315

-------------------------------------------------------------------------------

(INIT  SET  syntax  'LOCAL').  This has been tested on a Racal Vadic

VA212.

3. Break pin 8 (RCD) and loop DTR (pin 20) on the CPU side to RCD  (pin

8)  on  the  CPU side.  Then use the command SET DTR in Kermit-11 to

get RSTS to raise DTR and thus loop it's DTR  signal  back  to  RCD.

See the next note regarding this.

For  those of you who have port switches such as the Gandalf type, there is one

additional problem.  For Gandalf, suppose you want to connect a DZ11 line to to

an  AMTB2.  You will have a problem, in that the Gandalf AMTB2 wants to see RCD

(DCD) asserted to make a connection.  What you may need to do is this:

Make a cable for the DZ11 to AMTB2 port as follows:

CPU side                        AMTB2 side

20--|

8---|-----------8

7---------------7

3---------------2

2---------------3

Note that 20 is tied to 8 on the CPU side.  Also, 2 is swapped for 3.

Then, the Kermit-11 command SET DTR, which forces RSTS  to  raise  DTR  for  30

seconds,  will  cause  the  DTR signal to loop back to the RCD (DCD) signal and

thus tell RSTS that there is carrier detect which will raise DTR  (the  chicken

or  egg question) and get things rolling.  The Kermit-11 HANGUP (or DISCONNECT)

command will drop DTR and force the modem to break the connection.

RSX AND MODEMS

While the author's experience on RSX is limited, the following notes may be  of

use.   Dialing out on a LOCAL line will often require that the modem assert in-

ternally DTR.  If a line is set REMOTE on RSX, the driver will assert  DTR  and

RTS.   For a modem, like a VA212PAR strapped at the factory defaults, this will

cause the modem to assert DSR and RCD.  On the VADIC in particular,  the  modem

will  drop  RCD  during a DIAL command unless the modem is configured to assert

RCD continuously.  For dialing out, ideally the modem should be able to  assert

RCD  via an option or internally settable strap or switch.  If this is not pos-

sible, an alternative is to break line 8 (RCD) and jumper DTR (20) to  RCD  (8)

on  the CPU side.  This will force RSX to always see carrier detect and allow a

dial sequence to complete.  The Kermit-11 command DISCONNECT (or  HANGUP)  will

still  disconnect  the  modem as the modem will drop from the line when it sees

DTR go low (assuming the modem is not strapped to assert DTR internally).

12.8. Typical Kermit-11 Transfer Rates

Some sample timings for Kermit-11 and long packet support.  The packet size  in

the RSTS/E to P/OS was 500 bytes, the size from RSTS/E to RSTS/E was 700 bytes.

These sizes are somewhat arbitrary, they depend more on the system's  buffering

capabilities than anything else.

Host buffering capabilities:

12.8. Typical Kermit-11 Transfer Rates                                 Page 316

-------------------------------------------------------------------------------

P/OS                          500 (estimated)

RSTS/E 9.0 or later           up to 7000, given sufficient system pool

RSX-11M+                      255 (I/D space CPU only)

RSX-11M                       34

RT-11                         134 (could be larger with mod to XC/XL)

As  it can be seen, large packets make sense only for RSTS/E, P/OS and RSX-11M+

if one wishes to avoid XON/XOFF overhead at high speeds.  It should be possible

to run larger packets on M+ and RT-11 at lower speeds.

File  transferred:  K11POS.TSK,  size  102,400 bytes (200 disk blocks).  Actual

data packet characters AFTER prefixing was 120,857.

Time    Speed   Data rate       Comments

seconds baud

1436    1200    84/sec          11/44 to PRO/350, 'Classic' Kermit

local phone call

1237    1200    97/sec          11/44 to PRO/350, 500 Char packets

local phone call

2915    1200    41/sen          11/44 to PRO/350, 'Classic' Kermit

local call, 1 second ACK delay.

1492    1200    81/sec          11/44 to PRO/350, 500 Char packets

local call, 1 second ACK delay.

304     9600    397/sec         11/44 to 11/44, 'Classic' Kermit,

connected locally via Gandalf switch.

245     9600    493/sec         11/44 to 11/44, 700 char packets,

connected locally via Gandalf switch.

The last two timings are much lower than the line speed due to the fact the the

PDP  11/44  is  running  100%  busy trying to keep up with character interrupts

using a normal terminal driver.  A special  purpose  driver,  such  as  the  XK

driver  found on P/OS, would have lower overhead and allow somewhat faster data

rates.

Long packets were chosen for Kermit-11 due to the lack  of  suitable  interrupt

driven  i/o  (at  this  time)  under one of the operating systems, RSTS/E.  The

Sliding Windows would likely function better in those situations where the cir-

cuit delay is much higher, or when the circuit can not accommodate large packet

sizes.

12.9. Common Problems

CONNECTION FAILS

Check modem control signals.  RSX needs TC.DLU set to two to talk to a dial out

modem,  otherwise you will need to strap or jumper signals in the modem to have

carrier detect set high.  RSTS/E also should  have  the  modem  assert  carrier

detect.  If not, see the previous notes about modems.  If all else fails, put a

breakout box in the line and observe what signals are present.

12.9. Common Problems                                                  Page 317

-------------------------------------------------------------------------------

FILE TRANSFER FAILS.

If the file transfer aborts on retries  immediately,  there  may  be  a  parity

problem.   If the problem shows up on binary files, try a SET PAR SPACE command

to Kermit; that will force eight bit data to be prefixed into seven bits.    If

you  instead  get  a retry about once every 10 seconds, the other Kermit is not

responding and your Kermit is timing out.  Check to see if your  connection  is

still present, and try the SET PARITY command.

If  you  are  sending  binary data between unlike Kermits, you will most likely

have to give the proper command to each to prepare them for  the  binary  data;

this  is  the  SET FILE command; for Kermit-11 it's SET FIL BIN (or SET FIL TYP

FIX); for VMS Kermit it's SET FIL TYP FIX.

If your Kermit's packets are being echoed back, try a SET SEND START value com-

mand for your Kermit, and a SET REC START samevalue for the other Kermit.  This

will force Kermit to ignore any echoed packets as they won't have  the  default

start of packet character (a CONTROL A, octal 1).

13. Apple II Kermit                                                    Page 318

-------------------------------------------------------------------------------

13. Apple II Kermit

Authors:    Antonino N. J. Mione (Stevens Institute of Technology),

Peter Trei (Columbia University),

Ted Medin (NOSC)

Version:    3.81

Date:       December 1987

Kermit-65 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            No

Alternate block checks:            No

Terminal emulation:                Yes (VT52, VT100)

Communication settings:            Yes

Transmit BREAK:                    Yes

IBM communication:                 Yes

Transaction logging:               No

Session logging (raw download):    Yes

Raw upload:                        No

Act as server:                     Yes

Talk to server:                    Yes

Advanced commands for servers:     Yes

Long packets:                      Yes

Sliding windows:                   No

Local file management:             Yes

Handle file attributes:            No

Command/init files:                Yes

Printer control:                   Yes

Kermit-65  is  a  program that implements the Kermit file transfer protocol for

the Motorola 6502 processor family (hence the name, Kermit-65) on the Apple  II

microcomputer  system.   It is written in 6502 assembly language and should run

on any Apple II or compatible running DOS 3.3 or PRODOS 8.  This  section  will

describe  the things you should know about the file system in order to make ef-

fective use of Kermit, and then it will describe the special  features  of  the

Kermit-65 program.

13.1. Supported Systems and Devices                                    Page 319

-------------------------------------------------------------------------------

13.1. Supported Systems and Devices

There  are  several  different Apple II's which can run Kermit-65.  Kermit will

have no problems running on an Apple II, II+, //e or //c system.  Of  the  dif-

ferent communication devices available for the Apple II, Kermit-65 supports the

ones shown in Table 13-1.

-------------------------------------------------------------------------------

Apple Com Serial Card

Apple Super Serial Card & //c Serial Port

Apple//gs Serial Port

CCS 7710 Serial Card

CPS Serial Card

D.C. Hayes Micromodem.

Microtek sv-622 Card

Prometheus Versacard (Uses the Apple Com Card driver)

Table 13-1:   Apple II Communication Cards Supported by Kermit-65

-------------------------------------------------------------------------------

It is possible that other  cards  may  have  operational  characteristics  very

similar  or  identical to one of the devices above. If this is the case, it may

work using one of the currently available device drivers.  The user may want to

try  each  of  the above options to see if any of them work.  Kermit-65 must be

told in which slot the card resides.  This may be done with the  'SET'  command

(documented below).

13.2. The DOS 3.3 File System

Items  of  importance which will be discussed in this section include filenames

and file characteristics.

Apple DOS Filenames

Filenames under Apple DOS may contain almost  any  ASCII  character  (including

space). It is not recommended that special characters, (i.e. control characters

or spaces) be used in a filename to be transferred by Kermit-65 since they  may

cause problems when parsing the filename.  Filenames may be up to 40 characters

in length.

Apple DOS File Characteristics

All files in Apple DOS have a file type associated with them which is contained

in  the  directory  entry  for the file but is not part of the filename itself.

There are four types of files in DOS 3.3. They are:

1. APPLESOFT BASIC

2. INTEGER BASIC

3. BINARY

13.2. The DOS 3.3 File System                                          Page 320

-------------------------------------------------------------------------------

4. TEXT

All file types have their data stored in eight-bit bytes although  not  all  of

them need the eighth bit. The two file types containing basic programs required

the eighth bit due to the nature of the data being  stored.  BINARY  files  are

images  of  memory  copied into a file. Often, these are machine code programs.

These files require all eight bits. TEXT files normally contain only  printable

or  carriage control characters. They are stored in the form of seven-bit ASCII

characters but the eighth bit should always be set since Apples manipulate  all

text internally as 'Negative ASCII'.  When transmitting non-text files the user

must insure that both Kermits are handling eight-bit data  so  that  no  infor-

mation  is  lost.  If  an eight-bit data path is not available (i.e. the remote

Kermit needs to do parity checking with the eigth bit), then eight-bit  quoting

should  be used.  Of course, BINARY files as well as Apple BASIC files will not

have much meaning on a different system.  If the user desires to edit  a  BASIC

file  on  a mainframe, for instance, s/he must convert it to a TEXT file before

sending it over. After receiving the file back on the Apple, the user may  con-

vert  it back to BASIC once again.  The reason BASIC files would be meaningless

to a different machine is that  the  Apple  stores  BASIC  keywords  as  single

character tokens to save space and processing time.  To convert a BASIC program

to and from a TEXT file, consult the Apple DOS 3.3 Manual.    File  information

can be obtained by issuing the CATALOG command.  For example:

]CATALOG

DISK VOLUME 010

*A 002 HELLO

B 078 KERMIT

A 002 READER

T 005 TESTFILE

]

When  Kermit-65 is receiving a file, the file it creates on diskette will be of

the type indicated by the FILE-TYPE parameter.  The file will always be left in

an  unlocked  state  after  it  is  closed  by Kermit-65.  When sending a file,

Kermit-65 will use the FILE-TYPE  parameter  to  determine  how  to  detect  an

End-of-file  condition.  Thus, it is important to have this set properly in all

cases.

Recommendations for Archiving Files

When using a large system for archiving purposes, there is no reason to convert

Apple Basic programs into text files before sending them if there is no need to

edit them on the mainframe.  The FILE-TYPE parameter must always  be  set  cor-

rectly when sending and receiving files.  The procedure for archiving files is:

1. Run Kermit on remote system.

2. SET FILE-TYPE TEXT (or APPLESOFT or ...) on Kermit-65.

3. Send the files.

13.3. The PRODOS File System                                           Page 321

-------------------------------------------------------------------------------

13.3. The PRODOS File System

The  PRODOS  system is essentialy the same as the DOS system with the exception

that performance has been improved, hardware usage has been expanded  and  file

names have different syntax.  File names are the major importance to the Kermit

system.  File names have the following syntax:

/volname/subdirectory1/.../subdirectoryn/filename

where "volname" is the volume name where the file is located.   Subdirectory(n)

is  a  subdirectory  on the volume and may be omitted.  Filenames are much more

restrictive than DOS filenames.  PRODOS filenames are limited to 15  characters

with  no imbedded spaces and few special characters, and must begin with an al-

phabetic character.  Volname may be omited from the  filename  by  use  of  the

PREFIX command.

Binary  file  transfer  using  PRODOS  has its dangers when creating new files.

PRODOS keeps the file's size and starting location in the directory which is of

course  not  transferred.    Therefore a new binary file will have its starting

location 0 which can cause some interesting problems if you try  and  BRUN  the

file.  Basic files all start at $801 (it says here) so Kermit creates new basic

files with a starting address of $801.

13.4. Program Operation

Prior to using Kermit-65 for transferring files, the modem  interface  must  be

set  to  handle  data  in a certain manner.  First, the data format should be 8

data bits and 1 stop bit.  Second, the card should be set to no  parity.    The

baud  rate  (if  adjustable) must be set to whatever rate the modem can handle.

For the D.C. Hayes Micromodem, these parameters are set correctly  by  default,

so  very  little has to be done.  For the Apple Super Serial Card these are set

from within Kermit-65 except the interrupt switch (sw6-2) which must be set for

interupts  on.    For the Microtek SV-622, all applicable parameters are set by

Kermit-65.  Some mainframes may need parity checking (i.e. most IBM  machines).

In  this  case  some  parity  setting (other than none) will be required.  When

talking with such mainframes, binary and basic files on  the  Apple  cannot  be

transferred  unless  Eighth-bit-quoting is acceptable to the host.  If you have

the parameters set correctly then the "connect" command will start Kermit talk-

ing out the communication port.

File  transfer  is  very  dependent  upon  parity. Make sure the host and local

parity are the same. Following is one IBM site's method for file transfer.

We have an IBM 3033 and 4381 and use both 3705/3725 and 7171 or Series/1  front

ends.  The differences in front ends as far as any microcomputer Kermit is con-

cerned duplex (local-echo on for the 3705,  local-echo  off  for  the  7171  or

Series/1), parity (the two front ends might use different parity, e.g. Mark for

the 3705 and Even for the 7171), and flow control (None for the 3705,  XON/XOFF

for the 7171).

In Kermit-65, IBM mainframe users need to set the following parameters:

BAUD            Whatever is supported.

PARITY          EVEN, ODD, or MARK, whatever your front end requires.

13.4. Program Operation                                                Page 322

-------------------------------------------------------------------------------

FLOW            XON for the 7171, NONE for the 3705.

FLOW DELAY      00

LOCAL-ECHO      OFF for 7171, ON for 3705

Conversing With Kermit-65

Kermit-65  reads  file  KERMIT.INIT  from  the default drive when started.  The

lines of this file are executed one at a time starting at the begining.    This

file  should  be  an  ASCII  text  file and contain commands to set up Kermit's

parameters as desired.  It will also execute Kermit's other commands.  However,

any command which reads a file (like MODEM) or leaves local mode (like CONNECT)

will terminate reading of this file and continue with  the  command  specified.

Use your favorite editor to produce this file.  Here's a sample:

set display 80 3

set keyboard 2e

set baud 4800

modem

Kermit-65's prompt is "Kermit-65>".  To run Kermit-65 and issue commands to it,

type "brun kermit".  Example:

]BRUN KERMIT

NOSC/STEVENS/CU - APPLE ][ KERMIT-65 - VER 3.81

Kermit-65>send testfile

(file is sent...)

Kermit-65>status

(performance statistics are printed...)

Kermit-65>(other commands...)

.

.

.

Kermit-65>exit

]

Like many Kermit programs,  Kermit-65  uses  a  DEC-20  style  command  parser.

During  interactive  operation,  you may use the ?-prompting help feature ("?")

and recognition (ESC) features while typing commands.  A question mark typed at

any  point in a command displays the options available at that point; typing an

ESC character causes the current keyword to be completed (or default  value  to

be  supplied).  If you have not typed sufficient characters to uniquely specify

the keyword or filename (or if there is no default value) then a beep  will  be

sounded and you may continue typing.  Keywords may be abbreviated to any prefix

that is unique.

13.4. Program Operation                                                Page 323

-------------------------------------------------------------------------------

Remote and Local Operation

Kermit-65 is normally run in local mode. It may be run as a  remote  Kermit  as

well  although  there  is no advantage to doing things that way. Kermit-65 sup-

ports User-mode commands for talking to a Server, and it does support a limited

server mode.

13.5. Kermit-65 Commands

THE SEND COMMAND

Syntax: SEND filespec

The  SEND command causes a file to be sent from the Apple to the remote system.

The Filespec is the name of the file on the Apple diskette to  be  sent.    The

parser  will  not accept control characters and certain special characters in a

filename (like comma), so the user may have to rename the  file  before  it  is

sent.    The  user may also have problems in filename compatibility with remote

Kermits. If the remote Kermit does not have the facilities to beat the filename

into  a  format that its system likes, the user may have to rename the file be-

fore sending it.  Thanks to Dick Atlee, wildcards are now acceptable when send-

ing files (they have always been acceptable when receiving files). The "*" is a

multiple character wildcard and the "=" is a single character wildcard.

The default disk drive is used for file transfers this can be changed with  the

'SET  DEFAULT-DISK'(DOS) or 'SET PREFIX'(PRODOS) command (explained below).  As

a file is being sent, the screen displays 'SENDING  PACKET...'    and  'WAITING

PACKET...'  followed by the absolute packet number since start of transmission.

If a packet must be transmitted several times and it reaches the maximum  retry

count,  the  transfer will fail and the 'Kermit-65>' prompt will return. If the

remote Kermit sends an error packet, the text of the packet will  be  displayed

on  the screen, the transfer will fail, and the prompt will return.  Currently,

a packet can be retransmitted manually by typing anything on the keyboard. If a

'Q' is typed, the entire transmission will be aborted.

THE RECEIVE COMMAND

Syntax: RECEIVE [filespec]

The  RECEIVE  command  tells Kermit-65 to receive a file or file group from the

other system.  If only one file is being received, you may include the optional

filespec  as  the name to store the incoming file under; otherwise, the name is

taken from the incoming file header.  If the name in the header is not a  legal

filename,  Kermit-65  will  attempt  to  change  it  into  something legal.  If

FILE-WARNING is on and an incoming file has a name identical to a file  already

existing  on  the  diskette, Kermit-65 will issue a warning to the user and at-

tempt to modify the filename to make it unique.  Currently,  a  packet  can  be

retransmitted  manually by typing anything on the keyboard.  If a 'Q' is typed,

the entire transmission will be aborted.

13.5. Kermit-65 Commands                                               Page 324

-------------------------------------------------------------------------------

THE GET COMMAND

Syntax: GET remote-filespec

The GET command requests a remote Kermit server to send the file or file  group

specified by remote-filespec.  This command can be used with a Kermit server on

the other end.  The remote filespec is any string that  can  be  a  legal  file

specification for the remote system; it is not parsed or validated locally.  So

if  the  remote  Kermit  supports  wildcards  you  can  specify  them  in   the

remote-filespec.  If the remote Kermit is not capable of server functions, then

you will probably get an error message back from it like "Illegal packet type".

In this case, you must connect to the other Kermit, give a SEND command, escape

back, and give a RECEIVE command.  Currently, a  packet  can  be  retransmitted

manually  by  typing  anything  on  the keyboard. If a 'Q' is typed, the entire

transmission will be cancelled.

THE CONNECT COMMAND

Syntax:  CONNECT

Establish a terminal connection to the remote system using all the current  SET

paramaters  for  terminal  type,  speed, parity, etc.  Get back to Kermit-65 by

typing the escape character followed by the letter C. The escape  character  is

Control-@   by   default.     When  you  type  the  escape  character,  several

single-character commands are possible.  These are shown in Table 13-2.

-------------------------------------------------------------------------------

?     List all the possible single-character arguments.

B     Send a BREAK signal.

C     Close the connection and return to Kermit-65.

D     Drop the phone line to the remote and return to Kermit-65.

K     TOGGLE Keypad application-mode on/off.

P     Toggle the Printer on/off.

R     pRint the screen, >= //e required

S     Show Status of the connection.

W     sWap the del and backspace key.

0     Send a null (ASCII 0).

^@    (or whatever the Connect-Escape character is):  Send the

Connect-Escape character itself.

Table 13-2:   Kermit-65 Single-Character CONNECT Escape Commands

-------------------------------------------------------------------------------

You can use the SET ESCAPE command to  define  a  different  escape  character.

When CONNECTed, Kermit-65 will be passing characters entered on the keyboard to

the remote system, and passing characters from the remote system to  the  Apple

screen.    Incoming  characters are interpreted according the selected terminal

type (see SET TERMINAL).

On an Apple II+ with an incomplete keyboard, special characters can be typed by

prefixing  regular  characters  with a right-arrow.  On uppercase-only screens,

13.5. Kermit-65 Commands                                               Page 325

-------------------------------------------------------------------------------

uppercase characters are shown in inverse and  lowercase  characters  are  dis-

played as normal uppercase characters.

Here are the rules for using the special 2/2+ input, to get all printable ASCII

characters, and how they appear on the screen.  Special meanings are applied in

various  contexts to certain characters.  The left and right arrow keys do spe-

cial things, and sometimes the escape key does  as  well.    For  letters,  the

keyboard  is always in either default UPPERCASE mode or default lowercase mode.

When in UPPERCASE, all letters typed are sent out as uppercase.  In  lowercase,

all  letters  are sent as lowercase. To reverse the case for the next character

only, hit the right-arrow ("prefix") key.  To switch the default case, hit  the

prefix-key  twice  in a row.  For funny characters, the prefix key is also used

to get the unusual punctuation characters which are not on the Apple  keyboard.

Table  13-3  shows the Apple II/II+ keyboard escapes; the letter "p" represents

the prefix character.

-------------------------------------------------------------------------------

To Get                        Type      Appearence

Left Square Bracket            p(          [

Right Square Bracket           p)          ]

Left Curly Bracket             p<          {

Right Curly Bracket            p>          }

Underline                      p-          _

Backslash                      p/          \

Tilde (wiggle)                 p^          ~

Vertical Line                  p.          |

Table 13-3:   Apple II/II+ Keyboard Escapes

-------------------------------------------------------------------------------

The left-arrow key sends a rubout (ASCII 127).  With left-arrow and right arrow

doing  special  things, it's a little hard to enter their characters (^H and ^U

respectively).  There is therefore an escape from prefix mode sequence.  If you

type prefix-ESC, the next character is sent without any interpretation.  If you

have the capability for upper/lower case, etc, then use the 'SET KEYBOARD'  and

'SET DISPLAY' commands to specify complete keyboards.

THE HELP COMMAND

Syntax: HELP

Typing HELP alone prints a brief summary of the Kermit-65 commands.

13.5. Kermit-65 Commands                                               Page 326

-------------------------------------------------------------------------------

THE MODEM COMMAND

Syntax: MODEM

Typing  MODEM causes the file KERMIT.MODEM in the default drive to be used as a

menu.  You will be able to select any line in the file to be sent to the modem.

Sorry,  you can't back up to a previous menu, you will have to Quit and execute

MODEM again.  A "connect" response from the modem will cause  Kermit  to  leave

the  modem  command  and  execute the CONNECT command.  For a Hayes modem, this

file should have commands using text status responses (not numbers).  One  com-

mand  per  line  with  comments allowed after the first space (blank). Use your

favorite editor to produce this ASCII text file.   Since  the  attention  Hayes

command  (AT)  requires a delay the "&" character takes one time delay for Ker-

mit.  Each "&" causes a delay of one second on a 6502 chip. If you have a  //gs

or  an  accelerator  board the timing may change and you may require several to

produce a one second delay.  Normally Kermit will wait for 27 seconds (again on

a  6502 chip) for the modem to respond, but any character typed on the keyboard

will terminate this wait.  You may hear the busy signal and there is  no  sense

waiting any longer, so hit (crash-not so hard) any key on the keyboard.

Following is an example of the KERMIT.MODEM file:

+++&&ATH This should get the Hayes modem's attention and then hang up.

ATDP1234567 Call your local BBS with pulse dialing.

ATDT8901234 Call your work dialup phone with touch tone dialing.

THE CATALOG COMMAND

Syntax: CATALOG

Typing CATALOG produces a catalog (directory) listing of your default drive.

THE DELETE COMMAND

Syntax: DELETE filespec

Typing DELETE causes the file specified to be deleted.

THE SERVER COMMAND

Syntax: SERVER

Typing  SERVER  alone turns Kermit into a file server to a remote Kermit.  Cur-

rently server mode will handle remote "send", "get", "remote"  and  "fin"  com-

mands.    Variants of the above commands will probably work but file serving is

very limited at present.  Because the Apple requires knowledge  of  file  types

you  can  use  the  "remote Kermit" (or whatever the remote Kermit's syntax is)

command to set the file-type on the server.  Yes, the server will  execute  any

command so you can really get the server into trouble (this is not a BBS).  You

must have the appropiate file type set before transfering files.  You can  exit

server  mode  by  typing  Control-C  (^C)  when not doing file transfers or the

remote can of course terminate via the "fin" command.

13.5. Kermit-65 Commands                                               Page 327

-------------------------------------------------------------------------------

THE REMOTE COMMAND

Syntax: REMOTE [option character-string]

The only option  currently  is  "kermit".  This  command  submits  the  command

"character-string"  to  the  remote Kermit's command processor.  Currently only

short replys are supported so returned messages about the size of the  "receive

packet  size"  are  the  limit.    The obvious usage is for setting and showing

parameters on the remote Kermit.

THE EXIT AND QUIT COMMANDS

Syntax: EXIT or QUIT

Exit from Kermit-65.  You can restart the program,  provided  you  haven't  run

anything else, by typing 'CALL 4096'.

THE SET COMMAND

Syntax: SET parameter [option [value]]

Establish  or  modify  various parameters for file transfer or terminal connec-

tion.  You can examine their values with  the  SHOW  command.    The  following

parameters may be SET:

APPLICATION-MODE   Set VT100 gs keypad in/out of application mode.

BAUD   Which baud rate should the com card use?

CURSOR-KEYS-VT100   In VT100 mode cursor keys give VT100 sequences.

DEBUGGING   TERSE or VERBOSE packet information.

DEFAULT-DISK   Which Diskette drive is used for DOS 3.3 file transfer?

DISPLAY   Which type of screen display is being used?

ESCAPE   Character for terminal connection.

FILE-TYPE   Type of Apple DOS file being sent/received.

FILE-WARNING   Warn users if incoming file exists?

FLOW   Should xon/xoff flow control be used to the remote?

KEYBOARD   II+ or //e keyboard.

KEYPAD   Is there a gs style keypad?

LOCAL-ECHO   Full or half duplex switch.

PARITY   Character parity to use

PREFIX   Which default prefix to use with PRODOS?

PRINTER   Should the printer be used for the display?

RECEIVE   Various parameters for receiving files

SEND   Various parameters for sending files

SLOT   Which slot # is communication device in?

TIMER   Should Kermit observe the receive timeout value?

TERMINAL   Which type of terminal should Kermit emulate?

13.5. Kermit-65 Commands                                               Page 328

-------------------------------------------------------------------------------

SET APPLICATION-MODE

Syntax: SET APPLICATION-MODE {ON, OFF}

For  VT100 emulation with a gs-style keypad you can set the keypad in or out of

application mode. Some computer systems set this via escape sequences so it may

not be necessary to use this command.

SET BAUD

Syntax: SET BAUD value

Value  is  the baud rate for your communication card.  For the super serial and

the microtek it can be 300 to 19200.  The actual values will  depend  upon  the

com card you are running with.

SET CURSOR-KEYS-VT100

Syntax: SET CURSOR-KEYS-VT100 {ON, OFF}

In VT100 emulation the cursor keys can also emulate the VT100 cursor keys.

SET DEBUGGING

Syntax: SET DEBUGGING {TERSE, VERBOSE, OFF}

Record  the  packet  traffic on your terminal.  Options are: TERSE, Show packet

info only (brief).  VERBOSE displays packet field descriptions with packet info

(lengthy).    OFF  disables  display  of  debugging  information  (this  is the

default).

SET DEFAULT-DISK

Syntax: SET DEFAULT-DISK {SLOT, VOLUME, DRIVE} value

This DOS command will tell Kermit-65 which disk drive should be used  for  file

transfers.    The three parameters which may be set separately are SLOT, VOLUME

and DRIVE.  The value for SLOT ranges from 1 to 7. The value for DRIVE  is  ei-

ther 1 or 2.  The value for VOLUME ranges from 0 to 255.

SET ESCAPE

Syntax: SET ESCAPE hexidecimal-number

Specify  the  control character you want to use to "escape" from remote connec-

tions back to Kermit-65.  The default is 0 (Control-@).  The number is the  hex

value  of  the ASCII control character, 1 to 37, for instance 2 is Control-B, B

is Control-K.

13.5. Kermit-65 Commands                                               Page 329

-------------------------------------------------------------------------------

SET FILE-TYPE

Syntax: SET FILE-TYPE {APPLESOFT, INTEGER, TEXT, BINARY, OTHER hex-value}

This will inform Kermit-65 what type of file is being sent or received.  It  is

important  that this is set correctly since Kermit-65 must create a file of the

appropriate type when receiving (and it has no way of knowing what kind of file

it  is).    When Kermit-65 is sending, it must also know the type of file since

that tells it how to detect the actual end-of-file.    The  keywords  for  this

parameter are listed below.  OTHER includes an added hex-value so that the user

may specify the hex value of the file-type.  This has meaning  only  in  PRODOS

and  allows  the  user  to specify any of the many different file types used in

PRODOS. I would list the hex values vs file type if I had them.  So if you have

them send them along and I will include them here.

APPLESOFT       The file being transfered is an Applesoft Basic program.

INTEGER         The file being sent/received is an Integer Basic program.

TEXT            The file being sent/received is an ASCII Text file.

BINARY          The file being sent/received is a Binary image.

OTHER           The  type  of  file being sent/received is specified by the

hex-value.

SET FILE-WARNING

Syntax: SET FILE-WARNING {ON, OFF}

This tells Kermit-65 whether to warn the user  about  incoming  filenames  con-

flicting with existing files or not.  If there is a conflict Kermit-65 will at-

tempt to change the file name to something unique.

SET FLOW

Syntax: SET FLOW {OFF, XON, DELAY number}

SET FLOW allows one to use the XON/XOFF protocol when  connected  to  a  remote

site.  Delay timings are part of this command.  Using delay times is probably a

desperation move to keep the screen/printer from losing  characters.    Setting

the timings will have to be set by experience.  Perhaps the best way to set the

timings is to bring the values down until you get failures and then double  the

timing figure.  Both LOG and SET PRINTER will probably depend on flow control.

OFF             Turn off flow control

XON             Turn on xon/xoff flow control with the remote

DELAY number    Delay the micro until XOFF takes effect

Delay  followed by a number (including 0) delays the program for that many mil-

liseconds after the XOFF is given to the remote.  This delay allows the XOFF to

take effect before the program continues.

13.5. Kermit-65 Commands                                               Page 330

-------------------------------------------------------------------------------

SET KEYBOARD

Syntax: SET KEYBOARD {2P, 2E}

SET  KEYBOARD tells Kermit-65 if the user has a full keyboard (2E) or not (2P).

If the user is on an Apple II+,  this  should  be  set  to  2P  (which  is  the

default).   When set to that, character translations are available by using the

right-arrow key as a prefix character, as shown in Table 13-3.

SET KEYPAD

Syntax: SET KEYPAD {ON, OFF}

SET KEYPAD tells Kermit-65 if the user has an Apple//gs-style keypad available.

This  is  automatically set on a gs but must be set manually on other machines.

With keypad set ON then "VT100 keypad on an Apple keyboard"  (see  below)  will

not be used but the actual keypad will be.

SET PRINTER

Syntax: SET PRINTER {ON, SLOT} number

or SET PRINTER OFF

This  allows  one  to turn the printer on for printing what is displayed on the

screen.  With all the different printers and printer cards there will be a  lot

of  variability  here but flow control (XON/XOFF) is probably required when you

are connected to a remote site. The printer can also be toggled on/off via  the

ESCAPE character followed by the command "P".

Remember  when you use your printer there are a lot of variables here. What was

being sent to the screen now is being sent  to  your  printer.    If  you  were

emulating  the  VT52  your  printer  may  not know how to handle the escape se-

quences, tabs, etc. It may be you can tell the host you are a tty or some  such

device that will give produce control codes that your printer can handle.  Some

printers may require the flow control and delay to get readable printing.

ON              Turn the printer on, slot number is required.

OFF             Turn the printer off.

SLOT number     Printer card is in slot "number".

SET SLOT

Syntax: SET SLOT number

This option tells Kermit-65 in which slot the communication device is  located.

The range for the number parameter is 1-7.

13.5. Kermit-65 Commands                                               Page 331

-------------------------------------------------------------------------------

SET TIMER

Syntax: SET TIMER {ON, OFF}

SET  TIMER will turn on or off the timeout checking for receive file transfers.

Since there is no clock for exact timing a loop of instructions has been set up

assuming a 1 megacycle CPU.  CPUs which run faster will have to make allowances

via the SET RECEIVE TIMEOUT command.

SET TERMINAL

Syntax: SET TERMINAL {MONITOR, NONE, VT100, VT52}

When TERMINAL is NONE, then all incoming characters (except nulls)  are  passed

directly to the display.

MONITOR  emulation  simply displays all the characters received from the remote

(except nulls) without any formating of the screen (40  or  80  characters  per

line).  Control characters are displayed inverse.

VT100 Emulation

The  Kermit-65  VT100  emulator is a small but working set of a true VT100 ter-

minal.  It appears to work with most of the standard full  screen  editors  and

processors  on  BSD  UNIX  and  VAX/VMS machines.  Ah Apple//e, //c, or //gs is

probably required with the Apple 80 column text card.  Sorry, but  the  II  and

II+  will  probably  have  to use one of the other terminal options.  The VT100

keypad has also been defined for the application mode via the  OA/CA/game  but-

ton.    Figure  13-1  shows the Apple keypad looks like to ED (VMS), and Figure

13-2 shows the layout on an Apple//gs keyboard.  When using EVE (VMS) the mean-

ing of the keys will of course change.

As  you  can see the keypad is physically laid out like the VT100 keypad except

for the lower right corner.  Notice that above the keys are  the  VT100  labels

while within the box (key) is the Apple key label.  Also the arrow keys work as

VT100 arrow keys with the OA/CA/game button.

VT52 Emulation

SET TERMINAL VT52 will turn on the VT52 terminal emulation.  One thing that  is

required  is  your  80-column  card  must  handle  the $16 command in order for

reverse scrolling to work.  The Apple//e 80 column card handles this fine.  The

VT52  keypad  has  been defined using the open/closed Apple.  For II or II+ one

will have to have a game paddle or joy stick (key shift mod too????)   and  use

the  buttons.  When a button/open/closed Apple is pushed then the keys starting

with 6,7,8 & 9 form the top of the keypad. Key 6 is the blue key key 7  is  the

red  key etc. The keys directly below the 6,7,8 & 9 and shifted one-half key to

the right form the second row of the keypad etc. Every thing is fine until  you

get to the last row on the keypad. There the sp bar is 0 and the other two keys

are moved to the upper right as the 0 & - keys.  This way the  arrow  keys  are

available  as  VT52 keys with the OA/CA/game button combination (thanks to Dick

Atlee for this idea).  With those  two  exceptions  the  keypad  is  physically

similar  to  a  VT52 keypad.  Remember the open/closed Apple or the game button

13.5. Kermit-65 Commands                                               Page 332

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

PF1     PF2     PF3     PF4     .(period)

------- ------- ------- ------- -------

|       |       |findnxt|delline|select |

|   6   |   7   |   8   |   9   |   0   |

| gold  | help  | find  |undelln| reset |

------- ------- ------- ------- -------

7       8       9       -

------- ------- ------- -------

| page  |section|append |delword|

|   Y   |   U   |   I   |   O   |

|command| fill  |replace|undelwd|

------- ------- ------- -------

4       5       6       ,

------- ------- ------- -------

|advanc|backup |  cut  |delchar|

|  H   |   J   |   K   |   L   |

|bottom|  top  | paste |undelch|

------- ------- ------- -------

1       2       3       ENTER

------- ------- ------- -------

| word  |  eol  | char  | enter |

|   N   |   M   |   ,   |   .   |

|chgcase|deleol |specins|  sub  |

------- ------- ------- -------

0

---------------------------------------

bline      |

space bar                         |

openline    |

---------------------------------------

Figure 13-1:   VT100 Keypad on an Apple Keyboard

-------------------------------------------------------------------------------

must be pushed (like the cntl key) to get the keypad emulation.    Figure  13-3

should clear up the questions.

The arrow keys work as VT52 arrow keys with the OA/CA/game button.

THE SHOW COMMAND

Syntax: SHOW [option]

The SHOW command displays various information:

ALL                     All parameter settings (this is quite long).

BAUD                    Baud rate of the com card.

CURSOR-KEYS-VT100       Are  the  cursor  keys  emulating  the VT100 cursor

13.5. Kermit-65 Commands                                               Page 333

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

PF1     PF2     PF3     PF4

------- ------- ------- -------

|       |       |findnxt|delline|

| CLEAR |   =   |   /   |   *   |

| gold  | help  | find  |undelln|

------- ------- ------- -------

7       8       9       -

------- ------- ------- -------

| page  |section|append |delword|

|   7   |   8   |   9   |   +   |

|command| fill  |replace|undelwd|

------- ------- ------- -------

4       5       6       ,

------- ------- ------- -------

| advanc|backup |  cut  |delchar|

|   4   |   5   |   6   |   -   |

| bottom|  top  | paste |undelch|

------- ------- ------- -------

1       2       3       ENTER

------- ------- ------- -------

| word  |  eol  | char  |       |

|   1   |   2   |   3   |       |

|chgcase|deleol |specins|       |

------- ------- -------| enter |

0          .    | ENTER |

-----------------------|  sub  |

|    bline      |select |       |

|      0        |   .   |       |

|  openline     | reset |       |

-------------------------------

Figure 13-2:   VT100 Keypad on an Apple//gs or Equivalent Keypad

-------------------------------------------------------------------------------

keys?

DEBUGGING               Debugging mode.

DEFAULT-DISK            Which Diskette drive is used for file transfer?

DEVICE-DRIVER           Which communication device is being used?

DISPLAY                 Which screen display is being used?

ESCAPE                  Character for terminal connection.

FILE-TYPE               Of Apple DOS/PRODOS file being sent/received.

FILE-WARNING            Warn users if incoming file exists?

FLOW                    Should XON/XOFF flow control be used to the remote?

13.5. Kermit-65 Commands                                               Page 334

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

^ is up arrow    BLUE     RED     GRAY     ^    .(period) ENTER

------- ------- ------- ------- ------- -------

|       |       |delline|  up   |select | enter |

|   6   |   7   |   8   |   9   |   0   |   -   |

| gold  | help  |undelln|replace| reset |  sub  |

------- ------- ------- ------- ------- -------

v is down arrow     7       8       9       v

------- ------- ------- -------

| page  |findnxt|delword| down  |

|   Y   |   U   |   I   |   O   |

|command| find  |undelwd|section|

------- ------- ------- -------

4       5       6       ->

------- ------- ------- -------

|advanc|backup |delchar| right |

|  H   |   J   |   K   |   L   |

|bottom|  top  |undelch|specins|

------- ------- ------- -------

1       2       3      <-

------- ------- ------- -------

| word  |  eol  |  cut  | left  |

|   N   |   M   |   ,   |   .   |

|chgcase|deleol | paste |append |

------- ------- ------- -------

0

---------------------------------------

bline      |

space bar                         |

openline    |

---------------------------------------

Figure 13-3:   VT52 Keypad on an Apple Keyboard

-------------------------------------------------------------------------------

KEYBOARD                II+ or //e keyboard.

KEYPAD                  Does a gs-style keypad exist?

LOCAL-ECHO              Full or half duplex switch.

PARITY                  Character parity to use

PREFIX                  Which default prefix to use with PRODOS?

PRINTER                 Should the printer be used for the display?

RECEIVE                 Various parameters for receiving files

SEND                    Various parameters for sending files

SLOT                    Which slot # is communication device in?

13.5. Kermit-65 Commands                                               Page 335

-------------------------------------------------------------------------------

TIMER                   Is the receive timeout on or off?

TERMINAL                Which terminal (if any) should Kermit emulate?

The above options are analogous to the equivalent SET commands.

THE STATUS COMMAND

Syntax: STATUS

Give statistics about the most recent file transfer. This includes  information

such   as  number  of  characters  sent/received,  number  of  data  characters

sent/received, and last error encountered.

THE LOG COMMAND

Syntax: LOG filespec

When connected to a remote  site,  log  the  remote  session's  output  to  the

specified  file.    The file type and file warning protocols are observed. This

command is dependent upon the flow control (XON/XOFF) working.    Without  flow

control  there  is little possibility of getting a correct copy of the terminal

session.  The logging begins when you connect to the remote and  is  terminated

when  you escape back to the local Kermit with the ESCAPE character followed by

the "C" command.

13.6. Standard Installation

To bootstrap Kermit to the Apple, get the files APP3xx.1 thru APP3xx.2 on a DOS

3.3  diskette,  where  xx  is the current version.  Make sure the diskette is a

master diskette and empty.  Rename the files to ker3xx.1 thru ker3xx.2.   Then:

EXEC KER3xx.1,R25

You  will be asked several questions about your hardware and the program should

execute and produce a binary and kermit.init with some  starting  instructions.

If  you  want to run Kermit on PRODOS simply use the PRODOS conversion routines

to move the binary and kermit.init to PRODOS.  If you want other options  as  a

regular  thing then change file "kermit.init" with your changes. Then make sure

the file is on the same disk as the binary.

Files Supplied for Kermit-65

The following files should be supplied on the distribution tape:

APPAAA.HLP     List of files (like this one)

APP381.1       Easy install file 1 (an exec file for DOS 3.3)

APP381.2       Easy install file 2 (an exec file for DOS 3.3)

APPACC.HEX     Apple com card hex

APPACC.M65     Apple com card source

APPCCS.HEX     CCS 7710 com card hex

APPCCS.M65     CCS 7710 com card source

13.6. Standard Installation                                            Page 336

-------------------------------------------------------------------------------

APPCPS.HEX     CPS com card hex

APPCPS.M65     CPS com card source

APPGS.HEX      GS serial port hex

APPGS.M65      GS serial port source

APPHMM.HEX     Hayes micro modem card hex

APPHMM.M65     Hayes micro modem card source

APPLE.DOC      Complete documentation (it says here)

APPLE.MSS      Scribe text formatter source for documentation

APPMAI.HEX     Main kermit pgm hex

APPMAI.M65     Main kermit pgm source

APPMAK.UNX     Make file for UNIX cross assembly (to assemble Kermit)

APPMSV.HEX     Microtec com card hex

APPMSV.M65     Microtec com card source

APPSSC.HEX     Super serial com card hex

APPSSC.M65     Super serial com card source

APPXAS.1       65c02 cross assembler for UNIX system part 1

APPXAS.2       65c02 cross assembler for UNIX system part 2

APPXAS.3       65c02 cross assembler for UNIX system part 3

The syntax of the filenames may vary.  On UNIX systems, the filenames  will  be

in lowercase.  On VM/CMS systems, the period will be replaced by a space.

Alternate Installation

The  main  problem exists in getting the hex files onto your diskette as a text

file.  But again that is a test of your creativity. If you have  a  version  of

Kermit running then GET or RECIEVE the file as a text file and you are in busi-

ness.  Since Kermit has been separated into two assemblies then two  hex  files

will  have  to be present on the diskette. Get the main hex file APPMAI.HEX and

select which com card hex you will need. First "exec APPMAI.HEX".   Your  Apple

(or  compatable)  will  go  into  monitor and show you *'s for several minutes.

This is the monitor loading the hex into binary.  If you  get  beeps  from  the

monitor  its  probably because you didnt get a good copy of the text file.  Now

EXEC the com card driver you are going to use.  You will have to get back  into

basic(aha  another  test  for you,try "3d0G") to do this.  And you will see the

monitor loading the com driver. The order of EXEC's is important.  The com card

should   be   loaded  last.    Next  get  back  into  basic  and  do  a  "bsave

kermit,A$1000,L$6e00".  You may have to specify the drive  to  do  this  binary

save, with a slot or drive on the end of the BSAVE (aha another test).  You now

run Kermit via "brun kermit".

If you want to customize Kermit for your equipment, the recommended  method  is

to use file "kermit.init" or do all your SETs, etc, and then do an "exit".  Now

you should be back in BASIC.  At this point do a "bsave name,A$1000,L$6e00" and

when  you  do  a  "brun name" all your setups will be remembered.  NOTE: If you

save your current settings via "bsave kermit ..." you may find that moving that

binary  to  another type of Apple (e.g. from a //e to an //e+) will not be pos-

sible.  So make sure you keep the  original  binary  to  move  between  machine

types.

Since  the org is now $1000 if you have been using Kermit and then went back to

basic for some trivial thing a "CALL  4096"  should  start  up  Kermit  without

having to reload it.

In summary:

13.6. Standard Installation                                            Page 337

-------------------------------------------------------------------------------

1. EXEC APPMAI.HEX

2. Choose the com card driver you will use.  For example APPSSC.HEX.

3. 3D0G

4. EXEC APPSSC.HEX

5. BSAVE kermit,A$1000,L$6e00

And you should be in business.  Remember there is the command HELP and whenever

you are into a command a "?" will give you the  posible  options  available  at

that  point of a command.  The escape key will finish typing an option if it is

possible. The syntax of all the  commands  and  options  only  requires  enough

characters to make that command or option unique.

13.7. Problems

Installation

NOTE:  When  using  the  super  serial driver you must have the cards sw6-2 on.

This allows the card to use interupts. The rest of the switches  are  set  from

within  Kermit.  It  appears that you can run your Apple 2 with sw6-2 on and in

99% of the cases will cause no problems.  This is because DOS runs with  inter-

upts  locked  out  ("sei" in assembly language) and the program must explicitly

give a "cli" for interupts to work (the super serial driver does).

The Microtek driver is a super serial look alike which does not run with inter-

upts.    If you have trouble with the super serial driver you might try the MSV

driver.  For you people with the MSV-622c card, you might try running a  jumper

from  the UART 6551 pin 26 to the card edge pin 30.  This will enable interupts

just like the SSC sw2-6, and then you can use the super serial driver.

The Prometheus card will work with the Apple com driver.  However you will have

to  set  the switches on the card for baud etc.  Evidently this card can not be

programed by the software. If that is not true then here is an  oportunity  for

you  to write a better driver. If you do please pass it on for other Prometheus

users.

Unconfirmed reports have it that the Apple Cat will work  with  the  Apple  com

driver.  Would appreciate a confirmation.

Some  have  noted  the Apple com card must be initialized via the "IN#x" before

starting Kermit. Ike has now updated this driver and the initialization is  now

done within the Apple com driver.  Thanks Ike.

Usage

There  is the command HELP and whenever you are into a command a "?"  will give

you the posible options available at that point of a command.  The  escape  key

will  finish typing an option if it is possible. The syntax of all the commands

and options only requires enough characters to  make  that  command  or  option

unique.

13.7. Problems                                                         Page 338

-------------------------------------------------------------------------------

When  using flow control you may appear to hang. Type a ^Q (Control-Q) and that

may free you up.

Remember when you use your printer there are a lot of variables here.  What was

being  sent  to  the  screen  now  is  being sent to your printer.  If you were

emulating the VT52 your printer may not know  how  to  handle  the  escape  se-

quences,  tabs  etc. It may be you can tell the host you are a tty or some such

device that will give carrage returns etc that your printer can handle.    Some

printers may require the flow control and delay to get readable printing.

File  transfer  is  very  dependent  upon  parity. Make sure the host and local

parity are the same.

13.8. Customizing Kermit-65

The source code to Kermit-65 is in 6502 Assembler. It has been formatted for  a

cross  assembler  which  runs on a unix 2's complement machine.  Files appxas.1

thru appxas.3 are the cross assembler for UNIX.  Get the files on a UNIX system

and  then  look  at the documentation at the start. They will easily make you a

xasm for Kermit.  The file appmak.unx is the makefile to use with the  xasm  to

reassemble all of Kermit's parts.

Kermit-65 3.x has been separated into two assemblies, the main routines and the

com card routines for the devices shown in Table 13-1.  A vector has  been  set

up in low memory for the two assemblies to communicate. Look at the working com

drivers for tips on how to incorporate your version of  the  com  driver.  some

things  to note: It is probably best to buffer the input from the remote and to

get input characters from the remote every chance you get.  Note  the  Microtek

SV-622  driver, whenever the input is checked for a character and has a charac-

ter the character is put into the buffer immeadiatly.  Also when the output  is

checked  for  ready  to  output,  if the card is not ready to output then it is

checked for a character to input.  All this should help prevent losing  charac-

ters.

Communications card vector area:

address size  module   function

1003    byte   main     This is the baud rate index as follows:

3 - 110

4 - 135.4

5 - 150

6 - 300

7 - 600

8 - 1200

9 - 1800

10 - 2400

11 - 3600

12 - 4800

13 - 7200

14 - 9600

15 - 19200

for example:

if index is 6 then line should be 300 baud

1004    byte            unused

1005    word   driver   Address of a null terminated string.

13.8. Customizing Kermit-65                                            Page 339

-------------------------------------------------------------------------------

address should point to a capitalized

string of the drivers id

1007    byte   main     Com slot in the form $n0 where n is slot #.

1008    byte   main     Force initialization flag when 0.

init routine should always initialize when

this flag is 0 & then set flag non-zero.

1009    word   main     Address of the end of Kermit main routine.

100b    byte   main     Flow control is on when high bit is set.

100c    word   driver   Address of the end of the com driver.

1020  3 bytes  driver   Jump to initialization routine.

1023  3 bytes  driver   Jump to command routine.  A reg has command

0 - hang up the line

$0b - set baud rate

$0c - set break on the line

$91 - do xon on the line

$93 - do xoff on the line

routine returns false (P reg zero flag)

if unable to do command.

1026  3 bytes  driver   Jump to check for input from the line.

routine returns false (P reg zero flag)

if no character on line

1029  3 bytes  driver   Jump to get input character from line.

routine returns character in A reg

102c  3 bytes  driver   Jump to put character in A reg on line.

102f  3 bytes  driver   Jump to reset com driver.

1040  3 bytes  main     Jump to Apple ROM wait rtn. microsecs delay

=1/2(26+27A+5A*A) where A is the accumulator

1043  3 bytes  main     Jump to routine to print null-terminated string.

X reg contains least significant byte of address

Y reg contains most significant byte of address

routine does not issue a carriage return.

1046  3 bytes  main     Jump to routine to read the keyboard.

A reg contains the character read

1049  3 bytes  main     Jump to routine to print carrage rtn & line feed.

104f  3 bytes  main     Jump to routine to set characters parity.

A reg contains the character before and after.

All  the routines should return with the "rts" instruction.  Routines which can

return a true/false indication should return with the P reg zero flag  set  ap-

propiately.  That is: a "beq" instruction will branch on a false indication and

the "bne" will branch on a true indication.  The com driver  should  start  its

routines  above  the  main routines and tell where the end of the com driver is

via location $100c.  If your com driver gets too large then the  bsave  address

would have to be changed when you are saving the binary to diskette.

14. MACINTOSH KERMIT                                                   Page 340

-------------------------------------------------------------------------------

14. MACINTOSH KERMIT

Program:        Bill   Catchings,   Bill   Schilit,  Frank  da  Cruz  (Columbia

University),

Davide Cervone (University of Rochester),

Matthias Aebi (ECOFIN Research and Consulting, Ltd., Zuerich),

Paul Placeway (Ohio State University).

Language:       C (MPW)

Documentation:  Christine Gianone, Frank da Cruz, Paul Placeway

Version:        0.9(40)

Date:           May 26, 1988

MacKermit Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes (server mode only)

Login scripts:                     No

Transfer text files:               Yes

Transfer binary files:             Yes

MacBinary transfers:               No

Wildcard send:                     Yes (whole HFS folders)

File transfer interruption:        Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                Yes (VT100,VT102)

Communication settings:            Yes (Speed, Parity, Echo)

XON/XOFF:                          Yes

Transmit BREAK:                    Yes

IBM mainframe communication:       Yes

Transaction logging:               Yes

Session logging:                   Yes

Debug logging:                     No

Packet logging:                    No

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         Yes

Local file management:             Yes

Command/Init files:                No

Key redefinition/macros:           Yes

File attributes packets:           No

Command macros:                    No

Raw file transmit:                 No

Long packets:                      Yes

Sliding windows:                   No

14.1. Introduction                                                     Page 341

-------------------------------------------------------------------------------

14.1. Introduction

Macintosh Kermit, or "MacKermit", is  an  implementation  of  the  Kermit  file

transfer protocol for the Apple Macintosh family of computers. It was developed

at Columbia University, based on C-Kermit (which also forms the nucleus of Unix

Kermit  and  many other Kermit programs).  Version 0.9 of MacKermit runs on the

Macintosh 512, XL (Apple Lisa running MacWorks), 512e, Plus, SE, and II,  under

the regular Finder and the Multifinder, with which it can transfer files in the

background.  MacKermit 0.9 probably does not run on a 128k (original,  classic)

Macintosh,  due  to lack of sufficient memory, but should run OK on a "fat Mac"

(a 128K Mac upgraded to 512K).  Version 0.8 should be retained for 128K Macs.

This manual assumes you are acquainted with your Macintosh, and  that  you  are

familiar with the general ideas of data communication and Kermit file transfer.

A very brief overview is given here, but for details consult the early chapters

of  the  Kermit  User  Guide (of which this document is a chapter), or the book

Kermit, A File Transfer Protocol, by Frank da Cruz, Digital Press (1987).   For

further  information  about  Kermit  documentation,  updates,  lists of current

available versions, and ordering information, write to:

Kermit Distribution

Columbia University Center for Computing Activities

612 West 115th Street

New York, NY  10025  (USA)

14.2. Installation

Before you can use Macintosh Kermit or any other communication program on  your

Mac,  you must have a way to connect it to the other computers you wish to com-

municate with.  This means either a direct cable connection  (usually  using  a

"null  modem" cable), or a modem connected to your Mac and to a telephone.  The

Macintosh poses two special problems at this level.  First, its connectors  are

not the standard 25-pin RS-232 style, but either 9-pin or 8-pin special connec-

tors which you need special adapters for.    Second,  the  Macintosh  does  not

supply a Data Terminal Ready (DTR) signal, which is required by most modems be-

fore they will operate.  To use your Mac with a  modem  that  is  not  designed

specifically  for the Mac, you have to either (a) configure the modem to ignore

the DTR signal, or (b) feed some other active signal into the modem's  DTR  in-

put.  The former is usually done with DIP switches on the modem, the latter can

be done in the connector that plugs into the modem by installing a jumper  wire

between  DTR  (pin  20) and DSR (pin 6), or by connecting the Mac's +12V output

(pin 6 on the Mac's 9-pin connector) to DTR (pin 20) on the modem end.

If you have received a Macintosh  Kermit  diskette  from  Columbia  University,

there's no special software installation procedure -- just insert the diskette,

double-click on the appropriate start-up file, or on MacKermit itself, and  go.

If  all  the  communication  and  other  settings agree with your requirements,

there's nothing else you need to do.  This process is illustrated in  the  next

section, just below.

MacKermit is not copy-protected, and nothing out of the ordinary is required to

make copies onto other diskettes, or onto your hard disk if you have one.  Just

use the ordinary Macintosh methods of copying files, folders, etc.

Later, you may wish to create settings files tailored to your communication en-

14.2. Installation                                                     Page 342

-------------------------------------------------------------------------------

vironment, and you might also want to  customize  the  keyboard  configuration.

Use  the  various Settings options for this, and then select Save Settings from

the File menu.  Settings and settings files are explained in Sections 14.10 and

14.11.

14.3. Getting Started

Kermit programs perform two major functions, terminal emulation and file trans-

fer.  Before transferring files between two systems you must establish  a  ter-

minal  connection  from  your  system  to  the other one, either direct or else

dialed up using a modem.  Then to transfer files, login to the remote system if

necessary,  start  up  a  Kermit  program  there,  and then tell the two Kermit

programs which files to transfer, and in what direction.

Most Kermit programs present you with a prompt, in response to which you type a

command, repeating the process until you exit from the program.  If you want to

establish a terminal connection to another system, you must  give  the  CONNECT

command.    Unlike  these programs, MacKermit is always connected, and whatever

keystrokes you type are sent to the other system.  To give  commands  to  MacK-

ermit  itself, you must use the mouse to pull down menus from the menu bar that

overlays your terminal session, or type special Command-key equivalents.

The following example shows how to transfer a file with MacKermit.  The  remote

computer is a Unix system, but the method is the same with most others.

- First  insert the MacKermit diskette.  It should appear on the screen

as a diskette icon titled Kermit 0.9(40).  Click on it twice to  open

if it did not open automatically when you inserted it in the drive.

- Once  the  disk  is opened, you will see three MacKermit icons across

the top of the screen.  For the Unix system and most others  you  can

use  the  "Normal Settings" icon -- to start the Kermit program click

twice on it.  For linemode connections to IBM mainframes,  you  would

click twice on the "IBM Mainframe Linemode Settings" icon.

- You  will  see a white backround with menus stored under the headings

File, Edit, Settings, Remote, and Log.

- Move  the  mouse  pointer   to   the   Settings   menu   and   select

Communications... by clicking on it once.

- MacKermit  normally  sets  the  communication  speed to 9600 bits per

second.  Click on the circle in front of 1200 (or whatever speed  you

need  to  match  the  baud  rate of your modem and/or remote system).

Check to see that the other communication settings like parity are as

required, and make any necessary changes.

- Click on the "OK" box to accept the settings.

- If you have a Hayes-like dialout modem, follow the next two steps:

1. Type  AT  (uppercase)  and then press the Enter key.  The modem

should respond with "OK" or  the  digit  "0"  (zero).    If  it

doesn't,  check  the  cable, the modem, etc (consult your modem

manual for details).

14.3. Getting Started                                                  Page 343

-------------------------------------------------------------------------------

2. Now type ATDT 7654321 followed by Enter (replace 7654321 by the

actual phone number).  If the connection succeeds, you'll get a

message like CONNECT (or the digit "1"), otherwise  you'll  see

an error message like NO CARRIER, ERROR, etc, or a digit like 3

or 4 (see your modem manual).

For non-Hayes-compatible modems,  follow  the  instructions  in  your

modem manual.  For direct connections, skip this step altogether.

Now  you  should  be talking to the Unix system.  Type a carriage return to get

its attention, and it will issue its login prompt.  In the examples below,  un-

derlining is used to show the parts that you would type.

Login: christin          Login to the host.

password:                (Passwords normally don't echo.)

% kermit                 Run Kermit on the host.

C-Kermit>receive         Tell it to receive a file.

Now tell MacKermit what file to send:

- Use  the  mouse to point to the File menu and select the Send File...

option.  You can either type in the name of the file (if you know the

name)  or  select  the  alternate  drive to see what files are on the

disk.  Once you see the file you want to send, click on the  filename

and then click on the SEND option (or you can just click twice on the

filename).

- A "File Transfer Status" box will appear to report  the  progress  of

the  transer.    NOTE: If the number of retries is increasing but not

the number of packets, you should check your  Communications...  set-

tings under the Settings menu.

- When  the  file transfer is completed, the "File Transfer Status" box

should disappear and the C-Kermit prompt should reappear.

You have just transferred a file from the Macintosh to the  Unix  system.    To

transfer a file in the other direction, use the "send filename" command on Unix

instead of "receive", and click on "Receive File..."  from the Mac's File menu,

instead of "Send File...".

After  the  file  is  transferred,  your  terminal  connection is automatically

resumed.  Once your Unix session is complete, you can log out,  and  then  exit

from MacKermit:

C-Kermit>exit

% ^D                     Logout from Unix by typing Ctrl-D.

1. Select the Quit option in the File menu by clicking on it.

2. Select the Close option in the File menu by clicking on it (assuming

you want to close the current folder).

3. Select the Eject option in the File menu by clicking on it (assuming

14.3. Getting Started                                                  Page 344

-------------------------------------------------------------------------------

you ran Kermit from a diskette that you want to eject).

That's  the  easiest  and  quickest way to use Kermit.  If this simple scenario

does not work for you, look for any obvious incorrect settings (speed, parity),

fix them, and try again.  Otherwise, read on.

14.4. The Macintosh File System

The  Macintosh  file system consists of one or more disks, each disk containing

files.  There are actually two Macintosh file systems, which work slightly dif-

ferently.

Disks  formatted  for  the  older  Macintosh  File System (MFS) are essentially

"flat".  All files on one of these disks must have a unique name.  Files may be

collected together into "folders", but folders are not analogous to directories

on other file systems, and no two folders on the same disk may contain files of

the  same name; the folders exist only to make things look nicer in the Finder.

All Macintoshes have support for MFS.

Disks formatted with the newer Hierarchical File System (HFS) are  not  "flat";

each  folder is a directory.  There may not be more than one file with the same

name in a single folder, but there may be identically named files in  different

folders.

Macintosh  file names may contain practically any printable characters, includ-

ing space and punctuation -- but colon (":") may not be used;  it  is  used  in

device names and as the HFS path element separator.

14.5. Menus

The  major  menus  are Apple, File, Edit, Settings, Remote, and Log.  The Apple

menu gives some information about the program, including the MacKermit  version

number  and  the  C-Kermit  protocol mudule version number (useful in reporting

bugs).  It also shows statistics about the most recent file transfer.

The File menu invokes Kermit's file transfer functions, Send, Get, and Receive.

It  also  allows settings to be saved and restored, and like most Macintosh ap-

plications, includes a "quit" entry for leaving the program, and  a  "transfer"

entry for transferring to another program without going through the Finder.

The Edit menu provides support for Macintosh desk accessories that need to have

this menu to do cut and paste.  This menu does not  currently  do  anything  in

MacKermit.

The  Settings menu provides dialog boxes for file, communications, and protocol

settings; these will be discussed below.

The Remote menu has the commands that can be sent to Kermit servers, as well as

an option to turn Macintosh Kermit itself into a server (also discussed below).

The  Log  menu contains commands to start and stop session and transaction log-

ging.  It also has an entry to dump the current screen  image  to  the  session

log, which is only enabled when the session log is open.

14.6. Terminal Emulation                                               Page 345

-------------------------------------------------------------------------------

14.6. Terminal Emulation

Before you can transfer files, you have to establish a terminal connection with

the other computer.  You don't have to give MacKermit any special command to do

this,  just  start  the program.  Assuming you have a physical connection, then

the software will use it.  If you think you have  a  physical  connection,  but

don't  see any results, click on the Settings menu and select Communications to

make sure you have the right speed and parity.  If you have to dial out to make

the  connection,  you  must do this yourself -- Mac Kermit won't do it for you.

Depending on the type of modem, you must either type  dialing  commands  to  it

directly  (like the Hayes ATDT command in the example in section 14.3), or else

dial the phone manually, wait for an answer and a carrier tone,  and  then  put

the modem in data mode.

Once  you've  made  the  connection, you can use MacKermit's terminal emulator,

which conforms to ANSI standard X3.64, providing a subset of  the  features  of

the DEC VT102 terminal (a VT100 with line and character insert and delete func-

tions added).  The functions provided are sufficient to allow MacKermit to  act

as a terminal for the EMACS full-screen editor as it exists on most timesharing

systems, and for most host-resident display-oriented applications  that  expect

to  do cursor positioning and editing on the VT100 or VT102 screen, such as VAX

TPU.  MacKermit does not currently support the following VT100/102 functions:

- Double height or double width lines

- Blinking

- 132 columns

- DEC-style line wrapping

- Control characters embedded in escape sequences

- VT52 mode

(this is not an exhaustive list)

The keyboard is set up by default as follows: If your Macintosh has  a  Control

key  (ie.  an  SE or II), Kermit uses it, and the Command (Fan, Cloverleaf) key

can be used for keyboard equivalents for menus.  If your Mac does  not  have  a

Control  key,  then  the Command key is used as the Control key.  The CAPS LOCK

key forces all alphabetic characters to upper  case.    The  terminal  emulator

sends  ESC  (escape) when the "`" (accent grave) key is pressed unshifted (even

if your keyboard has an ESC key).  The character "`" can be sent by typing Con-

trol  (or Command) and the same key.  The Backspace key sends a Delete (Rubout)

and Control-Backspace sends a Backspace.  On the original  Mac  keyboards,  the

main  keypad Enter key sends a "short" (250ms) BREAK signal.  The Mac+, Mac SE,

and Mac II do not have a main keypad Enter key, so the BREAK function  must  be

reassigned to another key.

You  can modify the keyboard layout any way you like, defining keyboard macros,

defining or moving  the  Control  and  Escape  keys,  etc.,  using  MacKermit's

built-in  key  configuration  features.    Older  MacKermits  (version  0.8 and

earlier) came with a separate key configuration program called  CKMKEY.    This

should  not  be  used, because it does not understand the format of the 0.9 and

later keyboard configuration software.

MacKermit includes a mouse-controlled cursor positioning feature for use during

terminal emulation.  If the "Mouse -> Arrow Keys" feature is turned on (via the

Terminal entry of the Settings menu), then when the mouse  button  is  pressed,

the  program  acts as if you typed the VT100 keypad arrow keys to move the ter-

14.6. Terminal Emulation                                               Page 346

-------------------------------------------------------------------------------

minal cursor to where the mouse cursor is.  MacKermit does this by sending  the

absolute strings for arrow keys, independant of what is bound to the actual ar-

row keys of the keyboard.

MacKermit sets the Mac hardware to do 8-bit data communication with no  parity,

and then correctly sets the parity bit of each character itself in software, as

requested in the Communication settings menu.  This has the benefit of avoiding

the  problem of a machine which requires a different input parity than it sends

back.  MacKermit will correctly receive all of the characters sent  to  it,  no

matter which parity they are.

To allow useful coexistence of desk accessories and Kermit, the terminal emula-

tion window may be dragged using the drag bar.  If a  desk  accessory  overlays

the  emulation  window,  the emulation window can be clicked upon to move it in

front of the DA, and later dragged to reveal the hidden desk accessory so  that

it can be restored to the foreground.  The same thing can be done with Kermit's

own remote response window as well.  Note that Kermit's terminal emulation win-

dow does not accept input when any other window is in the foreground.

MacKermit  uses XON/XOFF (control-Q and control-S) flow control during terminal

emulation and file transfer.  If the other system does not understand XON/XOFF,

problems may result at high speeds.  The terminal emulator can normally keep up

at 9600 baud, and has a very large input buffer, but after  several  continuous

scrolling  screens at this speed, some characters may be lost.  When running at

high baud rates on a system that does not understand XON/XOFF flow control, ei-

ther  keep  your  terminal in page mode, use a text paging program such as Unix

"more", or view text with a non-scrolling screen editor.  Also, don't drag  the

terminal emulation window while characters are arriving; if you do, the charac-

ters may be lost and the display may become confused.

During terminal emulation, the characters displayed on the screen may  also  be

saved on the disk.  This allows you to record interactions with the remote sys-

tem, or to "capture" files that you can't transfer with  Kermit  protocol,  for

example  when  the  remote  system does not have a Kermit program.  Use the Log

menu, and choose session logging to activate this feature.    The  result  goes

into  a file called "Kermit Session" in the current folder, which is always ap-

pended to, rather than overwritten.  To create a new  session  log,  delete  or

rename the old one first.

The  following  features  are missing from the MacKermit terminal emulator, and

may be added in subsequent releases:

- Restoration of character attributes such as underlining or highlight-

ing.

- Cutting text from screen to clipboard.

- Transmission of raw text to host (e.g. pasting to screen).

- Screen rollback.

- Screen resizing.

- Explicit modem or dialer control.

- Login scripts.

- Printer support.

- Ability to use the printer port for terminal emulation.

- A  way  to  disable  XON/XOFF flow control, or select other flow con-

trols.

14.7. File Transfer                                                    Page 347

-------------------------------------------------------------------------------

14.7. File Transfer

Like most Kermit programs, MacKermit allows you to send and receive text or bi-

nary  files singly or in groups.  It will interact with a remote Kermit server,

and it can act as a server itself.  However, due to the unique  nature  of  the

Macintosh file system, there are some special considerations:

- Mode - Text or Binary.  Binary means the data is sent or stored with-

out modification.  Text means that every  carriage  return  character

(CR)  in a Macintosh file is translated to a carriage-return-linefeed

(CRLF) sequence when sending, and every CRLF in an incoming  file  is

turned  into  a  CR  when  stored  on  the  Mac disk.  A text file is

produced when you save a file from  MacWrite  or  other  applications

using  the "text only" option; text files are not associated with any

particular Macintosh application and can be sent in a useful  fashion

to other kinds of computers.

A  word of caution about Macintosh text files: The Macintosh supports

an extended version of ASCII, with characters like accented  and  um-

lauted vowels in the 128-255 range.  These characters allow represen-

tation of Roman-based languages other than English, but they  do  not

follow any of the ISO standards for extended character sets, and thus

are only useful on a Mac.  When transferring text files,  you  should

ensure  that  either there are no extended characters in the file, or

that the other system can understand the Mac's 8-bit characters.

- Fork - Data or Resource.  Macintosh files may have two "forks".   The

data  fork  contains  data for an application; the resource fork con-

tains icons, strings, dialog boxes, and so forth.   For  instance,  a

MacWrite  document  contains  text  and formatting information in the

data fork, and fonts in the resource fork.  For applications, the ex-

ecutable code is stored in the resource fork.

File  transfer  is  initiated when you select Send file..., Receive File..., or

Get file from server... from MacKermit's File menu.

File transfers can be canceled by clicking on the Cancel File or  Cancel  Group

buttons.   These will always work when sending.  When receiving, they will work

if the opposite Kermit honors this (optional) feature of the protocol.    There

is  also an "emergency exit" from any protocol operation, which can be taken at

any time by typing  "Command-."  --  that  is,  hold  down  the  Command  (Fan,

Cloverleaf) key and type period.

The  progress  of  file transfer operations can be logged into a Macintosh file

called a "transaction log".  This log will show the names of the  files  trans-

ferred,  the  date and time, and the completion status.  This feature is useful

with long unattended transfers -- you can come back later and read  the  trans-

action  log  to  find out what happened.  The transaction log is called "Kermit

Log".

The current version of Mac Kermit can only send one fork of a file at  a  time.

When  a  file  has  two  forks,  there  is  no provision for sending both forks

together.  This restriction may be lifted in future releases of MacKermit,  for

example by converting applications to MacBinary format during transmission.

14.7.1. Sending Files                                                  Page 348

-------------------------------------------------------------------------------

14.7.1. Sending Files

To  send files, first put the remote Kermit in server mode, or else give it the

RECEIVE command.  Then use the mouse to select Send file... from the File menu.

This  will  give  you  a  MacKermit  file-open box, which includes the standard

Macintosh dialog items -- a file list, Disk and Eject buttons, etc.    You  can

either  send  one  file at a time, by clicking on its name in the file list, or

send the entire contents of the current HFS folder  (for  HFS  disks  only,  of

course).   Clicking the Disk button will switch the file list to another physi-

cal disk.  If desired, you can type an alternate name to send the  file  under.

When  you select a file, MacKermit examines its type; if the type is APPL, then

MacKermit expects to send the resource fork in binary mode, otherwise the  data

fork in text mode.  The Mode and Fork radio buttons will display these choices;

you may change them before clicking the Send button.

14.7.2. Receiving Files

You can receive or get multiple files, providing the opposite Kermit is capable

of  sending  multiple  files  in  a  single transaction (most are).  To receive

files, first give the remote Kermit a SEND  command  and  then  select  Receive

file...  from  the File menu.  To get files from a server, first put the remote

Kermit into server mode, then select the Get file from  server...  option  from

the  File menu, and type in the name of the file you want to get, or a wildcard

designator for multiple files, in the remote system's filename syntax.

As each file arrives at the Mac, it will be decoded according  to  the  current

mode  (text or binary), and stored in the default fork (data or resource).  The

file names will be either the names the files arrive with (overwriting existing

files  of  the same names) or new unique names (when name conflicts occur), ac-

cording to the current default for name collisions.  You may also elect to per-

form  an  "attended" receive, in which you have an opportunity to override file

defaults on a per-file basis (do this in the Protocol section of  the  Settings

menu).    But  attended  operation must be used with caution -- if you take too

long (more than about a minute) to execute an incoming file's dialog  box,  the

opposite Kermit could time out and terminate the transaction.  If this happens,

tell the opposite Kermit to send again and try again with the receive dialog.

The folder for new files is the same as the location of the settings  file,  or

if  no settings file was used then the new files appear on the desktop.  If you

are transferring a lot of files and  want  to  keep  them  together,  create  a

folder,  drag the settings file into it, and double click on the settings file;

all created files will appear in that folder.

14.8. Remote Commands

When connected to a Kermit server, MacKermit is capable of issuing special file

management  and other commands to it.  The Remote menu contains these commands.

You may request directory listings, you can delete files,  change  directories,

etc,  on  server's  machine.  The response from these commands (if any) is dis-

played in a special pop-up window.  Responses to multiple Remote  commands  are

separated  by  a  dashed line.  The response window can be scrolled, sized, and

positioned, and can be hidden by clicking the menu item "Hide Response" or  the

window's  go-away box; all text remains intact and will be appended to the next

time you do a Remote command; it can also  be  brought  to  the  foreground  by

14.8. Remote Commands                                                  Page 349

-------------------------------------------------------------------------------

clicking  the  Show  Response  menu  item.    Note  that typein to the terminal

emulator will not take effect when the response window -- or any  other  window

(such as a desk accessory) -- is up front.  This is not a bug, but a feature of

the Macintosh user interface guidelines.

If the response buffer gets too full (greater than  30,000  characters),  MacK-

ermit  will  remove  enough  text from the beginning of the buffer, in 512 byte

chunks, to make it less than 30,000 characters again.

A Remote command can be canceled by taking the Emergency Exit (Command-.).   To

disengage  from  the remote Kermit server, click on Bye or Finish in the Remote

menu.

14.9. Server Operation

MacKermit may itself act as a Kermit server.  Just set the  desired  parameters

in  the Settings menu, then click on Be a Server in the Remote menu.  The MacK-

ermit server can respond to SEND, GET, REMOTE DIRECTORY, FINISH, and  BYE  com-

mands.    You  can send single or multiple files to a MacKermit server, and you

can get a single file from it by name.  You can also get all the files  in  the

current folder by using a colon (":") as the file specification in the GET com-

mand:

GET :

If you give the FINISH command, MacKermit will return to terminal mode.  If you

give the BYE command, the Macintosh will reboot itself.

You  can  take MacKermit out of server mode from the Mac keyboard by typing the

emergency exit sequence, Command-dot.

14.10. Settings

You can change File, Communications, Protocol, Terminal, Keyboard  macros,  and

Keyboard  modifier settings by using the Settings pull-down menu.  You can save

and load these settings by invoking the appropriate selection in the File menu.

If  the  "bundle  bit"  has been correctly set on your version of MacKermit (it

should be), then you can double-click on the resulting document to start  MacK-

ermit with those settings.

The File settings establish the defaults for file transfer:

- Attended versus Unattended operation for incoming files.

- Naming:  When doing unattended file reception, whether incoming files

should supersede existing files of the same name,  or  a  new  unique

name  should  be  assigned  to  them.  If the latter, the new name is

formed by adding a dot and a number to the end.  For instance,  if  a

file  called FOO exists and a file called FOO arrives, MacKermit will

store the arriving file as FOO.1; if FOO.1 exists, then FOO.2, etc.

- Mode: text or binary.  Used for received files only.   When  sending,

MacKermit  tries to figure out an appropriate mode for the file being

sent (but then lets you override it the Send File dialog).

14.10. Settings                                                        Page 350

-------------------------------------------------------------------------------

- Fork: which fork -- data or resource -- to send, or to store  an  in-

coming file into.

The  Communications  settings  allow you to set the baud rate (anywhere between

300 baud and 57.6K baud, except 38.4K  baud),  and  parity  (odd,  even,  mark,

space,  or  none).    When  the  parity  is  set  to none the Macintosh uses an

8-bit-wide connection.  All other parity settings tell the Macintosh to  use  a

7-bit-wide connection, and to request 8th-bit prefixing when transferring 8-bit

data.  If the remote host or the communication path uses any  kind  of  parity,

then you won't be able to transfer files successfully unless you tell MacKermit

(and in most cases also the Kermit on the other  end)  about  it.    Duplex  is

selected in the Terminal settings.

The  Protocol settings allow you to set packet parameters for both incoming and

outbound packets.  These include the block check type (1 or 2 character  check-

sum,  3-character  16-bit  CRC-CCITT), line turnaround handshake character (for

file transfer with half duplex systems), packet start and end characters,  pad-

ding,  packet  length,  timeout  interval,  and  packet length.  Characters are

specified by entering their ASCII value in decimal, e.g. 1  for  Control-A,  13

for  Control-M  (Carriage Return), etc.  The RECEIVE parameters are conveyed by

MacKermit  to  the  other   Kermit.      For   instance,   if   you   set   the

receive-packet-length  to  500,  MacKermit  will  tell the other Kermit to send

500-character packets.  The SEND parameters are  used  to  override  negotiated

values, and need rarely be used.

Long packets are selected by setting the RECEIVING packet length between 95 and

1000.  Normally, you should not change the sending  length  because  MacKermit,

and  most  other  Kermits, will configure themselves correctly.  Note also that

the fastest file transfers will happen  with  long  packets  in  the  range  of

300-500.    Very  long  packets  actually end up being much slower, because the

operating systems in both the Mac and the other machine have to do more work to

cope  with  such  long  inputs, and, under noisy conditions, the probability is

higher that a longer packet will be struck by noise, and will  take  longer  to

retransmit.

The Terminal settings let you modify the characteristics of the VT102 emulator,

such as auto-linefeed, autowrap, autorepeat keys, block  vs  underline  cursor,

blinking  vs steady cursor, inverted screen (reverse video), and smooth scroll-

ing.  There is also a "visible bell" for those who can't hear the audible  bell

produced  upon receipt of a Control-G, and an option to display control charac-

ters visibly by showing their numeric ASCII values (in  decimal)  in  a  single

character  cell.   If local echo is needed, as in half-duplex connections, that

must be specified here also.

14.11. Settings Files

You can start MacKermit with all its "factory settings" by double  clicking  on

the  MacKermit  icon.    Factory settings are designed for direct communication

with most other microcomputers, DEC minis and mainframes,  etc:  9600  bps,  no

parity,  XON/XOFF,  remote  echo,  etc.    You  can  change  the communication,

protocol, file, keyboard, and terminal settings by going through the options in

the  Settings  menu.  Once you have set all parameters as desired, you can save

your settings in a "MacKermit settings file"  by  selected  "Save  Settings..."

from the File menu.  A settings file is, in Macintosh terminology, a "MacKermit

document".  You'll recognize it because it looks  like  a  dog-eared  piece  of

14.11. Settings Files                                                  Page 351

-------------------------------------------------------------------------------

paper  with  the  MacKermit icon superimposed.  You can have more than one set-

tings file.

There are two ways to use a settings file.  First, you can double-click on  it,

just  as  you  can  double-click on a MacWrite document to start up MacWrite to

edit a particular file.  This method starts up MacKermit  with  all  the  saved

settings.  The other method is to click on the "Load Settings..." option in the

File menu from inside MacKermit.  This lets you change settings without leaving

and  restarting  the  program.   Load Settings...  shows all MacKermit settings

files in the selected folder.  Opening one of  them  loads  all  its  settings,

removing all current settings.

You  can "edit" a MacKermit settings file by loading it, going through the Set-

tings menu, and then saving the settings either in a new file,  or  overwriting

the same file.

As  distributed  by Columbia, Mac Kermit comes with two settings files.  One is

called "Normal Settings", and is pretty much identical to Mac Kermit's  factory

settings.    The  other  is "IBM Mainframe Linemode Settings".  It selects mark

parity, local echo, XON half-duplex line turnaround handshake.    You  can  use

these  files as-is, customize them for your own environment, or create new set-

tings files for all the different kinds of systems that you use.

14.12. Reconfiguring the Keyboard

Beginning with version 0.9,  MacKermit  has  keyboard  configuration  functions

built  in.  These are accessed through the Set Key Macros and the Set Modifiers

entries in the Settings menu.

The Macintosh keyboard is composed of normal keys and modifier keys.   Modifier

keys are those keys that, when held down, change the meaning of other keys.  On

the Mac these are: SHIFT, CAPS LOCK, OPTION, CONTROL (only on the  Mac  II  and

SE),  and  COMMAND  (also known as APPLE, CLOVER, or FAN).  Normal keys are the

letters, numbers, special symbols, arrow keys, space bar,  and  function  keys.

Only  one  normal key can be typed at a time, but one or more modifier keys can

be pressed down along with it.

When  you  type  a  key,  Kermit  reads  both  the   ASCII   value,   and   the

keyboard-independent  scan code for that key.  Kermit looks in its table of key

macros to see if there is a macro for this combination of  key  and  modifiers,

and  if  so  sends  the  macro.  If there is no macro, Kermit then looks in its

modifier table to see if any of the modifiers do special things to the  charac-

ter;  if so, it does these to the character.  Finally, Kermit sends the charac-

ter.  In the normal case when there is no macro and  no  modifiers  apply,  the

character sent is simply the ASCII value for that character.

It  is  important to keep in mind that if the parity setting is something other

than none, the high (8th) bit will be stripped off of the characters when  they

are  transmitted.  Since most systems do not understand characters in the range

128 -- 255 (decimal), you should avoid  using  the  Apple  extended  characters

(accented vowels, for example) during terminal connection.

14.12.1. Defining Key Macros                                           Page 352

-------------------------------------------------------------------------------

14.12.1. Defining Key Macros

To  define  a new key macro, select the Key Macros entry.  A dialog window will

appear, asking you to press the key to define.  Type the key (including any  of

the  modifiers).   A new dialog will appear, with an editable text field in it.

Enter the definition for the key here.   Your  definition  may  be  up  to  255

characters long, and can include all of the control characters (including NUL).

Special characters can be included in the macro by entering a "\"  (backslash),

followed  by  up  to  3 octal (base 8) digits for the value (just like in the C

programming language).  For example, an ASCII NUL (value 0) would be written as

"\000",  carriage  return  (ASCII 13) would be written "\015" (1 x 8 + 5 = 13).

Also, control characters may be entered with a backslash, followed by  a  caret

(or  circumflex,  "^"), followed by the corresponding letter.  Thus a Control-G

(value 7) could be entered as "\007", "\^G", or "\^g".  To  include  a  literal

backslash in a definition, type in two of them: "\\".

BREAK  conditions  are  also  programmable  as macros.  If the entire macro the

string is "\break", then typing the defined key will send a short (1/4  second)

break.    A  long (3.5 second) BREAK is defined with "\longbreak".  Note that a

macro can define either a BREAK, or a string  of  normal  characters,  but  not

both.

14.12.2. Defining Key Modifiers

Skip  ahead  to  the  next section if you already know about things like SHIFT,

CAPS LOCK, CONTROL, and META.

On a typewriter the only modifier key is SHIFT.  Typing  a  character  with  no

modifier  key  depressed selects a lowercase letter or the character printed on

the lower face of the keytop (say, the digit "4").   Typing  a  character  with

SHIFT depressed selects an uppercase letter or the character printed on the up-

per face of the keytop (say, a dollar sign).  Some keyboards also have a  SHIFT

LOCK key, which stays down once pressed and pops up the next time it's pressed;

its operation is equivalent to holding down SHIFT.  And some keyboards  have  a

CAPS LOCK key which operates like SHIFT LOCK, but only upon letters.

Computer  terminals  also  have  a  modifier key called CONTROL (or CTRL).  Its

function is a little less obvious: it is intended to  produce  one  of  the  33

characters  in  the  "control range" of the ASCII alphabet.  Control characters

are not graphic -- they are intended for use as format effectors (like carriage

return, formfeed, tab, backspace), for transmission control, or for device con-

trol.  The remaining 95 characters -- letters, digits, punctuation,  and  space

--  are  the  graphic  characters.   When a character is typed with the CONTROL

modifier pressed, its "control equivalent" (if any) is transmitted.  By conven-

tion,  the control equivalent of A is Control-A, B is Control-B, etc, and there

are also seven special control characters generally associated with punctuation

characters  or special keys.  For the "alphabetic" control characters Control-A

through Control-Z, SHIFT or CAPS LOCK modifiers are ignored;  for  the  others,

operation varies from terminal to terminal.

The  SHIFT and CONTROL modifiers allow all 128 ASCII characters to be sent from

a normal typewriter-like keyboard that has about 50  keys.    However,  certain

host-resident  computer  applications  --  notably  the full screen text editor

EMACS and its descendents -- can be  used  to  greater  advantage  with  a  256

character  8-bit alphabet (EMACS responds to single-character commands, and the

14.12.2. Defining Key Modifiers                                        Page 353

-------------------------------------------------------------------------------

more characters a terminal can send, the more commands are directly available).

For this purpose, some terminals also provide a META modifier key.    This  key

simply  causes  the high-order ("8th") bit of the selected 7-bit ASCII value to

be set to 1 upon transmission.  This can  only  work  when  the  connection  is

8-data-bits-no-parity.    When parity is in use, EMACS allows a sequence of two

7-bit ASCII characters to represent a single meta character.  The advantage  of

having  a  real  META modifier key is that it can be held down while the actual

key is struck repeatedly or even autorepeats, whereas a use of a "meta  prefix"

such  as  <escape> requires much more typing.  To illustrate, suppose META-F is

the command to go forward one word.  If you  want  to  execute  this  operation

repeatedly, just hold down META and F and let it autorepeat.  If you don't have

a META key, then you'd have to type <escape>F<escape>F<escape>F..., etc.

A common problem faced by computer users who switch from one terminal or PC  to

another  is  the  placement of the modifiers and other special keys.  DEC, IBM,

Apple, and other manufacturers consistently  move  these  keys  around  on  new

models of their keyboards.  MacKermit allows you to assign any of various func-

tions to any of the Mac's modifier keys, and to assign any desired character or

character  sequence  to  the regular keys, so that you can tailor the layout of

your Mac's keyboard to suit your taste.

14.12.3. Modifiers Dialog

To change the action of any of the modifier keys,  select  Modifiers  from  the

Settings  menu.  A dialog will appear that looks roughly like the one in Figure

14-1 (the "%" represents the Apple or Clover key).

-------------------------------------------------------------------------------

Modifier Pattern:  -->   Modification:

Ctrl Opt Lock Shift  %      Unmodify Caps Ctrl Meta Prefix string:

____________

[X]  [ ] [ ]  [ ]   [ ]   |   [ ]    [ ]  [X]  [ ]  [____________]

[ ]  [ ] [ ]  [ ]   [X]   |   [ ]    [ ]  [X]  [ ]  [____________]

[ ]  [X] [ ]  [ ]   [ ]   |   [x]    [ ]  [ ]  [ ]  [\033        ]

[ ]  [ ] [ ]  [ ]   [ ]   |   [ ]    [ ]  [ ]  [ ]  [____________]

(Cancel)   (Help)   ( OK )

Figure 14-1:   MacKermit Key Modifier Dialog

-------------------------------------------------------------------------------

The check boxes are divided into rows, each one describing a modification.  The

left half of each row describes the modifier combination to look for; a checked

box means that this key is down, and an unchecked box means "don't care".  Note

that  there is no way to specify a key being up, and lines with nothing checked

on the left side will be ignored; the character will be modified in the  normal

Macintosh way.

The  right  half  describes what modification to do to the characters.  The Un-

14.12.3. Modifiers Dialog                                              Page 354

-------------------------------------------------------------------------------

modify modification says "make this the character that would be sent  from  the

same  key  with  no modifer keys pressed".  In other words, un-Option, un-Caps,

un-Control, and un-Shift this character.  The Caps modification translates  all

letters  to upper case, Ctrl makes the letter a contol character, Meta sets the

high (8th) bit on the character, and if a Prefix string is present, it is  sent

before the character is.

Hints about modifiers:

- Beware of the Option key.  It changes the value of any characters you

use with it.  If you type Option-F, the Mac will send  a  D,  if  you

type  Option-B, the Mac will send a ":", etc.  If you want to use the

option key as a modifier, be sure to check the "Unmodify" box.

- To use MacKermit with a version of EMACS that does not  accept  8-bit

Meta  characters, define a key, like Option, to be unmodified, with a

prefix string of \033 (ASCII Escape), as in Figure 14-1.    Then  you

can hold down Option and type F (or any other key) repeatedly, or let

it autorepeat, and MacKermit will send the  correct  prefix-Meta  se-

quence.

- When  interpreting a keystoke, MacKermit checks the list of modifiers

from top to bottom, applying the first one that matches.  This  means

that  if  you  want  a different modifier for Command-Option and just

plain Command, you must put the definition for  Command-Option  first

in the list.

14.13. Bootstrapping

This  section applies if you do not have a MacKermit diskette, but MacKermit is

available for downloading from some other computer.

MacKermit is distributed in source form for building on  a  Macintosh,  running

Apple's  Macintosh  Programmers Workbench (in MPW C), in  .HQX "BinHex 4" form,

and sometimes also as a binary resource file.  Those who want to work from  the

source are referred to the file CKMKER.BLD for instructions.

If you're downloading, it's best to work with CKMKER.HQX, a textual encoding of

the MacKermit application.  Download this using any technique available to  you

-- an old release of Kermit, an Xmodem implementation, even raw screen capture.

Then run  BinHex (version 4) to convert it into a working  application  (select

Upload  -> Application from the File menu).  Eveything will be set up correctly

-- icons, forks, etc.

If you don't have the .HQX file available, but you do have access to the binary

resource   file   (its  name  will  be  CKMKER.RSRC,  ckmker.rsrc,  CKMKER.RSR,

ckmker.rsr, %ckmker or some variation on these, depending on what  system  it's

stored  on  and  how it got there), AND if you have "MacPut" on your system and

MacTerminal on your Mac, AND if you have an 8-bit-wide (no  parity)  data  path

between  your  Mac and your system, then you can use MacPut to download the bi-

nary resource file to  your  Mac  using  MacTerminal's  "MacBinary"  format  (a

variant  of  XMODEM).  After doing this you must use a program such as  SetFile

or  ResEdit on the Mac to set the author to KR09, the type to APPL, and turn on

the  bundle bit.  Do not bother with the CKMKEY program, as it is not used with

newer MacKermits.  If you have an earlier release of MacKermit, you may use  it

14.13. Bootstrapping                                                   Page 355

-------------------------------------------------------------------------------

in place of MacTerminal and MacPut.

14.14. Differences Between Versions 0.8 and 0.9

MacKermit 0.8(34) runs on the 128K Mac, the 512K Mac, and the Mac Plus, but not

on the Macintosh II or SE.  MacKermit 0.9(40) runs on all Macs except the  128K

original.  You should use version 0.9 unless you have a 128K Mac.

The second major difference is that the program is has been translated into Ap-

ple MPW C, so that it can be edited, compiled, and built on the  Macintosh  it-

self.   This was done originally by Jim Noble of Planning Research Corporation,

who converted MacKermit from SUMACC C (which had to be cross compiled on a UNIX

system)  to  Megamax  C. Jim's version was converted to MPW C by Matthias Aebi,

who also added most of the new features listed below.  Paul Placeway integrated

the  program  with  the current (long packet) version of C-Kermit and added ad-

ditional new features.

Besides these important differences, there were many other changes from version

0.8 to version 0.9, including:

- The Cursor with open desk accessories now works correctly

- Long packet support

- New program icon

- New settings files are no longer TEXT

- Settings can now be written back to an already existing settings file

- Key redefinition function built in to Kermit, no more CKMKEY

- Server mode directory listing feature

- Multifile (folder) send

- Server "Delete" file command

- Server "Space" command

- Get whole folder content from the server with filename ":"

- Recognition of all the different Mac keyboards

- Support of menu command keys (key macros)

- Terminal settings dialog separated from communication settings

- Non-transparent terminal mode

- Display of statistics and protocol version to "About Kermit" dialog.

- Parity problems fixed

- Session logging

- Transaction logging

- Multifinder support

- Additions to the VT102 emulator (smooth scrolling, etc)

- Rearrangement of menus and displays

- Program no longer hangs if remote response window gets too full

- Program now works correctly on 64K ROM machines

- A new manual

This  manual  applies  in  large part to version 0.8(34), except that the older

version is missing the new features listed above, and it comes in  two  pieces:

CKMKER  and  CKMKEY.   The CKMKEY program is used to program the keys, like the

Set Key Macros... and Set Modifiers described in this  manual,  and  creates  a

settings  file  which Kermit itself uses.  The old version only works well with

early Macintosh keyboards.

15. CP/M-80 KERMIT                                                     Page 356

-------------------------------------------------------------------------------

15. CP/M-80 KERMIT

Program:    Bill  Catchings,  Columbia  University,  with  contributions   from

Charles  Carvalho  (ACC),  Bernie Eiben (DEC), Nick Bush (Stevens),

John  Bray  (University  of  Tennessee),  Bruce  Tanner   (Cerritos

College),  Greg Small (University of California at Berkeley), Kimmo

Laaksonen (Helskini  University  of  Technology),  Brian  Robertson

(Aberdeen University), A.J. Cole (Leeds University), John Shearwood

(Birmingham University), Tony Addyman (Salford University), Godfrey

Nix and Martin Carter (Nottingham University), Ian Young (Edinburgh

University), Chris Miles (Manchester University), Richard  Russell,

Dave Roberts, and many, many others.

Language:   8080 Assembler, M80, or MAC80

Version:    4.09

Date:       11th January, 1988

Documentation:

Charles Carvalho, ACC;  Frank  da  Cruz,  Columbia,  Bertil  Schou,

Loughborough  University  (with  lots  of help from Jon Warbrick of

Plymouth Polytechnic), and many others.

KERMIT-80 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Partial, Auto-receive only

Login scipts:                      No

Transfer text files:               Yes

Transfer binary files:             Yes

Wildcard send:                     Yes

File transfer interruption:        Yes

Filename collision avoidance:      Yes, poor

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            No

Alternate block checks:            Yes

Terminal emulation:                Yes, VT52 and others

Communication settings:            Yes; duplex, parity

Support for dial-out modems:       No

Transmit BREAK:                    Yes; most versions

IBM communication:                 Yes

Transaction logging:               No

Debug logging:                     No

Session logging:                   Yes

Raw file transmit:                 Yes

Act as server:                     No

Talk to server:                    Yes; SEND, GET, FIN, BYE

Advanced commands for servers:     No

Command/init files:                Yes

Command macros:                    No

Local file management:             Yes; DIR, ERA, TYPE, PRINT, COPY

Handle file attributes:            No

Extended packets:                  No

Sliding Windows:                   No

15. CP/M-80 KERMIT                                                     Page 357

-------------------------------------------------------------------------------

Printer control:                   Yes, better, but not perfect

15.1. Summary of CP/M

There are essentially two versions of CP/M - Versions 2.2  and  3.0  (sometimes

also called CP/M PLUS.)

CP/M-80  Version  2.2  is  run in a single 64 Kbyte "page", usually the largest

amount of memory on Z80 or 8080 systems.  The BIOS (Basic input/output system),

BDOS  (Basic  Disk  Operating  System)  and CCP (Command console processor) all

share memory with any transient program the user may wish to run.   Some  basic

commands  are  available  through  the  CCP, like DIR, ERA etc,while others are

loaded from disk into the transient program area and run as a program, like PIP

or STAT.

CP/M  Version  3.0 (or CP/M PLUS) effectively removes the requirement of having

the CCP and BDOS along with a chunk of the BIOS  code  being  resident  in  the

single  64k  byte  page of memory.  This allows even more space for programs in

the TPA, but still a little less than the maximum of 64k.  It is  substantially

different  from  CP/M version 2.2, with lots of added features.  Kermit-80 uses

very few additional version 3.0 features, and only where absolutely necessary.

CP/M file specifications are of the form DEV:XXXXXXXX.YYY, where

DEV:            is a device name, normally the A: or B: floppy.    If  omitted,

the device name defaults to your connected diskette.

XXXXXXXX        is a filename of up to 8 characters.

YYY             is the file type, up to 3 characters.

File names and file types may contain letters, digits, and some special charac-

ters, including dash, dollar sign, and underscore, but no imbedded spaces.  Up-

per and lower case letters are equivalent.

"Wildcard" file-group specifications are permitted in file names and file types

(but not device names) within certain contexts; a "*" matches a whole field,  a

"?"   matches a single character, including space.  Examples: "*.F??" specifies

all files whose types start with F and are 1, 2, or 3 characters  long;  "F?.*"

specifies  all  files whose names start with F and are no more than two charac-

ters long (before the trailing spaces).

The five CP/M commands are:

DIR file        Lists the the names of the specified files.  The  default  file

specification is "*.*".  Example: "DIR B:*.FOR".

ERA file        Erases (deletes) the specified file(s); wildcards allowed.

REN new old     Changes the name of a file from old to new, e.g.

"REN NEW.FOR=OLD.FOR".

SAVE            Saves  the specified number of memory blocks into a file.  (Not

on CP/M Plus systems)

15.1. Summary of CP/M                                                  Page 358

-------------------------------------------------------------------------------

TYPE file       Types the specified file on the screen, e.g.  "TYPE FOO.TXT".

The most important programs are:

STAT            Gives statistics on disk usage; sets and displays IOBYTE.  (Not

on CP/M Plus systems)

PIP             Peripheral  Interchange Program.  Copies files.  In response to

the "*" prompt, give a command of the form

disk:outfile=disk:infile

Wildcards ("*" for a whole field or "?"  for a letter)  can  be

used.   Examples: "A:=B:*.*" to copy a whole disk, "A:=B:*.FOR"

to copy all the Fortran programs from disk B to disk A. If  the

disk  specification  is  omitted,  your "connected" disk is as-

sumed.  Command line arguments are  also  accepted,  e.g.  "PIP

A:=B:*.*".

There  are  equivalent  commands  for CP/M Version 3.0, but are not loaded into

memory in the same way as for CP/M Version 2.2.   For  further  information  on

CP/M, consult your microcomputer manual or a CP/M handbook.

15.2. Kermit-80 Description

Since  Kermit-80  runs  on  a  standalone micro, it is always in control of the

screen -- it is always local.  Thus, it always keeps the  screen  updated  with

the file name and the packet number, whether sending or receiving.

Kermit-80  is  capable  of an imprecise or "fuzzy" timeout on an input request,

and can break deadlocks automatically.  In most cases, this is  not  important,

because  the  KERMIT  on  the  other  side  is  most  likely able to handle the

timeouts.  The timeouts done by Kermit-80 are fuzzy because they depend on  the

speed of the processor and other factors that can vary from system to system.

If,  despite  the timeout capability, the transmission appears to be stuck (and

you can tell that this has happened if the screen fails to change for a  while)

you  can type carriage return to have the micro do what it would have done on a

timeout, namely NAK the expected packet to cause theforeign  host  to  send  it

again  (or,  if  the  micro is sending, to retransmit the last packet).  Micro/

micro or micro/IBM-mainframe transfers could require this kind of manual inter-

vention.

File transfers may be interrupted in several ways.

Control-C       This will return you to Kermit-80 command level immediately, so

that you can connect back to the remote  system,  or  take  any

other desired action.

Control-X       When  sending  a  file,  this will terminate the sending of the

current file with a signal to the KERMIT on the other  side  to

discard  what  it  got  so  far.  If there are more files to be

sent, KERMIT-80 will go on to the next one.  When  receiving  a

file, KERMIT-80 will send a signal to the remote KERMIT to stop

sending this file.  If the remote KERMIT understands this  sig-

15.2. Kermit-80 Description                                            Page 359

-------------------------------------------------------------------------------

nal  (not  all  implementations  of KERMIT do), it will comply,

otherwise the file will keep coming.  In any case,  the  remote

KERMIT will go on to the next file in the group, if any.

Control-Z       Like  Control-X,  except  if a file group is being transmitted,

this will stop the transmission of the entire group.  If only a

single  file  is  being  transmitted,  it  works  exactly  like

Control-X.

Carriage Return If you type a carriage return Kermit-80 will resend the current

packet.    You  may  do this repeatedly, up to the packet retry

limit (somewhere between 5  and  16  times)  for  a  particular

packet.

KERMIT-80 COMMANDS

KERMIT-80  uses  the DECSYSTEM-20 keyword style command language.  Each keyword

may be abbreviated to its minumum unique length.  "?" may be typed to request a

menu  of the available options for the current field at any point in a command.

ESC may be typed at any point in a command to fill out the current  keyword  or

filename;  if sufficient characters have not been typed to identify the current

field uniquely, KERMIT-80 will sound a beep and allow you to continue from that

point.

BREAK   Send  a  Break condition to the remote computer.  This is only possible

if your system is capable of sending breaks.  It is intended to be used

with  PAUSE,  STRING etc and the TAKE command to do weird and wonderful

things, like automatic logging on to a remote host.

BYE     When talking to a remote Kermit Server, this  command  shuts  down  the

server  and  logs it out, and also exits from Kermit-80 to CP/M command

level.

CONNECT Establish a "virtual terminal" connection to any host that may be  con-

nected  to the serial port, i.e. pass all typein to the serial port and

display all input from the serial port on the screen.  Also, emulate  a

DEC   VT52   to   allow  cursor  control,  screen  clearing,  etc.,  if

VT52-EMULATION is ON (see below), in which case  you  should  also  set

your  terminal type on the remote host to VT52.  (Some versions emulate

other terminals.)

Warning: VT52 emulation is only successful if your system  or  its  at-

tached  terminal  can  do the same sort of functions as a genuine VT52.

Things to beware of are cursor addressing, clear to end of page and end

of  line, clear screen, home cursor, and clear-and-home functions.  The

useability of VT52 emulation depends entirely on how many of  the  VT52

functions can be emulated by your terminal.

The  escape  character  differs from micro to micro; when you issue the

CONNECT command, the micro will print a message telling you how to  get

back.    The  escape  sequence  is generally an uncommonly-used control

character, like CTRL-backslash  or  CTRL-rightbracket,  followed  by  a

single letter "command":

C   Close Connection, return to Kermit-80> command level.

15.2. Kermit-80 Description                                            Page 360

-------------------------------------------------------------------------------

S   Display Status of connection, but maintain remote connection.

?   List available single-character commands.

0   (zero) Send a null (0) character.

B   Send a BREAK signal.  Most systems provide this function.

D   Drop the line.  Used on the Apple with modem.  Automatically closes

the connection after dropping the line.  The TORCH system  acknowl-

edges this command but does nothing.

P   Toggle  printer on or off.  Allows you to copy whatever goes to the

screen to the printer.

S   Temporarily suspend logging to the log file.

Q   Restart logging to the log file

^]  (or whatever - a second copy of the escape character) Send the  es-

cape character itself to the remote host.

DIRECTORY

This provides a directory listing of the specified files.  If no  files

are  specified,  all files on the default disk are listed.  File sizes,

in K, are included.  You may interrupt  the  listing  at  any  time  by

typing any character.  The listing (even if interrupted) concludes with

a display of the amount of free storage left on the disk.  You can  in-

hibit the display of file sizes by SET DIRECTORY OFF.

ERASE filespec

This executes the CP/M ERA command on the specified file(s).  The names

of the files being erased are not displayed.

EXIT    Quit back to CP/M.  The return is made by a JMP 0 (Warmstart)

FCOPY source destination

Copy a named file to another file, either on the same drive or  another

drive.

FINISH  Like  LOGOUT,  but shuts down the remote server without logging it out.

Leaves you at Kermit-80 command level; subsequent CONNECT commands will

put you back at host system command level.

GET filespec (local_filespec)

When Kermit-80 is talking to a Kermit Server on the  host,  you  should

use the GET command to request the server to send files to you, for ex-

ample:

get hlp:k*.hlp

You may specify a local filename if you want to save  the  remote  file

under  a  different  filename.  Limitation: If you request an alternate

block check type using the SET BLOCK command, the GET command will  not

communicate  it  to the remote server.  If you want to have type 2 or 3

block checks done when getting files from the server, you have to issue

the  appropriate  SET BLOCK command to the remote KERMIT before putting

it in server mode.

HELP    List all these commands, with a short description on what the  commands

do.    A  question  mark will do the same.  If you have already typed a

command but do not know what the parameters are, type a space  (to  in-

dicate  the  end  of the command) and a question mark.  You will be in-

formed of what Kermit can expect at that stage.

15.2. Kermit-80 Description                                            Page 361

-------------------------------------------------------------------------------

INPUT time_delay string

Setup  a  string and time delay for your CP/M system to expect from the

host, then wait for that sting to be sent to your CP/M-80 system.

LOG filespec

When  CONNECTed  to a foreign host as a terminal, log the terminal ses-

sion to the specified diskette file.   This  functionality  depends  to

some  extent  on the remote host's ability to do XON/XOFF flow control,

and does not guarantee a complete transcript (after  all,  that's  what

the  KERMIT  protocol is for).  The log file is closed when the connec-

tion is closed by typing the escape character followed by  the  single-

character command "C".

LOG (append)

It is possible to temporarily suspend  logging  during  connect  state.

Typing  an escape sequence can turn file logging on (<escape-character>

R for Resume) or off (<escape-character> Q for quiet).

Re-entering connect state will re-open the previously opened  log  file

and append to that file.

LOGOUT  Like BYE, but leaves you at Kermit-80 command level.

PAUSE delay_period

If this command is issued your CP/M system will  wait  a  while  before

proceeding with another command.  This is intended for use in TAKE com-

mands, where you may want to pause for a while before  proceeding  with

the  rest  of the TAKE file.  The actula delay is very variable between

systems, and values should be determined on a trial and error basis.

PRINT   Print a file to the console and printer.  Output to the printer is buf-

fered by the Kermit maintained printer buffer This routine is identical

to TYPE but characters are echoed to the printer  as  well  as  to  the

screen.  Suspending and aborting output is as described in TYPE.

RECEIVE filespec

Receive file(s) from the remote Kermit, and save them under  the  names

provided  in  the file headers supplied by the remote host.  If a local

filespec is given, the file is saved under the given filename.  If  the

names  aren't legal, use as many legal characters from the name as pos-

sible (see the description of SET FILE-WARNING below).   If  there's  a

conflict,  and  FILE-WARNING  is  ON,  warn the user and try to build a

unique name for the file by adding "&" characters to the name.

SEND filespec

Send  file(s) specified by filespec to the remote Kermit.  The filespec

may contain CP/M wildcards.

SET parameter [value]

Set the specified parameter to the specified value.  Possible parameter

settings:

AUTORECEIVE

ON  (or  OFF).    Allows  several  files to be received without

having to type RECEIVE on the receiving machine.   The  routine

simply  looks for activity on the serial line, and if so fudges

15.2. Kermit-80 Description                                            Page 362

-------------------------------------------------------------------------------

a RECEIVE command.  The packet sent by the sender will be lost.

BAUD-RATE value

Change  the baud rate of the communications port.  This command

only works on some systems.  value is  the  numeric  baud  rate

(300,  9600,  etc.) desired.  Type SET BAUD followed by a ques-

tion mark for a list of supported baud rates.  On systems  that

do  not  support  this command, you must set the port baud rate

from CP/M or other setup mechanism outside of KERMIT-80.

BLOCK-CHECK-TYPE option

The options are:

1-CHARACTER-CHECKSUM

Normal, default, standard 6-bit checksum.

2-CHARACTER-CHECKSUM

A 12-bit checksum encoded as two characters.

3-CHARACTER-CRC-CCITT

A 16-bit CCITT-format Cyclic Redundancy Check,  encoded

as 3 characters.

BUFFER-SIZE value

This allows you to set a buffer size during transfer  of  data.

On  some systems it takes so long that the remote end times out

while the local system is reading or writing to disk.  The size

is  the  number of 128 disk sectors (nominal) and can be from 1

(128 bytes) to 64 (8 kbytes).

CASE-SENSITIVITY ON (or OFF)

Kermit-80  usually  maps  lower  case  characters to upper case

character, but there may be times when  lower  case  should  be

left  alone.  SET CASE-SENSITIVITY ON will do this-but only for

some functions.

CP/M-80 filenames will still be mapped to uppercase characters.

DEBUG   ON (or OFF).  Enables/disables displaying  of  packets  on  the

screen during file transfer.  Not performed if the QUIET option

has been set for the terminal (ie SET TERMINAL QUIET)

DEFAULT-DISK drive letter

This  allows you to set the default disk as source and destina-

tion of file transfers.   In  addition,  issuing  this  command

causes  you  to  switch  to  the  specified disk and log it in,

write-enabled.  The colon must be included  in  the  disk  name

(A:).   The selected disk appears in your KERMIT-80 prompt, for

instance

Kermit-80 14A:>

DIRECTORY-FILE-SIZE ON (or OFF).

By setting DIRECTORY-FILE-SIZE OFF you can  get  an  abreviated

listing of your disk drive.  File sizes are not calculated, and

five files are shown on a line.  Setting this  option  ON  will

show file sizes of each file.

15.2. Kermit-80 Description                                            Page 363

-------------------------------------------------------------------------------

Both options will list the free space remaining.

ESCAPE  Change  the  escape character for virtual terminal connections.

Kermit-80 will prompt you for the new escape  character,  which

you enter literally.

FILE-MODE option

Tells KERMIT-80 what kind of file it is sending, so that KERMIT

can  correctly  determine the end of the file.  SET FILE BINARY

means to send all the 128-byte blocks (ie logical CP/M sectors)

of the file, including the last block in its entirety; SET FILE

ASCII is used for text files, and transmission stops  when  the

first  Control-Z  is  encountered anywhere in the file (this is

the CP/M convention for marking the end of a text file).

SET FILE-MODE DEFAULT tells Kermit to attempt to determine  the

file  type  by  examining  the  file  being  transmitted.  If a

Control-Z appears before the last block of the file, it is  as-

sumed  to  be  BINARY;  if, when the first Control-Z is encoun-

tered, the remainder of the file contains only control-Z's,  it

is  assumed to be a text file.  Unfortunately, not all programs

fill the remainder of the last  record  of  a  text  file  with

Control-Z's, so this algorithm is not always successful.

If  binary transmission is used on a text file, or a compressed

file (eg a .DQC file) some extraneous characters (up to 127  of

them) may appear at the end of the file on the target system.

If  ASCII  transmission  is used on a binary file, any 8th bits

set will be stripped and a warning sent to the console.    When

the  first  control-Z is encountered, the file is assumed to be

at the end, even if it is not.

FLOW-CONTROL ON (or OFF)

Sets  XON/XOFF  flow  control on or off.  If set ON the host is

expected to respond to an XOFF or XON sent by  Kermit-80.    If

set  off,  no  flow  control is assumed and any XON/XOFF is ig-

nored.

IBM ON (or OFF)

Allow  the  transfer of files to and from an IBM mainframe com-

puter.  This  makes  Kermit-80  wait  for  the  IBM  turnaround

character (XON), ignore parity on input, add appropriate parity

to output, and use local  echoing  during  CONNECT.    As  dis-

tributed, KERMIT-80 uses MARK parity for IBM communication.  If

you don't give this command, IBM mode is OFF.  Since IBM VM/CMS

KERMIT  does not have timeout capability, SET IBM ON also turns

on the "fuzzy timer" automatically.

LOCAL-ECHO ON (or OFF)

When  you  CONNECT to a remote host, you must set LOCAL-ECHO ON

if the host is half  duplex,  OFF  if  full  duplex.    OFF  by

default.

LOGGING ON (or OFF)

Cease or resume logging whenever connect mode is entered.  This

15.2. Kermit-80 Description                                            Page 364

-------------------------------------------------------------------------------

is  really only applicable after a LOG command is no longer re-

quired.

NO-EXIT This command is applicable only for  Kermit  initiated  with  a

command tail.  For example, if Kermit was initiated by:

KERMIT ;SEND HELLO;NO-EXIT

Kermit would first seek out and execute the KERMIT.INI file (if

present), then send file HELLO to a  remote  system.    Usually

Kermit would exit back to CP/M, but NO-EXIT over-rides this.

Note  the  leading  semicolon.  This clears leading spaces from

the first command.

PORT port name

Allows  you  to  switch  between different communication ports.

This command is not available on all systems.  Type SET PORT  ?

for  a  list  of valid options for your system. (Note:  If your

system does not support several ports, this command will return

a "Not implemented" error if you try to set a port.)

PRINTER ON (or OFF)

Turns copying of CONNECT session to printer on and off.  It  is

also  possible  to  toggle  the printer on/off from the connect

state, by typing <escape character> followed by P.

PARITY option

Sets  parity  for  outgoing characters to one of the following:

NONE, SPACE, MARK, EVEN, or ODD.  On input, if parity is  NONE,

then  the  8th  bit is kept (as data), otherwise it is stripped

and ignored.  The parity setting applies to both terminal  con-

nection and file transfer.  If you set parity to anything other

than none, KERMIT-80 will attempt to use "8th bit prefixing" to

transfer  binary files.  If the other KERMIT is also capable of

8th bit prefixing, then binary files can  be  transferred  suc-

cessfully;  if  not, the 8th bit of each data byte will be lost

(you will see a warning on your screen if this happens).

RECEIVE parameter [value]

Set a RECEIVE parameter.

PAD-CHAR

Set the PAD character to  use  while  receiving  files.

Currently a dummy, as for SET SEND PAD-CHAR.

PADDING [value]

Set the number of PAD characters to use while receiving

files.  Same as SET SEND PADDING.

START-OF-PACKET [value]

Set the default start of Packet character for receiving

files.   Apply the same rules and considerations as for

SET SEND START-OF-PACKET.

SEND parameter [value]

Set a SEND parameter.

15.2. Kermit-80 Description                                            Page 365

-------------------------------------------------------------------------------

PAD-CHAR

Set the Pad character to be used while  sending  files.

It  is  currently  a  dummy entry, and does not do any-

thing.

PADDING [value]

Set  the  number  of PAD-CHARS to be used while sending

files.  This too does nothing.

START-OF-PACKET

Set  the  default  start of packet character to another

character than control-A.  This  may  be  necessary  on

systems  (including  intervening  networks)  that  trap

control-A characters.  Choose a control  character  not

otherwise used, ie not carriage return (13D, ODH), line

feed (10D, OAN), tabs (09D, 09H), backspace (08H),  and

bell  (07H)  or  any  other  used  between you and your

remote system.

TACTRAP set the TAC intercept character.  If you are attached to a  TAC

it  will swallow the intercept character (commercial AT sign by

default) so Kermit sends it twice.  With this command  you  can

set  the  intercept  character  (ie  the  one to send twice) to

another character.

TERMINAL option

Select one of the following terminal characteristics:

OFF     sets emulation off, and its up to the attached terminal

to respond to escape sequences  sent  from  the  remote

host system.

DUMB    Like  off, but carriage return and line feed characters

are the only control characters accepted.    All  other

control characters are simply ignored. (Really a "Glass

TTY").

EXTERNAL

Emulation  is  provided  for by a routine in the system

dependent part of Kermit.  Attempting to set  this  op-

tion  without  having  and  externally supplied routine

will returna "Not Implemented" error.

VT52    When connected as a terminal to  a  foreign  host,  the

micro  emulates  a  VT52.    VT52  emulation  is set by

default, except on micros that  already  have  terminal

functionality  built in, such as the DEC VT180 and DEC-

mate (these act as VT100-series terminals).  Some  sys-

tems emulate other terminals, like the ADM3A; see table

15-5.

QUIET   Do not display any file transfer information  onto  the

console.    This  mode is useful if you console takes a

long time to update the display.  Only the file name is

displayed.  DEBUGging information is not displayed even

if selected.

REGULAR Inverse of  QUIET.    All  packets  etc  displayed,  as

15.2. Kermit-80 Description                                            Page 366

-------------------------------------------------------------------------------

ususal.

TIMER ON (or OFF)

Enable or disable the "fuzzy timer".    The  timer  is  off  by

default,  because in the normal case KERMIT-80 is communicating

with a mainframe KERMIT that has its own timer.  Mainframe KER-

MIT  timers  tend  to  be more precise or adaptable to changing

conditions.  You should SET TIMER ON if you  are  communicating

with a KERMIT that does not have a timer.  You should SET TIMER

OFF if you are communicating over a network with long delays.

USER new user number

Sets another user number to be active.  Acceptable user numbers

are 0 to 31, though it is recommended to use user numbers 0  to

15  only.    This  is really only useful for Winchester Systems

with high disk capacities.

WARNING ON (or OFF)

Warn  user  of  filename  conflicts  when  receiving files from

remote host, and attempt to generate a unique  name  by  adding

"&" characters to the given name.  ON by default.

SHOW    Display  all  setable  parameters.    You  will get a page or so of the

status af all parameters that can be set using the SET command.

STATUS  The same function as Show.

STRING line of text

Send  a character string to the host.  This simply copies the string to

the correct line, and assumes all appropriate parameters have been  set

to  be  used  eg baudrate, parity etc.  It is intended as an option for

the TAKE command.

TAKE filespec

Take  characters  and  commands from the specified file as if they were

entered from the keyboard.  This is useful if you  want  to  set  up  a

batch  job.    A command file can send, get, receive, set functions etc

automatically.

An automatic "TAKE KERMIT.INI" is executed from the default drive  when

Kermit-80  is  loaded.   This can be used to set defaults of band rate,

parity, filetype, default drive etc.

If KERMIT.INI does not exist, control is given directly to the user.

TRANSMIT filespec wait character string

Send  the  specified file to the system on the other end of the connec-

tion as though it were being typed at the terminal, one line at a time.

Each line sent is terminated with a carriage return, and any line feeds

are stripped from the file sent.  After each line has been sent  Kermit

waits  for a character string from the host (eg a acrriage return).  If

not specified, a carriage return is assumed.  No KERMIT protocol is in-

volved.   An asterix (star) is sent to the console for every line sent,

to indicate how the transfer is progressing.  This is useful for  send-

ing  files  to systems that don't have a KERMIT program.  During trans-

mission, you may type one of these single-character commands:

15.2. Kermit-80 Description                                            Page 367

-------------------------------------------------------------------------------

Control-C

Cease transmission, and drop into terminal emulation mode.

<CR>

Re-transmit the previous line.

TYPE    Type a file to the console.  Typing any character other than  Control-C

while  the  file  is  being displayed will suspend the output.  Another

character will resume output.  A Control-C will abort the rest  of  the

output.

VERSION Show  the  name,  edit  number, and edit date of several of the modules

that make up Kermit-80.

15.3. Kermit-80 Flavors

Many of the systems supported use an external terminal, rather than a  built-in

console.    Kermit  may be further customized for these systems by defining (at

assembly time) the terminal type to be used.  If the terminal type  is  unknown

or  does  not match any of the existing terminal options, the generic "CRT" op-

tion may be selected.  In this case, Kermit  cannot  do  fancy  screen  control

during  file transfer; it simply types the file names, packet numbers, and mes-

sages in sequence across and down the screen.  This works best if you  can  put

your  micro  or  terminal in "autowrap" mode; otherwise the packet numbers will

pile up in the rightmost column; the filenames and messages will always  appear

on a new line, however.  If no specific terminal has been selected, Kermit can-

not do VT52 emulation; it can act as a  "dumb  terminal"  (sometimes  called  a

"glass  TTY"),  or else its own built in terminal firmware provides cursor con-

trol functions independent of the Kermit program.

15.3.1. Generic Kermit-80

"Generic Kermit-80" is an implementation of Kermit that should run on any 8080-

compatible CP/M 2.2 system with no modification at all, or perhaps only a minor

one.  Unlike other Kermit-80 implementations, it contains  no  system-dependent

manipulation  of  the  serial  port.    All I/O is done with standard CP/M BIOS

calls, and I/O redirection is done using the CP/M IOBYTE function,  which,  ac-

cording  to  the  Digital Research CP/M Operating System Manual, is an optional

feature of any particular CP/M implementation.  If your system does not provide

the IOBYTE function, Generic Kermit-80 will not work; furthermore, not all sys-

tems that implement IOBYTE do so in the same way.  The SET PORT command may  be

used  to  select the devices to be used for input and output.  Table 15-1 lists

the options to the SET PORT command and their effects.

-------------------------------------------------------------------------

SET PORT xxx    input from      output to

CRT            CRT:            CRT:

PTR            PTR:            PTP:

TTY            TTY:            TTY:

UC1            UC1:            UC1:

UR1            UR1:            UP1:

UR2            UR2:            UP2:

15.3.1. Generic Kermit-80                                              Page 368

-------------------------------------------------------------------------------

Table 15-1:   Kermit-80 SET PORT Options

-------------------------------------------------------------------------

The default is SET PORT PTR.  In all cases, the console (CON:) and list  (LST:)

devices used are those selected when Kermit is started.

The  reason all Kermit-80 implementations aren't generic is that a good deal of

speed is sacrificed by getting all services from the operating system.  While a

specific  implementation  of Kermit-80 may be able to operate at 4800, 9600, or

even 56 Kilo baud, generic Kermit will fail to work on some systems  at  speeds

in  excess  of  1200  baud.   In addition, many features of Kermit require more

specific knowledge of the hardware involved.   Generic  Kermit  cannot  send  a

BREAK signal, or change the baud rate, for example.

15.3.2. CP/M 3 Kermit

CP/M-3  Kermit  (also  known  as  CP/M-Plus  Kermit)  is  a  version of generic

Kermit-80, and should run on most CP/M-3 (CP/M-Plus)  systems.    It  uses  the

auxilliary  port  (AUX:) to communicate to the remote Kermit.  The SET BAUD and

SET PORT commands are not supported; nor can a BREAK be  sent.    Like  generic

Kermit-80, a terminal may be selected at assembly time.

15.3.3. System-Specific Versions

There  are  also many versions of Kermit-80 tailored to specific systems.  Most

of these operate  uniformly,  but  some  of  them  take  advantage  (or  suffer

limitations) of the specific system.  Here are some of the special features for

particular systems:

Amstrad: -- Two versions:

PCW 8256

The PCW 8256/8512 with the serial inerafce attached.

CPC 6128

The 664 with add on memory and 6128 are both supported.   Both  systems

must  run  CP/M  Plus, so the 664 will need an add on RAM pack and CP/M

upgrade.  A high speed transfer rate of 38k baud can  be  used  between

Amstrad computers.

ACCESS:

Access Matrix computer using port J5.  Supports SET BAUD-RATE for rates  of

300-9600 baud.

Apple II -- four variations:

APMMDM:

Apple with Z80 Softcard and Micromodem II in slot 2 Dialout  capability

provided  in connect command; user is prompted for phone number if car-

rier is not present.  During connect mode, ^]D drops carrier.  BYE com-

mand also causes carrier to be dropped.

AP6551:

15.3.3. System-Specific Versions                                       Page 369

-------------------------------------------------------------------------------

Apple with Z80 Softcard, and one of  several  6551-based  communication

cards; the slot number is a compile-time parameter (default is slot 2).

SET BAUD-RATE supported; speeds are 110-19200 baud.

APCPS:

Apple with Z80 Softcard and CP Multi-Function Card.  The slot number is

again a compile-time parameter.  SET BAUD-RATE is  supported  for  baud

rates from 50 baud to 19200 baud.

AP6850:

Apple II with Z80 Softcard and a 6850-based USART in  slot  2-the  slot

being a compile-time parameter.  SET BAUD-RATE is not supported.

BBC:

Acorn Computers BBC  Computer  with  Acorn  Z80  second  processor  running

CP/M-80.  Supports SET BAUD-RATE and can send breaks.

BigBoard II:

Uses serial port A. To use port B, change mnport, mnprts,  and  baudrt  and

reassemble.    Can  generate  BREAK.    SET BAUD-RATE supported; speeds are

300-38400 baud.

Cifer:

Originally  coded  for Cifer 1886 using the VL: port set as TTYI: and TTYO:

but works successfully on 18xx and 28xx series machines.

There are now two versions, each with two variations: Either  running  CP/M

Version  2.2 or 3.0, and either using the VL: or AUX: ports.  The VL:  port

version can only use seven bits of data, so parity  prefixing  is  required

for  binary  file transfers.  This restriction is removed by using the AUX:

port.  For those interested, the problem is due to the interprocessor  link

between  the video and CPU (!) boards.  The VL: port is on the video board,

and the AUX: port on the CPU board, and the inter processor link  can  only

transfer seven bits of data.

Supports SET BAUD-RATE, and can generate breaks on some models with a BREAK

key.

Comart:

Comart Communicator-Similar to Northstar equipment.  Can generate BREAK.

Compupro:

Based on Kermit 3.x, and has been merged into V4.09

CPT-85xx word processors:

Can generate BREAK.  SET BAUD-RATE supported; speeds are 50-9600 baud.

Cromemco:

Cromemco computers with TU-ART card.    Supports  SET  BAUD-RATE  (110-9600

baud).

DEC DECmate II word processor (with Z80 card):

Can generate BREAK.

DEC VT180 (Robin):

Three  output  ports,  referred to as COMMUNICATIONS, GENERAL, and PRINTER.

15.3.3. System-Specific Versions                                       Page 370

-------------------------------------------------------------------------------

Can generate BREAK.

Digicomp Delphi 100:

SET BAUD-RATE supported; speeds are 50-19200 baud.

Discovery:

Action Computer Enterprises "Discovery" Multi-user Computer.  Uses  Port  B

on  an  83U  user  board.    Supports SET BAUD-RATE for 50-19200 baud.  Can

generate BREAK.

Epson:

Epson  PX-8 with LCD display.  Although it is quite different in displaying

of Packet Information, it works as any other CP/M-80 Kermit.  Supports  SET

BAUD-RATE and can generate BREAK.

Generic Kermit:

Two versions, one for CP/M version 2.2 and version 3.   These  systems  use

IOBYTE  flipping  (V2.2)  and the AUX:  device to communicate to the serial

line.  You may have to SET PORT xxx before the version 2.2  will  work,  as

Kermit needs to know what device is the serial line.

Genie:

Eaca Video Genie.

Heath: Three Versions:

H8QUAD

for  Heath-8  systems  with  the  quad  io board.  This system has been

derived from V3.x code.  Note that this version will not run "as is" on

H89 systems.

H89 For Heath-89 machines suports baud rates from 50 to 56,000 baud.

Z100

For Z-100 running CP/M-85.  This version does not  support  setting  of

baud rates.

Intertec Superbrain: Two Versions:

BRAINA

For superbrain using AUX port.  Breaks and SET BAUD both supported

BRAINM

As above, but using the MAIN port.

Ithaca:

Ithaca Intersystems based computer using the VIO card for  all  IO  to  the

outside  world.  The system is strictly speaking a home-brew variant of the

Ithaca machine, using an S100 cardcage without a front panel.  It uses  the

Extended  BIOS  by  EuroMicro of London.  However, I see no reason for this

version not running on a genuine Ithaca Intersystems machine.    There  are

patches needed to the EuroMicro BIOS to make this version work.

Kaypro:

Should work on  most  Kaypro  models,  as  well  as  some  related  systems

(Ferguson  BigBoard  I,  Xerox  820).   For the newer Kaypros with multiple

15.3.3. System-Specific Versions                                       Page 371

-------------------------------------------------------------------------------

ports, Kermit uses the one labeled "serial data"; it cannot use the  serial

printer  or  internal  modem ports (but it should be possible to modify the

values for mnport, mnprts, and baudrt to do this).    Can  generate  BREAK.

SET BAUD-RATE supported; speeds are 50-19200 baud.

Lobo:

Lobo MAX-80.  Supports SET BAUD-RATE and can generate BREAKS.

Merlin:

British  Telecom  Merlin  M2215 (also Rair Black Box, possibly also the ICL

PC?).  Requires a terminal.

Micromate:

PMC 101 Micromate.  Supports SET BAUD-RATE and can generate BREAK.

Micromint: Two versions

S6  The  Ciarcia/Micromint sb-180 board with a 6Mhz procoessor.  System re-

quires a terminal.

S9  As above, but with a 9Mhz processor.

NCR:

Decisionmate 5.  Uses the 2651 and is largely the same as the Ithaca Inter-

systems machine implementation.

Northstar: -- There are four versions available:

NORTHS:

Northstar  Horizon  with  HS10-4 board.  Supports SET BAUD-RATE and SET

PORT.

HORIZON:

Northstar  Horizon  using  the  serial  ports on the Mother board.  Can

generate BREAK.

BASICNS:

Basic Northstar Horizon using the printer port.  Can generate BREAK.

ADVANT:

Northstar Advantage.  Supports SET BAUD-RATE and  can  generate  BREAK.

Traps Control-0 in the system filter.

Morrow Decision I:

Uses the Multi-I/O board.  Port 1 is the console, port 3 is the  communica-

tions line.  SET BAUD-RATE supported; speeds are 75-56000 baud.

Morrow Micro Decision I:

Nokia MicroMikko:

Will not echo control-O (which locks keyboard).  SET  BAUD-RATE  supported;

speeds are 75-9600 baud.

Ohio Scientific:

Doesn't have screen control.

15.3.3. System-Specific Versions                                       Page 372

-------------------------------------------------------------------------------

Osborne 1:

Uses  serial  line,  not  internal  modem.   Left-arrow key generates <DEL>

("delete" or "rubout" character) during connect mode.  SET  BAUD-RATE  sup-

ported; speeds are 300 and 1200 baud.  Now supports multi-sector buffering.

Research Machines: Two Versions:

RM380ZM:

380Z and 5.25" disks supports SET BAUD.RATE

RM380ZF:

380Z and 8" disks, otherwise as above.

Sanyo:

Sanyo MBC-1100.  This version derived from Kermit V3.x

ScreenTyper:

Details unkown.

TRS-80: Three versions:

TRS80LB:

TRS-80 with Lifeboat CP/M

TRS80PT:

TRS-80 with Pickles and Trout CP/M

TRSM4:

TRS-80 Model 4 with Montezuma CP/M

Teletek:

Teletek Systemaster. Supports SET BAUD.

Telcon:

TELCON ZOBRA portable computer.

Torch:

Torch Unicorn 5 initially, but the code runs on other Z80 based CP/N (as in

Nut!) systems.  It uses the BBC Computer as  a  "Base  processor",  and  is

similar  to the BBC version.  The base processors RS423 port is used rather

than any built in Modem.  (UK telecoms legislation effectively makes  modem

control  software  tricky  business...).   Two potential versions exist-one

using cursor positioning codes for a MCP and CCCP ROM combination of  revi-

sion  less  than  1.00,  the  other  version  uses  the additional facility

MCP/CCCP versions greater than 1.  Supports SET BAUD-RATE and can  generate

BREAKs.

Note  that  binary  files  must be transferred using SET PARITY to anything

other than NONE!  Parity is neither generated nor checked.

US Micro Sales:

S-100-8 based computer.

Vector Graphics:

Vector

15.3.3. System-Specific Versions                                       Page 373

-------------------------------------------------------------------------------

Xerox:

Xerox 820.

Z80MU:

Development Kermit on a PC running the Z80MU Z80 and CP/M  2.2  development

system.  Allows development of the system independent modules to be done on

an IBM PC or clone.  Also allows the generation of new .HEX files, that may

then  be  KERMITed  to  the target system.  Note:  Not all the BDOS or BIOS

routines are supported, so avoid "unusual" BIOS/BDOS calls.  (For  example,

DIR from within Kermit will fail as one of the BIOS routines returning disk

parameters is not supported.)

15.4. Installation of Kermit-80

Kermit-80 was written originally for the Intertec SuperBrain in  lowest-common-

denominator   8080 code with the standard assembler, ASM (single source module,

no macros, no advanced instructions), so that it  could  be  assembled  on  any

CP/M-80  system  (the  8080  assembler  is  distributed  as  a standard part of

CP/M-80, whereas the fancier Z80 or macro assemblers  are  normally  commercial

products).    It  has since been modified to run on many other systems as well.

Kermit-80 should be able to run on any 8080-, 8085- or Z80-based  microcomputer

under    CP/M with appropriate minor changes to reflect the port I/O and screen

control for the system (see below).

The proliferation of new systems supported by Kermit-80 made the  program  grow

so  large  and  complicated that it had to be broken up into system-independent

and system-dependent modules, as of version 4 (this was done  by  Charles  Car-

valho  of  ACC).   Each module is composed of multiple files.  This has reduced

the time and disk space necessary for assembly; Kermit-80 may once again be as-

sembled  on a CP/M system with roughly 250Kbytes of space.  The majority of the

code does not need to be reassembled to support a new system.    Unfortunately,

it can no longer be assembled with ASM, since ASM does not support multiple in-

put files.  To allow it to be assembled on any CP/M system,  the  public-domain

assembler  LASM  is included in the distribution kit. Kermit-80 may also be as-

sembled with Microsoft's M80 (not supplied).  In  theory,  any  8080  assembler

supporting the INCLUDE directive ought to work, as well.

All versions of Kermit-80 are assembled from the same set of sources, with sys-

tem dependencies taken care of by assembly-time conditionals within the system-

dependent module (eventually, the system-dependent module will itself be broken

up into multiple files, one for each system).  The most important system depen-

dencies  are  terminal emulation (when CONNECTed to the remote host) and screen

handling, which are dependent on the individual  micro's  escape  codes  (these

features  are table driven and easily modified for other CP/M systems), and the

lowest level I/O routines  for  the  serial  communications  port.    The  port

routines  are  best  done  only  with BDOS calls, but some systems do not allow

this, primarily because the BDOS routines strip the parity bit during port I/O,

and the parity bit is used for data when transmitting binary files.

Kermit-80's  I/O routines must check the port status and go elsewhere if no in-

put is available; this allows for virtual terminal connection, keyboard  inter-

ruption  of  stuck  transmissions,  etc.    On systems that fully implement I/O

redirection via the optional CP/M IOBYTE facility, this may be done by  switch-

ing  the IOBYTE definition.  On others, however, IN/OUT instructions explicitly

referencing the port device registers must be used.

15.4. Installation of Kermit-80                                        Page 374

-------------------------------------------------------------------------------

CP/M-80 KERMIT versions 3.8 and later include a "fuzzy  timer"  that  allows  a

timeout to occur after an interval ranging from 5 to 20 seconds (depending upon

the speed of the processor and the operating system routines) during which  ex-

pected  input does not appear at the port.  In this case, retransmission occurs

automatically.  In any case, you may type a carriage return during transmission

to simulate a timeout when the transfer appears to be stuck.

15.4.1. Organization of Kermit-80

Kermit-80  consists  of  two  modules, each of which is generated from multiple

source files.  The first  module  contains  the  system-independent  code;  the

second module is configured for a particular system and merged with the system-

independent module to produce a customized Kermit-80.

The distribution kit contains:

- the system-independent module, CPSKER.HEX;

- the system-dependent modules, CPV*.HEX (see table 15-2 and 15-3);

- the source files, CPS*.ASM and CPX*.ASM,

- the public-domain CP/M assembler, LASM.*,

- the public-domain CP/M load/patch utility, MLOAD.*

15.4.1. Organization of Kermit-80                                      Page 375

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Symbol  Filename System

ACCESS  CPVACC  Access Matrix

ADVANT  CPVADV  Northstar Advantage

AP6551  CPVAPL  Apple II, Z80 Softcard, 6551 ACIA in serial interface

AP6850  CPVA65  Apple II, Z80 Softcard, 6850 ACIA in Serial Iiterface

APMMDM  CPVAPM  Apple II, Z80 Softcard, Micromodem II in slot 2

APCPS   CPVCPS  Apple II, Z80 Softcard, with CPS multifunction card

BASICNS CPVBNS  Northstar Horizon (terminal required)

BBC     CPVBBC  Acorn "BBC" computer with Acorn Z80 second processor

BBII    CPVBB2  BigBoard II (terminal required)

BRAINM  CPVBRM  Intertec Superbrain using the main port

BRAINA  CPVBRA  Intertec Superbrain using the Aux port

CIFER2  CPVCIF  Cifer 1886 using the VL: Serial port and CP/M V2.2

CIFER3  CPVCI3  Cifer 1886 using the VL: Serial port and CP/M V3.0

CIFER2  CPVCA2  Cifer 1886 using the AUX: Serial port and CP/M V2.2

CIFER3  CPVCA3  Cifer 1886 using the AUX: Serial port and CP/M V3.0

CMEMCO  CPVCRO  Cromemco with TU-ART card. Terminal required)

COMART  CPVCOM  Comart Communicator (terminal required)

COMPRO  CPVPRO  Compupro with Interfacer 4 (or 3).  Terminal required.

CPC     CPVCPC  Amstrad CPC 664 and 6128 and CP/M 3

CPM3    CPVCP3  "Generic": CP/M 3.0 (CP/M Plus) systems (terminal req'd)

CPT85XX CPVCPT  CPT-85xx wordprocessor with CP/M

DELPHI  CPVDEL  Digicomp Delphi 100 (terminal required)

DISC    CPVDIS  Action Computer Enterprises "Discovery" (terminal req'd)

DMII    CPVDM2  DECmate II with CP/M option

GENER   CPVGEN  "Generic": CPM 2.2 systems with IOBYTE (terminal req'd)

GENIE   CPVGNI  Video Genie

H8QUAD  CPVH8Q  Heath-8 with Quad 8 i/o board

HEATH   CPVH89  Heath/Zenith H89

HORIZON CPVHOR  Northstar Horizon (terminal required)

KPII    CPVKPR  Kaypro-II (and 4; probably supports all Kaypro systems)

LOBO    CPVLBO  Lobo Max-80

"symbol" is the symbol used to select the target system, in CPVTYP.ASM;

"filename" is the name under which the module is supplied in the distribution.

Table 15-2:   Systems supported by Kermit-80 (Part 1)

-------------------------------------------------------------------------------

15.4.1. Organization of Kermit-80                                      Page 376

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Symbol  Filename System

M2215   CPVMRL  British Telecom Merlin/Rair Black Box (terminal required)

MDI     CPVMDI  Morrow Decision I (terminal required)

MIKKO   CPVMIK  MikroMikko

MMATE   CPVMM   PMC 101 Micromate (terminal required)

MMDI    CPVUD   Morrow Micro Decision I (terminal required)

NCRDMV  CPVDMV  NCR Decision Mate V.  (Terminal required?)

NORTHS  CPVNS   Northstar Horizon with HSIO-4 card (terminal req'd)

OSBRN1  CPVOSB  Osborne 1

OSI     CPVOSI  Ohio Scientific

PCI2651 CPVPCI  Ithaca Intersystems with VI0 card (terminal required)

PCW     CPVPCW  Amstrad PCW 8256/8512 with serial interface

PX8     CPVPX8  Epson PX-8

RM380ZM CPVRMM  Research Machines 380Z with MDS (5.25" discs)

RM380ZF CPVRMF  Research Machines 380Z with FDS (8" discs)

ROBIN   CPVROB  DEC VT180

S1008   CPVUSM  US Microsales S-100-8 (terminal required)

SANYO   CPVSAN  Sanyo MBC-1100

SB6     CPVSB6  Micromint SB-180 with 6Mhz CPU (terminal required)

SB9     CPVSB9  Micromint SB-180 with 9Mhz CPU (terminal required)

SCNTPR  CPVSCN  Screentyper

TELCON  CPVTEL  TELCON Zobra portable

TELETEK CPVTET  Teletek Systemaster

TORCH   CPVTRC  Torch computers BBC-B with Z80 second processors

TRS80LB CPVTLB  TRS-80 model II with Lifeboat 2.25C CP/M Display

TRS80PT CPVTPT  TRS-80 model II with Pickles + Trout CP/M Display

TRSM4   CPVTM4  TRS-80 model IV

VECTOR  CPVVEC  Vector Graphics

XER820  CPVXER  Xerox 820

Z100    CPVZ00  Z-100 under CP/M-85

Z80MU   CPVZ80  Z80MU development system on a PC

"symbol" is the symbol used to select the target system, in CPXTYP.ASM;

"filename" is the name under which the module is supplied in the distribution.

Table 15-3:   Systems supported by Kermit-80 (Part 2)

-------------------------------------------------------------------------------

15.4.1. Organization of Kermit-80                                      Page 377

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Symbol  Terminal type

CRT     Dumb terminal type.  Does not do cursor addressing

ADM3A   Lear Seigler ADM 3A

ADM22   Lear Seigler ADM 22

AM230   Ampro 230

H1500   Hazeltine 1500

SMRTVD  Netronics Smartvid

SOROQ   Soroq IQ-120

TVI912  Televideo 912

TVI925  Televideo 925 or Freedom 100

VT52    Dec VT52 or equivalent (H19)

VT100   Dec VT100 or equivalent

WYSE    Wyse 100

"symbol" is the symbol used to select the target system, in CPXTYP.ASM;

"Terminal type" is the type of terminal "symbol" selects.

Table 15-4:   Terminals supported by Kermit-80

-------------------------------------------------------------------------------

15.4.2. Downloading Kermit-80

You'll need either a pre-configured .COM file or the system-independent module,

CPSKER,  in binary (.COM) or hex (.HEX) format and the system-dependent overlay

for your system (from Tables 15-2 and 15-3).  If your system is not  listed  in

the  table,  get  the generic CP/M 2.2 Kermit or the generic CP/M 3 Kermit.  If

you already have a version of Kermit on your micro and you want  to  install  a

new  version,  simply  use your present version to get the new files.  Transfer

the files to your system and skip ahead to "merging the modules".

If you do not have a copy of Kermit on your micro, and you cannot borrow a Ker-

mit  floppy  but  you do have access to a mainframe computer with a copy of the

Kermit-80 distribution, you should read this section.

There are several ways to get CP/M Kermit from a host  system  to  your  micro.

The easiest is to "download" the necessary "hex" files into your micro's memory

and then save them on the disk.  If you have a  terminal  emulator  program  on

your  micro which can save a copy of the session to disk, connect to your host,

and type the necessary files.  Exit from the emulator, saving the session  log,

and  edit the session log to extract the hex files.  Skip ahead to "merging the

files".

The following is a procedure which, though far from foolproof, should allow you

to  get a version of Kermit to your CP/M based micro.  It depends upon the host

prompt, or at least the first character of the host prompt, being some  charac-

ter  that  cannot  appear in a hex file (the valid characters for hex files are

the digits 0-9, the upper case letters A-F, the colon ``:'',  carriage  return,

and line feed).  As soon the prompt character is encountered, the transfer will

terminate.  If your host does not issue a prompt  that  will  accommodate  this

scheme,  you  can achieve the same effect by adding an atsign ``@'' to the very

15.4.2. Downloading Kermit-80                                          Page 378

-------------------------------------------------------------------------------

end of the hex file before sending it from the host.  The program  below  looks

for  an  atsign  (the  normal DEC-20 prompt, hex 40).  DECSYSTEM-10 users would

look for a dot, hex 2E; VAX/VMS or UNIX users would look for a dollar sign, hex

24; UNIX C-Shell users would look for a percent sign, hex 26.

1. For  CP/M  2.2 systems, connect to a floppy disk with plenty of free

space.  Run DDT and type in the following (the comments  should  not

be  typed  in;  they  are  there just to tell you what's happening):

(Note that this wont work for CP/M Plus or 3.0 systems!)

----------------------------------------------------------------

-a100                   ;Begin assembling code at 100

0100    LXI H,2FE       ;Where to store in memory

0103    SHLD 200        ;Keep pointer there

0106    MVI E,D         ;Get a CR

0108    MVI C,4         ;Output to PUNCH (send to HOST)

010A    CALL 5

010D    MVI C,3         ;Input from READER (read from HOST)

010F    CALL 5

0112    ANI 7F          ;Strip parity bit

0114    PUSH PSW        ;Save a and flags

0115    MOV E,A         ;Move char to E for echo

0116    MVI C,2         ;Output to screen

0118    CALL 5

011B    POP PSW         ;Restore A and flags

011C    CPI 40          ;(or 4E,24,26,etc) System prompt?

011E    JZ 127          ;Yes, have whole file in memory

0121    CALL 17A        ;No, store another byte

0124    JMP 10D         ;Read another byte

0127    MVI A,1A        ;Get a Control-Z (CP/M EOF mark)

0129    CALL 17A        ;Store it in memory

012C    LXI H,300       ;Get memory pointer

012F    SHLD 202        ;Store as DMA pointer

0132    LDA 201         ;Get 'HI' byte of memory pointer

0135    STA 200         ;and store it as 'LO' one

0138    XRA A

0139    STA 201         ;Zero 'HI' byte (slow *256)

013C    MVI C,16        ;Make NEW file

013E    LXI D,5C        ;With FCB1

0141    CALL 5

0144    CALL 15E        ;Write 128 bytes (sector)

0147    CALL 15E        ;Write another sector

014A    LXI H,FFFF      ;Get a 16-bit Minus One

014D    XCHG            ;into DE

014E    LHLD 200        ;Get 256-byte counter

0151    DAD D           ;Decrement

0152    SHLD 200        ;and store back

0155    MVI A,2         ;Check if

0157    CMP L           ; 256-byte counter down to offset

0158    JZ 183          ;Yes, we're done

015B    JMP 144         ;Keep writing..

015E    LHLD 202        ;Get file-pointer

0161    XCHG            ;into DE

0162    MVI C,1A        ;Set DMA-address

0164    CALL 5

15.4.2. Downloading Kermit-80                                          Page 379

-------------------------------------------------------------------------------

0167    MVI C,15        ;Write sector (128 bytes)

0169    LXI D,5C        ;using FCB1

016C    CALL 5

016F    LHLD 202        ;Get file-pointer

0172    LXI D,80        ;128-bytes

0175    DAD D           ;added to file-pointer

0176    SHLD 202        ;and save

0179    RET             ;and return

017A    LHLD 200        ;Get Memory-pointer

017D    MOV M,A         ;Store character

017E    INX H           ;Increment Pointer

017F    SHLD 200        ;and save

0182    RET             ;and return

0183    MVI C,10        ;CLOSE file

0185    LXI D,5C        ;using FCB1

0188    CALL 5

018B    JMP 0           ;Force WARM BOOT

0179

-^C                     ;(Type Control-C) Return to CP/M

A>SAVE 1 FETCH.COM      ;Save program, we need to run it twice.

Figure 15-1:   Bootstrap program for Kermit-80 and CP/M Version 2.2

----------------------------------------------------------------

Alternatively, an assembler source file for  this  program  is  dis-

tributed  with  CP/M Kermit as CPKFET.ASM.  You might prefer to type

the assembler version in and assemble and load it (ASM CPKFET,  LOAD

CPKFET,  or  MASM  CPKFET,  MLOAD  CPKFET), to let the assembler and

loader catch any typing errors.

2. Connect to your host  using  a  terminal  or  a  terminal  emulation

program.  Ensure that your host does not have your terminal in "page

mode" (does not pause at the end of each screenful).

3. Tell the host to display the first hex file (the  system-independent

module)  at your terminal, e.g. give a command like TYPE CPSKER.HEX,

without a terminating carriage return.

4. Return to your micro by switching the cable from the terminal to the

micro, or by terminating the micro's terminal program.

5. Make sure your IOBYTE is set so that RDR: and PUN: correspond to the

I/O port that is connected to the host (this would normally  be  the

case unless you have done something special to change things).

6. Load  the program you entered in the first step with DDT, and use it

to capture the first hex file:

DDT FETCH.COM

-icpsker.hex            ;Setup FCB for file CPSKER.HEX

-g100,179               ;Execute the program.

Now there should be a file CPSKER.HEX on your connected disk.

7. Return to the host, and tell it to display the second hex file  (the

system-dependent module for your configuration).  Again, do not type

15.4.2. Downloading Kermit-80                                          Page 380

-------------------------------------------------------------------------------

the terminating carriage return.

8. Return to your micro, and run the capture program again:

DDT FETCH.COM

-icpxovl.hex            ;Setup FCB to create CPXOVL.HEX

-g100,179               ;Execute the program.

Now there should be  a  file  CPXOVL.HEX  on  your  connected  disk.

Replace CPXOVL.HEX in this example with the appropriate overlay file

for your system.

Merging the files:

1. For purposes of illustration, we will  assume  the  system-dependent

overlay  is  called "cpxovl.hex".  The two hex files may be combined

with MLOAD or DDT.  If you already have a running  Kermit,  you  can

transfer  MLOAD.HEX  to  your system and create MLOAD.COM by running

LOAD.  If you're bootstrapping Kermit, you could transfer  MLOAD.HEX

to  your  system  the same way you got the other two .HEX files, but

it's probably simpler to use DDT to  get  Kermit  running,  and  get

MLOAD later if you need it.

2. Using MLOAD, the two pieces may be easily merged:

A>mload kermit49=cpsker,cpxovl

(Some messages about program size, etc...)

A>

3. If you don't have MLOAD running, it's a bit more complex:

A>ddt cpsker.hex

NEXT  PC

3500 0100

-icpxovl.hex

-r

NEXT  PC

xxxx 0000

-^C

A>save dd kermit49.com

The  page  count  ("dd") used in the SAVE command is calculated from

the last address ("xxxx") given by DDT in response to the R command:

drop  the last two digits and add 1 if they were not zero, then con-

vert from hexadecimal (base 16) to decimal (base 10):  684F  becomes

69 hex, which is 105 decimal (5 times 16 plus 9) -- but 6700 becomes

67 hex, or 103 decimal (consult an introductory  computing  book  if

you don't understand number base conversion).

4. If  you  are  using the Z80MU CP/M and Z80 development toolkit on an

IBM PC or clone, then follow the same instructions as for a  genuine

CP/M  system.  When you have loaded your file, you will have to ship

the .COM or two .HEX files to  the  target  CP/M  system.  (Possibly

using a previous issue of Kermit?)

5. Note that CP/M hex files have checksums on each line.  If there were

15.4.2. Downloading Kermit-80                                          Page 381

-------------------------------------------------------------------------------

any transmission errors during the downloading process, MLOAD or DDT

will  notice a bad checksum and will report an error (something like

"Illegal Format").  If you get any errors during loading, either fix

the hex file locally with an editor, or repeat the transfer.

You now should have a running version of Kermit-80, called KERMIT49.COM.

Test  your new Kermit by running it.  If it gives you a prompt, it might be OK.

(don't delete your old one yet...).  Instead of a prompt, you could get one  of

two messages indicating that the configuration information is invalid:

?Kermit has not been configured for a target system

or

?Consistency check on configuration failed

Of  course,  neither  of these messages should appear if you're building Kermit

from the distribution kit.  The first message indicates that  the  overlay  was

not found where the system-independent module expected to find it, probably be-

cause the overlay address is incorrect; the second indicates that  the  version

of  CPXLNK used in the system-dependent module is incompatible with the system-

independent module.

Once you are satisfied that KERMIT40 works correctly, you  should  rename  your

old  KERMIT.COM to something else, like OKERMIT.COM, and rename KERMIT40.COM to

KERMIT.COM.

15.4.3. Assembling Kermit-80 from the sources

Kermit-80 is built in two pieces from the following files:

The system-independent files:

CPSKER.ASM    header file

CPSDEF.ASM    definitions for both KERMIT and KERSYS

CPSMIT.ASM    initialization, main loop, miscellaneous commands  (BYE,  EXIT,

LOG, SET, SHOW, STATUS, and VERSION)

CPSCOM.ASM    second part of commands, status and set file

CPSPK1.ASM    part   1   of  the  KERMIT  protocol  handler  (SEND,  RECEIVE,

LOGOUT,and FINISH commands)

CPSPK2.ASM    part 2 of the KERMIT protocol handler

CPSREM.ASM    REMOTE routines (FINISH, BYE and LOGOUT in CPXPK*.ASM)

CPSSER.ASM    SERVER routines (for the future)

CPSTT.ASM     the transparent commands (TRANSMIT, CONNECT)

CPSCPM.ASM    CP/M commands (DIR, ERA, USER, TYPE, PRINT, COPY)

CPSWLD.ASM    the wildcard handler

CPSCMD.ASM    the command parser

CPSUTL.ASM    utility routines and data

CPSDAT.ASM    data space and the overlay definitions

CPXLNK.ASM    linkage area description

The system-dependent files:

CPXTYP.ASM    system selection

CPXLNK.ASM    system overlay specification and jump table

CPXCOM.ASM    common routines for all systems

15.4.3. Assembling Kermit-80 from the sources                          Page 382

-------------------------------------------------------------------------------

CPXSWT.ASM    system selector or switcher

One of:

CPXSYS.ASM    family file for some system-specific code

CPXTOR.ASM    family file for Torch, Superbrain, PCI2651 etc

CPXNOR.ASM    family file for Northstar and Comart machines

CPXMRL.ASM    family file for British Telecom merlin/Rair Black Box

CPXSB.ASM     family file for Micromint SB-180 systems

CPXCIF.ASM    family file for Cifer systems

CPXHEA.ASM    family file for Heath/Zenith systems

CPXAPP.ASM    family file for Apple II systems

CPXPCW.ASM    family file for Amstrad PCW 8256/8512 machines

CPXBBI.ASM    family file for BigBoard, Kaypro and Xerox 820 systems

CPXSYO.ASM    family file for Sanyo MBS-1100 systems

CPXTM4.ASM    family file for Tandy Model 4 with CP/M systems

CPXGNI.ASM    family file for Video Genie systems

CPXPRO.ASM    family file for Compupro systems

CPXZ80.ASM    family file for the Z80MU development system

and if you use a terminal,

CPXVDU.ASM    display codes for VDUs etc.  Not always required

The system-independent module contains all of the system-independent files  ex-

cept  for  CPXLNK.ASM,  which  is assembled into the system-dependent module to

provide the structures needed to connect the two modules.  As distributed,  the

system-independent  module  is  named  CPSKER.HEX.    If  you  have  a  copy of

CPSKER.HEX, you do not need to reassemble the system-independent module to con-

figure Kermit for your system.

The  system-dependent  module  consists  of CPXTYP.ASM, CPSDEF.ASM, CPXLNK.ASM,

CPXSWT.ASM,  CPSCOM.ASM,  one  of  the  family  files  CPXSYS.ASM,  CPXTOR.ASM,

CPXMRL.ASM,   CPXSB.ASM,   CPXCIF.ASM,   CPXHEA.ASM,   CPXBBI.ASM,  CPXTM4.ASM,

CPXGNI.ASM, CPXNOR.ASM, CPXAPP.ASM, CPXPCW.ASM,  or  CPXPRO.ASM,  and  possibly

CPXVDU.ASM,  if  your  system uses a terminal for the console.  One copy of the

system-dependent module is supplied already assembled for each  supported  sys-

tem;  the filename may be obtained from tables 15-2 and 15-3.  If a terminal is

required for a system, a CRT (glass TTY device) has been selected.

After assembling the two pieces separately, they are combined with DDT or MLOAD

into a system-specific Kermit.

If  you  want to rebuild the system-independent module, the only change you may

need to make is to select the assembler to be used, in CPSKER.ASM.  Define  one

of MAC80, M80, or LASM to TRUE to select it as the assembler; the others should

be defined FALSE.

Assuming you have the Microsoft Macro Assembler package (M80/L80), you'll  need

to do the following:

A>m80 cpsker=cpsker.asm

A>l80 /p:100,cpsker,cpsker/n/e

This will produce CPSKER.COM.

If you are using LASM instead, do this:

15.4.3. Assembling Kermit-80 from the sources                          Page 383

-------------------------------------------------------------------------------

A>lasm cpsker

LASM  will  generate  CPSKER.HEX  and  CPSKER.PRN.    LASM allows options to be

specified in the same way as the standard assembler, ASM, so the command

A>lasm cpsker.abz

will read the source files from drive A, send the .HEX file  to  drive  B,  and

suppress the listing file.

If  you  are using the Z80MU development system on an IBM PC or clone, then as-

semble your files using either LASM and MLOAD or M80 and L80, as  if  you  were

using  a  genuine CP/M-80 system.  Note that you will still have the problem of

transferring your assembled files to the target CP/M system.

If you want to generate a system-dependent overlay for a particular system,  or

want  to  change  the  terminal  supported, you'll need to check three areas in

CPXTYP.ASM:

First, the overlay start ADDRESS.  The symbol "ovladr" is EQUated  to  the  ad-

dress  of "LNKFLG" in the system-independent module, as the starting address of

the overlay (6000H for version 4.09).  You'll need to know this value if you're

building  the  overlay  with  M80/L80.   You won't normally need to change this

value.

Second, the assembler being used.  Again, define one of MAC80, M80, and LASM to

be  TRUE  to  select  it,  and  define the others to be FALSE.  The two modules

(system-independent and system-dependent) do not need to be built with the same

assembler.

Third,  the  system configuration.  Locate your system in tables 15-2 and 15-3,

then define the appropriate symbol TRUE, and the rest FALSE.    If  the  system

comes  with a builtin console terminal, define all the terminal switches FALSE.

If the system uses an external terminal as the console, locate the terminal  in

table 15-5 and define the appropriate symbol TRUE, and the remainder FALSE.  If

the terminal is not listed in table 15-5, use the CRT  switch;  in  this  case,

VT52 emulation is not supported.

In  addition,  there are a few general and system-specific symbols which may be

altered to fit your system:

APSLOT          For Apple with 6551 ACIA, defines the slot number of the serial

card

CPUSPD          Processor  speed  in  units  of 100KHz (currently used only for

bbII and kpII for timing loops)

TAC             For users connecting through ARPAnet TACs: set to TRUE  if  you

wish the default TACTRAP status to be ON. (This may be overrid-

den with the SET TACTRAP command).  If  you're  not  connecting

through a TAC, set tac to FALSE and ignore tacval.

TACVAL          For  ARPANET  TAC  users:  defines  the  default  TAC intercept

character (may be overridden with the SET TACTRAP command).

If you are just assembling an  existing  configuration,  you'll  need  to  edit

15.4.3. Assembling Kermit-80 from the sources                          Page 384

-------------------------------------------------------------------------------

CPXTYP.ASM  only.    If you are adding support for a new system, you should not

modify CPSDEF.ASM or CPXLNK.ASM; if you do, you'll have to change  the  system-

independent  module  also.   Eventually, CPXSYS.ASM will be split into separate

files, each of which will generate one or more related systems.  When this hap-

pens,  you'll  want to pick the one closest to your system to use as a starting

point.

After editing CPXTYP.ASM as necessary, assemble and link the  overlay  as  fol-

lows:

- With M80 (where "xxxx" is the hex value of ovladr from CPXLNK.ASM):

A>m80 cpxtyp=cpxtyp.asm

A>l80 /p:xxxx,cpxtyp,cpxtyp/n/x/e

- With LASM:

A>lasm cpxtyp

With an IBM PC or clone using the Z80MU softwrae, follow the instructions as if

you were using a real CP/M system.

The overlay (CPXTYP.HEX) may then be merged with the system-independent  module

as described above (creating a runnable Kermit from the distribution kit).

If  you are using the Z80MU development system on a PC, and already have a run-

ning Kermit-80 v3.9 or later, you can merge the two .HEX files into a .COM file

with LINK80 (TOPS 10/20), MLOAD (Z80MU), L80 (Z80MU), and transfer the new .COM

file to your micro with Kermit:

- Z80MU on a PC and MLOAD:

@MLOAD KERNEW=CPSKER,CPXTYP

- Z80MU on a PC and C80:

@L80 /P:xxxx,CPXTYP,CPXTYP/N/X/E

producing KERNEW.COM.

15.5. Adding Support For A New System

Kermit-80 is built from a common set  of  source  files;  the  system-dependent

module  makes  heavy  use  of  conditional  assembly (this complication will be

removed in future releases).  The system dependencies arise  from  attempts  to

answer some questions:

1. What kind of terminal is to be supported?

For  many micros, the console is an integral part of the system, but

others can use an external terminal.  In either case,  the  commands

to  manipulate  the  screen  (position the cursor, erase the screen,

etc) must be defined.

2. How is the serial line accessed?

15.5. Adding Support For A New System                                  Page 385

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Symbol         Terminal description

crt            Basic CRT, no cursor positioning

adm3a          ADM3A Display or lookalike

adm22          ADM22 Display or lookalike

am230          Ampro 230

h1500          Hazeltine 1500

smrtvd         Netronics Smartvid-80

soroq          Soroq IQ-120

tvi912         TVI 912

tvi925         TVI 925, Freedom 100

vt52           VT 52 or VT52 emulator such as Heath H19, H29, etc.

vt100          VT 100 or emulator (most ANSI terminals should work)

wyse           Wyse 100

Table 15-5:   Terminals known to Kermit-80

-------------------------------------------------------------------------------

For systems supporting the IOBYTE function, this is straightforward;

the  symbol "IOBYT" is defined TRUE.  If the serial line is accessed

with IN and OUT instructions, it may be possible to use  the  simple

I/O  routines provided.  In this case, the symbol "INOUT" is defined

TRUE, the MNPORT and MNPRTS are defined to be the data  and  control

addresses,  respectively,  and bit masks for testing for "input data

available" and "output buffer empty" must be defined.  If the inter-

face is strange, leave IOBYT and INOUT set to FALSE, and provide the

I/O routines.

3. What initialization is necessary?

You may wish to set the baud rate or configure the  serial  line  at

startup.  Examples for a number of devices are present.

4. What special features are to be supported?

You  may  want  to  provide  the capability to select one of several

serial lines with the SET PORT command, or to change  the  speed  of

the  serial line with the SET BAUD-RATE command.  To do this, you'll

need to build a command table, using the systems  already  supported

as  examples.    The  ability  to  send a BREAK signal is desirable.

Again, examples for several different interfaces  (ACIA,  SIO,  etc)

are present.

5. Do you want to design an external terminal type?

There  is  a  jump  entry  in the overlay file to allow users to add

their own termainl emulator.  If you write  the  code  for  such  an

emulator,  you  must load this jump address with the address of your

emulator, and SET TERMINAL EXTERNAL from within Kermit.  All charac-

ters will be passed to this routine during connect mode.

15.6. Notes on New Features in Kermit-80 Version 4                     Page 386

-------------------------------------------------------------------------------

15.6. Notes on New Features in Kermit-80 Version 4

- Debugging aids:  SET DEBUG ON will add two fields to the SEND/RECEIVE

display, labelled "Spack" and "Rpack".  These display the last packet

sent  and  received.   Of course, this slows down the transfer, espe-

cially if the console is an external terminal.  SET DEBUG OFF removes

these  fields.    The VERSION command displays the name, edit number,

and edit date of several of the modules that make up Kermit.

- TAC support: ARPAnet TACs (and many other communication devices  such

as  terminal  concentrators,  modems,  port contention units, network

PADs, etc) use a  printing  character  (like  "@")  as  an  intercept

character,  to allow commands to be issued to the TAC, or modem, etc.

In order to send this character to the host, it must be typed  twice.

The  command  "SET TAC CHARACTER" to Kermit enables the "TACtrap" and

asks the user to specify the TAC intercept character.  This character

will be automatically doubled when it appears in Kermit protocol mes-

sages (sent by the SEND or RECEIVE commands) or when it appears in  a

file  being  sent with the TRANSMIT command.  It is not automatically

doubled when typed by the user in CONNECT mode.  "SET TAC ON" enables

the TACtrap but does not change the TAC intercept character, which is

initially "@".  "SET TAC OFF" disables the TACtrap.

- File buffering:  Previous versions of  Kermit-80  buffered  only  one

sector  (128  bytes) at a time during file transfer operations.  This

version buffers 16Kbytes at a time, reducing the number of times  the

floppy  drive  must be spun up and down, and increasing the effective

throughput of the link.  If the  disk  transfer  rate  is  too  slow,

however, the remote Kermit may time out and retransmit packets.  This

will show up on the screen in the "Retries:" field;  if  this  occurs

after  disk  activity,  you may want to increase the timeout value on

the remote Kermit, SET BUFFER <new value> while  in  Kermit,or  reas-

semble  Kermit  with  a smaller value for MAXSEC (in CPSDEF.ASM) This

buffer is also used by the TRANSMIT command; the log file enabled  by

the LOG command is still written a sector at a time.

This  section is intended for people wanting to implement their own versions of

Kermit-80 for computers not already defined.

The system independent code communicates to  routines  for  a  specific  system

through  a  set  of tables.  These tables are defined in CPXLNK.ASM, and should

not be modified between revisions of Kermit.  If an entry is added or  deleted,

then  the whole of Kermit-80 needs reassembling.  Make sure that the changes to

CPXLNK.ASM are duplicated in  CPSUTL.ASM,  which  has  the  system  independent

equivalent of CPXLNK.ASM.

The  following  entries/definitions  apply  to  revision 4.09.  There have been

three additional entries since revision 4.05.

The table is split into three sectors;  The  first  section  defines  two  byte

"words"  giving  16 bits of interface data; The second set is a set of jumps to

various functions, and finally the third set a set of pure data bytes.

15.6.1. Interface Data.                                                Page 387

-------------------------------------------------------------------------------

15.6.1. Interface Data.

LNKFLG  Must be first entry in overlay at overlay address.   Is  a  two

byte  address  giving  the  size of the linkage table.  This is

used to check for consistency of overlay's

ENTSIZE Length of entry table, also used for consistency checking after

the overlay.  Currently 6

SYSEDT  The  address  of  a dollar-terminated string giving the overlay

revision  level  and  date.    Points   to   a   string   like:

CPXSYS.ASM(33) 4-JUN-1986$

FAMILY  The  address  of  a  dollar-terminated string giving the Family

overlay  revision  level  and  date.    If  the  system  is  in

CPXSYS.ASM  rather  than  a  particular  Family  overlay, it is

simply a pointer to $

15.6.2. Jump Table.

This is split into three main sectors-

1. Input/Output routines

2. Screen formatting routines

3. other system dependent routines

SELMDM

Parameters    None

Returns       None

Description   selects the modem port.  Most systems  do  nothing  and  simply

return.  HL,DE and BC registers preserved.

OUTMDM

Parameters    None

Returns       None

Description   Output  the character in E register to the communications line.

BC,DE,HL registers preserved.

INPMDM

Parameters    None

Returns       Accumulator either 0 or character from comms line if available

Description   Check modem for character and if so, return it in  A.  HL,DE,BC

registers preserved, flags and accumulator lost.

FLSMDM

Parameters    None

Returns       None

Description   Clear  any  pending  characters  in  the  input buffer from the

modem.  No registers are preserved.

15.6.2. Jump Table.                                                    Page 388

-------------------------------------------------------------------------------

SELCON

Parameters    None

Returns       None

Description   Select the console.  This is a null subroutine  for  most  sys-

tems, but for IOBYTE systems selects the console.

OUTCON

Parameters    Character in E

Returns       None

Description   Send  the  character in E to the console.  Any quirks of system

responding in an odd manner should be handled.    No  registers

preserved.

INPCON

Parameters    None

Returns       Zero or character in A.

Description   Get a character from the console or return a null if no charac-

ter to be read.  No registers are preserved.

OUTLPT

Parameters    Character in E

Returns       None

Description   Send the character in  E  to  the  printer.    The  console  is

selected.  Only DE registers are preserved

LPTSTAT

Parameters    None

Returns       00H or 0FFH in A register

Description   Test  the  printer to see if it is ready to receive a character

to be printed.  If a 00H is returned then the printer is  ready

to receive a character.

EXTTER

Parameters    Character  to be sent to the user supplied terminal emulator in

the E register

Returns       None

Description   If the user has supplied a terminal  emulator  in  the  overlay

code, EXTTER will be a JMP <non zero address>.  If SET TERMINAL

EXTERNAL has been set, all caharcters will be  passed  verbatim

to  this  terminal emulator.  If there is no external emulator,

this code will never be called.  The user should reset terminal

conditions on initialisation of both the system and before CON-

NECT.  All registers should be preserved.

XBDOS

Parameters    Any required for calling BDOS

Returns       Any expected from the called BDOS routine

Description   This is an alternative entry to BDOS.   This  entry  will  also

check  the  printer  status etc.  For full details see the code

for the BDOS trap in CPSUTL.ASM.

2b)

CLRLIN

Parameters    None

15.6.2. Jump Table.                                                    Page 389

-------------------------------------------------------------------------------

Returns       None

Description   Clear the current line on the terminal

CLRSPC

Parameters    None

Returns       None

Description   Erase the current position (after a backspace)

DELCHR

Parameters    None

Returns       None

Description   Make delete (7FH) look like a backspace.   Some  systems  do  a

backspace,  space, backspace automatically others have to simu-

late it

CLRTOP

Parameters    None

Returns       None

Description   Clear the screen and place the cursor at the top LH corner

SCREND

Parameters    None

Returns       None

Description   Place the cursor on the line for the Kermit-80 prompt  after  a

file transfer.  (Usually line 13)

SCRERR

Parameters    None

Returns       None

Description   Move  cursor  to  the  error message field on the file transfer

format screen

SCRFLN

Parameters    None

Returns       None

Description   Move the cursor to the filename field

SCRNP

Parameters    None

Returns       None

Description   Move the cursor to the packet count field

SCRNRT

Parameters    None

Returns       None

Description   Move cursor to the retry count field

SCRST

Parameters    None

Returns       None

Description   Move cursor to the status field

RPPOS

Parameters    None

Returns       None

Description   Move to the receive packet field (debugging use)

15.6.2. Jump Table.                                                    Page 390

-------------------------------------------------------------------------------

SPPOS

Parameters    None

Returns       None

Description   Move to the send packet field (for debugging use)

2c)

SYSINIT

Parameters    None

Returns       None

Description   Initialize  the  system  specific  items.    No  registers  are

preserved.  Any initialization is done once only when Kermit-80

is first loaded.

SYSEXIT

Parameters    None

Returns       None

Description   Program termination.  De-initialize anything in preparation for

a return to CP/M

SYSCON

Parameters    None

Returns       None

Description   Initialize anything before entering the connect state.

SYSCLS

Parameters    None

Returns       None

Description   System dependent close routine when exiting connect state

SYSINH

Parameters    None

Returns       None

Description   Help  routine  to  test  for  any extensions to the escape menu

during the connect state.  If a system has any special  feature

it  can  use  during  connect  mode,  then  it can be tested as

<escape-character>xxx.  This entry is a string for printing  to

the console for an <escape-character>?  Often used for generat-

ing breaks or controlling a modem.

SYSINT

Parameters    None

Returns       None

Description   This is a test-and-jump on receipt of an  escape  sequence  not

understood  by  Kermit-80.  If the character in A is not recog-

nized by your version of Kermit=80, do a rskip

SYSFLT

Parameters    Character in E

Returns       Character in E. Either a 00H or anything else in A

Description   Test the character in E. If it may not be printed to  the  con-

sole, set A to zero.  All other registers preserved.

NB <XON>,<XOFF>,<DEL>,<NULL> are always rejected.

SYSBYE

15.6.2. Jump Table.                                                    Page 391

-------------------------------------------------------------------------------

Parameters    None

Returns       None

Description   System  dependent  processing for the BYE command.  (eg hang up

the phone)

SYSSPD

Parameters    Value from table in DE

Returns       None

Description   The system dependent code for baud rate change.    DE  contains

the  two  byte  value  from the baud rate table.  This value is

also stored in "SPEED"

SYSPRT

Parameters    Value in DE

Returns       None

Description   The system dependent code for setting the port.  The parameters

are passed in DE, which are obtained from the port tables

SYSSCR

Parameters    String pointer in DE

Returns       None

Description   Setup the screen display for file transfer.  The Kermit version

string is pointed to by DE.  If the terminal is not capable  of

cursor  addressing  (eg dumb glass TTY) then only the screen is

cleared and the version string is printed.

CSRPOS

Parameters    Row number in B, column number in C

Returns       None

Description   Move the cursor to row B, column C  where  B=1,C=1  is  top  LH

corner  of  screen.    The  routine  should first end a "cursor

position" leading string (up to four characters) then  use  the

parameters given to complete the versions cursor position func-

tion

SYSSPC

Parameters    None

Returns       K bytes free in HL

Description   Get the amount of free disk space on the selected  disk  drive.

This  could  be  in the system independent code.  Automatically

detects CP/M V2.2 or V3.0.  No registers saved.

MOVER

Parameters    Source Pointer in HL

Destination Pointer in DE

Byte count in BC

Returns       None

Description   Move (BC) bytes from (HL) to (DE) Z80 based systems do an LDIR,

while 8080 systems do it as a loop.  All registers destroyed

PRTSTR

Parameters    $ terminated string pointed to by DE

Returns       None

Description   Print the string onto the console.

3)

15.6.2. Jump Table.                                                    Page 392

-------------------------------------------------------------------------------

PTTAB         WORD      Points to VT52 equivalent escape sequences.

SPDTAB        WORD      Address  of  baud-rate  command  table, or 0 if table

does not exist

SPDHLP        WORD      Address  of  baud-rate  help  table,  or  0  if   SET

BAUD-RATE is not supported.

PRTTAB        WORD      Address of port command table or 0 if SET PORT is not

supported.

PRTHLP        WORD      Address of port help table or 0 if SET  PORT  is  not

supported

TIMOUT        BYTE      FUZZY-TIMER.    Set  to value suitable to your system

(depends largely on CPU speed)

VTFLG         BYTE      VT52 emulation flag.  Set to 0 if terminal emulates a

VT52,  01  if  emulation is required, or 0FFH if emulations not

possible (eg for "CRT")

ESCCHR        BYTE      default escape character-usually control-] but  some-

times control-\

SPEED         WORD      Storage  space  for baud-rate.  Set to 0FFFFH as baud

rates are initially unknown.  Note that the STATUS routine only

looks at the first (least significant) byte.

PORT          WORD      Storage  space  for port.  Set to 0FFFFH as ports may

not be implemented, and is initially unknown

PRNFLG        BYTE      Printer copy flag-if O no copy.    Anything  else  =>

copy to printer

DBGFLG        BYTE      Debugging  flag.   If O then no debugging to be done.

(ie writing of debugging info during a file transfer)

ECOFLG        BYTE      Local ECHO flag (default is off)

FLWFLG        BYTE      File warning flag.  If set to 1  will  not  overwrite

files  already  existing  on  disk  with some-named files being

transferred

IBMFLG        BYTE      IBM system is the host-assume IBM file transfers etc

CPMFLG        BYTE      Flag indicating type of CP/M files to be transferred.

Default setting - DEFAULT

PARITY        BYTE      Type of parity in use

0 = Even parity

3 = Mark parity

6 = No parity (8th bit is data)

9 = Odd parity

12 = Space parity

15.6.2. Jump Table.                                                    Page 393

-------------------------------------------------------------------------------

SPSIZ         BYTE      Size of send packet

RPSIZ         BYTE      Size of receive packet

STIME         BYTE      Send timer (time-out)

RTIME         BYTE      Receive timer (time-out)

SPAD          BYTE      Send Padding (default=0)

RPAD          BYTE      Receive Padding (default=0)

SPADCH        BYTE      Send Padding character (default=NULL)

RPADCH        BYTE      Receive Padding character (default=NULC)

SEOL          BYTE      Send EOL character (default=CR)

REOL          BYTE      Receive EOL character (default=CR)

SQUOTE        BYTE      Send quote character (default=#)

RQUOTE        BYTE      Receive quote character (default=#)

CHKTYP        BYTE      Ascii value of checktype

31H="1"=checktype1 (6bits)

32H="2"=checktype2 (12bits)

33H="3"=CCITT checksum (CRC)

Default is 31H("1")

TACFLG        BYTE      If  set to on (non zero) send the TACCHR twice.  This

is for ARPA TAC users, where  the  TAC  swallows  one  "wakeup"

character.    If sent twice the TAC will pas one on and go back

to normal mode.

TACCHR        BYTE      Desired TAC character.  It is ignored if TAC trapping

is  turned  off. Value to send twice if TAC interception is set

on.  Default=0, but set to commercial AT if the conditional as-

sembly flag TAC is set true

BUFADR        WORD      Address of Multi-Sector buffering for I/O

BUFSEC        BYTE      The  number  of  bytes  the  big  buffers  can  hold.

Default is 1. (0=256 sectors).

FFUSSY        BYTE      Indicates if funny characters may  be  used  in  CP/M

file  names (eg <>.,;?#[]) If zero, allow anything.  Default is

nonzero.

BMAX          SPACE:(2bytes) Highest block number on selected disk drive

BMASK         SPACE:(1byte) (Records/block)-1

BSHIFTF       SPACE:(1byte) Number of shifts to multiply by rec.block

15.6.2. Jump Table.                                                    Page 394

-------------------------------------------------------------------------------

NNAMS         SPACE:(1byte) Counter for file-names per line

15.7. Future Work

Work that needs to be done in future releases includes:

- Merge in support for additional CP/M-80 systems,  particularly  those

for which support was recently added to the monolithic v3.x source.

- Break  up  CPXSYS  into  discrete  source files, one for each system.

These source files should serve as simple models for  adding  support

for  new  systems  to Kermit-80 -- only the very basic screen defini-

tions, flags, i/o primitives, initializations, and  so  forth  should

appear in each system-dependent file.

- Addition  of  missing  features -- compression of repeated characters

during  packet  transmission,   transmission   of   file   attributes

(particularly  size, so that "percent done" can be displayed for both

incoming and outbound files), advanced commands for  servers  (REMOTE

DIRECTORY,  etc),  command  macros  and  initialization  files, login

scripts, remote operation and server mode, etc etc.  Any offers??

16. CP/M-86 KERMIT                                                     Page 395

-------------------------------------------------------------------------------

16. CP/M-86 KERMIT

Authors:    Bill Catchings, Columbia University; Ron  Blanford,  University  of

Washington; Richard Garland, Columbia University.

Language:   Digital Research ASM86

Version:    2.9

Date:       December 1984

Documentation:

Frank da Cruz, Columbia

This version of KERMIT is designed to support any CP/M-86 system.   So  far  it

supports  the DEC Rainbow-100 and the NEC Advanced Personal Computer (APC).  It

is very similar to CP/M-80 and MS DOS KERMIT.

CP/M-86 KERMIT-86 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  No

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            No

Alternate block checks:            No

Terminal emulation:                Yes, uses PC firmware (VT100)

Communication settings:            Yes; duplex, parity

Transmit BREAK:                    Yes

IBM communication:                 Yes

Transaction logging:               No

Session logging (raw download):    Yes

Raw upload:                        No

Act as server:                     No

Talk to server:                    Yes; SEND, GET, FIN, BYE

Advanced commands for servers:     No

Local file management:             Yes

Handle file attributes:            No

Command/init files:                Yes

Printer control:                   No

CP/M-86 KERMIT DESCRIPTION

Since Kermit-86 runs on a standalone micro, it is  always  in  control  of  the

screen  --  it  is always local.  Thus, it always keeps the screen updated with

the file name and the packet number, whether sending or receiving.    Kermit-86

is  capable  of  timing  out  an  input  request,  and can thus break deadlocks

automatically.  In most cases, however, this is not desirable because the  KER-

MIT on the other side is most likely better able to handle the timeouts; there-

fore, Kermit-86's timer is normally not used.

If despite the timeout capability, the transmission appears to  be  stuck  (and

you  can  tell  that this has happened if the screen fails to change for a long

16. CP/M-86 KERMIT                                                     Page 396

-------------------------------------------------------------------------------

while) you can type carriage return to have the micro do  what  it  would  have

done  on a  timeout, namely NAK the expected packet to cause to foreign host to

send it again (or, if the micro is sending, to  retransmit  the  last  packet).

Micro/micro  or micro/IBM-mainframe transfers could require this kind of manual

intervention.

File transfers may be interrupted in several ways.

Control-C       This will return you to Kermit-86 command level immediately, so

that  you  can  connect  back to the remote system, or take any

other desired action.

Control-X       When sending a file, this will terminate  the  sending  of  the

current  file  with a signal to the KERMIT on the other side to

discard what it got so far.  If there  are  more  files  to  be

sent,  KERMIT-86  will go on to the next one.  When receiving a

file, KERMIT-86 will send a signal to the remote KERMIT to stop

sending  this file.  If the remote KERMIT understands this sig-

nal (not all implementations of KERMIT  do),  it  will  comply,

otherwise  the  file  will  keep  coming.   In either case, the

remote KERMIT will go on to the next file in the group, if any.

Control-Z       Like Control-X, except if a file group  is  being  transmitted,

this will stop the transmission of the entire group.  If only a

single  file  is  being  transmitted,  it  works  exactly  like

Control-X.

Carriage Returns

If you type carriage return repeatedly Kermit-86 will retry the

current  packet  up to its retry limit (somewhere between 5 and

16 times) and then, if no valid response was  received,  return

to Kermit-86 command level.

When  KERMIT-86 is started, it looks for the file KERMIT.INI.  If found, it ex-

ecutes KERMIT-86 commands from it before  prompting  you  for  commands.    The

KERMIT-86 prompt looks like this:

Kermit-86 B3>

in  which  "B" is your current default disk and "3" is the current default user

number.

16.1. Kermit-86 Commands

KERMIT-86 uses the DECSYSTEM-20 keyword style command language.   Each  keyword

may be abbreviated to its minumum unique length.  "?" may be typed to request a

menu of the available options for the current field at any point in a  command.

ESC  may  be typed at any point in a command to fill out the current keyword or

filename; if sufficient characters have not been typed to identify the  current

field uniquely, KERMIT-86 will sound a beep and allow you to continue from that

point.

CONNECT Establish a "virtual terminal" connection to any host that may be  con-

nected  to the serial port, i.e. pass all typein to the serial port and

display all input from  the  serial  port  on  the  screen,  using  the

16.1. Kermit-86 Commands                                               Page 397

-------------------------------------------------------------------------------

system's  own  built-in  support  for ANSI (VT100-like) screen control.

When you issue the CONNECT command, the PC will print a message telling

you how to get back by typing an an escape sequence, an uncommonly-used

control character, normally CTRL-backslash, followed by a single letter

"command".

C   Close Connection, return to Kermit-86> command level.

?   List available single-character commands.

B   Send a BREAK signal.

Q   Quit logging the remote session.

R   Resume logging the remote session.

L   Toggle logging.

^\  (or  whatever - a second copy of the escape character) Send the es-

cape character itself to the remote host.

SEND filespec

Send  file(s)  specified  by  filespec  to the remote Kermit, using the

prevailing file mode (ASCII or BINARY; see SET).  The filespec may con-

tain CP/M wildcards.

RECEIVE Receive  file(s)  from  the  remote Kermit.  Store them under the names

provided in the file headers supplied by the remote host.  If the names

aren't  legal,  use  as many legal characters from the name as possible

(see the description of SET FILE-WARNING below).   If  there's  a  con-

flict,  and FILE-WARNING is ON, warn the user and try to build a unique

name for the file by adding "&" characters to the name.  You  may  also

provide  an  optional  file name in the RECEIVE command; if you do, the

incoming file will be stored under the name you specify.  If more  than

one  file  arrives, only the first will be stored under the given name,

unless you included wildcard characters in  the  RECEIVE  filespec;  in

that  case, the filespec will be used as a mask for incoming filenames.

For instance, you told the remote Kermit to send *.ASM, you could  tell

KERMIT-86  to "receive *.A86", thereby changing the filetype of all the

incoming files.

GET filespec

When  Kermit-86  is  talking to a Kermit Server on the host, you should

use the GET command to request the server to send files to you, for ex-

ample: get hlp:k*.hlp

BYE     When  talking  to  a  remote Kermit Server, this command shuts down the

server and logs it out, and also exits from Kermit-86 to  CP/M  command

level.

LOGOUT  Like BYE, but leaves you at Kermit-86 command level.

FINISH  Like  LOGOUT,  but shuts down the remote server without logging it out.

Leaves you at Kermit-86 command level;  a  subsequent  CONNECT  command

should put you back at host system command level.

EXIT    Exit from KERMIT-86 back to CP/M.

QUIT    Synonym for EXIT.

SET parameter [value]

Set the specified parameter to the specified value.  Possible settings:

16.1. Kermit-86 Commands                                               Page 398

-------------------------------------------------------------------------------

BAUD    Change the baud rate of the communications port.  This  command

only  works  on some systems, and its actual operation can vary

from system to system.  Type SET BAUD followed  by  a  question

mark,  and  follow the directions.  On systems that do not sup-

port this command, you must set the port baud rate from CP/M or

other setup mechanism outside of KERMIT-86.

DEBUG   ON  or  OFF.    If  ON,  displays incoming and outbound packets

during file transfer.  OFF by default.

DEFAULT-DISK disk/user

Specify default disk and user number for subsequent file recep-

tion and transmission.  The specification following the command

must be in one of the following forms:

d:  = go to drive d (A through P) without changing user

u:  = go to user u (0 through 15) without changing drive

du: = go to drive d and user u

:   = go to the defaults when Kermit was loaded

Whenever  a  drive  is specified, even if it is the same as the

current default drive, the drive is logged in so that disks can

be  swapped  without  exiting Kermit to type control-C.  Kermit

restores the original drive and user upon termination.

ESCAPE  Change the escape character for virtual  terminal  connections.

Select  a  character  in the control range that you will not be

likely to need at the  remote  host;  type  the  new  character

literally.    Certain  characters,  like  Control-X,  cannot be

specified.

FILE-TYPE

Tells KERMIT-86 what kind of file it is sending, so that KERMIT

can correctly determine the end of the file.  SET  FILE  BINARY

means  to  send  all the 128-byte blocks of the file, including

the last block in its entirety; SET FILE ASCII is used for text

files,  and  transmission stops when the first Control-Z is en-

countered anywhere in the file (this is the CP/M convention for

marking  the  end  of  a text file).  If binary transmission is

used on a text file, some extraneous characters (up to  127  of

them)  may  appear at the end of the file on the target system.

If ASCII transmission is used on a binary file, the entire file

will  not  be sent if it happens to contain any data bytes that

correspond to Control-Z.  ASCII is the default.

FLOW-CONTROL

Select  the desired type of flow control to be used on the com-

munication line.  The choices are NONE and XON/XOFF.   XON/XOFF

is  the  default.    If the remote system is not full duplex or

cannot do XON/XOFF, you should use NONE.

IBM ON (or OFF)

Allow  the  transfer of files to and from an IBM mainframe com-

puter.  This  makes  Kermit-86  wait  for  the  IBM  turnaround

character (XON), ignore parity on input, add appropriate parity

to output, and use local  echoing  during  CONNECT.    As  dis-

16.1. Kermit-86 Commands                                               Page 399

-------------------------------------------------------------------------------

tributed, KERMIT-86 uses MARK parity for IBM communication.  If

you don't give this command, IBM mode is OFF.  Since IBM VM/CMS

KERMIT  does not have timeout capability, SET IBM ON also turns

on the timeout facility automatically, as if you had typed "SET

TIMER ON".

LOCAL-ECHO ON (or OFF)

When you CONNECT to a remote host, you must set  LOCAL-ECHO  ON

if  the  host  is  half  duplex,  OFF  if  full duplex.  OFF by

default.

LOG     Specify a log file on the  current  CP/M  disk  into  which  to

record  incoming characters during CONNECT.  If the remote host

can do XON/XOFF, then the log file will normally capture  every

character  shown  on  the screen.  When connected to the remote

system, several single-character arguments to the  connect  es-

cape  character  can  be used to control logging -- Q (quit), R

(resume), L (toggle).  If you use R or L during connect without

having previously specified a log file name, then KERMIT.LOG is

used.  An open log is closed when you escape back to the PC.

PARITY  Sets parity for outgoing characters to one  of  the  following:

NONE,  SPACE, MARK, EVEN, or ODD.  On input, if parity is NONE,

then the 8th bit is kept (as data), otherwise  it  is  stripped

and  ignored.  The parity setting applies to both terminal con-

nection and file transfer.  If you set parity to anything other

than NONE, Kermit-86 will attempt to use "8th bit prefixing" to

transfer binary files.  If the other KERMIT is also capable  of

8th  bit  prefixing,  then binary files can be transferred suc-

cessfully; if not, the 8th bit of each data byte will  be  lost

(you will see a warning on your screen if this happens).

PORT    Allows  you  to switch between different communication ports on

the PC.  This command is not available on all systems.

TIMER ON (or OFF)

Enable  or  disable  the timeout facility.  The timer is off by

default, because in the normal case KERMIT-86 is  communicating

with a mainframe KERMIT that has its own timer.  Mainframe KER-

MIT timers tend to be more precise  or  adaptable  to  changing

conditions.    You should SET TIMER ON if you are communicating

with another KERMIT that does not have a timer.  You should SET

TIMER  OFF  if  you  are communicating over a network with long

delays.

WARNING ON (or OFF)

Warn  user  of  filename  conflicts  when  receiving files from

remote host, and attempt to generate a unique  name  by  adding

"&" characters to the given name.  OFF by default.

SHOW    Show the current settings of the SET parameters.

TAKE    Take  KERMIT-86  commands from the specified file.  The file should not

contain any TAKE commands; nested command files do not work.

LOCAL   This is a prefix for local file  management  commands,  to  distinguish

16.1. Kermit-86 Commands                                               Page 400

-------------------------------------------------------------------------------

them  from  remote  file  management commands (which aren't implemented

yet).  The LOCAL prefix is optional; if left off, the commands will  be

performed locally.

SPACE           Show  how  much space is used and remaining on the cur-

rent disk.

DIRECTORY       Provide a directory listing for the current disk, show-

ing  the name and size of each file.  A filespec may be

given to select only a certain file  or  wildcard  file

group.

DELETE          Delete the specified files from the current disk.

TYPE            A wildcard filespec is accepted and files displayed al-

phabetically.  The display is  paged  in  Unix  fashion

with "--more--" displayed on the last line.  Typein op-

tions at that point can be obtained by hitting a '?'.

16.2. Installation:

CP/M-86 KERMIT is broken up into several source modules:

C86CMD.A86          Command parser

C86FIL.A86          File handler

C86Xxx.A86          System Dependent I/O

C86KER.A86          Main Program

C86PRO.A86          Protocol Module

C86TRM.A86          Terminal Emulation

C86UTL.A86          Utilities

The main program module, C86KER.A86, contains INCLUDE directives for the  other

files.    The  C86Xxx module is stored with "xx" replaced by codes denoting the

machine for which the program is being built -- RB for Rainbow, AP for NEC APC,

etc.    The  program  may  be  built on the CP/M-86 system by obtaining all the

source files listed above, storing them on the current disk with the names  in-

dicated,  renaming  the  appropriate C86Xxx.A86 file to be C86XXX.A86, and then

doing:

ASM86 C86KER $PZ  (takes about 6 minutes on the Rainbow)

GENCMD C86KER     (takes less than a minute)

and, if desired,

REN KERMIT.CMD=C86KER.CMD

16.3. DEC Rainbow 100 Support

Kermit-86 runs on the DEC Rainbow 100 or 100+ under CP/M-86/80, version 1 or 2,

on  the  8088 side.  It uses the built-in firmware to emulate a VT102 ANSI ter-

minal during CONNECT, and runs well at speeds up to 9600 baud.

You should be able to download the program using the old KERMIT on the Z80 side

(Rainbow  Kermit,  VT180 Kermit, or generic CP/M-80 Kermit will do the job, but

16.3. DEC Rainbow 100 Support                                          Page 401

-------------------------------------------------------------------------------

only under DEC CP/M-86/80 version 1.0), or an earlier version of Kermit-86.

If you don't have an earlier version of KERMIT, then follow the directions  for

installing  KERMIT-80  (yes,  KERMIT-80) in the KERMIT-80 section of the Kermit

User Guide, but send the Kermit-86 hex file instead.  This  works  because  the

Rainbow can run CP/M-80 programs like DDT.

Another  way to get Kermit onto your Rainbow for the first time would be from a

DEC VT-180 diskette.  A VT-180 can use its own Kermit to  load  Rainbow  Kermit

onto  its  disk, which can then be read directly by a Rainbow.  Also, note that

VT-180 Kermit-80 can actually run on the Rainbow on  the  Z80  side  under  DEC

CP/M-86/80  version  1 (but not version 2 or higher), at speeds of 1800 baud or

lower.

16.4. NEC Advanced Personal Computer Support

(Contributed by Ron Blanford, University of Washington)

Currently only the standard serial port is supported, and not the H14 auxiliary

port.  The SET PORT command is not implemented.

While  in  Kermit's  terminal emulation mode, local commands are initiated by a

two-character sequence consisting of the "escape  character"  followed  by  one

other  character  identifying the command.  (Make the second character a '?' to

see a list of the valid commands.)  As distributed, the standard Kermit-86 uses

the  control-backslash character as the escape character in terminal mode.  The

trouble is that the CP/M-86 BIOS  in  the  APC  ignores  a  keyboard  entry  of

Control-\  (i.e.  holding down the CTRL key while striking the '\' key), making

it difficult (impossible) to use this method to get out of terminal mode.

One solution is to perform a "SET ESCAPE ^" command  before  entering  terminal

mode  to change the escape character to a caret (or any other character the APC

keyboard will generate).  This command could be placed in your KERMIT.INI  file

for automatic execution every time Kermit is started.

The simpler solution is to realize that the character code for a Control-\ is a

hexadecimal 1C, and that this is the code generated  by  the  INS  key  on  the

numeric keypad.  Once you can remember that every reference to Control-\ should

be interpreted as a reference to the INS key, this is actually  easier  to  use

than the two-key Control-\ sequence.

In  the  standard  CP/M-86  BIOS,  the  unshifted DEL key generates a Control-X

character (hexadecimal 18).  This is the CP/M command to erase the current  in-

put  line, and is very useful for local processing.  Most mainframes do not use

the Control-X character at all, so it becomes much less useful during  terminal

emulation.    The  DEL  character (hexadecimal 7F), on the other hand, is often

used by mainframes and can only be generated on the APC  by  holding  down  the

SHIFT key while striking the DEL key (this capability is not mentioned anywhere

in the documentation).

Because the Control-X character is so seldom used while the  DEL  character  is

commonly  used,  the initialization procedure in Kermit-86 modifies the CP/M-86

BIOS so that the DEL key generates the DEL character whether  shifted  or  not.

Control-X  can  still  be  generated  if necessary by holding down the CTRL key

while striking the 'X' key.  The CP/M-86 BIOS is returned to its original state

16.4. NEC Advanced Personal Computer Support                           Page 402

-------------------------------------------------------------------------------

when Kermit terminates.

The  APC uses escape sequences which have been standardized by the American Na-

tional Standards Institute (ANSI) to control cursor movement,  screen  erasing,

and  character  attribute  manipulation.  Perhaps the best-known other terminal

which follows ANSI guidelines is the DEC VT100.  The APC only recognizes a  few

of  the  more important ANSI commands, and not the complete set which the VT100

supports.

The ANSI/VT100 features that the NEC APC supports are:

- direct cursor addressing (by row and column)

- relative cursor addressing (up, down, left, right)

- line erasing (cursor to end, beginning to cursor, entire line)

- screen erasing (cursor to end, beginning to cursor, entire screen)

- character attributes (underline, reverse video, blink, but not bold)

In addition, the first four grey function keys (unshifted) generate the  escape

sequences  associated  with  PF1  through PF4 on the VT100 keyboard.  The arrow

keys and numeric keypad DO NOT generate the corresponding VT100 sequences.

These functions are enough to support simple command line editing on most  sys-

tems,  and allow mailers or paged file display programs to clear the screen be-

fore each display.  Underlining and reverse video are also useful in  some  ap-

plications.    This is not enough to support the more sophisticated screen con-

trol required by screen editors such as EMACS or KED.  In addition,  due  to  a

bug  in the implementation of the CP/M-86 BIOS, the sequence ordinarily used to

home the cursor (esc [ H) does not work correctly; a patch for CP/M to  correct

this problem is distributed with APC Kermit-86.

I. The ASCII Character Set                                             Page 403

-------------------------------------------------------------------------------

I. The ASCII Character Set

ASCII Code (ANSI X3.4-1968)

There  are 128 characters in the ASCII (American national Standard Code for In-

formation Interchange) "alphabet".  The characters are listed in order of ASCII

value; the columns are labeled as follows:

Bit             Even parity bit for ASCII character.

ASCII Dec       Decimal (base 10) representation.

ASCII Oct       Octal (base 8) representation.

ASCII Hex       Hexadecimal (base 16) representation.

EBCDIC Hex      EBCDIC hexadecimal equivalent for Kermit translate tables.

Char            Name or graphical representation of character.

Remark          Description of character.

The first group consists of nonprintable 'control' characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   000   000   00   00    NUL   ^@, Null, Idle

1   001   001   01   01    SOH   ^A, Start of heading

1   002   002   02   02    STX   ^B, Start of text

0   003   003   03   03    ETX   ^C, End of text

1   004   004   04   37    EOT   ^D, End of transmission

0   005   005   05   2D    ENQ   ^E, Enquiry

0   006   006   06   2E    ACK   ^F, Acknowledge

1   007   007   07   2F    BEL   ^G, Bell, beep, or fleep

1   008   010   08   16    BS    ^H, Backspace

0   009   011   09   05    HT    ^I, Horizontal tab

0   010   012   0A   25    LF    ^J, Line feed

1   011   013   0B   0B    VT    ^K, Vertical tab

0   012   014   0C   0C    FF    ^L, Form feed (top of page)

1   013   015   0D   0D    CR    ^M, Carriage return

1   014   016   0E   0E    SO    ^N, Shift out

0   015   017   0F   0F    SI    ^O, Shift in

1   016   020   10   10    DLE   ^P, Data link escape

0   017   021   11   11    DC1   ^Q, Device control 1, XON

0   018   022   12   12    DC2   ^R, Device control 2

1   019   023   13   13    DC3   ^S, Device control 3, XOFF

0   020   024   14   3C    DC4   ^T, Device control 4

1   021   025   15   3D    NAK   ^U, Negative acknowledge

1   022   026   16   32    SYN   ^V, Synchronous idle

0   023   027   17   26    ETB   ^W, End of transmission block

0   024   030   18   18    CAN   ^X, Cancel

1   025   031   19   19    EM    ^Y, End of medium

1   026   032   1A   3F    SUB   ^Z, Substitute

0   027   033   1B   27    ESC   ^[, Escape, prefix, altmode

1   028   034   1C   1C    FS    ^\, File separator

0   029   035   1D   1D    GS    ^], Group separator

0   030   036   1E   1E    RS    ^^, Record separator

1   031   037   1F   1F    US    ^_, Unit separator

The  last  four  are  usually associated with the control version of backslash,

right square bracket, uparrow (or circumflex),  and  underscore,  respectively,

but some terminals do not transmit these control characters.

I. The ASCII Character Set                                             Page 404

-------------------------------------------------------------------------------

The following characters are printable:

First, some punctuation characters.

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   032   040   20   40    SP    Space, blank

0   033   041   21   5A    !     Exclamation mark

0   034   042   22   7F    "     Doublequote

1   035   043   23   7B    #     Number sign, pound sign

0   036   044   24   5B    $     Dollar sign

1   037   045   25   6C    %     Percent sign

1   038   046   26   50    &     Ampersand

0   039   047   27   7D    '     Apostrophe, accent acute

0   040   050   28   4D    (     Left parenthesis

1   041   051   29   5D    )     Right parenthesis

1   042   052   2A   5C    *     Asterisk, star

0   043   053   2B   4E    +     Plus sign

1   044   054   2C   6B    ,     Comma

0   045   055   2D   60    -     Dash, hyphen, minus sign

0   046   056   2E   4B    .     Period, dot

1   047   057   2F   61    /     Slash

Numeric characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   048   060   30   F0    0     Zero

1   049   061   31   F1    1     One

1   050   062   32   F2    2     Two

0   051   063   33   F3    3     Three

1   052   064   34   F4    4     Four

0   053   065   35   F5    5     Five

0   054   066   36   F6    6     Six

1   055   067   37   F7    7     Seven

1   056   070   38   F8    8     Eight

0   057   071   39   F9    9     Nine

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   058   072   3A   7A    :     Colon

1   059   073   3B   5E    ;     Semicolon

0   060   074   3C   4C    <     Left angle bracket

1   061   075   3D   7E    =     Equal sign

1   062   076   3E   6E    >     Right angle bracket

0   063   077   3F   6F    ?     Question mark

1   064   100   40   7C    @     "At" sign

I. The ASCII Character Set                                             Page 405

-------------------------------------------------------------------------------

Upper-case alphabetic characters (letters):

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   065   101   41   C1    A

0   066   102   42   C2    B

1   067   103   43   C3    C

0   068   104   44   C4    D

1   069   105   45   C5    E

1   070   106   46   C6    F

0   071   107   47   C7    G

0   072   110   48   C8    H

1   073   111   49   C9    I

1   074   112   4A   D1    J

0   075   113   4B   D2    K

1   076   114   4C   D3    L

0   077   115   4D   D4    M

0   078   116   4E   D5    N

1   079   117   4F   D6    O

0   080   120   50   D7    P

1   081   121   51   D8    Q

1   082   122   52   D9    R

0   083   123   53   E2    S

1   084   124   54   E3    T

0   085   125   55   E4    U

0   086   126   56   E5    V

1   087   127   57   E6    W

1   088   130   58   E7    X

0   089   131   59   E8    Y

0   090   132   5A   E9    Z

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   091   133   5B   AD    [     Left square bracket

0   092   134   5C   E0    \     Backslash

1   093   135   5D   BD    ]     Right square bracket

1   094   136   5E   5F    ^     Circumflex, up arrow

0   095   137   5F   6D    _     Underscore, left arrow

0   096   140   60   79    `     Accent grave

Kermit User Guide                                                      Page 406

-------------------------------------------------------------------------------

Lower-case alphabetic characters (letters):

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   097   141   61   81    a

1   098   142   62   82    b

0   099   143   63   83    c

1   100   144   64   84    d

0   101   145   65   85    e

0   102   146   66   86    f

1   103   147   67   87    g

1   104   150   68   88    h

0   105   151   69   89    i

0   106   152   6A   91    j

1   107   153   6B   92    k

0   108   154   6C   93    l

1   109   155   6D   94    m

1   110   156   6E   95    n

0   111   157   6F   96    o

1   112   160   70   97    p

0   113   161   71   98    q

0   114   162   72   99    r

1   115   163   73   A2    s

0   116   164   74   A3    t

1   117   165   75   A4    u

1   118   166   76   A5    v

0   119   167   77   A6    w

0   120   170   78   A7    x

1   121   171   79   A8    y

1   122   172   7A   A9    z

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   123   173   7B   C0     {    Left brace (curly bracket)

1   124   174   7C   4F     |    Vertical bar

0   125   175   7D   D0     }    Right brace (curly bracket)

0   126   176   7E   A1     ~    Tilde

Finally, one more nonprintable character:

0    127   177  7F   07    DEL   Delete, rubout

Kermit User Guide                                                    Page cdvii

-------------------------------------------------------------------------------

List of Figures

Figure 1-1:   A Kermit Packet                                              7

Figure 1-2:   Kermit File Transfer                                         8

Figure 4-1:   Local and Remote Kermits                                    25

Figure 5-1:   MS-Kermit File Transfer Display Screen                      76

Figure 5-2:   MS-Kermit Script for Dialing Up and Logging In             110

Figure 5-3:   MS-DOS Batch File Invoking Kermit to Send VAX Mail         111

Figure 5-4:   MS-Kermit Script for Logging into VAX and Sending Mail     112

Figure 5-5:   An Advanced MS-Kermit Initialization File                  113

Figure 11-1:   DECSYSTEM-20 Word/Byte Organization                       253

Figure 11-2:   DEC-20 Kermit Local Operation                             257

Figure 13-1:   VT100 Keypad on an Apple Keyboard                         332

Figure 13-2:   VT100 Keypad on an Apple//gs or Equivalent Keypad         333

Figure 13-3:   VT52 Keypad on an Apple Keyboard                          334

Figure 14-1:   MacKermit Key Modifier Dialog                             353

Figure 15-1:   Bootstrap program for Kermit-80 and CP/M Version 2.2      379

Kermit User Guide                                                   Page cdviii

-------------------------------------------------------------------------------

List of Tables

Table 5-1:   The US ASCII Character Set (ANSI X3.4-1977)                  61

Table 5-2:   Kermit-MS Single-Character CONNECT Escape Commands           72

Table 5-3:   Adapters Supported by IBM PC MS-Kermit for Tektronix         75

Emulation

Table 5-4:   Kermit-MS Verbs for the IBM PC Family                        97

Table 5-5:   Kermit-MS Terminal Emulation Options                        114

Table 5-6:   Kermit-MS Screen Scroll Keys                                115

Table 5-7:   Kermit-MS Verbs for the DEC Rainbow                         116

Table 5-8:   Response of MS-Kermit Tektronix Emulator to Received        131

Characters

Table 5-9:   Tektronix Dot-Drawing Commands                              133

Table 5-10:   MS-Kermit Tektronix Coordinate Interpretation              135

Table 5-11:   IBM PC/XT/AT Serial Port Numbers                           140

Table 7-1:   Error messages and codes for Kermit-370                     197

Table 12-1:   Kermit-11 File Types                                       301

Table 13-1:   Apple II Communication Cards Supported by Kermit-65        319

Table 13-2:   Kermit-65 Single-Character CONNECT Escape Commands         324

Table 13-3:   Apple II/II+ Keyboard Escapes                              325

Table 15-1:   Kermit-80 SET PORT Options                                 368

Table 15-2:   Systems supported by Kermit-80 (Part 1)                    375

Table 15-3:   Systems supported by Kermit-80 (Part 2)                    376

Table 15-4:   Terminals supported by Kermit-80                           377

Table 15-5:   Terminals known to Kermit-80                               385