THE TRUTH ABOUT KERMIT FILE TRANSFER PERFORMANCE

Frank da Cruz

In the early 1980s, the first generation of Kermit software used the basic

Kermit protocol: stop-and-wait exchange of short packets.  The basic protocol

is easily implemented and highly robust, and led to its rapid proliferation to

hundreds of hardware and software platforms where it proved a success in

transferring files under even the most difficult conditions.

The new generation of Kermit software improves on the original robust

qualities and dramatically boosts performance without sacrificing

compatibility with the earlier generation.  Protocol capabilities are

negotiated automatically so the newest, most advanced versions can still

exchange files with the earliest or most minimal versions.

Kermit's performance features include long packets, sliding windows,

control-prefixing selection, locking shifts, and compression.  The first three

have the potential for causing problems, and are not used unless you ask for

them.  This article describes Kermit's performance features and tests them

against other popular protocols.  The results might surprise you.

Long Packets and Sliding Windows

The maximum packet length in the basic Kermit protocol is 94, chosen to

prevent data loss when the receiver has small input buffers or lacks an

adequate method of flow control, and also to reduce vulnerability to noise.

But since 1985, Kermit's long-packet extension has allowed packets up to 9024

bytes in length to be used when conditions permit.

Longer packets reduce the ratio of protocol overhead to actual data,

increasing the potential file transfer efficiency (the ratio of file

characters transferred per second to the actual connection speed) from 86%

(for 94-byte packets) to 95% (with 2500-byte packets).  When conditions

deteriorate on the connection, the packet length is automatically adjusted.

Original, basic Kermit was a stop-and-wait protocol because it had to work on

half-duplex as well as full-duplex connections.  But connections through

satellites or packet-switched networks can have delays that seriously impede

the efficiency of a stop-and-wait packet protocol.  For example, suppose

packets are 90 bytes = 900 bits long, and there is a one-second transmission

delay.  For one packet and its response, the round-trip delay is 2 seconds.

At 300 bits per second (bps), the 3 seconds required to transmit the packet

plus the 2-second delay make 5 seconds, so throughput is 180 bps, 60%

efficiency.  At 9600 bps, it takes only 1/10 second to transmit the same

packet, but the delay is still 2 seconds.  Throughput is only 428 bps, 4.5%

efficiency.  When connections have delays, efficiency can be improved by

lengthening the packets, but only if the connection is clean.  On a noisy

connection, longer packets are more likely to be damaged in transmission and

take longer to retransmit.

On full-duplex connections, the new generation of Kermit software (IBM

mainframe Kermit excluded, which always has a half-duplex connection) can

transmit packets in a steady stream, processing the acknowledgements later as

they arrive, thus eliminating the effects of transmission delays, and also

eliminating the overhead of the acknowledgements themselves, since they are

"on the wire" at the same time as the data packets and therefore don't take up

any extra transmission time.  This technique is called sliding windows,

because multiple packets are kept in a buffer (window) that "slides" forward

whenever the oldest packet in the window is acknowledged.

Using 94-byte packets without sliding windows on a connection that has a

1-second delay results (according to actual measurements) in an efficiency of

about 8%.  Raising the packet length to 1500 on the same connection increases

the efficiency to 63%.  Using sliding windows on the same connection raises

the efficiency to 82-90%, depending on the packet length.

------------------------------------------------------------------------

Optimum performance can be achieved on any given connection by choosing

the right combination of packet length and window size.

------------------------------------------------------------------------

To see a dramatic speed improvement using MS-DOS Kermit 3.13 and/or C-Kermit

5A, simply give these commands to each Kermit before file transfer:

SET WINDOW 3

SET RECEIVE PACKET-LENGTH 1000

Adjust as necessary.  Longer delays require larger windows; noisier

connections (or devices with small input buffers) need shorter packets.

MS-DOS Kermit 3.13 and most versions of C-Kermit 5A allow the theoretical

maximum sizes, 31 and 9024 respectively, sufficient to overcome any reasonable

delay (for example, between the earth and the moon).

Compression

To reduce transmission overhead, the Kermit protocol uses a simple, but often

surprisingly effective, compression technique: repeated byte values are

represented by a count+byte combination.  This technique is easy to program

and inexpensive in memory and CPU cycles, and is therefore implemented in most

Kermit software, including MS-DOS Kermit, C-Kermit, and IBM mainframe Kermit,

and is used automatically when available.

Analysis of large volumes of both textual and binary data shows an average

compression of 15-20%.  Dramatic savings are achieved in certain types of

files, including tabular text (or any other type of text with lots of repeated

characters) and executable programs containing large amounts of pre-zeroed

data.

Prefixing

To achieve its ability to push data through even the most restrictive types of

connections--for example, to mainframes that are sensitive to certain control

characters, or on 7-bit connections, or on very noisy ones (one user said

recently, "Kermit will send data over a communication channel that is only

slightly better than a pair of tin cans connected with a wet string")--Kermit

formats its packets as lines of printable text.  This is done by encoding each

control character as a sequence of two printable characters and, on 7-bit

connections only, encoding 8-bit characters as a sequence of two printable

7-bit bytes.

On some connections it is safe to transmit certain control characters "bare,"

without prefixing or encoding.  "Unprefixing" of control characters can speed

up the transfer of binary files, particularly precompressed files, which tend

to contain a lot of bytes in the control range.  MS-DOS Kermit 3.13 and

C-Kermit 5A(189) give you the ability to specify which control characters are

to be prefixed and which are not.  In the benchmarks on pages 7 and -SPEEDY,

only three control characters are prefixed:

SET CONTROL UNPREFIXED ALL

SET CONTROL PREFIXED 0 1 129

This technique can be used even if the Kermit program on the other end doesn't

know anything about it, since well-designed Kermit software will, indeed,

accept bare control characters literally.  The three exceptions above are NUL

(0), which is used internally by C-Kermit for string termination, and SOH (1)

and SOH+parity (129), Kermit's normal packet-start indicator.  It takes some

experimentation to find the maximum safe set.  That's why Kermit prefixes all

control characters by default: first make it work, then make it fast.

On 8-bit connections, Kermit transfers 8-bit data with no additional overhead.

On 7-bit connections, which are quite common--these are the connections that

use even, odd, mark, or space parity, often without the user's

knowledge--8-bit data is encoded using a single-shift technique, a prefix

character for each byte whose 8th bit is 1, similar to holding down the

------------------------------------------------------------------------

The Kermit protocol implementations found in many of the popular

commercial and shareware PC communication software packages are minimal

and perfunctory, usually lacking some or all of the performance

features...

------------------------------------------------------------------------

Shift key on your keyboard for each 8-bit character.  This allows Kermit to

work where most other protocols fail. The amount of prefixing ranges from 0%

up to 100%, depending on the type of file.

Locking Shifts

To avoid the high overhead of transferring 8-bit text, particulary Cyrillic,

Hebrew, or Kanji, on 7-bit connections, a new "locking-shift" feature works

like the Caps Lock key on your PC:  a special shift prefix applies to a entire

run of 8-bit characters, no matter how long, rather than to a single

character.  Locking shifts are used in combination with single shifts to

achieve the most compact encoding.

Locking shifts are supported by MS-DOS Kermit 3.13, C-Kermit 5A, and IBM

Mainframe Kermit 4.2.4, and are negotiated automatically when parity is in use

(including when parity is detected automatically).  They reduce the 8th-bit

prefixing penalty anywhere from 0% to 100%, depending on the groupings of the

8-bit characters within the file.

So Why the Bad Reputation?

The Kermit protocol implementations found in many of the popular commercial

and shareware PC communication software packages are minimal and perfunctory,

usually lacking some or all of the performance features just described.  Many

of these same packages also include XMODEM, YMODEM, or ZMODEM protocol, which

(when they work at all) usually perform better than the basic short-packet,

stop-and-wait, prefix-everything Kermit protocol.  Using a limited Kermit

implementation is like filling your bathtup from a dripping faucet instead of

turning the water on full blast.  It is easy to see why users of such packages

might conclude that Kermit file transfers are slow.  Nothing could be further

from truth; turn the page and see for yourself.

TRUE-LIFE BENCHMARKS

Table 1 illustrates the performance of the Kermit protocol implementations

found in different PC software packages.  These measurements were made on a

direct 19200-bps serial connection, downloading a typical ASCII text file (the

VM/CMS Kermit-370 manual), 135087 bytes in length, from a Sun SPARCserver-10

with C-Kermit 5A(189) to the hard disk of an IBM PS/2 Model 70.

Table 1:  Kermit Implementations Compared

------------------------------------------------------------------------------

Window Packet Time Speed

PC Software    Size  Length secs  cps  Effic. Remarks

------------------------------------------------------------------------------

Telix           1     94   131   1052   55%  Long packets and s/w not avail

MTEZ            1     94   119   1158   60%  Long packets and s/w not avail

Smartcom III    1     94   113   1220   64%  Long packets and s/w not avail

PROCOMM PLUS   14   1000    77   1790   93%  Window size not selectable

Zstem 340       2   1000    74   1863   97%  Maximum window size 2

MS-DOS Kermit   3   1000    72   1915   99%  Full control-character prefixing

MS-DOS Kermit   3   1000    69   1999  104%  Only 0, 1, and 129 prefixed

------------------------------------------------------------------------------

The results speak for themselves.

If you thought Kermit file transfer was slow, you were probably not

using real Kermit software!

The UNIX-resident copy of the file, like all UNIX text files, uses only

linefeed (LF) for line termination.  During text-mode transfer, each LF

becomes carriage return and linefeed (CRLF).  There are 2814 lines in the

file, so the actual data size during (and after) transfer is 137901.  Since

the connection runs at 1920 characters per second (10 bits per character), a

100%-efficiency transfer should take 137901 / 1920 = 71.8 seconds.  The

following PC communications software was used:

MS-DOS Kermit 3.13     Columbia University, New York, NY, USA

MTEZ 1.16              MagicSoft, Inc., Lombard, IL, USA

PROCOMM PLUS 2.0       Datastorm Technologies, Inc., Columbia, MO, USA

Smartcom III 1.0A      Hayes Microcomputer Products, Inc, Norcross, GA, US

Telix 3.21             deltaComm Development, Cary, NC, USA

Zstem 340 1.0.4        KEA Systems Ltd., Burnaby, BC, Canada

Kermit and X-Y-ZMODEM

XMODEM, YMODEM, and ZMODEM are the file tranfer protocols most commonly

compared with Kermit, and which are found in numerous shareware and commercial

communication software packages.  XMODEM and YMODEM are stop-and-wait

protocols; XMODEM uses short blocks (128 data bytes), YMODEM uses longer ones

(1024 data bytes).  ZMODEM is a streaming protocol.

The tables on page 8 compare XMODEM, YMODEM, ZMODEM, and Kermit transfers

between the PC and UNIX.  The file transfer software on the UNIX system is sx

(XMODEM) / sb (YMODEM) / sz (ZMODEM) 3.24 (June 1993) and C-Kermit 5A(189).

On the PC, X-, Y- and ZMODEM transfers were done with Telix and PROCOMM PLUS

(which gave exactly the same results).  For fairness, four types of files are

transferred:

ASCII Text:    IKCKER.DOC 137901 bytes  Our original ASCII text file

UNIX Binary:   uuencode    24576 bytes  A Sun SPARC binary executable program

PC Binary:     KERMIT.EXE 197928 bytes  An MS-DOS binary executable program

Precompressed: KERMIT.ZIP 238584 bytes  A compressed ZIP archive

Tests were performed on four types of connections and in each trial, Kermit

transfers used a window size of 5 and a packet length of 5000, and control

prefixing was disabled except for NUL (0), Ctrl-A (1), and 129.  As the tables

show, Kermit outperforms the competition every time.

Table 2 shows the figures for transferring all four files with each of the

four protocols on same direct connection used for Table 1.  In this and the

following tables, the secs column shows the elapsed time of transfer in

seconds, the cps column shows actual file characters transferred per second,

and the eff column shows the percent efficiency (file characters per second

divided by the connection speed).

Table 2:  X- Y- and ZMODEM vs Kermit on a 19200-bps Direct Connection

------------------------------------------------------------------------------

XMODEM          YMODEM          ZMODEM           KERMIT

File Type     secs  cps  eff  secs  cps  eff  secs  cps  eff   secs  cps  eff

------------------------------------------------------------------------------

ASCII Text      89 1549  81%    76 1814  95%    73 1889  98%     69 1999 104%

UNIX Binary     15 1638  85%    13 1890  98%    13 1890  98%      9 2648 138%

PC Binary      127 1558  81%   109 1816  95%   107 1850  96%    100 1979 103%

Precompressed  153 1559  81%   133 1794  93%   130 1835  96%    129 1849  96%

-------------------------------------------------------------------------------

Table 3 shows the results for a local-call dialup connection using Telebit

T3000 modems, V.32bis modulation (14400 bps), V.42 error correction, V.42bis

compression, RTS/CTS hardware flow control, and an interface speed of 57600

bps.  The efficiencies in this table are based on the modem's 14400-bps

connection speed, and therefore also reflect the modem's compression methods.

Table 3:  X- Y- and ZMODEM vs Kermit with High-Speed Modems

------------------------------------------------------------------------------

XMODEM          YMODEM          ZMODEM           KERMIT

File Type     secs  cps  eff  secs  cps  eff  secs  cps  eff   secs  cps  eff

------------------------------------------------------------------------------

ASCII Text     221  624 43%     79 1746 121%    42 3283 228%     39 3535 246%

UNIX Binary     32  768 53%     13 1890 131%    15 1638 114%      3 8192 569%

PC Binary      346  572 40%    129 1534 106%    83 2385 166%     80 2474 172%

Precompressed  500  477 33%    208 1147  79%   149 1601 111%    148 1612 112%

------------------------------------------------------------------------------

So far we've looked only at connections with no delays.  Table 4 (also see

cover, left group) shows the results for a V.32 9600-bps cross-country dialup

connection from the same PC to a PC/486-50 running UNIX System V R4, with the

same C-Kermit, sx, sb, and sz software as on the Sun.  The round-trip delay is

a fraction of a second.  No error correction or compression is done by the

modems, but the connection is clean and no errors occurred.

Table 4:  X- Y- and ZMODEM vs Kermit with Delays at 9600 bps

------------------------------------------------------------------------------

XMODEM          YMODEM          ZMODEM           KERMIT

File Type     secs  cps  eff  secs  cps  eff  secs  cps  eff   secs  cps  eff

------------------------------------------------------------------------------

ASCII Text     422  327  33%   253  545  57%   217  635  66%    151  913  95%

UNIX Binary     73  337  35%    41  599  62%    32  768  80%      8 3072 320%

PC Binary      536  369  38%   319  620  65%   271  730  76%    207  956  99%

Precompressed  710  336  37%   363  657  68%   314  759  79%    284  840  87%

------------------------------------------------------------------------------

But if we always had clean connections, why would we need error-correcting

file-transfer protocols?  Table 5 (and middle group, cover) shows the results

for the same cross-country connection, same settings, but with short bursts of

noise injected every two seconds, which cause errors and retransmissions in

all four protocols.

Table 5:  X- Y- and ZMODEM vs Kermit with Delays and Noise at 9600 bps

------------------------------------------------------------------------------

XMODEM          YMODEM          ZMODEM           KERMIT

File Type     secs  cps  eff  secs  cps  eff  secs  cps  eff   secs  cps  eff

------------------------------------------------------------------------------

ASCII Text    3346   41   4%  fail    0   0%   438  315  33%    206  669  70%

UNIX Binary    573   43   4%    58  424  44%   144  171  18%      9 2736 284%

PC Binary     5154   42   4%  fail    0   0%   566  350  36%    281  706  74%

Precompressed 5917   40   4%  fail    0   0%   694  344  36%    385  621  65%

------------------------------------------------------------------------------

What about 7-Bit Connections?  No Contest!

The foregoing benchmarks were conducted in environments where XMODEM, YMODEM,

and ZMODEM can work, namely 8-bit transparent connections that are not

sensitive to any control characters.  Now let's look a different, but very

common, situation.  Table 6 (and right group, cover) shows the results of

downloading the same files from an IBM Mainframe running VM/CMS and Kermit-370

4.2.5 to the PS/2 over a 19200-bps serial connection through an IBM 7171

protocol converter, which uses even parity and Xon/Xoff flow control.

Kermit's window size is 1 because the mainframe can operate only in half

duplex, and the packet length is 1920, the largest allowed by the 7171.  All

control characters are prefixed.

Table 6:  File Transfer on a 7-Bit Connection

------------------------------------------------------------------------------

XMODEM          YMODEM          ZMODEM           KERMIT

File Type     secs  cps  eff  secs  cps  eff  secs  cps  eff   secs  cps  eff

------------------------------------------------------------------------------

ASCII Text      -     0   0%    -     0   0%    -     0   0%     81 1702  88%

UNIX Binary     -     0   0%    -     0   0%    -     0   0%      9 2730 142%

PC Binary       -     0   0%    -     0   0%    -     0   0%    162 1221  63%

Precompressed   -     0   0%    -     0   0%    -     0   0%    243  981  51%

------------------------------------------------------------------------------

The table shows Kermit file transfer to be infinitely more efficient than

X-Y-Z-MODEM transfer with IBM mainframes, because X-Y-Z-MODEM implementations

do not work with IBM mainframe operating systems such as VM/CMS, MVS/TSO, or

CICS, whereas Kermit works with all of them.  Of course, 7-bit connections are

not peculiar to IBM mainframes.  They are also used by other types of

mainframes and front ends as well as many types of networks and devices,

including some X.25-based public data networks and certain terminal servers.

You can use Kermit to transfer files on these connections, but not X-Y-Z-MODEM

protocols.

Locking Shifts

Although Kermit, unlike X-Y-Z-MODEM, can transfer 8-bit data over 7-bit

connections, there is often a performance penalty.  This penalty is

particularly unfair to people whose written languages are encoded primarily in

8-bit characters, as are Russian, Hebrew, and Japanese.  Russian text encoded

in any of the commonly used 8-bit Cyrillic character sets typically consists

of about 80%  8-bit characters and Japanese Kanji text often consists of

nearly 100% 8-bit characters.

Table 7 shows the results of attempting to upload typical Russian and Japanese

8-bit text files over a 19200-bps 7-bit serial connection to an IBM mainframe

using X-Y-Z-MODEM (it can't be done), Kermit with only single shifts (SS), and

Kermit with locking shifts (LS).  The Kermit window size is 1 and the packet

length is 1920.  In these cases, locking shifts improve the speed of transfer

30-40%.

Table 7:  Effect of Locking Shifts

-------------------------------------------------------------------------------

X-Y-Z-MODEM       KERMIT (SS)       KERMIT (LS)

File Type      Size      secs  cps  eff    secs  cps  eff    secs  cps  eff

-------------------------------------------------------------------------------

Russian Text   52046       -     0   0%      55  946  49%      39 1334  69%

Japanese Text  29706       -     0   0%      34  873  45%      20 1485  77%

-------------------------------------------------------------------------------

Conclusion

Kermit protocol works in practically every communication and computing

environment.  You don't have to be a data communications expert to transfer a

file with Kermit software.  Its first priority is getting your data through

safe and sound, and its second is efficiency.  Kermit's conservative protocol

defaults reflect these priorities:   First make it work, then make it fast.

But as the tests show, today's Kermit software, when given several simple

commands to enable its efficiency features, outperforms X-, Y-, and ZMODEM

protocol transfers every time.  And real Kermit software also outperforms the

Kermit protocol implementations found in commercial and shareware

communications programs.  Skeptical?  Run your own tests!