/****************************************************************************

** $Id: qt/doc/qws.doc   2.2.2   edited 2000-10-03 $

**

** Qt/Embedded (Qt on QWS) documentation

**

** Copyright (C) 1992-2000 Trolltech AS.  All rights reserved.

**

** This file is part of the Qt GUI Toolkit.

**

** This file may be distributed under the terms of the Q Public License

** as defined by Trolltech AS of Norway and appearing in the file

** LICENSE.QPL included in the packaging of this file.

**

** This file may be distributed and/or modified under the terms of the

** GNU General Public License version 2 as published by the Free Software

** Foundation and appearing in the file LICENSE.GPL included in the

** packaging of this file.

**

** Licensees holding valid Qt Enterprise Edition or Qt Professional Edition

** licenses may use this file in accordance with the Qt Commercial License

** Agreement provided with the Software.

**

** This file is provided AS IS with NO WARRANTY OF ANY KIND, INCLUDING THE

** WARRANTY OF DESIGN, MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.

**

** See http://www.trolltech.com/pricing.html or email sales@trolltech.com for

**   information about Qt Commercial License Agreements.

** See http://www.trolltech.com/qpl/ for QPL licensing information.

** See http://www.trolltech.com/gpl/ for GPL licensing information.

**

** Contact info@trolltech.com if any conditions of this licensing are

** not clear to you.

**

**********************************************************************/

/*! \page install-qws.html

\title Installing Qt/Embedded

<p>

This installation procedure is written for Linux. It may need

to be modified for other platforms.

<h3>  Unpack the archive if you have not done so already:</h3>

<pre>

cd &lt;anywhere&gt;

gunzip qt-embedded-<i>version</i>-commercial.tar.gz    # uncompress the archive

tar xf qt-embedded-<i>version</i>-commercial.tar       # unpack it

</pre>

This document assumes that the archive is installed as <tt>~/qt-<i>version</i></tt>.

</p>

<h3> Compile the Qt/Embedded library and examples</h3>

<pre>

cd ~/qt-<i>version</i>

export QTDIR=~/qt-<i>version</i>

./configure

make

</pre>

<p>

The configuration system is designed to allow platform-specific options

to be added, but in general all Linux system which have framebuffer

support can use the "linux-generic-g++" platform.

The configuration system also supports cross-compilers:

to build <i>on</i> Linux/x86 <i>for</i> the Linux/MIPSEL target, you would use:

<pre>

./configure -platform linux-x86-g++ -xplatform linux-mips-g++

</pre>

<p>

Only a small number of configurations are predefined, all much the same.

Configurations files are found in <tt>configs/</tt>.

<h3> Enable framebuffer support</h3>

<p>

You may need to recompile your kernel to enable the framebuffer.

This document does not describe how to do this; the

<a href=HOWTO-framebuffer.html>HOWTO-Framebuffer page</a>

contains a short description. (You should see

a penguin logo at boot time when the frame buffer is enabled.)

<p>

For Matrox G100/G200/G400 use the matrox frame buffer driver.

<p>

For NVidia TNT cards use the nvidia frame buffer driver.

<p>

For Mach64 and most other cards, use the vesafb driver.

<p>

Note that some cards are only supported in VGA16 mode, this will

not work with the current version of Qt/Embedded, since VGA/16 is

not yet supported. You may need to upgrade your kernel, or even

switch to an experimental kernel.

<p>

The frame buffer must also be enabled with a boot parameter. See

<tt>/usr/src/linux/Documentation/fb</tt> for details.

<p>

The fbset program, which should be included in Linux distributions,

may be used to switch video modes without rebooting the system. The

video mode active when the server is started will be used. (8-bit

modes are still experimental.) NOTE: fbset does not work with the vesafb

driver.

</p>

<h3> Change permissions</h3>

<p>

To run Qt/Embedded, you need write access to the framebuffer device

<tt>/dev/fb0</tt>.

<p>

You also need read access to the mouse device. (Note that

<tt>/dev/mouse</tt> is normally a symbolic link; the actual mouse device

must be readable.)

</p>

<h3> How to run the demonstration program</h3>

<p>

Log into a virtual console and do:

<pre>

cd ~/qt-<i>version</i>/

./start-demo

</pre>

<h3> Miscellaneous troubleshooting and known bugs</h3>

<p>

To kill gpm, run the following command as root:

<pre>

gpm -k

</pre>

<p>

In some cases, if the server does not work, it will work when run as root.

<p>

Some example programs may not compile with GCC 2.95.

<p>

Show processes using the framebuffer:

<pre>

fuser -v /dev/fb0

</pre>

<p>

Kill such processes:

<pre>

fuser -vk /dev/fb0

</pre>

or harsher:

<pre>

fuser -k -KILL /dev/fb0

</pre>

<p>

Show existing semaphores:

<pre>

ipcs

</pre>

<p>

Remove semaphores:

<pre>

ipcrm

</pre>

<p>

The communication between client and server is done through the

named pipe <tt>/tmp/.QtEmbedded</tt>; sometimes it may need to be deleted

(eg. if you run Qt/Embedded as root then later as an unprivileged user).

</p>

<h3> Customization  </h3>

<p>

The Qt/Embedded library can be reduced in size by

<a href=features.html>removing unneeded features</a>.

*/

/*! \page fonts-qws.html

\title Fonts in Qt/Embedded

<h2>Supported formats</h2>

<p>

Qt/Embedded supports four font formats:

<ul>

<li><b>TrueType (TTF)</b> - the scalable font technology now standard

on MS-Windows and Apple Macintosh, and becoming popular on X11.

<li><b>Postscript Type1 (PFA/PFB)</b> - scalable fonts often used by printers,

also popular on X11. These are similar in functionality to TTF

fonts and are not discussed further in this document.

<li><b>Bitmap Distribution Format fonts (BDF)</b> - a standard format

for non-scalable fonts. A large number of BDF fonts are

supplied as part of standard X11 distributions - most of

these can be used with Qt/Embedded.

<li><b>Qt Prerendered Font (QPF)</b> - a light-weight non-scalable font

format specific to Qt/Embedded.

</ul>

<p>

Support for each of these font formats,

except QPF which is always enabled,

can be enabled or disabled independently by using the

<a href=features.html>Qt/Embedded Features Definition</a>

There is support in

Qt/Embedded for writing a QPF font file from any font, thus you can

initially enable TTF and BDF formats, save QPF files for the fonts and

sizes you need, then remove TTF and BDF support.

<h2>Memory requirements</h2>

<p>

With TTF fonts, each character in the font at a given point size is

only rendered when first used in a drawing or metrics operation. With

BDF fonts all characters are rendered when the font is used.

With QPF fonts, the characters are stored in the same format as Qt uses

when drawing.

<p>

As an example, a 10-point Times font containing the ASCII characters uses

around 1300 bytes when stored in QPF format.

<p>

Taking advantage of the way the QPF format is structured, Qt/Embedded

memory-maps the data rather than reading and parsing it.

This reduces the RAM consumption even further.

<p>

The scalable fonts use a larger amount of memory per font, but

these fonts can give a memory saving if many different sizes of each

font are needed.

<h2>Smooth fonts</h2>

<p>

TTF, PFA, and QPF fonts can be rendered as <i>smooth</i> anti-aliased

fonts to give superior readability, especially on low-resolution devices.

The difference between smooth and non-smooth fonts is illustrated below

(you may need to change your display to low resolution to see the difference).

<center>

<table>

<tr align=center>

<td><img src=smooth.png>

<td width=10%>

<td><img src=unsmooth.png>

</table>

</center>

<p>

In Qt/Embedded 2.2.1, smooth fonts use 8 times as much memory as non-smooth

fonts. This multiplier will be reduced to a configurable

2 or 4 (ie. 4-level and 16-level shading rather than the current excessive

256-level shading).

<h2>Unicode</h2>

All fonts used by Qt/Embedded use the Unicode character encoding. Most fonts

available today do not actually contain all the Unicode characters, but they

do use the Unicode characters ordering. A single <i>complete</i> 16-point

Unicode font uses over 1Mb of memory.

<h2>The font definition file</h2>

When Qt/Embedded applications run, they look for a file called

<tt>$QTDIR/etc/fonts/fontdir</tt> or

<tt>/usr/local/qt-embedded/etc/fonts/fontdir</tt>. This file defines the

fonts available to the application. It has the following format:

<p>

<blockquote>

<i>name</i> <i>file</i> <i>renderer</i> <i>italic</i> <i>weight</i> <i>size</i> <i>flags</i>

</blockquote>

<p>

where:

<blockquote>

<p>

<table>

<tr><td valign=top>      <i>name</i> <td> <b>Helvetica</b>, <b>Times</b>, etc.

<tr><td valign=top>      <i>file</i> <td> <b>helvR0810.bdf</b>, <b>verdana.ttf</b>, etc.

<tr><td valign=top>  <i>renderer</i> <td> <b>BDF</b> or <b>TTF</b>

<tr><td valign=top>    <i>italic</i> <td> <b>y</b> or <b>n</b>

<tr><td valign=top>    <i>weight</i> <td> <b>50</b> is Normal, <b>75</b> is Bold, etc.

<tr><td valign=top>      <i>size</i> <td> <b>0</b> for scalable or 10 times pointsize (eg. <b>120</b> for 12pt)

<tr><td valign=top>     <i>flags</i> <td> Flag characters:<br>

<b>s</b> = smooth (anti-aliased)<br>

<b>u</b> = unicode range when saving (default is Latin 1)<br>

<b>a</b> = ascii range when saving (default is Latin 1)<br>

</table>

</blockquote>

<p>

The font definition file does not specify QPF fonts - these are loaded

directly from the directory containing the <tt>fontdir</tt> file, and must

be named <i>name</i>_<i>size</i>_<i>weight</i><i>italicflag</i>.qpf, where:

<blockquote>

<p>

<table>

<tr><td valign=top>      <i>name</i> <td> <b>helvetica</b>, <b>times</b>, etc. (lowercase)

<tr><td valign=top>      <i>size</i> <td> 10 times pointsize (eg. <b>120</b> for 12pt)

<tr><td valign=top>    <i>italicflag</i> <td> <b>i</b> for italic or nothing.

<tr><td valign=top>    <i>weight</i> <td> <b>50</b> is Normal, <b>75</b> is Bold, etc.

</table>

</blockquote>

<p>

If an application is run with the <tt>-savefonts</tt> command-line option,

then whenever a font other than a QPF font is used, a corresponding QPF file

is saved. This allows you to easily find the font usage of your applications

and to generate QPF files so that you can eventually reduce the memory

usage of your applications by disabling TTF and BDF support from Qt/Embedded.

option, or by modifying the initialization of <tt>qws_savefonts</tt> in

<tt>kernel/qapplication_qws.cpp</tt> of the Qt/Embedded library source code.

In extreme cases of memory-saving, it is possible to save partially-rendered

fonts (eg. only the characters in "Product Name<sup>TM</sup>") if you are

certain that these are the only characters you will need from the font.

See QMemoryManager::savePrerenderedFont() for this functionality.

<h2>Notes</h2>

<p>

The font definition file, naming conventions for font files, and the format

of QPF files may change in versions of Qt/Embedded after 2.2.1.

<p>

When enabled, Qt/Embedded uses the powerful FreeType2 library to implement

TrueType and Type1 support.

*/

/*! \page running-qws.html

\title Running Qt/Embedded applications

A Qt/Embedded application requires a master application to be running

or to be a master application itself.  The

master application is primarily responsible for managing top-level window

regions, pointer and keyboard input.

<p>

Any Qt/Embedded application can be a

master application by constructing the QApplication object with the

<i>QApplication::GuiServer</i> type, or running the application with the

<i>-qws</i> command line option.

<p>

This document assumes you have the Linux framebuffer configured correctly

and no master process is running.  If you do not have a working Linux

framebuffer you can use the

<a href="qvfb-qws.html">Qt/Embedded virtual framebuffer</a>.

<p>

Change to a Linux console and select an example to run, e.g. examples/widgets.

Make sure $QTDIR is set to the directory where you installed Qt/Embedded

and add the $QTDIR/lib directory to $LD_LIBRARY_PATH, e.g.:

<pre>

export QTDIR=$HOME/qt-<i>version</i>

export LD_LIBRARY_PATH=$QTDIR/lib:$LD_LIBRARY_PATH

</pre>

<p>

Run the application with the <i>-qws</i> option:

<p>

<pre>

cd $QTDIR/examples/widgets

./widgets -qws

</pre>

<p>

You should see the widgets example appear.  If your mouse doesn't work

correctly you need to specify the type of mouse to use.

You can exit the master application at any time using ctrl+alt+backspace.

<p>

If you wish to run additional applications you should run them as clients

i.e. without the <i>-qws</i> option.

<h2>Displays</h2>

Qt/Embedded allows multiple displays to be used simultaneously by running

multiple Qt/Embedded master processes.  This is achieved using the -display

command line parameter or the $QWS_DISPLAY environment variable.

<p>

The -display parameter's syntax is:

<p>

<tt>[gfx driver][:driver specific options][:display number]</tt>

<p>

for example if you want to use the mach64 driver on fb1 as display 2:

<p>

<tt>$ ./launcher -display Mach64:/dev/fb1:2</tt>

<p>

To try this functionality you can do the following:

<ol>

<li>Change to VC 1 and run the launcher:<br>

<tt>$ cd examples/launcher</tt><br>

<tt>$ ./launcher</tt>

<li>Switch to VC 2 and run another one:<br>

<tt>$ cd examples/launcher</tt><br>

<tt>$ ./launcher -display :1</tt><p>

Another launcher will be started.  Start an application in this launcher.

<li>Press ctrl+alt+F1 - back to display 0.  You can also start additional

applications on a particular display by specifying the display id. Change

to VC 3:<br>

<tt>$ cd examples/widgets</tt><br>

<tt>$ ./widgets -display :1</tt><br>

will display the widgets example on dislpay :1 (VC 2).

</ol>

<p>

Only the master process needs to specify the driver/device part explicitly.

The clients get the information they need from the master when they connect.

So once you have a master server running using a particular driver, you can

just use "client -display :n" to use display n.

<h2>Mouse Input</h2>

At the time of writing Qt/Embedded supports MouseMan (default), Microsoft,

IntelliMouse and some other devices specific to certain hardware (e.g. Vr

touch panel).  To specify the mouse to use set the $QWS_MOUSE_PROTO environment

variable, e.g.:<p>

<pre>

export QWS_MOUSE_PROTO=IntelliMouse

</pre>

<p>

See <a href="pointer-qws.html">Qt/Embedded Pointer Handling</a> for more

information.

*/

/*! \page porting-qws.html

\title Porting your applications to Qt/Embedded

Existing Qt applications should require no porting provided there is no

platform dependent code.  Platform dependent code includes system calls,

calls to the underlying window system (Windows or X11), and Qt platform

specific methods such as QApplication::x11EventFilter().

<p>

For cases where it is necessary to use platform dependent code there are

macros defined that can be used to enable/disable code for each platform

using #ifdef directives:

<table>

<tr><th>Platform</th><th>Macro</th></tr>

<tr><td>Qt/X11</td><td>_WS_X11_</td></tr>

<tr><td>Qt/Windows</td><td>_WS_WIN_</td></tr>

<tr><td>Qt/Embedded</td><td>_WS_QWS_</td></tr>

</table>

*/

/*! \page pointer-qws.html

\title Qt/Embedded Pointer Handling

<p>

Pointer handling in Qt/Embedded works for any mouse-like device such as

a touchpanel, a trackball, or real mouse.

<p>

In a real device, only a small number of pointer devices (usually one)

would be supported, but for demonstration purposes, Qt/Embedded includes

a large number of supported devices.

<h2>Mouse Protocols</h2>

<p>

Qt/Embedded normally auto-detects the mouse type and device if it is one of

the supported types on <tt>/dev/psaux</tt> or one of the <tt>/dev/ttyS?</tt>

serial lines. If multiple mice are detected, all may be used simultaneously.

<p>

Alternatively, you may set the environment variable QWS_MOUSE_PROTO to

determine which mouse to use. This environment variable may be set to:

<blockquote>

<i>&lt;protocol&gt;</i><tt>:</tt><i>&lt;device&gt;</i>

</blockquote>

where <i>&lt;protocol&gt;</i> is one of:

<ul>

<li>MouseMan

<li>IntelliMouse

<li>Microsoft

</ul>

and <i>&lt;device&gt;</i> is the mouse device, often <tt>/dev/mouse</tt>.

If no such variable is specified, the built-in default

is <tt>Auto</tt> which enables auto-detection of the mouse protocol

and device.

<p>

To add another protocol, new subclasses of QAutoMouseSubHandler or

QMouseHandler can be written in <tt>kernel/qwsmouse_qws.cpp</tt>.

<h2>Touch Panels</h2>

Qt/Embedded ships with support for the NEC Vr41XX touchpanel

and the iPAQ touchpanel.

These are subclasses of QCalibratedMouseHandler which is in turn

a subclass of QMouseHandler in <tt>kernel/qwsmouse_qws.cpp</tt>.

*/